3GPP TSG-RAN WG1 Meeting #112	R1-2301702

Athens, Greece, February 27 – March 3, 2023

Agenda Item:	5

Source:	Huawei, HiSilicon, CBN

Title:	Discussion on SPS configuration for unicast and multicast

Document for:	Discussion and Decision

Introduction

During RAN2 meeting#120, RAN2 discussed unicast and multicast SPS configuration and reached the following agreements for Rel-17 NR MBS:

Check with RAN1 whether NW can configure SPS in one BWP using sps-Config and sps-ConfigMulticastToAddModList-r17 simultaneously. Check with RAN1 whether other indexes than 0 can be used if a single DL SPS configuration for multicast is configured.

In the description for sps-Config (IE for single unicast SPS) in the current RRC spec, there’s a restriction that NW cannot configure SPS in one BWP using sps-Config and sps-ConfigToAddModList (IE for one or multiple unicast SPS) simultaneously:

sps-Config UE specific SPS (Semi-Persistent Scheduling) configuration for one BWP. Except for reconfiguration with sync, the NW does not reconfigure sps-Config when there is an active configured downlink assignment (see TS 38.321 [3]). However, the NW may release the sps-Config at any time. Network can only configure SPS in one BWP using either this field or sps-ConfigToAddModList.

RAN2 asked two questions regarding the SPS configurations for unicast or multicast in the incoming LS [1]:

Q1: RAN2 would like to ask RAN1’s view on whether similar restriction is required when configuring SPS for both unicast and multicast in one BWP, i.e., network cannot use sps-Config to configure unicast SPS and simultaneously use sps-ConfigMulticastToAddModList-r17 to configure multicast in one BWP. Q2: Another question RAN2 would like to ask RAN1 is that when a single multicast SPS is configured for a group of UEs and there is no SPS configuration for unicast for at least one UE in the group, whether sps-ConfigIndex other than 0 can be configured by network for the single multicast SPS.

This paper discusses the issues raised in the above two questions and proposes the feedback per RAN2’s request.

Discussions

Regarding configurations for multicast SPS, the relevant agreements are as follows:

Agreement: from UE feature discussion: - Support up to 8 SPS group-common PDSCH configurations per CFR for multicast per cell. - The total number of SPS configurations for both multicast and unicast is no larger than 8 in a BWP of a serving cell, and activated SPS group-common PDSCH configurations is no larger than M. - The total number of SPS configurations for both multicast and unicast in a cell group is no larger than 32. Agreement: @RAN1#104e From RAN1 perspective, the CFR (common frequency resource) for multicast of RRC-CONNECTED UEs, which is confined within the frequency resource of a dedicated unicast BWP and using the same numerology (SCS and CP), includes the following configurations: - Starting PRB and the number of PRBs - One PDSCH-config for MBS (i.e., separate from the PDSCH-Config of the dedicated unicast BWP) - One PDCCH-config for MBS (i.e., separate from the PDCCH-Config of the dedicated unicast BWP) - SPS-config(s) for MBS (i.e., separate from the SPS-Config of the dedicated unicast BWP) - FFS: Other configurations and details including whether signaling of starting PRB and the length of PRBs is needed when CFR is equal to the unicast BWP - FFS: Whether a unified CFR design is also used for broadcast reception for RRC_IDLE/INACTIVE and RRC_CONNECTED - FFS: Whether Coreset(s) for CFR in addition to existing Coresets in UE dedicated BWP is needed - Note: The terminology of CFR is only aiming for RAN1 discussion, and the detailed signaling design is up to RAN2 - Note: This agreement does not negate any previous agreements made on CFR Agreement: @RAN1#104be For RRC_CONNECTED UE supporting MBS, support up to 8 configured SPS configurations in a BWP of a serving cell for unicast and MBS in total. - It is up to gNB implementation to configure the SPS configuration indexes for unicast and MBS, respectively. Agreement: @RAN1#106e If a SPS-config for MBS is configured in CFR, one G-CS-RNTI is associated with the SPS-config. - FFS: Multiple G-CS-RNTIs associated with one SPS-config Agreement: @RAN1#106e The association between a G-CS-RNTI and a SPS-Config-Multicast is indicated by the activation GC-PDCCH for SPS GC-PDSCH, i.e., a value of the HARQ process number field in a DCI format indicates an activation for a SPS GC-PDSCH configuration for multicast with a same value as provided by sps-ConfigIndex in a SPS-Config-Multicast. Agreement @RAN1#107e For multicast in RRC_CONNECTED state, - Only SPS-Config-Multicast(s) configured in CFR for multicast can be activated/deactivated by GC-PDCCH with G-CS-RNTI. - SPS-Config-Multicast(s) configured in CFR for multicast cannot be activated by unicast PDCCH with CS-RNTI, but can be deactivated by unicast PDCCH with CS-RNTI. Conclusion @RAN1#107be For multicast of RRC_CONNECTED UEs, the value range of sps-ConfigIndex in SPS-Config-Multicast is {0-7}, and sps-ConfigIndex in sps-Config and SPS-Config-Multicast cannot be configured with the same value.

For either unicast SPS or multicast SPS, supporting one SPS configuration and more than one SPS configuration are separate UE capabilities. Therefore, it is possible that UE supports one SPS configuration for unicast but supports multiple SPS configuration for multicast, or vice versa.

Note that for unicast SPS, UE may be provided sps-Config (without sps-ConfigIndex) or sps-ConfigList (with sps-ConfigIndex) depending on whether UE supports multiple SPS configuration. sps-ConfigList can provide one or more than one unicast SPS configuration.

With the conclusion that sps-ConfigIndex cannot be configured with the same value for unicast SPS and multicast SPS, based on the relevant RAN1 agreements above, the possible combinations are as follows: for a given UE within the group,

- UE is not provided unicast SPS configuration but provided one or more than one multicast SPS configurations;

  - As long as one UE (e.g., R15 legacy supporting multicast SPS) within the group is provided one unicast SPS configuration and not supporting more than one unicast SPS configuration, UE should be configured with SPS-ConfigMulticastlist

- UE is provided one unicast SPS configuration and one multicast SPS configuration;

  - UE should be configured with SPS-ConfigMulticastlist for multicast SPS though provides only one multicast SPS configuration.

- UE is provided one unicast SPS configuration and more than one multicast SPS configurations;

  - UE should be configured with SPS-ConfigMulticastlist for multicast SPS.

- UE is provided more than one unicast SPS configuration and one multicast SPS configurations;

  - UE should be configured with sps-ConfigList for unicast SPS and configured with SPS-ConfigMulticastlist for multicast SPS though provides only one multicast SPS configuration.

- UE is provided more than one unicast SPS configuration and more than one multicast SPS configurations.

  - UE should be configured with sps-ConfigList for unicast SPS and configured with SPS-ConfigMulticastlist for multicast SPS.

Keeping the legacy procedures for unicast SPS unchanged as much as possible, it can be derived that for multicast SPS configuration provided by SPS-ConfigMulticastlist,

- When only includes one multicast SPS configuration, the SPS index indicated by sps-ConfigIndex can be other values than zero.

- When includes multiple multicast SPS configurations, the sps-ConfigIndex can be non-consecutive values for the multiple configurations in the list.

- For a given UE, the sps-ConfigIndex for unicast SPS and multicast SPS configurations can be non-consecutive values.

Regarding the two questions from RAN2,

Q1: RAN2 would like to ask RAN1’s view on whether similar restriction is required when configuring SPS for both unicast and multicast in one BWP, i.e., network cannot use sps-Config to configure unicast SPS and simultaneously use sps-ConfigMulticastToAddModList-r17 to configure multicast in one BWP.

RAN1’s view is that: network CAN use sps-Config to configure unicast SPS and simultaneously use sps-ConfigMulticastToAddModList-r17 to configure multicast in one BWP, considering that UE may be the legacy Rel-15 UE supporting multicast SPS but only supporting more than one SPS configuration for unicast.

Q2: Another question RAN2 would like to ask RAN1 is that when a single multicast SPS is configured for a group of UEs and there is no SPS configuration for unicast for at least one UE in the group, whether sps-ConfigIndex other than 0 can be configured by network for the single multicast SPS.

RAN1’s view is that: Network can configure sps-ConfigIndex other than 0 for the single multicast SPS, when a single multicast SPS is configured for a group of UEs and there is no SPS configuration for unicast for at least one UE in the group

Proposal 1: To Q1 from the LS in, providing RAN1’s view as

- network CAN use sps-Config to configure unicast SPS and simultaneously use sps-ConfigMulticastToAddModList-r17 to configure multicast in one BWP, considering that UE may be the legacy Rel-15 UE supporting multicast SPS but only supporting more than one SPS configuration for unicast.

Proposal 2: To Q2 from the LS in, providing RAN1’s view as

- Network can configure sps-ConfigIndex other than 0 for the single multicast SPS, when a single multicast SPS is configured for a group of UEs and there is no SPS configuration for unicast for at least one UE in the group.

Conclusions

This papers discusses the SPS configurations for unicast or multicast from RAN1 perspective and leads to the following proposals to respond the LS in from RAN2:

Proposal 1: To Q1 from the LS in, providing RAN1’s view as

- network CAN use sps-Config to configure unicast SPS and simultaneously use sps-ConfigMulticastToAddModList-r17 to configure multicast in one BWP, considering that UE may be the legacy Rel-15 UE supporting multicast SPS but only supporting more than one SPS configuration for unicast.

Proposal 2: To Q2 from the LS in, providing RAN1’s view as

- Network can configure sps-ConfigIndex other than 0 for the single multicast SPS, when a single multicast SPS is configured for a group of UEs and there is no SPS configuration for unicast for at least one UE in the group.

- References

R1-2300015/R2-2213108, LS on SPS configuration for unicast and multicast, RAN2, Toulouse, France, November 14 – 18, 2022.