3GPP TSG-RAN WG1 Meeting #112	R1-2301759

# Athens, Greece, February 27 –  March 3, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 38.212 | CR | xxxx | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the parameter name alignment for SRS in TS 38.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The availableSlotOffset-based triggering mechanism for aperiodic SRS was introduced in Rel-17, the enabling condition of which is the existence of specific higher layer parameter in the configuration of at least one SRS resource set. However, the aforementioned specific higher layer parameter is not aligned in 38.212 and 38.214, where both AvailableSlotOffset and availableSlotOffsetList appear in the enabling condition of availableSlotOffset-based triggering mechanism, which may cause misunderstanding. Since AvailableSlotOffset is the entry of availableSlotOffsetList according to 38.331, it’s more appropriate to unify the higher layer parameter in enabling condition to be availableSlotOffsetList. In addition, there is a typo in section 7.3.1.2.3 that should be modified. |  |  |  |  |  |  |  |  |
| e |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | To unify the higher layer parameter in the enabling condition of availableSlotOffset-based triggering mechanism to be availableSlotOffsetList. To modify the typo in section 7.3.1.2.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The higher layer parameter in the enabling condition of availableSlotOffset-based triggering mechanism is not aligned, which may cause misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: No inter-operatability issue is identified. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

7.3.1.1.2	Format 0_1

========================= Unchanged parts =========================

-	SRS offset indicator – 0, 1 or 2 bits.

-	0 bit if higher layer parameter availableSlotOffsetList is not configured for any aperiodic SRS resource set in the scheduled cell, or if higher layer parameter availableSlotOffsetList is configured for at least one aperiodic SRS resource set in the scheduled cell and the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) is 1;

-	otherwise, log2(K) bits are used to indicate available slot offset according to Table 7.3.1.1.2-37 and Clause 6.2.1 of [6, TS 38.214],  where K is the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) in the scheduled cell;

========================= Unchanged parts =========================

7.3.1.1.3	Format 0_2

========================= Unchanged parts =========================

-	SRS offset indicator – 0, 1 or 2 bits.

-	0 bit if higher layer parameter availableSlotOffsetList is not configured for any aperiodic SRS resource set in the scheduled cell, or if higher layer parameter availableSlotOffsetList is configured for at least one aperiodic SRS resource set in the scheduled cell and the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) is 1;

-	otherwise, log2(K) bits are used to indicate available slot offset according to Table 7.3.1.1.2-37 and Clause 6.2.1 of [6, TS 38.214], where K is the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) in the scheduled cell;

========================= Unchanged parts =========================

7.3.1.2.2	Format 1_1

========================= Unchanged parts =========================

-	SRS offset indicator – 0, 1 or 2 bits.

-	0 bit if higher layer parameter availableSlotOffsetList is not configured for any aperiodic SRS resource set in the scheduled cell, or if higher layer parameter availableSlotOffsetList is configured for at least one aperiodic SRS resource set in the scheduled cell  and the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) is 1;

-	otherwise, log2(K) bits are used to indicate available slot offset according to Table 7.3.1.1.2-37 and Clause 6.2.1 of [6, TS 38.214],  where K is the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) in the scheduled cell;

========================= Unchanged parts =========================

7.3.1.2.3	Format 1_2

========================= Unchanged parts =========================

-	SRS offset indicator – 0, 1 or 2 bits.

-	0 bit if higher layer parameter availableSlotOffsetList is not configured for any aperiodic SRS resource set in the scheduled cell, or if higher layer parameter availableSlotOffsetList is configured for at least one aperiodic SRS resource set in the scheduled cell and the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) is 1;

-	otherwise, log2(K) bits are used to indicate available slot offset according to Table 7.3.1.1.2-37 and Clause 6.2.1 of [6, TS 38.214],  where K is the maximum number of entries of availableSlotOffsetList configured for all aperiodic SRS resource set(s) in the scheduled cell;

========================= Unchanged parts =========================