3GPP TSG RAN WG1 #112	        		   R1-2301787

# Athens, Greece, February 27th – March 3rd, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on HARQ-ACK for multicast SPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2023-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the latest spec, it seems to mean that the HARQ-ACK for multicast SPS PDSCH with PDCCH scheduling can be disabled by RRC or DCI indication. However, if the HARQ-ACK for multicast SPS PDSCH with PDCCH scheduling is disabled, the NW will not determine whether the multicast SPS is activated successfully by the UE receiving the multicast SPS services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding wording “associated with the multicast SPS PDSCH without PDCCH scheduling” and sentence that ” A UE can be scheduled a multicast SPS PDSCH by PDCCH with CRC scrambled by G-CS-RNTI, the UE provides HARQ-ACK information for the PDSCH reception.” to reflec the HARQ-ACK for multicast SPS cotrrectly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the HARQ-ACK for multicast SPS PDSCH with PDCCH scheduling is disabled, the NW will not determine whether the multicast SPS is activated successfully by the UE receiving the multicast SPS services. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Unchanged text is omitted ***

# 18	Multicast Broadcast Services

*** Unchanged text is omitted ***

A UE can be configured per G-RNTI for multicast or per G-CS-RNTI associated with the multicast SPS PDSCH without PDCCH scheduling, by harq-FeedbackEnablerMulticast with value set to 'enabled', to provide HARQ-ACK information for PDSCH receptions. When the UE is not provided harq-FeedbackEnablerMulticast for a G-RNTI for multicast or G-CS-RNTI associated with multicast SPS PDSCH without PDCCH scheduling and pdsch-HARQ-ACK-Codebook = dynamic for multicast HARQ-ACK information, the UE does not provide HARQ-ACK information for respective PDSCH receptions. If a UE is provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI for multicast or a G-CS-RNTI associated with multicast SPS PDSCH without PDCCH scheduling, the UE provides HARQ-ACK information for PDSCH receptions scheduled by multicast DCI format 4_1 associated with the G-RNTI or the G-CS-RNTI, and determines whether or not to provide the HARQ-ACK information for PDSCH receptions scheduled by multicast DCI format 4_2 based on an indication by the multicast DCI format 4_2 associated with the G-RNTI for multicast or the G-CS-RNTI [4, TS 38.212]. If a UE is provided pdsch-HARQ-ACK-Codebook = semi-static for multicast HARQ-ACK information, the UE does not expect to be provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI or a G-CS-RNTI. A UE can be scheduled a multicast SPS PDSCH by PDCCH with CRC scrambled by G-CS-RNTI, the UE provides HARQ-ACK information for the PDSCH reception.

*** Unchanged text is omitted ***