3GPP TSG RAN WG1 #112	                                           R1-2301799 (revised from R1-2300476)

Athens, Greece, February 27th – March 3rd, 2023

Source:	vivo

Title:	Evaluation methodologies for R18 LP-WUS/WUR

Agenda Item:	9.13.1

Document for:	Discussion and Decision

- Introduction

In RAN1 #111 meeting, the agreements given in Appendix D for R18 low-power Wake-up Signal and Receiver for NR was approved [1]. In this contribution, the target use cases, design targets are firstly discussed. Then we give our views on the power evaluation methodologies for both RRC idle/inactive and connected modes, followed by the initial simulation results. After that, LLS and coverage evaluation methodology for LP-WUS is discussed as well.

- Target use cases

Power saving and low latency are two critical target requirements for the LP-WUS/LP-WUR use cases. For some unreachable or non-rechargeable IoT devices, the battery life of them especially need to be extended as longer as possible. Besides, for latency sensitive IoT cases e.g., fire alarm sensors, low latency within seconds is desirable. XR is another typical target use case with stringent requirements for both power consumption and latency. In addition to power and latency, mobility of these use case is another aspect needs to be considered. For IoT devices, the mobility requirement of them should be stationary or nomadic. For wearables, XR devices or smart phones, the mobility requirement for LP-WUS/WUR is 3~100km/h. As above analysis, Table 1 summarizes the use cases of LP-WUR/WUS.

Table 1. Use cases for LP-WUR/WUS

| Use case | Power | Latency | Mobility |
| --- | --- | --- | --- |
| Unreachable, small form-factor or non-rechargeable sensors | Last for years | Within tens of seconds | Stationary, nomadic |
| Fire alarms sensors | Last for years | Within 1 or 2 seconds | Stationary, nomadic |
| Controllers | Last for years | Within several seconds | Stationary, nomadic |
| Wearable | Last for a few weeks | Within several seconds | low/medium speed |
| XR glasses | Last for days | Within several milliseconds | low/medium speed |
| Smart phone | Last for days | Within tens or hundreds of milliseconds | low/medium speed |

Proposal 1: Capture the following use cases in the TR of LP-WUS/WUR:

- For IoT cases: The battery should last at least few years; Latency would be within several or tens of seconds; For latency sensitive IoT cases, the latency requirement is within 1 or 2 seconds; The mobility would be stationary or nomadic.

- For wearable cases: The battery should last for a few weeks; Latency need to be within several seconds; Low/medium speed is required.

- For XR or smart phone cases: support to provide higher power saving gains on top of existing solutions; Latency need to be within several milliseconds for XR traffic and within tens of milliseconds for other eMBB traffics; Low/medium speed is required.

- Design targets

- Relative power of LP-WUR “ON” state

The overall target for the power consumption of LP-WUR is to strive for a longer battery life or standby time of device. For legacy UEs in RRC idle/inactive, the average UE power consumption is currently on several or tens of mW e.g., the average power of NB IoT device is lower than 1mW. To achieve substantial power saving gain, a reasonable target power consumption of LP-WUR “ON” state need to be reduced by a factor of at least 100 to 1000, which is tens or hundreds of μW. From the point of view of evaluation, it will be more appropriate to design the target relative power of LP-WUR “ON” than its absolute power.

Based on the evaluation results provided in section 4.2 and 4.4, to achieve substantial power saving gain, tens or hundreds of units of the target relative power of LP-WUR “ON” state is desirable.

On the other hand, although for some use case, a higher relative power of LP-WUR “ON” state greater than 0.5 unit may also achieve significant power saving gain, e.g., higher traffic arrival case, it is important to keep the “ON” state power consumption as less as possible. For a use for general purpose, the LP-WUR should address all traffic arrival cases for IoT/Wearable/eMBB. Especially, when the traffic arrival is sparse, battery life is important. Thus, keeping the “ON” state power consumption as less as possible is important as the results shown in the section 4.2.

Observation 1: In the case that the relative power of LP-WUR “ON” state is 0.03~0.5 unit,

- substantial power saving gain e.g., up to 80% can be achieved.

- LP-WUR should address all traffic arrival cases for IoT/Wearable/eMBB, thus keeping the “ON” state power consumption as low as possible is important for increasing the battery life of the device.

Proposal 2: The target relative power of LP-WUR “ON” state should be less than 1 unit.

- Latency

The latency caused by adopting LP-WUS/WUR is the time gap start from data arrival time to being ready for MR to do paging reception, which consists of two parts as follows:

- Wake-up delay:

- LP-WUS monitoring cycle

- MR transition time:

- The ramp-up transition time of main radio

- The sync/resync time of main radio (only for RRC idle/inactive mode)

Proposal 3: The latency brought by using LP-WUS/WUR consists of two parts: wake-up delay and MR transition time.

For RRC idle/inactive mode, since there are variety of use cases with different latency requirements, it will be difficult to set certain latency targets. As mentioned in section 2, for IoT cases, latency in the order of seconds is required. Specially, for latency-sensitive IoT cases, target latency may need to be hundreds of milliseconds if necessary. In addition, latency within several seconds is promising for wearable cases.

For RRC connected mode, considering the stringent latency requirement of XR traffic, the total latency by adopting LP-WUS/WUR should be no more than several milliseconds. For example, to be able to recover data transmission quickly, MR is supposed to enter micro or light sleep state during continuously LP-WUS monitoring, so as to control latency within 3ms (i.e., half of light sleep transition time). Apart from XR use case, the target latency of other eMBB use cases can be tens of milliseconds which is mainly contributed by the LP-WUS monitoring cycle.

Proposal 4: Consider the following as the latency target for LP-WUS/WUR:

- Within several or tens of seconds for IoT cases; And within 1 or 2 seconds for latency-sensitive IoT cases;

- Within several seconds for wearable cases;

- Within several milliseconds for XR use case; Within tens of milliseconds for other eMBB use cases;

- Data rate

For idle UEs, the maximum data rate for paging can be calculated as R = 48 bits (UE ID length) * 32 (UEs per PO) * 4(POs per radio frame)/10ms = 614kbps. While for LP-WUS, highest data rate may not necessary, e.g., full UE ID delivery may be not required. Hence, around 100kbps data rate design target can be considered.

Proposal 5: Around 100kbps data rate can be considered as design target for LP-WUS.

- Evaluation methodologies and results

  - Assumptions for RRC idle/inactive mode

In this section, we firstly discuss and clarify the remaining evaluation assumptions to be determined for RRC idle/inactive mode.

  - Sync/re-sync time and energy assumption for MR

In the last meeting, it has been agreed that number of SSBs for sync/re-sync of MR is up to 10. For evaluation, assuming two different numbers of SSB to cover different SINR cases is suitable e.g., 5 SSBs for High SINR case and 10 SSBs for Low SINR case. Note that these SSBs can be used for both synchronization and RRM measurement. Besides, as shown in Figure 1, the detailed processing timeline of sync/re-sync is assumed as: UE needs to do continuously detection to determine the slot and symbol boundary of SSB from the beginning of sync/re-sync process; After acquiring the timing information, the rest SSBs can be measured discontinuously i.e., UE can enter sleep state between the time gap of two SSB bursts. According to these analyses, we provide the sync/re-sync time and energy as the following Table 2.

Figure 1. Sync/re-sync timeline including RRM measurement procedure by MR

Table 2. The assumptions of Sync/re-sync time and energy

|  | Sync/re-sync time [ms] | Sync/re-sync energy [ms*units] |
| --- | --- | --- |
| Number of SSBs for sync/re-sync =5[1] | 100 | 1760, 1960, 2260 |
| Number of SSBs for sync/re-sync =10[1] | 200 | 2860, 3060, 3360 |
| Note 1: the continuous monitoring duration from the beginning of sync-re-sync time is 5/10/20ms. |  |  |

Proposal 6: For sync/re-sync time and energy, Table 2 of R1-2300476 can be assumed.

  - Ramp-up/down transition time and energy for ultra-deep sleep state of MR

Companies views on ramp-up transition time and energy are converged into the following three alternatives in the last meeting as shown in Table 3. Alt 1 that has been discussed from the first RAN1 LP-WUS meeting seems to be acceptable by majority. In our opinions, since the sync/re-sync process is split from ramp-up transition, the ramp-up transition may only include the time for MR’s hardware to be tuned on e.g., boot, memory load and etc. Hence, we think Alt 1 will be more realistic.

Table 3. Power model of MR

| Power State | Relative Power (unit) | Ramp-up and down transition energy (Note1): (unit multiplied by ms) | Ramp-up time | # SSB for sync/re-sync |
| --- | --- | --- | --- | --- |
| Ultra-deep sleep | 0.015 | Alt 1: 10000-40000, 25000 as start point for evaluation, Alt 2: [50000-125000] as start point for evaluation Alt 3: [2000] | Alt 1: 400ms, Alt 2: [800-2000ms] Alt 3: [100ms] | Up to 10 SSB |

Proposal 7: Adopt Alt1 for ramp-up/down transition time and energy for ultra-deep sleep state of MR.

  - Transition time between LP-WUR ‘on’ and ‘off’ states for LP-WUR

For the transition from LP-WUR ‘off’ to ‘on’ state, it mainly includes the start and settling process of MCU or DSM. The MCUs boot up for a fixed number of cycles, or a fixed time period, to enable an external oscillator to become stabilized. After that, a DSM will initialize the runtime environment of the device etc. Accordingly, when assuming the stabilization of the digital circuit needs 1024 cycles and the clock is on the order of Msps, the transition time is approximately tens of milliseconds e.g., 10ms or 20ms. And in Table 4, the corresponding transition energy can be calculated as [TLR, ramp-up *(PON-POFF)/2], wherein p_WUROff =0.001unit for p_WUROn= 0.01/0.05/0.1/0.5/1/2/4 units  and p_WUROff =0.1unit for p_WUROn= 10/20 units.

Table 4. Examples of additional transition energy and transition time between LP-WUR ‘on’ and ‘off’ states.

| Transition energy [unit*ms] | The relative power of LP-WUR “ON” state [unit], i.e., p_WUROn |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.01 | 0.05 | 0.1 | 0.5 | 1 | 2 | 4 | 10 | 20 |
| TLR, ramp-up (ms)=10ms | 0.005 | 0.025 | 0.5 | 2.5 | 5 | 10 | 20 | 49.5 | 99.5 |
| TLR, ramp-up (ms)=20ms | 0.01 | 0.05 | 1 | 5 | 10 | 20 | 40 | 99 | 199 |

Proposal 8: Transition time between LP-WUR ‘on’ and ‘off’ states can be assumed as 10 or 20ms.

  - RRM measurement assumption

In RRC idle/inactive mode, to meet RRM measurement requirements defined by RAN4[X], a legacy UE is supposed to perform measurement by MR every several I-DRX cycles. In the case of adopting LP-WUS monitoring for UE power saving, to avoid frequent transitions from/to ultra-deep sleep of MR which will result in plenty of consumption, RRM measurement performed by MR is expected to be relaxed by means of partially or totally offloading it to LP-WUR. The following is three RRM measurement options.

Option 1 (legacy behavior): RRM measurement is only performed by MR.

Option 2: MR performs relaxed RRM measurement every X I-DRX cycles, where X can be 10 or 20. And LP-WUR performs assistant RRM measurement based on periodic lower power signal e.g., beacon or LP-SS.

Option 3: RRM measurement is totally performed by LP-WUR.

Additionally, the feasibility and detailed design of RRM measurement performed by LP-WUR need to be further evaluated and discussed. Note that we assume RRM measurement performed by MR is included in the process of sync-/re-sync as shown in Figure 1. And the relative power of RRM measurement by LP-WUR is the same as that of LP-WUS detection.

Proposal 9: For RRM measurement assumptions, the following options can be considered.

- Option 1: RRM measurement is only performed by MR.

- Option 2: LP-WUR performs RRM measurement based on periodic lower power signal e.g., LP-SS. MR performs relaxed RRM measurement every X I-DRX cycles, where X can be 10 or 20. And

- Option 3: RRM measurement is only performed by LP-WUR.

  - Power evaluation results for RRC idle/inactive mode

Based on the agreed evaluation methodologies for RRC idle/inactive mode and the aforementioned additional assumptions, we provide the power evaluation results considering the following aspects which will do impact on power consumption and latency performance.

- Baselines: PO monitoring with i-DRX, e-DRX, with or without PEI

- The relative power of LP-WUR “ON” state: [0.01/0.05/0.1/0.5/1/2/4/10/20] units

- MR Ramp-up time: [100ms, 400ms, 800ms] and Ramp-up and down transition energy: [2000, 20000, 50000]

- Sync/re-sync time X: X= [100ms, 200ms] and sync/re-sync energy: 100ms[1760, 1960, 2260], 200ms[2860, 3060, 3360].

- RRM:

- For baselines: option 1: RRM measurement is only performed by MR.

- For LP-WUS scheme:

Option 2: MR performs relaxed RRM measurement every X I-DRX cycles, where X can be 10 or 20. And LP-WUR performs assistant RRM measurement based on periodic lower power signal e.g., LP-SS.

Option 3: RRM measurement is totally performed by LP-WUR.

- The false-alarm rate (FAR) of LP-WUS: [0, 0.1%, 1%, 10%]

- Per UE paging rate i.e., RE=RE, REF= [1%, 0.1%, 0.01% or 0.001%], Y=YREF = 1.28s

- Per group paging rate i.e., RG=1 – (1 – RE)N , N is the number of UEs in the group: [10, 20]

- LP-WUS monitoring:

  - Option 1: continuously monitoring;

  - Option 2: discontinuously monitoring

- [200ms, 600ms, 1000ms] as the period for an on-and-off cycle, and for each cycle the duty cycle ratio is [0.1%, 2%, 10%, 60%, 100%].

- LP-WUR on-off transition time: 10ms or 20ms.

The following is the detail of evaluation results and observations considering the mentioned aspects.

- Comparing with i-DRX, e-DRX, with or without PEI

Figure 2 shows the simulation results with different baseline schemes under the configurations in Table 5.

Table 5. Configurations for evaluation with different baselines.

| Parameters | Ramp-up time and transition energy | Sync/re-sync time and energy | FAR of LP-WUS | LP-WUS monitoring configuration | Per UE paging rate | RRM |
| --- | --- | --- | --- | --- | --- | --- |
| Values | 400ms; 20000 * | 200ms; 3360* | 0.1% | Option 1 | 0.001% | Option 1 for baseline scheme; Option 3 for LP-WUS scheme. |

*Note: the value assumed for this simulation is to know the worst case for LP-WUS. It does not necessary mean to a down-selection or intension to it. Other values can be also considered for study. But the power saving gain for LP-WUS may be less than this value.

Figure 2.  Initial simulation results of power consumption and latency for different schemes

According to the results, the following observations are given:

Observation 2: Compared with I-DRX paging, LP-WUR/WUS scheme with continuously monitoring configuration can achieve around 50%~98% power saving gain when the relative power of LP-WUR “ON” state is no more than 1 unit, with marginal latency increase.

Observation 3: Compared with eDRX, LP-WUR/WUS scheme with continuously monitoring configuration can largely reduce the paging latency (22x), with comparable UE power consumption.

Observation 4:  LP-WUR/WUS scheme provides a much better trade-off between latency and power consumption when relative power of LP-WUR “ON” state is no more than 1unit, compared with I-DRX paging and eDRX scheme.

- The relative power of LP-WUR “On” state: [0.01/0.05/0.1/0.5/1/2/4] units

Figure 3 shows the simulation results with different relative power of LP-WUR under configurations in Table 6.

Table 6. Configurations for evaluation with different relative power of LP-WUR “ON” state.

| Parameters | Baseline: | Ramp-up time and transition energy | Sync/re-sync time and energy | FAR of LP-WUS | LP-WUS monitoring configuration | Per UE paging rate | LP-WUR on-off transition time | RRM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Values | I-DRX paging with PEI | 400ms; 20000 | 100ms; 2260 | 0%, 0.1% | Option 1; Option2 | 0.001% | 20ms | Option 1 for baseline scheme; Option 3 for LP-WUS scheme. |
| Notes: Considering latency sensitive use cases, the configuration for Option 2 is: [200ms] as the duty cycle, and [4ms] as the active time for monitoring LP-WUS every cycle. |  |  |  |  |  |  |  |  |

Note

(Notes: With discontinuously monitoring operation, the main reason for the high power consumption of WUR is the high transition power consumption of WUR between “ON” and “OFF”; When the WUR ON power=10/20, it is assumed that WUR OFF power = 0.1, which is different to other WUR ON power value cases.)

Figure 3.  The performance of LP-WUS/WUR scheme with different relative powers of LP-WUR.

According to the results, the following observations are given:

Observation 5: High relative power of LP-WUR “ON” state is not feasible when continuously LP-WUS monitoring is configured.

Observation 6: Discontinuous LP-WUS monitoring can significantly reduce UE power consumption of LP-WUS scheme, but latency will increase accordingly, compared with continuous LP-WUS monitoring configuration.

- LP-WUS monitoring configuration: Duty-cycled LP-WUS with various duty cycle length and ratio

Figure 4 shows the simulation results with different duty-cycled LP-WUS with various duty cycle length and ratio under configurations in Table 7.

- [200ms, 600ms, 1000ms] as the period for an on-and-off cycle, and for each cycle the duty cycle ratio is [0.1%, 2%, 10%, 60%, 100%].

Table 7. Configurations for evaluation with different LP-WUS monitoring configurations.

| Parameters | Baseline: | Ramp-up time and transition energy | Sync/re-sync time and energy | FAR of LP-WUS | Relative power of LP-WUR ON | Per UE paging rate | LP-WUR on-off transition time | RRM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Values | I-DRX paging with PEI | 400ms; 20000 | 100ms; 2260 | 0.1% | 0.1, 2 units | 0.001% | 20 | Option 1 for baseline scheme; Option 3 for LP-WUS scheme. |

Figure 4. The power and latency performances of LP-WUS/WUR scheme with different DRX cycle lengths and ratio.

According to the results, the following observations are given:

Observation 7: With fixed duty cycle ratio and relative power of LP-WUR “ON” state, different LP-WUR DRX cycle lengths have no or less impact on UE power consumption of LP-WUS scheme, and the difference only depends on the number of LP-WUR ON-OFF transition.

Observation 8错误!未指定顺序。: For LP-WUS scheme, latency will be reduced with the increase of duty cycle ratio of LP-WUS monitoring, while UE power consumption will increase accordingly.

Observation 9错误!未指定顺序。: Up to 65% power saving gain can be achieved by LP-WUS scheme when duty cycle ratio is extremely low e.g., 0.1% or 2% even in the case of high relative power of LP-WUS “ON” state e.g., 20units.

- The false-alarm rate (FAR) of LP-WUS: [0, 0.1%, 1%, 10%]

Figure 5 shows the simulation results with different FAR under configurations in Table 8

Table 8. Configurations for evaluation with different FAR.

| Parameters | Baseline: | Ramp-up time and transition energy | Per UE paging rate | Sync/re-sync time and energy | LP-WUS monitoring configuration | Relative power of LP-WUR ON | RRM |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Values | I-DRX paging with PEI | 400ms; 20000 | 0.001% | 100ms; 2260 | Option1 | 0.1 or 4units | Option 1 for baseline scheme; Option 3 for LP-WUS scheme. |

Figure 5.  The power and latency performances of LP-WUS/WUR scheme with different FAR.

(Note: FAR here refers to the FAR of channel which not contain the FAR caused by group paging. If we both consider FAR and group wake-up, UE power consumption of LP-WUS scheme will sharply increase.)

According to the results, the following observations are given:

Observation 10: As FAR for LP-WUS increases, UE power consumption goes up. And a sharp raising of UE power consumption is observed when FAR reaches 10%.

- RRM

Figure 6 shows the simulation results with different RRM schemes under configurations in Table 9.

Table 9. Configurations for evaluation with different RRM assumptions.

| Parameters | Baseline | Relative power of LP-WUR | Ramp-up time and transition energy | Sync/re-sync time and energy | FAR of LP-WUS | LP-WUS monitoring configuration | Per UE paging rate |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Values | I-DRX paging with PEI | 0.1/2 unit | 400ms; 20000 | 100ms; 2260 | 0.1% | Option 1 Option 2 | 0.001% |
| the configuration for Option 2 is: [200ms] as the duty cycle, and [4ms] as the active time for monitoring LP-WUS every cycle. |  |  |  |  |  |  |  |

Figure 6.  The power and latency performances of LP-WUS/WUR scheme with different RRM measurement.

According to the results, the following observations are given:

- Power: UE power consumption of LP-WUS scheme will significantly increase, once RRM performed by MR since plenty of MR transition times will be resulted in. But, as the RRM measurement by MR is gradually relaxed, UE power consumption decreases somewhat. Until RRM measurement completely offload to WUR i.e., RRM measurement is only performed by WUR, there will be the most power saving gain by adopting LP-WUS.

- Latency: Latency decreases as RRM measurement period decreases since it is assumed that MR and LP-WUR can monitoring at the same time.

Observation 11: Even if RRM measurement is performed by main radio at a relaxed level, the total UE power consumption of LP-WUS scheme increases distinctly.

Observation 12: RRM measurement performed by WUR only will greatly help reduce UE power consumption of LP-WUS scheme.

- Per UE paging rate: [1%, 0.1%, 0.01%, 0.001%] and per group paging rate: N= [1, 10, 20]

Figure 7 shows the simulation results with different paging rate under configurations in Table 10.

Table 10. Configurations for evaluation with different paging rate.

| Parameters | Baseline | Relative power of LP-WUR ON | Ramp-up time and transition energy | Sync/re-sync time and energy | FAR of LP-WUS | LP-WUS monitoring configuration | RRM |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Values | I-DRX paging with PEI | 0.1 unit | 400ms; 20000 | 100ms; 2260 | 0.1% | Option 1 | Option 1 for baseline scheme; Option 3 for LP-WUS scheme. |
| Note: RE = 0.001% or 1%, N = [1 10 20] when calculate per group paging rate. |  |  |  |  |  |  |  |

Figure 7. The power and latency performances of LP-WUS/WUR scheme with different paging rate.

According to the results, the following observations are given:

- Power: Obviously, UE power consumption of LP-WUS scheme will increase with the per UE or group paging rate.

- Latency: the latency slightly decreases due to more frequent PO monitoring caused by group paging.

Observation 13: As the paging rate and the number of UE in group increase, power consumption of LP-WUS scheme will increase but latency will slightly decrease.

- Ramp-up time: [100ms, 400ms, 800ms] and Ramp-up/down transition energy: [2000, 20000, 50000]; sync/re-sync time: [100ms 200ms] and corresponding energy: [2260, 3360]

Figure 8 shows the simulation results with different paging rate under configurations in Table 11.

Table 11. Configurations for evaluating different ramp-up and sync time/energy.

| Parameters | Baseline | Relative power of LP-WUR ON | Per UE paging rate | Ramp up time | Sync/re-sync time and energy | FAR of LP-WUS | LP-WUS monitoring configuration | RRM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Values | I-DRX paging with PEI | 0.1 unit | 1% | 100ms; 2000 400ms; 20000 800ms; 50000 | 100ms; 2260 200ms; 3360 | 0.1% | Option 1 | Option 1 for baseline scheme; Option 3 for LP-WUS scheme. |
| Note: RE = 1%, N = [10] when calculate per group paging rate. |  |  |  |  |  |  |  |  |

Figure 8. The power and latency performances of LP-WUS/WUR scheme with different ramp-up time/energy and sync/re-sync time/energy.

According to the results, the following observations are given:

- Power: In the case of very low paging rate, UE power consumption will not be affected much as ramp up and sync timer/energy increase due to few MR transition times. However, things will be opposite when paging rate is high.

- Latency: There is a linear relationship between delay and ramp-up/sync time.

Observation 14: The UE power consumption and latency of LP-WUS scheme will increase as ramp-up and sync/re-sync time/energy increase.

Observation 15: The impact of ramp up and sync time/energy on UE power consumption of LP-WUS scheme will be remarkable in intensive traffic scenario (i.e., larger paging rate) due to larger number of MR transition.

  - Assumptions for RRC connected mode

As mentioned earlier, XR service with stringent requirements on both power consumption and latency is one of the target use cases for RRC connected mode. Per the discussion in R17/18 XR, one of the identified pain point issues is XR data packets have a wide range of unpredictable jitter. That is, UE has to perform intensive PDCCH monitoring in the entire jitter range in order to receive the scheduling of XR traffic timely and avoid packet loss caused by exceeding the PDB (e.g., 10ms for XR video traffic). However, the existing R15/16/17 power saving schemes cannot completely avoid the unnecessary PDCCH monitoring actually. In this regard, the motivation to study LP-WUS/WUR in RRC connected mode is to seek a better solution to reduce the unnecessary power consumption caused by uncertain jitter.

Observation 16: The motivation to study LP-WUS/WUR in RRC connected mode for XR service is to reduce the excessive PDCCH monitoring due to unpredictable jitter.

As shown in Figure 9(b), enhanced C-DRX, i.e. aligned C-DRX cycle to XR traffic periodicity is adopted. To further reduce the unnecessary PDCCH monitoring during the jitter range (covered by DRX on duration), a UE can firstly monitor LP-WUS instead of PDCCH at the beginning of DRX onduration. And after LP-WUS is detected, the UE just start to monitor PDCCH. As such, by replacing the unnecessary PDCCH monitoring with LP-WUS detection, UE will have more opportunity to achieve more power saving gain, compared to the existing power saving schemes as depicted in Figure 9 (a).

Figure 9.  Working procedure for LP-WUS/WUR based jitter handling

The power model of LP-WUR agreed in the last meeting can also be adopted for power evaluation in RRC connected mode. Moreover, to guarantee the latency requirement of XR traffic, main radio need to enter micro or light sleep state rather than the ultra-deep sleep state during LP-WUS monitoring. Accordingly, the latency caused by LP-WUS detection will be 0 or 3ms. And the sum of relative power of LP-WUR “ON” state and MR in micro/light sleep state will be close to 45 or 20 units, which is less than half of the relative power of MR PDCCH-only state i.e., 100 units.

Proposal 10: For R18 LP-WUS/WUR power evaluation in RRC connected mode, during the LP-WUS monitoring, the power state of main radio can be micro or light sleep.

Apart from XR traffic, FTP model 3 is another one of typical eMBB traffics for applying LP-WUS in RRC connected mode. By means of LP-WUS monitoring, UE can be woken up right after data arrives at the gNB side so that the long latency caused by CDRX cycle (e.g., 160ms or 320ms) can be avoided. In this sense, the transmission latency can be reduced deeply by applying LP-WUS, which is conducive to improving the performance of UE perceived throughput accordingly.

  - Power evaluation results for RRC connected mode

In the following evaluation, performance metrics including power consumption, latency, capacity, UPT are considered. Besides, both XR traffic and eMBB traffic are evaluated, the detail evaluation methodologies and assumptions can be found in TR 38.838 and 38.840, respectively.

- Evaluation for XR traffic model (refer to TR38.838 and the agreed jitter model in R18 XR SI)

Based on the R17 and R18 XR power evaluation methodologies and assumptions, we further evaluate power consumption and system capacity of different cases. And the detailed configurations can be founded in Appendix C. Note that for capacity results of LP-WUS/WUR scheme (combined with main receiver light sleep) corresponding to the UE satisfaction metric with both 95% and 99% packet successful rate are shown. Evaluations for both low load and high loads are given, corresponding to 5 and 10 UEs per cell, respectively.

Figure 10.  Power saving gain and system capacity results for R17 PDCCH monitoring adaption and LP-WUS/WUR schemes

Figure 11.  Power saving gain and system capacity results for R17 PDCCH monitoring adaption and LP-WUS/WUR schemes

According to the results shown in Figure 11, compared to always-On scheme (no DRX configuration), R17 PDCCH monitoring adaptation scheme can obtain 10~19% power saving gain, while LP-WUS monitoring schemes can achieve about 25% or 30% power saving gain, meaning up to 15% additional power saving gain.

For both low load ang high load cases, comparing to R17 PDCCH monitoring adaptation scheme, LP-WUS/WUR schemes with main radio enters micro sleep during LP-WUS monitoring can bring {6%~15%} additional UE power saving gain with no capacity loss.

In addition, for low load case, comparing to R17 PDCCH monitoring adaptation scheme, LP-WUS/WUR schemes with main radio enters light sleep during LP-WUS monitoring can bring {10%~22%} additional UE power saving gain with 10% (UE satisfaction metric as 99% packet successful rate) or no (UE satisfaction metric as 95% packet successful rate) capacity loss. But, for high load case, LP-WUS/WUR schemes with main radio enters light sleep during LP-WUS monitoring can achieve similar additional UE power saving gain with as that in low load, but the capacity loss is significant.

In this sense, the LP-WUS monitoring combined with main receiver micro sleep can be applied to all the cases, while the combination with main receiver light sleep are mainly useful for low load scenarios. Moreover, it can be observed that the larger jitter range, the more additional power saving gain can be obtained by LP-WUS/WUR.

Observation 17: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring combined with main receiver micro sleep can bring {6%~15%} additional UE power saving gain with no capacity loss in both low load and high load cases.

Observation 18: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring combined with main receiver light sleep can bring {10%~22%} additional UE power saving gain, with acceptable capacity loss at least in low load case.

- Evaluation for general eMBB traffic model (refer to TR 38.840)

Based on FTP model 3 traffic model (200ms mean arrival interval) and its corresponding CDRX configuration provided in TR 38.840, we further evaluate UE perceived throughput (UPT) and power consumption with the following schemes:

- Always-On (baseline): Without adopting any power saving mechanism.

- C-DRX:

| DRX cycle (ms) | drx-onDurationTimer (ms) | drx-InactivityTimer(ms) |
| --- | --- | --- |
| 160 | 8 | 100 |

- C-DRX +DCI format 2_6: the time gap between DCI format 2_6 and DRX onduration is 3ms, and the DCI format 2_6 monitoring duration is 2ms.

- R17 PDCCH monitoring adaptation + C-DRX +DCI format 2_6: PDCCH skipping duration is 100ms, SSSG with PDCCH monitoring every slot.

- LP-WUS monitoring+ PDCCH skipping: During PDCCH skipping duration, the UE performs continuous LP-WUS monitoring. Once detecting LP-WUS, UE will start PDCCH monitoring after a wake-up delay e.g., 0ms, 3ms, or 10ms (which corresponds to the ramp-up delay of main receiver from micro/light/deep sleep states).  The assumptions on the relative power of LP-WUR “ON” state is 0.01/1/10/20/30/40 units.

Figure 12. UPT and power consumption of evaluation schemes

Table 12. PSG and UPT loss compared with always-On scheme

| Schemes | relative power of LP-WUR “ON” | PSG (compared to “AlwaysON”) | UPT loss |
| --- | --- | --- | --- |
| C-DRX | - | 52.19% | 60.93% |
| C-DRX + DCI 2_6 | - | 53.78% | 63.74% |
| R17 PDCCH monitoring adaptation + C-DRX | - | 58.42% | 66.92% |
| R17 PDCCH monitoring adaptation + C-DRX + DCI 2_6 | - | 77.35% | 86.73% |
| LP-WUS scheme (wake-up delay 0ms) | 0.01 | 44.00% | 0.00% |
|  | 1 | 42.95% |  |
|  | 10 | 33.42% |  |
|  | 20 | 22.83% |  |
|  | 30 | 12.24% |  |
|  | 40 | 1.65% |  |
| LP-WUS scheme (wake-up delay 3ms) | 0.01 | 69.00% | 25.98% |
|  | 1 | 67.95% |  |
|  | 10 | 58.42% |  |
|  | 20 | 47.83% |  |
|  | 30 | 37.24% |  |
|  | 40 | 26.65% |  |
| LP-WUS scheme (wake-up delay 10ms) | 0.01 | 86.73% | 57.20% |
|  | 1 | 85.68% |  |
|  | 10 | 76.15% |  |
|  | 20 | 65.56% |  |
|  | 30 | 54.98% |  |
|  | 40 | 44.39% |  |

As shown in Figure 12 and Table 12, the following observations are given in terms of UPT :

- Compared to existing power saving schemes, LP-WUS can achieve higher UPT gain, especially for cases with low wake-up delay under LP-WUR scheme.

- It can also be seen that with LP-WUR scheme (wake-up latency 0ms), the same UPT as always-on schemes can be achieved.

As shown in Figure 12 and Table 12, the following observations are given in terms of power consumption when relative power of WUR “ON” is 0.01 unit:

- By applying advanced UE power saving schemes such as DCI format 2_6, the power saving gain will be further increased, with the cost of increased latency, or decreased UPT.

- Comparing with C-DRX scheme, LP-WUR schemes with main radio enters light/deep sleep can bring {16.81% /34.54%} additional UE power saving gain.

- Comparing with C-DRX + DCI format 2_6 scheme, LP-WUR schemes with main radio enters light/deep sleep can bring {15.22% /32.95%} additional UE power saving gain.

- Compared with R17 PDCCH monitoring adaptation + C-DRX + DCI 2_6 scheme, LP-WUR schemes with main radio enters deep sleep during LP-WUS monitoring can bring 9.38% additional UE power saving gain.

Observation 19: UPT of LP-WUR scheme (wake-up latency 0ms case) is the same as always-on scheme which means it can effectively reduce power consumption without affecting network scheduling.

Observation 20: When the relative power of WUR 'On' state is no more than 1unit, LP-WUS monitoring with PDCCH skipping scheme can achieve the best trade-off performance in both UPT and power saving, compared with the existing UE power saving schemes.

Observation 21: When the relative power of WUR 'On' state is larger than 1unit i.e., 10 or 20 units, LP- WUS monitoring with PDCCH skipping scheme has no power saving gain advantage, compared with the existing power saving schemes.

  - System performance (Overhead and gNB Energy consumption)

- System resource overhead

In order to study the system resource overhead by LP-WUS, we perform the following resource overhead calculation by means of the following assumptions:

- The bandwidth of LP-WUS denoted as BWLP-WUS is 4.32MHz +2RB (30kHz) = 5.04MHz.

- The bandwidth of a BWP in a serving cell denoted as BWCell is 20MHz or 100MHz.

- The duration of LP-WUS represented by DLP-WUS is 4 symbols.

- For RRC idle/inactive mode, paging configuration:

  - The legacy i-DRX paging is as reference configuration, i.e., setting the number of paging in a certain time is the same for i-DRX paging and LP-WUS.

  - Reference paging configuration (worst case which requires worst overhead)

    - Assuming all the N POs every paging frame (PF) are used for paging, which is the extreme worse case. Here, N=4

  - Equivalent supported UEs NUE_idle can be calculated as follows,

    - For per UE paging rate is RE=1%;

    - Paging i-DRX cycle is Tpaging = 1.28s.

    - UE number NUE_idle then can be calculated as 51200 (1280/10*4/1% = 51200) (assuming the total paging frames is 128 and 4 POs in a PF), which should be large enough.

- For RRC connected mode, UE number per cell NUE_connected is assumed to be 10. Different UEs are using orthogonal LP-WUS resources which are not overlapped with each other.

- The traffic interval TTrafficInterval of XR video with 16.67ms (30FPS) and eMBB FTP3 with 200ms are assumed for RRC connected mode.

Hence, the resource overhead ratios of LP-WUS to the overall system resource can be calculated as below. And Table 13 and Table 14 provide some reference values based on the above assumptions.

R=BWLP_WUS*DLP_WUS*RE*NUE_idle BWCell*Tpaging or BWLP_WUS*DLP_WUS*NUE_connected BWCell*TTrafficInterval

- RRC Idle/inactive mode

Assuming the resource of one LP-WUS is:

- Alt 1: 5MHz * 4 OFDM symbol (30kHz)

- Alt 2: 5MHz * 14 OFDM symbol (30kHz)

Table 13.  Resource overhead ratio to the overall system resource for RRC idle/inactive mode.

| Resource overhead ratio, R | Alternative 1: 5MHz * 4 OFDM sybol |  |  | Alternative 2: 5MHz * 14 OFDM sybol |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1 PO per PF | 2 PO per PF | 4 PO per PF | 1 PO per PF | 2 PO per PF | 4 PO per PF |
| 20MHz | 0.36% | 0.72% | 1.44% | 1.26% | 2.52% | 5.04% |
| 100MHz | 0.072% | 0.144% | 0.288% | 0.25% | 0.50% | 1.01% |

- RRC Connected mode:

Assuming the resource of one LP-WUS is: 5MHz * 4 OFDM symbol (30kHz) (Considering the coverage is compared to PUSCH)

Table 14.  Resource overhead ratio to the overall system resource for RRC connected mode.

| Resource overhead ratio, R | RRC Connected mode |  |
| --- | --- | --- |
|  | XR traffic | eMBB traffic |
| 100MHz, 30KHz SCS | 0.43% | 0.036% |

Observation 22: For IDLE/INACTIVE mode, the upper bound of resource overhead used for LP-WUS is less than 1% for 100MHz system BW even in worst case such as the paging arrives every POs in every PF (i.e., 51200 idle/inactive UEs). For CONNECTED mode, the resource overhead of LP-WUS is less than 0.5% even in high load cases such as 10 XR UEs per cell.

| Resource overhead ratio, R | RRC Idle/inactive mode | RRC Connected mode |  |
| --- | --- | --- | --- |
|  |  | XR traffic | eMBB traffic |
| 20MHz, 30KHz SCS | 0.36% ~ 5.04% | - | - |
| 100MHz, 30KHz SCS | 0.072% ~ 1.01% | 0.43% | 0.036% |

- Network energy consumption

Network energy consumption for transmitting LP-WUS depends on the 	LP-WUS occasion configuration and gNB implementation. For example, if the LP-WUS is allowed to be transmitted in the slot with SSB/SIB transmission, it will not cause serve additional network energy.  According to the current specification, it has many occasions for SSB transmission, so it is flexible for gNB to transmit LP-WUS in these occasions.

Observation 23: Additional network energy consumption caused by LP-WUS/WUR operation can be minimized, e.g., gNB can transmit LP-WUS in the slot with existing NR signal e.g., SSB.

- Link performance and coverage evaluation

To achieve the same MIL as PUSCH, certain target receiver sensitivity should be achieved. In [3], The MIL is defined as:

MIL=Total transmit power-Receiver sensitivity-Tx Loss-Rx Loss+gNB antenna gain component 2+3+4+UE antenna gain

To achieve comparable coverage between LP WUS and PUSCH, MIL of LP-WUS should be no less than MIL of PUSCH, and targeting receiver sensitivity for LP-WUS can be derived accordingly.

- MILPUSCH≤ MILLP-WUS=Total transmit powerLP-WUS-Receiver sensitivityLP-WUS-Tx Loss-Rx Loss+gNB antenna gain component 2+3+4+UE antenna gain

Receiver sensitivityLP-WUS ≤ Total transmit powerLP-WUS-Tx Loss-Rx Loss+gNB antenna gain component 2+3+4+UE antenna gain- MILPUSCH

For PUSCH 1Mbps for 2.6GHz Urban, the representative MIL value is 139.66dB [3]. Thus, the Receiver sensitivity for LP-WUS should be lower than ~-81dBm to achieve comparable coverage as PUSCH with 1Mbps data rate in Urban scenario.

Observation 24: About -81 dBm receiver sensitivity is required for LP-WUS, to achieve the same coverage as PUSCH 1Mbps in 2.6GHz Urban scenario.

In previous meetings, there are proposals to compare the MIL of LP-WUS with that of paging PDCCH. The paging PDCCH mainly performed in idle state. Although UE supports 4Rx in some frequency band, e.g., 2.6GHz, 4GHz, UE typically use 2 Rx antennas in RRC idle state, due to the RRM measurements in RAN4 are defined assuming 2 Rx antennas. Hence, UE typically only use 2 Rx antennas in RRC idle state in real deployment, 2 Rx should be assumed when coverage of PDCCH is used to compared with that of LP-WUS.

Proposal 11: For normal UE supporting 4 Rx, 2 Rx should be assumed in evaluation when the coverage of paging PDCCH is used to compared with that of LP-WUS.

  - Assumptions in coverage Evaluation

The coverage of LP-WUS depends on many factors, including data rates, waveform, signal bandwidth and duration, receiver type, and receiver parameters. We will discuss how these parameters impacts the coverage in this section.

- Waveform/Modulation/Code scheme

For waveform of LP-WUS, the OOK, ASK, etc., based waveform which can be decoded with low power components, e.g., envelop detector, is widely used for ultra-low power receivers. Each waveform may associate with certain kind of receiver structure, and have different performance on certain aspects, e.g., interference suppression. Hence, the waveform of LP-WUS should be reported in coverage evaluation.

For OOK based waveform, some line coding, e.g., Manchester coding are usually used to avoid consecutive zeros. Hence, one symbol carrying information 0/1 is composed of two chips. Furthermore, repetition of the symbol waveform may be considered within a WUS channel to ensure the reliability of LP-WUS. The coding scheme should be reported for each evaluated case.

Proposal 12: For coverage evaluation, waveform/modulation type should be reported, e.g., OOK, ASK, etc.

Proposal 13: For coverage evaluation, code scheme should be reported, e.g., Manchester code, repetition, etc.

- Data rate and LP-WUS channel structure

Like legacy channels, LP-WUS with different data rates means different detection performance and coverage. Hence, the data rate should be one important parameter which should be also reported in coverage evaluation.

Figure 13.  Example of a LP-WUS channels structure

The channel structure for LP-WUS highly depends on how many information bits are delivered. As discussed in [4], sequence-based LP-WUS can be considered if limited information bits are conveyed. While control information payload may be required if a few tens of bits would be delivered. For LP-WUS with control information payload, the sync sequence part at the beginning may be still needed considering accurate timing maintenance cannot be expected in WUR. Longer sync sequence field means better timing acquisition performance and higher detection reliability at the expense of lower raw data rate.

Note that, if the LP-WUS is composed of multiple parts, e.g., preamble, control information, and CRC bits/symbols, the raw data rate should be calculated counting all the overhead for a WUS transmission.

Proposal 14:  For coverage evaluation, LP-WUS channel structure should be reported, and the following structures can be considered

- [Sync (X chips)] + data (Y bits) + CRC (Z bits);

- Sequence only (X chips)

- Signal bandwidth/guard band

OOK signal with different bandwidth means different power allocation and overhead for LP-WUS channel. Hence, the channel bandwidth should be reported. To ensure reliability of LP-WUS channel, enough signal bandwidth should be allocated, while excessive overhead should be avoided. For evaluation, the LP-WUS bandwidth can be selected in number of continuous RBs, e.g., {4, 8, 12, 16, …}.

Proposal 15:  For coverage evaluation, the LP-WUS bandwidth can be selected in number of continuous RBs, e.g., {4, 8, 12, 16, …}.

- Filter/Adjacent subcarrier interference

Since the LP-WUS channel may not occupy the whole channel bandwidth, FDM multiplexing between LP-WUS and existing DL channels may be required for better coexistence. While the WUR radio typically demodulates the LP-WUS in time domain, the FDMed DL signals would be present as adjacent subcarrier interference (ASCI) to LP-WUS. Typically, an IF LPF/BPF can be included in WUR in some receiver structure, e.g. the heterodyne architecture with IF envelope detection, to suppress the adjacent subcarrier interference. Legacy DL channel, e.g., PDSCH, can be FDMed with the LP-WUS channel to model the interference from adjacent subcarriers. For example, the PDSCH can be mapped on RBs not used for LP-WUS and guard band, and the EPRE ratio between LP-WUS and that of PDSCH can be further evaluated.

Figure 14.  Illustration of WUS channel multiplexing with NR DL channels

In general, the lower power receiver is equipped with low power consumption and low accuracy oscillator, e.g., ring oscillator, which the accuracy is far worse than the high-power consumption oscillators used in main receiver. Typically, the frequency offset of a ring oscillator is up to around 200ppm. According to AI 9.13.2, BPF or LPF should be implemented in the receiver to reject ASCI. Considering the frequency uncertainty at WUR, the bandwidth of the filter needs to be wider than the signal bandwidth to make sure the desired LP-WUS signal is kept, while adjacent subcarrier interference is blocked.

To avoid interference between LP-WUS and FDMed DL channels, guard band should be reserved on each side of LP-WUS bandwidth, and the guard band size depends on the LPF/BPF characteristics in WUR, which should be reported in the coverage evaluation. For example, 5-th order Butterworth filter with 4.32MHz bandwidth is considered in the

Proposal 16: For modeling adjacent subcarrier interference in LLS, PDSCH can be mapped on RBs not used for LP-WUS and guard band.

Proposal 17:  The BPF/LPF parameters and guard band size should be reported in coverage evaluation.

- BPF/LPF parameters may include bandwidth, filter order and filter type (e.g., Butterworth, …);

- The guard band size can be a number of RBs/subcarriers.

- Frequency error/frequency drift

In WUR, the oscillator is typically not accurate due to reduced power consumption and complexity. The inaccuracy of the oscillator may impact the detection performance on following perspectives.

As discussed earlier, WUR may rely on BPF to filter out the interference from the co-existed NR DL channels. The BPF may be unable to extract the signals in the WUS BW and exclude the interference from adjacent frequency due to frequency error. And the detection performance can be degraded.

Besides, the frequency error may also lead to timing drift in WUR. If WUR monitoring is performed in DRX manner, UE may be unable to capture the WUS transmission due to time drifting. To avoid missing the WUS occasion, UE is required to start receiving the signals earlier than the timing derived from the inaccurate oscillator, resulting in some impacts on the FAR and MDR for LP-WUS. The impact of timing error is shown in following figure.

Figure 15.  Starting detection in advance due to time drifting

For evaluation, the following assumptions can be used to derive the frequency and time drifting, as proposed in last meeting  [5].

#### [H] Proposals 1C*-v3: For evaluation, clock 1 and clock 2 can be used to model frequency error of LR as follows, - Clock 1: - maximum frequency error [20, 200] ppm - Frequency drifting [1] ppm/s - Clock 2: - maximum frequency error [5] ppm - Frequency drifting [0.05] ppm/s - Note: power consumption of clock 2 is not accounted in LR power consumption, i.e., it is accounted in MR power consumption. - Clock 2 cannot be available when MR is in ultra-deep sleep. - [Clock 2 can be available for LR when assuming LR relative power for ‘on’ is 1/2/4] - Clock 1 can be available for all states for MR and LR. - [FFS: how to model minimum frequency error] - Note: Company to report assumption on how to use clock 1 and 2 for on/off state and different DRX mode (I-DRX/eDRX).

The method for calculating the amount of time drifting can be referred to  [6]. Once UE successfully detects a LP-WUS or beacon, the timing drift can be assumed to be zero.

Proposal 18:  For coverage evaluation, clock 1 and clock 2 can be used to model frequency error and time drift for LR as follows,

- Clock 1:

  - maximum frequency error [20, 200] ppm

  - Frequency drifting [1] ppm/s

- Clock 2:

  - maximum frequency error [5] ppm

  - Frequency drifting [0.05] ppm/s

  - Clock 2 cannot be available when MR is in ultra-deep sleep.

- Sampling rate/ADC bit-width

The OOK/ASK based waveform is detected based on amplitude of samples in time domain, the number of samples used for symbol determination directly impacts the performance of LP-WUS. The sampling rate of digital unit in WUR is lower compared with legacy NR receivers which support tens or hundreds MHz channel bandwidth. Around 4/2/1/0.5MHz sampling rates can be assumed in evaluation. For easier modeling in LLS, the sampling rate can be 3.84MHz, 1.92MHz and etc., which are obtained via the typical sampling rate of 30.72MHz in NR divided by S= 8 and 16, respectively. Other S values, which belongs to 2^(N) can be reported.

Proposal 19:  For easier modeling in LLS, the typical value in NR, i.e., 30.72MHz divided by S can be considered as the sampling rate for LP-WUS, where S belongs to 2^(N) can be reported.

For LP-WUR, the most popular candidate waveform is OOK, and the information is delivered simply through signal amplitude, i.e., on/off. Theoretically, the on/off state can be easily detected by a comparator followed by envelop detector. However, if the interference is strong enough, the comparator cannot work well, and the detection performance will be greatly degraded in high SINR region. If multi-bit ADC is used instead of comparator, better performance can be achieved. Due to limitation by complexity and low power consumption, ADC with long bit length may be not preferable. Hence, a 1-bit comparator or 2/4-bits ADC can be assumed in evaluation. The ADC bit-width can be reported for the evaluated cases.

Proposal 20:  The ADC bit width can be selected among {1, 2, 4, 8, …} in coverage evaluation.

- Noise Figure

In previous meetings, it was agreed that noise figure can be selected from [9, 12, 15, 18, 21, 24] db. One of the above values should be reported together with MIL.

Proposal 21:  The noise figure among [9, 12, 15, 18, 21, 24] dB should be reported in coverage evaluation.

Besides, the assumed FAR should be reported, which has impact in both coverage and power evaluation.

Based on the discussion above, the simulation assumptions to be reported are summarized in following table. Some candidate values and settings are provided in Table 10, and other assumptions are not precluded.

Proposal 22:  Adopt Table 15 in R1-2300476 as the assumptions for coverage evaluation for LP-WUS.

Table 15. Simulation assumptions for coverage evaluation for LP-WUS

| Attributes | Assumptions |
| --- | --- |
| Carrier Frequency | 2.6GHz/4GHz/700MHz |
| Waveform | OOK, etc… |
| Channel structure | - Sync+ payload + CRC - Sync length: 16 chips - Payload: 12, 32, 48, … bits (2) - CRC length: {5, 8, 16, … bits - Sequence only: - Sequence length: {14, 16, 28, 32, …} chips - Number of information bits delivered: {1, 2, 4, …} |
| # of segments in one OFDM symbol (M)/chip rates | M = {1, 2, 4, 8…} Note: the chip rates can be derived accordingly, e.g., chip rates are {28, 56, 112, 224} kbps, respectively |
| WUS duration | Number of symbols: e.g., 4, 8 symbols |
| Performance metric | FAR: {[0.1%, 1%, 10%]}; MDR: 1% |
| Code scheme | 1/2, 1/4, … code rate Manchester coding Note: For information bits and CRC bits |
| SCS of OFDM generator | 30kHz |
| gNB Channel BW | 20MHz (50 RB), FFS other values |
| WUS BW | {4, 6, 8, 10, 12, …} RBs |
| Guard band | E.g., 6 subcarriers, {1, 2, …} RBs on each side of LP-WUS. |
| Filter | X-th Order Butterworth with Y MHz bandwidth X = {2, 3, 4, 5, …} Y can be WUS BW + Z, where Z is the delta value on WUS BW. |
| Adjacent subcarrier interference | PDSCH mapped on RBs not used for LP-WUS and guard band; EPRE of LP-WUS vs EPRE of PDSCH =ρ, where ρ= {0, 3, 6} dB; |
| Sampling Rate | S*30.72MHz, where S = (2^-N) *30.72MHz, and N = 2, 3, 4, 5, …. |
| ADC bit width | {1, 2, 4, 8, …} |
| Channel Model | TDL-C 300ns |
| Noise Figure for WUR | Among [9, 12, 15, 18, 21, 24] dB. |

  - Preliminary Evaluation results

In the section, we provide our preliminary evaluation results. Detailed simulation assumptions are provided in Appendix C and several configurations definition are listed as followed:

- Config-1:preamble length-16 chips, payload-12bits, CRC-8bits

- Config-2: sequence length 28 chips

- Config-3: sequence length 8 chips

The chip rate for all the three configurations is 56kbps, which means there are two segments for on/off states within one OFDM symbol.

We provide the coverage comparison between LP-WUS and legacy NR signals for both normal UE and Redcap UE. As shown in following figures, the LP-WUS configs can achieve higher MIL than PUSCH. In the three configurations above, LP-WUS Config-2 with length 28, can achieve the best performance. The MIL of LP-WUS con cannot achieve comparable MIL as PDCCH AL16, while it can be expected that the MIL can be comparable with PDCCH with low aggregation levels for normal UE. For R18 Redcap UE with 1Rx, the MIL of LP-WUS can be comparable with PDCCH AL8.

Figure 16. MIL comparison between PUSCH/PDCCH and LP-WUS for Urban 2.6GHz for normal UE

Figure 17. MIL comparison between PUSCH/PDCCH and LP-WUS for Rural 700MHz for normal UE

Figure 18. MIL comparison between PUSCH/PDCCH and LP-WUS for Urban 2.6GHz for Redcap UE

Figure 19. MIL comparison between PUSCH/PDCCH and LP-WUS for Rural 700MHz for Redcap UE

- Conclusion

In this contribution, the target use cases, applicable RRC states, design targets/KPIs and evaluation methodologies for R18 LP-WUS/WUR study are discussed with the following observations and proposals:

Observation 1: In the case that the relative power of LP-WUR “ON” state is 0.03~0.5 unit,

- substantial power saving gain e.g., up to 80% can be achieved.

- LP-WUR should address all traffic arrival cases for IoT/Wearable/eMBB, thus keeping the “ON” state power consumption as less as possible is important for increasing the battery life of the device.

Observation 2: Compared with I-DRX paging, LP-WUR/WUS scheme with continuously monitoring configuration can achieve around 50%~98% power saving gain when the relative power of LP-WUR “ON” state is no more than 1 unit, with marginal latency increase.

Observation 3: Compared with eDRX, LP-WUR/WUS scheme with continuously monitoring configuration can largely reduce the paging latency (22x), with comparable UE power consumption.

Observation 4:  LP-WUR/WUS scheme provides a much better trade-off between latency and power consumption when relative power of LP-WUR “ON” state is no more than 1unit, compared with I-DRX paging and eDRX scheme.

Observation 5: High relative power of LP-WUR “ON” state is not feasible when continuously LP-WUS monitoring is configured.

Observation 6: Discontinuous LP-WUS monitoring can significantly reduce UE power consumption of LP-WUS scheme, but latency will increase accordingly, compared with continuous LP-WUS monitoring configuration.

Observation 7: With fixed duty cycle ratio and relative power of LP-WUR “ON” state, different LP-WUR DRX cycle lengths have no or less impact on UE power consumption of LP-WUS scheme, and the difference only depends on the number of LP-WUR ON-OFF transition.

Observation 8: For LP-WUS scheme, latency will be reduced with the increase of duty cycle ratio of LP-WUS monitoring, while UE power consumption will increase accordingly.

Observation 9: Up to 65% power saving gain can be achieved by LP-WUS scheme when duty cycle ratio is extremely low e.g., 0.1% or 2% even in the case of high relative power of LP-WUS “ON” state e.g., 20units.

Observation 10: As FAR for LP-WUS increases, UE power consumption goes up. And a sharp raising of UE power consumption is observed when FAR reaches 10%.

Observation 11: Even if RRM measurement is performed by main radio at a relaxed level, the total UE power consumption of LP-WUS scheme increases distinctly.

Observation 12: RRM measurement performed by WUR only will greatly help reduce UE power consumption of LP-WUS scheme.

Observation 13: As the paging rate and the number of UE in group increase, power consumption of LP-WUS scheme will increase but latency will slightly decrease.

Observation 14: The UE power consumption and latency of LP-WUS scheme will increase as ramp-up and sync/re-sync time/energy increase.

Observation 15: The impact of ramp up and sync time/energy on UE power consumption of LP-WUS scheme will be remarkable in intensive traffic scenario (i.e., larger paging rate) due to larger number of MR transition.

Observation 16: The motivation to study LP-WUS/WUR in RRC connected mode for XR service is to reduce the excessive PDCCH monitoring due to unpredictable jitter.

Observation 17: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring combined with main receiver micro sleep can bring {6%~15%} additional UE power saving gain with no capacity loss in both low load and high load cases.

Observation 18: Compared to the existing R15/16/17 power saving schemes, LP-WUS monitoring combined with main receiver light sleep can bring {10%~22%} additional UE power saving gain, with acceptable capacity loss at least in low load case.

Observation 19: UPT of LP-WUR scheme (wake-up latency 0ms case) is the same as always-on scheme which means it can effectively reduce power consumption without affecting network scheduling.

Observation 20: When the relative power of WUR 'On' state is no more than 1unit, LP-WUS monitoring with PDCCH skipping scheme can achieve the best trade-off performance in both UPT and power saving, compared with the existing UE power saving schemes.

Observation 21: When the relative power of WUR 'On' state is larger than 1unit i.e., 10 or 20 units, LP- WUS monitoring with PDCCH skipping scheme has no power saving gain advantage, compared with the existing power saving schemes.

Observation 22:  For IDLE/INACTIVE mode, the upper bound of resource overhead used for LP-WUS is less than 1% for 100MHz system BW even in worst case such as the paging arrives every POs in every PF (i.e., 51200 idle/inactive UEs). For CONNECTED mode, the resource overhead of LP-WUS is less than 0.5% even in high load cases such as10 XR UEs per cell.

| Resource overhead ratio, R | RRC Idle/inactive mode | RRC Connected mode |  |
| --- | --- | --- | --- |
|  |  | XR traffic | eMBB traffic |
| 20MHz, 30KHz SCS | 0.36% ~ 5.04% | - | - |
| 100MHz, 30KHz SCS | 0.072% ~ 1.01% | 0.43% | 0.036% |

Observation 23: Additional network energy consumption caused by LP-WUS/WUR operation can be minimized, e.g., gNB can transmit LP-WUS in the slot with existing NR signal e.g., SSB.

Observation 24: About -81 dBm receiver sensitivity is required for LP-WUS, to achieve the same coverage as PUSCH 1Mbps in 2.6GHz Urban scenario.

Proposal 1: Capture the following use cases in the TR of LP-WUS/WUR:

- For IoT cases: The battery should last at least few years; Latency would be within several or tens of seconds; For latency sensitive IoT cases, the latency requirement is within 1 or 2 seconds; The mobility would be stationary or nomadic.

- For wearable cases: The battery should last for a few weeks; Latency need to be within several seconds; Low/medium speed is required.

- For XR or smart phone cases: support to provide higher power saving gains on top of existing solutions; Latency need to be within several milliseconds for XR traffic and within tens of milliseconds for other eMBB traffics; Low/medium speed is required.

Proposal 2: The target relative power of LP-WUR “ON” state should be less than 1 unit.

Proposal 3: The latency brought by using LP-WUS/WUR consists two parts: wake-up delay and MR transition time.

Proposal 4: Consider the following as the latency target for LP-WUS/WUR:

- Within several or tens of seconds for IoT cases; And within 1 or 2 seconds for latency-sensitive IoT cases;

- Within several seconds for wearable cases;

- Within several milliseconds for XR use case; Within tens of milliseconds for other eMBB use cases;

Proposal 5: Around 100kbps data rate can be considered as design target for LP-WUS.

Proposal 6: For sync/re-sync time and energy, Table 2 of R1-2300476 can be assumed.

Proposal 7: Adopt Alt1 for ramp-up/down transition time and energy for ultra-deep sleep state of MR.

Proposal 8: Transition time between LP-WUR ‘on’ and ‘off’ states can be assumed as 10 or 20ms.

Proposal 9: For RRM measurement assumptions, the following options can be considered.

- Option 1: RRM measurement is only performed by MR.

- Option 2: LP-WUR performs RRM measurement based on periodic lower power signal e.g., LP-SS. MR performs relaxed RRM measurement every X I-DRX cycles, where X can be 10 or 20. And

- Option 3: RRM measurement is only performed by LP-WUR.

Proposal 10: For R18 LP-WUS/WUR power evaluation in RRC connected mode, during the LP-WUS monitoring, the power state of main radio can be micro or light sleep.

Proposal 11: For normal UE supporting 4 Rx, 2 Rx should be assumed in evaluation when the coverage of paging PDCCH is used to compared with that of LP-WUS.

Proposal 12: For coverage evaluation, waveform/modulation type should be reported, e.g., OOK, ASK, etc.

Proposal 13: For coverage evaluation, code scheme should be reported, e.g., Manchester code, repetition, etc.

Proposal 14:  For coverage evaluation, LP-WUS channel structure should be reported, and the following structures can be considered

- [Sync (X chips)] + data (Y bits) + CRC (Z bits);

- Sequence only (X chips)

Proposal 15:  For coverage evaluation, the LP-WUS bandwidth can be selected in number of continuous RBs, e.g., {4, 8, 12, 16, …}.

Proposal 16: For modeling adjacent subcarrier interference in LLS, PDSCH can be mapped on RBs not used for LP-WUS and guard band.

Proposal 17:  The BPF/LPF parameters and guard band size should be reported in coverage evaluation.

- BPF/LPF parameters may include bandwidth, filter order and filter type (e.g., Butterworth, …);

- The guard band size can be a number of RBs/subcarriers.

Proposal 18:  For coverage evaluation, clock 1 and clock 2 can be used to model frequency error and time drift for LR as follows,

- Clock 1:

  - maximum frequency error [20, 200] ppm

  - Frequency drifting [1] ppm/s

- Clock 2:

  - maximum frequency error [5] ppm

  - Frequency drifting [0.05] ppm/s

  - Clock 2 cannot be available when MR is in ultra-deep sleep.

Proposal 19:  For easier modeling in LLS, the typical value in NR, i.e., 30.72MHz divided by S can be considered as the sampling rate for LP-WUS, where S belongs to 2^(N) can be reported.

Proposal 20:  The ADC bit width can be selected among {1, 2, 4, 8, …} in coverage evaluation.

Proposal 21:  The noise figure among [9, 12, 15, 18, 21, 24] dB should be reported in coverage evaluation.

Proposal 22:  Adopt Table 15 in R1-2300476 as the assumptions for coverage evaluation for LP-WUS.

Table 15. Simulation assumptions for coverage evaluation for LP-WUS

| Attributes | Assumptions |
| --- | --- |
| Carrier Frequency | 2.6GHz/4GHz/700MHz |
| Waveform | OOK, etc… |
| Channel structure | - Sync+ payload + CRC - Sync length: 16 chips - Payload: 12, 32, 48, … bits (2) - CRC length: {5, 8, 16, … bits - Sequence only: - Sequence length: {14, 16, 28, 32, …} chips - Number of information bits delivered: {1, 2, 4, …} |
| # of segments in one OFDM symbol (M)/chip rates | M = {1, 2, 4, 8…} Note: the chip rates can be derived accordingly, e.g., chip rates are {28, 56, 112, 224} kbps, respectively |
| WUS duration | Number of symbols: e.g., 4, 8 symbols |
| Performance metric | FAR: {[0.1%, 1%, 10%]}; MDR: 1% |
| Code scheme | 1/2, 1/4, … code rate Manchester coding Note: For information bits and CRC bits |
| SCS of OFDM generator | 30kHz |
| gNB Channel BW | 20MHz (50 RB), FFS other values |
| WUS BW | {4, 6, 8, 10, 12, …} RBs |
| Guard band | E.g., 6 subcarriers, {1, 2, …} RBs on each side of LP-WUS. |
| Filter | X-th Order Butterworth with Y MHz bandwidth X = {2, 3, 4, 5, …} Y can be WUS BW + Z, where Z is the delta value on WUS BW. |
| Adjacent subcarrier interference | PDSCH mapped on RBs not used for LP-WUS and guard band; EPRE of LP-WUS vs EPRE of PDSCH =ρ, where ρ= {0, 3, 6} dB; |
| Sampling Rate | S*30.72MHz, where S = (2^-N) *30.72MHz, and N = 2, 3, 4, 5, …. |
| ADC bit width | {1, 2, 4, 8, …} |
| Channel Model | TDL-C 300ns |
| Noise Figure for WUR | Among [9, 12, 15, 18, 21, 24] dB. |

# References

- Chairman’s Notes of RAN1 #111 (Online meeting, November 14th – 18th, 2022).

- RP-222644, Revised SID: Study on low-power Wake-up Signal and Receiver for NR, RAN#97e, vivo.

- TR 38.830, Study on NR coverage enhancements, v17.0.0.

- R1-2211032, Discussion on physical signal and procedure for low power WUS, vivo.

- R1-2213005, Final FL summary of evaluation on low power WUS, RAN1#111, vivo.

- R1-1712984, Downlink channel power efficiency for MTC, RAN1#90, vivo.

# Appendix A – Processing timeline for the schemes in RRC idle/inactive mode

The assumed processing timelines of I-DRX paging, eDRX and LP-WUS/WUR schemes.

# Appendix B – Evaluation assumption for RRC connected mode

The detail configurations for R17 PDCCH monitoring adaptation and LP-WUS/WUR schemes are illustrated as below.

R17 PDCCH monitoring adaptation scheme includes both PDCCH skipping indication and SSSG switching indication, and its key assumptions are listed below:

- Default/sparse SSSG: do PDCCH monitoring in every two slots;

- Dense SSSG: do PDCCH monitoring in every slot.

- Two candidate PDCCH skipping durations: 4ms and 6ms.

- Enhanced DRX (i.e., aligning DRX cycle with non-integer traffic periodicity) is configured and the length of DRX onduration is equal to the jitter range.

For LP-WUS/WUR scheme, the key assumptions are introduced below:

- The LP-WUS monitoring is performed within DRX onduration.

- The main receiver enters into the micro sleep when doing LP-WUS monitoring.

- The total relative power for LP-WUS monitoring is assumed as 45 or 20 power units (i.e., the main radio enters micro or light sleep).

- And the latency from receiving LP-WUS to be ready for PDCCH monitoring is set to 1 slot.

- R17 PDCCH skipping indication is adopted. And three candidate PDCCH skipping durations are 2ms, 4ms and 6ms respectively.

- Enhanced DRX (i.e., aligning DRX cycle with non-integer traffic periodicity) is configured and the length of DRX onduration is equal to the jitter range.

- System capacity is assumed as the maximum number of users per cell with at least 90% of UEs being satisfied. And a UE can be regarded as a satisfied UE if more than 99% or 95% of packets are successfully transmitted within a given air interface PDB.

Table 14. Simulation assumption for FR1 Indoor Hotspot scenario

| Parameter | value |
| --- | --- |
| Scenarios | Indoor Hotspot, 12 nodes in 50 m x 120 m |
| Channel model | InH |
| Carrier frequency | 4GHz |
| Bandwidth | 100 MHz, 1.72% Guard Band |
| Subcarrier spacing | 30 KHz |
| Frame structure | DDDSU (S: 10D:2G:2U) |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 32T: (4,4,2,1,1;4,4), (dH,dV) = (0.5, 0.5)λ |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,1/2,2,1,1;1,1/2), (dH, dV) = (0.5, N/A) λ |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi |
| BS Power | 24 dBm per 20MHz |
| UE max Power | 23 dBm |
| UE Power | Max Tx power: 23 dBm, (P0 = -80, alpha = 0.8) |
| ISD | 20 m |
| BS height | 3 m |
| UE height | 1.5 m |
| Noise Figure | BS:5 dB, UE:9 dB |
| Max MCS | 256QAM |
| Device deployment | 100% indoor |
| Down-tilt | 90 degrees |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic |
| Target BLER | 10% |
| UE speed | 3 km/h |

Table 15. The DL video traffic models with 60 FPS adopted in R17 XR SI

| Traffic model | VR/AR | VR/AR |
| --- | --- | --- |
| Data rate (Mbps) | 30 | 45 |
| Packet size distribution | Truncated Gaussian distribution |  |
| Mean packet size (Bytes) | 62500 | 93750 |
| STD of packet sizes (Bytes) | 6562 | 9844 |
| Maximum packet size (Bytes) | 93750 | 140625 |
| Minimum packet size (Bytes) | 31250 | 46875 |
| Packet arrival interval (ms) | 16.67 | 16.67 |
| PDB (ms) | 10 |  |
| Jitter distribution | Truncated Gaussian distribution |  |
| Jitter Mean (ms) | 0 |  |
| Jitter STD (ms) | 2 or 5 |  |
| Jitter Range (ms) | [-4, +4], [-6, +6] or [-8, +8] |  |

# Appendix C – Evaluation Results on Coverage for LP-WUS

In the evaluation results shown in Appendix D, some common parameters are assumed, provided as follows

| Attributes | Assumptions |  |  |
| --- | --- | --- | --- |
| Carrier Frequency | 2.6GHz |  |  |
| Case name | LP-WUS Config1 | LP-WUS Config2 | LP-WUS Config3 |
| Channel structure | sync: 16 chips data: 12bits(24 chips) CRC: 8 bits (16 chips) | Sequence only: 28 chips | Sequence only: 8 chips |
| Chip rate | 56kbps | 56kbps | 56kbps |
| WUS duration | 2 slots | 1 slot | 4 symbols |
| Performance metric | FAR <0.1% MDR 1% | FAR < 1% MDR 1% | FAR < 1% MDR 1% |
| Coding | 1/2 rate Manchester coding (For information bits and CRC bits) |  |  |
| maximum frequency error | 200 ppm |  |  |
| Frequency drifting | 1 ppm/s |  |  |
| Beacon periodicity | 1 sec Note: beacon periodicity is used to calculate the time drift for WUS monitoring |  |  |
| SCS | 30kHz |  |  |
| gNB Channel BW | 20MHz (50 RB) |  |  |
| WUS BW | 12RB ~4.32MHz |  |  |
| Guard band | 1RB on each side of LP-WUS bandwidth |  |  |
| Filter | 5th Order Butterworth with 4.32MHz bandwidth |  |  |
| ASCI | PDSCH mapped on RBs not used for LP-WUS and guard band; EPRE of LP-WUS vs EPRE of PDSCH = 1:1. |  |  |
| Sampling Rate | 3.84 MHz |  |  |
| ADC bitwidth | 4 bit ADC |  |  |
| Channel Model | TDL-C 300 |  |  |

Detailed MIL calculation for LP-WUS for above configurations, are provided in the following attachment which is calculated according to the template provided in [3]

# Appendix D – Agreements of RAN1#111 meeting

Agreement For system impact analysis, the following performance metrics are considered to be provided, Performance Metric; Note. System overhead; expressed as percentage of used part of all REs for LP-WUS (including guard band or time or others resource used for LP-WUR if any) among all resources Other assumptions related to the system overhead analysis can be reported, e.g., the LP-WUR raw data rate evaluated in the coverage evaluations.. Capacity impact; Evaluate the system capacity impact due to introducing of LP-WUS Note: it is for UEs which are in connected mode. Definition is the same as in XR TR.. FFS: NW power consumption / Energy Efficiency; [Impact of LP-WUS/WUR operation on gNB energy consumption as performance metric in system impact analysis.]. For power and latency evaluation of the LP-WUS, the following performance metrics definitions provided for future study Performance Metric; Note. Power consumption; Relative power consumption in units. The power consumption includes main radio and LP-WUR. For comparison, the relative power consumption and evaluation period for baseline schemes should also be provided, as well as the power saving gain (i.e., percentage of power consumption reduction of the proposed power saving scheme from the baseline scheme).. Latency; For IDLE/INACTIVE state, - the latency is the time interval between the data arrival time at the gNB and the time of the first PO UE can monitor the paging message - alternatively, if UE is not required to monitor a PO after wake-up, company to report detailed procedure and definition of the latency . In RAN1#111, there are no definitions being precluded - sync/re-sync for main radio is included. UPT; The definition is the same as in [TR38.840] Note: it is for connected mode purpose.. Companies to report baseline scheme, e.g., PO monitoring with i-DRX, e-DRX, with or without PEI Companies to report the power consumption / power saving gain considering the FAR impact, latency considering MDR impact Other performance metrics (e.g., mobility) can be reported by companies (if any) Agreement Update the IDLE/INACTIVE state traffic model option 1 as follows and remove traffic model option 2, - The traffic arrival is modeled as a Poisson Arrival Process where inter-arrival times are exponentially distributed, the mean arrival time is P = YREF / RE, REF, where - RE, REF= 1%, 0.1%, 0.01% or 0.001% and YREF = 1.28s - Per group paging probability RG = 1 – (1 – RE)N, where N is the number of UEs in the group - FFS: Value of N - For LP-WUS - Both per group and UE paging can be assumed. Note： - For i-DRX with i-DRX cycle duration Y second, - Per UE paging probability RE = 1 – (1 – RE, REF )Y/YREF - Per group paging probability RG = 1 – (1 – RE)N, where N is the number of UEs in the group - For e-DRX with K i-DRX cycles duration, L PTW duration of L i-DRX cycles, and an i-DRX cycle duration Y second - Per UE paging probability is - RE = 1 – (1 – RE, REF )(K-L)Y/YREF for the first i-DRX cycle within the PTW - RE = 1 – (1 – RE, REF )LY/YREF for each of the remaining L-1 i-DRX cycles within the PTW - Per group paging probability RG = 1 – (1 – RE)N, where N is the number of UEs in the group - L=4 (as agreed in RAN1#110bis) Agreement For MR, at least for FR1 evaluation, - Number of SSBs for sync/re-sync for MR is up to 10 - Companies to report timeline and energy consumption - Companies to provide feasibility analysis for transition time and transition energy with aim to converge to one or two set of values in RAN1#112 Agreement The following power model for LP-WUR is used for evaluation for FR1, Power State; Relative Power (unit); Transition energy: (unit multiplied by ms); Ramp-up time TLR, ramp-up (ms). Off; 0.001; [TLR, ramp-up *(PON+POFF)/2]; TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up. On; 0.005/0.01/0.02/0.03/0.05/0.1/0.2/0.5/1/2/4 FFS: If other values are needed. FFS: whether further categorization/sub-categorization is needed and how.