3GPP TSG RAN WG1 #112                                                               R1-2301800

Athens, Greece, February 27th – March 3rd, 2023

Agenda item:	8.16.1

Source: 	Moderator (NTT DOCOMO, INC.)

Title: 	Summary#1 on UE features for NR MBS

Document for:	Discussion and Decision

- Introduction

This document summarizes contributions submitted to AI 8.16.1 regarding UE features for NR MBS and captures company views based on the announcement in the following email thread.

[112-R17-UE_features_1] To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc – Hiroki (DOCOMO)

Based on the latest RAN1 UE features list on Rel-17 NR MBS in [1] and contributions submitted to AI 8.16.1, following remaining issues are identified.

- Reporting type of following FGs are not yet decided [2, 4, 6, 7, 8, 9, 10].

  - 33-5-1e	Dynamic Slot-level repetition for SPS group-common PDSCH for multicast

  - 33-5-1f	NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast

  - 33-5-1g	DCI-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast

  - 33-5-1i	Multicast SPS scheduling with DCI format 4_2

  - 33-5-2	Multiple SPS group-common PDSCH configuration on PCell

  - 33-8-1	PUCCH resource configuration for multicast feedback for dynamically scheduled multicast

  - 33-9	Supporting unicast PDCCH to release SPS group-common PDSCH

- Brackets on component 1 of FG33-5-1a remain, and it may impact to potential additional component of FG33-5-1 and/or 33-5-3 [3, 5, 6, 7, 8, 9, 10].

- FFSs on “Consequence if the feature is not supported by the UE” remain for FG33-3-3a/33-3-3b [4, 7, 9, 10].

- There are proposals to update component of FG33-5-1f and FG33-5-1j [5, 6, 9, 10].

- There are proposals to update FG33-8-1 [9, 10].

- There is a proposal to update component and note of FG33-1-2 [5].

- There is a proposal to update component, prerequisite FGs and note of FG33-3-2 [5].

- There is a proposal to update FG name and component of FG33-4 [5].

- There is a proposal to update reporting type and prerequisite of FG33-6-x [6].

- There is a proposal to update reporting type and note of FG33-5-1h/33-10 [7].

- There is a proposal to update note of FG33-1 [8].

- There is a proposal to update component of FG33-2 [9].

- There is a proposal to update prerequisite of FG33-5-1f [10].

- Discussion on UE features for NR MBS

2.1	Reporting type of FGs that have not been decided yet

FG33-5-1e

In [1], FG 33-5-1e is captured as below.

| 33. NR_MBS | 33-5-1e | Dynamic Slot-level repetition for SPS group-common PDSCH for multicast | Support up to X times dynamic slot-level repetition for SPS group-common PDSCH for multicast. | 33-5-1 | Yes |  |  | [Per UE] | [No] | [No] |  | Candidate values for X is: {8, 16} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Following views are provided in contributions for the RAN1#112 meeting.

| [2] | Nokia, Nokia Shanghai Bell | - 33-5-1e/f/g/i: - Per UE |
| --- | --- | --- |
| [4] | ZTE | The reporting type for FG33-5-1e, 33-5-1f, 33-5-1g, 33-5-1i, 33-5-2, 33-8-1, 33-9 is still FFS. From our perspective, all of them can be reported as per UE without FR1/FR2 separation and without TDD/FDD separation. Proposal 2: The reporting type for FG33-5-1e, 33-5-1f, 33-5-1g, 33-5-1i, 33-5-2, 33-8-1, 33-9 is per UE without FR1/FR2 separation and without TDD/FDD separation. |
| [6] | Qualcomm Incorporated | 33. NR_MBS; 33-5-1e; Dynamic Slot-level repetition for SPS group-common PDSCH for multicast; Support up to X times dynamic slot-level repetition for SPS group-common PDSCH for multicast.; 33-5-1; Yes; Per BC; N/A; N/A; Candidate values for X is: {8, 16}; Optional with capability signalling. |
| [7] | NTT DOCOMO, INC. | For dynamic slot-level repetition, we do not see any reason to adopt different reporting types between dynamic scheduling (FG 33-3-1) and SPS (FG 33-5-1e). For FG 33-5-1e, ‘per UE’ should be supported with FDD/TDD and FR1/FR2 differentiations as in FG 33-3-1. In addition, as we did for FGs 33-3-1/33-3-1a, TN/NTN differentiation and licensed/unlicensed differentiation can be done by adding separate FG for NTN/unlicensed with per-band. Proposal 5: The reporting type of FG 33-5-1e is per UE with FDD/TDD and FR1/FR2 differentiations, and the note “This FG is reported for TN and licensed” is added. Proposal 6: Following FG is added. 33. NR_MBS; 33-5-1k; Dynamic Slot-level repetition for SPS group-common PDSCH for multicast for NTN and unlicensed; Support up to X times dynamic slot-level repetition for SPS group-common PDSCH for multicast for NTN and unlicensed.; 33-5-1; Yes; Per band; N/A; N/A; Candidate values for X is: {8, 16} This FG is reported for NTN and unlicensed; Optional with capability signalling. |
| [8] | MediaTek Inc. | 33. NR_MBS; 33-5-1e; Dynamic Slot-level repetition for SPS group-common PDSCH for multicast; Support up to X times dynamic slot-level repetition for SPS group-common PDSCH for multicast.; 33-5-1; Yes; Per UE; N/A; N/A; Candidate values for X is: {8, 16} Reporting type of FGs 33-5-1e is per UE with [FDD/TDD,] FR1/FR2, licensed/unlicensed, and TN/NTN differentiation, detail signalling is up to RAN2; Optional with capability signalling. |
| [9] | Ericsson | Regarding the reporting of FGs 33-5-1e/f/g/i, and 33-5-2 we propose following the template of 33-5-1h and use per UE with FDD/TDD , licensed unlicensed, NTN/TN and FR1/2 differentiation - FG 33-5-1e/f/g/i and 33-5-2 are reported as Per UE with FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation. |
| [10] | Huawei, HiSilicon | 33. NR_MBS; 33-5-1e; Dynamic Slot-level repetition for SPS group-common PDSCH for multicast; Support up to X times dynamic slot-level repetition for SPS group-common PDSCH for multicast.; 33-5-1; Yes; Per BC; N/A; N/A; Candidate values for X is: {8, 16}; Optional with capability signalling. |

Based on above, companies’ views can be summarized as below.

- Reporting type of FG33-5-1e

  - Alt.1: Per UE [2, 4, 7, 8, 9]

    - 1-1: Per UE with FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation [7, 8, 9]

      - Apply same approach as 33-3-1/1a, i.e., add separate per-band FG for unlicensed and NTN [7]

      - Apply same approach as 33-5-1h/33-10, i.e., detailed signaling is up to RAN2 [8, 9]

    - 1-2: Per UE without FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation [2, 4]

  - Alt.2: Per BC [6, 10]

The moderator’s proposal is to take Alt.1-1.

Proposal 2-1:

- Reporting type of FG33-5-1e is Per UE with FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation.

| Company | Comment |
| --- | --- |
| DCM | Support |
| Spreadtrum | We are fine with per BC. |

FG33-5-1f

In [1], FG 33-5-1f is captured as below.

| 33. NR_MBS | 33-5-1f | NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast | 1) Support NACK-only based HARQ-ACK feedback, and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling 2) Support of PTM retransmission associated with G-CS-RNTI for SPS multicast | 33-5-1 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Following views are provided in contributions for the RAN1#112 meeting.

| [2] | Nokia, Nokia Shanghai Bell | - 33-5-1e/f/g/i: - Per UE |
| --- | --- | --- |
| [4] | ZTE | The reporting type for FG33-5-1e, 33-5-1f, 33-5-1g, 33-5-1i, 33-5-2, 33-8-1, 33-9 is still FFS. From our perspective, all of them can be reported as per UE without FR1/FR2 separation and without TDD/FDD separation. Proposal 2: The reporting type for FG33-5-1e, 33-5-1f, 33-5-1g, 33-5-1i, 33-5-2, 33-8-1, 33-9 is per UE without FR1/FR2 separation and without TDD/FDD separation. |
| [6] | Qualcomm Incorporated | 33. NR_MBS; 33-5-1f; NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast; 1) Support NACK-only based HARQ-ACK feedback, and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling a) A single TB with NACK-only feedback transmitted in PUCCH b) multiple TBs with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits 3. One or multiple TB with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits 4. One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits when multiplexing with other UCI; 33-5-1a; Yes; Per BC; N/A; N/A; Optional with capability signalling. |
| [7] | NTT DOCOMO, INC. | For FG 33-5-1f, we do not see any reason to adopt different reporting types between ACK/NACK based feedback (FG 33-5-1a) and NACK-only based feedback (FG 33-5-1f). ‘per BC’ should be supported as in FG 33-5-1a. Proposal 3: The reporting type of FG 33-5-1f is per BC. |
| [8] | MediaTek Inc. | 33. NR_MBS; 33-5-1f; NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast; 1) Support NACK-only based HARQ-ACK feedback, and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling 2) Support of PTM retransmission associated with G-CS-RNTI for SPS multicast; 33-5-1; Yes; Per BC; N/A; N/A; Optional with capability signalling. |
| [9] | Ericsson | Regarding the reporting of FGs 33-5-1e/f/g/i, and 33-5-2 we propose following the template of 33-5-1h and use per UE with FDD/TDD , licensed unlicensed, NTN/TN and FR1/2 differentiation - FG 33-5-1e/f/g/i and 33-5-2 are reported as Per UE with FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation. |
| [10] | Huawei, HiSilicon | 33. NR_MBS; 33-5-1f; NACK-only based HARQ-ACK feedback for multicast with ACK/NACK transforming and RRC-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast; 1) Support NACK-only based HARQ-ACK feedback, including: a) A single TB with NACK-only feedback transmitted in PUCCH b) multiple TBs with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits c) One or multiple TBs with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits. d). One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits when multiplexing with other UCI 2) Support of shared SPS-PUCCH-AN-List configuration from unicast SPS. 3) support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling 4) Support of PTM retransmission associated with G-CS-RNTI for SPS multicast; 33-5-1a; Yes; Per BC; N/A; N/A; Optional with capability signalling. |

Based on above, companies’ views can be summarized as below.

- Reporting type of FG33-5-1f

  - Alt.1: Per UE [2, 4, 9]

    - 1-1: Per UE with FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation [9]

    - 1-2: Per UE without FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation [2, 4]

  - Alt.2: Per BC [6, 7, 8, 10]

The moderator’s proposal is to take Alt.2.

Proposal 2-2:

- Reporting type of FG33-5-1f is Per BC.

| Company | Comment |
| --- | --- |
| DCM | Support |
| Huawei, HiSilicon | ok |
| Spreadtrum | Ok |

FG33-5-1g

In [1], FG 33-5-1g is captured as below.

| 33. NR_MBS | 33-5-1g | DCI-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast | Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC signaling via DCI format 4_2 | 33-5-1f, 33-5-1i | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Following views are provided in contributions for the RAN1#112 meeting.

| [2] | Nokia, Nokia Shanghai Bell | - 33-5-1e/f/g/i: - Per UE |
| --- | --- | --- |
| [4] | ZTE | The reporting type for FG33-5-1e, 33-5-1f, 33-5-1g, 33-5-1i, 33-5-2, 33-8-1, 33-9 is still FFS. From our perspective, all of them can be reported as per UE without FR1/FR2 separation and without TDD/FDD separation. Proposal 2: The reporting type for FG33-5-1e, 33-5-1f, 33-5-1g, 33-5-1i, 33-5-2, 33-8-1, 33-9 is per UE without FR1/FR2 separation and without TDD/FDD separation. |
| [6] | Qualcomm Incorporated | 33. NR_MBS; 33-5-1g; DCI-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast; Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC signaling via DCI format 4_2; 33-5-1f, 33-5-1i; Yes; Per band; N/A; N/A; Optional with capability signalling. |
| [7] | NTT DOCOMO, INC. | For FG 33-5-1g, from similar reason to the above, ‘per band’ should be supported as in FG 33-5-1b. Proposal 4: The reporting type of FG 33-5-1g is per band. |
| [8] | MediaTek Inc. | 33. NR_MBS; 33-5-1g; DCI-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast; Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC signaling via DCI format 4_2; 33-5-1f, 33-5-1i; Yes; Per band; N/A; N/A; Optional with capability signalling. |
| [9] | Ericsson | Regarding the reporting of FGs 33-5-1e/f/g/i, and 33-5-2 we propose following the template of 33-5-1h and use per UE with FDD/TDD , licensed unlicensed, NTN/TN and FR1/2 differentiation - FG 33-5-1e/f/g/i and 33-5-2 are reported as Per UE with FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation. |
| [10] | Huawei, HiSilicon | 33. NR_MBS; 33-5-1g; DCI-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast; Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC signaling via DCI format 4_2; 33-5-1f, 33-5-1i; Yes; Per BC; N/A; N/A; Optional with capability signalling. |

Based on above, companies’ views can be summarized as below.

- Reporting type of FG33-5-1g

  - Alt.1: Per UE [2, 4, 9]

    - 1-1: Per UE with FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation [9]

    - 1-2: Per UE without FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation [2, 4]

  - Alt.2: Per band [6, 7, 8]

  - Alt.3: Per BC [10]

The moderator’s proposal is to take Alt.2.

Proposal 2-3:

- Reporting type of FG33-5-1g is Per band.

| Company | Comment |
| --- | --- |
| DCM | Support |
| Huawei, HiSilicon | ok |
| Spreadtrum | Support |

FG33-5-1i

In [1], FG 33-5-1i is captured as below.

| 33. NR_MBS | 33-5-1i | Multicast SPS scheduling with DCI format 4_2 | 1.Support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling 2. Retransmission scheduled by DCI format 4_2 with CRC scrambled with G-CS-RNTI | 33-5-1 | Yes |  |  | FFS | FFS | FFS |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Following views are provided in contributions for the RAN1#112 meeting.

| [2] | Nokia, Nokia Shanghai Bell | - 33-5-1e/f/g/i: - Per UE |
| --- | --- | --- |
| [4] | ZTE | The reporting type for FG33-5-1e, 33-5-1f, 33-5-1g, 33-5-1i, 33-5-2, 33-8-1, 33-9 is still FFS. From our perspective, all of them can be reported as per UE without FR1/FR2 separation and without TDD/FDD separation. Proposal 2: The reporting type for FG33-5-1e, 33-5-1f, 33-5-1g, 33-5-1i, 33-5-2, 33-8-1, 33-9 is per UE without FR1/FR2 separation and without TDD/FDD separation. |
| [6] | Qualcomm Incorporated | 33. NR_MBS; 33-5-1i; Multicast SPS scheduling with DCI format 4_2; Support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling Retransmission scheduled by DCI format 4_2 with CRC scrambled with G-CS-RNTI; 33-5-1; Yes; Per band; N/A; N/A; Optional with capability signalling. |
| [7] | NTT DOCOMO, INC. | According to the same logic as above, ‘per band’ should be supported for FG 33-5-1i as in FG 33-2f. Proposal 7: The reporting type of FG 33-5-1i is per band. |
| [8] | MediaTek Inc. | 33. NR_MBS; 33-5-1i; Multicast SPS scheduling with DCI format 4_2; 1.Support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling 2. Retransmission scheduled by DCI format 4_2 with CRC scrambled with G-CS-RNTI; 33-5-1; Yes; Per band; N/A; N/A; Optional with capability signalling. |
| [9] | Ericsson | Regarding the reporting of FGs 33-5-1e/f/g/i, and 33-5-2 we propose following the template of 33-5-1h and use per UE with FDD/TDD , licensed unlicensed, NTN/TN and FR1/2 differentiation - FG 33-5-1e/f/g/i and 33-5-2 are reported as Per UE with FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation. |
| [10] | Huawei, HiSilicon | 33. NR_MBS; 33-5-1i; Multicast SPS scheduling with DCI format 4_2; 1.Support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling 2. Retransmission scheduled by DCI format 4_2 with CRC scrambled with G-CS-RNTI; 33-5-1; Yes; Per band; N/A; N/A; Optional with capability signalling. |

Based on above, companies’ views can be summarized as below.

- Reporting type of FG33-5-1i

  - Alt.1: Per UE [2, 4, 9]

    - 1-1: Per UE with FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation [9]

    - 1-2: Per UE without FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation [2, 4]

  - Alt.2: Per band [6, 7, 8, 10]

The moderator’s proposal is to take Alt.2.

Proposal 2-4:

- Reporting type of FG33-5-1i is Per band.

| Company | Comment |
| --- | --- |
| DCM | Support |
| Huawei, HiSilicon | ok |
| Spreadtrum | Support |

FG33-5-2

In [1], FG 33-5-2 is captured as below.

| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration on PCell | 1. Support up to 8 SPS group-common PDSCH configuration per CFR for multicast 2. Support M>=1 activated SPS group-common PDSCH configurations per CFR for multicast 3. The total number of SPS configurations for both multicast and unicast is no larger than 8 in a BWP of a serving cell, and activated SPS group-common PDSCH configurations is no larger than M. 4. The total number of SPS configurations for both multicast and unicast in a cell group is no larger than 32 | 33-5-1 | Yes |  |  | [Per UE] | [No] | [No] |  | Candidate value set for M is {1, 2, …, 8} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Following views are provided in contributions for the RAN1#112 meeting.

| [2] | Nokia, Nokia Shanghai Bell | - 33-5-2: - Per UE |
| --- | --- | --- |
| [4] | ZTE | The reporting type for FG33-5-1e, 33-5-1f, 33-5-1g, 33-5-1i, 33-5-2, 33-8-1, 33-9 is still FFS. From our perspective, all of them can be reported as per UE without FR1/FR2 separation and without TDD/FDD separation. Proposal 2: The reporting type for FG33-5-1e, 33-5-1f, 33-5-1g, 33-5-1i, 33-5-2, 33-8-1, 33-9 is per UE without FR1/FR2 separation and without TDD/FDD separation. |
| [6] | Qualcomm Incorporated | 33. NR_MBS; 33-5-2; Multiple SPS group-common PDSCH configuration on PCell; 1. Support up to 8 SPS group-common PDSCH configuration per CFR for multicast 2. Support M>=1 activated SPS group-common PDSCH configurations per CFR for multicast 3. The total number of SPS configurations for both multicast and unicast is no larger than 8 in a BWP of a serving cell, and activated SPS group-common PDSCH configurations is no larger than M. 4. The total number of SPS configurations for both multicast and unicast in a cell group is no larger than 32; 33-5-1; Yes; Per FS; N/A; N/A; Candidate value set for M is {1, 2, …, 8}; Optional with capability signalling. |
| [7] | NTT DOCOMO, INC. | The reporting type of FG for support of up to 8 SPS configurations for unicast (FG 12-2) is per band; correspondingly the reporting type of FG 33-5-2 should also be per band. Proposal 8: The reporting type of FG 33-5-2 is per band. |
| [8] | MediaTek Inc. | 33. NR_MBS; 33-5-2; Multiple SPS group-common PDSCH configuration on PCell; 1. Support up to 8 SPS group-common PDSCH configuration per CFR for multicast 2. Support M>=1 activated SPS group-common PDSCH configurations per CFR for multicast 3. The total number of SPS configurations for both multicast and unicast is no larger than 8 in a BWP of a serving cell, and activated SPS group-common PDSCH configurations is no larger than M. 4. The total number of SPS configurations for both multicast and unicast in a cell group is no larger than 32; 33-5-1; Yes; Per FS; N/A; N/A; Candidate value set for M is {1, 2, …, 8}; Optional with capability signalling. |
| [9] | Ericsson | Regarding the reporting of FGs 33-5-1e/f/g/i, and 33-5-2 we propose following the template of 33-5-1h and use per UE with FDD/TDD , licensed unlicensed, NTN/TN and FR1/2 differentiation - FG 33-5-1e/f/g/i and 33-5-2 are reported as Per UE with FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation. |
| [10] | Huawei, HiSilicon | 33. NR_MBS; 33-5-2; Multiple SPS group-common PDSCH configuration on PCell; 1. Support up to 8 SPS group-common PDSCH configurations per CFR for multicast 2. Support M>=1 activated SPS group-common PDSCH configurations per CFR for multicast 3. The total number of SPS configurations for both multicast and unicast is no larger than 8 in a BWP of a serving cell, and activated SPS group-common PDSCH configurations is no larger than M. 4. The total number of SPS configurations for both multicast and unicast in a cell group is no larger than 32; 33-5-1; Yes; Per BC; N/A; N/A; Candidate value set for M is {1, 2, …, 8}; Optional with capability signalling. |

Based on above, companies’ views can be summarized as below.

- Reporting type of FG33-5-2

  - Alt.1: Per UE [2, 4, 9]

    - 1-1: Per UE with FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation [9]

    - 1-2: Per UE without FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation [2, 4]

  - Alt.2: Per band [7]

  - Alt.3: Per BC [10]

  - Alt.4: Per FS [6, 8]

The moderator’s proposal is to take Alt.1-1, Alt.2 or Alt.3 as compromise.

Proposal 2-5:

- Reporting type of FG33-5-2 is [Per UE with FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation or Per band or Per BC].

| Company | Comment |
| --- | --- |
| DCM | We prefer ‘per band’, but OK with ‘per UE’. |
| Huawei, HiSilicon | We guess per BC would be the middle ground for compromise. |
| Spreadtrum | We are fine with either per and or per BC or per FS. |

FG33-8-1

In [1], FG 33-8-1 is captured as below.

| 33. NR_MBS | 33-8-1 | PUCCH resource configuration for multicast feedback for dynamically scheduled multicast | Support of a PUCCH-Config for multicast HARQ-ACK feedback, separate from that of unicast configurations | 33-2a | Yes |  |  | [Per band or per FSPC] | [No] | [No] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Following views are provided in contributions for the RAN1#112 meeting.

| [2] | Nokia, Nokia Shanghai Bell | - 33-8-1: - Per band as a compromise |
| --- | --- | --- |
| [4] | ZTE | The reporting type for FG33-5-1e, 33-5-1f, 33-5-1g, 33-5-1i, 33-5-2, 33-8-1, 33-9 is still FFS. From our perspective, all of them can be reported as per UE without FR1/FR2 separation and without TDD/FDD separation. Proposal 2: The reporting type for FG33-5-1e, 33-5-1f, 33-5-1g, 33-5-1i, 33-5-2, 33-8-1, 33-9 is per UE without FR1/FR2 separation and without TDD/FDD separation. |
| [6] | Qualcomm Incorporated | 33. NR_MBS; 33-8-1; PUCCH resource configuration for multicast feedback for dynamically scheduled multicast; Support of a PUCCH-Config for multicast HARQ-ACK feedback, separate from that of unicast configurations; 33-2a; Yes; Per BC; N/A; N/A; Optional with capability signalling. |
| [7] | NTT DOCOMO, INC. | We do not see any necessity to make the reporting type of FG 33-8-1 finer granularity than per BC. Besides, at least typically separate PUCCH-config would be configured for NACK-only feedback since PUCCH resource for NACK-only feedback would be shared among UEs. FGs 33-4/4a are per BC; thus, the same granularity should be reasonable. Proposal 9: The reporting type of FG 33-8-1 is per BC. |
| [8] | MediaTek Inc. | 33. NR_MBS; 33-8-1; PUCCH resource configuration for multicast feedback for dynamically scheduled multicast; Support of a PUCCH-Config for multicast HARQ-ACK feedback, separate from that of unicast configurations; 33-2a; Yes; Per FSPC; N/A; N/A; Optional with capability signalling. |
| [9] | Ericsson | Regarding the signalling of the capability, we think that per band would be enough, since the pre-requisite capability already gives per BC granularity. - In FG 33-8-1, the signalling is per BC, and the pre-requisite FG is kept to 33-2a. |
| [10] | Huawei, HiSilicon | Proposal 3: Updating FG33-8-1 as follows in red: 33. NR_MBS; 33-8-1; PUCCH resource configuration for multicast feedback for dynamically scheduled multicast; Support of a PUCCH-Config for multicast HARQ-ACK feedback, separate from that of unicast configurations; 33-2a or 33-4; Yes; Per BC; No; No; Note: With 33-2a or 33-4 as prerequisite FG, this FG33-8-1 includes the case of ACK/NACK for multicast or NACK-only mode1 for multicast.; Optional with capability signalling. |

Based on above, companies’ views can be summarized as below.

- Reporting type of FG33-8-1

  - Alt.1: Per UE without FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation [4]

  - Alt.2: Per band [2]

  - Alt.3: Per BC [6, 7, 9, 10]

  - Alt.4: Per FSPC [8]

The moderator’s proposal is to take Alt.3.

Proposal 2-6:

- Reporting type of FG33-8-1 is Per BC.

| Company | Comment |
| --- | --- |
| DCM | Support |
| Huawei, HiSilicon | ok |
| Spreadtrum | Support |

FG33-9

In [1], FG 33-9 is captured as below.

| 33. NR_MBS | 33-9 | Supporting unicast PDCCH to release SPS group-common PDSCH | Supports unicast PDCCH scrambled with CS-RNTI to release SPS group-common PDSCH | 33-5-1, 12-2 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Following views are provided in contributions for the RAN1#112 meeting.

| [2] | Nokia, Nokia Shanghai Bell | - 33-9: - Per UE |
| --- | --- | --- |
| [4] | ZTE | The reporting type for FG33-5-1e, 33-5-1f, 33-5-1g, 33-5-1i, 33-5-2, 33-8-1, 33-9 is still FFS. From our perspective, all of them can be reported as per UE without FR1/FR2 separation and without TDD/FDD separation. Proposal 2: The reporting type for FG33-5-1e, 33-5-1f, 33-5-1g, 33-5-1i, 33-5-2, 33-8-1, 33-9 is per UE without FR1/FR2 separation and without TDD/FDD separation. |
| [6] | Qualcomm Incorporated | We suggest the changes for the remaining FFSs of FG 33-9 as 33. NR_MBS; 33-9; Supporting unicast PDCCH to release SPS group-common PDSCH; Supports unicast PDCCH scrambled with CS-RNTI to release SPS group-common PDSCH; 33-5-1, 12-2; Yes; Per BC; N/A; N/A; Optional with capability signalling. |
| [7] | NTT DOCOMO, INC. | The process of checking if it is a DCI for SPS release would be band-independent. Reporting type per UE without FDD/TDD and FR1/FR2 differentiations would be sufficient. Proposal 10: The reporting type of FG 33-9 is per UE without FDD/TDD and FR1/FR2 differentiations. |
| [8] | MediaTek Inc. | 33. NR_MBS; 33-9; Supporting unicast PDCCH to release SPS group-common PDSCH; Supports unicast PDCCH scrambled with CS-RNTI to release SPS group-common PDSCH; 33-5-1, 12-2; Yes; Per BC; N/A; N/A; Optional with capability signalling. |
| [9] | Ericsson | Finally, in 33-9-1, the discussion in RAN1#111 also remained open with choices between reporting per UE (alt1), BC(alt3), or per UE with differentiation for FDD/TDD, NTN, licensed/unlicensed, and FR1/FR2 (alt2). In our view, Alt2 is a good compromised considering the pre-requisites. - FG 33-9-1 is reported as Per UE with FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation. |
| [10] | Huawei, HiSilicon | 33. NR_MBS; 33-9; Supporting unicast PDCCH to release SPS group-common PDSCH; Supports unicast PDCCH scrambled with CS-RNTI to release SPS group-common PDSCH; 33-5-1, 12-2; Yes; Per BC; N/A; N/A; Optional with capability signalling. |

Based on above, companies’ views can be summarized as below.

- Reporting type of FG33-9

  - Alt.1: Per UE [2, 4, 7, 9]

    - 1-1: Per UE with FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation [9]

    - 1-2: Per UE without FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation [2, 4, 7]

  - Alt.2: Per BC [6, 8, 10]

The moderator’s proposal is to take Alt.1-1 as compromise.

Proposal 2-7:

- Reporting type of FG33-9 is Per UE with FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation.

| Company | Comment |
| --- | --- |
| DCM | OK for progress. |
| Spreadtrum | Fine with per UE without FDD/TDD, FR1/FR2, licensed/unlicensed, TN/NTN differentiation |

2.2	Brackets on component 1 of FG33-5-1a

In [1], FG 33-5-1/33-5-1a/33-5-3 are captured as below.

| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast on PCell | 1. Support one SPS group-common PDSCH configuration for multicast 2. Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH | 33-2 | Yes |  |  | Per FS | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-5-1a | Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | 1. Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, [SPS group-common PDSCH activation, and SPS release PDCCH] 2. Support of PTM retransmission for SPS multicast associated with G-CS-RNTI 3. Support of Type-1 and Type-2 HARQ-ACK CB for SPS multicast feedback only 4. Support of shared SPS-PUCCH-AｄN-List configuration from unicast SPS | 33-5-1 | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-3 | One SPS group-common PDSCH configuration for multicast for Scell | 1. Support one SPS group-common PDSCH configuration for multicast for Scell. 2. Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH for Scell. | 33-5-1, 33-2h | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |

Following views are provided in contributions for the RAN1#112 meeting.

| [3] | Spreadtrum Communications | Firstly, after further checking the latest the spec and the previous agreements, we have not found any descriptions pointing out that ACK/NACK-based HARQ-ACK feedback for SPS group-common PDCCH activation and SPS release must be supported if supporting SPS group-common PDSCH for multicast. On the contrary, we have agreements in RAN1#109e to explicitly state that ACK/NACK-based HARQ-ACK feedback for SPS group-common PDCCH activation and SPS release is in included in FG33-5-1a, which is optional. Agreement - FG 33-5-1a is confirmed as follows 33. NR_MBS; 33-5-1a; Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast; Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, SPS group-common PDSCH activation, and SPS release PDCCH; 33-5-1; Yes; FFS; FFS; FFS; Optional with capability signalling.. Secondly, we have not seen any serious issue for the case where no feedback for SPS group-common PDSCH activation PDCCH, and SPS release PDCCH. Up to 32 AL can be configured for PDCCH, and the reliability of PDCCH can be very very high. The event that PDCCH cannot be decoded nearly would not take place, even in real deployment. In addition, in light of FG33-5-1a, SPS group-common PDSCH without PDCCH scheduling also can be w/o feedback. When it happens, it can be interpreted that gNB would ensure the reliability of PDSCH, or gNB does not care about whether UE can success to receive PDSCH. If it is the former, we believe gNB also could ensure the reliability of activation/release PDCCH. If it is the latter, there is also no need to require the confirmation of activation/release PDCCH. Thirdly, considering so complex HARQ mechanisms introduced for MBS, including multiplexing with unicast, we think it may be not good choice to develop HARQ mechanism only to support the confirmation of activation/release PDCCH. Based on the above, we have the following proposal: Proposal 1: Remove the bracket in the component 1 of FG 33-5-1a. |
| --- | --- | --- |
| [5] | vivo | For SPS group-common PDSCH for multicast activation and release, the following agreement was achieved in previous RAN1 meeting. One reason to make such agreement includes whether there can be no HARQ-ACK feedback for multicast SPS activation/deactivation. It is clear that UE should feedback HARQ-ACK information for multicast SPS activation/deactivation based on the agreement, i.e., disabled HARQ-ACK is not applicable for multicast SPS activation/deactivation, which should be included in 33-5-1a. Agreement: @RAN1-107 For multicast SPS activation/deactivation, only ACK/NACK based feedback is supported Proposal 4: Update 33-5-1a to include the UE capability for enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling. 33. NR_MBS; 33-5-1a; Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast; 1. Support of ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH and SPS release PDCCH, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling 2. Support of PTM retransmission for SPS multicast associated with G-CS-RNTI 3. Support of Type-1 and Type-2 HARQ-ACK CB for SPS multicast feedback only 4. Support of shared SPS-PUCCH-AN-List configuration from unicast SPS; 33-5-1; Yes; Per BC; N/A; N/A; Optional with capability signalling. |
| [6] | Qualcomm Incorporated | 33. NR_MBS; 33-5-1; SPS group-common PDSCH for multicast on PCell; 1. Support one SPS group-common PDSCH configuration for multicast 2. Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH 3. Support of group-common PDCCH/PDSCH with CRC scrambled by G-CS-RNTI(s) for multicast 4. Support of DCI format 4_1 with CRC scrambled with G-CS-RNTI for multicast 5. ACK/NACK-based HARQ-ACK feedback for SPS group-common PDCCH activation and SPS release associated with G-CS-RNTI; 33-2; Yes; Per FS; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-5-1a; Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast; 1. Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, 2. Support of PTM retransmission for SPS multicast associated with G-CS-RNTI 3. Support of Type-1 and Type-2 HARQ-ACK CB for SPS multicast feedback only 4. Support of shared SPS-PUCCH-AN-List with unicast; 33-5-1; Yes; Per BC; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-5-3; One SPS group-common PDSCH configuration for multicast for Scell; 1. Support one SPS group-common PDSCH configuration for multicast for Scell. 2. Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH for Scell. 3. Support of group-common PDCCH/PDSCH with CRC scrambled by G-CS-RNTI(s) for multicast 4. Support of DCI format 4_1 with CRC scrambled with G-CS-RNTI for multicast 5. ACK/NACK-based HARQ-ACK feedback for SPS group-common PDCCH activation and SPS release associated with G-CS-RNTI; 33-5-1, 33-2h; Yes; Per FSPC; N/A; N/A; Optional with capability signalling; Optional with capability signalling. |
| [7] | NTT DOCOMO, INC. | For FGs 33-5-1/1a, it seems that the discussion point is not UE capability perspective but exact specification-based UE behavior. For HARQ feedback enabling/disabling and SPS activation/release, the following description can be found in 38.213. A UE can be configured per G-RNTI for multicast or per G-CS-RNTI, by harq-FeedbackEnablerMulticast with value set to 'enabled', to provide HARQ-ACK information for PDSCH receptions. When the UE is not provided harq-FeedbackEnablerMulticast for a G-RNTI for multicast or G-CS-RNTI and pdsch-HARQ-ACK-Codebook = dynamic for multicast HARQ-ACK information, the UE does not provide HARQ-ACK information for respective PDSCH receptions. If a UE is provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI for multicast or a G-CS-RNTI, the UE provides HARQ-ACK information for PDSCH receptions scheduled by multicast DCI format 4_1 associated with the G-RNTI or the G-CS-RNTI, and determines whether or not to provide the HARQ-ACK information for PDSCH receptions scheduled by multicast DCI format 4_2 based on an indication by the multicast DCI format 4_2 associated with the G-RNTI for multicast or the G-CS-RNTI [4, TS 38.212]. If a UE is provided pdsch-HARQ-ACK-Codebook = semi-static for multicast HARQ-ACK information, the UE does not expect to be provided harq-FeedbackEnablerMulticast with value set to 'dci-enabler' for a G-RNTI or a G-CS-RNTI.. Based on the above text with red color, especially ‘for respective PDSCH receptions’, the correct UE behavior can be interpreted as whether HARQ feedback for activation (i.e., for initial SPS PDSCH after activation) is determined by the enabling/disabling signaling and HARQ feedback for release PDCCH is always performed regardless of the enabling/disabling signaling. If this interpretation is common among companies, activation part should be maintained in FG 33-5-1a while release part should be moved to FG 33-5-1. Proposal 2: Update FGs 33-5-1 and 33-5-1a as follows. 33. NR_MBS; 33-5-1; SPS group-common PDSCH for multicast on PCell; 1. Support one SPS group-common PDSCH configuration for multicast 2. Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH 3. ACK/NACK-based HARQ-ACK feedback for SPS release associated with G-CS-RNTI; 33-2; Yes; Per FS; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-5-1a; Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast; 1. Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, [and SPS group-common PDSCH activation (i.e., initial SPS PDSCH after activation), and SPS release PDCCH] 2. Support of PTM retransmission for SPS multicast associated with G-CS-RNTI 3. Support of Type-1 and Type-2 HARQ-ACK CB for SPS multicast feedback only 4. Support of shared SPS-PUCCH-AN-List configuration from unicast SPS; 33-5-1; Yes; Per BC; N/A; N/A; Optional with capability signalling. |
| [8] | MediaTek Inc. | Regarding whether HARQ-ACK can be enabled/disabled for the SPS activation/release as discussed in the FG33-5-1a, we prefer the legacy unicast SPS behaviour is reused for the multicast SPS, which means that the HARQ-ACK is needed for the multicast SPS activation/release. Otherwise, the network cannot determine whether the UE is activated/released successfully. Proposal 3: The HARQ-ACK is needed and cannot be disabled for the multicast SPS activation/release, and the corresponding FGs can be modified as follows: 33. NR_MBS; 33-5-1a; Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast; 1. Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, 2. Support of PTM retransmission for SPS multicast associated with G-CS-RNTI 3. Support of Type-1 and Type-2 HARQ-ACK CB for SPS multicast feedback only 4. Support of shared SPS-PUCCH-AN-List configuration from unicast SPS; 33-5-1; Yes; Per BC; N/A; N/A; Optional with capability signalling. |
| [9] | Ericsson | During the past meetings, we discussed including additional components in FG for SPS 33-5-1 as follow: High priority proposal 2-11-2: - Components of FG 33-5-1 are revised as - Add a component “Support of group-common PDCCH/PDSCH with CRC scrambled by G-CS-RNTI(s) for multicast” - Add a component “Support of DCI format 4_1 with CRC scrambled with G-CS-RNTI for multicast” - Add a component “ACK/NACK-based HARQ-ACK feedback for SPS group-common PDCCH activation and SPS release associated with G-CS-RNTI” The last component is controversial for some companies, with the argument that HARQ for SPS is captured in FG 33-5-1a. however, HARQ for the SPS activation and release and HARQ for SPS PDSCH transmissions are separately specified. For SPS activation and release, HARQ is not optional. The following agreement was struck at RAN1#107e: Agreement For multicast SPS activation/deactivation, only ACK/NACK based feedback is supported.. We therefore support that the basic FG for SPS must support HARQ response for activation and release, and support the original proposal by the UE feature FL. Similarly, FG 33-5-3 for SPS in SCell should be modified accordingly. - Components of FG 33-5-1 are revised as - Add a component “Support of group-common PDCCH/PDSCH with CRC scrambled by G-CS-RNTI(s) for multicast” - Add a component “Support of DCI format 4_1 with CRC scrambled with G-CS-RNTI for multicast” - Add a component “ACK/NACK-based HARQ-ACK feedback for SPS group-common PDCCH activation and SPS release associated with G-CS-RNTI” - In component 1 in 33-5-1, remove the following text: [SPS group-common PDSCH activation, and SPS release PDCCH] - Components of FG 33-5-3 are revised as - Add a component “Support of group-common PDCCH/PDSCH with CRC scrambled by G-CS-RNTI(s) for multicast” - Add a component “Support of DCI format 4_1 with CRC scrambled with G-CS-RNTI for multicast” - Add a component “ACK/NACK-based HARQ-ACK feedback for SPS group-common PDCCH activation and SPS release associated with G-CS-RNTI” - In component 1 in 33-5-1, remove the following text: [SPS group-common PDSCH activation, and SPS release PDCCH] |
| [10] | Huawei, HiSilicon | Whether the disabling HARQ-ACK feedback applies to multicast SPS activation or release was discussed in the last meeting as well as the corresponding update in the UE feature group 33-5-1a, but it was not concluded in the last meeting. As discussed in the accompany paper [2], it is proposed that disabling HARQ-ACK feedback is NOT applied to multicast SPS activation or release given there is no benefit to do so. Accordingly, the first component in FG33-5-1a needs to update by deleting “[SPS group-common PDSCH activation, and SPS release PDCCH]”. 33. NR_MBS; 33-5-1a; Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast; 1. Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, [SPS group-common PDSCH activation, and SPS release PDCCH] 2. Support of PTM retransmission for SPS multicast associated with G-CS-RNTI 3. Support of Type-1 and Type-2 HARQ-ACK CB for SPS multicast feedback only 4. Support of shared SPS-PUCCH-AN-List configuration from unicast SPS; 33-5-1; Yes; Per BC; N/A; N/A; Optional with capability signalling. |

Based on above, companies’ views can be summarized as below.

- Component 1 of FG33-5-1a

  - Alt.1: Remove brackets i.e., keep ”SPS group common PDSCH activation, and SPS release PDCCH” in component 1 of FG33-5-1a [3]

  - Alt.2: Remove “SPS group common PDSCH activation, and SPS release PDCCH” from component 1 of FG33-5-1a [5, 6, 8, 9, 10]

    - 2-1: Additionally, following components are added for FG33-5-1 and FG33-5-3 [6, 9]

      - 3. Support of group-common PDCCH/PDSCH with CRC scrambled by G-CS-RNTI(s) for multicast

      - 4. Support of DCI format 4_1 with CRC scrambled with G-CS-RNTI for multicast

      - 5. ACK/NACK-based HARQ-ACK feedback for SPS group-common PDCCH activation and SPS release associated with G-CS-RNTI

  - Alt.3: Remove only “and SPS release PDCCH” from component 1 of FG33-5-1a and keep “SPS group common PDSCH activation (i.e., initial SPS PDSCH after activation)” in component 1 of FG33-5-1a [7]

    - 3-1: Additionally, “ACK/NACK-based HARQ-ACK feedback for SPS release associated with G-CS-RNTI” is added as component 3 of FG33-5-1 [7]

The moderator’s proposal is to take Alt.2-1 as starting point for the discussion.

Proposal 2-8:

- Remove “SPS group common PDSCH activation, and SPS release PDCCH” from component 1 of FG33-5-1a

- Additionally, following components are added for FG33-5-1 and FG33-5-3

  - 3. Support of group-common PDCCH/PDSCH with CRC scrambled by G-CS-RNTI(s) for multicast

  - 4. Support of DCI format 4_1 with CRC scrambled with G-CS-RNTI for multicast

  - 5. ACK/NACK-based HARQ-ACK feedback for SPS group-common PDCCH activation and SPS release associated with G-CS-RNTI

| Company | Comment |
| --- | --- |
| DCM | It seems that there is discussion on related CR in maintenance agenda. We can wait for the outcome. If HARQ feedback shall be performed for initial SPS PDSCH regardless of the enabling/disabling configuration, the feedback should be mandatory for MBS. Regarding SPS release PDCCH, we do not think enabling/disabling feedback is not applied to PDCCH since the parameter only covers feedback for PDSCH, as we pointed out it in our contribution. Regardless of the CR discussion, SPS PDCCH release should not be included here. |
| Huawei, HiSilicon | Ok with the proposal |
| Spreadtrum | The issue is being discussed in maintenance agenda, we can wait the progress. |

2.3	FFSs on “Consequence if the feature is not supported by the UE” remain for FG33-3-3a/33-3-3b

In [1], FG 33-3-3a/33-3-3b are captured as below.

| 33. NR_MBS | 33-3-3a | FDM-ed Type-1 and Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast | 1. Support of FDM-ed Type-1 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and ACK/NACK-based HARQ-ACK for multicast on PUCCH or PUSCH 2. Support of Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast on PUCCH or PUSCH with max number X of G-RNTIs | 33-3-2, at least one of {33-2a, 33-4, 33-5-1a, 33-5-1f} | Yes |  | FFS | Per BC | N/A | N/A |  | Note1: FDM-ed Type-1 HARQ-ACK codebook is generated by concatenating the Type-1 sub-codebook for unicast and the Type-1 sub-codebook for multicast. Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast. Candidate values of X is {1, 2, 3, 4} with X no larger than max number of G-RNTIs of FG33-2e Note: the value of X should be common across FG33-2a, 33-3-3a and 33-3-3b if reported | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-3-3b | Mode 2 TDM-ed Type-1 and Type-2 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast | 1. Support of Mode 2 TDM-ed Type-1 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and ACK/NACK-based HARQ-ACK for multicast on PUCCH or PUSCH 2. Support of Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast on PUCCH or PUSCH with max number X of G-RNTIs | 33-2a or 33-4 or 33-5-1a or 33-5-1f | Yes |  | FFS | Per BC | N/A | N/A |  | Note1: Mode 2 TDM-ed Type-1 HARQ-ACK codebook is generated based on the union TDRA tables from unicast and multicast and the union of k1 sets from unicast and multicast. Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast. Candidate values of X is {1, 2, 3, 4} with X no larger than max number of G-RNTIs of FG33-2e Note: the value of X should be common across FG33-2a, 33-3-3a and 33-3-3b if reported | Optional with capability signalling |

Following views are provided in contributions for the RAN1#112 meeting.

| [4] | ZTE | The “Consequence if the feature is not supported by the UE” for FG33-3-33a and FG33-3-3b is still FFS. Type-2 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast is included in both FG 33-3-3a and 33-3-3b. It is kind of “default UE behaviour”. The “Consequence if the feature is not supported by the UE” for FG 33-3-3a and FG 33-3-3b can be updated as following: If UE doesn’t indicate support of FG33-3-3a nor FG33-3-3b, UE doesn’t support multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast. Proposal 1: Add the following for “Consequence if the feature is not supported by the UE” for FG 33-3-3a and FG 33-3-3b - If UE doesn’t indicate support of FG33-3-3a nor FG33-3-3b, UE doesn’t support multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast. |
| --- | --- | --- |
| [7] | NTT DOCOMO, INC. | The remaining issue for FGs 33-3-3a/3b is what is mentioned in the column of ‘Consequence if the feature is not supported by the UE’. For this issue, necessity to add some text there seems to be unclear in the first place. There are a lot of FGs for MBS without any text in the column. Just removing ‘FFS’s would be sufficient. Proposal 1: Remove ‘FFS’ from FGs 33-3-3a/3b. |
| [9] | Ericsson | The two FGs details have been agreed during RAN1#111 but the column for consequence if not supported need to be clarified. Our view is that at least one of the two FGs have to be supported in order to support multiplexing of unicast and multicast in the same type-2 codebook. If one of the feature is not supported, what differs is how type 1 is supported. Therefore we propose the following for the description of the UE behaviour in case the FGs are not supported: - For the UE behaviour when FG 33-3-3a or FG 33-3-3b is not supported: - If 33-3-3a is not supported, FDM-ed Type-1 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and ACK/NACK-based HARQ-ACK for multicast on PUCCH or PUSCH are not supported - If FG 33-3-3b is not supported, Mode 2 TDM-ed Type-1 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and ACK/NACK-based HARQ-ACK for multicast on PUCCH or PUSCH is not supported - If neither are supported, multiplexing of type-1 unicast feedback and type 1 multicast feedback in a type1 codebook is not supported, and multiplexing of type-2 unicast feedback and type 2 multicast feedback in a type2 codebook is not supported - If either are supported, muxing unicast and multicast in type 2 HARQ feedback is supported. |
| [10] | Huawei, HiSilicon | The remaining issue for FG33-3-3a/3b is the consequence if such FGs are not supported by UE and the reason suggested to consider the consequence was because something might be different comparing with HARQ-ACK feedback for unicast, since it is mandatory for UE to support Type-1 and Type-2 HARQ-ACK feedback for unicast. Now FG33-3-3a/3b are separate UE capabilities from others, e.g., FG33-2a, FG33-4/4a, so UE may support 33-2a or FG33-4 but not support the multiplexing with the HARQ-ACK for unicast. However, network scheduling is expected to be flexible as well even with such UE implementation possibility. Considering both UE implementation and network scheduling, the first preference is that UE shall support FG33-3-3a/3b when UE supports at least one of {33-2a, 33-4, 33-5-1a, 33-5-1f}. The second preference is that network may disable HARQ-ACK feedback for multicast if UE does not support FG33-3-3a/3b instead of UE always expecting network would not schedule HARQ-ACK feedback multiplexing in the same slot. Proposal 1: Updating FG33-3-3a/3b as follows in red: 33. NR_MBS; 33-3-3a; FDM-ed Type-1 and Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast; 1. Support of FDM-ed Type-1 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and ACK/NACK-based HARQ-ACK for multicast on PUCCH or PUSCH 2. Support of Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast on PUCCH or PUSCH with max number X of G-RNTIs; 33-3-2, at least one of {33-2a, 33-4, 33-5-1a, 33-5-1f}; Yes; 1st preference: UE supports FG33-3-3a when UE supports at least one of {33-2a, 33-4, 33-5-1a, 33-5-1f} 2nd preference: If UE does not support FG33-3-3a, network may disable HARQ-ACK feedback for multicast.; Per BC; N/A; N/A; Note1: FDM-ed Type-1 HARQ-ACK codebook is generated by concatenating the Type-1 sub-codebook for unicast and the Type-1 sub-codebook for multicast. Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast. Candidate values of X is {1, 2, 3, 4} with X no larger than max number of G-RNTIs of FG33-2e Note: the value of X should be common across FG33-2a, 33-3-3a and 33-3-3b if reported; Optional with capability signalling. 33. NR_MBS; 33-3-3b; Mode 2 TDM-ed Type-1 and Type-2 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast; 1. Support of Mode 2 TDM-ed Type-1 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and ACK/NACK-based HARQ-ACK for multicast on PUCCH or PUSCH 2. Support of Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast on PUCCH or PUSCH with max number X of G-RNTIs; 33-2a or 33-4 or 33-5-1a or 33-5-1f; Yes; 1st preference: UE supports FG33-3-3b when UE supports at least one of {33-2a, 33-4, 33-5-1a, 33-5-1f} 2nd preference: If UE does not support FG33-3-3b, network may disable HARQ-ACK feedback for multicast.; Per BC; N/A; N/A; Note1: Mode 2 TDM-ed Type-1 HARQ-ACK codebook is generated based on the union TDRA tables from unicast and multicast and the union of k1 sets from unicast and multicast. Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast. Candidate values of X is {1, 2, 3, 4} with X no larger than max number of G-RNTIs of FG33-2e Note: the value of X should be common across FG33-2a, 33-3-3a and 33-3-3b if reported; Optional with capability signalling. |

Based on above, companies’ views can be summarized as below.

- Consequence if FG33-3-3a/33-3-3b is/are not supported by the UE

  - Alt.1: Clarification is necessary [4, 9, 10]

    - 1-1: If UE doesn’t indicate support of FG33-3-3a nor FG33-3-3b, UE doesn’t support multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast [4, 9]

    - 1-2: [9]

      - If 33-3-3a is not supported, FDM-ed Type-1 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and ACK/NACK-based HARQ-ACK for multicast on PUCCH or PUSCH are not supported

      - If FG 33-3-3b is not supported, Mode 2 TDM-ed Type-1 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and ACK/NACK-based HARQ-ACK for multicast on PUCCH or PUSCH is not supported

    - 1-3: [10]

      - UE supports FG33-3-3a when UE supports at least one of {33-2a, 33-4, 33-5-1a, 33-5-1f}

      - UE supports FG33-3-3b when UE supports at least one of {33-2a, 33-4, 33-5-1a, 33-5-1f}

    - 1-4: [10]

      - If UE does not support FG33-3-3a, network may disable HARQ-ACK feedback for multicast

      - If UE does not support FG33-3-3b, network may disable HARQ-ACK feedback for multicast

  - Alt.2: Clarification is not necessary, i.e., just remove FFS [7]

The moderator’s proposal is to take Alt.1-1 as starting point for the discussion.

Proposal 2-9:

- Consequence if FG33-3-3a/33-3-3b is/are not supported by the UE is that “If UE doesn’t indicate support of FG33-3-3a nor FG33-3-3b, UE doesn’t support multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast”.

| Company | Comment |
| --- | --- |
| DCM | It seems OK. If controversial, just a brank would be fine. |
| Huawei, HiSilicon | Fine for progress |
| Spreadtrum | In our understanding, even if FG33-3-3a/33-3-3b is not supported, model-1 TDM-ed Type-1 and Type-2 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast can be supported. We not understand why consider the consequence of the proposal. Hope proponent can clarify this. |

2.4	Proposals to update components of FG33-5-1f/33-5-1j

FG33-5-1f

In [1], FG 33-5-1f is captured as below.

| 33. NR_MBS | 33-5-1f | NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast | 1) Support NACK-only based HARQ-ACK feedback, and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling 2) Support of PTM retransmission associated with G-CS-RNTI for SPS multicast | 33-5-1 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Following views are provided in contributions for the RAN1#112 meeting.

| [5] | vivo | For NACK-only based HARQ-ACK feedback for dynamically scheduled group-common PDSCH for multicast 33-4 and 33-4a are defined for NACK-only mode 1 and mode 2 respectively. For SPS group-common PDSCH for multicast, similar UE capability should be defined to support NACK-only feedback for multiple TBs. Proposal 5: Update 33-5-1f to include NACK-only based HARQ-ACK feedback for SPS group-common PDSCH for multicast with ACK/NACK transforming. 33. NR_MBS; 33-5-1f; NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast with ACK/NACK transforming; 1) Support NACK-only based HARQ-ACK feedback, and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling including: a) A single TB with NACK-only feedback transmitted in PUCCH b) One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits 2) Support of shared PUCCH resource configurations with unicast 3) One or multiple TB with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits 4) One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits when multiplexing with other UCI; 33-5-1a; Yes; Per BC; N/A; N/A; Optional with capability signalling. |
| --- | --- | --- |
| [6] | Qualcomm Incorporated | 33. NR_MBS; 33-5-1f; NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast; 1) Support NACK-only based HARQ-ACK feedback, and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling a) A single TB with NACK-only feedback transmitted in PUCCH b) multiple TBs with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits 3. One or multiple TB with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits 4. One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits when multiplexing with other UCI; 33-5-1a; Yes; Per BC; N/A; N/A; Optional with capability signalling. |
| [9] | Ericsson | For the support of SPS and NACK only (FG 33-5-1f and 1j), we discussed adding the clarifications that multiple TBs HARQ feedback is transmitted by transforming NACK only to ACK NACK, transmitted over PUCCH or over PUSCH. With think such description is necessary to avoid confusion between the FGs in 33-5-1f and 33-5-1j. additionally, there were some comments that NACK only+ SPS should be limited to 2TBs, but that only applies when the HARQ bits are mapped to a single PUCCH resource in our view, i.e. for 33-5-1j. - In FG 33-5-1f, add the following components: - One or multiple TB with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits - One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits when multiplexing with other UCI |
| [10] | Huawei, HiSilicon | FG33-5-1f is expected to be the basic FG for NACK-only feedback for multicast SPS, as FG33-4 defined as the basic FG for NACK-only feedback for multicast dynamic scheduling and FG33-4a defined as additional UE capability for NACK-only mode2. As revised to FG33-4, for FG33-5-1f, the components needed to be added include a single TB with NACK-only feedback transmitted in PUCCH, multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits, support of shared SPS-PUCCH-AN-List configuration from unicast SPS, and support of PTM retransmission associated with G-CS-RNTI for SPS multicast. 33. NR_MBS; 33-5-1f; NACK-only based HARQ-ACK feedback for multicast with ACK/NACK transforming and RRC-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast; 1) Support NACK-only based HARQ-ACK feedback, including: a) A single TB with NACK-only feedback transmitted in PUCCH b) multiple TBs with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits c) One or multiple TBs with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits. d). One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits when multiplexing with other UCI 2) Support of shared SPS-PUCCH-AN-List configuration from unicast SPS. 3) support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling 4) Support of PTM retransmission associated with G-CS-RNTI for SPS multicast; 33-5-1a; Yes; Per BC; N/A; N/A; Optional with capability signalling. |

Based on above, companies’ views can be summarized as below.

- Components of FG33-5-1f

  - Alt.1: Add following components [5, 6, 9, 10]

    - One or multiple TB with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits

    - One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits when multiplexing with other UCI

  - Alt.2: Alt.1 + additionally add following components [5, 6, 10]

    - A single TB with NACK-only feedback transmitted in PUCCH

    - multiple TBs with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits

  - Alt.3: Alt.1 + Alt.2 + additionally add following component [5, 10]

    - Support of shared PUCCH resource configurations with unicast

      - FG name is also updated to add “with ACK/NACK transforming”

The moderator’s proposal is to take Alt.3 as starting point for the discussion.

Proposal 2-10:

- Add following components for FG33-5-1f

  - One or multiple TB with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits

  - One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits when multiplexing with other UCI

  - A single TB with NACK-only feedback transmitted in PUCCH

  - multiple TBs with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits

  - Support of shared PUCCH resource configurations with unicast

| Company | Comment |
| --- | --- |
| DCM | OK |
| Huawei, HiSilicon | ok |
| Spreadtrum | Generally we are OK. |

FG33-5-1j

In [1], FG 33-5-1j is captured as below.

| 33. NR_MBS | 33-5-1j | NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission for SPS group-commmon PDSCH for multicast | 1. Support NACK-only based HARQ-ACK feedback for SPS PDSCH for multicast, including: a) Multiple TBs with NACK-only feedback transmitted in PUCCH by select one PUCCH resource. 2. Support of separate SPS-PUCCH-AN-List from unicast | 33-5-1f | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Following views are provided in contributions for the RAN1#112 meeting.

| [5] | vivo | Considering that at most 4 PUCCH resources can be configured by sps-PUCCH-AN-ListMulticast, it cannot support the case of more than 2 HARQ-ACK information bits in a slot corresponding to a specific sequence or a PUCCH transmission in FG 33-5-1j, which should be clarified. Proposal 6: Update 33-5-1j to include NACK-only based HARQ-ACK feedback for SPS group-common PDSCH with only up to 2 TBs 33. NR_MBS; 33-5-1j; NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission for SPS group-common PDSCH for multicast; 1. Support NACK-only based HARQ-ACK feedback for SPS PDSCH for multicast, including: a) Up to 2 TBs with NACK-only feedback transmitted in PUCCH by select one PUCCH resource. 2. Support of separate SPS-PUCCH-AN-List from unicast 3. Single TB with NACK-only feedback transmitted in PUCCH 4. Up to 2TBs with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits; 33-5-1f; Yes; Per BC; N/A; N/A; Optional with capability signalling. |
| --- | --- | --- |
| [9] | Ericsson | For the support of SPS and NACK only (FG 33-5-1f and 1j), we discussed adding the clarifications that multiple TBs HARQ feedback is transmitted by transforming NACK only to ACK NACK, transmitted over PUCCH or over PUSCH. With think such description is necessary to avoid confusion between the FGs in 33-5-1f and 33-5-1j. additionally, there were some comments that NACK only+ SPS should be limited to 2TBs, but that only applies when the HARQ bits are mapped to a single PUCCH resource in our view, i.e. for 33-5-1j. - In FG 33-5-1j, add the following components: - Single TB with NACK-only feedback transmitted in PUCCH - Up to 2TBs with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits |
| [10] | Huawei, HiSilicon | As revised to FG33-4a, for FG33-5-1j, the components should also include b) single TB with NACK-only on PUCCH because it is for the separate SPS-PUCCH-AN-list from unicast, , and c) up to 4TBs with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits. 33. NR_MBS; 33-5-1j; NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission for SPS group-commmon PDSCH for multicast; 1. Support NACK-only based HARQ-ACK feedback for SPS PDSCH for multicast, including: a) Multiple Up to 4 TBs with NACK-only feedback transmitted in PUCCH by select one PUCCH resource. b) Single TB with NACK-only feedback transmitted in PUCCH c) Up to 4TBs with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits. 2. Support of separate SPS-PUCCH-AN-List from unicast; 33-5-1f; Yes; Per BC; N/A; N/A; Optional with capability signalling. |

Based on above, companies’ views can be summarized as below.

- Components of FG33-5-1j

  - Alt.1: Add following components and modify “Multiple” in component 1-a to “Up to 2” [5, 9]

    - Single TB with NACK-only feedback transmitted in PUCCH

    - Up to 2TBs with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits

  - Alt.2: Add following components and modify “Multiple” in component 1-a to “Up to 4” [10]

    - Single TB with NACK-only feedback transmitted in PUCCH

    - Up to 4TBs with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits

The moderator’s proposal is to take Alt.1 as starting point for the discussion.

Proposal 2-11:

- Add following components for FG33-5-1j

  - Single TB with NACK-only feedback transmitted in PUCCH

  - Up to 2TBs with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits

- Modify “Multiple” in component 1-a of FG33-5-1j to “Up to 2”

| Company | Comment |
| --- | --- |
| DCM | OK |
| Huawei, HiSilicon | ok |
| Spreadtrum | Generally we are OK. |

2.5	Proposals to update FG33-8-1

In [1], FG 33-8-1 is captured as below.

| 33. NR_MBS | 33-8-1 | PUCCH resource configuration for multicast feedback for dynamically scheduled multicast | Support of a PUCCH-Config for multicast HARQ-ACK feedback, separate from that of unicast configurations | 33-2a | Yes |  |  | [Per band or per FSPC] | [No] | [No] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Following views are provided in contributions for the RAN1#112 meeting.

| [9] | Ericsson | During RAN1#111, we discussed whether to add FG 33-4 as prerequisite for 33-8-1. In our view, 33-8-1 is the capability for separate PUCCH configuration for multicast ACK NACK feedback, i.e., the capability signals the UE ability to receive a configuration in pucch-ConfigurationListMulticast1. Therefore, FG 33-4-1 should not be a prerequisite for 33-8-1. However, the description of 33-8-1 could be extended to include NACK-only mode 1, since the same kind of PUCCH resource is required for both ACK NACK and NACK only mode 1. Alternatively, a separate capability for the configuration of a distinct PUCCH resource for NACK only in pucch-ConfigurationListMulticast2 when moreThanOneNackOnlyMode is not configured can be configured. - Regarding the configuration of a separate PUCCH for NACK only mode 1 feedback, select either of the following: - In FG 33-8-1, the first component is modified as follow: Support of a PUCCH config for multicast HARQ ACK or NACK only feedback, separate from that of unicast configurations. - Note: NACK only refers to the mode supported as described in FG 33-4. - Prerequisite include FG 33-4 - A new FG 33.8.1a is introduced as follow: 33-8-1a; PUCCH resource configuration for multicast feedback for dynamically scheduled multicast with NACK only feedback with ACK NACK transforming; Support of a PUCCH-Config for multicast NACK only feedback, separate from that of unicast configurations, where feedback, with multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits; 33-4; Yes; [Per band ]; [No]; [No]; Optional with capability signalling. |
| --- | --- | --- |
| [10] | Huawei, HiSilicon | FG33-4 is defined for NACK-only mode1 with shared PUCCH resources configured for unicast. Support of separately configured PUCCH resources from unicast for NACK-only mode1 is expected to be a separate UE FG. Given FG33-8-1 is already defined that was initially intended for at least ACK/NACK based feedback for multicast, such FG can be expanded to include the case for NACK-only mode1. Overall, the change can be made to the prerequisite FG to include FG33-2a or FG33-4 for ACK/NACK based and for NACK-only mode1 feedback, respectively. Proposal 3: Updating FG33-8-1 as follows in red: 33. NR_MBS; 33-8-1; PUCCH resource configuration for multicast feedback for dynamically scheduled multicast; Support of a PUCCH-Config for multicast HARQ-ACK feedback, separate from that of unicast configurations; 33-2a or 33-4; Yes; Per BC; No; No; Note: With 33-2a or 33-4 as prerequisite FG, this FG33-8-1 includes the case of ACK/NACK for multicast or NACK-only mode1 for multicast.; Optional with capability signalling. |

Based on above, companies’ views can be summarized as below.

- Alt.1: [9]

  - Component of FG33-8-1 is modified to “Support of a PUCCH config for multicast HARQ ACK or NACK only feedback, separate from that of unicast configurations.

    - Note: NACK only refers to the mode supported as described in FG 33-4.”

    - Prerequisite include FG33-4

  - New FG 33-8-1a is introduced for “PUCCH resource configuration for multicast feedback for dynamically scheduled multicast with NACK only feedback with ACK NACK transforming”

- Alt.2: [10]

  - Add FG33-4 as prerequisite FG for FG33-8-1

  - Add following note for FG33-8-1

    - Note: With 33-2a or 33-4 as prerequisite FG, this FG33-8-1 includes the case of ACK/NACK for multicast or NACK-only mode1 for multicast.

The moderator would like to hear companies’ views on above proposals.

Proposal 2-12:

  - TBD

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | As long as UE supports separate resources from unicast, then such resources can be used for ACK/NACK based or NACK-only mode1 based. |

2.6	Other proposals

FG33-1

In [1], FG 33-1 is captured as below.

| 33. NR_MBS | 33-1 | Broadcast | 1. Support of group-common PDCCH/PDSCH for broadcast with CRC scrambled by MCCH-RNTI. 2. Support of group-common PDCCH/PDSCH for broadcast with CRC scrambled by G-RNTI(s) for MTCH. 3. Support of CFR configuration for broadcast. 4. Support of CORESET and common search space for broadcast. 5. Support of DCI format 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. 6. Support of inter-slot TDM between unicast PDSCH and MCCH group-common PDSCH or MTCH group-common PDSCH, or between MCCH group-common PDSCH and MTCH group-common PDSCH, or among unicast PDSCH and MCCH group-common PDSCH and MTCH group-common PDSCH in different slots. 7. Support MCCH change notification indication via DCI. 8. support of higher layer configured slot-level repetition up to 8 for MTCH 9. One G-RNTI per UE is supported for broadcast reception 10. Support of FDMed MCCH and PBCH 11. Support of up to 64QAM for FR1/FR2 |  | Up to RAN2 |  |  | Up to RAN2 | Up to RAN2 | Up to RAN2 |  | It is up to RAN2 whether/how to introduce the capability for support of N > 1 G-RNTIs for broadcast for a UE | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Following proposal is provided in a contribution for the RAN1#112 meeting.

| [8] | MediaTek Inc. | Regarding the CFR number for broadcast reception, it is no clear motivation to support multiple CFR. We had agreed that the number of CFRs for multicast is no more than one, and the corresponding agreement is copied as following. We suggest the similar mechanism can be reused for broadcast reception since we also have agreed that the CFR frequency for MCCH and MTCH can be configured by SIBx. Agreement: The number of CFRs for multicast is no more than one per dedicated unicast BWP in Rel-17. Agreement For broadcast reception with RRC_IDLE/RRC_INACTIVE UEs: - The CFR frequency resources used for MCCH and MTCH are configured by SIBx;. Proposal 4: For FG 33-1, adding a note that “For component 3, only one CFR frequency resource is supported for broadcast and the CFR frequency resource is configured by SIBx”. 33. NR_MBS; 33-1; Broadcast; 1. Support of group-common PDCCH/PDSCH for broadcast with CRC scrambled by MCCH-RNTI. 2. Support of group-common PDCCH/PDSCH for broadcast with CRC scrambled by G-RNTI(s) for MTCH. 3. Support of CFR configuration for broadcast. 4. Support of CORESET and common search space for broadcast. 5. Support of DCI format 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. 6. Support of inter-slot TDM between unicast PDSCH and MCCH group-common PDSCH or MTCH group-common PDSCH, or between MCCH group-common PDSCH and MTCH group-common PDSCH, or among unicast PDSCH and MCCH group-common PDSCH and MTCH group-common PDSCH in different slots. 7. Support MCCH change notification indication via DCI. 8. support of higher layer configured slot-level repetition up to 8 for MTCH 9. One G-RNTI per UE is supported for broadcast reception 10. Support of FDMed MCCH and PBCH 11. Support of up to 64QAM for FR1/FR2; Up to RAN2; Up to RAN2; Up to RAN2; Up to RAN2; It is up to RAN2 whether/how to introduce the capability for support of N > 1 G-RNTIs for broadcast for a UE For component 3, only one CFR frequency resource is supported for broadcast and the CFR frequency resource is configured by SIBx; Optional without capability signalling. |
| --- | --- | --- |

The moderator would like to hear companies’ views on above proposal.

Proposal 2-13:

  - TBD

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | Seems clear from specification. But ok to have if agreeable to others. |

FG33-1-2

In [1], FG 33-1-2 is captured as below.

| 33. NR_MBS | 33-1-2 | FDM-ed unicast PDSCH and group-common PDSCH for broadcast | 1. Support FDM between one unicast PDSCH and one group-common PDSCH for broadcast in RRC CONNECTED mode in a slot. | 33-1 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Following proposal is provided in a contribution for the RAN1#112 meeting.

| [5] | vivo | Regrading FDMed unicast PDSCH and group-common PDSCH, the following agreements were achieved in previous meeting that the unicast PDSCH and group-common PDSCH can only be dynamically scheduled [1]. Thus, for 33-1-2 for FDM-ed unicast PDSCH and group-common PDSCH for broadcast, it should be clarified that the unicast PDSCH should be dynamically scheduled. Agreement It is clarified that for the FDMed unicast PDSCH and group-common PDSCH capability of RRC_CONNECTED UE, the unicast PDSCH and group-common PDSCH can only be dynamically scheduled. Further down-selection between the following: - Opt 1: the unicast/multicast SPS retransmission PDSCH is included in the dynamically scheduled PDSCH for FDMed scheduling - FFS: whether new UE capability is needed - Opt 2: the unicast/multicast SPS retransmission PDSCH is NOT included in the dynamically scheduled PDSCH for lFDMed scheduling. Proposal 1: Update 33-1-2 to include the UE capability for FDMed unicast dynamically scheduled PDSCH and group-common PDSCH for broadcast. 33. NR_MBS; 33-1-2; FDM-ed unicast PDSCH and group-common PDSCH for broadcast; 1. Support FDM between one unicast dynamically scheduled PDSCH and one group-common PDSCH for broadcast in RRC CONNECTED mode in a slot.; 33-1; Yes; Per FSPC; N/A; N/A; NOTE: SPS PDSCH retransmission is included in dynamically scheduled PDSCH; Optional with capability signalling. |
| --- | --- | --- |

The moderator would like to hear companies’ views on above proposal.

Proposal 2-14:

  - TBD

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | Ok |

FG33-2

In [1], FG 33-2 is captured as below.

| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast for PCell | 1. Support of group-common PDCCH/PDSCH for multicast with CRC scrambled by G-RNTI for PCell. 2. Support of CFR configuration for multicast. 3. Support of CORESET and common search space configuration for multicast. 4. Support of DCI format 4_1 with CRC scrambled with G-RNTI for multicast. 5. Support of inter-slot TDM between group-common PDSCH for multicast and other PDSCHs in different slots. 6. Support {2, 4, 8} times semi-static slot-level repetition for group-common PDSCH for multicast |  | Yes |  |  | Per FS | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Following proposal is provided in a contribution for the RAN1#112 meeting.

| [9] | Ericsson | when FG 33-2 is signaled, the components do not state how many GRNTIs may be used. In broadcast, a component exists and states a single GRNTI is supported for broadcast reception. We propose to use similar wording for 33-2. - add the following component to FG 33-2: - One G-RNTI per UE is supported for multicast reception |
| --- | --- | --- |

The moderator would like to hear companies’ views on above proposal.

Proposal 2-15:

  - TBD

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | Ok |

FG33-3-2

In [1], FG 33-3-2 is captured as below.

| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and one group-common PDSCH for multicast | 1. Support FDM between one unicast PDSCH and one group-common PDSCH for multicast in RRC CONNECTED mode in a slot. | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  | Note: this FG does not support FDMed SPS | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Following proposal is provided in a contribution for the RAN1#112 meeting.

| [5] | vivo | Regrading FDMed unicast PDSCH and multicast PDSCH, the following agreements were achieved in previous meeting that the unicast PDSCH and group-common PDSCH can only be dynamically scheduled [1]. One remaining issue is for SPS PDSCH retransmission, whether it is included in the dynamically scheduled PDSCH for FDMed scheduling. Agreement It is clarified that for the FDMed unicast PDSCH and group-common PDSCH capability of RRC_CONNECTED UE, the unicast PDSCH and group-common PDSCH can only be dynamically scheduled. Further down-selection between the following: - Opt 1: the unicast/multicast SPS retransmission PDSCH is included in the dynamically scheduled PDSCH for FDMed scheduling - FFS: whether new UE capability is needed - Opt 2: the unicast/multicast SPS retransmission PDSCH is NOT included in the dynamically scheduled PDSCH for lFDMed scheduling. Regarding SPS PDSCH retransmission, it is usually deemed as dynamically scheduled PDSCH in current specification for unicast scheduling. For example, when determining number of PDSCHs supported in a slot for TDM case, or determining HARQ-ACK timing/PUCCH resource, or PDSCH aggregation factor, etc., same mechanism is used for SPS PDSCH retransmission and dynamically scheduled PDSCH. SPS PDSCH retransmission is scheduled by a certain DCI, it is natural to include it in the dynamically scheduled PDSCH for FDMed scheduling, i.e. Opt 1 should be adopted. Proposal 2: Update 33-3-2 to include the UE capability for unicast/multicast SPS retransmission PDSCH for FDMed unicast PDSCH and group-common PDSCH for multicast. 33. NR_MBS; 33-3-2; FDM-ed unicast PDSCH and one group-common PDSCH for multicast; 1. Support FDM between one dynamically scheduled unicast PDSCH and one dynamically scheduled group-common PDSCH for multicast in RRC CONNECTED mode in a slot.; 33-2, or at least one of {33-5-1a, 33-5-1f}; Yes; Per FSPC; N/A; N/A; Note: this FG does not support FDMed SPS. SPS PDSCH retransmission is included in dynamically scheduled PDSCH; Optional with capability signalling. |
| --- | --- | --- |

The moderator would like to hear companies’ views on above proposal.

Proposal 2-16:

  - TBD

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | Ok |

FG33-4

In [1], FG 33-4 is captured as below.

| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast with ACK/NACK transforming | 1. Support NACK-only based HARQ-ACK feedback for dynamic scheduling for multicast, including: a) A single TB with NACK-only feedback transmitted in PUCCH b) multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits 2. Support of shared PUCCH resource configurations with unicast 3. One or multiple TB with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits 4. One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits when multiplexing with other UCI | 33-2a | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Following proposal is provided in a contribution for the RAN1#112 meeting.

| [5] | vivo | For NACK-only based HARQ-ACK feedback for dynamically scheduled group-common PDSCH for multicast, RRC-based enabling/disabling NACK-only based feedback is supported according to RAN1’s agreements, which should be clarified in 33-4. Proposal 3: Update 33-4 to include the UE capability for NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling with ACK/NACK transforming. 33. NR_MBS; 33-4; NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling with ACK/NACK transforming; 1. Support NACK-only based HARQ-ACK feedback and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for dynamic scheduling for multicast, including: a) A single TB with NACK-only feedback transmitted in PUCCH b) multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits 2. Support of shared PUCCH resource configurations with unicast 3. One or multiple TB with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits 4. One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits when multiplexing with other UCI; 33-2a; Yes; Per BC; N/A; N/A; Optional with capability signalling. |
| --- | --- | --- |

The moderator would like to hear companies’ views on above proposal.

Proposal 2-17:

  - TBD

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | Ok |

FG33-5-1f

In [1], FG 33-5-1f is captured as below.

| 33. NR_MBS | 33-5-1f | NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast | 1) Support NACK-only based HARQ-ACK feedback, and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling 2) Support of PTM retransmission associated with G-CS-RNTI for SPS multicast | 33-5-1 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Following proposal is provided in a contribution for the RAN1#112 meeting.

| [10] | Huawei, HiSilicon | 33. NR_MBS; 33-5-1f; NACK-only based HARQ-ACK feedback for multicast with ACK/NACK transforming and RRC-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast; 1) Support NACK-only based HARQ-ACK feedback, including: a) A single TB with NACK-only feedback transmitted in PUCCH b) multiple TBs with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits c) One or multiple TBs with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits. d). One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits when multiplexing with other UCI 2) Support of shared SPS-PUCCH-AN-List configuration from unicast SPS. 3) support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling 4) Support of PTM retransmission associated with G-CS-RNTI for SPS multicast; 33-5-1a; Yes; Per BC; N/A; N/A; Optional with capability signalling. |
| --- | --- | --- |

The moderator would like to hear companies’ views on above proposal regarding prerequisite FG (other parts are to be discussed as proposals 2-2 and 2-10).

Proposal 2-18:

  - TBD

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | Ok |

FG33-5-1h/33-10

In [1], FG 33-5-1h and 33-10 are captured as below.

| 33. NR_MBS | 33-5-1h | Multiple G-CS-RNTIs for SPS group-common PDSCHs | Max number of G-CS-RNTIs for SPS multicast | 33-5-1 | Yes |  |  | Per UE | [Yes] | Yes |  | Reporting type of FGs 33-5-1h is per UE with [FDD/TDD,] FR1/FR2, licensed/unlicensed, and TN/NTN differentiation, detail signalling is up to RAN2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-10 | Support group-common PDSCH RE-level rate matching for multicast | 1) Support of SP ZP-CSI-RS for group-common PDSCH RE-mapping patterns 2) Support of P ZP-CSI-RS for group-common PDSCH RE-mapping patterns 3) Support p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config-Multicast same as or different from the p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config Note 1: The total number of semi-persistent ZP-CSI-RS-ResourceSet that a UE can be configured with is the same as for unicast in Rel-16 4) Support of AP ZP-CSI-RS for group-common PDSCH RE-mapping patterns | 2-33a, 33-2 | Yes |  |  | Per UE | [Yes] | Yes |  | Reporting type of FG 33-10 is per UE with [FDD/TDD,] FR1/FR2, licensed/unlicensed, and TN/NTN differentiation, detail signalling is up to RAN2 | Optional with capability signalling |

Following proposal is provided in a contribution for the RAN1#112 meeting.

| [7] | NTT DOCOMO, INC. | It was agreed that details on reporting type for FGs 33-5-1h and 33-10 are up to RAN2 discussion, and in the latest 38.306, we can see the following corresponding capabilities. Just to clean up the UE feature list to avoid any potential confusion, the following proposals are submitted. Note that although it is basically not necessary to update RAN1 UE features list according to RAN2 specifications, the update here would be better in order to avoid unnecessary confusion due to remaining brackets and texts in the note. maxNumberG-CS-RNTI-r17 Defines maximum number of G-CS-RNTIs for SPS multicast. For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands, associated with supported shared and non-shared spectrum respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands. A UE supporting this feature shall also indicate support of sps-Multicast-r17.; Band; No; N/A; N/A. re-LevelRateMatchingForMulticast-r17 Indicates whether the UE supports group-common PDSCH RE-level rate matching for multicast, comprised of the following functional components: - Supports SP ZP-CSI-RS for group-common PDSCH RE-mapping patterns; - Supports P ZP-CSI-RS for group-common PDSCH RE-mapping patterns; - Supports p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config-Multicast same as or different from the p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config; - Supports AP ZP-CSI-RS for group-common PDSCH RE-mapping patterns. For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands, associated with supported shared and non-shared spectrum respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands. A UE supporting this feature shall also indicate support of dynamicMulticastPCell-r17. A UE supporting this feature in FR1 bands shall also indicate support of pdsch-RE-MappingFR1-PerSymbol or pdsch-RE-MappingFR1-PerSlot. A UE supporting this feature in FR2 bands shall also indicate support of pdsch-RE-MappingFR2-PerSymbol or pdsch-RE-MappingFR2-PerSlot. NOTE: The total number of semi-persistent ZP-CSI-RS-ResourceSet that a UE can be configured with is the same as for unicast in Rel-16.; Band; No; N/A; N/A. Proposal 11: FGs 33-5-1h and 33-10 are updated as described in the latest 38.306. - The reporting type is per band. - The text in Note column is updated as the following. - For TN, the UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands and all TDD-FR2 bands, associated with supported shared and non-shared spectrum respectively. For NTN, UE shall set the capability value consistently for all FDD-FR1 NTN bands. |
| --- | --- | --- |

The moderator would like to hear companies’ views on above proposal.

Proposal 2-19:

  - TBD

| Company | Comment |
| --- | --- |
| Huawei, HiSilicon | Ok |

FG33-6-x

In [1], FG 33-6-x are captured as below.

| 33. NR_MBS | 33-6-1 | DL priority indication for multicast in DCI | 1. Support of priority indicator field configured in DCI formats 4_2 with CRC scrambled with G-RNTI for multicast. 2. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast and multicast at a UE | 33-2a, 33-2f | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-6-1a | DL priority configuration for SPS multicast | Support of priority indicator field configured in DCI format 4_2 for multicast HARQ-ACK feedback of SPS multicast | 33-6-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-2 | Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for unicast and multicast at a UE | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for unicast and multicast at a UE. | 33-6-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-3 | More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot | 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot. | 33-6-1, 33-6-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |

Following proposal is provided in a contribution for the RAN1#112 meeting.

| [6] | Qualcomm Incorporated | Given the potential UE testing differentiation among FDD/TDD, licensed, unlicensed, and TN/NTN band, we suggest the following changes of FG33-6-x as 33. NR_MBS; 33-6-1; DL priority indication for multicast in DCI; 1. Support of priority indicator field configured in DCI formats 4_2 with CRC scrambled with G-RNTI for multicast. 2. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast and multicast at a UE; 33-2a, 33-2f; Yes; Per band; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-6-1a; DL priority configuration for SPS multicast; Support of priority indicator field configured in DCI format 4_2 for multicast HARQ-ACK feedback of SPS multicast; 33-5-1a, 33-5-1i; Yes; Per band; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-6-2; Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for unicast and multicast at a UE; 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for unicast and multicast at a UE.; 33-6-1; Yes; Per band; N/A; N/A; Optional with capability signalling. 33. NR_MBS; 33-6-3; More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot; 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot.; 33-6-1, 33-6-2; Yes; Per band; N/A; N/A; Optional with capability signalling. |
| --- | --- | --- |

The moderator would like to hear companies’ views on above proposal.

Proposal 2-20:

  - TBD

| Company | Comment |
| --- | --- |

- Conclusions

TBD

References

[1]	R1-2212895	Updated RAN1 UE features list for Rel-17 NR after RAN1 #111	Moderators (AT&T, NTT DOCOMO, INC.)

[2]	R1-2300140	Remaining issues for UE features topics 1	Nokia, Nokia Shanghai Bell

[3]	R1-2300202	UE features for R17 NR MBS	Spreadtrum Communications

[4]	R1-2300337	Discussion on UE features for topics 1	ZTE

[5]	R1-2300432	Remaining issues on Rel-17 MBS UE features	vivo

[6]	R1-2301392	Discussion on Rel-17 UE features topic 1	Qualcomm Incorporated

[7]	R1-2301476	Discussion on remaining issues regarding Rel-17 RAN1 UE features topics 1	NTT DOCOMO, INC.

[8]	R1-2301615	Views on UE feature Topic 1	MediaTek Inc.

[9]	R1-2301677	UE capabilities for NR MBS	Ericsson

[10]	R1-2301703	Remaining issues for UE features topics 1	Huawei, HiSilicon

Appendix: Latest RAN1 UE features list for Rel-17 NR MBS in R1-2212895

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1 | Broadcast | 1. Support of group-common PDCCH/PDSCH for broadcast with CRC scrambled by MCCH-RNTI. 2. Support of group-common PDCCH/PDSCH for broadcast with CRC scrambled by G-RNTI(s) for MTCH. 3. Support of CFR configuration for broadcast. 4. Support of CORESET and common search space for broadcast. 5. Support of DCI format 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. 6. Support of inter-slot TDM between unicast PDSCH and MCCH group-common PDSCH or MTCH group-common PDSCH, or between MCCH group-common PDSCH and MTCH group-common PDSCH, or among unicast PDSCH and MCCH group-common PDSCH and MTCH group-common PDSCH in different slots. 7. Support MCCH change notification indication via DCI. 8. support of higher layer configured slot-level repetition up to 8 for MTCH 9. One G-RNTI per UE is supported for broadcast reception 10. Support of FDMed MCCH and PBCH 11. Support of up to 64QAM for FR1/FR2 |  | Up to RAN2 |  |  | Up to RAN2 | Up to RAN2 | Up to RAN2 |  | It is up to RAN2 whether/how to introduce the capability for support of N > 1 G-RNTIs for broadcast for a UE | Optional without capability signalling |
| 33. NR_MBS | 33-1-1 | DCI indicated slot-level repetition up to 16 for broadcast MTCH | Support up to 16 times dynamic slot-level repetition for broadcast MTCH. | 33-1 | Up to RAN2 |  |  | Per FSPC | N/A | N/A |  |  | Up to RAN2 |
| 33. NR_MBS | 33-1-2 | FDM-ed unicast PDSCH and group-common PDSCH for broadcast | 1. Support FDM between one unicast PDSCH and one group-common PDSCH for broadcast in RRC CONNECTED mode in a slot. | 33-1 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast for PCell | 1. Support of group-common PDCCH/PDSCH for multicast with CRC scrambled by G-RNTI for PCell. 2. Support of CFR configuration for multicast. 3. Support of CORESET and common search space configuration for multicast. 4. Support of DCI format 4_1 with CRC scrambled with G-RNTI for multicast. 5. Support of inter-slot TDM between group-common PDSCH for multicast and other PDSCHs in different slots. 6. Support {2, 4, 8} times semi-static slot-level repetition for group-common PDSCH for multicast |  | Yes |  |  | Per FS | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2a | Support of ACK/NACK based HARQ-ACK feedback andRRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | 1) Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling 2) Support of PTM retransmission for multicast 3) support of Type-1 and Type-2 HARQ-ACK CB for multicast feedback only 4) Support of shared PUCCH resource configurations with unicast 5) Support of Type-2 HARQ-ACK codebook for multicast on PUSCH/PUCCH with max number X of G-RNTIs | 33-2 | Yes |  |  | Per BC | N/A | N/A |  | Candidate values of X is {1, 2, 3, 4} with X no larger than max number of G-RNTIs of FG33-2e Note: the value of X should be common across FG33-2a, 33-3-3a and 33-3-3b if reported | Optional with capability signalling |
| 33. NR_MBS | 33-2b | DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling via DCI format 4_2 | 33-2a, 33-2f | Yes |  |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2d | PTP retransmission for multicast dynamic scheduling | Support of PTP retransmission for multicast on the same cell as multicast initial transmission | 33-2a | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2e | Multiple G-RNTIs for group-common PDSCHs | Capability on number of G-RNTI for multicast | 33-2 | Yes |  |  | Per UE | [Yes] | Yes |  | Reporting type of FG 33-2e is per UE with [FDD/TDD,] FR1/FR2, licensed/unlicensed, and TN/NTN differentiation, detail signalling is up to RAN2 | Optional with capability signalling |
| 33. NR_MBS | 33-2f | Dynamic multicast with DCI format 4_2 | Support of DCI format 4_2 with CRC scrambled with G-RNTI for multicast | 33-2 | Yes |  |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2g | MIMO layers for multicast PDSCH | Supported maximal number of MIMO layers for multicast PDSCH | 33-2 | Yes |  | UE supports 1 MIMO layer only for multicast PDSCH | Per FSPC | N/A | N/A |  | Candidate values: {2,4,8} Note: If UE supports up to 8 layers, the UE supports TB2 | Optional with capability signalling |
| 33. NR_MBS | 33-2h | Dynamic scheduling for multicast for SCell | Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI for SCell. | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2i | Supported maximal modulation order for multicast PDSCH | 1. For FR1, up to 1024QAM is supported, candidate values {256QAM, 1024QAM} 2. For FR2, up to 256QAM is supported, candidate values {64QAM, 256QAM} | 33-2 | Yes |  | The UE supports the same modulation order as unicast | Per band | N/A | N/A |  | Note: A UE shall support the corresponding mandatory maximum modulation for unicast. | Optional with capability signalling |
| 33. NR_MBS | 33-2j | Supported maximum modulation order used for maximum data rate calculation for multicast PDSCH | 1. For FR1, up to 1024QAM is supported as maximum modulation order used for maximum data rate calculation for multicast PDSCH, candidate values {256QAM, 1024QAM} 2. For FR2, up to 256QAM is supported as maximum modulation order used for maximum data rate calculation for multicast PDSCH, candidate values {64QAM, 256QAM} | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-1 | Dynamic Slot-level repetition for group-common PDSCH | 1. Support up to X times dynamic slot-level repetition for group-common PDSCH for multicast. | 33-2 | Yes |  |  | Per UE | Yes | Yes |  | Candidate values for X is: {8, 16} This FG is reported for TN and licensed. | Optional with capability signalling |
| 33. NR_MBS | 33-3-1a | Dynamic Slot-level repetition for group-common PDSCH for NTN and unlicensed | 1. Support up to X times dynamic slot-level repetition for group-common PDSCH for multicast for NTN and unlicensed | 33-2 | Yes |  |  | Per band | N/A | N/A |  | Candidate values for X is: {8, 16} This FG is reported for NTN and unlicensed | Optional with capability signalling |
| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and one group-common PDSCH for multicast | 1. Support FDM between one unicast PDSCH and one group-common PDSCH for multicast in RRC CONNECTED mode in a slot. | 33-2 | Yes |  |  | Per FSPC | N/A | N/A |  | Note: this FG does not support FDMed SPS | Optional with capability signalling |
| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | 1. Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. 2. Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC 3. Support TDM among N (N>1) group-common PDSCHs in a slot per CC 4. Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC 5. The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). - Note: The max number of (M+1), N, (K+L) are determined based on the numbers reported by FG5-11 and/or FG5-11a and/or FG5-11b. 6. up to one broadcast PDSCH is supported in a slot. 7. For any two consecutive slots n and n+1, if there are more than 1 broadcast/multicast/unicast PDSCH in either slot, whether to require the minimum time separation between starting time of any two broadcast/multicast/unicast PDSCHs within the duration of these slots is 4 OFDM symbol for 30kHz and 7 OFDM symbol for 60kHz | 33-1 and/or 33-2, 5-11 and/or 5-11a and/or 5-11b | Yes |  |  | Per FSPC | N/A | N/A |  | Candidate value for component 7: require the minimum time separation time {yes, no} | Optional with capability signalling |
| 33. NR_MBS | 33-3-3a | FDM-ed Type-1 and Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast | 1. Support of FDM-ed Type-1 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and ACK/NACK-based HARQ-ACK for multicast on PUCCH or PUSCH 2. Support of Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast on PUCCH or PUSCH with max number X of G-RNTIs | 33-3-2, at least one of {33-2a, 33-4, 33-5-1a, 33-5-1f} | Yes |  | FFS | Per BC | N/A | N/A |  | Note1: FDM-ed Type-1 HARQ-ACK codebook is generated by concatenating the Type-1 sub-codebook for unicast and the Type-1 sub-codebook for multicast. Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast. Candidate values of X is {1, 2, 3, 4} with X no larger than max number of G-RNTIs of FG33-2e Note: the value of X should be common across FG33-2a, 33-3-3a and 33-3-3b if reported | Optional with capability signalling |
| 33. NR_MBS | 33-3-3b | Mode 2 TDM-ed Type-1 and Type-2 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast | 1. Support of Mode 2 TDM-ed Type-1 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and ACK/NACK-based HARQ-ACK for multicast on PUCCH or PUSCH 2. Support of Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast on PUCCH or PUSCH with max number X of G-RNTIs | 33-2a or 33-4 or 33-5-1a or 33-5-1f | Yes |  | FFS | Per BC | N/A | N/A |  | Note1: Mode 2 TDM-ed Type-1 HARQ-ACK codebook is generated based on the union TDRA tables from unicast and multicast and the union of k1 sets from unicast and multicast. Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast. Candidate values of X is {1, 2, 3, 4} with X no larger than max number of G-RNTIs of FG33-2e Note: the value of X should be common across FG33-2a, 33-3-3a and 33-3-3b if reported | Optional with capability signalling |
| 33. NR_MBS | 33-3-4 | Mode 1 for type1 codebook generation | Supports type1-Codebook-Generation-Mode configured as mode 1 | 33-3-3b | Yes |  |  | Per BC | N/A | N/A |  | This FG is for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast on PUCCH or PUSCH | Optional with capability signalling |
| 33. NR_MBS | 33-3-5 | Feedback multiplexing for unicast PDSCH and group-common PDSCH for multicast with same priority and different codebook type | Support of multiplexing HARQ-ACK for unicast and for multicast with the same priority and different HARQ-ACK codebook types in a PUCCH or in a PUSCH | 33-2a or 33-4 or 33-5-1a or 33-5-1f | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast with ACK/NACK transforming | 1. Support NACK-only based HARQ-ACK feedback for dynamic scheduling for multicast, including: a) A single TB with NACK-only feedback transmitted in PUCCH b) multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits 2. Support of shared PUCCH resource configurations with unicast 3. One or multiple TB with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits 4. One or multiple TB with NACK-only feedback transmitted in PUCCH by transforming into ACK/NACK bits when multiplexing with other UCI | 33-2a | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-4a | NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission | 1. Support NACK-only based HARQ-ACK feedback for dynamic scheduling for multicast, including: a) Up to 4 TBs with NACK-only feedback transmitted in PUCCH by select one PUCCH resource. 2. Support of separate PUCCH resource configurations from unicast 3. Single TB with NACK-only feedback transmitted in PUCCH 4. up to 4TBs with NACK-only feedback transmitted in PUSCH by transforming into ACK/NACK bits | 33-4 | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-4-1 | DCI-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-RNTI by RRC signalling via DCI format 4_2 | 33-4 and 33-2f | Yes |  |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast on PCell | 1. Support one SPS group-common PDSCH configuration for multicast 2. Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH | 33-2 | Yes |  |  | Per FS | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1a | Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | 1. Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, [SPS group-common PDSCH activation, and SPS release PDCCH] 2. Support of PTM retransmission for SPS multicast associated with G-CS-RNTI 3. Support of Type-1 and Type-2 HARQ-ACK CB for SPS multicast feedback only 4. Support of shared SPS-PUCCH-AN-List configuration from unicast SPS | 33-5-1 | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1b | DCI-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC signaling via DCI format 4_2 | 33-5-1a, 33-5-1i | Yes |  |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1d | PTP retransmission for SPS group-common PDSCH for multicast | Support of PTP retransmission associated with CS-RNTI for SPS multicast on the cell same as multicast initial transmission | 33-5-1a | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1e | Dynamic Slot-level repetition for SPS group-common PDSCH for multicast | Support up to X times dynamic slot-level repetition for SPS group-common PDSCH for multicast. | 33-5-1 | Yes |  |  | [Per UE] | [No] | [No] |  | Candidate values for X is: {8, 16} | Optional with capability signalling |
| 33. NR_MBS | 33-5-1f | NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast | 1) Support NACK-only based HARQ-ACK feedback, and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling 2) Support of PTM retransmission associated with G-CS-RNTI for SPS multicast | 33-5-1 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1g | DCI-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast | Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC signaling via DCI format 4_2 | 33-5-1f, 33-5-1i | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1h | Multiple G-CS-RNTIs for SPS group-common PDSCHs | Max number of G-CS-RNTIs for SPS multicast | 33-5-1 | Yes |  |  | Per UE | [Yes] | Yes |  | Reporting type of FGs 33-5-1h is per UE with [FDD/TDD,] FR1/FR2, licensed/unlicensed, and TN/NTN differentiation, detail signalling is up to RAN2 | Optional with capability signalling |
| 33. NR_MBS | 33-5-1i | Multicast SPS scheduling with DCI format 4_2 | 1.Support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling 2. Retransmission scheduled by DCI format 4_2 with CRC scrambled with G-CS-RNTI | 33-5-1 | Yes |  |  | FFS | FFS | FFS |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1j | NACK-only based HARQ-ACK feedback for multicast corresponding to a specific sequence or a PUCCH transmission for SPS group-commmon PDSCH for multicast | 1. Support NACK-only based HARQ-ACK feedback for SPS PDSCH for multicast, including: a) Multiple TBs with NACK-only feedback transmitted in PUCCH by select one PUCCH resource. 2. Support of separate SPS-PUCCH-AN-List from unicast | 33-5-1f | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration on PCell | 1. Support up to 8 SPS group-common PDSCH configuration per CFR for multicast 2. Support M>=1 activated SPS group-common PDSCH configurations per CFR for multicast 3. The total number of SPS configurations for both multicast and unicast is no larger than 8 in a BWP of a serving cell, and activated SPS group-common PDSCH configurations is no larger than M. 4. The total number of SPS configurations for both multicast and unicast in a cell group is no larger than 32 | 33-5-1 | Yes |  |  | [Per UE] | [No] | [No] |  | Candidate value set for M is {1, 2, …, 8} | Optional with capability signalling |
| 33. NR_MBS | 33-5-3 | One SPS group-common PDSCH configuration for multicast for Scell | 1. Support one SPS group-common PDSCH configuration for multicast for Scell. 2. Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH for Scell. | 33-5-1, 33-2h | Yes |  |  | Per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-4 | Up to 8 SPS group-common PDSCH configurations per CFR for multicast for SCell | 1. Support up to 8 SPS group-common PDSCH configuration per CFR for multicast for Scell. 2. Support M>=1 activated SPS group-common PDSCH configurations per CFR for multicast for Scell. 3. The total number of SPS configurations for both multicast and unicast is no larger than 8 in a BWP of a serving cell, and activated SPS group-common PDSCH configurations is no larger than M. 4. The total number of SPS configurations for both multicast and unicast in a cell group is no larger than 32. | 33-5-3 | Yes |  |  | Per FSPC | N/A | N/A |  | Candidate value set for M is {1, 2, …, 8} | Optional with capability signalling |
| 33. NR_MBS | 33-6-1 | DL priority indication for multicast in DCI | 1. Support of priority indicator field configured in DCI formats 4_2 with CRC scrambled with G-RNTI for multicast. 2. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast and multicast at a UE | 33-2a, 33-2f | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-1a | DL priority configuration for SPS multicast | Support of priority indicator field configured in DCI format 4_2 for multicast HARQ-ACK feedback of SPS multicast | 33-6-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-2 | Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for unicast and multicast at a UE | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for unicast and multicast at a UE. | 33-6-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-3 | More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot | 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot. | 33-6-1, 33-6-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-8-1 | PUCCH resource configuration for multicast feedback for dynamically scheduled multicast | Support of a PUCCH-Config for multicast HARQ-ACK feedback, separate from that of unicast configurations | 33-2a | Yes |  |  | [Per band or per FSPC] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-8-2 | Up to 2 PUCCH resources configuration for multicast feedback for dynamically scheduled multicast | Support of a PUCCH-ConfigurationList for multicast HARQ-ACK feedback, separate from that of unicast configurations | 33-8-1, 33-6-1 | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-8-3 | PUCCH resource configuration for multicast feedback for SPS GC-PDSCH | Support of a SPS-PUCCH-AN-List for multicast HARQ-ACK feedback of all multicast SPS configuration(s), separate from that of SPS unicast configurations | 33-5-1a | Yes |  |  | Per BC | N/A | N/A |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-9 | Supporting unicast PDCCH to release SPS group-common PDSCH | Supports unicast PDCCH scrambled with CS-RNTI to release SPS group-common PDSCH | 33-5-1, 12-2 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-10 | Support group-common PDSCH RE-level rate matching for multicast | 1) Support of SP ZP-CSI-RS for group-common PDSCH RE-mapping patterns 2) Support of P ZP-CSI-RS for group-common PDSCH RE-mapping patterns 3) Support p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config-Multicast same as or different from the p-ZP-CSI-RS-ResourceSet configured in PDSCH-Config Note 1: The total number of semi-persistent ZP-CSI-RS-ResourceSet that a UE can be configured with is the same as for unicast in Rel-16 4) Support of AP ZP-CSI-RS for group-common PDSCH RE-mapping patterns | 2-33a, 33-2 | Yes |  |  | Per UE | [Yes] | Yes |  | Reporting type of FG 33-10 is per UE with [FDD/TDD,] FR1/FR2, licensed/unlicensed, and TN/NTN differentiation, detail signalling is up to RAN2 | Optional with capability signalling |