3GPP TSG RAN WG1 #112                                              	       R1-2301905

Athens, Greece, February 27th – March 4th, 2023

Source: 	Moderator (Nokia)

Title: 	Summary #1 of Correcting on TEG reporting

Agenda item:	8.5

Document for: 	Discussion and Decision

# Introduction

In this contribution, we provide a summary of the maintenance issues proposed in [1] on TEG reporting. We also make some initial proposals to facilitate RAN1 discussion.

Issues for discussion

## Issue: TEG Reporting

The following is observed in [1] as the reason for the change and summary of the proposed change:

| Reason for change: | In 38.214 everywhere that talks about TEG IDs refers to either TRP TEG ID or UE TEG ID exepct in two places. These two places involve Tx TEGs which may cause ambiguity if UE or TRP Tx TEGs are what is meant. In particular for multi-RTT both UE and TRP Tx TEGs can be involved and therefore should be clarified. |
| --- | --- |
| Summary of change: | In Clause 5.1.6.5, clarify that TRP Tx TEGs are what is provided to the UE. In Clause 6.2.1.4, clarify that UE Tx TEGs are what the UE can report as part of UE RxTx TEG reporting. |
| Consequences if not approved: | Unclear UE behvaior and assumptions when Tx TEG IDs are reported or provided to the UE. Misalignment with RRC and LPP specs. |

The draft CR to TS 38.214 from [1] is also copied here:

<omitted text>

#### 5.1.6.5	PRS reception procedure

<omitted text>

The UE may be provided with association information of DL PRS resource(s) with TRP Tx TEGs via higher layer parameter dl-prs-trp-Tx-TEG-ID for a dl-PRS-ID.

<omitted text>

#### 6.2.1.4	UE sounding procedure for positioning purposes

<omitted text>

If the UE reports a UE RxTx TEG ID with a UE Rx-Tx time difference measurement, the UE may report a UE Tx TEG ID.

<omitted text>

### Round #1 Discussion

FL Comments

In TS 38.214 there are careful references to UE Tx TEG ID or TRP Tx TEG in all places expect the two suggested in the draft CR. At least in the case of multi-RTT it could be ambiguious which type of Tx TEG is being referred to by the specification. It seems reasonable to clarify and to align the specification with the rest of the positioning clauses.

Proposal 1: Endorse the changes in draft CR R1-2301210.

Companies views:

| Company Name | Support - Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | - Yes |  |
| Huawei, HiSilicon |  | In general, the change is technically correct. However, we do not see the unclarity as stated in the box of “consequence if not approved”. We can be OK with the change if there is majority support. |
| CATT |  | This correction is not essential, since the details about the TRP Tx TEGs or UE Tx TEG ID had been clearly described in LPP specs TS 37.355. If the majority supports it, we prefer it should be editorial correction and considered for editor alignment CR. |

Conclusion

In this contribution, we provided a review of the submitted maintenance issue on TEG reporting and prepared an initial set of proposals to facilitate further discussion/decision by RAN1 during the RAN1#112.

Outcome (if any):

To be updated

References

- R1-2301210, Correction on TEG Reporting, Nokia, Nokia Shanghai Bell.