3GPP TSG RAN WG1 #112                                                  R1-2301971

Athens, Greece, February 27th – March 3rd, 2023

Source:          Moderator (ZTE)

Title:            Summary #1 of PRS configuration and editorial corrections

Agenda item:     8.5

Document for:	Discussion and Decision

- Introduction

This document provides the moderator summary of the following contribution.

- R1-2300801 Alignment CR for NR positioning in 38.214 ZTE

- Discussion

| Company | Proposals |
| --- | --- |
| ZTE[1,R1-2300801] | Reason for change:; - In current 37.355 specification, NR-DL-PRS-PositioningFrequency Layer is configured as follows: NR-DL-PRS-PositioningFrequencyLayer-r16 ::= SEQUENCE { dl-PRS-SubcarrierSpacing-r16 ENUMERATED {kHz15, kHz30, kHz60, kHz120, ...}, dl-PRS-ResourceBandwidth-r16 INTEGER (1..63), dl-PRS-StartPRB-r16 INTEGER (0..2176), dl-PRS-PointA-r16 ARFCN-ValueNR-r15, dl-PRS-CombSizeN-r16 ENUMERATED {n2, n4, n6, n12, ...}, dl-PRS-CyclicPrefix-r16 ENUMERATED {normal, extended, ...}, ... } This configuration means that resource sets in the same DL PRS positioning frequency layer have the same value of: dl-PRS-SubcarrierSpacing dl-PRS-CyclicPrefix dl-PRS-PointA dl-PRS-CombSizeN dl-PRS-ResourceBandwidth dl-PRS-StartPRB. But 38.214 specification only introduces 3 of them in NR-DL-PRS-PositionFrequencyLayer configuration description, i.e., dl-PRS-SubcarrierSpacing, dl-PRS-CyclicPrefix, dl-PRS-PointA. The remaining 3 IEs are included in NR-DL-PRS-ResourceSet configuration description part. But NR-DL-PRS-ResourceSet configuration are designed for the parameters that all DL PRS resources within one resource set have the same value. So we suggest move the configuration description of these 3 IEs to NR-DL-PRS-PositioningFrequencyLayer configuration description. - There are several misspelling problems and typos in the current 38.214.. Summary of change:; - Corrected the configuration description of NR-DL-PRS-Positioning FrequencyLayer and NR-DL-PRS-ResourceSet. Move the configuration description of dl-PRS-CombSizeN, dl-PRS- ResourceBandwidth dl-PRS-StartPRB from NR-DL-PRS-ResourceSet part to NR-DL-PRS-Positioning FrequencyLayer part. - Corrected several typos in current specification 38.214.. Consequences if not approved:; It’s unclear for UE to distinguish the exact configuration for NR-DL-PRS-PositioningFrequencyLayer and NR-DL-PRS-ResourceSet.. - 5.1.6.5 PRS reception procedure The UE can be configured with one or more DL PRS resource set configuration(s) as indicated by the higher layer parameters NR-DL-PRS-ResourceSet and NR-DL-PRS-Resource as defined by Clause 6.4.3 [17, TS 37.355]. Each DL PRS resource set consists of K≥1 DL PRS resource(s) where each has an associated spatial transmission filter. The UE can be configured with one or more DL PRS positioning frequency layer configuration(s) as indicated by the higher layer parameter NR-DL-PRS-PositioningFrequencyLayer. A DL PRS positioning frequency layer is defined as a collection of DL PRS resource sets which have common parameters configured by NR-DL-PRS-PositioningFrequencyLayer. The UE assumes that the following parameters for each DL PRS resource(s) are configured via higher layer parameters NR-DL-PRS-PositioningFrequencyLayer, NR-DL-PRS-ResourceSet and NR-DL-PRS-Resource. A DL PRS positioning frequency layer is configured by NR-DL-PRS-PositioningFrequencyLayer, consists of one or more DL PRS resource sets and it is defined by: - dl-PRS-SubcarrierSpacing defines the subcarrier spacing for the DL PRS resource. All DL PRS resources and DL PRS resource sets in the same DL PRS positioning frequency layer have the same value of dl-PRS-SubcarrierSpacing. The supported values of dl-PRS-SubcarrierSpacing are given in Table 4.2-1 of [4, TS38.211], excluding the values of 240kHz, 480 kHz, and 960 kHz. - dl-PRS-CyclicPrefix defines the cyclic prefix for the DL PRS resource. All DL PRS Resources and DL PRS Resource sets in the same DL PRS positioning frequency layer have the same value of dl-PRS-CyclicPrefix. The supported values of dl-PRS-CyclicPrefix are given in Table 4.2-1 of [4, TS38.211]. - dl-PRS-PointA defines the absolute frequency of the reference resource block. Its lowest subcarrier is also known as Point A. All DL PRS resources belonging to the same DL PRS resource set have common Point A and all DL PRS resources sets belonging to the same DL PRS positioning frequency layer have a common Point A. - dl-PRS-CombSizeN defines the comb size of a DL PRS resource where the allowable values are given in Clause 7.4.1.7.3 of [TS38.211]. All DL PRS resource sets belonging to the same DL PRS positioning frequency layer have the same value of dl-PRS-CombSizeN. - dl-PRS-ResourceBandwidth defines the number of resource blocks configured for DL PRS transmission. The parameter has a granularity of 4 PRBs with a minimum of 24 PRBs and a maximum of 272 PRBs. All DL PRS resources sets within a DL PRS positioning frequency layer have the same value of dl-PRS-ResourceBandwidth. - dl-PRS-StartPRB defines the starting PRB index of the DL PRS resource with respect to reference Point A, where reference Point A is given by the higher-layer parameter dl-PRS-PointA. The starting PRB index has a granularity of one PRB with a minimum value of 0 and a maximum value of 2176 PRBs. All DL PRS resource sets belonging to the same DL PRS positioning frequency layer have the same value of dl-PRS-StartPRB. The UE expects that it will be configured with dl-PRS-ID each of which is defined such that it is associated with multiple DL PRS resource sets. The UE expects that one of these dl-PRS-ID along with a nr-DL-PRS-ResourceSetID and a nr-DL-PRS-ResourceID-r16 can be used to uniquely identify a DL PRS resource. The UE may be configured by the network with nr-PhysCellID, nr-CellGlobalID, and nr-ARFCN [17, TS 37.355] associated with a dl-PRS-ID. - If nr-PhysCellID or nr-CellGlobalID is provided, and if nr-PhysCellID, nr-CellGlobalID and nr-ARFCN associated with the dl-PRS-ID, if provided, are the same as the corresponding information of a serving cell, the UE may assume that the DL PRS is transmitted from the serving cell; - Otherwise, the UE may assume that the DL PRS is not transmitted from a serving cell. If the UE assumes that the DL PRS is transmitted from a serving cell, and if the serving cell is the same as the serving cell defined by the SS/PBCH block, the UE may assume that the DL PRS and the SS/PBCH block are transmitted from the same serving cell. If the UE assumes that the DL PRS is not transmitted from a serving cell, and if nr-PhysCellID is provided, and is the same as physical cell ID of the SS/PBCH block from a non-serving cell of the same band as the DL PRS, the UE may assume that the DL PRS and the SS/PBCH block are transmitted from the same non-serving cell. A DL PRS resource set is configured by NR-DL-PRS-ResourceSet, consists of one or more DL PRS resources and it is defined by: - nr-DL-PRS-ResourceSetID defines the identity of the DL PRS resource set configuration. - dl-PRS-Periodicity-and-ResourceSetSlotOffset defines the DL PRS resource periodicity and takes values TperPRS∈2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 slots, where μ=0, 1, 2, 3 for dl-PRS-SubcarrierSpacing=15, 30, 60 and 120 kHz respectively and the slot offset for DL PRS resource set with respect to SFN0 slot 0. All the DL PRS resources within one DL PRS resource set are configured with the same DL PRS resource periodicity. The UE does not expect that the product of DL PRS resource periodicity TperPRS, the higher layer parameter dl-prs-MutingBitRepetitionFactor and the size of the bitmap of dl-PRS-MutingOption1 exceeds 2μ×10240, where μ=0, 1, 2, 3 for dl-PRS-SubcarrierSpacing=15, 30, 60 and 120 kHz respectively. - dl-PRS-ResourceRepetitionFactor defines how many times each DL-PRS resource is repeated for a single instance of the DL-PRS resource set and takes values TrepPRS∈1,2,4,6,8,16,32. All the DL PRS resources within one resource set have the same resource repetition factor. - dl-PRS-ResourceTimeGap defines the offset in number of slots between two repeated instances of a DL PRS resource with the same nr-DL-PRS-ResourceID within a single instance of the DL PRS resource set. The UE only expects to be configured with dl-PRS-ResourceTimeGap if dl-PRS-ResourceRepetitionFactor is configured with value greater than 1. The time duration spanned by one instance of a nr-DL-PRS-ResourceSet is not expected to exceed the configured value of DL PRS periodicity. All the DL PRS resources within one resource set have the same value of dl-PRS-ResourceTimeGap. - dl-PRS-MutingOption1 and dl-PRS-MutingOption2 define the time locations where the DL PRS resource is expected to not be transmitted for a DL PRS resource set. If dl-PRS-MutingOption1 is configured, each bit in the bitmap of dl-PRS-MutingOption1 corresponds to a configurable number provided by higher layer parameter dl-prs-MutingBitRepetitionFactor of consecutive instances of a DL PRS resource set where all the DL PRS resources within the set are muted for the instance that is indicated to be muted. The length of the bitmap can be {2, 4, 6, 8, 16, 32} bits. If dl-PRS-MutingOption2 is configured each bit in the bitmap of dl-PRS-MutingOption2 corresponds to a single repetition index for each of the DL PRS resources within each instance of a nr-DL-PRS-ResourceSet and the length of the bitmap is equal to the values of dl-PRS-ResourceRepetitionFactor. Both dl-PRS-MutingOption1 and dl-PRS-MutingOption2 may be configured at the same time in which case the logical AND operation is applied to the bit maps as described in Clause 7.4.1.7.4 of [4, TS 38.211]. - NR-DL-PRS-SFN0-Offset defines the time offset of the SFN0 slot 0 for the DL PRS resource set with respect to SFN0 slot 0 of reference provided by nr-DL-PRS-ReferenceInfo. - dl-PRS-ResourceList determines the DL PRS resources that are contained within one DL PRS resource set. - dl-PRS-NumSymbols defines the number of symbols of the DL PRS resource within a slot where the allowable values are given in Clause 7.4.1.7.3 of [4, TS38.211]. < Unchanged parts are omitted > - 5.2.2.2.5 Enhanced Type II Codebook < Unchanged parts are omitted > The precoding matrices indicated by the PMI are determined from L+Mυ vectors. L vectors, vm1i,m2(i),i=0,1,…,L-1, are identified by the indices q1,q2, n1, n2, indicated by i1,1, i1,2, obtained as in 5.2.2.2.3, where the values of Cx,y are given in Table 5.2.2.2.5-4. Mυ=pυN3R vectors, y0,lf,y1,lf,…,yN3-1,lfT, f=0,1,…,Mυ-1, are identified by Minitial (for N3>19) and n3,l l=1,…, υ where < Unchanged parts are omitted > - 5.2.2.2.6 Enhanced Type II Port Selection Codebook < Unchanged parts are omitted > For layer l, l=1,…,υ, the strongest coefficient i1,8,l, the amplitude coefficient indicators i2,3,l and i2,4,l, the phase coefficient indicator i2,5,l and the bitmap indicator i1,7,l are defined and indicated as in clause 5.2.2.2.5, where the mapping from kl,p(1) to the amplitude coefficient pl,p(1) is given in Table 5.2.2.2.5-2 and the mapping from kl,i,f(2) to the amplitude coefficient pl,i,f(2) is given in Table 5.2.2.2.5-3. < Unchanged parts are omitted > - 6.1.2.3.3 Transport Block repetition for uplink transmissions of TB processing over multiple slots with a configured grant The procedures described in this clause apply to PUSCH transmissions of TB processing over multiple slots with a Type 2 configured grant. The higher layer parameter repK-RV defines the redundancy version pattern to be applied to the repetitions. If the parameter repK-RV is not provided in the configuredGrantConfig, the redundancy version for uplink transmissions with a configured grant shall be set to 0. If the parameter repK-RV is provided in the configuredGrantConfig, the nth transmission occasion among N∙K transmissions occasions, n=0,1, …, N∙K -1, is associated with (mod((n-mod(n, N))/N,4)+1)th value in the configured RV sequence. When K=1, or when K>1 and the configured grant configuration is configured with startingFromRV0 set to 'off', the initial transmission of the transport block may only start at the first transmission occasion of the N∙K transmission occasions. Otherwise, the initial transmission of the transport block may start at - The first transmission occasion of the N∙K transmission occasions if the configured RV sequence is {0,2,3,1}. - Any transmission occasion n associated with RV=0, and for which n mod N =0, if the configured RV sequence is {0,3,0,3} or {0,0,0,0}. < Unchanged parts are omitted > - 6.1.7 UE procedure for determining time domain windows for bundling DM-RS < Unchanged parts are omitted > - The last symbol of a PUSCH transmission before the event, if an event occurs which causes power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots within the nominal TDW, and the PUSCH transmission is in a slot for PUSCH transmission of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots. < Unchanged parts are omitted > For PUCCH transmissions of PUCCH repetition, a nominal TDW consists of one or multiple actual TDWs. The UE determines the actual TDWs as follows: - The start of the first actual TDW is the first symbol of the first PUCCH transmission in a slot determined for PUCCH transmission within the nominal TDW. < Unchanged parts are omitted > - 6.2.2 UE DM-RS transmission procedure < Unchanged parts are omitted > - for MsgA PUSCH for Type-2 random access procedure the UE can be configured with a number of additional DM-RS for PUSCH by higher layer parameter msgA-DMRS-AdditionalPosition, which can be 'pos0', 'pos1', 'pos2', 'pos3' for single-symbol DM-RS or 'pos0', 'pos1' for double-symbol DM-RS. < Unchanged parts are omitted > |

FL comments:

The change is proposed to move the description of several NR-DL-PRS IEs from under DL PRS resource set to under DL PRS positioning frequency layer in TS 38.214, to align with the current 37.355, in which these three IEs are defined as under PFL, and also made some editorial corrections to spelling errors in TS 38.214.

- Proposal 2-1:

The changes in the draft CR R1-2300801 are agreed.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Qualcomm | No | Moving the text around is not needed. The text is not wrong and it is not a good precedence to move around text just because there could have been a better place to place it. Also, such a comment was made during the early days of Rel-16 TRs for 38.214 and it was the editor’s choice to not move it. We prefer to keep it where it currently is. |
| Huawei, HiSilicon | No | The change related to positioning was discussed in RAN1#103-e, and was not agreed. See R1-2009239 and section 2.5 of R1-2009419. The remaining change does not belong to positioning WI. |
| CATT |  | It seems that this issue on the configuration description of dl-PRS had been discussed at previous meeting. Therefore, we prefer not to re-open the discussion on this issue at this meeting. |
| Moderator |  | Moderator suggested conclusion: The change on positioning part in R1-2300801 is not pursued. It is up to editor to consider the remaining editorial change. |

- Conclusion

Based on the above comments, the change part on positioning is not pursued. Since the remaining change is not for positioning, moderator suggests to let editor to consider fixing them.

Moderator suggested conclusion:

The change on positioning part in R1-2300801 is not pursued. It is up to editor to consider the remaining editorial change.