3GPP TSG RAN WG1 #112bis-e			R1-2303322

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	5

Source:	Ericsson

Title:	Discussion on Reply LS on Comparison of SL-RSRP and SD-RSRP measurements

Document for:	Discussion, Decision

# 1	Introduction

RAN WG1 received an LS from RAN WG2 regarding the Comparison of SL-RSRP and SD-RSRP measurements in the context of the work on session continuity enhancements within NR_SL_relay_enh work item. SL resource (re)selection. Specifically, RAN 2 discussed about the triggering of a new measurement report, i.e.,

Event Z2: The measured RSRP of a candidate L2 U2N Relay UE becomes an offset better than measured RSRP of the serving L2 U2N Relay UE

This new event requires either the comparison of SD-RSRP (RSRP measurement on broadcast PC5 link used for discovery messages) measurements from different UEs, or the comparison of SL-RSRP (RSRP measurement on unicast PC5 link) and SD-RSRP measurements.

Before agreeing to the introduction of this measurement report trigger, RAN2 would like to ask feedback from RAN1 and RAN4 on the comparisons of RSRP measurements. The question from RAN WG2 is the following:

- Q1: Can the comparison of SL-RSRP and SD-RSRP measurement be used for the purposes of triggering a measurement report?

# 2	Discussion

According to TS 138 300 V17.3.0, for service continuity of L2 U2N Relay, during the L2 U2N Remote UE switching to direct path, the sidelink measurement quantity can be SL-RSRP of the serving L2 U2N Relay UE, and if SL-RSRP is not available, SD-RSRP is used. However, it is not straightforward to establish one-to-one mapping between SL-RSRP and SD-RSRP given that there exist power control and beamforming, etc., on the UE-to-UE link. Note that the network is not aware of the power control or beamforming parameters.

It is not straightforward to establish one-to-one mapping between SL-RSRP and SD-RSRP given that there exist power control and beamforming, etc., on the UE-to-UE link.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	It is not straightforward to establish one-to-one mapping between SL-RSRP and SD-RSRP given that there exist power control and beamforming, etc., on the UE-to-UE link.