3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2303423

# Online, 17 – 26 April, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | xxxx | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to multicast HARQ-ACK |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2023-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - For multiplexing HARQ-ACK information of different priorities associated with unicast DCI formats and with multicast DCI formats in a same PUCCH, it is unclear whether determining the PUCCH resources for transmission should be based on the PRI indicated in the last unicast DCI or DCI with higher priority. - It is currently unclear whether the semi-static PUCCH cell switching can support HARQ-ACK feedback for multicast. It can be clarified that the semi-static PUCCH cell switching can support the ACK/NACK-based HARQ-ACK feedback for multicast as currently specified in clause 9.A. - It is currently unclear whether deferring HARQ-ACK for SPS PDSCH can be applied to HARQ-ACK feedback for multicast. It can be clarified that the UE procedure for deferring HARQ-ACK for SPS PDSCH can be applied to ACK/NACK-based HARQ-ACK feedback for multicast as currently specified in clause 9.2.5.4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In clause 18, for multiplexing HARQ-ACK information of different priorities associated with unicast DCI formats and with multicast DCI formats in a same PUCCH, it is clarified that determining the PUCCH resources for transmission should be based on the PRI indicated in the last unicast DCI or DCI with higher priority. - In clause 18, it is clarified that the semi-static PUCCH cell switching can support the ACK/NACK-based HARQ-ACK feedback for multicast as currently specified in clause 9.A. - In clause 18, it is clarified that deferring HARQ-ACK for SPS PDSCH can be applied to ACK/NACK-based HARQ-ACK feedback for multicast as currently specified in clause 9.2.5.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How unicast/multicast HARQ-ACKs with different priorities are multiplexed remains unclear. In addition, whether the semi-static PUCCH cell switching can support HARQ-ACK feedback for multicast and whether deferring HARQ-ACK for SPS PDSCH can be applied to HARQ-ACK feedback for multicast remain unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

18	Multicast Broadcast Services

< Unchanged parts are omitted >

If a UE multiplexes in a PUCCH HARQ-ACK information of same priority associated with unicast DCI formats and with multicast DCI formats in a same PUCCH, the last DCI format that the UE uses to determine the PUCCH resource, as described in clause 9.2.3, is a last unicast DCI format.

If the UE multiplexes in a PUCCH only multicast HARQ-ACK information of same priority that is according to both the first and second HARQ-ACK reporting modes, the last DCI format that the UE uses to determine the PUCCH resource, as described in clause 9.2.3, is a last DCI format associated with multicast HARQ-ACK information that is according to the first HARQ-ACK reporting mode.

If a UE multiplexes in a PUCCH only first HARQ-ACK information associated with multicast SPS PDSCH receptions and second HARQ-ACK information associated with multicast DCI formats and having same priority value as the first HARQ-ACK information, and both the first and second HARQ-ACK information are according to the first HARQ-ACK reporting mode, the UE determines the PUCCH resource based on the last multicast DCI format, as described in clause 9.2.3.

If a UE multiplexes in a PUCCH only first HARQ-ACK information associated with multicast SPS PDSCH receptions and second HARQ-ACK information associated with multicast DCI formats and having same priority value as the first HARQ-ACK information, and the first and second HARQ-ACK information are indicated by harq-FeedbackOptionMulticast different HARQ-ACK reporting modes, the UE determines the PUCCH resource based on:

-	sps-PUCCH-AN-ListMulticast, or sps-PUCCH-AN-List if sps-PUCCH-AN-ListMulticast is not provided, if the first HARQ-ACK information is according to the first HARQ-ACK reporting mode or

-	the last multicast DCI format, as described in clause 9.2.3, if the second HARQ-ACK information is according to the first HARQ-ACK reporting mode.

If a UE multiplexes in a PUCCH first HARQ-ACK information associated with unicast SPS PDSCH receptions and second HARQ-ACK information associated with multicast DCI formats and having same priority value as the first HARQ-ACK information in a same PUCCH, the UE determines the PUCCH resource from

-	if provided, SPS-PUCCH-AN-List for unicast SPS PDSCH receptions as described in clause 9.2.1;

-	else, if provided, PUCCH-Config/PUCCH-ConfigurationList for multicast PDSCH receptions;

-	else, PUCCH-Config/PUCCH-ConfigurationList for unicast PDSCH receptions.

If a UE multiplexes in a PUCCH first HARQ-ACK information associated with unicast SPS PDSCH receptions and second HARQ-ACK information associated with multicast SPS PDSCH receptions and having same priority value as the first HARQ-ACK information in a same PUCCH, the UE determines the PUCCH resource from SPS-PUCCH-AN-List for unicast SPS PDSCH receptions as described in clause 9.2.1.

If a UE multiplexes in a PUCCH only HARQ-ACK information associated with multicast SPS PDSCHs receptions of same priority that is according to the first HARQ-ACK reporting mode and is not provided sps-PUCCH-AN-ListMulticast, the UE determines the PUCCH resource from the sps-PUCCH-AN-List provided for unicast SPS PDSCH reception as described in clause 9.2.1.

If a UE multiplexes in a PUCCH multicast HARQ-ACK information only according to second HARQ-ACK reporting modes and CSI reports, the UE determines a PUCCH resource as described in clause 9.2.5.2 for multiplexing CSI reports with HARQ-ACK information that is in response to PDSCH receptions without corresponding PDCCHs.

A UE is not required to multiplex in a PUCCH multicast HARQ-ACK information of a priority and unicast UCI of the priority if the UE is provided subslotLengthForPUCCH for PUCCH transmissions with unicast UCI of the priority.

If a UE multiplexes in a PUCCH HARQ-ACK information of different priorities associated with unicast DCI formats and with multicast DCI formats according to the first HARQ-ACK reporting mode in a same PUCCH, the UE uses to determine the PUCCH resource with higher priority.

If a UE is be provided a PUCCH-sSCell by pucch-sSCell and a periodic cell switching pattern for PUCCH transmissions by pucch-sSCellPattern, the UE determines PUCCH transmissions for multicast HARQ-ACK information according to the first HARQ-ACK reporting mode as currently specified in clause 9.A.

If a UE is provided sps-HARQ-Deferral for multicast SPS PDSCH, the UE determines PUCCH transmissions for the HARQ-ACK information associated with multicast SPS PDSCH receptions according to the first HARQ-ACK reporting mode as currently specified in clause 9.2.5.4.

< End >