3GPP TSG RAN WG1 Meeting #113	                                                               R1-2304854

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:		9.2.2.1

Source:	China Telecom

Title:	Evaluation on AI/ML for CSI feedback enhancement

Document for:		Discussion

Introduction

A study item of Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved in RAN#94e meeting [1] and revised in RAN#96 meeting [2]. Based on the agreements in previous meeting, CSI enhancement is a basic and important use case. In this contribution, we report our evaluation results of AI/ML-based CSI compression of 1-on-1 joint training without generalization/scalability verification. The same evaluation result is also submitted to the ongoing evaluation results collection. We provide the template excel together with our contribution.

Discussion

System model of AI/ML-based CSI feedback enhancement

In this section, we discuss the procedure of the AI/ML-based CSI feedback compression sub use cases. The AI/ML model is based on the two-sided autoencoder structure, including the CSI compression part and the CSI decompression part. In the training phase, UEs estimate the channel according to the received CSI-RS, and feedback the corresponding channel information (e.g., eigen vectors) as training input to gNB for model training. After the training, the AI/ML model was used for inference. UEs use the eigen vectors as input of the encoder for CSI compression. The compressed CSIs are fed back to gNB. The gNB uses the CSI decoder to decompress the CSI feedback to obtain the recovered channel eigen vectors, which will be used to process precoding and beamforming. Besides, for CSI compression involving the time domain, CSI compression and decompression also use historical channel information for training/inference.

AI/ML model design

The AI/ML model for CSI feedback enhancement includes an encoder at UE side and a decoder at gNB side. We use the model as shown in Figure 1, in which each eigen-vector of subband in the input CSI matrix is firstly put into CNN layer, processed by 8 self-attention based blocks and full-connect layer are sequentially introduced before a uniform quantization layer. As for the decoder part, after dequantization layer, a full connect layer is employed, followed by residual layer and CNN layer as well, then 8 self-attention based blocks are introduced. Finally, a reshape layer is implemented to obtain the output with the shape of the original CSI. The AI/ML model apply MSE as the loss function.

Figure 1. Illustration of AI model for CSI feedback enhancement

Dataset construction

Based on the agreements, system-level simulation is used to construct datasets for AI/ML-based CSI feedback evaluation. The scenario of Dense Urban is adopted as baseline. The eigen vector of each subband are generated for model training, model verification and model testing.

The detail evaluation assumptions are shown below:

Table 1. Key simulation parameters of system-level simulation

| Parameter | Value |
| --- | --- |
| Scenario | Dense Urban (Macro only) is a baseline |
| Carrier Frequency | 4.0GHz |
| ISD | 200m |
| Bandwidth | 10MHz |
| Subcarrier spacing | 15kHz |
| RB number | 48 |
| Sub-band number | 12 |
| Granularity of the frequency unit for averaging operation | 4 RBs |
| Antenna Configuration | 32 Tx ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ 4 Rx ports: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Channel Model | According to 38.901 |
| Rank | 1 |
| Channel Estimation | Ideal |

Total 600,000 samples are provided. Each sample includes 768 real numbers, which corresponds to a large eigenvector concatenated by 12 sub-bands as:

w=[w1, w2, ⋯w12]

Where  wk, 1≤k≤12 is the eigenvector for the k-th sub-band channel. Each wk has been processed as the following format:

wk=[Rewk,1,Imwk,1,Rewk,2,Imwk,2⋯Rewk,32,Imwk,32]

where Im{} and Re{}are the real and imaginary parts.

In this simulation, we consider Rel-16 Type II codebook as baseline and SGCS are considered to evaluate the results. Ideal DL channel estimation is considered for the evaluations of CSI feedback enhancement, and eigen vector are calculated based on ideal channel matrix. The AI/ML model input for training and inference is the eigen vector generated.

Simulation results

We evaluate the intermediate performance of AI/ML model mentioned above with different feedback bits assumption. The generalized cosine similarity (SGCS) between recovered eigenvectors and original eigenvectors are shown in Table 2. Besides, the SGCS for Rel-16 enhanced Type-II codebook under approximate feedback bits are also given here for comparison.

Table 2. The intermediate performance with different feedback bits

| Overhead (bit) | SGCS |  |
| --- | --- | --- |
|  | AI model | Rel-16 eType-Ⅱ |
| 48 | 0.90 | 0.81 |
| 60 | 0.91 | 0.84 |
| 120 | 0.94 | 0.89 |

As shown in Table 2, with the same feedback bits, AI based approach outperform the Rel-16 enhanced Type II codebook. With similar performance, AI based approach could reduce above 60% feedback overhead.

Observation 1: Compared with traditional Rel-16 enhanced Type II codebook, AI/ML based CSI feedback schemes could improve the CSI accuracy with the same feedback bits.

Observation 2: Compared with traditional Rel-16 enhanced Type II codebook, AI/ML based CSI feedback schemes could reduce CSI feedback bits with the similar CSI accuracy.

Table 3 Simulation results for complexity

|  | Model complexity | Computational complexity (FLOPs) |
| --- | --- | --- |
| Encoder | 1.6M | 18.9M |
| Decoder | 1.7M | 115.4M |
| Total | 3.3M | 134.3M |

Table 3 shows the AI/ML model complexity results. The reason that the computational complexity of decoder is much higher than the encoder is due to the higher dimension of self-attention block’s input at decoder. Because UE has limitation of power and computation capability, light weight design is more suitable for encoder model.

Proposal 1: The evaluation results are submitted and should be captured in the technical report.

Conclusions

In this contribution, we report our simulation results on AI/ML-based CSI feedback enhancement. Following observations and proposals are given:

Observation 1: Compared with traditional Rel-16 enhanced Type II codebook, AI/ML based CSI feedback schemes could improve the CSI accuracy with the same feedback bits.

Observation 2: Compared with traditional Rel-16 enhanced Type II codebook, AI/ML based CSI feedback schemes could reduce CSI feedback bits with the similar CSI accuracy.

Proposal 1: The evaluation results are submitted and should be captured in the technical report.

References

- [1] RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm (Moderator), RAN#94e.

- [2] RP-221348, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#96.