3GPP TSG RAN WG1 #113			                                       R1-2305082

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:	9.1.4.1

Source: 	CMCC

Title: 	Discussion on UL precoding indication for multi-panel transmission

Document for:	Discussion and Decision

Introduction

In the RAN1#112bis-e meeting, the UL precoding indication for multiple panel transmission was discussed. In this meeting, single DCI and Multiple DCI transmission, the configuration of SRS resource sets, layer combination and DMRS port indication for SDM scheme were discussed [1]. The detailed agreements are listed in the related section in the discussion.

In this contribution, we discussed our views about the UL precoding indication for the simultaneous multiple panel UL transmission.

Discussion

- 2.1 Single DCI based STxMP

SFN

In the previous meetings, the details of the single DCI based SFN scheme for STxMP PUSCH was discussed.

Agreement (#111) For the SFN scheme of single-DCI based STxMP PUSCH: - Configure two SRS resource sets for CB or NCB. - FFS: Number of SRS resources of SRS resource set, and number of SRS ports of SRS resource - The DCI indicates two SRI fields and TPMI fields for SFN transmission, - On the indication of number of layers for CB and NCB PUSCH: - Alt1: Similar to rel-17 mTRP TDM scheme, the number of layers is indicated by the first SRI field (for NCB PUSCH) or the first TPMI field (for CB PUSCH) Agreement (#112) On dynamic switching between STxMP SFN scheme and sTRP transmission: - The legacy maxRank/Lmax is applied to sTRP transmission - For configuration of SFN, - Alt2: Configure a separate parameter for the maximal number of layers for STxMP SFN

For the SFN scheme of single DCI based STxMP PUSCH, the two panels of the UE transmit the same PUSCH to different TRPs. Then the configuration of the SRS resource sets for CB or NCB could follow the same procedure of that of the single TRP. That is the number of SRS resources of the SRS resource set is two for CB based transmission and four for the NCB based transmission. The number of the SRS ports of the SRS resource could be 1~4 for CB PUSCH and single port for the NCB PUSCH. The maximum number of the SRS ports for CB PUSCH depends on the UE capability. The maximum rank for SFN transmission of single DCI has not been discussed, which depends on the UE capability and configuration. Considering that the SDM scheme of STxMP only support maximum 2+2 transmissions, for SFN scheme, the maximum rank could be limited to two following the same principle.

Proposal 1:

- The number of SRS resources of the SRS resource set is two for CB based transmission and four for the NCB based transmission.

- The number of the SRS ports of the SRS resource could be 1~4 for CB PUSCH and single port for the NCB PUSCH.

Proposal 2:

The max Rank of single DCI SFN transmission could be limited to 2.

SDM

Agreement The codepoints of “SRS resource set indicator” in DCI for dynamic switching between STxMP SDM and sTRP transmission are interpreted and the SRI/TPMI fields are designed as follows: - The codepoints 00 and 01 indicate sTRP transmission. 00 indicates the first SRS resource set and 01 indicates the second SRS resource set. For SRI/TPMI field design, down-select one from the following Alts: - Alt1: The DCI has two SRI fields and two TPMI fields. The first SRI field and first TPMI field are associated the first SRS resource set if codepoint = 00 or the second SRS resource set if codepoint = 01. The second SRI field and second TPMI fields are reserved. - Alt2: the DCI has only one SRI field and one TPMI field. The SRI and TPMI field are associated with the first SRS resource set if codepoint=00 or the second SRS resource set if codepoint = 01. - Alt3: The DCI has two SRI fields and two TPMI fields. The first SRI field and second SRI field are concatenated into one SRI field. The first TPMI field and second TPMI field are concatenated into one TPMI field. The concatenated SRI field and the concatenated TPMI field are associated with first SRS resource set if codepoint = 00 or the second SRS resource set if codepoint = 01. - FFS: If the concatenated bits are not sufficient, additional bits are appended to concatenated bits in order to support this feature - Alt4: the DCI has two SRI fields and two TPMI fields. - When the codepoint is 00, the first SRI field and first TPMI field are associated with the first SRS resource set. The second SRI field and second TPMI field are reserved. - When the codepoint is 01, the second SRI field and second TPMI field are associated with the second SRS resource set. The first SRI field and first TPMI field are reserved. - The codepoints 10 indicate SDM transmission with the first and second SRS resource set. - The first SRI field and first TPMI field are associated with the first SRS resource set and they indicate the precoder(s)/rank/SRI for the first SRS resource set. - The second SRI field and second TPMI field are associated with the first SRS resource set and they indicate the precoder(s)/rank/SRI for the second SRS resource set. - FFS: The codepoint 11 is reserved.

It was agreed that the codepoint 00 and 01 indicate single TRP transmission. 00 indicates the first SRS resource set and 01 indicates the second SRS resource set. For the SRI/TPMI field design, four alternatives are listed to be down selected. For Alt 2, since it is dynamic switching between single TRP and SDM, it will change the total bit size of the DCI due to different transmission mode. Alt 1 and Alt 4 are preferred considering that the bit size will not change due to the dynamic switch between single TRP and SDM transmission. Since RAN1 have no consensus to configure different number of SRS resources in the two SRS resources sets for CB or NCB for single DCI based STxMP. The difference between Alt 1 and Alt 4 is not much. The Alt 4 which has a clear association between different fields are slightly preferred. In the current situation, the codepoint 11 can be as reserved bits, since no other scenario or usage is observed.

Proposal 3:

The Alt 4 which has a clear association between different fields are slightly preferred.

- Alt4: the DCI has two SRI fields and two TPMI fields.

  - When the codepoint is 00, the first SRI field and first TPMI field are associated with the first SRS resource set. The second SRI field and second TPMI field are reserved.

  - When the codepoint is 01, the second SRI field and second TPMI field are associated with the second SRS resource set. The first SRI field and first TPMI field are reserved.

Proposal 4:

Confirm the codepoint 11 as reserved bits.

- 2.2 Multiple DCI based STxMP

Agreement The multi-DCI based STxMP PUSCH+PUSCH transmission supports fully/partially/non-overlapping in frequency domain and fully/partially overlapping in time domain. - FFS whether/how to handle the PUSCH power adjustment when two PUSCHs are fully/partially overlapped in time domain (Depending on RAN4’s input on Pcmax requirements). - Note: No symbol-level power adjustment within a PUSCH transmission occasion in the case of fully/partially overlapping in time domain

The multiple DCI based STxMP PUSH+PUSCH transmission supports fully/partially overlapping in the time domain. If there is a overlapping transmission, a predefined maximum transmit power limit could be defined to prevent the power adjustments during the transmission.

Proposal 5:

A predefined maximum transmit power could be configured for each of the panel, to prevent the power adjustments during the transmission.

In the last meeting, the issue of PUCCH overlapping with two overlapped PUSCHs in multiple DCI based STxMP PUSCH+PUSCH was discussed.

Agreement For case that one PUCCH overlaps with two overlapped PUSCHs in multi-DCI based STxMP PUSCH+PUSCH, down-select one for the UCI multiplexing: - Option 1: the UCI is multiplexed into the PUSCH associated with the same TRP. And among the PUSCHs associated with the same TRP, the legacy PUSCH priority order for UCI multiplexing is applied. FFS: determining the PUSCH associated with the same TRP. - Option 2: the legacy PUSCH priority order for UCI multiplexing is first applied and if at last, there are two PUSCHs with the same start time in one same CC, the UCI is multiplexed in (FFS: one or two) of these two PUSCHs, and FFS which one PUSCH. - Option 3: - When joint HARQ-ACK feedback is configured, the legacy PUSCH priority order for UCI multiplexing is first applied and if at last, there are two PUSCHs with the same start time in one same CC, the UCI is multiplexed in (FFS: one or two) of these two PUSCHs, and FFS which one PUSCH. - When separate HARQ-ACK feedback is configured, at least when the UCI includes HARQ-ACK, the UCI is multiplexed into the PUSCH associated with the same TRP. And among the PUSCHs associated with the same TRP, the legacy PUSCH priority order for UCI multiplexing is applied. FFS: determining the PUSCH associated with the same TRP. - FFS: When the UCI does not include HARQ-ACK (CSI and/or SR), whether to follow the same behavior as above, or to follow the behavior of the case that joint HARQ-ACK feedback is configured. - Note: Here using joint HARQ-ACK feedback and separate HARQ-ACK feedback is mainly for discussion purpose. FFS: whether to introduce a new RRC parameter to indicate that. - FFS the impact of the following legacy restriction on the above options: when separate HARQ feedback is configured, a PUCCH transmission triggered by DCI associated with one coresetPoolIndex cannot overlap in time with a PUSCH transmission triggered by DCI associated with another coresetPoolIndex. - Note: each of the above options is applied to the system when the system is configured with multi-DCI based STxMP PUSCH+PUSCH.

Considering that STxMP is supported in this release, the UCI can be multiplexed into the PUSCH associated with the same TRP. The legacy PUSCH priority rule can be applied for each panel’s transmission. The determination of which TRP should be associated could be based on the related coreset pool index of the PDCCH.

Proposal 6:

Considering the STxMP is supported, the UCI can be multiplexed into the PUSCH with the same TRP. Then Option 1 is preferred.

- Option 1: the UCI is multiplexed into the PUSCH associated with the same TRP. And among the PUSCHs associated with the same TRP, the legacy PUSCH priority order for UCI multiplexing is applied. FFS: determining the PUSCH associated with the same TRP.

Conclusions

In this contribution, we discussed the related issues about UL precoding indication for the simultaneous multi-panel UL transmission. The proposals are listed as below.

Proposal 1:

- The number of SRS resources of the SRS resource set is two for CB based transmission and four for the NCB based transmission.

- The number of the SRS ports of the SRS resource could be 1~4 for CB PUSCH and single port for the NCB PUSCH.

Proposal 2:

The max Rank of single DCI SFN transmission could be limited to 2.

Proposal 3:

The Alt 4 which has a clear association between different fields are slightly preferred.

- Alt4: the DCI has two SRI fields and two TPMI fields.

  - When the codepoint is 00, the first SRI field and first TPMI field are associated with the first SRS resource set. The second SRI field and second TPMI field are reserved.

  - When the codepoint is 01, the second SRI field and second TPMI field are associated with the second SRS resource set. The first SRI field and first TPMI field are reserved.

Proposal 4:

Confirm the codepoint 11 as reserved bits.

Proposal 5:

A predefined maximum transmit power could be configured for each of the panel, to prevent the power adjustments during the transmission.

Proposal 6:

Considering the STxMP is supported, the UCI can be multiplexed into the PUSCH with the same TRP. Then Option 1 is preferred.

- Option 1: the UCI is multiplexed into the PUSCH associated with the same TRP. And among the PUSCHs associated with the same TRP, the legacy PUSCH priority order for UCI multiplexing is applied. FFS: determining the PUSCH associated with the same TRP.

References

[1] Chair’s notes RAN1#112bis-e, 3GPP TSG RAN WG1 #112bis-e, Online, 17th - 26th April 2023