3GPP TSG-RAN WG1 Meeting #113	R1-2305940

Incheon, Korea, May 22 – 26, 2023

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on RS LS on supported for group-based reporting

Document for:	Discussion and Decision

Discussion

1. Overall Description: RAN4 is discussing requirements for Rel-17 group-based L1-RSRP measurement and report (mTRP-GroupBasedL1-RSRP-r17) as part of Requirement for NR FR2 multi-Rx chain DL reception WI. RAN4 would like to ask RAN1 to clarify the following questions on RAN1 specification on RS supported for group-based beam reporting. Question 1: RAN4 understands that two resource sets consisting of SSB + SSB or CSI-RS + CSI-RS can be configured for Rel-17 group-based L1-RSRP measurement and report. However, RAN4 is not clear whether the two resource sets configured for Rel-17 group-based L1-RSRP measurement and report can contain a mix of SSB and CSI-RS. Can RAN1 clarify whether a mix of SSB and CSI-RS can be configured for Rel-17 group-based L1-RSRP measurement and report? Question 2: When CSI-RS resources are configured as resource set for Rel-17 group-based L1-RSRP measurement and report, can the CSI-RS resource with repetition ON be configured for Rel-17 group-based L1-RSRP report? Question 3: Does RAN1 specification define UE reporting behaviour for the case when NW configures UE with Rel-17 group-based L1-RSRP measurement and report, and UE did not find any pair of beams for report (e.g., the measured L1-RSRP of at least one of the beams of any beam pair is lower than a certain value, e.g., -140dBm)?

In the last meeting, one LS [1] was sent from RAN4 to RAN1 for clarification of three questions. In this paper, we provide our views on these questions.

For Question 1：

In Rel-17 group-based L1-RSRP reporting, the reportQuantity parameter can be only configured to either ‘cri-RSRP’ or ‘ssb-Index-RSRP’. Hence, configuring both SSB and CSI-RS for Rel-17 group-based L1-RSRP reporting is not allowed. Otherwise, if a reported resource group contains a CRI and an SSB-index, it is contradictory with the configured reportQuantity parameter.

Proposal 1: The two resource sets configured for Rel-17 group-based L1-RSRP measurement and report cannot contain a mix of SSB and CSI-RS.

Agreement When the higher layer parameter groupBasedBeamReporting-r17 in CSI -ReportConfig is configured, the UE can be configured with the same or different value(s) of repetition in different CSI Resource Sets. - FFS: UE reports CRI regardless of whether repetition is set to ‘on’ or not - Whether/how to capture the above is up to editor.

For Question 2:

As agreed in RAN1 #108 meeting [2], a resource set configured for Rel-17 group-based L1-RSRP reporting can be configured with repetition = ‘on’. The two resource sets can be even configured with different value, i.e., one resource set configured with repetition = ‘on’ and the other resource set configured with repetition = ‘off’.

Proposal 2: When CSI-RS resources are configured as resource set for Rel-17 group-based L1-RSRP measurement and report, the CSI-RS resource can be configured with repetition = ‘on’.

For Question 3:

we don’t think RAN1 has defined the UE reporting behavior in the case mentioned in the question. UE reporting behavior is UE implementation.

Proposal 3: RAN1 has not defined the UE reporting behavior in the case mentioned in Question 3.

References

[1] R1-2304311, LS on RS supported for group-based reporting, April 17-26, 2023.

[2] RAN1 #108-e minutes, Chairman, February 21 - March 4, 2022.