3GPP TSG-RAN WG1 Meeting #113	R1-2305986

Incheon, Korea, 22 – 26 May, 2023

Agenda Item:	9.4.1.2

Source:	Moderator (Huawei)

Title:	FL summary#1 for AI 9.4.1.2 SL-U physical channel design framework

Document for:	Discussion and decision

# Introduction

The latest R18 WID on sidelink evolution (RP-230077) includes the following objective regarding support of sidelink on unlicensed spectrum (SL-U):

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - Focus on FR1 unlicensed bands (n46 and n96/n102). - Note: In sidelink unlicensed operation, the gNB does not perform Type 1 channel access to initiate and share a channel occupancy, neither Type 2 channel access to share an initiated channel occupancy, nor semi-static channel access procedures to access an unlicensed channel.

This contribution provides discussions related to SL-U physical channel design framework (AI 9.4.1.2), including summary of contributions, FL’s proposals, discussions, outcome of this meeting, etc. The related email thread is as below:

[113-R18-SL] Email discussion on NR sidelink evolution – Kevin (OPPO)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

# Issues

## Issue#1: SL bandwidth part and resource pool

### Background

- Some issues are mentioned, e.g., whether to use guardband PRBs for PSFCH/S-SSB transmission, whether to consider Uu TDD configuration, whether to set bitmap to all “1”s, whether to support one SL resource pool includes sub-set of PRBs of one RB set, etc.

- FL assumes such issues are not very urgent nor essential at this stage, and some of them can be discussed after the details are clearer (e.g., PSFCH/S-SSB transmission scheme, etc.). Considering we already have so many essential issues to be resolved at this meeting, FL does not organize proposals on these issues for now.

### Round 1: Discussions

So far, no proposals are given. If you have any comments, please provide in box below.

| Company | Comments |
| --- | --- |
| DCM | At least “whether to support one SL resource pool includes sub-set of PRBs of one RB set” can be discussed. |

## Issue#2: Slot structure

### Round 1: Proposals and background

#### [M] Proposal 2-1

Background: TBS determination

- Summary

  - On Lref

    - Lref replaces sl-LengthSymbols (6): vivo, LGE, ZTE, Apple, CATT, Huawei/HiSilicon

    - Lref replaces Nsymbsh (1): OPPO

  - On NsymbPSFCH

    - NsymbPSFCH is determined in the same way as in legacy NR SL (4): Huawei/HiSilicon, vivo, ZTE, NOK

    - NsymbPSFCH is always 0, i.e., ‘PSFCH overhead indication’ field in SCI-1 is always 0 bit (1): Docomo

- FL’s view

  - A proposal is given to reflect the above.

(Below is copied from TS 38.214) #### 8.1.3.2 Transport block size determination For the PSSCH assigned by SCI, if Table 5.1.3.1-2 is used and , or a table other than Table 5.1.3.1-2 is used and the UE shall first determine the TBS as specified below: The UE shall first determine the number of REs (NRE) within the slot. - A UE first determines the number of REs allocated for PSSCH within a PRB (NRE') by NRE'=NscRBNsymbsh-NsymbPSFCH-NohPRB-NREDMRS, where - NscRB=12 is the number of subcarriers in a physical resource block, - Nsymbsh = sl-LengthSymbols -2, where sl-LengthSymbols is the number of sidelink symbols within the slot provided by higher layers, - NsymbPSFCH = 3 if 'PSFCH overhead indication' field of SCI format 1-A indicates "1", and NsymbPSFCH = 0 otherwise, if higher layer parameter sl-PSFCH-Period is 2 or 4. If higher layer parameter sl-PSFCH-Period is 0, NsymbPSFCH=0. If higher layer parameter sl-PSFCH-Period is 1, NsymbPSFCH=3. - NohPRB is the overhead given by higher layer parameter sl-X-Overhead, - NREDMRS is given by Table 8.1.3.2-1 according to higher layer parameter sl-PSSCH-DMRS-TimePatternList. … (below is copied from TS 38.331) SL-BWP-Generic-r16 ::= SEQUENCE { sl-BWP-r16 BWP OPTIONAL, -- Need M sl-LengthSymbols-r16 ENUMERATED {sym7, sym8, sym9, sym10, sym11, sym12, sym13, sym14} OPTIONAL, -- Need M sl-StartSymbol-r16 ENUMERATED {sym0, sym1, sym2, sym3, sym4, sym5, sym6, sym7} OPTIONAL, -- Need M sl-PSBCH-Config-r16 SetupRelease {SL-PSBCH-Config-r16} OPTIONAL, -- Need M sl-TxDirectCurrentLocation-r16 INTEGER (0..3301) OPTIONAL, -- Need M ... } …

Agreement (in RAN1#112b-e) If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - TBS is determined based on a reference number of symbols (denoted as L_ref) - Support the followings - Alt 1: Support Option 4 only - Note: the options are as below - Option 4: The reference number of symbols is determined by (pre-)configuration - FFS details, e.g., in TS 38.214 Clause 8.1.3.2, whether L_ref replaces Nsymbsh or sl-LengthSymbols or other usage of L_ref, whether NsymbPSFCH is determined in the same way as in legacy NR SL, etc.

[M] Proposal 2-1

If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, in TS 38.214 Clause 8.1.3.2:

- L_ref replaces sl-LengthSymbols

  - Value range of L_ref is {7, 8, 9, 10, 11, 12, 13, 14} symbols

- NsymbPSFCH is determined in the same way as in legacy NR SL

| Company | Agree? | Comments |
| --- | --- | --- |
| Qualcomm | No | We don’t agree on ‘Value range of L_ref is {7, 8, 9, 10, 11, 12, 13, 14} symbols ’. L_ref needs to be chosen based on the 1st starting symbol so that the spectrum efficiency can be maximized. If Lref=7, the link only achieves half of the maximum throughput rate. If we really worry about the robustness of PSSCH decoding for the 1st slot of MCSt which may start at the 2nd starting symbol. The Tx UE could by implementation choosing lower MCS for the slot with potentially 2 starting location. We suggest the following [M] Proposal 2-1 If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, in TS 38.214 Clause 8.1.3.2: - L_ref replaces sl-LengthSymbols - Option 1: Value range of L_ref is {7, 8, 9, 10, 11, 12, 13, 14} symbols - Option 2: L_ref to be chosen based on 1st starting symbol |
| DCM | NO | We do not understand motivation to keep NsymbPSFCH. Dynamic determination is not aligned with the intention of (pre-)configuring L_ref. |
| LGE | Yes | In our understanding, both 1st starting symbol and L_ref will be provided by a (pre)configuration. Then, it is up to network how to the values are set. We are not so sure it is necessary to specify that L_ref is selected based on 1st starting symbol. On the dynamic indication, it is understood that some flexibility may or may not be helpful. At least, we think that the L_ref is set to the same as sl-LenghtSymbols, there is no reason not to use the existing dynamic determination. For a compromise, it can be considered that L_ref replaces sl-LengthSymbols if N_PSFCH=0. In this case, spectral efficiency for the PSFCH-slot could be maintained by using “sl-LengthSymbols-2- N_PSFCH”. |
| Panasonic | Yes |  |
| MediaTek | No | We share similar opinion as QC the value of L_ref should be configured based on the 1st starting symbol. Most of the transmissions on the slot(s) (occupying a high ratio of an initiated COT) rather than the 1st slot of an initiated COT will be started from the 1st starting symbol. While the 1st slot of an initiated COT only occupies a low ratio of that COT (e.g., 1/20 for a COT of 10ms under 30kHz). A L_ref value based on 1st starting symbol can maximize the spectrum efficiency. For the case that channel access is finished before the 2nd starting symbol(s), a repetition of a sub-set of the prepared TB can be transmitted and it is up to UE implementation to decode (e.g., for combination gain) the sub-set of the prepared TB or not (e.g., for power saving). At current stage, the modification from QC is OK for us. |
| CATT/GOHIGH | Yes |  |
| Lenovo | OK |  |
| Xiaomi | OK |  |
| NEC | Yes | Regarding comment "L_ref should be configured based on the 1st starting symbol" we don't think it's necessary. Anyway, even the condition is assumed, the network can configure a suitable value. We don't need to add restriction. Moreover, previous agreement said "Option 4: The reference number of symbols is determined by (pre-) configuration", the pending issue is to determine the value range. We also think L_ref replaces sl-LengthSymbols is a more simple and is reusing legacy way. |
| Spreadtrum | Yes |  |
| Sharp | No | Similarly to DCM we also don’t see the reason to keep NsymbPSFCH in this case. |
| Transsion | No | We share the same view as QC and MTK |

#### [M] Proposal 2-2

Background: AGC symbols

- Summary

  - Regarding Tx UE behaviour:

    - Option 1: The PSCCH/PSSCH transmission has 2 symbols for AGC purpose

      - Support (8): CATT, Huawei/HiSilicon, OPPO, CATT, CMCC, Spreadtrum, NEC, Panasonic

    - Option 2: The PSCCH/PSSCH transmission has only 1 symbol for AGC purpose

      - Support (8): Qualcomm, Ericsson, ZTE, Futurewei, [Docomo], MediaTek, Transsion Holdings, Sony

    - Option 3: The PSCCH/PSSCH transmission has 1 or 2 symbol(s) for AGC purpose depending on conditions, FFS details

      - Support (8): Samsung, Apple, vivo, Lenovo, InterDigital, ITL, Nokia, Fraunhofer

  - Regarding Rx UE behavior:

    - Option A: The Rx UE always monitors two AGC symbols in such slot

      - Support (3): CATT, Huawei/HiSilicon, NEC

    - Option B: The Rx UE monitors two AGC symbols in such slot by default, but could drop monitoring the 2nd AGC symbol at least if it detects a PSCCH/PSSCH transmission starting from the 1st starting symbol

      - Support (4): Qualcomm, Samsung, Lenovo, InterDigital

    - Option C: The Rx UE monitors two AGC symbols in such slot by default, but it is up to UE implementation whether to drop monitoring the 2nd AGC symbol

      - Support (2): OPPO, CMCC

    - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot

      - Support (10): Qualcomm, Ericsson, ZTE, Nokia, MediaTek, Futurewei, Spreadtrum, NEC, Sony, Transsion Holdings

- FL’s view

  - A proposal is given to reflect the above.

[M] Proposal 2-2

For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, support the following:

- Regarding Tx UE behaviour:

  - If PSCCH/PSSCH transmission starts from 1st starting symbol, the PSCCH/PSSCH transmission has 1 or 2 AGC symbols

    - FFS: whether this is subject to explicit or implicit (pre-)configuration, or under some conditions, etc.

- Regarding Rx UE behaviour:

  - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot

- Note: AGC symbol is duplication of next symbol

| Company | Agree? | Comments |
| --- | --- | --- |
| Qualcomm | Ok with Rx UE No on Tx UE | We don’t see the need of introducing 2nd AGC symbol for transmission staring from 1st starting symbol. Firstly, it increases overhead and the Rx UE implementation most likely cannot make use of it since retuning AGC hurts the phase continuity of channel estimation. Secondly, additional AGC symbols also require redesigning DMRS patterns to avoid getting punctured/replaced by AGC symbol (which happens at least for some combinations of second starting symbol location and DMRS pattern, e.g., #7 and 4-symbols DMRS). If companies are really worried about the AGC issue for the full slot receiver, the Tx UE could (by implementation) choose lower MCS and the Rx UE could (by implementation) use the PSSCH SCH REs in 2nd starting symbol for AGC retuning and puncture that symbol in PSSCH decoding. |
| DCM | NO | For TX side, only the first symbol of each TX should be duplication. No further duplication does not bring any benefit. For RX side, meaning of “monitor AGC symbol” is unclear. There is not such a text in the current spec. Without clarification, companies’ understanding of UE behavior may be different. |
| OPPO | No | For RX side, if it is up to UE implementation to monitor 1 or 2 AGC symbols, there will be following possibilities: - RX UE only monitor for 2nd AGC symbol, not monitor for 1st AGC symbol; - RX UE only monitor 1st AGC symbol, and not monitor 2nd AGC symbol even if it cannot detect any transmission from 1st AGC symbol. All of these possibilities will cause communication loss which should be avoided. Therefore, we prefer Option C. For TX UE, based on above analysis, correspondingly TX UE should consider 2 AGC symbols. Therefore, we prefer option 1. |
| LGE | Ok with Rx UE No on Tx UE | For compromise, we can add “RAN1 does not pursue specific enhancement on introducing new PSCCH/PSSCH structure”. In other words, the TX UE may consider the potential two AGC symbols for selecting DMRS pattern, 2nd SCI overhead, and so on. |
| Panasonic | No | For Tx side, AGC symbol is not necessary to be duplication of next symbol. For Rx side, it is up to UE implementation to use the symbol before 2nd stating symbol for AGC. |
| Mediatek | No for Tx | As analyzed in our paper, we do not see the motivation of introducing the 2nd AGC symbol with the following observations as partially mentioned by QC as well: - The spectrum efficiency will be degraded if a 2nd AGC symbol is introduced. - Considering WiFi is an asynchronization system, introducing a 2nd AGC symbol cannot solve such issue. - Phase jump issue maybe occurred if AGC is readjusted on the 2nd starting symbol. For the transmission started from the 1st starting symbol, the AGC gain will be reduced if a 2nd AGC symbol is used on the 2nd starting symbol. |
| CATT/GOHIGH | Comment | For Tx side, we prefer to use 2 AGC symbols to resolve the potential AGC issues, i.e. option 1. For Rx side, even we prefer option A, but we can accept leaving it by UE implementation. |
| Lenovo |  | If Tx UE can determine 1or 2 AGC symbols based on (pre-)configuration and frequency resource, for Rx UE, we should assume its (pre-)configuration for number of AGC symbol is same as Tx UE’s and can detect the frequency resource indication from Tx UE. So we prefer to same behavior for both Tx UE and Rx UE, but not up to UE implementation for Rx UE. |
| xiaomi | Ok with Rx UE No on Tx UE | We think we shall make a down-selection among above options. For the Tx UE behavior, we don’t support the proposal, we support option 1. Different UEs with different RB sets may start from 1st or 2nd starting symbol. If one UE starts from the 1st starting symbol in RB set1, and the another UE with RB set2 starts from 2nd starting symbol in the same slot, there shall also has a AGC in the 2nd stating symbol to ensure PSSCH reception.So there always have two AGC symbols in time domain whenever 2 candidate starting symbols are configured. For the Rx UE behavior, we support the proposal. |
| NEC | TX: No RX: Yes | Option 1 should be adopted. We still think it's necessary to have a 2nd AGC symbol for the case some UEs will access to the channel starting 2nd starting symbol. Therefore, a AGC is at least needed for the transmission starting from 1st starting symbol. |
| Spreadtrum | Ok with Rx UE | Regarding Tx UE behavior, we prefer 2 AGC symbols. |
| Transsion | OK | We are fine with the proposal as a compromise. |

#### [L] Proposal 2-3

Background: Tx UE behavior on using 1st or 2nd starting symbol

- Summary

  - Whether/how to support that for the remaining slots of a COT, the Tx UE only chooses the 1st starting symbol for PSCCH/PSSCH transmission

    - Support (8): Qualcomm, Huawei/HiSilicon, [vivo], [Docomo], [MTK], [Apple], [NOK], InterDigital

      - Vivo: The COT initiating UE should transmit only at the 1st starting symbol in the remaining COT. The responding UE can transmit at either the 1st or the 2nd starting symbol in the remaining COT.

      - Docomo: For the remaining slots of a COT, TX UE only chooses the 1st starting symbol for PSCCH/PSSCH, if the transmission is started with no greater than 16us gap from the previous transmission by any UE, including MCSt.

      - MTK, NOK: at least for transmission with no greater than 16us gap from the previous transmission by any UE

      - Apple: only if the gap between the PSCCH/PSSCH transmission and its previous transmission is less than 16 µs

    - Not Support (2): OPPO, SPRD

- FL’s view

  - A proposal is given to reflect the above.

[L] Proposal 2-3

Regarding Tx UE behavior:

- For the slots of a COT except the 1st slot, Tx UE only chooses the 1st starting symbol for PSCCH/PSSCH transmission, at least for transmission with no greater than 16us gap from the previous transmission by any UE.

| Company | Agree? | Comments |
| --- | --- | --- |
| Qualcomm | Ok |  |
| DCM | YES |  |
| vivo | comments | The intention of this proposal (only concerning the Tx UE) is not clear. Is it intended to preclude the Tx UE that failed to access the channel in the 1st symbol from retrying at the 2nd symbol? If yes, what is the benefit (without considering the Rx UE behavior)? |
| OPPO |  | Generally fine with the principle. While whether such proposal is needed? If the transmission gap is <16 us within COT, type 2C LBT will be performed (no LBT), then it is naturally that TX UE can select the 1st staring symbol. No agreement is needed any more. |
| LGE | No | If the previous transmission is performed by other UE, the TX UE can still perform Type 2B channel access procedure. In this case, it is also possible that the TX UE fail to access the channel. Then, the TX UE can still try to access the channel from the 2nd starting position. In this case, “any UE” needs to be replaced with “the UE”. |
| Panasonic | Yes |  |
| MediaTek | Yes |  |
| Fraunhofer | Yes |  |
| CATT/GOHIGH | Yes | We share similar understanding as OPPO, if the condition is “at least for transmission with no greater than 16us gap from the previous transmission by any UE.”, it seems that UE will always select the first symbol transmission by type 2c. |
| Lenovo |  | Please help to further clarify why need the limitation of ‘at least for transmission with no greater than 16us gap from the previous transmission by any UE.’? To our understanding, this proposal can only focus on ‘starting symbol for PSCCH/PSSCH transmission’, it doesn’t include to discuss the CPE transmission and the data transmission in previous slot. How to ensure the gap duration less than or equal to 16us can be solved by the proposal on CPE transmission and data transmission in previous slot. |
| Xiaomi | comment | MCSt is a transmission with no greater than 16us gap from the previous transmission, and we think the Docomo’s proposal is more specified and shall be supported: “For the slots of a COT except the 1st slot, Tx UE only chooses the 1st starting symbol for PSCCH/PSSCH transmission, at least for transmission with no greater than 16us gap from the previous transmission by any UE, including MCSt.” |
| NEC | Yes | We are fine with the principle. We think this proposal is intending to maintain the COT, not only for MCSt because there are also other approaches to achieve MSCt. Therefore, if we think maintaining a COT is necessary, we should keep this proposal and better to add "to maintain a COT" before the proposal. Otherwise, if it’s not necessary, we can also accept to abandon this proposal as mentioned by OPPO. |
| Spreadtrum | OK | We can accept this proposal. |
| Sharp | Yes |  |

#### Others

- Some other issues are mentioned:

  - Some companies (e.g., Qualcomm, ZTE, Samsung, vivo, etc.) mentioned some PSCCH blind decoding reduction designs, e.g., limit PSCCH search locations, having only 1 common PSCCH occasion in such slots, introducing preamble, etc.

  - Some companies (e.g., Apple, Docomo, etc.) mentioned the PSFCH location may be 1 slot further compared with legacy design due to processing time.

  - Some companies (e.g., OPPO, Sharp, etc.) mentioned some signaling for 1st and 2nd starting symbol should be different, e.g., DMRS pattern, etc.

  - Some companies (e.g., OPPO, Sharp, etc.) mentioned the OFDM symbols used for SL RSSI measurement should start from the next symbol of the 2nd candidate starting symbol.

  - Some companies (e.g., Qualcomm, etc.) mentioned using 2nd starting symbol is subject to UE capability.

- FL assumes such issues are not very urgent nor essential at this stage. Considering we already have so many essential issues to be resolved at this meeting, FL does not organize proposals on these issues for now.

If you have any other comments, please provide in box below.

| Company | Comments |
| --- | --- |

## Issue#3: PSCCH/PSSCH

### Round 1: Proposals and background

#### [M] Proposal 3-1

Background: IRB, frequency domain resource indication (sub-channel indication)

- Summary: views on sub-channel indication

  - FRIV

    - Support (9): Qualcomm, OPPO, LGE, ZTE, Apple, CATT, Sharp, Huawei/HiSilicon, ITL

  - RIV

    - Support (2): MTK, Lenovo

  - Bitmap

    - Support (3): MTK, Lenovo, ITL

    - Not support (2): Huawei/HiSilicon, OPPO

- Summary: others

  - Use non-contiguous interlaces

    - Support (3): LGE, Lenovo, HW

    - Not support (2): OPPO, Apple

  - Different transmissions use different number of RB set(s) while keeping total number of sub-channels unchanged between initial transmission and retransmission(s) for a TB

    - Support (3): Docomo, [ZTE], SPRD

    - Not support (5): vivo, OPPO, Apple, CATT, Huawei/HiSilicon

  - 1 sub-channel maps to K non-contiguous interlace:

    - Support (1): LGE

    - Not support (2): [ZTE], MTK,

- FL’s view

  - FL suggests to use FRIV to align with legacy NR SL design.

  - FL suggests to FFS using non-contiguous interlace via spare states of FRIV.

  - To make the design simpler, FL suggests not to pursue enhancements like “Different transmissions use different number of RB set(s) while keeping total number of sub-channels unchanged between initial transmission and retransmission(s) for a TB”, “1 sub-channel maps to K non-contiguous interlace”.

  - A proposal is given to reflect the above.

(below is from TS 38.214) The starting sub-channel nsubCH,0start of the first resource is determined according to clause 8.1.2.2. The number of contiguously allocated sub-channels for each of the N resources LsubCH≥1 and the starting sub-channel indexes of resources indicated by the received SCI format 1-A, except the resource in the slot where SCI format 1-A was received, are determined from "Frequency resource assignment" which is equal to a frequency RIV (FRIV) where. If sl-MaxNumPerReserve is 2 then FRIV=nsubCH,1start+i=1LsubCH-1N subchannel SL+1-i If sl-MaxNumPerReserve is 3 then FRIV=nsubCH,1start+nsubCH,2start⋅N subchannel SL+1-LsubCH+i=1LsubCH-1N subchannel SL+1-i2 where - nsubCH,1start denotes the starting sub-channel index for the second resource - nsubCH,2start denotes the starting sub-channel index for the third resource - N subchannel SL is the number of sub-channels in a resource pool provided according to the higher layer parameter sl-NumSubchannel If TRIV indicates N < sl-MaxNumPerReserve, the starting sub-channel indexes corresponding to sl-MaxNumPerReserve minus N last resources are not used.

[M] Proposal 3-1

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings:

- Use FRIVsubCH to indicate used sub-channel index(s)

- FRIVsubCH is conveyed in 1st stage SCI

- FRIVsubCH is calculated as below

  - If sl-MaxNumPerReserve is 2 then

    - FRIVsubCH=nsubCH,1start+i=1LsubCHRBset-1N subchannel RBset+1-i

  - If sl-MaxNumPerReserve is 3 then

    - FRIVsubCH=nsubCH,1start+nsubCH,2start⋅N subchannel RBset+1-LsubCHRBset+i=1LsubCHRBset-1N subchannel RBset+1-i2

  - where

    - nsubCH,1start denotes the starting sub-channel index for the second resource

    - nsubCH,2start denotes the starting sub-channel index for the third resource

    - N subchannel RBset is the number of sub-channels in one RB set

    - LsubCHRBset is the number of used sub-channels within one RB set for one PSCCH/PSSCH transmission

- FFS: whether/how to use spare states of FRIVsubCH to indicate using non-contiguous interlaces

| Company | Agree? | Comments |
| --- | --- | --- |
| Qualcomm | OK |  |
| DCM | YES | “Different transmissions use different number of RB set(s) while keeping total number of sub-channels unchanged between initial transmission and retransmission(s) for a TB” should also be discussed. |
| OPPO | Yes with comment | Whether the last FFS is needed? This mechanism is used in NR-U resource indication. While it is hardly to be applied in SL-U since up to 3 resource should be indicated in SL, while only 1 is indicated in NR-U. if non-contiguous interlace is supported, that will introduce large payload size considering up to 3 resources to be indicated. |
| LGE | OK |  |
| Panasonic | Yes |  |
| MTK | Comment | For FRIV manner, we are OK. For bitmap manner, it is also supported in NR-U (as copied below from TS 38.214 Clause 6.1.2.2.3), where one benefit compared to FRIV manner from our point of view is the bit consumption. For example, for 15kHz, 5 interlace within one RB set, 5 bits are needed for bitmap manner while 6 bits for FRIV manner (i.e., 3+3) for the indication of interlace/sub-channel. As current stage, we think bitmap manner for sub-channel indication should not be excluded. TS 38.214 Clause 6.1.2.2.3 - For µ=0, the X=6 MSBs of the resource block assignment information indicates to a UE a set of allocated interlace indices m0+l, where the indication consists of a resource indication value (RIV). For 0≤RIV<M(M+1)/2 , l=0,1,⋯L-1 the resource indication value corresponds to the starting interlace index m0 and the number of contiguous interlace indices (). The resource indication value is defined by: For µ=1, the X=5 MSBs of the resource block assignment information comprise a bitmap indicating the interlaces that are allocated to the scheduled UE. The bitmap is of size M bits with one bitmap bit per interlace such that each interlace is addressable, where M and interlace indexing is defined in Clause 4.4.4.6 in [4, TS 38.211]. The order of interlace bitmap is such that interlace 0 to interlace M-1 are mapped from MSB to LSB of the bitmap. An interlace is allocated to the UE if the corresponding bit value in the bitmap is 1; otherwise the interlace is not allocated to the UE. |
| CATT/GOHIGH | Comment | Regarding the meaning of the parameters in FRIV, because when PSCCH/PSSCH transmission may span over multiple RB sets, the IRB index in different used RB set is the same, so a slightly update as following on nsubCH,1start and nsubCH,2start is preferred. Regarding the spare states of FRIVsubch, we don’t agree to use them to indicate non-contiguous interlaces (or sub-channels). Such resource indication is fine for NR-U, because only one resource needs to be indicated for only one transmission. But for SL-U, where the resource reservation should be considered, the resource indication principles of R16 NR SL rather than NR-U should be applied, thus only contiguous interlaces(s) or sub-channels can be indicated. It is aligned with FL’s understanding (FL suggests to use FRIV to align with legacy NR SL design). Besides, if the resource indication of non-contiguous interlaces(s) or sub-channels is supported, then only the same interlaces (sub-channels) can be used for initial transmission and reserved re-transmissions. And different types of candidate single-slot resource should be designed, which will largely complicate the resource selection procedure. [M] Proposal 3-1 Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings: - Use FRIVsubCH to indicate used sub-channel index(s) - FRIVsubCH is conveyed in 1st stage SCI - FRIVsubCH is calculated as below - If sl-MaxNumPerReserve is 2 then - FRIVsubCH=nsubCH,1start+i=1LsubCHRBset-1N subchannel RBset+1-i - If sl-MaxNumPerReserve is 3 then - FRIVsubCH=nsubCH,1start+nsubCH,2start⋅N subchannel RBset+1-LsubCHRBset+i=1LsubCHRBset-1N subchannel RBset+1-i2 - where - nsubCH,1start denotes the starting sub-channel index for the second resource in one RB set - nsubCH,2start denotes the starting sub-channel index for the third resource in one RB set - N subchannel RBset is the number of sub-channels in one RB set - LsubCHRBset is the number of used sub-channels within one RB set for one PSCCH/PSSCH transmission FFS: whether/how to use spare states of FRIVsubCH to indicate using non-contiguous interlaces |
| Xiaomi | Yes |  |
| NEC | YES |  |
| Spreadtrum | Yes |  |
| Sharp | Yes with comments | The proposal should be talking about the indicated *resources* rather than the *PSCCH/PSSCH transmission*. We propose to change the last sub-sub-bullet of the last but one bullet as follows, LsubCHRBset is the number of used sub-channels within one RB set for each of the indicated resources one PSCCH/PSSCH transmission. |
| Transsion | Yes |  |

#### [M] Proposal 3-2

Background: IRB, frequency domain resource indication (RB set indication)

- Summary: views on RB set indication

  - FRIV

    - Support (6): Huawei/HiSilicon, Qualcomm, OPPO, LGE, ZTE, Apple,

  - RIV

    - Support (2): ITL, Lenovo

- FL’s view

  - FL suggests to use FRIV to align with legacy NR SL design.

  - A proposal is given to reflect the above.

[M] Proposal 3-2

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings:

- Use FRIVRBset to indicate used RB set index(s)

- FRIVRBset is conveyed in 1st SCI

- FRIVRBset is calculated as below

  - If sl-MaxNumPerReserve is 2 then

    - FRIVRBset=nRBset,1start+i=1LRBset-1NRBset+1-i

  - If sl-MaxNumPerReserve is 3 then

    - FRIVRBset=nRBset,1start+nRBset,2start⋅NRBset+1-LRBset+i=1LRBset-1NRBset+1-i2

  - where

    - nRBset,1start denotes the starting RB set index for the second resource

    - nRBset,2start denotes the starting RB set index for the third resource

    - NRBset is the number of RB sets in a resource pool

    - LRBset is the number of used RB sets for one PSCCH/PSSCH transmission

| Company | Agree? | Comments |
| --- | --- | --- |
| Qualcomm | Ok |  |
| DCM | YES | “Different transmissions use different number of RB set(s) while keeping total number of sub-channels unchanged between initial transmission and retransmission(s) for a TB” should also be discussed. |
| OPPO | yes |  |
| LGE | OK |  |
| Panasonic | Yes |  |
| MediaTek | OK |  |
| CATT/GOHIGH | Yes |  |
| Xiaomi | Yes |  |
| NEC | Yes |  |
| Spreadtrum | Yes |  |
| Sharp | Yes with comments | LRBset is the number of used RB sets for each of the indicated resource one PSCCH/PSSCH transmission |
| Transsion | Yes |  |

#### [M] Proposal 3-3

Background: IRB, frequency domain resource indication (others)

- FL’s view

  - Considering one PSSCH transmission may occupy one or multiple RB sets, RAN1 needs to discuss whether/how to re-define single-slot candidate resource, and update resource selection and/or signaling from MAC to PHY, etc.

    - This is discussed by vivo, ZTE, CATT, CMCC, HW, Apple, Sharp, OPPO, etc.

  - A proposal is given to reflect the above.

(TS 38.214) ### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 In resource allocation mode 2, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this PSSCH/PSCCH transmission: - the resource pool from which the resources are to be reported; - L1 priority, prioTX; - the remaining packet delay budget; - the number of sub-channels to be used for the PSSCH/PSCCH transmission in a slot, LsubCH; - optionally, the resource reservation interval, Prsvp_TX, in units of msec. … The following steps are used: 1) A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot t'ySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing full sensing, in a set of Y candidate slots within the time interval n+T1,n+T2 correspond to one candidate single-slot resource for UE performing periodic-based partial sensing together with contiguous partial sensing and resource (re)selection triggered by periodic transmission (Prsvp_TX≠0), or in a set of Y' candidate slots within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource for UE performing at least contiguous partial sensing and resource (re)selection triggered by aperiodic transmission (Prsvp_TX=0), where …

[M] Proposal 3-3

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings:

- MAC layer indicates LRBset to PHY layer, where LRBset is the number of used RB sets for one PSCCH/PSSCH transmission

- Regarding LsubCH in TS 38.214 Clause 8.1.4, down-select one of the followings in RAN1#113:

  - Alt 1: LsubCH is re-defined as “the number of sub-channels within one RB set to be used for the PSSCH/PSCCH transmission in a slot”

    - Note: LsubCHRBset=LsubCH

  - Alt 2: LsubCH is re-defined as “the number of sub-channels within all used RB sets to be used for the PSSCH/PSCCH transmission in a slot”

    - Note: LsubCHRBset=LsubCH/LRBset

  - Note: LsubCHRBset is the number of used sub-channels within one RB set for one PSCCH/PSSCH transmission

- Regarding candidate single-slot resource for transmission Rx,y in TS 38.214 Clause 8.1.4:

  - Rx,y is replaced by Rr,x,y, Rr,x,y is defined as a set of LsubCHRBset contiguous sub-channels in LRBset contiguous RB sets, with sub-channel x+j in RB set r+h in slot t'ySL where j=0,...,LsubCH-1, h=0, …, LRBset-1.

  - FFS: how to re-define Rx,y if using non-contiguous interlaces is supported

FL’s comment: please also indicate you prefer Alt 1 or Alt 2 (FL assumes Alt 1 and 2 are equivalent technically, it’s just a matter of clarification).

| Company | Agree? | Comments |
| --- | --- | --- |
| Qualcomm | OK | Alt 1 is preferred since additional LsubCHRBset needs to be introduced anyway. |
| DCM |  | PHY should perform resource identification for any number of RB sets, and MAC can select appropriate resources. We are not sure whether this is included in this proposal. |
| vivo |  | For Alt 1, given that the sub-channel is defined per RB set, it seems the LsubCH may become larger than the maximum sub-channel number. Does it seem to be a problem? E.g., how to derive the SCI field size? From this perspective, we slightly prefer Alt 2, but we are basically open to consider either one. |
| OPPO |  | Another possibility is to reuse legacy mechanism as much as possible. MAC indicate Lsubch to PHY layer, and PHY layer can determine the single slot resource structure, i.e., whether all Lsubch are within one RB set (if possible), or Lsubch are equally divided into multiple contiguous RB sets. Based on the determined single slot structure, PHY can determine S-A and report to MAC layer. |
| LGE |  | Alt 3 would be needed. Similar with number of subchannels, high layer could provide number of RB sets to PHY layer. So, if the more than one RB sets are associated with SL grant generated from MAC layer, Alt 1 is not correct. Alt 3: LsubCH is determined by “the number of sub-channels” and “the number of RB sets” to be used for the PSSCH/PSCCH transmission in a slot |
| Panasonic | OK | We are open to consider this issue and we slightly prefer alt 2. |
| CATT/GOHIGH |  | From our understanding, we don’t see clear motivation why we re-define LsubCH, what we should do here is to determine LsubCHRBset which will be used to determine candidate single-slot resource. The legacy parameter (the number of sub-channels to be used for the PSSCH/PSCCH transmission in a slot, LsubCH) and the additional RB set number indicated to PHY layer is enough. Thus, the following updated proposal is provided as follows. [M] Proposal 3-3 Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings: - MAC layer indicates LRBset to PHY layer, where LRBset is the number of used RB sets for one PSCCH/PSSCH transmission - Regarding LsubCH in TS 38.214 Clause 8.1.4, down-select one of the followings in RAN1#113: - Alt 1: LsubCH is re-defined as “the number of sub-channels within one RB set to be used for the PSSCH/PSCCH transmission in a slot” - Note: LsubCHRBset=LsubCH - Alt 2: LsubCH is re-defined as “the number of sub-channels within all used RB sets to be used for the PSSCH/PSCCH transmission in a slot” - Note: LsubCHRBset=LsubCH/LRBset - Note: LsubCHRBset is the number of used sub-channels within one RB set for one PSCCH/PSSCH transmission - Regarding candidate single-slot resource for transmission Rx,y in TS 38.214 Clause 8.1.4: - Rx,y is replaced by Rr,x,y, Rr,x,y is defined as a set of LsubCHRBset contiguous sub-channels in LRBset contiguous RB sets, with sub-channel x+j in RB set r+h in slot t'ySL where j=0,...,LsubCHRBset-1, h=0, …, LRBset-1. - LsubCHRBset is the number of used sub-channels within one RB set for one PSCCH/PSSCH transmission, which is determined by LsubCHRBset=LsubCH/LRBset FFS: how to re-define Rx,y if using non-contiguous interlaces is supported |
| Xiaomi |  | We follow the majority view, and we think this proposal is not necessary. The current specification “the number of sub-channels to be used for the PSSCH/PSCCH transmission in a slot, LsubCH” is clear, which is obvious to indicate the number of sub-channels within all used RB sets to be used for the PSSCH/PSCCH transmission in a slot” ” |
| NEC | YES | We support Alt 2. Alt 2 seems more in line with R16 TS. For example, the higher layer indicated sub-channels number is based on per resource pool level. We expect less TS impact across PHY and MAC layer |
| Spreadtrum |  | We prefer Alt 2 that LsubCH is the number of sub-channels to be used for the PSSCH/PSCCH transmission in a slot within all used RB sets. |
| Sharp |  | It is not necessary to redefine the existing LsubCH in spec. Alt 2 is preferred. In addition, the following LsubCH should be changed to LsubCHRBset. - Rx,y is replaced by Rr,x,y, Rr,x,y is defined as a set of LsubCHRBset contiguous sub-channels in LRBset contiguous RB sets, with sub-channel x+j in RB set r+h in slot t'ySL where j=0,...,LsubCHLsubCHRBset-1, h=0, …, LRBset-1. |
| Transsion | Yes | Alt 1 is preferred. |

#### [M] Proposal 3-4

Background: IRB, TBS determination

- Summary: views on Option A1~A4

  - Option A1: N_ref is a fixed value, e.g., 10, 11

    - Support (10): Qualcomm, LGE, ZTE, Docomo, CATT, NOK, NEC, Panasonic, ITL, Transsion Holdings

  - Option A2: N_ref is (pre-)defined

    - Support (3): [Ericsson], OPPO, NOK

      - Ericsson: sl-SubchannelSize / K

      - OPPO: N_ref = ceil (NPRB/Ninterlace) , where NPRB is the number of PRBs within one RB set, Ninterlace is number of interlace

  - Option A3: N_ref is (pre-)configured

    - Support (7): vivo, Docomo, Apple, NOK, SPRD, NEC, CMCC,

  - Option A4: N_ref is dynamically indicated by Tx UE

    - Support (2): Huawei/HiSilicon, Lenovo,

- Summary: others

  - For TBS determination, whether/how to handle the impact of additional available PRB(s) in intra-cell guard band(s) for PSCCH/PSSCH transmission across multiple RB sets

    - The PRBs in guard band can be dynamically indicated in SCI for TB transmission occupying multiple RB set (1): ZTE

    - No special handling (4): Huawei/HiSilicon, [Ericsson], Docomo, NOK,

- FL’s view

  - FL suggests to go with Option A3 for simplicity.

  - FL suggests not to pursue other enhancements.

  - A proposal is given to reflect the above.

Agreement (in RAN1#112b-e) For interlace RB-based PSCCH/PSSCH transmission in SL-U, considering 1 sub-channel equals K interlace(s), support the followings: - Option A: - TBS is determined based on a reference number of PRBs of one interlace within 1 RB set (denoted as N_ref), down-select one of the followings in RAN1#113: - Option A1: N_ref is a fixed value, e.g., 10, 11 - Option A2: N_ref is (pre-)defined - e.g., N_ref is the average number of PRBs per interlace, which is determined by total number of PRBs of the RP divided by the number of interlaces. - Option A3: N_ref is (pre-)configured - Option A4: N_ref is dynamically indicated by Tx UE - Note: The number of PRBs within a sub-channel can be different among sub-channels in a single resource pool subject to (pre-)configuration. - FFS: for TBS determination, whether/how to handle the impact of additional available PRB(s) in intra-cell guard band(s) for PSCCH/PSSCH transmission across multiple RB sets

[M] Proposal 3-4

For interlace RB-based PSCCH/PSSCH transmission in SL-U, support the followings:

- Option A3: N_ref is 10 or 11 subject to (pre-)configuration

| Company | Agree? | Comments |
| --- | --- | --- |
| Qualcomm | No | It is not clear whether/how two UEs can align their pre-configuration. With all the possible RB-set configs, most of the interlaced subchannels would have 10RBs. A1 is preferred so that at least Tx and Rx UEs would have the same assumption during the initial message exchange. Suggest to keep A1 [M] Proposal 3-4 For interlace RB-based PSCCH/PSSCH transmission in SL-U, support the followings: - Option A1: N_ref is a fixed value, e.g., 10, 11 - Option A3: N_ref is 10 or 11 subject to (pre-)configuration |
| DCM | YES |  |
| OPPO |  | It is OK that N_ref is (pre-)configured. While the value range can be further discussed. According to 38.211, “The UE expects that the number of common resource blocks in an interlace contained within bandwidth part i is no less than 10.” In NR-U, the PRBs of one interlace corresponds to all the PRBs associated to the same interlace index within the BWP. For example, if BWP includes 2 RB set, the PRBs of one interlace includes all the PRBs with same interlace index of the 2 RB sets and also PRBs of intra-cell guard band. While here N_ref is the number of PRBs of one interlace within one RB set. If the SL BWP includes 2 RB set (e.g., 40MHz BW), intra-cell guard band can be configured and the number of PRB of GB is up to 15. In this case, the number of PRBs per RB set is about 45-46 (for SCS=30kHz). And the number of PRB per interlace within one RB set is about 9, instead of 10 or 11. Therefore, we suggest the following modification: [M] Proposal 3-4 For interlace RB-based PSCCH/PSSCH transmission in SL-U, support the followings: - Option A3: N_ref is 10 or 11 subject to (pre-)configuration - FFS value range of N_ref GuardBand-r16 ::= SEQUENCE { startCRB-r16 INTEGER (0..274), nrofCRBs-r16 INTEGER (0..15) } |
| LGE |  | We prefer to use Rel-16/17 NR SL subchannel size for N_ref value(s). In this case, 11 needs to be excluded. |
| Panasonic | No | We think (pre-)configuration is not necessary. Option A1 (fixed value) is enough. |
| MediaTek | Comment | We prefer a (pre-)defined value of N_ref to make all UEs have a common understanding on the size of sub-channel. A sub-channel size of 10 is preferred with consideration of legacy SL sub-channel size. |
| CATT/GOHIGH |  | Prefer option A1, but OK for progress |
| Xiaomi | Yes |  |
| NEC | YES | We are OK with A1 and A3 with (10 or 11). We think the current value is proper which can achieve same purpose of A1. |
| Spreadtrum | Yes |  |
| Transsion | Comment | Share the same view as QC and MTK. It is difficult to guarantee all SL UEs have common sense ont the size of sub-channel especially when out of coverage. |

#### [M] Proposal 3-5

Background: CRB, usage of sub-channel(s) which include intra-cell guardband PRBs

- Summary

  - Option 2: Such sub-channel(s) can be used for PSCCH/PSSCH transmission

    - Support (10): Ericsson, OPPO, ZTE, Apple, CATT, CMCC, Panasonic, WILUS, Huawei/HiSilicon, ETRI

  - Option 3: Such sub-channel(s) cannot be used for PSCCH transmission, and can be used for PSSCH transmission

    - Support (9): Qualcomm, vivo, LGE, ZTE, Huawei/HiSilicon, MTK, Apple, Sharp, SPRD,

- FL’s view

  - FL suggests to support both Option 2 and 3.

  - Some companies discussed the impacts on resource selection, PSCCH mapping, etc. (see QC, HW, Apple, CATT, Sharp). Since the input is limited and views are divergent. FL adds an FFS to reflect this and suggests companies bring more details later.

  - A proposal is given to reflect the above.

[M] Proposal 3-5

For contiguous RB-based PSCCH/PSSCH transmission in SL-U, regarding sub-channel(s) which include intra-cell guardband PRBs, support the followings:

- If Nleft >= NPSCCH

  - Option 2: Such sub-channel(s) can be used for PSCCH/PSSCH transmission

    - Note: PRBs within intra-cell guard band are not used for PSCCH transmission as per previous agreement

- Otherwise,

  - Option 3: Such sub-channel(s) cannot be used for PSCCH transmission, and can be used for PSSCH transmission

- FFS other details, e.g., impacts on resource selection, PSCCH mapping, etc.

- Note:

  - Nleft : the number of remaining PRBs of a sub-channel belonging to a RB set after excluding the PRBs belonging to intra-cell guardband

  - NPSCCH : the number of PRBs for PSCCH transmission

| Company | Agree? | Comments |
| --- | --- | --- |
| Qualcomm | OK |  |
| vivo | No | In R16, the orphan RBs (remaining PRB that less than one sub-channel) are not used. If the same principle is still reused, it would be very strange that a ‘partial’ sub-channel (due to the GB) is used, but the orphan RBs are never be used (even if the number of orphan RBs is larger …), which also makes the design and implementation very complicated. We prefer a unified approach. |
| OPPO |  | We need to differentiate single slot candidate resource is 1 sub-channel or more than one sub-channel. For the later case (>1 sub-channel), a single slot candidate resource corresponding to sub-chanel 2 and sub-channel 3 is valid resource even if the left PRB within sub-channel 3 is less than PSCCH PRBs. |
| LGE |  | It seems like that the position of PSCCH can be changed depending on the guard band PRBs within a subchannel. We prefer not to change PSCCH location across subchannels. If we change it, we also need to discuss further whether how to allow FDM between PSCCH and PSSCH DMRS, which is quite related to the remaining PRBs in a subchannel after excluding PSCCH PRBs. In this point of view, the 1st bullet needs to be changed into “If PRBs for PSCCH is overlapping with guradband PRBs”. Or, simply, we can go to Option 3. Actually, we see some potential problem on Option 2. It will cause small number of PRBs within a RB set are used. Then, since the small power will be allotted to the RB set, the energy detected by another node would be quite small, and then the PSCCH will suffer from high interference. Small power moderate to high power Small energy detected moderate to high energy detected |
| Panasonic |  | We prefer single solution with option 2 or option 3. |
| Mediatek | Comment | As commented by some other companies, Option 3 is preferred regarding UE complexity. |
| ETRI | OK | We are ok with the current proposal. But if we have to choose one of the two options (option 2 and option 3), we prefer option 2 with a slight modification. The modification is to support additional PSCCH symbol allocation when the remaining PRBs are insufficient for PSCCH transmissions (Nleft<NPSCCH). |
| CATT/GOHIGH | Comment | Even if the condition of Nleft >= NPSCCH is met, it doesn’t mean such sub-channel can be certaily used for PSSCH transmission. For example, the PRBs number that used for PSSCH transmission is very small and the coding rate is larger than 1. So the condition should be if Nleft is larger than or equal to the (pre-)configured threshold, then such sub-channels can be used for PSCCH/PSSCH transmission. Additionally, we share similar views as LGE, the PSCCH mapping should not be changed. |
| xiaomi |  | We prefer option 2. |
| NEC | ok |  |
| Spreadtrum |  | We prefer only option 3 to reduce the impact on PSCCH transmission. In option 2, it allows the partial sub-channel transmission of PSCCH/PSSCH. It’s quite different with the principle of legacy SL that the minimum unit in frequency domain of PSCCH/PSSCH is one sub-channel. |
| Sharp |  | Similarly to LGE, we also don’t think “floating” of (candidate) PSCCH locations across sub-channels as introduced by Option 2 should be allowed. We prefer to simply go with Option 3. |
| Transsion |  | We share the similar view as LGE. We prefer Option 3 |

#### Others

- Some other issues are mentioned:

  - Some companies propose to add more values for K (1 sub-channel = K interlace). While some other companies are against this.

  - Some companies propose other enhancements, e.g., supporting PSCCH locates in every RB set of corresponding PSSCH, enhancing TRIV, etc. While some other companies are against this.

- FL assumes such issues are not very urgent nor essential at this stage. Considering we already have so many essential issues to be resolved at this meeting, FL does not organize proposals on these issues for now.

If you have any other comments, please provide in box below.

| Company | Comments |
| --- | --- |

## Issue#4: PSFCH and SL-HARQ

### Round 1: Proposals and background

#### [H] Proposal 4-1

Background: PSFCH transmission for 15/30 kHz, OCB requirement

- Summary: view on alternatives

  - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

    - Support (9): Huawei/HiSilicon, [Qualcomm], Docomo, Nokia, [Sharp], InterDigital, Futurewei, NEC, ITL

  - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

    - Support (8): [Qualcomm], Ericsson, vivo, Samsung, LGE, CATT, Xiaomi, WILUS

  - Alt 3-2b: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set

    - Support (7): OPPO, ZTE, Apple, Lenovo, Panasonic, [Sharp], SPRD

- Summary: others

  - PRB-level cyclic shift hopping as in NR-U to reduce PAPR

    - Support (4): Qualcomm, Docomo, ZTE, WILUS

- FL’s view

  - On down-selection among three Alts

    - Many companies point out Alt 2-3a has two major issues:

      - Issue#1: in many cases, Alt 2-3a cannot reuse legacy NR SL PSSCH-PSFCH mapping (i.e., each candidate resource is mapped to orthogonal PRB for PSFCH, see TS 38.213 as below)

        - Note: RAN1#112b-e already agreed “R16 NR SL PSSCH-PSFCH mapping is reused as baseline, FFS details”.

      - Issue#2: Alt 2-3a almost cannot support groupcast option 2

    - E.g., the analysis of Alt 2-3a is shown in Table 1 below (take 30 kHz SCS, 1 RB set as an example).

      - Only when PSFCH periodicity P=1 slot (see row 1-4), legacy NR SL PSSCH-PSFCH mapping (i.e., each candidate resource is mapped to orthogonal PRB for PSFCH) can be reused. But since the PSFCH periodicity P is already 1 slot, it seems unable/difficult support “more than 1 PSFCH occasion(s) per PSCCH/PSSCH transmission”.

      - When PSFCH periodicity P > 1 slot (see row 5-12): legacy NR SL PSSCH-PSFCH mapping (i.e., each candidate resource is mapped to orthogonal PRB for PSFCH) cannot be reused, i.e., PSFCH for different PSCCH/PSSCH transmission needs to occupy same interlace and be differentiated by CS pair, which is not legacy way. Even RAN1 intends to use new PSSCH-PSFCH mapping, it still has some issues:

        - When the number of CS pair is small (e.g., 1,2,3 CS pair, see row 5,9,11): NperCandidatePSFCH is smaller than 1, it means even unicast cannot be supported!

        - Even when the number of CS pair is large (e.g., 6 CS pair, see row 12): NperCandidatePSFCH<2, it means max group size for groupcast option 2 is only 1, which is impractical.

        - In addition, larger number of CS pairs leads to higher intra-PRB interference.

      - Overall, it can be observed that for Alt 2-3a, the max group size for groupcast option 2 is very small, i.e., 1~6.

    - On the contrary, the other two Alt (Alt 1-1b, 3-2b) do not have the above issues since there can be many dedicated PRBs in these two Alt.

    - In short, it seems Alt 2-3a is more complicated and will cause more RAN1 workload than the other two Alts, since RAN1 may need to design new PSSCH-PSFCH mapping rule for Alt 2-3a.

    - Thus, FL suggests to drop Alt 2-3a, and down-select between Alt 1-1b and 3-2b.

  - On detailed design for Alt 1-1b, 3-2b

    - Companies provided some further details for Alt 1-1b, 3-2b, e.g.,

      - For Alt 1-1b, 3-2b: multiple dedicated PRBs are in the same interlace (e.g., Sharp, ZTE, etc.)

      - For Alt 3-2b: multiple dedicated PRBs and common PRBs are in the same interlace (e.g., ZTE, Apple, Panasonic, etc.)

      - For Alt 3-2b: introduce gap between dedicated PRBs and common PRBs (e.g., OPPO, ZTE, etc.)

      - For Alt 1-1b, 3-2b: use decreased power for common PRBs (e.g., Nokia, ZTE, etc.)

    - FL suggests to do down-selection first, and then discuss such details. So far, Alt 1-1b and 3-2b remains the same as last meeting. After down-selection, we can further discuss whether/how to update it.

  - On PAPR reduction

    - Only 4 companies discussed this issue, FL suggests to continue FFS it.

  - A proposal is given to reflect the above.

(Below is copied from TS 38.213) 16.3.0 UE procedure for transmitting PSFCH with control information … A UE is provided by sl-PSFCH-RB-Set a set of MPRB, setPSFCH PRBs in a resource pool for PSFCH transmission with HARQ-ACK information in a PRB of the resource pool. A UE can be provided by sl-PSFCH-Conflict-RB-Set a set of MPRB, setPSFCH PRBs in a resource pool for PSFCH transmission with conflict information in a PRB of the resource pool. A UE expects that different PRBs are (pre)configured for conflict information and HARQ-ACK information. For a number of Nsubch sub-channels for the resource pool, provided by sl-NumSub-channel, and a number of PSSCH slots associated with a PSFCH slot that is less than or equal to NPSSCHPSFCH, the UE allocates the i+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH, i+1+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH-1 PRBs from the MPRB, setPSFCH PRBs to slot i among the PSSCH slots associated with the PSFCH slot and sub-channel j, where Msubch, slotPSFCH=MPRB, setPSFCHNsubch⋅NPSSCHPSFCH, 0≤i<NPSSCHPSFCH, 0≤j<Nsubch, and the allocation starts in an ascending order of i and continues in an ascending order of j. The UE expects that MPRB, setPSFCH is a multiple of Nsubch∙NPSSCHPSFCH. …

Table 1: Analysis on Alt 2-3a (30 kHz, 1 RB set)

| Row | Num of interlace (Ninterlace) | Num of sub-channels (NsubCH) | PSFCH periodicity / slots (P) | Candidate resources within 1 PSFCH period (i.e., each sub-channel on each slot) (Ncandidate= NsubCH*P) | Num of CS pairs (NCS) | Num of PSFCH resource per candidate (NperCandidatePSFCH=Ninterlace*NCSNcandidate) Note: this is also max group size for groupcast opt 2 | Can reuse legacy NR SL PSSCH-PSFCH mapping? (i.e., each candidate resource is mapped to orthogonal PRB for PSFCH) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 5 | 1 | 5 | 1 | 1 | - Can reuse legacy. - Seems unable/difficult to support “more than 1 PSFCH occasion(s) per PSCCH/PSSCH transmission” since PSFCH periodicity is already 1. |
| 2 | 5 | 5 | 1 | 5 | 2 | 2 |  |
| 3 | 5 | 5 | 1 | 5 | 3 | 3 |  |
| 4 | 5 | 5 | 1 | 5 | 6 | 6 |  |
| 5 | 5 | 5 | 2 | 10 | 1 | 0.5 | - Cannot reuse legacy |
| 6 | 5 | 5 | 2 | 10 | 2 | 1 |  |
| 7 | 5 | 5 | 2 | 10 | 3 | 1.5 |  |
| 8 | 5 | 5 | 2 | 10 | 6 | 3 |  |
| 9 | 5 | 5 | 4 | 20 | 1 | 0.25 |  |
| 10 | 5 | 5 | 4 | 20 | 2 | 0.5 |  |
| 11 | 5 | 5 | 4 | 20 | 3 | 0.75 |  |
| 12 | 5 | 5 | 4 | 20 | 6 | 1.5 |  |

[H] Proposal 4-1

Regarding PSFCH transmission with 15 kHz and 30 kHz SCS:

- RAN1 down-select one of followings in RAN1#113:

  - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

    - K3 is (pre-)configured, FFS value range

    - On the K3 dedicated PRB(s), multiple CS pairs can be used as in legacy NR SL PSFCH transmission

    - When a PRB of common interlace and a dedicated PRB locate within the same 1 MHz bandwidth, UE only transmits on the dedicated PRB

      - FFS: whether any impact on meeting OCB requirement

  - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

  - Alt 3-2b: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set

    - K2=2

    - K4 is (pre-)configured, FFS value range

    - FFS: how to meet PSD limitation

    - FFS: whether to introduce any restrictions on the locations of K4 dedicated PRB(s) and/or K2 common PRBs, e.g., whether/how they are on the same interlace

- PSFCH collision is not supported, i.e., PSSCH transmissions on non-overlapped resources are mapped to orthogonal dedicated PRBs for PSFCH transmission as in R16 NR SL

- FFS: regardless of which Alt above is selected, whether or not to support PRB-level cyclic shift hopping as in NR-U to reduce PAPR

FL’s comment: Proponent of Alt 2-3a, please check FL’s view above and Table 1, to see whether any misunderstanding.

FL’s comment: Please also indicate which Alt you support to help down-selection.

| Company | Agree? | Comments |
| --- | --- | --- |
| Qualcomm | No | Interlaced PSFCH waveform within one full interlace is the legacy NR-U waveform (NR-U PUCCH format 0) and spec support could be simple. We don’t see an issue for Alt 2-3a supporting unicast which is the main use case for commercial SL-U. Only for the case in which we want to support GC option 2 with larger group size, Alt 1-1b can be configured for the network. We support unified framework based on Alt 1-1b and Alt 2-3a in which K3 is configurable by the network. If K3=10, it becomes Alt 2-3a. |
| DCM | YES | Support Alt 1-1b |
| vivo | No | As commend and evaluated by simulation, the capacity is not an issue for Alt 2-3a, even for the GC opt-2. We are also open to the PRB-level cycling shift if it is really a concern. We also do not agree that the 2-3a is more complicated. The existing mapping rule can be reused as much as possible (e.g., by firstly mapping PSSCH to PSFCH RB, and then mapping PSFCH RB to the associated interlace). On the other hand, the other alternatives also require additional works to handle the other issues , e.g., coverage degradation, PSD issue, etc. |
| LGE | No | In our contribution, we provide some MCL analysis to compare coverage of PSSCH and the corresponding PSFCH. Then, for the single PSFCH transmission, the number of dedicated PRBs for Alt 1-1b and Alt 3-2b needs to be more than 2 to achieve a coverage compatible with PSSCH. For the simultaneous PSFCH transmission (e.g. 4 PSFCH TXs), the number of dedicated PRBs for Alt 1-1b and Alt 3-2b needs to be at least 8 to achieve a coverage compatible with PSSCH. In this case, Alt 1-1b and Alt 3-2b also have cases where the legacy PSSCH-to-PSFCH mapping cannot be reused. In other words, all the alternatives needs to modify the PSSCH-to-PSFCH mapping especially for the sufficient coverage of PSFCH. If not, the coverage of PSFCH will be much smaller than that of PSSCH. Then, if the dedicated PRB is always small for Alt 1-1b and Alt 3-2b, the PSFCH transmission would not be effective, and our thinking is that the PSFCH with OCB requirement itself does not need to be supported since TX UE cannot receive the PSFCH reliably. Moreover, according to TS38.213, the total set of PSFCH resources could be extended by using PSFCH resource candidates associated with subchannels allocated for PSSCH rather than the lowest subchannel. For compromise, we have similar view with QC. At this moment, one way to go would be allowing all the possibility by adjusting value of the number of common PRBs and dedicated PRBs. |
| Panasonic | Yes | We support Alt 3-2b. |
| CATT/GOHIGH | No | According to FL’s analysis, the assumption is that alt 2-3a is not support PRB-level cycling shift, and then the capacity may be an issue. From our understanding, the current alt 2-3a is mostly aligned with previous agreement. We are open for the discussion on capacity enhancement on alt 2-3a. |
| Lenovo | Yes | Alt 3-2b is preferred. |
| xiaomi | No | We also think the capacity of Alt2-3a is sufficient with applying PRB-level cyclic shift as Rel-16 sidelink. And it can reuse the legacy PSSCH-PSFCH mapping rule as vivo mentioned. But Alt 1-1b and Alt 3-2b have the IBE problem, the power on common interlace or common PSFCH PRBs could be high as many UEs will simultaneously transmit on these common interlace or common PSFCH PRBs, which might bring interference to the useful transmission. |
| NEC | Yes | We support Alt 1-1b and we agree on the analysis from FL about the capacity issue for Alt 2-3a. |
| Spreadtrum | Yes | We support Alt 3-2b. |
| Sharp |  | There is also a risk that power within 1Mhz would be shared between K3 dedicated PRBs if K3 >1. We suggest adding FFS to consider K3 dedicated PRBs are in a same interlace. - When a PRB of common interlace and a dedicated PRB locate within the same 1 MHz bandwidth, UE only transmits on the dedicated PRB - FFS: whether any impact on meeting OCB requirement - FFS: K3>1 dedicated PRBs are on the same interlace |
| Transsion | No |  |

#### [M] Proposal 4-2

Background: More than 1 PSFCH occasion per PSCCH/PSSCH transmission

- Summary

  - Value of N

    - N={1, 2} (5): ZTE, Apple, Huawei/HiSilicon, MTK, Panasonic

    - N is (pre-)configured, Tx UE indicates N1<N (1): Ericsson

  - Locations of N candidate PSFCH occasion(s) for one PSCCH/PSSCH transmission:

    - They are in different slots of the same RB set(s) (TDM) (15): Qualcomm, vivo, Samsung, OPPO, LGE, Docomo, MTK, Apple, NOK, Sharp, SPRD, Huawei/HiSilicon, Xiaomi, InterDigital, Panasonic

    - They are in different RB sets of the same slot (FDM) (7): Qualcomm, OPPO, LGE, NOK, Sharp, SPRD, InterDigital

  - PSSCH transmission and its related PSFCH occasion(s)

    - They are in the same RB set(s) (6): Docomo, MTK, NOK, Sharp, Huawei/HiSilicon, Panasonic

    - They can be in different same RB set(s) (2): NOK, Sharp

- FL’s view

  - UE behaviour on transmitting PSFCH is discussed by several companies (e.g., Samsung, ZTE, Apple, NOK, Xiaomi, etc.). FL reflected this in proposal.

  - Docomo proposed that “PSFCH resources with a PSSCH resource are not overlapped in freq-domain with PSFCH resources associated with other PSSCH resources in the same occasion, by (pre-)configuration”. This seems aligned with legacy NR SL PSFCH principle and can simplify RAN1 design. So FL reflects this in proposal.

  - Companies provided followings enhancements related to HARQ RTT:

    - Update the minimum time gap Z=a+b between any two selected resources of a TB in case PSFCH is configured for this resource pool (e.g., ZTE, Apple, CATT, Xiaomi, etc.)

    - Update the reference slot n for PUCCH transmission to report HARQ in Mode 1 (e.g., QC, ZTE, etc.)

    - Considering the number of interested companies is limited, FL captured them in proposal for further study. Companies are encouraged to check and provide more input in next meeting.

  - A proposal is given to reflect the above.

[M] Proposal 4-2

Regarding one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s) via (pre-)configuration:

- Value range of N is {1, 2}

  - By default (if no (pre-)configuration), N=1

- PSFCH collision is not supported, i.e., N associated candidate PSFCH occasion(s) for one PSCCH/PSSCH transmission have different time and/or frequency resource with the candidate PSFCH occasion(s) for another PSCCH/PSSCH transmission, if these two PSCCH/PSSCH transmissions are on non-overlapped resources

- Regarding locations of candidate PSFCH occasion(s):

  - PSSCH transmission and its related PSFCH occasion(s) are in the same RB set(s)

  - For one PSCCH/PSSCH transmission, at least support that its associated candidate PSFCH occasion(s) are in different slots of the same RB set(s)

    - FFS: whether to support its associated candidate PSFCH occasion(s) are in different RB sets of the same slot

- Regarding UE behaviour on transmitting PSFCH:

  - For one PSCCH/PSSCH transmission, PSCCH/PSSCH receiver UE attempts to transmit PSFCH on a candidate PSFCH occasion if and only if it fails to transmit on previous PSFCH occasion(s) due to LBT failure

- Regarding HARQ RTT restriction:

  - Further study whether/how to update the followings:

    - The minimum time gap Z=a+b between any two selected resources of a TB in case PSFCH is configured for this resource pool

    - The reference slot n for PUCCH transmission to report HARQ in Mode 1

| Company | Agree? | Comments |
| --- | --- | --- |
| Qualcomm | OK |  |
| DCM | YES |  |
| vivo | Comments | - Some larger values of N are needed, otherwise, it is possible that all the PSFCH candidate are blocked by a same COT which makes the design of additional PSFCH occasion become meaningless. - The 2nd bullet is unclear. What does it mean of “not supported”? Does it mean such case should not be configured, or no additional mechanism should be defined for this case? |
| OPPO | No | We don’t think it is reasonable to limit the PSFCH resource and its associated PSSCH to the same RB set. In legacy NR SL, PSFCH resource and PSSCH are in the same resource pool. For a PSSCH tx, its associated PSFCH is indexed within the whole (pre-)configured PRBs and CS pairs for PSFCH of the RP. We think this mechanism can be reused in SL-U. the candidate resources can be configured within the RP. When determine the PSFCH resource, it is not necessary to limit PSFCH resource within a RB set, it can be within the RP. Therefore, we suggest to replace “in same RB set(s)” by “in same RP”. [M] Proposal 4-2 Regarding one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s) via (pre-)configuration: - Value range of N is {1, 2} - By default (if no (pre-)configuration), N=1 - PSFCH collision is not supported, i.e., N associated candidate PSFCH occasion(s) for one PSCCH/PSSCH transmission have different time and/or frequency resource with the candidate PSFCH occasion(s) for another PSCCH/PSSCH transmission, if these two PSCCH/PSSCH transmissions are on non-overlapped resources - Regarding locations of candidate PSFCH occasion(s): - PSSCH transmission and its related PSFCH occasion(s) are in the same RB set(s) RP - For one PSCCH/PSSCH transmission, at least support that its associated candidate PSFCH occasion(s) are in different slots of the same RP RB set(s) - FFS: whether to support its associated candidate PSFCH occasion(s) are in same/different RB sets of the same slot - Regarding UE behaviour on transmitting PSFCH: - For one PSCCH/PSSCH transmission, PSCCH/PSSCH receiver UE attempts to transmit PSFCH on a candidate PSFCH occasion if and only if it fails to transmit on previous PSFCH occasion(s) due to LBT failure - Regarding HARQ RTT restriction: - Further study whether/how to update the followings: - The minimum time gap Z=a+b between any two selected resources of a TB in case PSFCH is configured for this resource pool - The reference slot n for PUCCH transmission to report HARQ in Mode 1 |
| LGE |  | Regarding 4th bullet, “if and only if” needs to be replaced with “at least if”. We think that the additional PSFCH occasion can be used for the case when RX UE skips PSFCH transmission due to UL/SL prioritization. Or, simply, we can remove “due to LBT failure”. |
| Panasonic | Yes |  |
| MediaTek | Comment | For the 4th bullet on regarding UE behaviour on transmitting PSFCH, to prevent COT losing, the behavior of receiver UE on the later candidate PSFCH occasion should not be limited. In this case, a later candidate PSFCH occasion can still be used by the receiver UE (e.g., with a duplication of PSFCH transmission) even the PSFCH transmission on the previous PSFCH occasion is successful. |
| Fraunhofer | Yes |  |
| Lenovo | Yes with comments | Suggest adding value of 4 in the range of N. |
| xiaomi | comment | We are generally fine this proposal. For the third bullet, even the legacy sidelink PSSCH-PSFCH is reused, the details of mapping rule is not clear, we think it is too early to limit the PSSCH transmission and its related PSFCH occasion(s) are in the same RB set(s), so we support that PSSCH transmission and its related PSFCH occasion(s) are in the same RP as oppo’s proposed. |
| Spreadtrum | Yes |  |
| Sharp |  | The primary motivation of configuring multiple PSFCH occasions is to address LBT failure. Once RB set(s) of PSSCH transmission is occupied by other UEs, additional PSFCH occasion configured in different RB set is also beneficial to address LBT failure in RB set(s) of PSSCH transmission. Therefore, at least for additional PSFCH occasion, we do not see the necessity to limit additional PSFCH occasions to be in same RB set(s) with PSSCH transmission. |
| Transsion | Yes |  |

#### [M] Proposal 4-3

Background: COT interruption by periodic PSFCH occasions

- Summary

  - Views on previous FFS “whether to support that COT initiating UE can dynamically indicate which subset of the (pre-)configured PSFCH occasions within its COT are available for PSFCH transmissions. Other associated candidate PSFCH occasion(s) within its COT are used for PSSCH transmissions”

    - Support (11): vivo, MTK, Futurewei, NEC, [Sony], Lenovo, InterDigital, [WILUS], [Fraunhofer] , Huawei/HiSilicon, Transsion Holdings

    - Not Support (8): OPPO, [LGE], Docomo, ZTE, CATT, [SPRD], Xiaomi, ITL

  - Views on previous FFS “how to avoid the risk of losing the COT if the COT is interrupted by periodic PSFCH occasions”

    - Transmit some signal (3): Qualcomm (PSFCH-like padding signal), ZTE (channel occupy signal), MTK (pre-defined sequence)

- FL’s view

  - The above two FFSs intends to address similar issue, i.e., when neither COT initiating UE nor responding UE intends to transmit PSFCH on some PSFCH occasion(s) within a COT (e.g., broadcast, blind retransmissions, etc.), the COT will be interrupted by such periodic PSFCH occasion(s).

  - A proposal is given to reflect the above.

[M] Proposal 4-3

When neither COT initiating UE nor responding UE intends to transmit PSFCH on some PSFCH occasion(s) within a COT, to avoid COT interruption, selects one or both of the followings:

- Option 1: COT initiating UE or responding UE transmits PSSCH on such PSFCH occasion(s)

  - FFS details, e.g., how PSSCH Rx UE knows such transmission, etc.

- Option 2: COT initiating UE or responding UE transmits a PSFCH-like sequence on such PSFCH occasion(s)

  - FFS details, e.g., signaling design, etc.

| Company | Agree? | Comments |
| --- | --- | --- |
| Qualcomm | OK | Transmitting PSSCH (as in Option 1) or other reference signal to fill the PSFCH gap risks the chance of causing interference to other links’ PSFCH reception (outside of ED range). Other link outside of ED range (SNR~=20dB away) may still clear the LBT and have their A/N transmitted in the PSFCH gap. Hence, the PSFCH gap padding signal should be PSFCH waveform compatible. We can either assign a common and dedicated PSFCH resource for PSFCH padding signal or choose some unused resourced based on the Tx UE’s ID. |
| DCM | YES only with Option 2 | By defining dedicated resource, Option 2 does not lead to conflict issue among UEs due to hidden-node issue, which occurs in Option 1. |
| OPPO | No | We agree that period PSFCH resource may cause COT interruption. While in our view, it is not essential issue, it is optimization issue. Even the COT is interrupted, the system will not break down. We need to deprioritized this proposal. |
| LGE | OK | We prefer Option 2. PSSCH transmission can be collided with other UE’s PSFCH transmission. |
| MediaTek | OK |  |
| Fraunhofer | OK | We prefer Option 2. |
| CATT/GOHIGH | Comment | We are not sure the impacts of other UE’s PSFCH performance. and we prefer at least support an alternative without such enhancement. |
| Lenovo | Yes | Option 1 is preferred as at least 4 symbols (two PSFCH symbols and two gap symbols around PSFCH symbols) can be used for data transmission. In that sense, the spectrum efficiency is quite higher compared with Option 2. |
| Xiaomi | No | We also think this is an optimization issue and shall be deprioritized. Even the some signal are transmitted in the PSFCH occasion to keep the COT, the LBT might be failure when the responding UE shares the COT, and the COT also be interrupted. |
| Spreadtrum | Yes with comments | We are open to discuss this optimization issue. |
| Sharp | OK | Option 2 is preferred. Same view with LGE that option 1 may lead to PSSCH transmission to be collided with other UE’s PSFCH transmission. |
| Transsion | Yes |  |

#### Others

- Some other issues are mentioned:

  - Some companies propose to introduce new PSFCH format, PSFCH prioritization, etc.

- FL assumes such issues are not very urgent nor essential. Considering we already have so many essential issues to be resolved at this meeting, FL does not organize proposals on these issues for now.

If you have any other comments, please provide in box below.

| Company | Comments |
| --- | --- |

## Issue#5: S-SSB and synchronization

### Round 1: Proposals and background

#### [H] Proposal 5-1

Background: S-SSB transmission, OCB requirement

- Summary

  - Views on repetition number N, gap value, location of S-SSB repetitions:

    - N=2, at two edges of an RB set (5): Qualcomm, ZTE, Apple, Sharp, Transsion Holdings

    - Gap is (pre-)configured, N is derived by gap implicitly: Huawei/HiSilicon, [OPPO]

    - Gap is (pre-)configured, N is (pre-)configured: vivo, [LGE]

    - Gap and N is (pre-)configured or pre-defined: CATT, [Panasonic]

    - N=8/4/2 for 15/30/60kHz, gap=2 RB: MTK

    - Support a (pre-)configuration of a list of frequency locations (e.g., sl-AbsoluteFrequencySSB-List) for S-SSB: Samsung

  - PAPR reduction

    - Support (5): Qualcomm, [Huawei/HiSilicon], [vivo], [ZTE], MTK

- FL’s view

  - On repetition number N, gap value, location of S-SSB repetitions:

    - FL provided 3 Alts below for down-selection.

    - Technically, it seems if ARFCN is given and Gap is known, UE can derive the location and number of other S-SSB repetitions implicitly.

      - Example#1: assume center of S-SSB configured by ARFCN is in PRB k, and Gap is N PRBs, then UE can know center of other S-SSB repetitions are in k+N, k-N, k+2N, k-2N, …, until it exceeds current RB set.

      - So it’s not very clear whether Alt 3 below is needed. Proponent of Alt 3 can explain.

  - On PAPR reduction

    - QC, MTK, etc. gave detailed evaluation on PAPR and propose to reduce PAPR. FL reflects this in proposal.

  - A proposal is given to reflect the above.

[H] Proposal 5-1

Regarding “Option 3-1(revised): Transmit legacy S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement”:

- Down-select one of the followings in RAN1#113

  - Alt 1: N=2, the two repetitions locate at two edges of an RB set

  - Alt 2: the value of gap is (pre-)configured, and the value of N is derived implicitly

    - FFS: value range for gap

  - [ Alt 3: the value of gap is (pre-)configured, and the value of N is (pre-)configured ]

    - FFS: value range for gap and N

- Support reducing PAPR of S-SSB transmission, FFS details

FL’s comment: Please indicate which Alt you support.

FL’s comment: Proponent of Alt 3, please explain why Alt 2 is not enough (see Example#1 above).

| Company | Agree? | Comments |
| --- | --- | --- |
| Qualcomm | Yes | We support Alt 1 and reducing the PAPR of S-SSB transmission |
| LGE |  | According to our MCL analysis, the S-SSB coverage needs to be sufficiently large to cover the coverage of other SL channels/signals. In this case, the gap of 0 also needs to be included. In other words, Alt 3 including the gap of 0 or only gap of 0 needs to be supported. |
| Panasonic | Yes | We support Alt 2. Depending on the sl-AbsoluteFrequencySSB, whether two repetition is necessary or not would be different. |
| MediaTek | Comment | For the configuration on transmission time N and gap length L, a high transmission power of synchronization signal and a unified configuration on gap length for all SCSs are desirable to achieve a comparable coverage range as that in legacy SL, and a simplified UE implementation, respectively. As analyzed in our paper, a unified gap length of 2 RB can be applied for all SCSs of 15/30/60 KHz (if supported) with the transmission time N equals to 8/4/2 respectively. Therefore, we suggest the following update regarding Alt 2. - Alt 2: the value of gap is (pre-)configured, and the value of N is derived implicitly - FFS: value range for gap (e.g., 2) |
| Lenovo | Yes | Alt 1 is supported. |
| NEC | Yes | Alt.2 |
| Sharp | Yes | Alt 1 is preferred. |
| Transsion | Yes | Alt 1 is preferred. |

#### [M] Proposal 5-2

Background: number/location of additional candidate S-SSB occasions

- Summary

  - Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS

    - Support (6): Docomo, OPPO, Spreadtrum, Samsung, ZTE, Panasonic

  - Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion, and the gap between them is (pre-)configured

    - Support (6): Huawei/HiSilicon, CATT, Apple, MediaTek, vivo, NEC

  - Option 3: The number and location(s) of additional candidate S-SSB occasions are separately (pre-)configured

    - Support (2): Nokia, vivo

  - Option 4: Introduce M contiguous candidate S-SSB occasions in one S-SSB period

    - Support (2): Qualcomm, Xiaomi

  - Option 5: the number of candidate S-SSB occasions is (pre-)configured, and locations are determined based on the (pre-)configured number

    - Support (2): Xiaomi, WILUS

- FL’s view

  - As mentioned by some companies (e.g., MTK, CATT, NOK, NEC, etc.), Tx UE behaviour on legacy S-SSB occasions and additional S-SSB occasions are different (see below).

    - On legacy S-SSB occasions: when to transmit S-SSB are specified in R16 NR SL synchronization procedure.

    - On additional S-SSB occasions: RAN1#112b-e made a conclusion that “…UE can attempt to transmit on additional candidate S-SSB occasion(s) regardless of whether or not it transmit-ted on R16/R17 S-SSB occasion(s).”

  - So locations of legacy S-SSB occasions and additional S-SSB occasions shall be differentiated so that Tx/Rx UE can behave accordingly. It seems Option 1 cannot differentiate legacy S-SSB occasions and additional S-SSB occasions. FL adds bracket to Option 1, and proponent of Option 1 can explain and suggest updates for Option 1.

  - A proposal is given to reflect the above.

Conclusion (RAN1#112b-e) Regarding additional candidate S-SSB occasions, in the same S-SSB period, UE can attempt to transmit on additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s). … (below is copied from TS 38.331) SL-SSB-TimeAllocation-r16 ::= SEQUENCE { sl-NumSSB-WithinPeriod-r16 ENUMERATED {n1, n2, n4, n8, n16, n32, n64} OPTIONAL, -- Need R sl-TimeOffsetSSB-r16 INTEGER (0..1279) OPTIONAL, -- Need R sl-TimeInterval-r16 INTEGER (0..639) OPTIONAL -- Need R } …

[M] Proposal 5-2

Regarding the number and location(s) of additional candidate S-SSB occasions, down-select one of the followings in RAN1#113:

- [ Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS ]

- Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion(s) in different time slot(s), and the gap between them is (pre-)configured

  - FFS details, e.g., value of K, details on gap length (including possibility of being 0), etc.

FL’s comment: Proponent of Option 1, please check FL’s view above. It seems Option 1 is not aligned with last meeting’s conclusion.

| Company | Agree? | Comments |
| --- | --- | --- |
| Qualcomm | Ok | We support option 2. Option 1 needs to transmit S-SSB in all S-SSB occasions (which is a legacy behavior) and the overhead is huge. |
| DCM | YES | For Option 1, one additional parameter to indicate which occasion is additional can be introduced. |
| LGE | OK |  |
| Panasonic | Yes | We supported option1. However, if Tx UE behaviour on legacy S-SSB occasions and additional S-SSB occasions should be different, we can accept option 2. |
| MediaTek | OK | Option 2 is more flexible than Option 1 and provides the opportunity to differentiate the legacy S-SSB occasion(s) and additional candidate S-SSB occasion(s). |
| CATT/GOHIGH | Yes | Option 2 |
| Lenovo | Ok | Option 1 is supported. If it is agreed to differentiate between the legacy S-SSB occasion(s) and additional S-SSB occasion(s), the differentiation can be done by applying a new parameter indicating occasion type. |
| Xiaomi | OK |  |
| NEC | YES | And we support option 2. |
| Spreadtrum | Yes |  |
| Sharp | OK |  |
| Transsion | Yes |  |

#### [H] Proposal 5-3

Background: more than one S-SSB on multiple RB sets

- Summary

  - UE may transmit S-SSB repetition in more than one RB set

    - Support (12): Qualcomm, Ericsson, vivo, Samsung, [Docomo], [MTK], CATT, Spreadtrum, InterDigital, Huawei/HiSilicon, Panasonic, [OPPO]

    - Not support (3): ZTE, LGE, NOK

- FL’s view

  - Based on the situation, FL suggests to support this. RAN1 can discuss further details later.

  - On PSBCH-RSRP fluctuation: QC, OPPO, MTK proposed some solution to avoid this. FL reflects this in proposal.

  - Note that SL-U Channel Access AI is waiting for this to determine the multi-channel access procedure for S-SSB.

  - A proposal is given to reflect the above.

SL-FreqConfigCommon-r16 ::=      SEQUENCE {

sl-SCS-SpecificCarrierList-r16   SEQUENCE (SIZE (1..maxSCSs)) OF SCS-SpecificCarrier,

sl-AbsoluteFrequencyPointA-r16   ARFCN-ValueNR,

sl-AbsoluteFrequencySSB-r16      ARFCN-ValueNR                                                       OPTIONAL, -- Need R

frequencyShift7p5khzSL-r16       ENUMERATED {true}                                                   OPTIONAL, -- Cond V2X-SL-Shared

valueN-r16                       INTEGER (-1..1),

sl-BWP-List-r16                  SEQUENCE (SIZE (1..maxNrofSL-BWPs-r16)) OF SL-BWP-ConfigCommon-r16  OPTIONAL, -- Need R

sl-SyncPriority-r16              ENUMERATED {gnss, gnbEnb}                                           OPTIONAL, -- Need R

sl-NbAsSync-r16                  BOOLEAN                                                             OPTIONAL, -- Need R

sl-SyncConfigList-r16            SL-SyncConfigList-r16                                               OPTIONAL, -- Need R

...

}

[H] Proposal 5-3

When the SL-BWP contains multiple RB sets, support the followings:

- When UE attempts to transmit S-SSB in a S-SSB occasion (e.g., R16/17 S-SSB occasion, R18 additional candidate S-SSB occasion)

  - UE may transmit S-SSB repetition in more than one RB set

    - At least the power for S-SSB transmission on anchor RB set is independent from the number of RB sets

      - FFS details, e.g., whether the power for S-SSB transmission on other RB set(s) is also independent from the number of RB sets

    - Locations of S-SSB repetitions in each RB set are the same as the locations of S-SSB repetitions in the anchor RB set

    - FFS: the relationship with UE’s COT

    - Note: anchor RB set refers to the RB set where S-SSB indicated by sl-AbsoluteFrequencySSB-r16 locates

    - Note: whether UE can transmit S-SSBs over non-contiguous RB sets is subject to RAN4’s reply, details can be found in RAN1’s LS to RAN4 in R1-2304218

| Company | Agree? | Comments |
| --- | --- | --- |
| Qualcomm | Ok |  |
| DCM |  | Intention of the sub-sub-bullets is unclear. Clarification is necessary first. |
| vivo |  | We don’t think the concept of anchor RB set is needed, and we don’t think special handling for this RB set is needed, either. Having different tx power for different S-SSB may complicate the implementation and may not be desirable from PAPR perspective. |
| LGE | No | On the 1st sub-sub-bullet, it seems that our concern is not resolved yet. In my reading the relevant contribution, they suggest to fix the transmit power of S-SSB within one RB set regardless of the number of RB sets. Why don’t we just say that at least when the TX power is limited, S-SSB repetition over multiple RB sets is not supported. When the power is not limited, it is FFS whether S-SSB repetition over multiple RB sets is supported or not. |
| Panasonic | Yes |  |
| MediaTek | Ok |  |
| CATT/GOHIGH | No | We share similar views as vivo, we prefer no different handing on the S-SSBs of different RB sets. |
| Lenovo | Ok |  |
| Xiaomi | OK |  |
| NEC | OK |  |
| Spreadtrum | Yes |  |
| Sharp |  | We are still not convinced that this is needed. On the proposal itself, in our view at least the network should be able to control the RB set(s) where transmission of S-SSB is allowed, e.g. when this is (pre-) configured to be allowed in only one RB set, the UE falls back to legacy SL, i.e. transmission of multiple S-SSB in the frequency domain is disabled. |
| Transsion | No | We share the same view as LGE. |

#### Others

So far, no other critical or essential issues are identified. If you have any other comments, please provide in box below.

| Company | Comments |
| --- | --- |

## Issue#6: Others

### Round 1: Proposals and background

#### [L] Proposal 6-1

Background: power control updates due to PSD limit

- Summary

  - Updates on power control are needed (4): vivo, Apple, MediaTek, OPPO

  - Updates on power control are not needed (1): ZTE

- FL’s view

  - The number of interested companies is limited. FL provides two Alt below to collect companies’ views.

  - A proposal is given to reflect the above.

[L] Proposal 6-1

Considering PSD limit in unlicensed spectrum regulation, down-select one of followings:

- Alt 1: RAN1 will update power control formulas of PSCCH/PSSCH/S-SSB/PSFCH by taking PSD limit into account

- Alt 2: RAN1 does not pursue specific enhancements on power control formulas of PSCCH/PSSCH/S-SSB/PSFCH to reflect PSD limit, i.e., RAN1 assumes PSD limit can be satisfied by other ways, e.g., UE implementation, etc.

FL’s comment: Please also indicate which Alt you support to help down-selection.

| Company | Agree? | Comments |
| --- | --- | --- |
| Qualcomm | Ok | We prefer Alt 2. In NR-U, no enhancement on power control is introduced. The Tx UE by UE implementation may adjust it Tx power to fulfill the regulation |
| DCM | YES | Alt 2 |
| vivo | comments | Some clarifications on alt 2 are needed. The problem seems not only to the power control aspect, but also to the PSFCH prioritization procedure. If by Alt 2 it is up to UE implementation to handle this issue, does it mean up to UE to drop which/how many PSFCHs to meet the PSD requirement? |
| LGE |  | At least we need to discuss whether the PSD requirement will further reduce the number of simultaneous PSFCH transmissions in a slot or each PSFCH TX power can be further reduced. |
| Panasonic | Yes | We prefer Alt 2. |
| MediaTek | Comments | For NR-U, the power of UE is somehow controlled by gNB, and the PSD requirement can be satisfied by the scheduling of gNB. But for SL-U, it is confused how to guarantee the PSD requirement is respected if it is left to UE implementation especially for CRB based transmission. |
| Fraunhofer | Ok |  |

#### Others

- On 60kHz SCS:

  - Positive on supporting 60kHz SCS (6): OPPO, Huawei/HiSilicon, Apple, Futurewei, Lenovo, [ZTE]

  - Deprioritize 60kHz SCS (2): Samsung, ITL

  - FL’s view: Some companies support 60 kHz SCS and give related design (e.g., for PSFCH, S-SSB). However, as discussed in previous meetings, let’s first focus on 15/30kHz SCS, and come back to 60 kHz SCS later if time allows. Companies are encouraged to check and provide simple/unified design for 60 kHz SCS.

- Others

  - Some other issues are mentioned, e.g., congestion control, CBR measurement, etc.

  - FL assumes such issues are not very urgent nor essential at this stage. Considering we already have so many essential issues to be resolved at this meeting, FL does not organize proposals on these issues for now.

If you have any other comments, please provide in box below.

| Company | Comments |
| --- | --- |

# Collection of Proposals

## [Closed] Proposals for Round 1 discussions

FL’s comment: To help delegates have an overview, the following is a collection of Proposals for Round 1 discussions. Based on input/offline/etc., the proposals and order may be updated before online session.

FL’s comment: Please provide input in the above box for each proposal.

[H] Proposal 4-1

Regarding PSFCH transmission with 15 kHz and 30 kHz SCS:

- RAN1 down-select one of followings in RAN1#113:

  - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

    - K3 is (pre-)configured, FFS value range

    - On the K3 dedicated PRB(s), multiple CS pairs can be used as in legacy NR SL PSFCH transmission

    - When a PRB of common interlace and a dedicated PRB locate within the same 1 MHz bandwidth, UE only transmits on the dedicated PRB

      - FFS: whether any impact on meeting OCB requirement

  - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

  - Alt 3-2b: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set

    - K2=2

    - K4 is (pre-)configured, FFS value range

    - FFS: how to meet PSD limitation

    - FFS: whether to introduce any restrictions on the locations of K4 dedicated PRB(s) and/or K2 common PRBs, e.g., whether/how they are on the same interlace

- PSFCH collision is not supported, i.e., PSSCH transmissions on non-overlapped resources are mapped to orthogonal dedicated PRBs for PSFCH transmission as in R16 NR SL

- FFS: regardless of which Alt above is selected, whether or not to support PRB-level cyclic shift hopping as in NR-U to reduce PAPR

[H] Proposal 5-3

When the SL-BWP contains multiple RB sets, support the followings:

- When UE attempts to transmit S-SSB in a S-SSB occasion (e.g., R16/17 S-SSB occasion, R18 additional candidate S-SSB occasion)

  - UE may transmit S-SSB repetition in more than one RB set

    - At least the power for S-SSB transmission on anchor RB set is independent from the number of RB sets

      - FFS details, e.g., whether the power for S-SSB transmission on other RB set(s) is also independent from the number of RB sets

    - Locations of S-SSB repetitions in each RB set are the same as the locations of S-SSB repetitions in the anchor RB set

    - FFS: the relationship with UE’s COT

    - Note: anchor RB set refers to the RB set where S-SSB indicated by sl-AbsoluteFrequencySSB-r16 locates

    - Note: whether UE can transmit S-SSBs over non-contiguous RB sets is subject to RAN4’s reply, details can be found in RAN1’s LS to RAN4 in R1-2304218

[H] Proposal 5-1

Regarding “Option 3-1(revised): Transmit legacy S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement”:

- Down-select one of the followings in RAN1#113

  - Alt 1: N=2, the two repetitions locate at two edges of an RB set

  - Alt 2: the value of gap is (pre-)configured, and the value of N is derived implicitly

    - FFS: value range for gap

  - [ Alt 3: the value of gap is (pre-)configured, and the value of N is (pre-)configured ]

    - FFS: value range for gap and N

- Support reducing PAPR of S-SSB transmission, FFS details

[M] Proposal 5-2

Regarding the number and location(s) of additional candidate S-SSB occasions, down-select one of the followings in RAN1#113:

- [ Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS ]

- Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion(s) in different time slot(s), and the gap between them is (pre-)configured

  - FFS details, e.g., value of K, details on gap length (including possibility of being 0), etc.

[M] Proposal 4-2

Regarding one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s) via (pre-)configuration:

- Value range of N is {1, 2}

  - By default (if no (pre-)configuration), N=1

- PSFCH collision is not supported, i.e., N associated candidate PSFCH occasion(s) for one PSCCH/PSSCH transmission have different time and/or frequency resource with the candidate PSFCH occasion(s) for another PSCCH/PSSCH transmission, if these two PSCCH/PSSCH transmissions are on non-overlapped resources

- Regarding locations of candidate PSFCH occasion(s):

  - PSSCH transmission and its related PSFCH occasion(s) are in the same RB set(s)

  - For one PSCCH/PSSCH transmission, at least support that its associated candidate PSFCH occasion(s) are in different slots of the same RB set(s)

    - FFS: whether to support its associated candidate PSFCH occasion(s) are in different RB sets of the same slot

- Regarding UE behaviour on transmitting PSFCH:

  - For one PSCCH/PSSCH transmission, PSCCH/PSSCH receiver UE attempts to transmit PSFCH on a candidate PSFCH occasion if and only if it fails to transmit on previous PSFCH occasion(s) due to LBT failure

- Regarding HARQ RTT restriction:

  - Further study whether/how to update the followings:

    - The minimum time gap Z=a+b between any two selected resources of a TB in case PSFCH is configured for this resource pool

    - The reference slot n for PUCCH transmission to report HARQ in Mode 1

[M] Proposal 4-3

When neither COT initiating UE nor responding UE intends to transmit PSFCH on some PSFCH occasion(s) within a COT, to avoid COT interruption, selects one or both of the followings:

- Option 1: COT initiating UE or responding UE transmits PSSCH on such PSFCH occasion(s)

  - FFS details, e.g., how PSSCH Rx UE knows such transmission, etc.

- Option 2: COT initiating UE or responding UE transmits a PSFCH-like sequence on such PSFCH occasion(s)

  - FFS details, e.g., signaling design, etc.

[M] Proposal 2-1

If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, in TS 38.214 Clause 8.1.3.2:

- L_ref replaces sl-LengthSymbols

  - Value range of L_ref is {7, 8, 9, 10, 11, 12, 13, 14} symbols

- NsymbPSFCH is determined in the same way as in legacy NR SL

[M] Proposal 3-1

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings:

- Use FRIVsubCH to indicate used sub-channel index(s)

- FRIVsubCH is conveyed in 1st stage SCI

- FRIVsubCH is calculated as below

  - If sl-MaxNumPerReserve is 2 then

    - FRIVsubCH=nsubCH,1start+i=1LsubCHRBset-1N subchannel RBset+1-i

  - If sl-MaxNumPerReserve is 3 then

    - FRIVsubCH=nsubCH,1start+nsubCH,2start⋅N subchannel RBset+1-LsubCHRBset+i=1LsubCHRBset-1N subchannel RBset+1-i2

  - where

    - nsubCH,1start denotes the starting sub-channel index for the second resource

    - nsubCH,2start denotes the starting sub-channel index for the third resource

    - N subchannel RBset is the number of sub-channels in one RB set

    - LsubCHRBset is the number of used sub-channels within one RB set for one PSCCH/PSSCH transmission

- FFS: whether/how to use spare states of FRIVsubCH to indicate using non-contiguous interlaces

[M] Proposal 3-2

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings:

- Use FRIVRBset to indicate used RB set index(s)

- FRIVRBset is conveyed in 1st SCI

- FRIVRBset is calculated as below

  - If sl-MaxNumPerReserve is 2 then

    - FRIVRBset=nRBset,1start+i=1LRBset-1NRBset+1-i

  - If sl-MaxNumPerReserve is 3 then

    - FRIVRBset=nRBset,1start+nRBset,2start⋅NRBset+1-LRBset+i=1LRBset-1NRBset+1-i2

  - where

    - nRBset,1start denotes the starting RB set index for the second resource

    - nRBset,2start denotes the starting RB set index for the third resource

    - NRBset is the number of RB sets in a resource pool

    - LRBset is the number of used RB sets for one PSCCH/PSSCH transmission

[M] Proposal 3-3

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings:

- MAC layer indicates LRBset to PHY layer, where LRBset is the number of used RB sets for one PSCCH/PSSCH transmission

- Regarding LsubCH in TS 38.214 Clause 8.1.4, down-select one of the followings in RAN1#113:

  - Alt 1: LsubCH is re-defined as “the number of sub-channels within one RB set to be used for the PSSCH/PSCCH transmission in a slot”

    - Note: LsubCHRBset=LsubCH

  - Alt 2: LsubCH is re-defined as “the number of sub-channels within all used RB sets to be used for the PSSCH/PSCCH transmission in a slot”

    - Note: LsubCHRBset=LsubCH/LRBset

  - Note: LsubCHRBset is the number of used sub-channels within one RB set for one PSCCH/PSSCH transmission

- Regarding candidate single-slot resource for transmission Rx,y in TS 38.214 Clause 8.1.4:

  - Rx,y is replaced by Rr,x,y, Rr,x,y is defined as a set of LsubCHRBset contiguous sub-channels in LRBset contiguous RB sets, with sub-channel x+j in RB set r+h in slot t'ySL where j=0,...,LsubCH-1, h=0, …, LRBset-1.

  - FFS: how to re-define Rx,y if using non-contiguous interlaces is supported

[M] Proposal 3-4

For interlace RB-based PSCCH/PSSCH transmission in SL-U, support the followings:

- Option A3: N_ref is 10 or 11 subject to (pre-)configuration

[M] Proposal 3-5

For contiguous RB-based PSCCH/PSSCH transmission in SL-U, regarding sub-channel(s) which include intra-cell guardband PRBs, support the followings:

- If Nleft >= NPSCCH

  - Option 2: Such sub-channel(s) can be used for PSCCH/PSSCH transmission

    - Note: PRBs within intra-cell guard band are not used for PSCCH transmission as per previous agreement

- Otherwise,

  - Option 3: Such sub-channel(s) cannot be used for PSCCH transmission, and can be used for PSSCH transmission

- FFS other details, e.g., impacts on resource selection, PSCCH mapping, etc.

- Note:

  - Nleft : the number of remaining PRBs of a sub-channel belonging to a RB set after excluding the PRBs belonging to intra-cell guardband

  - NPSCCH : the number of PRBs for PSCCH transmission

[M] Proposal 2-2

For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, support the following:

- Regarding Tx UE behaviour:

  - If PSCCH/PSSCH transmission starts from 1st starting symbol, the PSCCH/PSSCH transmission has 1 or 2 AGC symbols

    - FFS: whether this is subject to explicit or implicit (pre-)configuration, or under some conditions, etc.

- Regarding Rx UE behaviour:

  - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot

- Note: AGC symbol is duplication of next symbol

[L] Proposal 2-3

Regarding Tx UE behavior:

- For the slots of a COT except the 1st slot, Tx UE only chooses the 1st starting symbol for PSCCH/PSSCH transmission, at least for transmission with no greater than 16us gap from the previous transmission by any UE.

[L] Proposal 6-1

Considering PSD limit in unlicensed spectrum regulation, down-select one of followings:

- Alt 1: RAN1 will update power control formulas of PSCCH/PSSCH/S-SSB/PSFCH by taking PSD limit into account

- Alt 2: RAN1 does not pursue specific enhancements on power control formulas of PSCCH/PSSCH/S-SSB/PSFCH to reflect PSD limit, i.e., RAN1 assumes PSD limit can be satisfied by other ways, e.g., UE implementation, etc.

## Proposals for Tuesday offline

[H] Proposal 4-1

Regarding PSFCH transmission with 15 kHz and 30 kHz SCS:

- RAN1 down-select one of followings in RAN1#113:

  - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

    - K3 is (pre-)configured, FFS value range

      - Value range for K3 at least includes {1, [2], 5,10, [11]}, FFS whether to additionally include other values

    - FFS: whether K3 dedicated PRB(s) are on the same interlace

    - FFS: whether to introduce guardband PRB/RE between common PRB and dedicated PRB

    - FFS: whether to reduce power on common PRBs

    - On the K3 dedicated PRB(s), multiple CS pairs can be used as in legacy NR SL PSFCH transmission

    - When a PRB of common interlace and a dedicated PRB locate within the same 1 MHz bandwidth, UE only transmits on the dedicated PRB

      - FFS: whether any impact on meeting OCB requirement

  - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

  - Alt 3-2b: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set

    - K2=2

    - K4 is (pre-)configured, FFS value range

    - FFS: whether to introduce guardband PRB/RE between common PRB and dedicated PRB

    - FFS: how to meet PSD limitation

    - FFS: whether to introduce any restrictions on the locations of K4 dedicated PRB(s) and/or K2 common PRBs, e.g., whether/how they are on the same interlace

- PSFCH collision is not supported, i.e., PSSCH transmissions on non-overlapped resources are mapped to orthogonal dedicated PRBs for PSFCH transmission as in R16 NR SL

- FFS: regardless of which Alt above is selected, whether or not to support PRB-level cyclic shift hopping as in NR-U to reduce PAPR

==

Table 1: Analysis on Alt 2-3a (30 kHz, 1 RB set)

| Row | Num of interlace (Ninterlace) | Num of sub-channels (NsubCH) | PSFCH periodicity / slots (P) | Candidate resources within 1 PSFCH period (i.e., each sub-channel on each slot) (Ncandidate= NsubCH*P) | Num of CS pairs (NCS) | Num of PSFCH resource per candidate (NperCandidatePSFCH=Ninterlace*NCSNcandidate) Note: this is also max group size for groupcast opt 2 | Can reuse legacy NR SL PSSCH-PSFCH mapping? (i.e., each candidate resource is mapped to orthogonal PRB for PSFCH) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 5 | 1 | 5 | 1 | 1 | - Can reuse legacy. - Seems unable/difficult to support “more than 1 PSFCH occasion(s) per PSCCH/PSSCH transmission” since PSFCH periodicity is already 1. |
| 2 | 5 | 5 | 1 | 5 | 2 | 2 |  |
| 3 | 5 | 5 | 1 | 5 | 3 | 3 |  |
| 4 | 5 | 5 | 1 | 5 | 6 | 6 |  |
| 5 | 5 | 5 | 2 | 10 | 1 | 0.5 | - Cannot reuse legacy |
| 6 | 5 | 5 | 2 | 10 | 2 | 1 |  |
| 7 | 5 | 5 | 2 | 10 | 3 | 1.5 |  |
| 8 | 5 | 5 | 2 | 10 | 6 | 3 |  |
| 9 | 5 | 5 | 4 | 20 | 1 | 0.25 |  |
| 10 | 5 | 5 | 4 | 20 | 2 | 0.5 |  |
| 11 | 5 | 5 | 4 | 20 | 3 | 0.75 |  |
| 12 | 5 | 5 | 4 | 20 | 6 | 1.5 |  |

==

[H] Proposal 5-3

When the SL-BWP contains multiple RB sets, support the followings:

- When UE attempts to transmit S-SSB in a S-SSB occasion (e.g., R16/17 S-SSB occasion, R18 additional candidate S-SSB occasion)

  - UE may transmit S-SSB repetition in more than one RB set [to maintain its COT]

    - Down-select one of the followings:

      - Alt 1: At least the power for S-SSB transmission on anchor RB set does not change due to the number of used RB sets

        - FFS details, e.g., whether this can be satisfied by (pre-)configuration, whether the power for S-SSB transmission on other RB set(s) also does not change due to the number of used RB sets, etc.

      - Alt 2: The power for S-SSB transmission on each RB set does not change due to the number of used RB sets

        - FFS details, e.g., whether this can be satisfied by (pre-)configuration, etc.

    - Locations of S-SSB repetitions in each RB set are the same as the locations of S-SSB repetitions in the anchor RB set

    - FFS: the relationship with UE’s COT

    - Note: anchor RB set refers to the RB set where S-SSB indicated by sl-AbsoluteFrequencySSB-r16 locates

    - Note: whether UE can transmit S-SSBs over non-contiguous RB sets is subject to RAN4’s reply, details can be found in RAN1’s LS to RAN4 in R1-2304218

==

Alt 1: power for S-SSB, P1=P4=P6

Alt 2: power for S-SSB, P1=P4=P6=P2=P3=P5

==

[H] Proposal 5-1

Regarding “Option 3-1(revised): Transmit legacy S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement”:

- Down-select one of the followings in RAN1#113

  - Alt 1: N=2, the two repetitions locate at two edges of an RB set

    - FFS details

  - Alt 2: the value of gap is (pre-)configured, and the value of N is derived implicitly

    - FFS: value range for gap

    - FFS: whether N for different RB sets is different

  - Alt 3: the value of gap is (pre-)configured, and the value of N is (pre-)configured

    - FFS: value range for gap and N

  - Alt 4: N=2, the value of gap is (pre-)configured

    - FFS: value range for gap

- FFS: whether to support reducing PAPR of S-SSB transmission, FFS details

[M] Proposal 5-2

Regarding the number and location(s) of additional candidate S-SSB occasions, down-select one of the followings in RAN1#113:

- [ Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS ]

  - This is a new R18 parameter (denoted as SL-SSB-TimeAllocation-r18)

  - For UEs operating on unlicensed spectrum, UE shall use both SL-SSB-TimeAllocation-r16 and SL-SSB-TimeAllocation-r18

- Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion(s) in different time slot(s), and the gap between them is (pre-)configured

  - FFS details, e.g., value of K, details on gap length (including possibility of being 0), etc.

Conclusion (RAN1#112b-e) Regarding additional candidate S-SSB occasions, in the same S-SSB period, UE can attempt to transmit on additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s). … (below is copied from TS 38.331) SL-SSB-TimeAllocation-r16 ::= SEQUENCE { sl-NumSSB-WithinPeriod-r16 ENUMERATED {n1, n2, n4, n8, n16, n32, n64} OPTIONAL, -- Need R sl-TimeOffsetSSB-r16 INTEGER (0..1279) OPTIONAL, -- Need R sl-TimeInterval-r16 INTEGER (0..639) OPTIONAL -- Need R } …

==

[M] Proposal 4-2-1

Regarding one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s) via (pre-)configuration:

- Value range of N at least includes {1, 2, 3, 4}, FFS whether to additionally include other values

  - By default (if no (pre-)configuration), N=1

- PSFCH collision is not supported, i.e., N associated candidate PSFCH occasion(s) for one PSCCH/PSSCH transmission have different time and/or frequency resource with the candidate PSFCH occasion(s) for another PSCCH/PSSCH transmission, at least if these two PSCCH/PSSCH transmissions are on non-overlapped resources

[M] Proposal 4-2-2

Regarding one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s) via (pre-)configuration:

- Regarding locations of candidate PSFCH occasion(s):

  - Down-select one of the followings:

    - Alt 1: Associated PSFCH occasion(s) are in one of the RB set(s) occupied by PSSCH transmissions

      - FFS details

    - [ Alt 2: PSSCH transmission and its related PSFCH occasion(s) are in the same or different RB set(s) ]

  - For one PSCCH/PSSCH transmission, at least support that its associated candidate PSFCH occasion(s) are in different slots of the same RB set(s)

    - FFS: whether to support its associated candidate PSFCH occasion(s) are in different RB sets of the same slot

[M] Proposal 4-2-3

Regarding one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s) via (pre-)configuration:

- Regarding UE behaviour on transmitting PSFCH:

  - Down-select one of the followings:

    - Alt 1: For one PSCCH/PSSCH transmission, PSCCH/PSSCH receiver UE attempts to transmit PSFCH on a candidate PSFCH occasion if and only if it fails to transmit on previous PSFCH occasion(s) due to LBT failure

    - Alt 2: For one PSCCH/PSSCH transmission, PSCCH/PSSCH receiver UE attempts to transmit PSFCH on a candidate PSFCH occasion if and only if it fails to transmit on previous PSFCH occasion(s) (e.g., due to LBT failure, or due to UL/SL prioritization, etc.)

    - Alt 3: Do not specify UE behavior on transmitting PSFCH.

- FFS: UE behaviour on receiving PSFCH

- Regarding HARQ RTT restriction:

  - Further study whether/how to update the followings:

    - The minimum time gap Z=a+b between any two selected resources of a TB in case PSFCH is configured for this resource pool

    - The reference slot n for PUCCH transmission to report HARQ in Mode 1

[M] Proposal 4-3

When neither COT initiating UE nor responding UE intends to transmit PSFCH on some PSFCH occasion(s) within a COT, to avoid COT interruption, selects one or both of the followings:

- Option 1: COT initiating UE or responding UE transmits PSSCH on such PSFCH occasion(s)

  - FFS details, e.g., how PSSCH Rx UE knows such transmission, etc.

- Option 2: COT initiating UE or responding UE transmits a PSFCH-like sequence signal on such PSFCH occasion(s)

  - FFS details, e.g., signaling design, etc.

[M] Proposal 2-1

If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, for TBS determination, in TS 38.214 Clause 8.1.3.2:

- L_ref replaces sl-LengthSymbols

  - Value range of L_ref is {7, 8, 9, 10, 11, 12, 13, 14} symbols

- Down-select one of the followings:

  - Alt 1: NsymbPSFCH is determined in the same way as in legacy NR SL

  - Alt 2: NsymbPSFCH is always 0

Agreement (in RAN1#112b-e) If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - TBS is determined based on a reference number of symbols (denoted as L_ref) - Support the followings - Alt 1: Support Option 4 only - Note: the options are as below - Option 4: The reference number of symbols is determined by (pre-)configuration - FFS details, e.g., in TS 38.214 Clause 8.1.3.2, whether L_ref replaces Nsymbsh or sl-LengthSymbols or other usage of L_ref, whether NsymbPSFCH is determined in the same way as in legacy NR SL, etc.

(Below is copied from TS 38.214) #### 8.1.3.2 Transport block size determination For the PSSCH assigned by SCI, if Table 5.1.3.1-2 is used and , or a table other than Table 5.1.3.1-2 is used and the UE shall first determine the TBS as specified below: The UE shall first determine the number of REs (NRE) within the slot. - A UE first determines the number of REs allocated for PSSCH within a PRB (NRE') by NRE'=NscRBNsymbsh-NsymbPSFCH-NohPRB-NREDMRS, where - NscRB=12 is the number of subcarriers in a physical resource block, - Nsymbsh = sl-LengthSymbols -2, where sl-LengthSymbols is the number of sidelink symbols within the slot provided by higher layers, - NsymbPSFCH = 3 if 'PSFCH overhead indication' field of SCI format 1-A indicates "1", and NsymbPSFCH = 0 otherwise, if higher layer parameter sl-PSFCH-Period is 2 or 4. If higher layer parameter sl-PSFCH-Period is 0, NsymbPSFCH=0. If higher layer parameter sl-PSFCH-Period is 1, NsymbPSFCH=3. - NohPRB is the overhead given by higher layer parameter sl-X-Overhead, - NREDMRS is given by Table 8.1.3.2-1 according to higher layer parameter sl-PSSCH-DMRS-TimePatternList. … (below is copied from TS 38.331) SL-BWP-Generic-r16 ::= SEQUENCE { sl-BWP-r16 BWP OPTIONAL, -- Need M sl-LengthSymbols-r16 ENUMERATED {sym7, sym8, sym9, sym10, sym11, sym12, sym13, sym14} OPTIONAL, -- Need M sl-StartSymbol-r16 ENUMERATED {sym0, sym1, sym2, sym3, sym4, sym5, sym6, sym7} OPTIONAL, -- Need M sl-PSBCH-Config-r16 SetupRelease {SL-PSBCH-Config-r16} OPTIONAL, -- Need M sl-TxDirectCurrentLocation-r16 INTEGER (0..3301) OPTIONAL, -- Need M ... } …

==

[M] Proposal 3-1

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings:

- Use FRIVsubCH to indicate used sub-channel index(s)

- FRIVsubCH is conveyed in 1st stage SCI

- FRIVsubCH is calculated as below

  - If sl-MaxNumPerReserve is 2 then

    - FRIVsubCH=nsubCH,1start+i=1LsubCHRBset-1N subchannel RBset+1-i

  - If sl-MaxNumPerReserve is 3 then

    - FRIVsubCH=nsubCH,1start+nsubCH,2start⋅N subchannel RBset+1-LsubCHRBset+i=1LsubCHRBset-1N subchannel RBset+1-i2

  - where

    - nsubCH,1start denotes the starting sub-channel index for the second resource

    - nsubCH,2start denotes the starting sub-channel index for the third resource

    - N subchannel RBset is the number of sub-channels in one RB set

    - LsubCHRBset is the number of used sub-channels within one RB set for one PSCCH/PSSCH transmission

- FFS: whether/how to use spare states of FRIVsubCH to indicate using non-contiguous interlaces

- FFS: whether additionally support different number of RB set(s) in such case while keeping total number of sub-channels unchanged between initial transmission and retransmission(s) for a TB

- FFS: whether additionally support using bitmap

[M] Proposal 3-2

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings:

- Use FRIVRBset to indicate used RB set index(s)

- FRIVRBset is conveyed in 1st SCI

- FRIVRBset is calculated as below

  - If sl-MaxNumPerReserve is 2 then

    - FRIVRBset=nRBset,1start+i=1LRBset-1NRBset+1-i

  - If sl-MaxNumPerReserve is 3 then

    - FRIVRBset=nRBset,1start+nRBset,2start⋅NRBset+1-LRBset+i=1LRBset-1NRBset+1-i2

  - where

    - nRBset,1start denotes the starting RB set index for the second resource

    - nRBset,2start denotes the starting RB set index for the third resource

    - NRBset is the number of RB sets in a resource pool

    - LRBset is the number of used RB sets for one PSCCH/PSSCH transmission

[M] Proposal 3-3

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings:

- MAC layer indicates LRBset to PHY layer, where LRBset is the number of used RB sets for one PSCCH/PSSCH transmission

- Regarding LsubCH in TS 38.214 Clause 8.1.4, down-select one of the followings in RAN1#113:

  - Alt 1: LsubCH is re-defined as “the number of sub-channels within one RB set to be used for the PSSCH/PSCCH transmission in a slot”

    - Note: LsubCHRBset=LsubCH

  - Alt 2: LsubCH is re-defined as “the number of sub-channels within all used RB sets to be used for the PSSCH/PSCCH transmission in a slot”

    - Note: LsubCHRBset=LsubCH/LRBset

  - Note: LsubCHRBset is the number of used sub-channels within one RB set for one PSCCH/PSSCH transmission

- Regarding candidate single-slot resource for transmission Rx,y in TS 38.214 Clause 8.1.4:

  - Rx,y is replaced by Rr,x,y, Rr,x,y is defined as a set of LsubCHRBset contiguous sub-channels in LRBset contiguous RB sets, with sub-channel x+j in RB set r+h in slot t'ySL where j=0,...,LsubCH LsubCHRBset-1, h=0, …, LRBset-1.

  - FFS: how to re-define Rx,y if using non-contiguous interlaces is supported

[M] Proposal 3-4

For interlace RB-based PSCCH/PSSCH transmission in SL-U, support the followings:

- Option A3: N_ref is (pre-)configured

  - The value range for N_ref at least includes {10, 11}

  - FFS: whether to additionally include other values

[M] Proposal 3-5

For contiguous RB-based PSCCH/PSSCH transmission in SL-U, regarding sub-channel(s) which include intra-cell guardband PRBs, support the followings:

- Down-select one of the followings in RAN1#113

  - Alt 1: Support both Option 2 and Option 3 as below

    - If all the following conditions are satisfied, Option 2 is adopted

      - Condition 1: Nleft >= NPSCCH

      - [Condition 2: Lowest PRB of such sub-channel does not locate in intra-cell guardband]

    - Otherwise, Option 3 is adopted

  - Alt 2: Support only Option 3

- FFS other details, e.g., impacts on resource selection, PSCCH mapping, etc.

- Note:

  - Option 2: Such sub-channel(s) can be used for PSCCH/PSSCH transmission

    - Note: PRBs within intra-cell guard band are not used for PSCCH transmission as per previous agreement

  - Option 3: Such sub-channel(s) cannot be used for PSCCH transmission, and can be used for PSSCH transmission

  - Nleft : the number of remaining PRBs of a sub-channel belonging to a RB set after excluding the PRBs belonging to intra-cell guardband

  - NPSCCH : the number of PRBs for PSCCH transmission

==

[M] Proposal 2-2

For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, support the following:

- Regarding Tx UE behaviour:

  - If PSCCH/PSSCH transmission starts from 1st starting symbol, the PSCCH/PSSCH transmission has 1 or 2 AGC symbols

    - FFS: whether this is subject to explicit or implicit (pre-)configuration, or under some conditions, etc.

- Regarding Rx UE behaviour:

  - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot

- Note: AGC symbol is duplication of next symbol

[L] Proposal 2-3

Regarding Tx UE behavior:

- For the slots of a COT except the 1st slot, Tx UE only chooses the 1st starting symbol for PSCCH/PSSCH transmission, at least for transmission with no greater than 16us gap from the previous transmission by any UE.

[L] Proposal 6-1

Considering PSD limit in unlicensed spectrum regulation, down-select one of followings:

- Alt 1: RAN1 will update power control formulas of PSCCH/PSSCH/S-SSB/PSFCH by taking PSD limit into account

- Alt 2: RAN1 does not pursue specific enhancements on power control formulas of PSCCH/PSSCH/S-SSB/PSFCH to reflect PSD limit, i.e., RAN1 assumes PSD limit can be satisfied by other ways, e.g., UE implementation, etc.

# Conclusions

TBD

# References

- R1-2304342	On Physical Channel Design Framework for SL-U	Nokia, Nokia Shanghai Bell

- R1-2304364	Discussion on physical channel design for sidelink on unlicensed spectrum	FUTUREWEI

- R1-2304481	Physical channel design framework for sidelink on unlicensed spectrum	vivo

- R1-2304559	Discussion on Physical channel design for sidelink on unlicensed spectrum	Spreadtrum Communications

- R1-2304662	Physical channel design for sidelink operation over unlicensed spectrum	Huawei, HiSilicon

- R1-2304732	Discussion on physical channel design framework for sidelink on unlicensed spectrum	CATT, GOHIGH

- R1-2304903	Discussion on physical channel design for sidelink-unlicensed	xiaomi

- R1-2304982	Discussion on physical channel design framework	NEC

- R1-2305039	Discussion on physical channel design framework for SL-unlicensed	Sony

- R1-2305063	Physical layer design framework for sidelink on FR1 unlicensed spectrum	Lenovo

- R1-2305096	Discussion on physical channel design framework for sidelink on unlicensed spectrum	CMCC

- R1-2305153	SL U physical layer design framework	InterDigital, Inc.

- R1-2305244	Sidelink Physical Channel Design Framework for Unlicensed Spectrum	Apple

- R1-2305338	Physical Channel Design for Sidelink on Unlicensed Spectrum	Qualcomm Incorporated

- R1-2305380	Discussion on physical layer structures and procedures for SL-U	ZTE, Sanechips

- R1-2305422	Remaining issues on SL-U PHY channel designs and procedures	OPPO

- R1-2305515	On physical channel design framework for sidelink on FR1 unlicensed spectrum	Samsung

- R1-2305600	Discussion on channel design framework in SL-U	NTT DOCOMO, INC.

- R1-2305631	Discussion on physical channel design framework for sidelink on unlicensed spectrum	LG Electronics

- R1-2305673	Discussion on physical channel design framework	MediaTek Inc.

- R1-2305694	Physical channel design for sidelink on unlicensed spectrum	Panasonic

- R1-2305707	Discussion of physical channel design for sidelink in unlicensed spectrum	Transsion Holdings

- R1-2305759	Physical Channel Design framework for SL-U	ITL

- R1-2305761	NR Sidelink Unlicensed Physical Channel Design	Fraunhofer HHI, Fraunhofer IIS

- R1-2305795	Discussion on physical channel design framework for SL-U	ETRI

- R1-2305818	Discussion on PHY channel design framework for SL-U	WILUS Inc.

- R1-2305844	Discussion on physical channel design framework for NR sidelink evolution on unlicensed spectrum	Sharp

- R1-2305870	PHY channel design framework for SL-U	Ericsson

# Annex A: Contact Information

Please provide/update your contact information here so that delegates can better communicate with each other if needed.

| Company | Name | Email |
| --- | --- | --- |
| Huawei | Xiang Mi | Shawn.mixiang@huawei.com |
| HiSilicon | Fan Yang | james.yangfan@huawei.com |
| LGE | Daesung Hwang Seungmin Lee | daesung.hwang@lge.com edison.lee@lge.com |
| CATT, GOHIGH | Rui Zhao Shupeng Li Xiaoran Wen | zhaorui@goghigh.com.cn lsp@catt.cn wenxiaoran@gohigh.com.cn |
| OPPO | Zhenshan ZHAO Kevin Lin | zhaozhenshan@oppo.com kevin.lin@oppo.com |
| vivo | Zichao Ji | jizichao@vivo.com |
| ZTE,Sanechips | Yuzhou Hu | hu.yuzhou@zte.com.cn |
| Spreadtrum | Mimi Chen | mimi.chen@unisoc.com |
| Intel | Salvatore Talarico | salvatore.talarico@intel.com |
| CMCC | Pengyu Ji Jingwen Zhang | jipengyu@chinamobile.com zhangjingwen@chinamobile.com |
| Panasonic | Ayako Iwata | iwata.ayako@jp.panasonic.com |
| xiaomi | Wensu Zhao Qun Zhao | zhaowensu@xiaomi.com zhaoqun1@xiaomi.com |
| NTT DOCOMO | Shohei Yoshioka | shohei.yoshioka@docomo-lab.com syouhei.yoshioka.py@nttdocomo.com |
| Sharp | Liqing Liu | liu.liqing@sharp.co.jp |
| Nokia/NSB | Naizheng Zheng | naizheng.zheng@nokia-sbell.com |
| Ericsson | Ricardo Blasco Jose Leon Calvo | name.surname at company . com jose.leon.calvo@ericsson.com |
| Transsion | Xingya Shen | xingya.shen@transsion.com |
| InterDigital | Aata El Hamss | Aata.elhamss@interdigital.com |
| Lenovo | Xiaodong Yu Karthikeyan Ganesan Alexander Golitschek | Yuxd1@lenovo.com kganesan@lenovo.com aelbwart@lenovo.com |
| Samsung | Hongbo Si | hongbo.si@samsung.com |
| Futurewei | George Calcev | gcalcev@futurewei.com |
| Qualcomm | Giovanni Chisci Chih-Hao Liu Stelios Stefanatos | gchisci@qti.qualcomm.com chihliul@qti.qualcomm.com sstefana@qti.qualcomm.com |
| NEC | Zhaobang MIAO Jin YANG | miao_zhaobang@nec.cn yangjin@labs.nec.cn |
| WILUS | Minseok Noh | minseok.noh@wilusgroup.com |
| MediaTek | Tao Chen | tao.chen@mediatek.com |
| Fraunhofer | Tom Wirth | thomas.wirth@hhi.fraunhofer.de |
| China Telecom | Jing Guo | guojing6@chinatelecom.cn |
| Sony | Kazuyuki Shimezawa | kazuyuki.shimezawa@sony.com |
| Johns Hopkins University APL | Vijitha Weerackody | vijitha.weerackody@jhuapl.edu |
| National Spectrum Consortium | Amitav Mukherjee | Amitav.Mukherjee@nationalspectrumconsortium.org |
| ETRI | Junhyeong Kim | jhkim41jf@etri.re.kr |

# Annex B: Outcomes of RAN1 meetings

## RAN1#109-e (May 9 – 20, 2022)

Agreement

SL BWP, SL resource pool in R16/R17 NR SL and RB set in R16 NR-U are reused for SL-U as baseline

- Only one SL BWP is (pre-)configured within a carrier

- The SL BWP is (pre-)configured to include one or multiple SL resource pools

- At least support that one SL resource pool can be (pre-)configured to include integer number of RB sets

  - FFS: whether/how to support one SL resource pool can include sub-set of PRBs of one RB set

  - FFS: the applicable resource pool

  - FFS: the impact on sub-channel size and number of sub-channels in a resource pool if sub-channel is supported

- PRBs within intra-cell guard band of two adjacent RB sets belong to a resource pool if the resource pool includes the two adjacent RB sets

  - FFS details, e.g., how such PRBs are used, the applicable resource pool, etc.

- FFS: whether R16/R17 NR SL S-SSB slots and/or new S-SSB slots (if supported) are excluded from resource pool

- FFS: which slots belong to resource pool, e.g., how to set the value of bitmap, whether to consider SL-U/NR-U operating in the same carrier and whether TDD configuration are considered, etc.

- FFS: the impact of PSCCH/PSSCH mapping to frequency resources on resource pool configuration, on sub-channel definition if sub-channel is supported, etc.

Agreement

For PSCCH and PSSCH in SL-U:

- Both R16/R17 NR SL contiguous RB-based and R16 NR-U interlace RB-based transmissions are considered as starting point

  - RAN1 strives to have unified design for both contiguous RB-based and interlace RB-based transmissions

  - FFS: whether/how to address IBE (In Band Emission) impact

Agreement

For PSCCH and PSSCH in SL-U:

- For interlace RB-based transmission (if supported), at least the following candidates can be discussed:

  - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission

    - FFS: Other resource allocation granularity, e.g., RB-level

  - 1 sub-channel equals K interlaces if sub-channel is supported

    - FFS details

  - Other candidates are not precluded

  - FFS: mapping of PSCCH to frequency resources

  - FFS: resource indication in time/frequency domain, e.g., how to handle using one RB set or multiple RB sets, etc.

Agreement

For slot structure in SL-U:

- At least R16/R17 NR SL slot-based PSCCH/PSSCH transmission is supported

  - FFS: whether/how to support additional starting symbol(s) within a slot for the PSCCH/PSSCH transmission

Agreement

For PSFCH and SL-HARQ in SL-U:

- At least R16 NR SL PSFCH format 0 is supported

  - FFS whether to introduce new PSFCH format

- FFS: how to meet OCB and PSD requirement for PSFCH transmission, e.g., using interlaced RB transmission, whether/how to avoid too small PSFCH capacity, etc.

- FFS: the locations of PSFCH resources, e.g., (pre-)configured, dynamically indicated, etc.

- FFS: whether/how to address PSFCH transmission dropping due to LBT failure, e.g., whether to have multiple PSFCH occasions for a PSSCH and the related PSSCH-PSFCH mapping relationship, impact on SL HARQ-ACK reporting to the gNB for Mode 1, etc.

- FFS: whether/how to address PSFCH and related PSSCH in different COTs

Agreement

For S-SSB and synchronization in SL-U:

- FFS the time domain locations of S-SSB resources, e.g., whether/how to introduce more candidate occasions compared with R16/R17 NR SL design, etc.

- Down-selection at least one of the following solutions to meet OCB and PSD requirement for S-SSB transmission

  - Option 1: Using interlaced RB transmission

  - Option 2: S-SSB multiplexing with other SL transmissions in the same slot

  - Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain

  - Option 4: S-PSS/S-SSS/PSBCH with wider bandwidth

- FFS: whether to support 4 symbols S-SSB

  - Note: 4 symbols S-SSB can be considered with options 1/2/3/4 above

- FFS whether the temporary exemption of OCB requirement is applicable for S-SSB transmission

- FFS whether any changes to R16/R17 NR SL synchronization procedure

## RAN1#110 (August 22 – 26, 2022)

Agreement

For PSCCH and PSSCH in SL-U:

- Both R16/R17 NR SL contiguous RB-based and interlace RB-based transmissions similar to R16 NR-U are supported

Agreement

For PSCCH and PSSCH in SL-U:

- For interlace RB-based transmission

  - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission

  - 1 sub-channel equals K interlace

    - FFS: whether K is fixed as 1 or (pre-)configured

  - Discuss whether one or both of the following alternatives are supported

    - Alt 1: 1 sub-channel is confined within 1 RB set

    - Alt 2: 1 sub-channel spans 1 or multiple RB set(s) belonging to a resource pool

Agreement

To meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS, FFS details.

Agreement

If RAN1 decides that LBT is performed for S-SSB transmission, in addition to the S-SSB occasions in R16/R17 NR SL design, support additional candidate S-SSB occasions

- FFS the number and locations of additional candidate S-SSB occasions

- FFS when a UE transmits S-SSB on such additional candidate S-SSB occasions, and the related Rx UE’s behavior

Agreement

Regarding PSFCH transmission, at least the followings alternatives can be further studied

- Alt 1: each PSFCH transmission occupies a common interlace and zero or one or more dedicated PRB(s)

- Alt 2: each PSFCH transmission occupies an interlace, and may or may not further apply code domain enhancement (e.g., OCC, PRB-level cyclic shifts)

- Alt 3: each PSFCH transmission occupies some dedicated PRBs and some common PRBs

- FFS details of above alternatives

Agreement

If RAN1 decides that LBT is performed for PSFCH transmission, for the time and frequency domain locations of PSFCH resources, at least the followings alternatives can be further studied

- Alt 1: PSFCH resources are (pre-)configured

- Alt 2: PSFCH resources are dynamically indicated

- Combination of above alternatives are not precluded

- FFS details of above alternatives

Agreement

For S-SSB and synchronization in SL-U:

- No changes on R16 NR SL S-PSS/S-SSS sequence generation

- Continue studying the 4 options from the previous agreement and whether/how temporary exemption of OCB requirement is applicable for S-SSB transmission, e.g., how to meet the minimum of 2 MHz requirement under 15 kHz SCS

Agreement

For PSCCH and PSSCH resource indication in time/frequency domain:

- For time domain: R16 NR SL TRIV is reused as baseline

- For frequency domain:

  - further study sub-channel indexing and resource indication

- FFS: whether any enhancement needed on R16 NR SL TRIV/FRIV if new feature is introduced in SL-U, e.g., multi-slot consecutive transmission

## RAN1#110b-e (October 10 – 19, 2022)

Agreement

For interlace RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding 1 sub-channel equals K interlace(s)

  - At least K=1 and K=2 is supported for 15 kHz SCS

  - At least K=1 is supported for 30 kHz SCS

  - FFS: details related to multiple RB sets

Working assumption:

Support maximum 2 candidate starting symbols within a slot for a PSCCH/PSSCH transmission.

- RAN1 strives to have unified design for PSCCH/PSSCH transmission from 1st or 2nd starting symbol

- The candidate starting symbol(s) are intended for AGC purpose

  - FFS: other potential uses of the candidate starting symbol(s)

- FFS other details, e.g., applicable scenarios (including SCS), position of 2nd starting symbol, TBS determination, PSCCH blind decoding complexity, processing time constraints, etc.

- FFS whether 2 candidate starting symbols is also supported for slots with PSFCH

Agreement

To meet OCB and PSD requirement for S-SSB transmission, down-select between the followings for 15 kHz and 30 kHz SCS:

- Option 1: Using interlaced RB transmission for S-PSS/S-SSS/PSBCH

- Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain

- FFS: whether/how the above options apply to all or subset of channel type of S-PSS/S-SSS/PSBCH

- Note: RAN1 further study the relationship between above options and temporary OCB exemption, and the discussion on temporary OCB exemption can continue even if option 1 or option 3 is supported

FFS: how to handle 60 kHz SCS (if needed, not limited to option 1 or option 3)

Agreement

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission:

- When more than one RB set is used for transmissions, down-select one of the followings

  - Option A: Support that the used interlace index(s) in different RB sets are always the same

  - Option B: Support that the used interlace index(s) in different RB sets can be different

- FFS details

Agreement

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission:

- Down-select one of the followings

  - Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s)

  - Option 2: Support explicitly indicating at least the used sub-channel index(s)

    - At least RB set index(s) is not explicitly indicated

- FFS details

Agreement

For PSCCH and PSSCH in SL-U:

- PSCCH is transmitted within 1 sub-channel

- At least support Option 1 below

  - Option 1: PSCCH locates in the lowest sub-channel of lowest RB set of corresponding PSSCH

    - Note: the lowest sub-channel may not be entirely contained in the lowest RB set

- FFS whether/how to handle the case where UEs supporting different bandwidths can use the same resource pool to communicate with each other, e.g., whether/how to additionally support Option 2 below

  - Option 2: PSCCH locates in every RB set of corresponding PSSCH

- Note: the above options do not imply any restriction on the mapping of sub-channels to PRBs.

- FFS other details

Agreement

Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets:

- Such PRBs can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission

  - FFS details, e.g., handling of potential unequal sub-channel size, for interlaced RB based transmission, whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets

- Such PRBs are not used for PSCCH transmission

  - FFS: whether or not such PRBs are used for PSFCH/S-SSB transmission

Agreement

At least R16/R17 NR SL S-SSB slots are excluded from SL resource pool.

- Note: whether or not additional candidate S-SSB occasions are excluded from resource pool will be discussed after the details of additional candidate S-SSB occasions are clearer

Agreement

At least there is 1 PSFCH occasion per PSCCH/PSSCH transmission, FFS details

Agreement

To address PSFCH transmission dropping due to LBT failure, the followings are to be studied:

- Alt 1: Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission

- Alt 2: PSFCH resources are dynamically indicated

- Alt 3: Convey SL-HARQ feedback information in PSCCH/PSSCH, e.g., new SCI or new MAC-CE

- Alt 4: drop PSFCH transmission

- Alt 5: Support trigger based HARQ feedback reporting for non-numerical HARQ FB and one shot HARQ FB

- Combination of above alternatives are not precluded

- FFS details of above alternatives

Agreement

Regarding additional candidate S-SSB occasions:

- Their number and time domain locations are (pre-)configured or pre-defined

## RAN1#111 (November 14 – 18, 2022)

Agreement

For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- Regarding the location of 1st starting symbol, down-select one of the followings:

  - Option 1: it is fixed as symbol#0

  - Option 2: it is indicated by sl-StartSymbol as in R16 NR SL

- Regarding the location of 2nd starting symbol, down-select one of the followings:

  - Option A: it is a fixed location

    - FFS the location, e.g., symbol#4, #7, etc.

  - Option B: it is a (pre-)configured location per resource pool

    - FFS the details of candidate locations

- Note: assume symbol index in a slot starts from #0

Agreement

If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- TBS is determined based on a reference symbol length, down-select one of the followings:

  - Option 1: The reference symbol length is dynamically indicated by Tx UE

  - Option 2: The reference symbol length is determined based on 1st starting symbol

  - Option 3: The reference symbol length is determined based on 2nd starting symbol

  - Option 4: The reference symbol length is (pre-)configured

Agreement

Regarding PSFCH transmission under 15 kHz and 30 kHz SCS, RAN1 continues studying the following updated alternatives:

- Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

  - FFS: value of K3

- Alt 2-1a: each PSFCH transmission occupies 1 interlace, and further apply frequency-domain OCC

  - FFS: details of FD-OCC, e.g., OCC length, RB-level, RE-level, etc.

- Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift

  - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace

  - FFS: value of K1

- Alt 2-3a: each PSFCH transmission occupies 1 interlace

- Alt 2-4a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift

  - A UE uses different cyclic shifts on different PRBs in the interlace

- Alt 3-1a: each PSFCH transmission occupies 1 dedicated PRB and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set

  - The above dedicated PRB and common PRBs are within 1 interlace

  - FFS: value of K2

- Alt 3-2a: each PSFCH transmission occupies 1 dedicated PRB and 2 common PRBs, where 2 common PRBs locate at the two edges of a RB set

- FFS: the impact of PSD limit, e.g., whether/how to handle the case when common PRB and dedicated PRB locate within the same 1 MHz bandwidth

- FFS: whether IBE issue exists and whether/how to address it

- Note: in the above descriptions

  - The dedicated PRB/cyclic shift conveys ACK/NACK information

- Note: as previously agreed: to meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS.

Agreement

Slots with PSFCH symbols only have 1 candidate starting symbol for PSCCH/PSSCH.

Agreement

For interlace RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding mapping between sub-channel and interlace, RAN1 further study the followings:

  - Option 1: 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool

  - Option 2: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly within an RB set, then across different RB sets within the resource pool

  - Option 3: 1 sub-channel is defined across all RB sets within the resource pool, i.e., 1 sub-channel includes K interlace(s) across all RB sets within the resource pool

  - Option 4: 1 sub-channel is defined within 1 RB set or 2 adjacent RB sets, and is incrementally indexed firstly within an RB set, then across different RB sets within the resource pool

  - Option 5: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly across different RB sets within the resource pool, then across different interlaces in the RB set

  - FFS: whether/how to use intra-cell guardband PRBs

Agreement

For S-SSB transmission, down-select one or more of the following for 15 kHz and 30 kHz SCS:

- Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH

- Option 1-2: Using interlaced RB transmission for PSBCH only, and apply OCB exemption to S-PSS and S-SSS

- Option 3-1: Repeat S-PSS/S-SSS/PSBCH N times in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement

  - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of N (e.g., N=2)

  - FFS gap of 0

- Option 3-2: Repeat only S-PSS/S-SSS K times in frequency domain, and PSBCH is rate matched. There is a gap between the repetition(s) to meet OCB requirement

  - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of K

  - FFS gap of 0

  - FFS PSBCH resource

- Option 3-3: keep the legacy S-PSS/S-SSS/PSBCH while repeating PSBCH N times in frequency domain and rate-matching PSBCH to S-PSS/S-SSS symbols, and there is a gap between the PSBCH repetition(s) to meet OCB requirements

  - FFS details, e.g. the length of gap is (pre-)configured or pre-defined, value of N

- Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH

- For Option 1-1 and 1-2 above

  - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set

- FFS: whether transient period issue exists and whether/how to address it

Agreement

For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- Regarding Tx UE behaviour:

  - If PSCCH/PSSCH transmission starts from 1st starting symbol, down-select one of the followings

    - Option 1: The PSCCH/PSSCH transmission has 2 symbols for AGC purpose

    - Option 2: The PSCCH/PSSCH transmission has only 1 symbol for AGC purpose

    - Option 3: The PSCCH/PSSCH transmission has 1 or 2 symbol(s) for AGC purpose depending on conditions, FFS details

  - If PSCCH/PSSCH transmission starts from 2nd starting symbol, the PSCCH/PSSCH transmission has only 1 symbol for AGC purpose

- Regarding Rx UE behaviour, down-select one of the followings:

  - Option A: The Rx UE always monitors two AGC symbols in such slot

  - Option B: The Rx UE monitors two AGC symbols in such slot by default, but could drop monitoring the 2nd AGC symbol at least if it detects a PSCCH/PSSCH transmission starting from the 1st starting symbol

    - FFS details

  - Option C: The Rx UE monitors two AGC symbols in such slot by default, but it is up to UE implementation whether to drop monitoring the 2nd AGC symbol

  - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot

Agreement

To address PSFCH transmission dropping due to LBT failure, RAN1 down-select one of followings, or support the combination of followings:

- Alt 1: Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission

  - FFS other details, e.g., HARQ-ACK timeline

- Alt 2: PSFCH occasions are dynamically indicated

  - FFS: Whether/how to handle the case where some TB’s corresponding PSFCH cannot be transmitted within the same or different COT

  - FFS other details, e.g., dynamically indicate one or more PSFCH transmission(s), container of the indication, etc.

- FFS: Whether such PSFCH occasions are within the same or different COT of corresponding PSSCH

- FFS: Whether/how to address PSFCH collision if any

- FFS: Whether/how to handle the linearly decreased PSFCH capacity

Agreement

Regarding the number and location(s) of additional candidate S-SSB occasions, RAN1 further study the followings:

- Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS

- Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion, and the gap between them is (pre-)configured

  - FFS details, e.g., value of K, details on gap length, etc.

- Option 3: The number and location(s) of additional candidate S-SSB occasions are separately (pre-)configured

- Option 4: Introduce M contiguous candidate S-SSB occasions in one S-SSB period

- Option 5: the number of candidate S-SSB occasions is (pre-)configured, and locations are determined based on the (pre-)configured number

Agreement

Regarding additional candidate S-SSB occasions:

- In the same S-SSB period, RAN1 further study the followings:

  - Alt 1: UE attempts to transmit on all or some of additional candidate S-SSB occasion(s) only when it fails to transmit on R16/R17 S-SSB occasion(s)

  - Alt 2: UE attempts to transmit on all additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s)

  - Alt 3: UE can attempt to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s)

  - Alt 4: upon LBT failure on a (candidate) S-SSB occasion, a UE attempts to transmit on the subsequent additional candidate S-SSB occasion if within a period S-SSB transmission has not been transmitted in any prior occasions

- FFS details

Agreement

For contiguous RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding mapping between sub-channel and PRBs, further study the following options:

  - Option 1 (sub-channel aligns with resource pool boundary): Same as in legacy NR SL, i.e., the mapping of sub-channel starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size

    - FFS: whether/how to use sub-channel(s) which include intra-cell guardband PRBs

    - FFS: whether/how to handle the case when the number of PRBs of the resource pool cannot be divided by sub-channel size

  - Option 2 (sub-channel aligns with RB set boundary): In each RB set, the mapping of sub-channel starts from the first PRB of the RB set and mapped sequentially within the RB set according to the sub-channel size

    - FFS: whether/how to use intra-cell guardband PRBs

    - FFS: whether/how to handle the case when the number of PRBs of one RB set cannot be divided by sub-channel size

  - Option 3 (sub-channel aligns with RB set boundary): In each RB set, the mapping of sub-channel starts from the first PRB of the RB set and mapped sequentially within the RB set and/or guardband PRB according to the sub-channel size

    - FFS: how to use intra-cell guardband PRBs

    - FFS: how to use the subchannel including PRBs in guardband

Agreement

Regarding PSFCH transmission under 60 kHz SCS, further study the following alternatives:

- Alt 1: Each PSFCH transmission occupies K dedicated PRB(s) and some common PRBs

  - FFS details

- Alt 2: Each PSFCH transmission occupies some dedicated PRBs

  - FFS details

Agreement

Regarding S-SSB, RAN1 further study the following:

- How to transmit S-SSB when a SL BWP contains multiple RB sets

## RAN1#112 (February 27 – March 3, 2023)

Agreement

For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- The location of 1st starting symbol can be (pre)configured from {#0,#1,#2,#3,#4,#5,#6} per BWP

  - By default (if no (pre)configuration), the location of the 1st starting symbol is symbol#0

- The location of 2nd starting symbol is (pre-)configured from {#3,#4,#5,#6,#7} per BWP

  - It shall be configured such that within a slot, the number of symbols used for PSCCH/PSSCH transmission from 2nd starting symbol is not smaller than 6

  - It shall be configured such that within a slot, the 2nd starting symbol is later than the 1st starting symbol

- PSCCH/PSSCH transmission starting from 1st or 2nd starting symbol shall have the same ending symbol within a slot

- Note: assume symbol index in a slot starts from #0

Agreement

For interlace RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding mapping between sub-channel and interlace, 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool

Agreement

Regarding PSFCH transmission with 15 kHz and 30 kHz SCS, RAN1 down-select one of followings, or support the combination of followings:

- Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

  - FFS: value of K3

- Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift

  - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace

  - FFS: value of K1

- Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

- Alt 2-4a: each PSFCH transmission occupies 1 dedicated interlace and adopt PRB-level cyclic shift hopping as in NR-U

- Alt 3-2a: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set

  - FFS: value of K2, K4

- FFS: the impact of PSD limit, e.g., whether/how to handle the case when common PRB and dedicated PRB locate within the same 1 MHz bandwidth, e.g., drop common PRB or reduce power on common PRB in such case

- FFS: whether/how to reduce PAPR of PSFCH transmission

Agreement

To address PSFCH transmission dropping due to LBT failure:

- Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission

  - Down-select one or support both of the followings

    - Option 1: Such PSFCH occasion(s) are (pre-)configured

    - Option 2: Such PSFCH occasion(s) are (pre-)configured and dynamically indicated

    - FFS applicable scenarios, e.g., considering the applicability of COT sharing, MCSt, etc.

  - FFS other details

Agreement

For contiguous RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding mapping between sub-channel and PRBs, down-select one of the followings during RAN1#112:

  - Option 1 (sub-channel aligns with resource pool boundary): Same as in legacy NR SL, i.e., the mapping of sub-channel starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size

    - FFS: how to deal with the remaining PRBs, e.g. for meeting OCB requirements

Agreement

For S-SSB transmission within 1 RB set, down-select to one or more of the following for 15 kHz and 30 kHz SCS:

- Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH

  - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set, e.g. whether 1 or 2 interlaces will be used for S-SSB

- Option 3-1: Transmit S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement

  - FFS details, e.g., the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), how to reduce PAPR, etc.

  - FFS gap of 0

- Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH

  - Continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS.

Working assumption

If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- At least for COT initiation, TBS is determined based on a reference number of symbols as follows:

  - Option 4: The reference number of symbols is determined by (pre-)configuration

  - FFS: value range

  - FFS: whether a different reference number of symbols is needed for transmission in a shared COT

Agreement

Down-select one or support both of the followings:

- Option 1: Additional candidate S-SSB occasions are excluded from resource pool

- Option 2: Additional candidate S-SSB occasions belong to resource pool

- Note: Companies are encouraged to consider aspects including: S-SSB resource overhead, Tx/Rx UE behavior (e.g., whether any blind detection in Option 2), applicable scenarios, etc.

Agreement

RAN1 further study the followings:

- Whether/how to maintain a COT when the COT contains multiple RB sets and includes S-SSB slot(s), e.g., whether to transmit S-SSB repetitions in more than one RB set, etc.

Agreement

RAN1 further study the followings:

- Whether any updates on power control are necessary considering PSD limit in unlicensed spectrum regulation.

## RAN1#112b-e (April 17 - 26, 2023)

Agreement

Considering PSD limit in unlicensed spectrum regulation, RAN1 further study whether updates on power control is needed especially for PSFCH.

Agreement

Regarding PSFCH transmission with 15 kHz and 30 kHz SCS:

- RAN1 down-select one of followings in RAN1#113:

  - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

    - K3 is (pre-)configured, FFS value range

    - On the K3 dedicated PRB(s), multiple CS pairs can be used as in legacy NR SL PSFCH transmission

    - When a PRB of common interlace and a dedicated PRB locate within the same 1 MHz bandwidth, UE only transmits on the dedicated PRB

      - FFS: whether any impact on meeting OCB requirement

  - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

  - Alt 3-2b: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set

    - K2=2

    - K4 is (pre-)configured, FFS value range

    - FFS: how to meet PSD limitation

    - FFS: whether to introduce any restrictions on the locations of K4 dedicated PRB(s) and/or K2 common PRBs, e.g., whether/how they are on the same interlace

- R16 NR SL PSSCH-PSFCH mapping is reused as baseline, FFS details

  - Note: companies are encouraged to give more details and analyze the specification impact

    - E.g., whether PSSCH transmissions on non-overlapped resources are mapped to non-orthogonal PSFCH resources, i.e., whether PSFCH collision may happen and whether/how to address it, etc.

    - E.g., whether introducing more than 6 CS pairs is needed

    - E.g., for group cast option 2, what’s the maximum group size that can be supported

    - E.g., how to support “more than 1 PSFCH occasion(s) per PSCCH/PSSCH”

- FFS: regardless of which Alt above is selected, whether or not to support PRB-level cyclic shift hopping as in NR-U to reduce PAPR

- FFS: whether IBE issue exists and whether/how to address it

  - E.g., whether to introduce guardband PRB/RE between common PRB and dedicated PRB

Working assumption

Additional candidate S-SSB occasions are excluded from resource pool

Agreement

A SL-BWP is (pre-)configured with either contiguous RB-based or interlace RB-based PSCCH/PSSCH transmission, i.e., not both.

Agreement

For interlace RB-based PSCCH/PSSCH transmission in SL-U, considering 1 sub-channel equals K interlace(s), support the followings:

- Option A:

  - TBS is determined based on a reference number of PRBs of one interlace within 1 RB set (denoted as N_ref), down-select one of the followings in RAN1#113:

    - Option A1: N_ref is a fixed value, e.g., 10, 11

    - Option A2: N_ref is (pre-)defined

      - e.g., N_ref is the average number of PRBs per interlace, which is determined by total number of PRBs of the RP divided by the number of interlaces.

    - Option A3: N_ref is (pre-)configured

    - Option A4: N_ref is dynamically indicated by Tx UE

    - Note: The number of PRBs within a sub-channel can be different among sub-channels in a single resource pool subject to (pre-)configuration.

    - FFS: for TBS determination, whether/how to handle the impact of additional available  PRB(s) in intra-cell guard band(s) for PSCCH/PSSCH transmission across multiple RB sets

Agreement

Regarding more than 1 PSFCH occasion per PSCCH/PSSCH transmission, support the followings:

- One PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s) via (pre-)configuration

  - FFS value range of N

  - FFS detailed design of such N associated candidate PSFCH occasion(s)

    - E.g., they are in different slots of the same RB set, or in different RB sets of the same slot, or combination thereof, etc.

    - E.g., whether PSSCH transmission and its related PSFCH occasion(s) are in the same RB set(s)

- FFS: whether to support that COT initiating UE can dynamically indicate which subset of the (pre-)configured PSFCH occasions within its COT are available for PSFCH transmissions.

  - FFS: whether other associated candidate PSFCH occasion(s) within its COT are used for PSSCH transmissions, and applicable scenarios.

  - FFS: whether AGC issue and PSFCH/PSSCH collision issue exist, and whether/how to address them

- FFS other details

  - E.g., how to meet the HARQ RTT restriction

  - E.g., UE behavior on transmitting PSFCH

  - E.g., how to avoid the risk of losing the COT if the COT is interrupted by periodic PSFCH occasions

Agreement

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings:

- Option A: Support that for one PSSCH transmission, the used interlace index(s) in different used RB sets are always the same

- Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s)

  - Frequency domain resource of PSSCH transmission is determined by an intersection of the resource blocks of the indicated sub-channel(s) and the union of the indicated set of RB sets and intra-cell guard bands between the indicated RB sets, if any

  - For a TB, the initial transmission and reservation of the resource(s) for retransmission(s) use the same number of sub-channel(s) and same number of RB set(s)

    - FFS: whether additionally support different number of RB set(s) in such case while keeping total number of sub-channels unchanged between initial transmission and retransmission(s) for a TB

  - Use X bits for indicating sub-channel index(s), and use Y bits for indicating contiguous RB set index(s)

    - R16 NR SL FRIV is reused as baseline

    - FFS details, e.g., signaling design, bit size, whether to consider bitmap design, whether/how the used interlace(s) can be non-contiguous, etc.

- FFS others

  - E.g., considering one PSSCH transmission may occupy one or multiple RB sets, whether or not to re-define single-slot candidate resource, and update resource selection and/or signaling from MAC to PHY, etc.

Agreement

For contiguous RB-based PSCCH/PSSCH transmission in SL-U, regarding sub-channel(s) which include intra-cell guardband PRBs, down-select one or more of the followings in RAN1#113:

- Option 2: Such sub-channel(s) can be used for PSCCH/PSSCH transmission

  - Note: PRBs within intra-cell guard band are not used for PSCCH transmission as per previous agreement

- Option 3: Such sub-channel(s) cannot be used for PSCCH transmission, and can be used for PSSCH transmission

- FFS details, e.g., conditions to apply the above Option(s)

- FFS impacts on definition of candidate resource, and resource selection

Agreement

If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- TBS is determined based on a reference number of symbols (denoted as L_ref)

- Support the followings

  - Alt 1: Support Option 4 only

- Note: the options are as below

  - Option 4: The reference number of symbols is determined by (pre-)configuration

- FFS details, e.g., in TS 38.214 Clause 8.1.3.2, whether L_ref replaces Nsymbsh or sl-LengthSymbols or other usage of L_ref, whether NsymbPSFCH is determined in the same way as in legacy NR SL, etc.

Agreement

For interlace RB-based PSCCH/PSSCH transmission in SL-U, regarding details of mapping between sub-channel and interlace:

- In a resource pool with multiple RB sets, sub-channel with the same index is mapped to K interlace(s) with the same index(s) in different RB sets.

- In a resource pool, support the following

  - At least for the agreed case where one SL resource pool can be (pre-)configured to include integer number of RB sets

    - Option 2: sub-channel#0 is mapped to K interlace(s) starting from interlace#0

      - sub-channel#1 is mapped to K interlace(s) starting from interlace#K, and so on

      - At least support that the above K interlace(s) are contiguous

        - FFS: whether/how to support the above K interlace(s) are non-contiguous

  - FFS: if RAN1 agrees to support that one SL resource pool can be (pre-)configured to include sub-set of PRBs of one RB set, the mapping between sub-channel and interlace for this case will be further discussed

- Interlace is indexed as per NR-U

Agreement

When the SL-BWP contains multiple RB sets, study the followings:

- When UE attempts to transmit S-SSB in a S-SSB occasion (e.g., R16/17 S-SSB occasion, R18 additional candidate S-SSB occasion)

  - Alt 1: UE may transmit S-SSB repetition in more than one RB set

    - FFS details, e.g., location of such S-SSB repetition(s) (e.g., (pre-)configured and/or pre-defined), whether/how to address potential power reduction and/or potential fluctuation of PSBCH-RSRP

    - FFS: the relationship with UE’s COT

    - FFS: the scenario that UE may or may not transmit S-SSB repetition in more than one RB set

    - Note: whether UE can transmit S-SSBs over non-contiguous RB sets is subject to RAN4’s reply, details can be found in RAN1’s LS to RAN4 in R1-2304218

Agreement

For S-SSB transmission within 1 RB set, for 15 kHz and 30 kHz SCS, Alt6 is supported:

- Alt 6: Support both Option 3-1(revised) and Option B, and enable one of them by (pre-)configuration

Note: the Options are as below

- Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH

  - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set, e.g. whether 1 or 2 interlaces will be used for S-SSB

- Option 3-1(revised): Transmit legacy S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement

  - FFS the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), whether/how to reduce PAPR.

  - FFS gap of 0

- Option A: Legacy S-SSB

  - Continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS for OCB exemption.

- Option B: Legacy S-SSB

  - RAN1 does not pursue further study on how to meet the minimum 2 MHz requirements under 15 kHz SCS for OCB exemption.

Note: Option A and B are applicable in region with no OCB requirement, or with OCB exemption.

Conclusion

Regarding additional candidate S-SSB occasions, in the same S-SSB period, UE can attempt to transmit on additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s).

Agreement

For interlace RB-based PSCCH/PSSCH transmission in SL-U, support the following:

- Option 1: lowest sub-channel is the sub-channel with smallest sub-channel index

Agreement

Regarding Tx UE behavior, at least when it initiates a COT:

- For the 1st slot of a COT, the Tx UE chooses the earliest starting symbol for PSCCH/PSSCH transmission after clearing LBT.

  - Note: in the same slot, Tx UE can use the 2nd starting symbol only if LBT fails at the 1st starting symbol

- FFS: whether/how to support that for the remaining slots of a COT, the Tx UE only chooses the 1st starting symbol for PSCCH/PSSCH transmission.

  - FFS applicable scenarios

    - e.g., at least for MCSt with no greater than 16us gap

    - e.g., at least for transmission with no greater than 16us gap from the previous transmission by any UE

  - FFS: Rx UE behavior

FFS: COT sharing case