3GPP TSG RAN WG1 #113			R1-230nnnn

Incheon, Korea, May 22nd – May 26th, 2023

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes for 9.16.16

Agenda Item:		9.16.16

Document for: 	Endorsement

- 9.16.16	Others

Including UE features for endorsed TEI proposals.

Agreement

  - Introduce FG55-3 for multiple PUSCHs scheduling by single DCI for non-consecutive slots in FR1

| 55. TEI18 | 55-3 | Multiple PUSCHs scheduling by single DCI for non-consecutive slots in FR1 | 1. Multi-PUSCH scheduling by single DCI format 0_1 for the operation with non-contiguous allocation | [10-17] | Yes | N/A | For operation on FR1, scheduling multiple PUSCHs by a DCI format 0_1 in non-contiguous slots is not supported. | Per Band | N/A | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

  - FG 10-17 is confirmed as the prerequisite FG of FG 55-3

R1-2304416	UE feature for endorsed TEI-18 on single DCI scheduling multiple PUSCHs	Ericsson, Qualcomm

R1-2305729	On UE features for TEI-18	Nokia, Nokia Shanghai Bell

R1-2305945	UE feature for non-contiguous multi-PUSCH scheduling with single DCI 0_1 in FR1	Huawei, HiSilicon