3GPP TSG RAN WG1 #113			R1-230nnnn

Incheon, Korea, May 22nd – May 26th, 2023

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes for 9.16.9

Agenda Item:		9.16.9

Document for: 	Endorsement

- 9.16.9	UE features for MC enhancements

Proposal 2-1:

- Component 2 in FG 49-1/49-2 is confirmed as: Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is PCell or an SCell if set of cells includes only SCells.

- Add notes in FG 49-1/49-2:

  - When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell

  - For any scheduled cell in any set of cells configured for multi-cell scheduling, UE is not expected to be configured with more than one scheduling cell to monitor PDCCH candidates for the scheduled cell, regardless of the DCI format

Agreement

- Candidate value set for component 3 in FG 49-1/49-2 is confirmed as: {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2}

Agreement

- Component 3 is revised as: Scheduling cell and co-scheduled cells have same SCS/carrier type: value set: {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2}, UE reports one or multiple of values from the value set

Agreement

- Reporting type of FG 49-1/49-1b/49-2/49-2b is per BC

Proposal 2-3:

- “FFS whether this component is reported per reported value in component 3” is removed from component 4 in FG 49-1/49-2

- “FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b” is removed from component 4 in FG 49-1b/49-2b

Proposal 2-4:

- Component 4 in FG 49-1/49-1b is revised as: Max number of [co-scheduled] cells in a DL cell list for the UE is reported with candidate value set of {2, 3, 4}

  - FFS whether to introduce another component for Max number of co-scheduled cells by a DCI format 1_3 for the UE is reported with candidate value set of {2, 3, 4}

- Component 4 in FG 49-2/49-2b is revised as: Max number of [co-scheduled] cells in a UL cell list for the UE is reported with candidate value set of {2, 3, 4}

  - FFS whether to introduce another component for Max number of co-scheduled cells by a DCI format 0_3 for the UE is reported with candidate value set of {2, 3, 4}

Proposal 2-5:

- Component 5 in FG 49-1/49-1b/49-2/49-2b is confirmed as: Max number of sets of cells supported by UE [per PUCCH group]: Candidate value set of {[1, 2, 3, 4]}

  - “FFS whether to separately report for primary and secondary PUCCH cell groups” is removed from component 5 in FG 49-1/49-1b/49-2/49-2b

  - “FFS whether this component is reported per reported value in component 3” is removed from component 5 in FG 49-1/49-2

  - “FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b” is removed from component 5 in FG 49-1b/49-2b

- Add component 5a in FG 49-1/49-1b/49-2/49-2b as: Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, …, 8}

Proposal 2-6:

- Component 6 in FG 49-1/49-1b/49-2/49-2b is confirmed as: Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {[1, 2, 3, 4]}

  - “FFS whether this component is reported per reported value in component 3” is removed from component 6 in FG 49-1/49-2

  - “FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b” is removed from component 6 in FG 49-1b/49-2b

Proposal 2-7:

- Followings are removed from FG 49-1/49-2

  - “[Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether this component is reported per reported value in component 3]”

  - “[Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether this component is reported per reported value in component 3]”

- Followings are removed from FG 49-1b/49-2b

  - “[Max total number of cells, across different sets of cells, supported by UE per PUCCH group: Candidate value set of {[2, 3, …, 16]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b]”

  - “[Max total number of cells, across different sets of cells, supported by UE for a same scheduling cell: Candidate value set of {[2, 3, …, 8]}, FFS whether to report separately for the reported combinations between scheduling and scheduled cells in components 3a/3b]”

Proposal 2-8-a:

- “FFS Type 2 HARQ codebook” is removed from component 7 in FG 49-1/49-1b

- Confirm FG 49-5 as follows

| 49. NR_MC_enh | 49-5 | Type 2 HARQ CB support for DCI format 1_3 | HARQ feedback based on Type 2 HARQ codebook for PDSCHs scheduled by DCI format 1_3 | At least one of {49-1, [49-1a], 49-1b} | Yes |  | UE does not support HARQ feedback based on Type 2 HARQ codebook for PDSCHs scheduled by DCI format 1_3 | [Per UE] | No | No | No |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2-9:

- Component 9 in FG 49-1/49-1b is confirmed as: Supported types Type-2 for ‘Antenna port(s)’ field: Candidate value set of {Type-2, Type 1A and Type-2}

- Component 8 in FG 49-2/49-2b is confirmed as: Supported types Type-2 for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields: Candidate value set of {Type-2, Type 1A and Type-2}

- Introduce following FG

| 49. NR_MC_enh | 49-4c | Type-1A for ‘Antenna port(s)’ field | 1) Support Type-1A for ‘Antenna port(s)’ field for DCI format 1_3 2) Support Type-1A for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields for DCI format 0_3 | At least one of {49-1, 49-1b, 49-2, 49-2b} | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2-11:

- “and FFS for the case when same SCS but different carrier types between scheduling cell and set of cells” is removed from FG 49-1/2

Proposal 2-12-b:

- Following component is introduced in FG 49-1

  - The number of unicast DCI to process for a set of cells configured for multi-cell PDSCH scheduling by a DCI format 1_3

    - One unicast DCI per slot of scheduling cell for the set of cells for FDD/TDD scheduling cell

  - FFS whether to introduce advanced UE capability for larger number of unicast DCI

  - FFS whether to count DCI format 1_3 only or both legacy DCI formats and DCI format 1_3

- Following component is introduced in FG 49-2

  - The number of unicast DCI to process for a set of cells configured for multi-cell PUSCH scheduling by a DCI format 0_3”

    - One unicast DCI per slot of scheduling cell for the set of cells for FDD scheduling cell

    - Two unicast DCI per slot of scheduling cell for the set of cells for TDD scheduling cell

  - FFS whether to introduce advanced UE capability for larger number of unicast DCI

  - FFS whether to count DCI format 0_3 only or both legacy DCI formats and DCI format 0_3

Proposal 2-13:

- Add a component in FG 49-1/49-1b: Monitoring SS set(s) for DCI formats 0_0/1_0 on PCell/PSCell for which the UE is configured to monitor SS set(s) for DCI format 1_3

- Add a component in FG 49-2/49-2b: Monitoring SS set(s) for DCI formats 0_0/1_0 on PCell/PSCell for which the UE is configured to monitor SS set(s) for DCI format 0_3

  - FFS Monitoring SS set(s) for DCI formats 0_1/1_1/0_2/1_2 on PCell/PSCell

  - FFS Monitoring SS set(s) on SCell

  - FFS whether scheduled cell by legacy DCI format(s) is {reference cell only, any-cell}

  - FFS scheduling cell by DCI format 0_3/1_3 is within the set of cells or outside the set of cells

  - FFS reporting granularity {per reported value in component 3, others}

[Stable] Proposal 2-15:

- Candidate value set for component 3b in FG 49-1b/49-2b is updated as: [Bitmap] Indication of support/not support for each of applicable combinations of scheduling cell from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} and scheduled cells from {FR1 licensed FDD, FR1 licensed TDD, FR1 unlicensed TDD, FR2-1, FR2-2} from the band combinations

- “FFS: relation between 3a and 3b” is removed from component 3a/3b in FG 49-1b/49-2b

- “FFS: whether/how to indicate support of scheduling on unlicensed band(s)” is removed from component 3a/3b in FG 49-1b/49-2b

Proposal 2-16-b:

- Following component is introduced in FG 49-1b

  - The number of unicast DCI to process for a set of cells configured for multi-cell PDSCH scheduling by a DCI format 1_3

    - For low-to-high SCS

      - One unicast DCI per slot of scheduling cell for the set of cells for FDD/TDD scheduling cell

    - For high-to-low SCS

      - One unicast DCI per N consecutive slots of scheduling cell for FDD/TDD scheduling cell, where

        - N = 2 for (30, 15), (60, 30), (120, 60)

        - N = 4 for (60, 15), (120, 30)

        - N = 8 for (120, 15)

        - [N = 16 for (240, 15), (480, 30)]

        - [N = 32 for (480, 15)]

  - FFS whether to introduce advanced UE capability for larger number of unicast DCI

  - FFS whether to count DCI format 1_3 only or both legacy DCI formats and DCI format 1_3

- Following component is introduced in FG 49-2b

  - The number of unicast DCI to process for a set of cells configured for multi-cell PUSCH scheduling by a DCI format 0_3

    - For low-to-high SCS

      - One unicast DCI per slot of scheduling cell for the set of cells for FDD/TDD scheduling cell

      - Two unicast DCI per slot of scheduling cell for the set of cells for TDD scheduling cell

    - For high-to-low SCS

      - One unicast DCI per N consecutive slots of scheduling cell for FDD/TDD scheduling cell, where

        - N = 2 for (30, 15), (60, 30), (120, 60)

        - N = 4 for (60, 15), (120, 30)

        - N = 8 for (120, 15)

        - [N = 16 for (240, 15), (480, 30)]

        - [N = 32 for (480, 15)]

  - FFS whether to introduce advanced UE capability for larger number of unicast DCI

  - FFS whether to count DCI format 0_3 only or both legacy DCI formats and DCI format 0_3

Proposal 2-17:

- Introduce following FG

| 49. NR_MC_enh | 49-4a | Nominal RBG size of Configuration 3 for FDRA type 0 | 1) Support of nominal RBG size of Configuration 3 for FDRA type 0 | At least one of {49-1, 49-1b, 49-2, 49-2b} | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2-18:

- Introduce following FG

| 49. NR_MC_enh | 49-4b | FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 1_3/0_3 | 1) Support of FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 1_3/0_3 | At least one of {49-1, 49-1b, 49-2, 49-2b} | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- FG 49-X is updated as follows

| 49. NR_MC_enh | 49-X | Supported switching option for each band pair in the band combination for UL Tx switching across more than 2 bands | Indicate supported switching option for each band pair in the band combination for UL Tx switching across more than 2 bands Candidate value set is {switchedUL, dualUL, both} |  | Yes |  | [UL Tx switching across more than 2 bands cannot be supported for the band pair in the band combination] | [Per band pair per band combination, details up to RAN2] | [N/A] | [N/A] | [N/A] | This FG is based on the following agreements. RAN1 will not discuss the detail of this FG and the detail is up to RAN2 [Agreement Ask RAN2 to consider following alternatives for UE capability reporting about the supported UL Tx switching options - Alt.1: report {switchedUL, dualUL, both} for each band pair in the band combination] [Agreement in RAN2#121 For UE capability of switching options, introduce a per-band-pair UE capability to report supported switching options for Rel-18 UL Tx switching.] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement

- If the UE reports 0us in FG 49-Y, the minimum separation time is not applied

  - FFS the consequence if UE does not report FG 49-Y

Ad-hoc chair’s note

- RAN1 does not send reply LS in R1-2304333 from RAN2 in RAN1#113

R1-2304513	Discussion on UE features for Multi-carrier enhancements	vivo

R1-2304584	Discussion on UE features for multi-carrier enhancement	Spreadtrum Communications

R1-2304603	Discussion on UE feature for MC enhancements	ZTE

R1-2304671	UE features for MC enhancements	Huawei, HiSilicon

R1-2304891	Disucssion on UE features for MC enhancements	xiaomi

R1-2305149	Discussion on UE features for MC enhancements	LG Electronics

R1-2305280	On UE features for Rel-18 multi-carrier enhancements	Apple

R1-2305373	UE features for MC enhancements	Qualcomm Incorporated

R1-2305471	Discussion on UE features for multi-carrier enhancement	OPPO

R1-2305545	UE features for multi-carrier enhancements	Samsung

R1-2305578	UE features for MCE	Ericsson

R1-2305624	Discussion on UE features for Multi-carrier enhancements	NTT DOCOMO, INC.

R1-2305661	On UE feature discussion for Rel-18 MC enhancements	MediaTek Inc.

R1-2305724	On UE features for Multi-carrier Enhancements	Nokia, Nokia Shanghai Bell