3GPP TSG RAN WG1 #114	R1-2306875

Toulouse, France, August 21-25, 2023

Agenda item:		9.3.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On subband non-overlapping full duplex for NR

Document for:		Discussion and Decision

# Introduction

RAN has agreed a new Rel-18 Study Item on Evolution of NR Duplex Operation with the following objectives [1]:

The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

In this contribution, we discuss open issues related to subband non-overlapping full duplex (SBFD). The performance of SBFD is discussed in Section 2.1. Section 2.2 includes system design considerations and our view on the enhancements required to enable optimized SBFD operation, also based on the discussions and agreements from previous RAN1 meetings. Finally, our proposals and observations are summarized in Section 3.

# Discussion

## SBFD performance

Simulation results for co-channel FR1 Urban Macro (UMa) and FR1 and FR2 Indoor Office scenarios have been presented in detail in our companion contribution R1-2306874 [2]. The key simulation assumptions and observations are included below:

FR1 UMa

For FR1 UMa, the simulations consider the agreements up to RAN WG1 #113 meeting. Deployment Case 1 is considered where one single operator is simulated and all the cells belonging to the operator use SBFD operation with the same SBFD subband configuration. For the comparison between TDD, we considered a DDDSU frame configuration, and for SBFD, a XXXXX with X denoting a SBFD with ~20% UL RBs; corresponding to ‘Alt 4’ agreed by RAN1 where the goal is to have the same UL/DL resource ratio between Legacy TDD and SBFD. With respect to gNB antenna configurations, an antenna array of (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) is assumed for TDD, while for SBFD we considered Option 2 (twice area than TDD with same TxRUs) and Option 3 (same area than TDD with half TxRUs).

Regarding the inter-sector isolation, the following settings are considered:

- Setting 1 - Optimistic: 93 dB isolation

- Setting 2 - Realistic: 75 dB isolation

The results are depicted in Figure 1 below, which shows the average DL and UL UPT for 5th, 50th and 95th percentile with low load, medium load and high load for the UMa FR1 scenario with the settings described above. Figure 1 highlights the effect of the SBFD BS antenna configuration, SBFD inter-sector isolation assumptions and the impact of UE transmit power. We consider two options for the UE transmit power: the default option, with 23 dBm, and the performance of PC2, with 26 dBm tx power. The 3dB power boost is possible when the UL duty cycle is less than 50 %. In the results presented here, we consider the BS ICS equal to 62 dB.

The results in Figure 1 show that the DL performance is either maintained or degraded by SBFD when compared to static TDD. In general, SBFD performs much worse than static TDD due to the presence of strong UE-UE CLI in the UMa cluster scenario. UL coverage limited UEs generally transmit over a few resource blocks with full power, resulting in a very high in-band emission (IBE) and blocking nearby UEs.

Considering the UL performance, the only case in which the SBFD performance surpasses the static TDD performance (when the UE transmits with 26 dBm) is in the 5th percentile of low traffic load. In medium and high loads, the performance assessed in all percentiles show that static TDD, with either 23 dBm or 26 dBm, always outperforms SBFD.

Figure 1: Average DL and UL UPT for static TDD and different configurations of SBFD for FR1 UMa cluster scenario

With clustered UE distribution in UMa Scenario, the presence of co-channel inter-sector and co-channel inter-gNB inter-subband CLI impacts the SBFD performance and hinders any gain on the UL UPT in medium and high loads with respect to static TDD. Gains are only observed in the 5th percentile UL UPT for a low traffic load scenario, when SBFD is compared to static TDD with 23 dBm transmit power, in which the SBFD UL throughput is ~4.5 times larger than the static TDD UL UPT. However, the improvement is only observed when assuming optimistic inter-sector isolation conditions, and the same antenna gain. In this scenario, static TDD with PC2 UEs achieves 80% of the gain that SBFD provides without any degradation in the DL UPT performance. In all other scenarios, static TDD with PC2 UEs outperforms SBFD.

With clustered UE distribution in UMa Scenario, there is significant degradation of UE DL throughput due to UE-UE CLI even at low load. This mainly occurs when one or more coverage limited UEs transmit over a few, e.g. 4, RBs with full 23 dBm UL transmit power which generates large amount of UL leakage interference to other UEs receiving in DL.

FR1 Indoor Office

For the FR1 Indoor Office scenario, the performance considering different payload sizes was investigated in our companion contribution[2], the main results and discussion are summarized below. In the results shown in we consider two alternatives for the payload size, small and large, and two SBFD alternatives:

- Alt-2: SBFD XXXXU, TDD DDDSU

- Alt 4: SBFD XXXXX, TDD DDDSU

Figure 2 - FR1 Indoor Office DL and UL UPT performance with large packet (0.5MB for DL and 0.125MB for UL) and BS ICS of 46 dB.

Considering a large payload size (125 kB in the UL and 500 kB in the DL), SBFD Alt-4 (XXXXX) shows slightly worse UL user perceived throughput (UPT), despite of providing UL opportunities in every slot, reporting an average of just above 1% performance degradation as compared to static TDD. The main reason for this degradation is the fact that the FTP3 payloads can actually be transmitted faster (i.e. with lower latency) with TDD than with SBFD, since an entire slot is dedicated to UL. SBFD Alt-2 (XXXXU), on the other hand, provides a significant improvement on the UL throughput, between 50% and 70%, at the cost of DL UPT degradation, of approximately 20% at low and medium load and 70% in the 5%-ile DL average UPT at high loads. SBFD Alt-2 provides more UL opportunities than SBFD Alt-4 and static TDD. In this Figure, we also provide results for dynamic TDD. Dynamic TDD has a comparable performance to static TDD in the DL, but a remarkable UL UPT gain, between 100% and 150% as compared to static TDD and SBFD Alt-4.

For Deployment Case 1 FR1 Indoor Office scenario and large packet size:

- SBFD XXXXX and static TDD DDDSU provide almost identical UL and DL performance.

- SBFD XXXXU provides 50% to 70% UL UPT improvement at the expense of a DL UPT degradation of approximately 20% at low and medium load, and up to 70% in the 5%-ile DL average UPT at high loads.

- For low and medium loads, dynamic TDD provides a UL UPT gain between 100% to 150% as compared static TDD and SBFD XXXXX, and up to 65% gain over SBFD XXXXU. Contrary to SBFD XXXXU, dynamic TDD does not come at the expense of significantly worse DL average UPT performance.

For a scenario with small payload size (1 kB in the UL and 4 kB in the DL), contrary to the large payload results, SBFD gives an overall improvement of UL throughput compared to static TDD of at least 50%, in both SBFD Alt-2 and Alt-4, without degradation in the DL. In this case, a 3% to 10% DL UPT performance improvement is observed in SBFD Alt-2 when compared to static TDD. The reason for the UL UPT performance improvement is that the transmission of the entire 1 kB payload can fit a single slot for both SBFD and TDD. The main difference is the UL resource availability and the impact on the queuing delay. In SBFD, an UL packet is transmitted almost immediately in the case of SBFD, while there is generally some waiting time in the case of TDD UL (an average of 2 slots to get the next UL scheduling opportunity). This problem becomes even larger for the high load cases in which packets are queued for more than 1 full radio frame before they get served in TDD.

Figure 3: FR1 Indoor Office DL and UL UPT performance with small packet (4 kB for DL and 1 kB for UL) and BS ICS of 46 dB.

For Deployment Case 1 FR1 Indoor Office scenario and small packet size:

- SBFD XXXXX and XXXXU provides approximately 50% higher UL 5%/50%/95%-ile UL UPT performance compared to static TDD.

- SBFD XXXXX provides also a small 3% to 10% DL UPT gain over static TDD, while SBFD XXXXU performs worse than static TDD in DL in most of the cases.

FR2 Indoor Office

For the FR2-1 Indoor Office scenario, different payload sizes were considered and discussed in [2], Here, we focus on the discussion of the scenario with small payload size (1 kB in the UL and 4 kB in the DL). We focus the results on SBFD Alt-4 (XXXXX)

Similar as for the FR1 InH scenario, the SBFD benefits are highly visible in UL for the small payload size. Both the UL UPT and the UL packet latency are improved. The reason for this is a combination of the following: good link budget that results in high SINR, sufficient self-interference isolation, 100% availability of UL resources at any SBFD slot and UL packets mostly fitting into 1 radio slot.

In downlink, as shown in Figure 4, we noted that SBFD provides very similar performance DL UPT and DL packet latency for low and medium loads. The reason is that, in most of the cases, 1 slot is enough to transmit the 4 kB DL packet. The benefit of SBFD comes from the 100% availability of DL resource at any slot. At high loads, degradation is visible for SBFD due to the presence of inter-subband UE-to-UE CLI, whose power is higher than observed legacy DL interference. This is especially noticeable for those UEs with lowest SINR, and it is shown in the 5%-ile DL UPT.

Figure 4 - FR2-1 Indoor Office DL and UL UPT performance with small packet (4 KB for DL and 1KB for UL).

For Deployment Case 1 FR2 Indoor Office scenario and small packet size:

- SBFD offers up to 50% UL throughput improvement for all load conditions.

- In the DL direction, SBFD outperforms static TDD most of the time by a small margin of around 5%.

Dynamic SBFD

The final set of results presented here comprises the evaluation of dynamic SBFD in an FR1 indoor office scenario, and compares it with the performance obtained with static TDD DDDSU, semi-static SBFD Alt-2 and dynamic TDD.

The following was agreed in RAN1 #112:

Agreement: For dynamic SBFD, For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in a symbol configured as DL in TDD-UL-DL-ConfigCommon based on the following options: Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) and UL transmissions outside semi-statically configured UL subband are allowed or not in the symbol configured as flexible in TDD-UL-DL-ConfigCommon based on the following options: Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed and UL transmissions outside semi-statically configured UL subband are not allowed Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed UL transmissions outside the semi-statically configured UL subbands are not allowed Option 3: DL receptions outside semi-statically configured DL subband(s) are allowed UL transmissions outside the semi-statically configured UL subbands are allowed Dynamic SBFD should be compared with dynamic TDD and/or semi-static SBFD in terms of performance, implementation complexity, switching latency. For each option, additional conditions may apply to determine whether the option is applicable.

In the dynamic SBFD implementation, ‘XXXXU’ is assumed as the baseline SBFD frame structure, as in Option 2 from the RAN1 agreement in RAN1 #112. Prior to the start of each ‘X’ slot, in the absence of UL traffic and presence of DL traffic, the RB configuration for the upcoming slot is configured as DL-only (X → D). Otherwise, ‘X’ corresponds to a SBFD slot with <ND, NU, NG >=<104, 55, 5>; the UL/DL ratio in SBFD slot is fixed regardless of the traffic volume.

This implementation corresponds to the case where a SBFD system with XXXXU can fallback to using DL heavy subframes as in legacy TDD (e.g. DDDDU is used in the absence of UL traffic during four consecutive slots).

The results show that in UL, we see identical performance to semi-static SBFD XXXXU and over 70% improvement over semi-static SBFD XXXXX and static TDD DDDSU. In DL, the benefits of dynamic SBFD come to light as it provides over 35% improvement compared to semi-static SBFD with XXXXU and 8% gain over semi-static XXXXX. In other words, dynamic SBFD Option 2 allows to significantly improve the UL performance to the same level as XXXXU without any penalty in DL performance. Note that some improvement over TDD DDDSU is observed in DL which is mainly a consequence of not modelling the guard symbols between DL to UL transition in the XXXXU dynamic SBFD case.

The dynamic SBFD results in a FR-1 indoor office scenario show that dynamic SBFD provides a DL performance improvement of 35% when compared to a semi-static SBFD configuration (XXXXU). In the UL, the results show that the performance of dynamic SBFD is comparable to the performance of semi-static SBFD (XXXXU) and provides over 70% improvement over semi-static SBFD XXXXX and static TDD DDDSU. In other words, dynamic SBFD provides comparable UL performance to semi-static SBFD, but without any degradation to the DL performance, when compared to static TDD.

The dynamic SBFD results in a FR-1 indoor office scenario show that in low load and medium load, the DL average UPT of Dynamic SBFD outperforms Dynamic TDD. When the UL average UPT is compared, dynamic SBFD only outperforms dynamic TDD in high load.

Figure 5 - FR1 Indoor Office DL and UL UPT performance with dynamic SBFD and other duplexing schemes. Large packet is assumed (0.5MB for DL and 0.125MB for UL).

## System design considerations & SBFD enhancements

### SBFD configuration

#### Semi-static and dynamic SBFD configuration

One issue that was discussed in the past meetings was dynamic SBFD, in which the semi-statically configured UL subband in a symbol or slot that is configured as DL or flexible in TDD-UL-DL-ConfigCommon can be dynamically disabled. In RAN1 #112bis[3], the following options were discussed, but not agreed:

### Proposal 1-8f Proposed Agreement: Study at least the following options for dynamic SBFD. - Option 1: Dynamic SBFD is achieved by scheduling DCI which is used to indicate whether the RBs in flexible subband where RBs in flexible subband can be dynamically are used for UL transmission and or DL transmission. - FFS definition of flexible subband, e.g. flexible subband is defined as 1 RB or a set of consecutive flexible RBs, which can be used for UL transmission, DL transmission, and guard band - FFS benefit of introducing flexible subband in addition to UL/DL subbands - Option 2: Dynamic SBFD is achieved by scheduling DCI which is used to determine whether DL receptions outside semi-statically configured DL subband and/or UL transmission outside semi-statically configured UL subband are allowed. - Option 3: Dynamic SBFD is achieved by non-scheduling DCI which indicates whether a symbol is SBFD symbol or not. Note: whether or not dynamic SBFD is beneficial from a performance perspective is a separate discussion

In our view, dynamically disabling the UL subband should be possible. As shown in the first part of this document, SBFD operation degrades the DL throughput, due to the partial allocation of DL resources to UL. If dynamic SBFD configuration in time is not possible, there is a risk that the network won’t be able to adapt to varying traffic and load conditions. It should be noted that disabling semi-statically configured UL subband in symbols configured as DL or Flexible in TDD-UL-DL-ConfigCommon corresponds to falling back to TDD, which is shown by several companies to provide, in specific circumstances, similar or even better performance as compared to SBFD

SBFD operation in symbols configured as DL improves the UL performance at the cost of DL performance. Dynamic adaptation of the frame structure based on traffic changing, for example, by converting SBFD symbols to DL-only, or allowing DL receptions outside the DL subband is beneficial, since it allows adaptation to the current traffic conditions by dynamically falling back to TDD.

Support dynamic indication of SBFD subbands in time-domain.

Regarding the three options discussed in RAN1 #112bis-e, in our view it is important that the network ensures that all the SBFD UEs understand whether the UL subband is active in each symbol or not to avoid collisions between UL and DL transmissions, as well as creating interference in different directions.

We do not see any benefit of Option 1. In our view, flexible symbols are already supported by the specification, and there is no gain in creating a flexible subband. We assume that it is not possible to configure a DL subband in an UL symbol, so assuming that the flexible subband could be configured in the middle of the band, in a DFD configuration for example, this symbol could only be used as SBFD (DUD), or DL-only, and this is already achievable by the two other options.

In Option 2, the symbol is not converted to a non-SBFD symbol, but the dynamic behavior is implemented by the scheduling DCI, for example, by allocating DL within the UL subband and guardbands. In our view, only the UEs that are dynamically scheduled in that symbol would be aware of the change, while other UEs with e.g. a persistent allocation (for UL transmission, or periodic DL receptions, such as CSI-RS) might not be aware of it, potentially causing collisions with dynamically scheduled transmissions. Another problem with Option 2 is that solutions based on rate matching around the UL subband and guardband(s) are being discussed for both type 1 and type 0 RA. For type 0 RA, partial use of an RBG overlapping with the DL subband boundary is being considered, while for type 1 RA, one of the options is that the UE is scheduled with a contiguous set of resources and applies rate matching around the UL subband and guardband(s), if any. In case Option 2 is agreed, there should be mechanisms for the UE to understand when resources overlapping with the DL or UL subbands in a SBFD symbols are to be rate matched, and when they are not.

In Option 3, in which the SBFD symbol is converted to a DL-only or UL-only symbol, in our view, there could be a lower layer signalling such as SFI or enhanced SFI, to support communicating to all the SBFD UEs that this symbol is converted to a DL-only/UL-only. The benefit of this alternative is that it ensures that the interference is controlled and that the right resources are used at a given time, i.e., collisions are avoided. In that case, this symbol may not be characterized as a SBFD symbol anymore, since there would not be different DL subband(s), UL subband and guardband(s) defined. This means, whatever is decided in terms of allowing or not-allowing DL/UL transmissions outside of DL/UL subbands in SBFD symbols configured as DL or Flexible in TDD-UL-DL-ConfigCommon, it does not apply to symbols that are dynamically converted to DL-only or UL-only symbols.

If the SBFD symbol is dynamically converted to a DL-only or UL-only symbol, it is not characterized by DL/UL subbands and guardband(s).

For a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon, support converting the SBFD symbol to DL-only symbol.

For a SBFD symbol configured as flexible in TDD-UL-DL-ConfigCommon, support converting the SBFD symbol to DL-only or UL-only.

Based on the above proposals, the frame structure signaling framework may need to be enhanced to support dynamic subband location in time domain, e.g. by providing a mechanism for dynamic adjustment of the sub-band configuration in line with traffic demands and using lower-layer signaling. For example, it could be possible to signal the frequency domain location of the UL subband via higher layer signaling, while the time domain location of the UL subband is provided with a combination of higher layer and lower layer signaling, the latter possibly being SFI or enhanced SFI. However, before RAN1 can further progress on the signaling framework, it must first reach consensus on the support for dynamic subband location in time (and/or frequency) domain.

For a SBFD aware UE in a symbol semi-statically configured as DL or Flexible in tdd-UL-DL-ConfigurationCommon, support mechanisms enabling dynamic configuration of the SBFD UL subband in the symbol.

Additionally, in previous meetings, there had been agreements on the UE behavior within DL subbands and UL subbands for SBFD operation in symbols configured as DL or flexible in TDD-UL-DL-ConfigCommon.In RAN1 #111, the following has been agreed for a UE configured with UL subband in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon:

Agreement For a SBFD aware UE semi-statically configured with UL subband in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study: · UL transmissions within UL subband are allowed in the symbol · UL transmissions outside UL subband are not allowed in the symbol · Frequency locations of DL subband(s) are known to the SBFD aware UE · The frequency location of DL subband(s) can be explicitly indicated or implicitly derived · DL receptions within DL subband(s) are allowed in the symbol Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol

Considering our Proposal 2: above, in which we discuss that the symbol can be converted to a DL-only symbol, we propose to update the agreement above as:

For a SBFD aware UE semi-statically configured with UL subband in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study:

- · UL transmissions within UL subband are allowed in the symbol

- · UL transmissions outside UL subband are not allowed in the symbol

- · Frequency locations of DL subband(s) are known to the SBFD aware UE

- · The frequency location of DL subband(s) can be explicitly indicated or implicitly derived

- · DL receptions within DL subband(s) are allowed in the symbol

- · The symbol can be converted to a DL-only symbol

- Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol

For a UE semi-statically configured with UL subband in a SBFD symbol configured as flexible, in RAN1 #112-bis-e the agreement was updated as:

Agreement: - Endorse the text proposal in R1-2303639 with the following update. 6.1.1.3 SBFD operation in symbols configured as flexible in TDD-UL-DL-ConfigCommon For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, the following optionsalternatives are studied for SBFD aware UEs, OptionAlt 1: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol - FFS: Whether DL receptions outside DL subband(s) are allowed or not in the symbol OptionAlt 2: - UL transmissions within UL subband are allowed in the symbol - The RBs outside the UL subband can be used as either UL, or DL excluding guardband(s) if used, in the symbol from gNB’s perspective, and the transmission direction for all those RBs is the same - FFS: SBFD aware UE behaviours - FFS: Whether or not signalling of guardband(s) is needed - FFS: Whether or not the symbol can be converted to a DL-only symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE DL receptions within DL subband(s) are allowed in the symbol

The first alternative is similar to the agreement reached for SBFD operation in symbols configured as DL. UL transmissions are allowed only within the UL subband, and the DL subband(s) frequency locations are known to the SBFD aware UE. There is still an FFS on whether DL receptions outside DL subband(s) are allowed or not in the symbol. With Alt 1, SBFD UEs cannot transmit in UL outside of the UL subband in symbols configured as Flexible in TDD-UL-DL-ConfigCommon, which limits the gNB flexibility as compared to legacy UEs. The second option, on the other hand, increases the SBFD operation flexibility by allowing all RBs outside of the UL subband to also be used for UL. The behavior in SBFD symbols outside the UL subband in Option 2 is similar to the behavior in legacy Flexible symbols, though it is not clear what “from gNB’s perspective” means in the agreement. It if means that RBs outside the UL subband can be used for UL by the gNB but not by SBFD aware UEs, then there is in practice no difference between Alt 1 and Alt 2. If RBs outside the UL subband can be used for UL by SBFD aware UEs, then there is no need to specify “from gNB’s perspective” in the agreement. Note that symbols configured as flexible in TDD-UL-DL-ConfigCommon can be used for DL or UL since Rel-15, and in this perspective, we think this should still be possible for SBFD operation in flexible symbols.

Symbols configured as flexible in TDD-UL-DL-ConfigCommon can be used for DL or UL since Rel-15.

Considering the options discussed in RAN1 #112-bis for SBFD operation in symbols configured as Flexible in TDD-UL-DL-ConfigCommon, we think it should be possible to convert the symbol to a DL-only or a UL-only symbol not to introduce unnecessary scheduling restrictions for SBFD-aware UEs as compared to legacy UEs . Therefore, our preferred alternative is Alternative 1 with the following update:

For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon:

- •	UL transmissions within UL subband are allowed in the symbol

- •	UL transmissions outside UL subband are not allowed in the symbol

- •	Frequency locations of DL subband(s) are known to the SBFD aware UE

- •	DL receptions within DL subband(s) are allowed in the symbol

- The symbol can be converted to a DL-only symbol or UL-only symbol

#### Enhancements to the current frame structure signalling

Next, we discuss the required enhancements to the current frame structure signaling. First, the signaling should guarantee backwards compatibility for legacy UEs, considering that the legacy UE may not be able to operate if the broadcast TDD configuration tdd-UL-DL-ConfigurationCommon included flexible symbols. To ensure legacy UE operation, the legacy time domain TDD UL/DL pattern e.g. DDDSU can be indicated by the gNB via tdd-UL-DL-ConfigurationCommon. For SBFD aware UEs, the gNB shall indicate the frequency and time location of the UL subband via either SIB or using dedicated RRC configuration.

As the configuration of the UL subband is cell specific, and as we support initial access in SBFD symbols (Section 2.2.2),  we think that signaling the frequency and time location of the UL subband via SIB should be assumed as baseline. Therefore, signaling via dedicated RRC can be considered as an additional (not alternative) option

Signaling of the frequency and time location of the UL subband via SIB is assumed as baseline. Signaling via dedicated RRC can be considered as an additional (not alternative) option.

When configuring the UL subband, the gNB indicates a set of UL resources (time and frequency domain) that converts a set of DL or flexible symbols in tdd-UL-DL-ConfigurationCommon in SBFD symbols by including a set of resource elements (REs, PRBs, etc.) that can be used for UL transmission, i.e. it creates a SBFD UL subband on DL (and flexible) symbols within the carrier/BWP. There are different options how this could be achieved, e.g.:

- a RE/symbol square consisting of two [symbol, PRB] tuples indicating respectively the starting and ending point (in both time and frequency domain) of the UL subband.

- a starting PRB and a starting symbol, together with a number of PRBs in frequency domain, and a number of symbols in time domain

  - the frequency/time square can be indicated in a similar way the PUSCH resource allocation is provided; a Resource Indicator Value (RIV) provides jointly a starting PRB and a number of PRBs within the active BWP or within the common RB (CRB) grid, and a corresponding Start and Length Indicator Value (SLIV) indicates the subband configuration in time domain

- a starting symbol and a frequency offset relative to the frequency allocated to CORESET0, and a number of PRBs in frequency domain and number of symbols in time domain. The frequency offset can be negative or positive relative to CORESET0.

In RAN1 #111 meeting, it has been agreed that for the purpose of this study, the understanding is that for semi-static configuration of subband frequency location, the frequency location of DL or UL subband is with reference to the common resource block (CRB) grid. This alternative allows the configuration of the UL/DL subbands in any part of the BWP, i.e., it allows for a large configurable range. However, in our view, not all PRBs are suitable for the configuration of UL subbands, such as the frequencies used for the allocation of the SSB or the CORESET0 (if allowed in SBFD symbols) and shouldn’t be considered for the allocation of the UL subband. This is the situation illustrated in Figure  below, in which SSBs are sent in the lower DL subband. Therefore, using the CRB as a reference for the UL subband could cause waste in the number of indicated bit resources compared to using a different reference, such as the PRBs used for the transmission of SSB or CORESET0, and an offset to configure the UL subband.

Not all PRBs are suitable for the configuration of the UL subband, for example, those configured for SSB or CORESET0 transmission (if allowed in SBFD symbols). Using the frequency allocated to the transmission of the SSB or CORESET0 as reference, and an offset for the configuration of the UL subband, can reduce the configurable range and number of bits used to indicate the UL subband to the UE.

Figure : SSB configuration in DL subband of SBFD symbol.

The frequency allocated to SSB or CORESET0 is used as reference for the configuration of the UL subband.

SBFD operation in SSB symbols

RAN1 #113 it was agreed to allow UL subband configuration in SSB symbols.

Agreement An UL subband can be configured in an SSB symbol. - Note: It is SSB from serving cell perspective, which can be CD-SSB or NCD-SSB. - Whether actual UL transmission can be done is for further discussion Agreement The following conclusion is to be captured in the TR If SBFD-aware UEs are not allowed to transmit in the SSB symbol but is allowed to receive within the DL BWP in the SSB symbol, negative impact on SSB detection and measurement can be avoided but UL performance may be degraded due to fewer UL opportunities. If SBFD-aware UE is allowed to transmit in the SSB symbol, the UE may only transmit UL in an UL subband depending on gNB scheduling, configuration, UE measurement or priority rule. There may be negative impact on SSB detection and measurement if the SBFD-aware UE is requested to transmit in the SSB symbol.

The first agreement lists the actual UL transmission during SSB symbols as for further discussion. As mentioned in our TDocs from previous RAN1 meetings, we have concerns with UL transmissions during SSB symbols. SSBs are used for serving cell and neighbor cells measurements by both SBFD aware UEs and legacy UEs. SSBs are broadcast signals periodically transmitted, which can increase the self-interference at the gNB if the UL subband is configured, and UEs are scheduled to transmit in this subband.

In case SBFD symbols overlap with SSB symbols, the potential UL transmissions by aggressor UEs in the UL subband could degrade the SSB based measurements performed by victim UEs (legacy or SBFD aware) in the DL subband due to UE-to-UE inter-subband CLI. Additionally, it is worth mentioning that SSBs can be used by the UE for automatic gain control (AGC). If the UE is subject to CLI while it is measuring SSBs, it can impact its AGC performance. To avoid degradation, in our view, it is important to protect not only the SSB symbols from UE-UE CLI, but also the symbols that are preceding the SSB symbols.  There are currently scheduling restrictions for UL transmissions during SSB symbols to be measured. TS 38.133 (for example, in clause 9.2.5.3) states that the UE is not expected to transmit on: SSB symbols to be measured, 1 data symbol before each consecutive SSB symbol to be measured and 1 data symbol after each consecutive SSB symbol to be measured.

In our view, the preferred behavior would be that the UEs are allowed to receive in DL within the UL subband, if the UL subband is configured in SSB symbols.

UL transmissions during SSB symbols are not allowed.

#### RB-set based and BWP based schemes

Also related to SBFD configuration, two main options have been discussed in the last meetings for the configuration of the time and frequency location of the UL/DL subbands to SBFD aware UEs: RB-set based and BWP based schemes.

- RB-set based: the gNB explicitly configures the time and frequency resource set within a carrier. The UE may observe DL and UL resources in a same symbol within a BWP.

- BWP based: This scheme relies on the Rel-15 NR BWP framework to ensure that the UE active UL and DL BWP is always aligned with the UL and/or DL subband within a slot or symbol so that the UE can only observe one direction (either UL or DL) per symbol within a BWP.

The RB-set based scheme does not require specific enhancements to the Rel-15 BWP framework. The frequency resources reserved for DL (or UL) can be modified from symbol to symbol and/or from slot to slot. There is no need for the UE to adapt its DL and/or UL BWP to the time-varying frequency resources. Essentially, the UE’s active BWP may include PRBs that are used by the serving cell for the opposite link direction (e.g., UL BWP may include PRBs where the gNB is transmitting in DL, and vice versa, DL BWP may include PRBs where the gNB is receiving in UL).

With the BWP based scheme, the cell/carrier can be seen as being divided into multiple subbands, each subband having its own TDD frame structure. The UE can be configured with one or more BWP pairs, each BWP pair overlapping with (or at least contained within) one of the subbands. This scheme also works with UEs supporting single configured DL and UL BWP pair with aligned center frequencies, which is the baseline assumption from RAN1 #110bis-e meeting. We think this option as the basic BWP based scheme. The basic BWP based scheme is schematically illustrated in Figure .

Figure  Baseline BWP based scheme

With the basic BWP-based scheme, a UE can work properly without new UE behavior to e.g. resolve the collision between UL/DL signal in SBFD symbols. Also, adaptation of (semi-statically) allocated resources based on the slot/symbol type is not needed, as the UE can be configured with different set of resources (PUCCH, CG-PUSCH, SS, CORESET, CSI-RS, etc.) per BWP, and can rely on BWP switching or BWP reconfiguration for determining which set resources should be used in a specific slot/symbol.

The main drawbacks of the basic BWP based scheme are the reduced UE bandwidth, which can result in a significant reduction of the available peak data rate in both UL and DL, and the potential increase in signaling overhead, as the cell may need to provide at least one SSB per subband to ensure proper UE operation.

The basic BWP based scheme is rather straightforward but has severe UE peak rate and signaling overhead issue.

Some companies have also proposed a BWP based scheme where the UE can (more or less) dynamically adapt its active DL and UL BWP to the portion of the cell spectrum which is allocated for DL transmission and UL reception, respectively, in each slot/symbol. We refer to this option as advanced BWP based scheme (Figure ). First, such advanced BWP based scheme requires more than one configured DL and UL BWP pair with unaligned center frequencies in cases with asymmetric UL-DL frequency partitioning such as D-U and U-D frequency domain configurations. This is not in line with the baseline assumptions agreed in RAN1 #110bis-e meeting. Besides that, one of the main challenges of such advanced BWP based scheme is that it requires very fast BWP switching delay. In practice, the UE shall be able to switch BWP nearly instantaneously from one symbol/slot to the next symbol/slot, e.g. in the transition from a SBFD slot to a TDD slot, and vice versa.

For the advanced BWP based scheme, there should be no additional gap due to BWP switch in the transition from SBFD slots to TDD slots, and vice versa.

Note that, based on current specifications, the BWP switch delay (in FR1) is between 0.75 ms and 3 ms, depending on the SCS and the UE capability. Within the BWP switch delay, the UE: (1) is not required to transmit UL signals or receive DL signals on the serving cell, and (2) may be allowed to cause interruptions of up to X slots on other active cells (X is between 0.5 ms and 1 ms depending on the SCS). More details can be found in TS 38.133, sections 8.6.2 and 8.2.2.2.5.

Note that one of the main advantages of matching the UE DL/UL BWP to the portion of the cell spectrum allocated for DL transmission/UL reception in each slot/symbol is that the UE may adjust its digital filter bandwidth to the active BWP, with consequent benefits in terms of reduced inter-subband UE-2-UE cross link interference. In case the UE has this capability, it may also be assumed the UE can adapt its digital filter bandwidth to match the UL/DL subband with the RB-set based scheme, if the UE is aware of the UL/DL frequency partitioning in SBFD symbols. Therefore, the ability to better suppress inter-subband UE-2-UE CLI is more related to the UE capability of dynamically and rapidly adapting its digital filter bandwidth to the time varying UL/DL subband, and not much to whether the RB-set based or the advanced BWP based scheme is assumed.

Figure  Advanced BWP based scheme

In terms of digital filter bandwidth adaptation at the UE, there is substantially no difference between the RB-set based and the advanced BWP based approach.

It should also be noted that in current NR specifications the in-band emission requirements are not depending on the BWP configuration. Many legacy devices do not adapt their digital filter’s bandwidth to the active BWP. Unless new in-band requirements are specified for SBFD aware UEs, we do not think that better UL transmission and DL reception filtering is in practice an advantage of any of the proposed schemes, though it may be easier to implement for the basic BWP based scheme.

Unless new in-band emission requirements are specified, better UL transmission and DL reception filtering should not be considered as an advantage of any of the proposed schemes.

In Table 1 below we summarize the pros/cons and specification impacts of the different options.

Table 1 Pros and cons of RB-set based and BWP based SBFD schemes

|  | RB-set based | Basic BWP based | Advanced BWP based |
| --- | --- | --- | --- |
| Peak data rate | High | Low (Limited by subband size) | High |
| Signaling overhead | Low | High (at least one SSB per subband) | Low |
| Spec modifications for UL/DL Collision Handling in SBFD slots/symbols | Yes (One slot/symbol can have both UL and DL resources) | No (Only UL/DL resources in a slot/symbol) | Yes (One slot/symbol can have both UL and DL resources) |
| Spec modifications for adaptation of (semi-statically) allocated resources to available UL/DL resources in SBFD slots/symbols | Yes (May only be needed in case of dynamic indication of subband location in time domain) | No | No |
| Spec modifications to BWP framework | No | Minor (Specify how to determine the TDD frame structure per BWP pair, BWP consisting of two non-adjacent RB sets, etc.) | Yes (E.g. support semi-static BWP switching, configuration of BWP pair with unaligned center frequencies for UL and DL BWP, BWP consisting of two non-adjacent RB sets, new BWP switch requirements, etc.) |
| Support dynamic indication of subband location in time domain (with or w/o adaptation of UE digital filter BW to UL/DL subband, depending on UE capability) | Yes | Yes | No (Depend on the feasibility of optimized BWP switching delay with dynamic indication of subband location in time domain) |
| Adaptation of UE digital filter BW to UL/DL subband | Possible (May depend on UE capability and may only be possible for semi-static SBFD configuration in frequency and time domain) | Possible | Possible (May depend on UE capability and may only be possible for semi-static SBFD configuration in frequency and time domain) |

Based on the above analysis and on the summary in Table 1, we conclude that:

- The basic BWP based scheme is the most straightforward from standard specification perspective but may have significant limitations in terms of UE peak data rate and signaling overhead (one SSB per subband). The basic BWP based scheme does not require support of DL and UL BWP pair with unaligned center frequencies, though some enhancements to BWP framework may be needed to e.g. configure a (DL) BWP consisting of at least two non-adjacent RB sets (e.g. for D-U-D SBFD configuration).

- Both the advanced BWP based and the RB-set based schemes have some standardization impact. Both schemes require specification of new UE behavior for handling UL/DL collision in SBFD slots. The advanced BWP-based scheme also requires enhancements/modifications to the Rel-15 BWP framework to enable e.g.: (1) semi-static BWP switching between two BWP pairs; (2) configuration of a BWP pair with unaligned center frequencies for UL and DL BWP (needed to support “non-symmetric” SBFD configurations, i.e. D-U and U-D); (3) configuration of a (DL) BWP consisting of at least two non-adjacent RB sets. The BWP-based scheme may also require specification of new BWP switching (delay and gaps) requirements.

- The RB-set based scheme may require additional standardization work to determine the validity of semi-statically allocated resources (and/or their adaptation to the available UL/DL frequency resources) in SBFD slots/symbols, at least if dynamic indication of subband location in time domain is supported.

- All options may support reduced UE-2-UE inter-subband CLI depending on the UE capability to adapt its digital filter BW to the UL/DL subband or BWP.

Based on the observations and summary above, we propose:

Assume RB-set based scheme as the baseline. With the RB-set based scheme, a UE’s active BWP may include PRBs that are used by the serving cell for the opposite link direction.

It should be noted that even assuming RB-set based SBFD configuration, it should still be possible to configure a UE with a BWP confined within a set of PRBs where the UE may only see one link direction per symbol within the BWP. In other words, operation according to the basic BWP based scheme is not precluded by RB set based signaling.

Operation according to the basic BWP based scheme is not precluded by RB-set based signaling.

### Initial Access

Support of initial access in UL subband has been discussed in RAN1 since the beginning of the SI. No agreement has been reached so far, despite most of the companies are in favor of studying potential benefits and enhancements for initial access in UL subband.

One example in which both legacy and SBFD UEs are configured with the same PRACH configuration index is shown in Figure , in which an initial access timeline is shown, in 4-step RACH for an SBFD UE (timeline in blue) and a legacy UE (timeline in orange). In this example, we show a DL heavy frame configuration. We assume that both UEs are ready to transmit PRACH by subframe 0 in the first frame. For simplicity in the figure, all processing delays are equal to 3 slots. RACH occasions are available in subframes 1,3,5,7,9. In this example, 30 kHz SCS is assumed. We assume PRACH configuration index 108, in which one PRACH slot is available in the subframe.

- Msg 1 is sent in the first available RO. The first available RO for the SBFD UE occurs in subframe 1 while the first available RO for the legacy UE occurs in subframe 9.

- Msg 2: is sent in the first available PDCCH,

- Msg3: is sent in the first UL opportunity after Msg 2.

- Msg4: is sent in the first DL opportunity, after Msg 3.

Figure  – Example of initial access delay for SBFD and Legacy UEs.

In this example, the total delay from slot 0 until msg4 is sent is 8 ms for the SBFD UE and 17 ms to the legacy UE. Though this figure oversimplifies the initial access delay, here we can see the aspects that impact it:

- PRACH configuration index, which determines the subframes that contain a RACH occasion (RO)

- The frame format, which determines which PRACH occasions are valid and in which slots it is possible to send the different messages related to PRACH.

For most PRACH configurations there is no difference between the valid ROs for a SBFD UE and for a legacy UE, because they are concentrated in subframes that are available to both (for example, 4 and 9). If we consider only the PRACH configurations with more than 4 subframes available in each frame, and repeat this exercise considering different PRACH configurations, processing times at the UE and gNB SSB periodicity and randomize the starting point in which both the SBFD UE and legacy UE are ready to send PRACH (starting point can be any slot from slot 0 to slot 40 – 2 full frames), we obtain the following results:

| a) | b) |
| --- | --- |
| Figure – (a) Example of initial access delay, SBFD UEs and Legacy UEs (b) initial access delay difference between SBFD UEs and Legacy UEs |  |

In 50% of the cases, a delay reduction of 3 slots is achieved when comparing a DXXXU configuration and DDDSU configuration. In 90th percentile, a delay reduction of 10 slots is observed. When considering retransmissions, this reduction can be even more accentuated.

As shown in the example above, Enabling PRACH transmission in SBFD symbols can reduce initial access delay. Moreover, PRACH coverage can also be enhanced if the UL subband in SBFD slots can be used for PRACH transmissions, as compared to only allowing PRACH transmissions in UL slots, e.g. by enabling long PRACH transmissions spanning over more than one consecutive UL slot. In [6][7], it is shown that UL coverage is limited by PRACH instead of PUSCH, so using SBFD slots for PRACH transmission can increase the UL coverage by 3 dB or 6dB if we increase the number of consecutive PRACH slots to 2 or 4.

Additionally, when using the SBFD symbols for sending PRACH, it can be assumed that not all SBFD UEs will be competing for PRACH occasions with legacy UEs, which leads to less collisions, and less power used for retransmissions.

Allowing SBFD aware UEs to send PRACH and Msg3 in SBFD symbols can result in a decrease in latency of at least 2 slots, and at most 18 slots when compared to static TDD DDDSU, even if no additional source of delays, such as repetitions, are considered.

UL coverage is limited by PRACH instead of PUSCH, so using SBFD slots for PRACH transmission can increase the UL coverage up to 3dB or 6dB if the number of slots used to consecutive PRACH is increased to 2 or 4.

Allowing SBFD aware UEs to send PRACH in SBFD symbols can reduce the number of collisions for sending PRACH since SBFD aware UEs would not necessarily be competing for PRACH resources with legacy UEs.

Enabling initial access in SBFD symbols can increase the PRACH capacity, PRACH coverage, as well as reduce initial access delay.

As in the example above, ROs could be configured using existing RACH configuration during legacy DL slots (i.e. slots configured as DL in tdd-UL-DL-ConfigurationCommon). Legacy UEs may consider the ROs colliding with DL symbols as invalid, while SBFD aware UEs may consider the ROs colliding with DL symbols as valid. Signaling the subband UL location in frequency and time domain to IDLE mode UEs (e.g. in SIB1) may facilitate the UE in determining the validity of ROs configured during SBFD symbols. On the other hand, having a single RACH configuration for legacy UL and SBFD symbols may create backward compatibility issue (legacy UEs do not expect ROs to be configured on symbols signaled as DL in tdd-UL-DL-ConfigurationCommon) and reduce the resource allocation flexibility.

A better approach would be to have different RACH configuration parameters for legacy UL and SBFD symbols/slots. This can be done, for example, by means of signaling a new SBFD RACH configuration via SIB1 message to be used by SBFD aware UEs to perform random access in SBFD slots/symbols. The UE may implicitly assume that the new SBFD RACH configuration applies to ROs colliding with SBFD symbols, or an explicit indication on the valid ROs can be included in the new SBFD RACH configuration. Details are FFS.

To handle cross link interference in SBFD symbols, the network could configure different criteria to consider a RO as invalid in SBFD slots unless certain conditions are met (by means of configuration of maximum transmit power, minimum coupling loss, priority rules, etc.). This can be done by defining new corresponding parameter(s) in the new SBFD RACH configuration.

In RAN1 #113, the following was proposed as a conclusion in the discussion summary, without an agreement. [5]

Proposal 1-12a Proposed Conclusion: If PRACH and Msg3 transmissions are allowed in UL subband for SBFD-aware UEs, it can potentially reduce the latency of random access, reduce the collision probability, improve the coverage of PRACH and Msg3 and avoid the UL resource fragmentation in full UL symbols. In order to support PRACH and Msg3 transmissions in UL subband for SBFD aware UEs in RRC_IDLE and RRC_INACTIVE states, SBFD time and frequency locations need to be configured in SIB and higher specification impact is expected.

Our view is that it is important to capture the discussions regarding the initial access in SBFD symbols in the TR, since it was mentioned by different companies during the study phase.

Capture in the TR the following conclusion: It is beneficial to allow PRACH and Msg3 transmissions in SBFD symbols. Initial access during SBFD symbols can increase PRACH capacity, PRACH/Msg3 coverage (e.g. by 3 or 6dB), reduce the collision probability as well as reduce latency of random access as much as 18 slots, depending on the configurations being compared.

### Impact and potential enhancements for transmissions and receptions

We think that support of SBFD introduces two main challenges from frequency domain resource allocation perspective:

During SBFD slots/symbols and in case the UL subband is located at the middle part of the carrier, there may be inefficiencies in the currently supported FDRA to support allocation of frequency resources in two non-adjacent DL subbands. This may require enhancements.

The frequency domain resources for DL/UL transmission in SBFD slots/symbols may be different as compared to non-SBFD symbols. For transmission whose FDRA is applicable in more than one slot (e.g. semi-statically configured resource allocations, multi-slot transmissions potentially including repetitions and/or frequency hopping, etc.), enhancements may be required to ensure that the allocated frequency resources do not overlap with a subband which is configured for the opposite link direction during SBFD slots/symbols.

Enhancements addressing each of the above-described problems are discussed in sections 2.3.3.1 and 2.3.3.2, respectively.

#### Frequency domain resource allocation

##### PDSCH

NR supports two types of Frequency Domain Resource Allocation (FDRA):

- Type 0 FDRA assumes RBG (Resource Block Group) based resource allocation. The BWP is divided into RBGs and the UE can be allocated one or multiple (contiguous or non-contiguous) RBGs. The first and the last RBG in a BWP may be with size less than the nominal RBG size P, while other RBG in the BWP will be with size P. The nominal RBG size P can be up to 16 PRBs, depending on the BWP size. The RBG size P is given by Table 5.1.2.2.1-1 in TS 38.214, copied below:

Nominal RBG size P

| Bandwidth Part Size | Configuration 1 | Configuration 2 |
| --- | --- | --- |
| 1 – 36 | 2 | 4 |
| 37 – 72 | 4 | 8 |
| 73 – 144 | 8 | 16 |
| 145 – 275 | 16 | 16 |

- Type 1 FDRA, the UE is allocated over a set of contiguous non-interleaved or interleaved virtual RBs within the UE BWP. A type 1 RA field consists of a RIV (Resource Indication Value) indicating the starting virtual RB (RBstart) and the number of contiguous RBs allocated to UE (LRBs).

PDSCH type 0 RA enhancements:

In RAN1 #112bis-e, the following was agreed on the FDRA:

Agreement For SBFD-aware UEs, Option 1 with update is agreed for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands for better resource utilization. For an RBG that overlaps the subband boundary, - Option 1 (with update): - The Part of the DL RBG inside the DL subband can be used - The Part of the UL RBG inside the UL subband can be used

For the part outside the respective DL or UL subband, in our view the conclusion depends on what is the agreed behavior for dynamic SBFD. For example, if it is agreed that the dynamic behavior is specified by allowing DL receptions outside of the DL subband, then it would make sense to study whether part of the DL RBG outside the DL subband can be used (at least in Dynamic SBFD). This could for example be achieved by the gNB by scheduling a RBG completely outside the DL subband. However, our preference is that the symbol is converted to a DL-only symbol, and in this case, the symbol wouldn’t be characterized by DL and UL subbands. Therefore, we propose:

For SBFD-aware UEs, for an RBG that overlaps the subband boundary:

- •	The UE assumes that the part of the UL RBG outside the UL subband cannot be used for transmissions

PDSCH type 1 RA enhancements:

Clearly, FDRA Type 1 cannot be used for frequency resource allocation in two non-contiguous DL subbands. Therefore, it also suffers from resource fragmentation in a DUD configuration. Study of RA type 1 enhancements were not agreed in the last meetings, though different enhancements were discussed among companies[3]:

1) Non-continuous PRB to CRB mapping

2) Enhanced rate matching: Rate matching around UL subband or puncturing the PRB/symbols within the UL subband: in this option, for non-interleaved RA, when there is an overlap of the allocated PDSCH with the active UL subband, the UE would perform rate match around an active UL subband.

3) Mirror image FDRA, where the DL grant indicates a 1st set of RBs and a 2nd set of RBs is determined by reflecting the 1st set of RBs across the middle of the BWP.

Current FDRA Type 1 does not support resource allocation across two DL subbands.

In our view, it should be possible to allocate resources in both DL sub bands in a DUD configuration, with only 1 RIV and this can in principle be achieved by all options above. It is also important to address whether the current DCI format can be maintained or not in each of the discussed options.

Another option to support non-contiguous allocation in SBFD symbols is to change the reference point for the FDRA Type 1 allocation in SBFD symbols/slots. Currently, the reference point is the first RB within the DL BWP The UE determines the set of RBs to be used for transmission based on an offset to the reference point and a number of contiguous RBs, which are signaled in the RIV. To optimize signaling overhead, RIV only supports signaling of an offset-length duple if it is feasible within the configured DL BWP. As an example, if the offset is pointing to the last RB within the DL BWP, the RIV only supports signaling of a length corresponding to 1 RB.

If the first RB in the upper DL subband is used as the reference point and the allocated RBs are determined based on the offset and the length signaled in the RIV by applying some modulo operation to ensure that the allocated RB index does not fall outside of the DL BWP, as in Figure , the existing RIV and corresponding DCI format can be used to signal non-contiguous allocation to SBFD aware UEs. This requires an SBFD aware UE to reinterpret the RIV in case the UL subband is located between two DL subbands in the DL BWP The new reference point and modulo operation to determine the index of the allocated RBs in SBFD slots/symbols can also be used for FDRA Type 0 to limit the number of RBGs overlapping with DL subband boundary.

By changing the reference RB in type 1 FDRA allocation in case the UL subband is located between two DL subbands in the BWP, and wrap-around the PRB index it is possible to use the RIV and DCI formats to provide non-contiguous allocation in type-1 FDRA.

Figure  – New FDRA starting point

FDRA type 1 enhancements are needed to support allocation across two DL subbands in a SBFD symbol with one RIV and existing DCI formats.

##### CSI-RS and CSI reporting

CSI-RS has similar frequency resource fragmentation problem as described for PDSCH in the previous section, especially in a DUD configuration. There are two alternatives being discussed for the CSI-RS resource allocation: contiguous or non-contiguous. Enhancements are needed to support non-contiguous allocation. Contiguous allocation is already supported in the specification, but in case of a DUD SBFD configuration would require that two different resources are configured, one in each sub-band.

In the RAN1 #112, two options have been agreed, and in in RAN1 #112bis the discussion continued with further agreements and conclusions:

RAN1 #112 Agreement: Study the frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs considering the following options: - Option 1: Two contiguous CSI-RS resources that are linked - Option 2: One CSI-RS resource - Option 2-1: Non-contiguous CSI-RS resource allocation - Option 2-2: One contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s) RAN1 #112bis-e Conclusion For the options agreed to study in RAN1#112 for frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, the following observations are agreed. - For all the options, there is no impact on CSI-RS sequence generation. - Option 1 requires additional signalling to link two CSI-RS resources in two DL subbands. - Option 2-1 requires new RRC structure to configure non-contiguous RBs for one CSI-RS resource, which may require additional signalling overhead. - Option 2-2 can reuse the existing signalling design for CSI-RS resource configuration. Option 2-2 can be used to resolve the potential unaligned boundaries between CSI-RS resource configuration and SBFD subbands - Further discussion is required on the UE complexity due to: - UE capability of maximum number of configured CSI-RS resources - Processing non-contiguous CSI-RS Agreement For SBFD-aware UEs, study the following options for CSI report associated with periodic/semi-persistent CSI-RS in case the periodicity is such that CSI-RS instances occur in both SBFD and non-SBFD symbols: - Option 1: two CSI-ReportConfigs, where one is associated with SBFD symbols and the other is associated with non-SBFD symbols - Option 1-1: One CSI-ReportConfig is associated with a CSI-RS restricted to SBFD symbols only and the second CSI-ReportConfig is associated with a second CSI-RS restricted to non-SBFD symbols only; - Option 1-2: Both CSI-ReportConfigs are associated with the same CSI-RS. The CSI report associated with one CSI-ReportConfig is derived based on CSI-RS instances in SBFD symbols only. The CSI report associated with the second CSI-ReportConfig is derived based on CSI-RS instances in non-SBFD symbols only. - Option 2: one CSI-ReportConfig associated with both SBFD symbols and non-SBFD symbols - Option 2-1: One CSI-ReportConfig is associated with two CSI-RSs which are restricted to SBFD symbols and non-SBFD symbols respectively. Separate CSI measurements are derived based on the first and second CSI-RSs respectively. - Option 2-2: One CSI-ReportConfig is associated with one CSI-RS. The CSI report is derived based on CSI-RS which can be in SBFD symbols or non-SBFD symbols in different time instances. - FFS impact on UE CSI processing and reporting timeline Note: Whether the CSI-RS resource can be used for SBFD and non-SBFD symbols may depend on, e.g., gNB implementation of same/different antenna configuration in both symbols. Option 1-1 can be supported according to existing specification by gNB configuration of appropriate periodicities to ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only. But it may restrict the gNB configuration flexibility and enhancements can be considered by additional indication or rules to determine the CSI-RS is valid within one symbol type and is invalid in the other symbol type. Option 2-2 can be supported according to existing specification to configure measurement restriction so that UE would not average CSI measurements across SBFD and non-SBFD symbols. Agreement - For semi-static SBFD, for a CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS resources within DL subband(s) are valid for SBFD-aware UE. - For semi-static SBFD, for a CSI reporting subband which overlaps with SBFD subband boundaries, CSI report is derived based on CSI-RS resources excluding CSI-RS resources outside DL subband(s).

When analyzing the different options agreed in RAN1 #112, it is important to consider different aspects such as whether the existing specification supports the concept and the signaling overhead. This has been captured in the conclusions agreed in the last meeting.

Our view is that configuring a contiguous CSI-RS resource over the entire bandwidth is possible. Rate matching around UL subband and guardband can be assumed, as also proposed for Type 1 FDRA. Essentially, CSI-RS resources are configured over the entire bandwidth, but the UE can assume CSI-RS is not transmitted on REs that are overlapping with UL subband and guardband in a SBFD slots/symbols, and report the CSI-RS based on the REs in the DL subband. In our view, this understanding is captured in Option 2-2.

For CSI-RS resource set frequency domain resource allocation and CSI reporting configuration across non-contiguous DL subband, CSI-RS resources are configured over the entire bandwidth, with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s).

Additionally, in the last RAN1 meeting different options were discussed regarding the CSI report. These options depend on the agreement on how the CSI-RS resources are configured. In our view, if it is agreed that the CSI-RS resources can be configured across non-contiguous DL subbands, and derived by excluding frequency resources outside DL subbands, it should be possible to agree that the periodic/ semi-persistent CSI-RS can be configured in such way that CSI-RS instances can occur in both SBFD and non-SBFD symbols.

However, we note that the interference conditions are different in both types of symbols, since SBFD symbols are subject to UE-to-UE CLI. Therefore, in our view, the UE should not average CSI measurements across SBFD and non-SBFD symbols. Our preferred option is that the UE is configured with two CSI-ReportConfig, both associated with the same CSI-RS, but one report is derived based on CSI-RS instances in SBFD symbols, and the other based on CSI-RS instances in DL symbols. This Option is covered by Option 1-2.

For CSI report associated with periodic/semi-persistent CSI-RS in case the periodicity is such that CSI-RS instances occur in both SBFD and non-SBFD symbols, two CSI-ReportConfig are configured. Both CSI-ReportConfigs are associated with the same CSI-RS. The CSI report associated with one CSI-ReportConfig is derived based on CSI-RS instances in SBFD symbols only. The CSI report associated with the second CSI-ReportConfig is derived based on CSI-RS instances in non-SBFD symbols only.

Additionally, as the interference for PDSCH may include UE-to- UE CLI from aggressor UE, it will be interesting how to include this UE-to- UE CLI from exact aggressor UE in the interference measurement for CSI process in SBFD cases.

#### Transmissions across SBFD and non-SBFD symbols

In this section we discuss, among others, whether a slot can consist of both SBFD and non-SBFD symbols, whether the transmissions and receptions can be configured across symbols/ slots of different types, and how to configure these resources.

##### Whether a slot can consist of both SBFD and non-SBFD symbols

In RAN1#112, it was agreed to study whether a slot can consist of both SBFD and non-SBFD symbols. In RAN1 #112bis-e and RAN1 #113, the following has been concluded:

RAN1 #112bis-e Conclusion The following RAN1 observation is made: One motivation for allowing that a slot can consist of both SBFD and non-SBFD symbols is for compatibility with symbol-level TDD UL/DL configuration. Frequent switching between SBFD and non-SBFD symbols may increase the implementation complexity and interruptions of transmissions/receptions during transition. - Further study whether limitation(s) on the maximum number of switching points between SBFD and non-SBFD symbols within a slot, a TDD UL/DL pattern period, and/or semi-static SBFD configuration period (if different from TDD UL/DL pattern period) are needed - Further study scenarios a guard period between SBFD and non-SBFD symbols is required/not required and the length of the guard period if required Note: Whether or not a physical channel/signal occasion is mapped to both SBFD and non-SBFD symbols within a slot is a separate discussion. RAN1 #113 Conclusion At least for semi-static SBFD, in order to avoid frequent switching between SBFD and non-SBFD symbols, potential limitation on the maximum number of switching transition points between SBFD and non-SBFD symbols can be considered from SBFD subband configuration perspective. Maximum of two switching transition points including one switching transition point from non-SBFD symbols to SBFD symbols and one switching transition point from SBFD symbols to non-SBFD symbols within a TDD UL/DL pattern period can be considered as a starting point where the switching transition point can be aligned with slot boundary or within a slot. - (Agreement) The usage of ‘switching point’ in previous conclusions/agreements are revised to ‘transition point’ A guard period between SBFD and non-SBFD symbols may or may not be required at gNB and/or UE side depending on gNB/UE implementation and/or SBFD operation.

Today, NR TDD allows for three types of slots: DL, UL and special slots. Special slots can consist of DL symbols, UL symbols and guard symbols, which are used to allow for the Rx-Tx switching at the UE.

NR specification allows for symbols of different types in special slots. A transition from DL to UL requires a slot to consist of different symbol types, at least in the special slots, if the configuration is kept compatible with legacy TDD DL/UL configuration.

In order to be compatible with a symbol-level TDD DL/UL configuration, there should be a possibility of allowing the same mechanism to be available for the transition from SBFD to UL-only slots. In this case, in special slots, there could be some symbols that are SBFD and others that are UL-only. This backward compatibility is important given that DL/UL transmissions in legacy subbands only (i.e., the subbands that use legacy TDD pattern) should follow legacy behaviour. Another benefit of allowing symbols of different types in the same slot, is the support of DL only symbols, for example, for sending SSBs and CSI-RS. As mentioned earlier, our preference is that SBFD is configured only in a subset of the DL symbols in a slot, not all of them. In that case, it is important to support CSI-RS transmission over the entire channel bandwidth in DL-only symbols, even if the slot also contains SBFD slots later. In a mixed DL slot, the first part of the slot could accommodate PDCCH, for example. Allowing for more scheduling flexibility in the SBFD portion of the slot.

A slot can consist of both SBFD and non-SBFD symbols.

##### Whether the transmission/reception occasion of a physical channel/signal can be mapped to SBFD and non-SBFD symbols within a slot for a UE

In RAN1#112bis, it has been agreed to study whether the transmission / reception occasion of a physical channel can be mapped to SBFD and non-SBFD symbols within a slot for a UE:

Agreement Study whether the transmission/reception occasion of a physical channel/signal can be mapped to SBFD and non-SBFD symbols within a slot for a UE, and whether a UE can transmit/receive in the occasion mapped to SBFD symbols and non-SBFD symbols including: - Use-case(s) including the locations and number of switching points of the SBFD and non-SBFD symbols in the slot. - Potential benefits if any - Phase continuity - Potential interruption of transmissions/receptions during transition - Required guard time if any - Potential impact on performance - Impact on link adaptation, channel estimation, and other procedures - UL transmission timing if any - Implementation complexity - Applicability for SBFD aware UE and non-SBFD aware UEs - NOTE: There are more than one scenario where a transmission overlaps SBFD and non-SBFD symbols and some may or may not face the aspects listed above - NOTE: This study doesn’t mean RAN1 agreement on a slot consisting of SBFD and non-SBFD symbols.

Use cases:

The first part of the study is to identify use cases in which a transmission can be mapped across SBFD and non-SBFD symbols within a slot. We refer to the mapping as cross-symbol-types mapping and refer to the slot as mixed slot hereinafter. One obvious use case is to allow a full-slot scheduling for UL transmission in the mixed slot. Given that all symbols in the mixed slot in the UL subband are UL symbols, full-slot scheduling should be allowed, following legacy behaviours, without scheduling restriction. The use case is important especially for the flexible slot near to the UL slot, where there may be SBFD symbols and UL symbols.

Similarly, this should also allow full-slot scheduling for DL transmission in DL subbands in the mixed slot.

The obvious use cases for allowing a transmission to be mapped across SBFD and non-SBFD symbols in a slot are to allow full-slot scheduling of UL or DL transmission in the slot, without scheduling restriction.

Another use case that would benefit from cross-symbol-type mapping is the UCI mapping. The SBFD symbols provide more opportunities for sending PUSCH or PUCCH, but non-SBFD symbols are more robust to interference.  In case the PUSCH or PUCCH is mapped into SBFD and non-SBFD symbols, the UCI can for example, be mapped in the non-SBFD symbols in this allocation. It is worth noting that in case PUSCH/ PDSCH covers two types of symbols, the DMRS should also cover SBFD and non-SBFD symbols.

Potential benefits:

The benefit for allowing a transmission/ reception occasion to be mapped across SBFD and non-SBFD symbols within a slot is that it avoids either scheduling restriction at the scheduler or specific handling rules at the UE. On the other hand, a common scenario would be that the mixed slot is (legacy) special slot, within which the number of SBFD symbols will be larger than number of non-SBFD symbols. Therefore, if cross-symbol-types mapping is not allowed, the scheduler would mostly schedule transmission only on SBFD symbols in the slot. However, performance of a transmission on SBFD symbols only may be worse than performance of a transmission with cross-symbol-types mapping. Indeed, given that interference is lower in non-SBFD symbol compared to its counterpart, interleaved encoded bits are spread across symbol types and may result in better performance. Another benefit could be that, in case of UCI multiplexing on PUSCH, the UCI may be multiplexed on the non-SBFD symbols with lower interference. However, it’s worth noting that this is not the only way of protecting UCI from CLI, e.g., an approach of avoiding mapping control bits in the PRBs that that close to DL subbands may help isolating at least inter-subband CLI as well, although it’s less ideal.

Several benefits of allowing a transmission to be mapped across SBFD and non-SBFD symbols in a slot could be observed, namely avoiding either scheduling restriction at the scheduler or specific handling rules at the UEs or offering better performance for data (or control bits, if any) thanks to interleaving the encoded bits across symbol types given that lower interference is expected in non-SBFD symbols.

Potential interruption/required guard time

From discussions in RAN1#112 and RAN1#12bis-e, our understanding on the potential interruption or required guard time is due to concerns on implementation, namely a transition time for potentially switching panels, tuning filter and adjusting timing and/or sampling rate when moving from one symbol type to another.

The conclusion from RAN1 #113 is that a guard period between SBFD and non-SBFD symbols may or may not be required at gNB and/or UE site depending on gNB/UE implementation and/or SBFD operation. As raised by different companies in the last RAN1 meeting, the need for a guard band would depend on what is the antenna configuration at the gNB, whether there is need for switching the TxRUs between an SBFD and non-SBFD symbol, and whether the NTA_offset is set to zero in SBFD symbols, and to a non-zero value in non-SBFD symbols. RAN1 should consider the minimum number of transition period needed and avoid unnecessary overhead.

In antenna configuration 2, method 2-1, and in antenna configuration 3, methods 3-1, one panel group is always used for DL and the other is always used for UL. In antenna configuration 1, and in antenna configuration 2, method 2-2 and antenna configuration 3, method 3-3, a panel group can be used for DL and UL, just as in TDD operation. RAN4 is discussing whether a transient period requirement needs to be defined from the switching between a non-SBFD to SBFD symbol and vice-versa. Apart from the gNB implementation, our view is that the length of the guard period would also depend on whether the NTA_offset in the UE is configured or not to be zero, i.e, whether NTA_offset assumes different values in SBFD or non-SBFD slots.

The exact value of the guard-period between SBFD and non-SBFD symbols can be discussed in the WI phase.

In our view, these are potential implementation issues, which may or may not exist depending on the Base station and UE implementation/capability. However, if a long guard time is introduced in the end, then allowing cross-symbol-types mapping may not be very beneficial anymore. Indeed, considering again the mixed slot as legacy special slot with 10D:2G:2U symbol split. If the 2 gap symbols are used for guard time then the benefit is clearly worse compared to the case when guard time is not introduced. This not only because less symbols are used for data transmission, but also because the number of low CLI symbols is reduced from 4 to 2 in this example.

If a long guard time is introduced to account for implementation limitation, the benefit of allowing a transmission to be mapped across SBFD and non-SBFD symbols in a slot would be less significant.

Issues with not allowing a transmission to be mapped across SBFD and non-SBFD symbols in a slot:

If a transmission is not allowed to be mapped across SBFD and non-SBFD symbols in a slot, then UE behaviour for TDRA determination for the transmission/repetition that spans across SBFD and non-SBFD symbols within a slot should be defined. Furthermore, legacy UEs are not aware of SBFD symbols, and would receive in DL slots (which might be configured as SBFD by the network). This is relevant, for example, for the case of one transmission spans multiple slots, TBoMS, and at least one of these slots consists of symbols of different types. Another relevant case is semi-static scheduling or, even more relevant, for the case of PUSCH/PDSCH transmission with repetitions given that the repetitions follows the same time domain resource allocation as allocated for the first repetition. For example, let’s take the case of Rel-17 PUSCH repetitions, in which all repetitions follow the allocation of the 1st one (allocated with full 14 symbols in the slot), and thus only slots with 14 UL symbols are counted as available as illustrated in Figure  below. Assuming that repetitions can be mapped in slots of different types, there could still be the case in which one of these repetitions fall within a mixed slot, which consists of SBFD and non-SBFD symbols as the 5th repetition in the figure below:

Figure  - Example of PUSCH repetition with L = 14

In this case, several options should be considered (for the 5th repetition) to define UE behaviour if a transmission is not allowed to span across symbol types, namely, whether the mixed slot is not considered as available for the repetition, therefore it is not counted for the repetition or whether the UE considers only part of the resources as available for the repetition (for example the UL part of the slot, or the SBFD part of the slot), or the UE considers only the longest part of the slot for that repetition (either the UL part or the SBFD part, depending on the number of symbols in each).

If a transmission is not allowed to be mapped across SBFD and non-SBFD symbols in a slot, RAN1 should further discuss UE behavior for TDRA determination for the transmission occasion that spans across symbol types in the slot, especially for semi-static PUSCH/PDSCH and PUSCH/PDSCH with repetitions and TBoMS.

Agreement

The following conclusion is to be captured in the TR

For a physical channel/signal occasion mapped to SBFD and non-SBFD symbols within a slot if any, the following options for UE transmission/reception can be considered in the normative stage

- Option 1: UE does not transmit or receive the physical channel/signal within the slot.

- Option 2: UE can transmit or receive the physical channel/signal within the slot only under certain conditions.

  - The conditions may depend on at least the following: whether or not phase continuity can be maintained across SBFD and non-SBFD symbols, whether or not there are same or different transmission/reception parameters e.g. power control, spatial/QCL, UL timing etc. applied in SBFD and non-SBFD symbols, and whether or not there is a guard period between the SBFD and non-SBFD symbols, etc.

- Other options are not precluded

In RAN1 #113 meeting, there were agreement as below to provide candidate options for a physical channel/signal mapped on SBFD and non-SBFD symbols within a slot.

Actually, there is one more option, i.e. a physical channel/signal occasion only mapped to either SBFD or non-SBFD symbol within a slot covering two types of symbols. In this option, the condition needed for option 2 will be no more limitation for the channel/signal transmission, which will provide easier chance to transmit in the slot covering two types of symbols and provide flexibility.

For a slot covering both SBFD symbol and non-SBFD symbol, RAN1 should also consider the option that UE can transmit or receive the physical channel/signal only in SBFD or non-SBFD symbol.

#### UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots

In RAN1#112 and RAN1 #112bis-e the following has been agreed:

RAN1 #112 Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) - Study the following options for SBFD-aware UEs: - Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only - Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols - UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols include the following: - PDSCH/PUSCH/PUCCH repetitions - SPS PDSCH/CG PUSCH - TBoMS - Multi-PUSCH/PDSCH scheduled by a single DCI - Periodic/semi-persistent SRS/CSI-RS/PUCCH - PDCCH RAN1 #112bis-e Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), if the transmissions/receptions can be in SBFD symbols and non-SBFD symbols with different available resources, study at least the following frequency resource allocation options for PDSCH, CSI-RS, PUSCH, PUCCH, SRS for SBFD-aware UE: - Option 1: Separate FDRA determination for SBFD slots and non-SBFD slots. - Option 1-1: Separate FDRA configurations/indications for SBFD slots and non-SBFD slots - Option 1-2: Separate frequency resources determined for SBFD slots and non-SBFD slots based on single FDRA configuration/indication - Option 1-3: single FDRA configuration/indication and RB offset(s) - Option 2: Perform rate matching or puncturing on the RBs outside DL/UL subbands for DL/UL channels/signals. - Option 3: A DL/UL channel/signal overlapping with RBs outside DL/UL subbands in a SBFD slot is dropped or postponed. Note: Different options can be studied for different signals/channels. Conclusion For the two options agreed in RAN1#112 for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols), the following observations are agreed. - Option 1 can be achieved by gNB configuration or scheduling to ensure that all transmission/reception occasions are confined to either SBFD symbols or non-SBFD symbols. Alternatively, Option 1 can be achieved by additional indication or rules to determine the transmission/reception occasions are valid within one symbol type and are invalid within the other symbol type. - The frequency resources, power control and beam/spatial relation for all the transmission/reception occasions can be the same for Option 1 but may be different for Option 2. If different, it may require additional specification efforts. - Option 1 may or may not increase the transmission/reception latency if the transmission/reception in the other symbol type is postponed and may degrade the performance if the transmission/reception in the other symbol type is dropped. Option 2 may or may not reduce the transmission/reception latency and improve coverage.

We first address the agreement from RAN1#112 related to whether UL transmissions or DL receptions are restricted to SBFD symbols or non-SBFD symbols only. In our view, there are advantages of not restricting transmissions/receptions to slots of the same type. SBFD symbols are subject to different interference conditions than non-SBFD symbols, and non-SBFD symbols are already used by legacy UEs. By allowing UL transmissions / DL receptions in slot of different types, the gNB has more flexibility to balance scheduling considering the interference conditions and the rate of resources used by legacy / SBFD UEs.

Therefore, we propose:

For SBFD aware UEs, for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots, the transmissions/receptions can be in SBFD symbols and non-SBFD symbols

Regarding the options agreed to be studied in the last meeting, in general, Option 1 (and the sub-options) provide more scheduling flexibility for the gNB to allocate the resources across symbols of different types. This may come at the cost of increased complexity and signaling overhead but depending on signal/channel and how the configuration is done, this overhead is not critical. Option 2, in which rate matching or puncturing on the RBs outside the DL/UL subband is applied, in our view, reduces the scheduling flexibility at the gNB, since in SBFD symbols the availability of the resources will differ from non-SBFD symbols, and the symbols will suffer from different SINR conditions. While this approach is suitable for some signals, for example CSI-RS, it might not be suitable for other channels such as DL SPS. Finally, in option 3 there will be an unnecessary performance impact since the transmissions/receptions will be dropped or postponed. In our view, option 3 is not preferred in any scenario.

In the next sections we discuss the alternatives for different signals/channels.

##### CG-PUSCH and DL SPS

A configured grant (CG)-PUSCH configuration can grant the UE with periodic resources for UL PUSCH transmissions. Two types of CG-PUSCH configurations are specified in NR: Type-1 CG and Type-2 CG. For both Type-1 and Type-2 CG, the CG-PUSCH allocation periodicity is configured via RRC. The resource allocation parameters (MCS, FD resource allocation, TD resource allocation) are provided via an ‘activation DCI’ for Type-2 CG, and via RRC signaling for Type-1 CG. In case of Type-2 CG, the resource allocation parameters apply for each of the subsequent periodic UL PUSCH transmissions following the ‘activation DCI’. The radio parameters can be adjusted by the gNB at any time by re-issuing a new DCI (a.k.a. ‘reactivation DCI’).

With SBFD, there is the problem of i) different resource availability in UL-only slots vs. SBFD slots with, as well as ii) potentially different SINR conditions in UL-only vs. SBFD due to the presence of new SBFD interference types, e.g. self-interference and inter-subband interference. To address this problem, one option is to introduce SBFD-specific CG-PUSCH configurations where, for each CG occasion, one or another set of radio parameters (FD resource allocation, TD resource allocation, MCS, etc.) is used depending on the corresponding slot format (e.g. UL-only or SBFD). This can be achieved by having e.g. two or more independently configured CG configurations (ConfiguredGrantConfig), where each of them is associated to one slot type.

Considering the different options agreed to be discussed in RAN1 #112bis-e, this is covered by Option 1-1 in the agreement in RAN1#112bis-e.

A similar approach may also be applied for DL SPS PDSCH.

For CG-PUSCH and DL SPS, introduce SBFD-specific configurations where one set of radio parameters is used depending on the corresponding slot format.

##### SRS/PUCCH/PUSCH in SBFD and non-SBFD symbols in different slots

For SRS/PUCCH/ PUSCH we also believe that Option 1 is more suitable. Option 1 increases the flexibility of the configuration but can result in a more complex design and overhead (especially in Option 1-1, and to a lesser extent in Option 1-3). In Option 2, only one configuration is provided, and it is avoided to increase the DCI overhead, but the scheduling flexibility is reduced. Option 3 is not preferred, because it will result in performance degradation.

We would be OK with either Option 1-1, in which separate FDRA configurations/ indications are given, or with option 1-3, in which a single FDRA configuration/indication and RB offsets are provided. This offset can be either configured by the network, or can be inferred by the UE based on, for example, the ratio between the bandwidth of the UL or DL subbands, and the bandwidth of the entire UL BWP. Option 1-3 increases the signaling overhead, but less than the first option, but without compromising the scheduling flexibility.

For SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots, introduce separate configurations (as in Option 1-1), or related configurations (as in Option 1-3).

##### PDSCH/PUSCH when the transmissions last more than one slot

While considering transmissions lasting for more than one slot, e.g. repetition or slot aggregation, the same transmission may be scheduled or granted to be transmitted on both SBFD slot and non-SBFD slot. How the PDSCH/PUSCH is mapped on different type of slots based on the same DCI or grant, should be studied. There are several scenarios such that a transmission may span across the two slot types, especially for UL transmission enhancement. Indeed, any transmission in SBFD slots may suffer from performance degradation due to CLI, which strongly impacts its coverage in these slots.

This issue is more relevant for UL than DL given the limited transmit power at UE side. Existing techniques for UL coverage enhancements can be applied for improving coverage of the transmissions in SBFD slot, namely PUSCH/PUCCH repetitions, TB processing over multiple slots (TBoMS) etc. This may lead to the case when some PUSCH/PUCCH repetitions are on SBFD slots and some other PUSCH/PUCCH repetitions are on non-SBFD slots, or a single TBoMS can span across SBFD and non-SBFD slots. Given that the potential performance degradation may happen only on SBFD slot and that non-SBFD slots offer wider bandwidth which can be leveraged to improve throughput, RAN1 can further study techniques to adapt the repetitions/TBoMS feature to better exploit/balance the different characteristic of SBFD and non-SBFD slots. One straightforward approach is to consider different number of RBs for the repetitions (or the slots of a TBoMS) in SBFD slots compared to the repetitions (or the slots of the TBoMS) in non-SBFD slots. Another straightforward approach would be to combine both repetitions and TBoMS, for example applying TBoMS for the repetitions of a PUSCH in SBFD slots, while the usual single-slot repetition is used for the repetitions of the PUSCH in non-SBFD slots. Other approaches, e.g., using different density of DMRS symbols or different waveforms for the transmissions on different slot types, should also be considered to complete the study.

It's worth also mentioning that, using different number of RBs for UL transmissions in SBFD slots (e.g., a smaller number of RBs) compared to that in non-SBFD slots has benefit in increasing energy per resource element (EPRE) to cope with CLI, however this benefit is only materialized when UE keeps the same transmit power for the transmissions in SBFD and non-SBFD slots. In other words, with a same transmit power, the transmission in SBFD slots with smaller number of RBs will have higher EPRE and cope better with CLI. However, in case the CLI is not critical (and it’s up to gNB measurement), different transmit powers could also be applied for the transmissions in SBFD and non-SBFD slots for power saving.

For Scheduled/configured TBoMS across SBFD symbols and non-SBFD symbols, with or without repetition:

- consider different number of RBs for the repetitions (or the slots of a TBoMS) in SBFD slots compared to the repetitions (or the slots of the TBoMS) in non-SBFD slots

- combine both repetitions and TBoMS, for example applying TBoMS for the repetitions of a PUSCH in SBFD slots, while the usual single-slot repetition is used for the repetitions of the PUSCH in non-SBFD slots

- Use of different density of DMRS symbols in SBFD and non-SBFD symbols

- Use of different waveforms for the transmissions on different slots

##### PDCCH

The control resource set (CORESET) and the Search Space (SS) configurations identify a set of physical frequency and time resources and a set of parameters that is used to carry PDCCH/DCI. The NR CORESET region can be localized to a specific region in frequency domain, which is not necessarily spread across the whole channel bandwidth, though it must be confined within the UE BWP. The CORESET configuration is quite flexible and uses a bitmap of 6 RBs per bit to indicate which PRBs are part of the CORESET.

With SBFD operation at the gNB, the portion of the cell bandwidth that is available for DL transmissions may dynamically change from slot/symbol to slot/symbol. While PDSCH transmissions can typically be contained in the corresponding DL subband by scheduling the UE in the corresponding frequency resources, adapting semi-statically allocated resources such as PDCCH/CORESET configuration may be more challenging. If, at any time during a SBFD symbol, the PDCCH CORESET overlaps with the UL subband, then it may occur that some PDCCH candidates are partially or fully on the UL subband, and they may therefore not be used. This has obviously negative impacts on the UE power consumption and on the effective use of PDCCH resources, which may indirectly result in a capacity loss.

To avoid this, some options exist with current specifications.

- PDCCH is only transmitted in DL-only symbols. This implicitly limits the availability of SBFD symbols, thus impacting the expected UL gain.

- Configuration of multiple non-overlapping SSs with different PDCCH CORESET configurations (each one associated to a different slot type). This approach has the clear disadvantage of not supporting dynamic subband location in time domain.

- To overcome this limitation, the network could configure multiple overlapping SSs with different CORESET configurations. As multiple CORESET configurations are available in one slot/symbol, the gNB can decide on which one to use depending on whether the slot/symbol is DL or SBFD. The downside of this approach is that the UE PDCCH blind decode budget may be hit as the UE has to run through two (or more) CORESET hypotheses when doing the blind search. If the UE blind decode budget is exceeded, this may imply that the gNB cannot use some of the CORESET configurations in the corresponding slot (SS with lower configuration index have higher priority), which brings us back to the original problem.

- Always confine the CORESET in the DL subband. This approach has the disadvantage of unnecessarily limiting the PDCCH capacity (due to limited PDCCH bandwidth) in DL-only slots/symbols.

- SSGS functionality specified in R16 for NR-U (and R17 UE power savings) could be reused. However, a field in DCI needs to be reserved to indicate on which resources the UE shall perform PDCCH monitoring. Also, there is a delay associated with the signaling. Therefore, SSGS functionality seems not optimized for very frequent SS group switching that may be needed with SBFD.

Options available with current standard specifications to provide different CORESET configurations in different slots/symbols all present limitations in terms of either limited flexibility or excessive signaling overhead.

In RAN#113, the following was captured in the TR:

Agreement The following conclusion is to be captured in the TR gNB can configure a CORESET and a search space in a way such that the MOs of the search space occur in either SBFD or non-SBFD symbols, or the MOs of the search space occur in both SBFD and non-SBFD symbols but the associated CORESET does not overlap the boundary of a DL subband in SBFD symbols. If it is agreed to be beneficial that a CORESET and a search space are configured that the MOs of the search space occur in both SBFD and non-SBFD symbols and the associated CORESET overlaps the boundary of a DL subband in SBFD symbols, at least the following options can be considered for SBFD-aware UE: - Option 1: Separate valid resources for the CORESET in SBFD symbols and in non-SBFD symbols. - Option 2: Rate matching or puncturing on the REG(s) of a PDCCH outside DL subband(s). - Option 3: UE does not monitor a PDCCH candidate if it is mapped to one or more REs that overlap with REs outside DL subband(s). - Option 4: Drop search space(s) when the associated CORESET overlaps with RBs outside DL subband(s) - Option 5: Separate search spaces associated with a CORESET in SBFD and non-SBFD symbols Note: These options are applicable to at least USS.

Considering there may be different available resource for PDCCH in SBFD symbol and non-SBFD symbol, we propose that RAN1 studies solutions enabling dynamic adaptation of PDCCH resources based on the occurrence of SBFD symbols and the corresponding DL-UL frequency domain partitioning. Thus it can be assumed that the UE knows different available resource for PDCCH in different slot type. For example, the UE could be provided with multiple PDCCH CORESET configurations per Search Space (SS), e.g. one CORESET configuration associated with DL-only slots/symbols, and one CORESET configuration associated with SBFD slots/symbols, in our view this is covered by Option 1 in the agreement from RAN1 #112bis-e meeting. The UE determines the CORESET to be used based on the slot/symbol type (signaled by the gNB), where the CORESET for SBFD symbol can be fully on DL subband. Alternatively, the UE may determine the CORESET to be used based on whether the CORESET time/frequency resources overlap or not with UL subband resources, where the issue of this way may be too much limitation on CORESET utilization as only partial resource in one CORESET may be not in the DL subband. Another option is to use different assumptions for PDCCH mapping in SBFD and non-SBFD slots, e.g. PDCCH candidate only map to available CCE/REG resources for PDCCH in the slot.

By any way above, the PDCCH should not be mapped to REG/CCE that is not in DL subband in SBFD symbol. We do not think Options 2 can work well as rate matching in PDCCH around REG/CCE that is not in DL subband will degrade PDCCH performance. Actually, the PDCCH candidate that may cause PDCCH overlap with REG/CCE not in DL subband should not be detected by UE and even should not be counted in blind detection, so that the performance of PDCCH detection is not degraded in SBFD cases. While for Option 4, when search space overlapped with RB(s) outside of DL subband will be dropped, it will be a big restriction on configure the search space, especially for common search space, which may cause always search space for SBFD slot and non-SBFD slot should be configured separately, or PDCCH performance will be impacted when search space is dropped.

For the CORESET/ search space configuration Option 2 and Option 4 captured in the conclusion of RAN1 #113, there is performance degradation when the PDCCH is mapped on REG/CCE outside of the DL subband

For the CORESET/ search space configuration options agreed to be captured in the TR in RAN1 #113, RAN1 should only consider PDCCH mapping/detection methods in which it can be guaranteed that the PDCCH is only mapped on DL resource and UE only detect the PDCCH candidate on DL resource, so that there is no impact on PDCCH detection performance.

### gNB self-interference and Tx/Rx timing alignment aspects

In NR, the UE determines the Timing advance (TA) as the sum of a cell specific TA offset and a UE specific TA command (absolute or relative). The cell specific TA offset can be configured by the network in SIB1 (n-TimingAdvanceOffset). If n-TimingAdvanceOffset is not provided by the serving cell, the UE assumes TA offset in Table 7.1.2-2 in 38.133. In case of TDD in FR1 with no LTE-NR coexistence, the value of the cell specific TA offset is 25600*Tc = 13.03 us.

While the UE specific TA is typically used to compensate for the propagation delay so that reception of UL signals from different UEs is synchronized at the gNB receiver, the cell specific TA offset is introduced to shift the relative timing between UL and DL so that the overall guard time that is necessary to allow a half-duplex UE to switch from downlink reception to uplink transmission and vice versa can be minimized.

In previous meetings, several companies raised the issue of FFT time misalignment between UL Rx and DL Tx when the gNB is simultaneously receiving in UL and transmitting in DL during SBFD slots and NTA, offset > 0. Not having the same symbol timing in DL and UL can effectively increase the self-interference at the gNB. Based on those discussions, the following conclusion was reached:

Conclusion Time misalignment at gNB between UL receptions and DL transmissions due to configuration of non-zero NTA,offset at UE can lead to increased interference assuming no gNB transmit chain side impairments and no filtering of DL subband(s) in the gNB Rx chain. - FFS the case with gNB transmit chain impairments and/or filtering of DL subband(s) in the gNB Rx chain - FFS whether/how to mitigate the interference increase, including impact to legacy UEs

Self-interference at the gNB is one of the main challenges behind practical deployment of SBFD. Even when gNB transmit chain impairments and/or filtering of DL subband(s) in the gNB Rx chain are considered, FFT time misalignment between UL receptions and DL transmissions at the gNB can increase the self-interference by several dBs, as compared to the case with time alignment. Therefore, we think that solutions to achieve time alignment between UL and DL signals at the gNB should be studied. In previous RAN1 meetings some companies suggested using NTA, offset = 0. This has two main limitations:

- Though in principle NTA, offset can be configured by the network to any value among 0, 25600*Tc and 39936*Tc by parameter n-TimingAdvanceOffset, in practice, legacy UEs always assume the value specified in Table 7.1.2-2 in 38.133 (for the corresponding frequency range and co-existence scenario) independently of what the network indicates in system information

- The guard time between the end of a DL transmission in a DL slot and the start of an UL reception in a subsequent SBFD slot (on the same frequency resources) is increased (as compared to legacy TDD with NTA, offset > 0)

Use of NTA, offset = 0 is only possible in SBFD slots and it increases the required guard time between the end of a DL transmission in a DL slot and the start of an UL reception in a SBFD slot.

To solve the backward compatibility issue with legacy UEs, some companies have proposed to configure and operate with two different values of NTA, offset in SBFD slots and TDD UL slots. However, changing the TA offset from 0 in SBFD slots to e.g. 13us in TDD UL slots may cause an overlap between two consecutive UL transmissions by the same UE in the transition from an SBFD slot to a TDD UL slot. This may result in one additional symbol lost in the transition between a SBFD and a TDD UL slot. Overall, the overhead required for switching between DL and UL (an vice versa) increases with SBFD operation as compared to TDD. Therefore, how to reduce such overhead such as enabling partial OFDM symbol transmission in the guard periods between SBFD and DL-/UL-only slots should be studied.

Use of NTA, offset = 0 in SBFD slots and NTA, offset > 0 in TDD UL slots further increases the required overhead needed to switch between legacy TDD DL/UL slots and SBFD slots.

Consider using NTA, offset = 0 in SBFD slots and NTA, offset > 0 in TDD UL to solve the FFT misalignment problem between UL Rx and DL Tx in SBFD slots.

If it is agreed to use NTA, offset = 0 in SBFD slots and NTA, offset > 0 in TDD UL, study how to reduce the increased overhead when switching between TDD DL/UL slots and SBFD slots.

### CLI mitigation enhancements

#### UE-2-UE CLI

Schemes for UE-to-UE CLI mitigation have been specified during Rel-16 WI on Cross link Interference (CLI) handling and Remote Interference Management (RIM) for NR. Enhancements to the Rel-16 UE-to-UE CLI framework are being discussed under A.I. 9.3.3 with focus on dynamic TDD, i.e. A.I. 9.3.3 is already discussing enhancements for improved handling of inter-cell UE-to-UE CLI. Though those enhancements may also be applicable to SBFD, we propose to focus in here on enhancements for improved handling of intra-cell inter-subband UE-to-UE CLI.

Advanced UE RF requirements

One obvious way to mitigate intra-cell inter-subband UE-to-UE CLI is to tighten the UE in-band emission requirements. Note that typically the UE TX is optimized for efficiency, so Power Amplifier (PA) nonlinearity is a dominant source of spectral regrowth. It may be feasible to reduce emissions bringing the PA into more linear operation at the expense of higher power consumption. The UE capability to adapt its UL/DL digital filter’s bandwidth to match the UL/DL subband in SBFD symbols may also help reducing emissions. In any case, the feasibility of defining stricter (in-band) emission requirements for SBFD operation is not for RAN1 to discuss.

One obvious way to mitigate intra-cell inter-subband UE-to-UE CLI is to tighten the UE in-band emission requirements. The UE being able to match its UL/DL digital filter’s bandwidth to the UL/DL subband in SBFD symbols may also help reducing the UE in/band emissions. However, such advance UE RF requirements are for RAN4 to be discussed.

Schemes for accurate measurement of CLI leakage

In RAN1 #112 and RAN1 #112bis-e, it has been agreed to study different methods for inter-UE inter-subband CLI measurements.

RAN1 #112 Agreement For inter-UE inter-subband CLI measurement, study at least the following methods: - Method#1: victim UE measures RSSI within DL subband - FFS: Whether SINR can be measured - Method#2: victim UE measures RSRP of aggressor UE within UL subband - Method#3: victim UE measures RSSI within UL subband - Note: the restriction in Rel-16 that CLI is only measured within DL BWP does not forbid UE to measure CLI in UL subband when UL subband is confined within DL BWP. RAN1 #112bis-e Agreement For inter-UE inter-subband CLI measurement, study Method#2 and Method#3 considering: - Necessity/benefit compared with measurement within DL subband - Whether/how to estimate CLI from RSRP/RSSI measurements within UL subband / guardband - Whether UE is required to measure RSRP/RSSI within UL subband and receive DL in DL subband(s) simultaneously - Whether existing CLI measurement and report framework can be reused to support RSRP/RSSI measurements within UL subband - If not, identify the potential impact Agreement For semi-static SBFD, a SBFD aware UE does not transmit UL channels/signals or receive DL channels/signals on the guardband(s) that the UE is aware of. - FFS: Measurement in guardband for the purpose of CLI measurement RAN1 #113 Agreement The following conclusion is to be captured in the TR - For the methods agreed to be studied for inter-UE inter-subband CLI measurement, Method #2 and Method #3 can be used for identifying the aggressor UE(s) if orthogonal resources are allocated for different aggressor UE(s); and Method #2 and #3 can at least provide higher interference signal strength than inter-subband interference leakage based measurements in Method #1. Furthermore, such measurement is not subject to inter-cell DL interference. - It is feasible for UE to measure RSRP/RSSI within UL subband if within active DL BWP and receive DL in DL subband(s) simultaneously similar as simultaneous RSRP/RSSI measurement and DL reception in Rel-16. - The existing CLI measurement and report framework can be reused to support RSRP/RSSI measurements within UL subband when UL subband is confined within active DL BWP.

For the inter-UE inter-subband CLI measurement, the methods agreed up to RAN1 #112bis-e consider the CLI measurements performed within the UL subband or within the DL subband. However, in RAN1 #112bis-e, it has been agreed to further study the measurements in the guardband for the purpose of CLI measurement. In RAN1 #113 conclusions on the advantages of the different methods were captured in the TR.

In addition to methods 1, 2 and 3, we see advantages of the measurements within the guardbands when compared to Method #1, in which the victim UE measures RSSI within the DL subband. In the DL subband, the UE would measure both the leakage from transmissions from the aggressor UEs, and the power received from the gNB DL transmissions – unless the gNB mutes its own resources during the RSSI measurements. Therefore, the obvious advantage of the CLI measurements within the guardband is the possibility of measuring the UL leakage without muting the DL transmissions at the gNB, so it leads to a better resource utilization. Furthermore, by allowing an RSSI measurement to be configured also in the guard-bands, the network could in principle allocate a wideband resource for the RSSI measurements, i.e. spanning through different subbands, and the UE could report the results per subband type, for example: DL, UL or Guardband. Using these reports, the gNB could compare the RSSI in different subbands, resulting in a more accurate measurement in the ed.

The possibility of configuring an RSSI measurement in DL subbands, UL subbands and guardbands allows the gNB to configure a wideband RSSI measurement, across different types of subbands. The UE can use the measurement results to assess simultaneously the RSSI in different types of subbands. One option to configure the measurements within the guardbands is to configure zero power CSI-RS. This wouldn’t lead to any transmissions within the guardband, so it would be in accordance with the agreement made in RAN1 #112bis-e.

As said in [4], we do not see the different methods discussed in RAN1 #112 meeting as mutually exclusive. In case of SBFD operation, measurements of both the CLI within the UL subband, and the CLI-leakage within the DL subband are important for the network to handle the UE-UE CLI, as the CLI within the UL subband will impact the UE AGC and the leakage in the DL subband will impact the DL SINR and quantization.

In Rel-16 inter-UE CLI measurements, there were two types of measurements: RSSI and CLI-SRS RSRP, and the same could be defined for SBFD aware UEs.

For inter-UE inter-subband CLI measurements, the victim UE measures at least:

- RSRP of the aggressor UE within UL subband

- RSSI within the guardbands

Additionally, in RAN1 #112bis-e, it has also been agreed to study the CLI-RSSI measurement/ report across DL subbands.

Agreement For UE-to-UE CLI-RSSI measurement/report across downlink subbands, study the following methods: - Method#1: separate CLI-RSSI measurement resources/reports in each DL subband - Note: supported in existing specifications - Method#2: CLI-RSSI measure/report in one DL subband only - Note: supported in existing specifications - Method#3: CLI-RSSI measurement/report based on non-contiguous CLI-RSSI resource across downlink subbands - FFS: report single or separate CLI-RSSI report(s) - FFS: details on determination of non-contiguous CLI-RSSI resource allocation

We think the discussion on CLI-RSSI measurement/report across downlink subbands in SBFD symbols is strictly related to the discussion on CSI-RS measurement/report, especially when it comes to resource configuration. Therefore, we propose to align the options for configuration of CLI-RSSI and CSI-RS resources across DL subbands in SBFD symbols. We support:

  - configuration of non-contiguous CLI-RSSI resource allocation equivalent to Option 2-2 for CSI-RS configuration (e.g. one contiguous CLI-RSSI resource allocation with non-contiguous CLI-RSSI resources across DL subbands derived by excluding frequency resources outside DL subband(s))

  - separate CLI-RSSI measurement reports in each DL subband (i.e. subband CLI-RSSI reporting)

#### gNB-2-gNB CLI

To avoid co-channel CLI in TDD deployments, coordinated scheduling might be achieved by simply aligning the TDD frame configuration among the neighbour cells. Concepts such as clustering of multiple gNBs is currently feasible by using the standardized Intended TDD DL-UL Configuration IE. In SBFD deployments, however, it is not currently possible to coordinate the sub-band configuration with the neighbour cells. The intra-subband cross-link interference can be completely removed if cells align the link direction of the sub-bands, which it is expected to provide performance benefits. Therefore, we propose to extend the current framework of exchange of the intended TDD frame configuration over Xn interference to also support the intended subband configuration.

Capture the following conclusion in the TR: exchange of intended SBFD configuration between the gNBs, e.g. over Xn interface, may be beneficial to reduce the inter-subband interference and improve the SBFD performance.

#### Self-interference

As discussed in our companion contribution on dynamic TDD enhancements [7], boosting the UE Tx power and/or reducing the gNB power in slots/symbols affected by gNB-to-gNB CLI from overlapping frequency resources can be a simple yet effective way to handle cross-link interference. Similar solutions may also be applicable with SBFD operation e.g. to compensate for the decrease in UL SINR due to gNB self-interference in SBFD symbols/slots. Therefore, we propose the following:

The potential benefits of boosting the UE Tx power and/or reducing the gNB power in SBFD slots/symbols shall be further investigated as a potential method to boost the UL received SINR in slots/symbols affected by gNB self-interference.

### Controlling access to SBFD cells

When a SBFD cell is serving both legacy UEs and SBFD aware UEs, to simplify gNB implementation and/or to limit the impact on the performance of legacy UEs, the SBFD cell may decide to schedule legacy UEs only during DL-only (and U-only) slots, while SBFD-aware UEs may also be scheduled during SBFD slots.

Also, it could happen that a cell TDD UL resources are close to full utilization while its SBFD UL resources are still (at least partially) unused.

In any of the above situations, the network may want to facilitate SBFD aware UEs to (re)select a SBFD capable cell, while discouraging legacy UEs from doing the same. In other words, SBFD aware UEs could have higher probability of (re)selecting to a SBFD capable cell than legacy UEs. To partially achieve this, some companies have proposed to indicate future release UEs about the cell duplex operation mode so that such indication can be used to facilitate UE’s cell selection, e.g. SBFD capable UE can prioritize SBFD cells over non-SBFD cells. An alternative solution, also enabling legacy UEs to de-prioritize SBFD cells, could be to signal SBFD-specific (re)selection offsets and/or priorities that SBFD aware UE shall apply when performing cell (re)selection. In any case, the specific solution is not for RAN1 to discuss. Therefore, we propose the following:

RAN1 to study the potential benefits of SBFD aware UEs prioritizing SBFD cells and/or legacy UEs de-prioritizing SBFD cells when performing cell (re)selection. Detailed solutions are for RAN2 to discuss and specify if/when normative work for SBFD is agreed.

### UL/DL collision handling in SBFD slots/symbols

In RAN1 #110bis-e meeting it was agreed to:

Agreement: Identify if there are any cases of time domain conflict of UE’s UL and DL operation in the same SBFD symbol for SBFD aware UE - If there are, whether/how to avoid/handle such collision cases (as second step)

In RAN1 #111, there was a discussion about collision handling without agreements.

Proposal 1-12 ​ Proposed Conclusion:​ At least the following cases of time domain conflict of UL transmission and DL reception in the same SBFD symbol are identified.​ - Dynamic DL reception vs. semi-statically configured UL transmission​ - Dynamic UL reception vs. semi-statically configured DL transmission​ - Semi-statically configured DL reception vs. semi-statically configured UL transmission​ Note: PRACH transmissions and SSB receptions are not included.

Based on current specifications, the gNB plays an essential role in ensuring that no conflicts resulting in error cases or unspecified UE behavior occur at the UE. Some of these error cases are described below, while remaining details can be found in TS 38.213 Clause 11.1 and 11.1.1. Essentially, the gNB needs to ensure no contradicting indications are signaled to the UE, e.g. DCI triggering a DL reception on resources configured in UL (by SFI/DCI, or RRC/SIB configuration). For semi-static signals (e.g. SRS, PUCCH, CSI-RS, etc.) some more flexibility is allowed as the gNB can ‘cancel’ the corresponding transmission/reception of those signals (with some cancellation timeline restrictions).

- The UE does not expect the gNB to request UL transmission on resources configured as DL by higher-layer signaling.

- The UE does not expect to receive a SFI indicating UL/DL direction for a symbol that is configured by higher-layers as DL/UL, respectively

- For a set of symbols of a slot that are indicated to a UE as flexible by higher-layer signaling, the UE does not expect to receive both dedicated higher layer parameters configuring transmission from the UE in the set of symbols of the slot and dedicated higher layer parameters configuring reception by the UE in the set of symbols of the slot.

- The UE does not expect to receive SFI indicating a set of symbols as uplink, and receive another DCI indicating the UE to receive PDSCH/CSI-RS in the set of symbols. And vice versa: receive SFI indicating DL symbols and other DCI indicating UL transmission.

- If a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols

  - The UE cancels the SRS, PUCCH, PUSCH or PRACH in the set of symbols but only if some cancellation timeline is met.

- If a UE is configured by higher layers to receive a CSI-RS or a PDSCH in a set of symbols of a slot and the UE detects a DCI format 2_0 with a slot format value other than 255 that indicates a slot format with a subset of symbols from the set of symbols as uplink or flexible, or the UE detects a DCI format indicating to the UE to transmit PUSCH, PUCCH, SRS, or PRACH in at least one symbol in the set of the symbols,

  - the UE cancels the CSI-RS reception in the set of symbols of the slot or cancels the PDSCH reception in the slot.

- For a set of symbols of a slot indicated to a UE by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the UE does not expect the set of symbols to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated.

- For a set of symbols of a slot indicated to a UE by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set, the UE does not expect to detect a DCI format 2_0 with an SFI-index field value indicating the set of symbols of the slot as uplink.

Table 2 UE behavior in case of UL/DL collision according to current specifications

|  | Dyn. DL | Dyn. UL | Semi-static DL | Semi-static UL | SSB | Type 0 CSS | Valid RO |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Dyn. DL |  |  |  |  |  |  |  |
| Dyn. UL | Not expected |  |  |  |  |  |  |
| Semi-static DL | - | Dynamic UL |  |  |  |  |  |
| Semi-static UL | Dynamic DL (within cancellation timeline) | - | Not expected |  |  |  |  |
| SSB | - | Not expected (SSB) | - | Not allowed (SSB) |  |  |  |
| Type 0 CSS | - | Not expected (Type 0 CSS) | - | Not expected (Type 0 CSS) | - |  |  |
| Valid RO | Dynamic DL (within cancellation timeline) | - | Not expected | - | Not allowed (SSB) | Not expected (Type 0 CSS) |  |

UL-DL collision handling according to currents specifications is summarized in Table 2. We consider only cases with UL-DL collisions. For valid RO, based on TS 38.213, a RO is invalid if it collides with SSB. Therefore, we assume collisions between valid RO and SSB are not allowed in current specifications. We assume the same applies for collision between valid RO and Type 0 CSS, as the UE does not expect Type 0 CSS in slots/symbols that are configured as UL by either higher layer signaling or SFI.

For the same reason, we consider as not allowed/expected collisions between dynamic or semi-static UL and SSB or Type 0 CSS. As a valid RO can in principle be configured in a set of symbols indicated as flexible by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, we assume that in case of dynamic DL colliding with valid RO, dynamic DL transmission is prioritized. On the other hand, collisions between valid RO and semi-static DL are not expected (for the same reason why collision between semi-static DL and UL is not expected).

One of the targets of SBFD is to reduce latency as compared TDD, especially in uplink. To achieve this, the network may need to configure the UE with semi-static UL transmission opportunities with very high periodicity, in principle up to every slot. In this case, as the UE is still half duplex, new collision rules may need to be specified to determine the conditions based on which the UE shall prioritize between semi-static UL and semi-static or dynamic DL.

Also, depending on whether SBFD operation in SSB symbols (and/or Type 0 CSS symbols) is allowed, new prioritization rules may be needed to determine when a UE is allowed to transmit in UL during SSB and/or Type 0 CSS symbols. Therefore, we propose:

For time domain conflict of SBFD aware UE’s UL and DL operation in the same SBFD symbol, at least study whether/how to avoid/handle the following collision cases:

- Semi-static UL (including valid RO) vs. semi-static or dynamic DL (including SSB/Type 0 CSS, if allowed)

- Dynamic UL vs. SSB/Type 0 CSS (if allowed)

Additionally, for the DL vs. UL transmissions collision handling, it is straightforward for UE to determine whether there is collision or not by determining whether there is at least one overlapping symbol between the two transmissions. This is not always the case for determining whether DL and UL transmissions are collided or not in SBFD operation, given that UE needs time for switching from reception to transmission. In SBFD operation, even if the symbols used for DL receptions and UL transmissions are not overlapping, DL and UL transmissions are collided if there is not enough time for the UE Rx-Tx switching.

In SBFD operation, new collision cases arise if the time between DL reception and UL transmission is less than the time required for Rx to Tx switching.

: For SBFD aware UE’s, study how to handle collision cases between DL and UL when the time between DL receptions and UL transmissions is less than the Rx-Tx switching time.

Finally, Rel-16 introduced different priorities namely high priority (HP) and low priority (LP) to handle the collisions in the UL. It was not necessary to configure priority index for the dynamic PDSCH because DL vs UL collision was not allowed. Even though SBFD is a feature that aims to enhance uplink performance and latency, i.e., it is expected that UL transmissions are prioritized, there might be scenarios in which the network wants to prioritize the DL or drop part of the DL.

In our view there are different alternatives to handle the semi-static UL vs. semi-static or dynamic DL collisions. In case it is decided that UL transmissions are prioritized by default, it should be possible to the gNB to configure priority index for the dynamic PDSCH as well, in case needed, as discussed above. A second alternative is to allow the configuration of the priorities for the transmission or reception of the different signals / channels. The priority can be configurable and valid during a determined period. Instead of configuring a priority for a given channel, the gNB can indicate a period under which a set of priorities apply. In addition, always assuming a fixed (default) set of collision handling rules (e.g., always prioritize UL transmissions) may be restrictive and suboptimal given that some critical DL channels (e.g., PDCCH) may be blocked. Therefore, RAN1 should also consider the possibility of reserving some slots, wherein a different set of collision handling rules is applied (e.g., DL channels are prioritized). This offers gNB the possibility to schedule DL channel with priority in those slots, if needed.

For SBFD aware UE’s, study configuration of collision handling rules between DL receptions and UL transmissions.

# Conclusion

In this contribution, we discussed subband non-overlapping full duplex (SBFD) for NR and included considerations on the feasibility, performance, and system design enhancements of SBFD. Based on the discussions, we have the following observations and proposals:

Proposals:

Proposal 1:	Support dynamic indication of SBFD subbands in time-domain.

Proposal 2:	For a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon, support converting the SBFD symbol to DL-only symbol.

Proposal 3:	For a SBFD symbol configured as flexible in TDD-UL-DL-ConfigCommon, support converting the SBFD symbol to DL-only or UL-only.

Proposal 4:	For a SBFD aware UE in a symbol semi-statically configured as DL or Flexible in tdd-UL-DL-ConfigurationCommon, support mechanisms enabling dynamic configuration of the SBFD UL subband in the symbol.

Proposal 5:	For a SBFD aware UE semi-statically configured with UL subband in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study:

· UL transmissions within UL subband are allowed in the symbol

· UL transmissions outside UL subband are not allowed in the symbol

· Frequency locations of DL subband(s) are known to the SBFD aware UE

· The frequency location of DL subband(s) can be explicitly indicated or implicitly derived

· DL receptions within DL subband(s) are allowed in the symbol

· The symbol can be converted to a DL-only symbol

Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol

Proposal 6:	For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon:

•	UL transmissions within UL subband are allowed in the symbol

•	UL transmissions outside UL subband are not allowed in the symbol

•	Frequency locations of DL subband(s) are known to the SBFD aware UE

•	DL receptions within DL subband(s) are allowed in the symbol

The symbol can be converted to a DL-only symbol or UL-only symbol

Proposal 7:	Signaling of the frequency and time location of the UL subband via SIB is assumed as baseline. Signaling via dedicated RRC can be considered as an additional (not alternative) option.

Proposal 8:	The frequency allocated to SSB or CORESET0 is used as reference for the configuration of the UL subband.

Proposal 9:	UL transmissions during SSB symbols are not allowed.

Proposal 10:	Assume RB-set based scheme as the baseline. With the RB-set based scheme, a UE’s active BWP may include PRBs that are used by the serving cell for the opposite link direction.

Proposal 11:	Capture in the TR the following conclusion: It is beneficial to allow PRACH and Msg3 transmissions in SBFD symbols. Initial access during SBFD symbols can increase PRACH capacity, PRACH/Msg3 coverage (e.g. by 3 or 6dB), reduce the collision probability as well as reduce latency of random access as much as 18 slots, depending on the configurations being compared.

Proposal 12:	For SBFD-aware UEs, for an RBG that overlaps the subband boundary:

•	The UE assumes that the part of the UL RBG outside the UL subband cannot be used for transmissions

Proposal 13:	FDRA type 1 enhancements are needed to support allocation across two DL subbands in a SBFD symbol with one RIV and existing DCI formats.

Proposal 14:	For CSI-RS resource set frequency domain resource allocation and CSI reporting configuration across non-contiguous DL subband, CSI-RS resources are configured over the entire bandwidth, with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s).

Proposal 15:	For CSI report associated with periodic/semi-persistent CSI-RS in case the periodicity is such that CSI-RS instances occur in both SBFD and non-SBFD symbols, two CSI-ReportConfig are configured. Both CSI-ReportConfigs are associated with the same CSI-RS. The CSI report associated with one CSI-ReportConfig is derived based on CSI-RS instances in SBFD symbols only. The CSI report associated with the second CSI-ReportConfig is derived based on CSI-RS instances in non-SBFD symbols only.

Proposal 16:	A slot can consist of both SBFD and non-SBFD symbols.

Proposal 17:	The exact value of the guard-period between SBFD and non-SBFD symbols can be discussed in the WI phase.

Proposal 18:	If a transmission is not allowed to be mapped across SBFD and non-SBFD symbols in a slot, RAN1 should further discuss UE behavior for TDRA determination for the transmission occasion that spans across symbol types in the slot, especially for semi-static PUSCH/PDSCH and PUSCH/PDSCH with repetitions and TBoMS.

Proposal 19:	For a slot covering both SBFD symbol and non-SBFD symbol, RAN1 should also consider the option that UE can transmit or receive the physical channel/signal only in SBFD or non-SBFD symbol.

Proposal 20:	For SBFD aware UEs, for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots, the transmissions/receptions can be in SBFD symbols and non-SBFD symbols

Proposal 21:	For CG-PUSCH and DL SPS, introduce SBFD-specific configurations where one set of radio parameters is used depending on the corresponding slot format.

Proposal 22:	For SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots, introduce separate configurations (as in Option 1-1), or related configurations (as in Option 1-3).

Proposal 23:	For Scheduled/configured TBoMS across SBFD symbols and non-SBFD symbols, with or without repetition:

	consider different number of RBs for the repetitions (or the slots of a TBoMS) in SBFD slots compared to the repetitions (or the slots of the TBoMS) in non-SBFD slots

	combine both repetitions and TBoMS, for example applying TBoMS for the repetitions of a PUSCH in SBFD slots, while the usual single-slot repetition is used for the repetitions of the PUSCH in non-SBFD slots

	Use of different density of DMRS symbols in SBFD and non-SBFD symbols

	Use of different waveforms for the transmissions on different slots

Proposal 24:	For the CORESET/ search space configuration options agreed to be captured in the TR in RAN1 #113, RAN1 should only consider PDCCH mapping/detection methods in which it can be guaranteed that the PDCCH is only mapped on DL resource and UE only detect the PDCCH candidate on DL resource, so that there is no impact on PDCCH detection performance.

Proposal 25:	Consider using NTA, offset = 0 in SBFD slots and NTA, offset > 0 in TDD UL to solve the FFT misalignment problem between UL Rx and DL Tx in SBFD slots.

Proposal 26:	If it is agreed to use NTA, offset = 0 in SBFD slots and NTA, offset > 0 in TDD UL, study how to reduce the increased overhead when switching between TDD DL/UL slots and SBFD slots.

Proposal 27:	For inter-UE inter-subband CLI measurements, the victim UE measures at least:

	RSRP of the aggressor UE within UL subband

	RSSI within the guardbands

Proposal 28:	Capture the following conclusion in the TR: exchange of intended SBFD configuration between the gNBs, e.g. over Xn interface, may be beneficial to reduce the inter-subband interference and improve the SBFD performance.

Proposal 29:	The potential benefits of boosting the UE Tx power and/or reducing the gNB power in SBFD slots/symbols shall be further investigated as a potential method to boost the UL received SINR in slots/symbols affected by gNB self-interference.

Proposal 30:	RAN1 to study the potential benefits of SBFD aware UEs prioritizing SBFD cells and/or legacy UEs de-prioritizing SBFD cells when performing cell (re)selection. Detailed solutions are for RAN2 to discuss and specify if/when normative work for SBFD is agreed.

Proposal 31:	For time domain conflict of SBFD aware UE’s UL and DL operation in the same SBFD symbol, at least study whether/how to avoid/handle the following collision cases:

	Semi-static UL (including valid RO) vs. semi-static or dynamic DL (including SSB/Type 0 CSS, if allowed)

	Dynamic UL vs. SSB/Type 0 CSS (if allowed)

Proposal 32:	: For SBFD aware UE’s, study how to handle collision cases between DL and UL when the time between DL receptions and UL transmissions is less than the Rx-Tx switching time.

Proposal 33:	For SBFD aware UE’s, study configuration of collision handling rules between DL receptions and UL transmissions.

Observations:

Observation 1:	With clustered UE distribution in UMa Scenario, the presence of co-channel inter-sector and co-channel inter-gNB inter-subband CLI impacts the SBFD performance and hinders any gain on the UL UPT in medium and high loads with respect to static TDD. Gains are only observed in the 5th percentile UL UPT for a low traffic load scenario, when SBFD is compared to static TDD with 23 dBm transmit power, in which the SBFD UL throughput is ~4.5 times larger than the static TDD UL UPT. However, the improvement is only observed when assuming optimistic inter-sector isolation conditions, and the same antenna gain. In this scenario, static TDD with PC2 UEs achieves 80% of the gain that SBFD provides without any degradation in the DL UPT performance. In all other scenarios, static TDD with PC2 UEs outperforms SBFD.

Observation 2:	With clustered UE distribution in UMa Scenario, there is significant degradation of UE DL throughput due to UE-UE CLI even at low load. This mainly occurs when one or more coverage limited UEs transmit over a few, e.g. 4, RBs with full 23 dBm UL transmit power which generates large amount of UL leakage interference to other UEs receiving in DL.

Observation 3:	For Deployment Case 1 FR1 Indoor Office scenario and large packet size:

-	SBFD XXXXX and static TDD DDDSU provide almost identical UL and DL performance.

-	SBFD XXXXU provides 50% to 70% UL UPT improvement at the expense of a DL UPT degradation of approximately 20% at low and medium load, and up to 70% in the 5%-ile DL average UPT at high loads.

-	For low and medium loads, dynamic TDD provides a UL UPT gain between 100% to 150% as compared static TDD and SBFD XXXXX, and up to 65% gain over SBFD XXXXU. Contrary to SBFD XXXXU, dynamic TDD does not come at the expense of significantly worse DL average UPT performance.

Observation 4:	For Deployment Case 1 FR1 Indoor Office scenario and small packet size:

	SBFD XXXXX and XXXXU provides approximately 50% higher UL 5%/50%/95%-ile UL UPT performance compared to static TDD.

	SBFD XXXXX provides also a small 3% to 10% DL UPT gain over static TDD, while SBFD XXXXU performs worse than static TDD in DL in most of the cases.

Observation 5:	For Deployment Case 1 FR2 Indoor Office scenario and small packet size:

	SBFD offers up to 50% UL throughput improvement for all load conditions.

	In the DL direction, SBFD outperforms static TDD most of the time by a small margin of around 5%.

Observation 6:	The dynamic SBFD results in a FR-1 indoor office scenario show that dynamic SBFD provides a DL performance improvement of 35% when compared to a semi-static SBFD configuration (XXXXU). In the UL, the results show that the performance of dynamic SBFD is comparable to the performance of semi-static SBFD (XXXXU) and provides over 70% improvement over semi-static SBFD XXXXX and static TDD DDDSU. In other words, dynamic SBFD provides comparable UL performance to semi-static SBFD, but without any degradation to the DL performance, when compared to static TDD.

Observation 7:	The dynamic SBFD results in a FR-1 indoor office scenario show that in low load and medium load, the DL average UPT of Dynamic SBFD outperforms Dynamic TDD. When the UL average UPT is compared, dynamic SBFD only outperforms dynamic TDD in high load.

Observation 8:	SBFD operation in symbols configured as DL improves the UL performance at the cost of DL performance. Dynamic adaptation of the frame structure based on traffic changing, for example, by converting SBFD symbols to DL-only, or allowing DL receptions outside the DL subband is beneficial, since it allows adaptation to the current traffic conditions by dynamically falling back to TDD.

Observation 9:	If the SBFD symbol is dynamically converted to a DL-only or UL-only symbol, it is not characterized by DL/UL subbands and guardband(s).

Observation 10:	Symbols configured as flexible in TDD-UL-DL-ConfigCommon can be used for DL or UL since Rel-15.

Observation 11:	Not all PRBs are suitable for the configuration of the UL subband, for example, those configured for SSB or CORESET0 transmission (if allowed in SBFD symbols). Using the frequency allocated to the transmission of the SSB or CORESET0 as reference, and an offset for the configuration of the UL subband, can reduce the configurable range and number of bits used to indicate the UL subband to the UE.

Observation 12:	The basic BWP based scheme is rather straightforward but has severe UE peak rate and signaling overhead issue.

Observation 13:	For the advanced BWP based scheme, there should be no additional gap due to BWP switch in the transition from SBFD slots to TDD slots, and vice versa.

Observation 14:	In terms of digital filter bandwidth adaptation at the UE, there is substantially no difference between the RB-set based and the advanced BWP based approach.

Observation 15:	Unless new in-band emission requirements are specified, better UL transmission and DL reception filtering should not be considered as an advantage of any of the proposed schemes.

Observation 16:	Operation according to the basic BWP based scheme is not precluded by RB-set based signaling.

Observation 17:	Allowing SBFD aware UEs to send PRACH and Msg3 in SBFD symbols can result in a decrease in latency of at least 2 slots, and at most 18 slots when compared to static TDD DDDSU, even if no additional source of delays, such as repetitions, are considered.

Observation 18:	UL coverage is limited by PRACH instead of PUSCH, so using SBFD slots for PRACH transmission can increase the UL coverage up to 3dB or 6dB if the number of slots used to consecutive PRACH is increased to 2 or 4.

Observation 19:	Allowing SBFD aware UEs to send PRACH in SBFD symbols can reduce the number of collisions for sending PRACH since SBFD aware UEs would not necessarily be competing for PRACH resources with legacy UEs.

Observation 20:	Enabling initial access in SBFD symbols can increase the PRACH capacity, PRACH coverage, as well as reduce initial access delay.

Observation 21:	Current FDRA Type 1 does not support resource allocation across two DL subbands.

Observation 22:	By changing the reference RB in type 1 FDRA allocation in case the UL subband is located between two DL subbands in the BWP, and wrap-around the PRB index it is possible to use the RIV and DCI formats to provide non-contiguous allocation in type-1 FDRA.

Observation 23:	NR specification allows for symbols of different types in special slots. A transition from DL to UL requires a slot to consist of different symbol types, at least in the special slots, if the configuration is kept compatible with legacy TDD DL/UL configuration.

Observation 24:	The obvious use cases for allowing a transmission to be mapped across SBFD and non-SBFD symbols in a slot are to allow full-slot scheduling of UL or DL transmission in the slot, without scheduling restriction.

Observation 25:	Several benefits of allowing a transmission to be mapped across SBFD and non-SBFD symbols in a slot could be observed, namely avoiding either scheduling restriction at the scheduler or specific handling rules at the UEs or offering better performance for data (or control bits, if any) thanks to interleaving the encoded bits across symbol types given that lower interference is expected in non-SBFD symbols.

Observation 26:	If a long guard time is introduced to account for implementation limitation, the benefit of allowing a transmission to be mapped across SBFD and non-SBFD symbols in a slot would be less significant.

Observation 27:	Options available with current standard specifications to provide different CORESET configurations in different slots/symbols all present limitations in terms of either limited flexibility or excessive signaling overhead.

Observation 28:	For the CORESET/ search space configuration Option 2 and Option 4 captured in the conclusion of RAN1 #113, there is performance degradation when the PDCCH is mapped on REG/CCE outside of the DL subband

Observation 29:	Use of NTA, offset = 0 is only possible in SBFD slots and it increases the required guard time between the end of a DL transmission in a DL slot and the start of an UL reception in a SBFD slot.

Observation 30:	Use of NTA, offset = 0 in SBFD slots and NTA, offset > 0 in TDD UL slots further increases the required overhead needed to switch between legacy TDD DL/UL slots and SBFD slots.

Observation 31:	One obvious way to mitigate intra-cell inter-subband UE-to-UE CLI is to tighten the UE in-band emission requirements. The UE being able to match its UL/DL digital filter’s bandwidth to the UL/DL subband in SBFD symbols may also help reducing the UE in/band emissions. However, such advance UE RF requirements are for RAN4 to be discussed.

Observation 32:	In SBFD operation, new collision cases arise if the time between DL reception and UL transmission is less than the time required for Rx to Tx switching.

# References

- RP-223041, “Study on Evolution of NR Duplex Operation”, CMCC

- R1-2306874, “On the evaluation methodology for NR duplexing enhancements”, Nokia, Nokia Shanghai Bell, RAN1 #113.

- R1-2304031, Summary #4 of subband non-overlapping full duplex, Moderator (CATT), RAN1 #112bis-e

- R1-2303016, “On subband non-overlapping full-duplex”, Nokia, Nokia Shanghai Bell, RAN1 #112bis-e

- R1-2306075, Summary #3 of subband non-overlapping full duplex, Moderator (CATT), RAN1 #113

- RSW-230026, GSMA - prospective on R19 from MNOs, GSM association, RAN Rel-19 workshop

- RWS-230337, SK telecom’s view on Rel-19, SK Telecom

# Annex A: Detailed simulation assumptions for SBFD

Table B1: Simulation assumptions for FR1 UMa Scenario

| Parameters | Value |
| --- | --- |
| Scenario | Urban Macro (TR 38.901) with 7x3=21 cells and 500 meter ISD. SBFD Deployment Case 1 with single operator and all gNBs using the same UL-DL SBFD sub-band partitioning |
| SCS | 30 kHz |
| Carrier bandwidth | 100 MHz, 273 RBs |
| gNB total transmit power | 53 dBm |
| UE Tx power | 23 dBm |
| UE position | UE clustering in line with RAN1#111 agreements: 10 UEs per cell at 1.5 meter height. 80% of the UEs in 1 cluster per macro cell area with 25 meter radius. UEs dropped within the UE cluster are indoor with 3km/h; UEs dropped outside the UE cluster are outdoor in car with 30km/h |
| Traffic model | FTP3 UL and DL traffic; 0.125 MB payload size in UL and 0.5 MB in DL |
| Channel modelling | gNB-UE: TR 38.901 UMa gNB-gNB: TR 38.901 UMa with replacement of the UE’s antenna height with gNB’s antenna height and updated angular spread. 75% of LOS probability for gNBs within ISD distance UE-UE: TR 38.901 UMi with O2I according to TR 38.802. Both large-scale and small-scale fading effects are modeled between all gNB-gNB links. Only large-scale fading is modeled between UE-UE links. |
| BS antenna configurations | TDD: 16 Tx/16 Rx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (8, 8, 2, 1, 1; 2, 4); dH = 0.5λ, dV = 0.8λ; SBFD: 16 Tx/16 Rx antenna ports (Opt 2) (M, N, P, Mg, Ng; Mp, Np) = (8, 8, 2, 1, 1; 1, 4) (per panel group) dH = 0.5λ, dV = 0.8λ; SBFD: 8 Tx/8 Rx antenna ports (Opt 3) (M, N, P, Mg, Ng; Mp, Np) = (8, 4, 2, 1, 1; 1, 4) dH = 0.5λ, dV = 0.8λ; |
| UE antenna configuration | 2 Tx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 1) 4 Rx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) dH=0.5 |
| UE & BS receiver | MMSE-IRC |
| Number of UEs per cell | 10 |
| UE power control | Open-loop power control with alpha = 0.7 and p0=-80 |
| DL/UL Transmission mode | DL: Single user MIMO with rank 2 UL: Single user MIMO with rank 1 |
| Frame structure | TDD: DDDSU with S=[12D:2G:0U] SBFD: XXXXX with X denoting a SBFD slot with DGUGU = [104, 5, 55, 5, 104] PRB assignment. “D”, “U” and “G” refers to downlink subband, uplink subband and guard bands, respectively. |
| SBFD interference modeling | Setting 1 (optimistic): 149 dB RSI and 135.5 dB for inter-sector isolation Setting 2 (realistic): 149 dB RSI and 117.5 dB for inter-sector isolation gNB-to-gNB inter-site: ACLR: 45 dB, ACS: 46 dB. UE ACLR: IBE requirements defined in TS 38.101-1 UE ICS = 33 dB |
| Noise figure | BS: piece-wise noise figure model with A = -43 dBm, B = -25 dBm, C = 5 dB, D = 14 dB UE: 9 dB |

Table B2: Simulation assumptions for FR1 Indoor Scenario

| Parameters | Value |
| --- | --- |
| Scenario | TR 38.901 Indoor Office of 120x50x3 meter with 12 cells deployed in the ceiling with 20 meter inter-site distance. SBFD Deployment Case 1 with single operator and all gNBs using the same UL-DL SBFD sub-band partitioning |
| SCS | 30 kHz |
| Carrier bandwidth | 100 MHz, 273 RBs |
| gNB total transmit power | 24 dBm |
| UE Tx power | 23 dBm |
| UE position | 120 randomly distributed UEs in the office area. |
| Traffic model | FTP3 UL and DL traffic; 0.125 MB payload size in UL and 0.5 MB in DL FTP3 UL and DL traffic; 1kB payload size in UL and 4kB in DL |
| Channel modelling | gNB-UE, UE-UE and gNB-gNB: TR 38.901 InH Both large-scale and small-scale fading effects are modeled between all gNB-gNB and UE-UE links. |
| BS antenna configurations | TDD: 32 Tx/32 Rx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (4, 4, 2, 1, 1; 4, 4); SBFD: 32 Tx/32 Rx antenna ports (Opt 2) (M, N, P, Mg, Ng; Mp, Np) = (4, 4, 2, 1, 1; 4, 2); (per panel group) dH = 0.5λ, dV = 0.5λ; 90 degree mechanical tilt (pointing to the floor) |
| UE antenna configuration | 2 Tx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 1) 4 Rx antenna ports: (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) dH=0.5 |
| UE & BS receiver | MMSE-IRC |
| UE power control | Open-loop power control with alpha = 0.6 and p0=-60 |
| DL/UL Transmission mode | DL: Single user MIMO with rank 2 UL: Single user MIMO with rank 1 |
| Frame structure | TDD: DDDSU with S=[12D:2G:0U] SBFD: XXXXX with X denoting a SBFD slot with DGUGU = [104, 5, 55, 5, 104] PRB assignment. “D”, “U” and “G” refers to downlink subband, uplink subband and guard bands, respectively. |
| SBFD interference modeling | gNB Self-interference RSI: 120 dB gNB-to-gNB inter-site: ACLR: 45 dB, ACS: 46 dB. UE-to-UE: IBE requirements defined in TS38.101-1 and TS38.101-2 for Tx model. UE selectivity model according to recent RAN1 working assumptions. UE ICS = 33 dB |
| Noise figure | BS: piece-wise noise figure model with A = -35 dBm, B = -17 dBm, C = 13 dB, D = 22 dB UE: 9 dB |

Table B3: Simulation assumptions for FR2-1 Indoor Scenario

| Parameters | Value |
| --- | --- |
| Scenario | TR 38.901 Indoor Office of 120x50x3 meter with 12 cells deployed in the ceiling with 20 meter inter-site distance. SBFD Deployment Case 1 with single operator and all gNBs using the same UL-DL SBFD sub-band partitioning |
| SCS | 120 kHz |
| Carrier bandwidth | 200 MHz, 132 RBs |
| gNB total transmit power | 24 dBm |
| UE Tx power | 23 dBm |
| UE position | 120 randomly distributed UEs in the office area. |
| Traffic model | FTP3 UL and DL traffic; 0.125 MB payload size in UL and 0.5 MB in DL FTP3 UL and DL traffic; 1kB payload size in UL and 4kB in DL |
| Channel modelling | gNB-UE, UE-UE and gNB-gNB: TR 38.901 InH Both large-scale and small-scale fading effects are modeled between all gNB-gNB and UE-UE links. |
| BS antenna configurations | TDD: (M, N, P, Mg, Ng; Mp, Np) = (16, 8, 2, 1, 1; 1, 1); SBFD (Opt 2): (M, N, P, Mg, Ng; Mp, Np) = (16, 8, 2, 1, 1; 1, 1); dH = 0.5λ, dV = 0.5λ; 90 degree mechanical tilt (pointing to the floor) |
| UE antenna configuration | 4Tx/4 Rx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (2, 4, 2, 1, 2; 1, 1) dH=0.5 |
| UE & BS receiver | MMSE-IRC |
| UE power control | Open-loop power control with alpha = 0.6 and p0=-60 |
| DL/UL Transmission mode | DL: Single user MIMO with rank 1 UL: Single user MIMO with rank 1 |
| Frame structure | TDD: DDDSU with S=[12D:2G:0U] SBFD: XXXXX with X denoting a SBFD slot with DGUGU = [52, 1, 26, 1, 52] PRB assignment. “D”, “U” and “G” refers to downlink subband, uplink subband and guard bands, respectively. |
| SBFD interference modeling | gNB Self-interference RSI: 120 dB gNB-to-gNB inter-site: ACLR: 28 dB, ACS: 23.5 dB. UE-to-UE: IBE requirements defined in TS38.101-1 and TS38.101-2 for Tx model. UE selectivity model according to recent RAN1 working assumptions. UE ICS = 23 dB |
| Noise figure | BS: piece-wise noise figure model with A = -58 dBm, B = -40 dBm, C = 10 dB, D = 19 dB UE: 10 dB |