3GPP TSG RAN WG1 #114			R1-2307489

Toulouse, France, August 21st – August 25th, 2023

Source:	NTT DOCOMO, INC.

Title:	Timing advance enhancement for inter-cell mobility

Agenda Item:	9.10.2

Document for: 	Discussion and Decision

- Introduction

In RAN#94-e meeting, a new Rel-18 WID of ‘Further NR mobility enhancements’ was approved and the WID was further updated in RAN#97-e meeting [1].

In this contribution, we mainly discuss the enhancements for timing advance for L1/L2 triggered inter-cell mobility (LTM).

- Initial TA acquisition of candidate cells

2.1 Enhancements of PDCCH ordered RACH

In previous RAN2 meeting, the following agreement was made. Thus, in this contribution, we discuss enhancements assuming PDCCH ordered RACH without RAR.

- RAN2 doesn’t see a need for a solution with RAR in for Rel-18.

In previous RAN1 meeting, enhancement for PDCCH ordered RACH of candidate cells in Rel-18 LTM was discussed. And RAN1 has made following agreements.

Agreement The PDCCH order from the source cell contains the indication of candidate cell. - The reserved bit(s) in DCI format 1_0 for PDCCH order can be used for indication of cell identity Agreement For PDCCH-order based PRACH for candidate cell study the following issues: - whether/how prioritizations for transmission power reduction for a PRACH transmission to a LTM candidate cell is performed - whether/how prioritizations for prioritization of a PARCH transmission to a LTM candidate cell compared to an overlapped (in time and frequency) serving cell UL transmission Agreement For PDCCH-order based PRACH for candidate cell, If UE capability does not support simultaneous/parallel transmissions, when the PRACH transmission to a candidate cell other than current serving cell(including any interruption due to processing time to build the PRACH transmission, carrier or/and BWP switching time if any, UL or DL RF retuning time if any, additional preparation time if any) happen to overlap over one or more symbols or have a time gap below a certain threshold (e.g., N symbols, FFS: the value of N) with following UL transmission to one of the serving cells - PRACH transmission - PUCCH/PUSCH transmission carrying HARQ-ACK, SR, P/SP CSI, aperiodic CSI - SRS transmission - Any other PUCCH/PUSCH transmission Down-select the UE behavior in this case - Alt 1: Dropping rule is needed - Alt 2: up to UE implementation

There are a few open issues related to PDCCH ordered RACH. The first FFS is how the indication of candidate cell is indicated by the reserved bit(s) in PDCCH order. To decide how to do it we should consider the following two perspectives:

- whether all RRC configured candidate cells in LTM could be triggered with PDCCH order

- how many candidate cells could be indicated in a PDCCH order

Firstly, if we consider the number in Rel-17 ICBM as baseline, there can be at least 7 candidate cells. And for Rel-18 LTM, larger than 7 candidate cells may be configured. Then, for all candidate cells, performing DL synchronization, L1 measurement/reporting, and UL synchronization would lead to UE complexity. And UL synchronization would be not necessary for candidate cells on which DL synchronization and/or L1 measurement/reporting is not performed. Thus, it would not be necessary that all RRC configured candidate cells in LTM could be triggered with PDCCH order RACH.

Observation 2.1

It would not be necessary that all RRC configured candidate cells in LTM could be triggered with PDCCH order RACH.

Based on above observation, UE needs to know candidate cells to which PDCCH order can be sent to trigger PRACH in a separate configuration from LTM candidate cell configuration. And the separate configuration of candidate cells for UL synchronization can be explicit or implicit. One possible implicit method is that the candidate cells configured for L1 beam measurement/reporting could be assumed as candidate cells for UL synchronization by UE. Thus, we propose

Proposal 2.1

Support explicit or implicit separate configuration of candidate cells for UL synchronization to which PDCCH order RACH can be triggered.

In addition to the limitation of candidate cells that require UL synchronization as described above, candidate cells for UL synchronization could be also selected by NW according to L1 measurement results. Considering multiple RACHs in parallel leads to great complexity, one candidate cell triggered by a PDCCH order would be considered sufficient.

Observation 2.2

It would be sufficient that one candidate cell is triggered by a PDCCH order.

If the number of candidate cells that require UL synchronization is limited, and one candidate cell is triggered by a PDCCH order, it would be better that the indication of candidate cell is indicated in PDCCH order by codepoint as shown in Figure1. And the bit size in DCI format 1_0 for cell indicator could be determined by the maximum number of configured candidate cells for UL synchronization. Thus, we propose

Proposal 2.2

The bit size of N in DCI format 1_0 for cell indicator is determined by the maximum number (e.g., C) of explicitly/implicitly configured candidate cells for UL synchronization, i.e., N = log2C.

Figure 1: Example of indication for cell indicator in PDCCH order.

The second FFS is how/whether prioritization for PRACH is performed. Firstly, we think prioritization for transmission power reduction for a PRACH transmission to a LTM candidate cell should be performed. The scenario focused on in R18 LTM is PCell switch, which could be a very essential mobility in terms of connection continuity. In addition, one of the objectives of R18 LTM is to shorten the interruption time. To achieve this, it would be very essential to obtain the TA before reception of the cell switch command. Therefore, we think that the PRACH transmission for the PDCCH ordered RACH to obtain the TA of a candidate cell that may become the target PCell should have the next highest priority after PRACH transmission to the PCell. Then, transmission power allocation of PRACH for a LTM candidate cell could have the next highest priority after PRACH transmission on the PCell. Secondary, for prioritization of a PRACH transmission to a LTM candidate cell compared to overlapped serving cell UL transmission if UE capability does not support simultaneous/parallel transmissions, we think dropping rule should be defined. If dropping rule is not determined, the NW side cannot assume that the UE will transmit and may not get the expected signal. And as for the specific dropping rules, the same as in the above analysis, PRACH transmission to LTM candidate cell should have the next highest priority after PRACH transmission on the PCell. Thus, we propose

Proposal 2.3

For PDCCH-order based PRACH for candidate cell,

- On prioritization for transmission power reductions, PRACH transmission to LTM candidate cell has the next highest priority after PRACH transmission on the PCell.

Proposal 2.4

For PDCCH-order based PRACH for candidate cell, if UE does not support simultaneous/parallel transmissions, in case of collision, support Alt1: Dropping rule is needed.

- UE drops PRACH transmission to LTM candidate cell when it overlaps with PRACH transmission on the PCell. Otherwise, UE transmits PRACH to LTM candidate cell and drops other UL channel/RS.

RAN1 has made following agreement for PDCCH ordered RACH without RAR.

Agreement For PDCCH ordered-RACH, if reception of RAR is not configured - Whether power ramping is performed or not is determined from PDCCH order - If power ramping is performed, - whether PRACH is an initial transmission or retransmission is explicitly indicated in PDCCH order (FFS exact indication mechanism) - power ramping-up value is configured - else, the power should be determined by open-loop power control Agreement On the determination of the PRACH transmission power when reception of RAR is not configured, a 1-bit field in PDCCH order explicitly indicating initial transmission or retransmission of PRACH, FFS - UE will increase the power with the value of power ramping configuration if it is indicated as re-transmission, unless the max allowed power is achieved - whether/how to reset the counter

There are two FFSs related to exact UE behaviour of power ramping for PRACH candidate cell. As there was some discussion at the last meeting, there are cases where power ramping should not be performed even when retransmission is indicated by the PDCCH order. We need to clarify these cases. First, there would be the following cases:

Case1: When the maximum allowed transmission power is achieved

Case2: When a PDCCH order is received that indicates a candidate cell index and/or SSB index different from the previously received PDCCH order

In these above cases, the UE may not need to increase the transmission power. In the previous meeting, the case when the PDCCH order for re-transmission is received without receiving a PDCCH order indicating initial transmission was also mentioned, but this case could be considered undesirable from the UE complexity point of view because the UE may need to remember two or more previous PDCCH order when power ramping is repeatedly indicated. In addition, the assumption of Case 2 could be sufficient to cover this case. Secondly, we think that there should be a specification as to when a counter should be reset since PRACH may be transmitted at a power that is not expected. To avoid ambiguity in transmit power, we can consider that two situations that require a reset would be when a PDCCH order is received that indicates the initial transmission and when Case 2 occurs. Based on analysis above, we propose

Proposal 2.5

For PDCCH ordered RACH without RAR, when UE receives the PDCCH order indicating re-transmission,

- UE increases the power with the value of power ramping configuration except when

  - the maximum allowed transmission power is achieved

  - a PDCCH order is received that indicates a candidate cell index and/or SSB index different from the previously received PDCCH order

Proposal 2.6

For PDCCH ordered RACH without RAR, UE resets the counter, when

- a PDCCH order is received that indicates the initial transmission

- a PDCCH order is received that indicates a candidate cell index and/or SSB index different from the previously received PDCCH order

2.2 UE-based TA measurement

In previous RAN1 meeting, UE-based TA measurement was supported as below.

Agreement Confirm the following Working Assumption, and sent LS to RAN4 to clarify the feasibility of supporting this mechanism Working Assumption From RAN 1 perspective, UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. - Corresponding UE capability is to be introduced to support UE-based TA measurement - For a UE reports support of this capability, configuration of UE-based TA measurement is supported - FFS: other impacts on RAN1 spec

So, we should further discuss details of it. Procedure of UE-based TA measurement in our assumption is shown in Figure 2. To make UE based TA measurement work, some enhancements are needed. Thus, we analyse it below. Firstly, UE needs to be notified by NW whether to perform UE based TA measurement or not. UE-based TA measurement was supported to reduce latency. However, several companies may be concerned about the accuracy of the derived TA when DL synchronization between serving cell and candidate cell has not been established. On the other hand, PDCCH ordered RACH has already been agreed upon to obtain TAs for candidate cells. This would be considered a means to obtain TAs with high accuracy. Given this situation, we think that UE-based TA measurement should not be set for all cells, but rather per cell. Moreover, for UE-based TA measurement, a reference cell needs to be set up for the UE to measure the DL difference timing. The reference cell to be set should be configured as part of the LTM candidate cell configuration, as it is simple for the UE to refer to the same cell after cell switch. Secondly, for UE based TA measurement, an offset value from the reference cell needs to be configured for the UE to calculate the DL difference timing when DL is unsynchronized between reference cell and a candidate cell. Since this offset value is a fixed value assuming that the reference cell is fixed, it is considered better to be configured as part of LTM candidate cell configuration as well.

Proposal 2.7

For UE based TA measurement, whether to perform UE based TA measurement or not is configured per cell.

Proposal 2.8

For UE based TA measurement, the reference cell and the offset values should be configured as part of LTM candidate cell configuration.

Figure 2: Assumed procedures of UE-based TA measurement.

Thirdly, for UE-based TA measurement, it needs to be considered when to trigger it. NW needs to know whether UE-based TA measurement is performed or not for candidate cell (i.e., NW needs to know whether UE has available TA for candidate cell.). Thus, UE-based TA measurement can be triggered by NW. For this purpose, two alternatives (Alt1: PDCCH order for serving cell, Alt2: Absolute TAC MAC CE for serving cell) can be considered as triggering signal by reusing ‘R’ field. We analyse these as shown in Table 1 below.

Table 1: Analysis of signal to trigger UE-based TA measurement

From analysis above, either signal can be supported considering small difference.

Proposal 2.9

Support either one of the following signals to trigger UE based TA measurement for candidate cell(s).

- PDCCH order for serving cell

- Absolute TAC MAC CE for serving cell

Fourthly, for UE-based TA measurement, it needs to be considered when to apply the TA of candidate cell. As for the TA derived by UE-based TA measurement, it is considered appropriate to be applied after cell switch command, since it is for a candidate cell. Also, after the cell switch, the candidate cell to which the TA is applied becomes a serving cell, and the subsequent TA derivation is considered to follow the legacy procedure, so it is better to apply it to the first UL transmission after cell switch command.

Proposal 2.10

For UE based TA measurement, TA of target cell is applied after cell switch command for first UL transmission.

Fifthly, we think UE-based TA measurement can be UE optional. And new UE capability can be introduced for it. For TA acquisition of candidate cells, PDCCH ordered RACH should be supported by default.

Proposal 2.11

For TA acquisition of candidate cells for Rel-18 LTM, PDCCH ordered RACH is supported by default.

- UE-based TA measurement is UE optional.

- Configuration for TA acquisition

In 110bis-e meeting, TA acquisition of the candidate cell(s) before cell switch command is received in L1/L2 based mobility was agreed, but whether this can be applied to the candidate cell when it is deactivated SCell was FFS. This is because some companies said that definition of deactivated SCell as candidate cells is ambiguous. On the other hand, in the RAN2#119bis meeting, the following agreements were made.

・A L1/L2 inter-cell mobility candidate (target) configuration is received within an RRC message before the L1/L2 dynamic switch is triggered. ・For L1L2 mobility, Target Pcell/SCell can be current SCell/PCell, i.e., current SCell/PCell can be configured as candidates.

From the agreements above, we can assume that current PCell/SCell can be configured as candidate cells by RRC signaling. In other words, these agreements clarify that deactivated SCell is supported as the candidate cell. So, we can support TA acquisition of candidate cell(s) before cell switch command is received in L1/L2 based mobility when it is deactivated SCell.

Proposal 3.1

Support TA acquisition of candidate cell(s) before cell switch command is received in L1/L2 based mobility when it is deactivated SCell.

RAN1 has made following agreement for association between TA/TAG and candidate cell.

Agreement For TA acquisition of a candidate cell before cell switch command is received, study at least the following alternatives of associating TA/TAG to candidate cell: - Alt1: Associate TA/TAG and candidate cell implicitly, e.g., - the association between TA/TAG and TCI states can be configured - Alt2: Associate TA/TAG and candidate cell explicitly, e.g., - the association is provided as a part of candidate cell(s) configuration - the association between TA/TAG and SSB(s)/TRS(s) is provided as a part of candidate cell(s) configuration

For the alternatives, some companies have concern on scenario, and this is not done down-select yet. We do not think this association is for multi-TRP. In our view, when NW knows that a candidate cell has same TA as other candidate cell, it would be beneficial to provide this information to UE. If UE has this information, when UE acquires a TA of a candidate cell, UE also applies this TA for other candidate cells. This function would be also useful for subsequent LTM, i.e., if after cell switch UE has TA for next target cell, TA acquisition is not necessary.  However, the key point is that if we follow legacy spec., the legacy TAG ID is configured for serving cell, so if different candidate cells have same TA, UE cannot know which candidate cells have same TA and cannot apply TA for other candidate cells. Thus, we could design a new TAG between candidate cells as shown in Figure3. Based on analysis above, we propose

Proposal 3.2

Support Alt2: Associate TA/TAG and candidate cell explicitly.

- New TAG between TAGs for each candidate cell is provided as part of configuration for candidate cell(s).

Figure 3: Example for new TAG for between candidate cells

- Conclusion

In this contribution, we discussed enhancements on timing advance management. Based on the discussion, we made following observations and proposals.

Observation 2.1

It would not be necessary that all RRC configured candidate cells in LTM could be triggered with PDCCH order RACH.

Proposal 2.1

Support explicit or implicit separate configuration of candidate cells for UL synchronization to which PDCCH order RACH can be triggered.

Observation 2.2

It would be sufficient that one candidate cell is triggered by a PDCCH order.

Proposal 2.2

The bit size of N in DCI format 1_0 for cell indicator is determined by the maximum number (e.g., C) of explicitly/implicitly configured candidate cells for UL synchronization, i.e., N = log2C.

Proposal 2.3

For PDCCH-order based PRACH for candidate cell,

- On prioritization for transmission power reductions, PRACH transmission to LTM candidate cell has the next highest priority after PRACH transmission on the PCell.

Proposal 2.4

For PDCCH-order based PRACH for candidate cell, if UE does not support simultaneous/parallel transmissions, in case of collision, support Alt1: Dropping rule is needed.

- UE drops PRACH transmission to LTM candidate cell when it overlaps with PRACH transmission on the PCell. Otherwise, UE transmits PRACH to LTM candidate cell and drops other UL channel/RS.

Proposal 2.5

For PDCCH ordered RACH without RAR, when UE receives the PDCCH order indicating re-transmission,

- UE increases the power with the value of power ramping configuration except when

  - the maximum allowed transmission power is achieved

  - a PDCCH order is received that indicates a candidate cell index and/or SSB index different from the previously received PDCCH order

Proposal 2.6

For PDCCH ordered RACH without RAR, UE resets the counter, when

- a PDCCH order is received that indicates the initial transmission

- a PDCCH order is received that indicates a candidate cell index and/or SSB index different from the previously received PDCCH order

Proposal 2.7

For UE based TA measurement, whether to perform UE based TA measurement or not is configured per cell.

Proposal 2.8

For UE based TA measurement, the reference cell and the offset values should be configured as part of LTM candidate cell configuration.

Proposal 2.9

Support either one of the following signals to trigger UE based TA measurement for candidate cell(s).

- PDCCH order for serving cell

- Absolute TAC MAC CE for serving cell

Proposal 2.10

For UE based TA measurement, TA of target cell is applied after cell switch command for first UL transmission.

Proposal 2.11

For TA acquisition of candidate cells for Rel-18 LTM, PDCCH ordered RACH is supported by default.

- UE-based TA measurement is UE optional.

Proposal 3.1

Support TA acquisition of candidate cell(s) before cell switch command is received in L1/L2 based mobility when it is deactivated SCell.

Proposal 3.2

Support Alt2: Associate TA/TAG and candidate cell explicitly.

- New TAG between TAGs for each candidate cell is provided as part of configuration for candidate cell(s).

Reference

[1] 3GPP RP-222332, “Revised WID on Further NR mobility enhancements”, RAN#97e, Sep. 2022.