3GPP TSG RAN WG1 #114	        		             R1-2307698

Toulouse, France, August 21st – 25th, 2023

Agenda item:	9.10.2

Source: 	Samsung

Title: 	Candidate cell TA acquisition for NR L1/L2 mobility enhancement

Document for:	Discussion and Decision

Background

In RAN1 #113, Multiple agreements made as following and it seems we are ready to complete the feature.

Agreement Confirm the following Working Assumption, and sent LS to RAN4 to clarify the feasibility of supporting this mechanism Working Assumption From RAN 1 perspective, UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. - Corresponding UE capability is to be introduced to support UE-based TA measurement - For a UE reports support of this capability, configuration of UE-based TA measurement is supported - FFS: other impacts on RAN1 spec Agreement From RAN 1 perspective, without performing PDCCH-ordered RACH for candidate cell(s), RACH-less mechanism can be supported by indicating TA value of target cell as TA=0 or keeping the same value as source cell in cell switch command. - Note 1: this doesn’t mean to preclude TA values other than 0 and the same value as source cell in cell switch command for PDCCH-ordered RACH when RAR is not configured for the PDCCH order. - Note 2: The feasibility and signalling can be further concluded by RAN2 Agreement For PDCCH order based PRACH to candidate cell, the candidate cell SSB indicated in the PDCCH order serves as the path loss RS for PRACH Tx power determination. Agreement On the determination of the PRACH transmission power when reception of RAR is not configured, a [1-bit] field in PDCCH order explicitly indicating initial transmission or retransmission of PRACH is supported. Note: From RAN1 perspective, when reception of RAR is configured, there may have 4 alternatives to determine the randomaccess response window - Alt1: Postpone the starting point of the random access response window - Alt2: Extend the length of the random access response window - Alt3: Length and offset of the starting point of RAR window can be configured by RRC - Alt4: if MAC CE is used to carry TA and PDCCH is scrambled by C-RNTI in USS, RAR window is not needed [Note: the random access response window for candidate cell(s) is separately configured from the normal RAR window.] Note: In addition to TA, when reception of RAR is configured, RAN1 discussed the following alternatives: - Alt 1: when there is only one ongoing RACH procedure at each time, the identification of candidate cell is not needed - Alt 2: when more than one RACH procedures are allowed at each time, the identification of candidate cell is contained in RAR Agreement On the determination of the PRACH transmission power when reception of RAR is not configured, a 1-bit field in PDCCH order explicitly indicating initial transmission or retransmission of PRACH, FFS - UE will increase the power with the value of power ramping configuration if it is indicated as re-transmission, unless the max allowed power is achieved - whether/how to reset the counter Agreement - For PDCCH-order based PRACH for candidate cell, If UE capability does not support simultaneous/parallel transmissions, when the PRACH transmission to a candidate cell other than current serving cell(including any interruption due to processing time to build the PRACH transmission, carrier or/and BWP switching time if any, UL or DL RF retuning time if any, additional preparation time if any) happen to overlap over one or more symbols or have a time gap below a certain threshold (e.g., N symbols, FFS: the value of N) with following UL transmission to one of the serving cells - PRACH transmission - PUCCH/PUSCH transmission carrying HARQ-ACK, SR, P/SP CSI, aperiodic CSI - SRS transmission - Any other PUCCH/PUSCH transmission - Down-select the UE behavior in this case - Alt 1: Dropping rule is needed - Alt 2: up to UE implementation

Though outlines of feature confirmed as shown above, several details stills needs to be clarified in this meeting. In this contribution, we suggest how to complete the remaining details without causing additional implementation complexity or unnecessary restriction.

Discussions

Remaining details on PRACH transmission

Serving cell triggered PRACH toward candidate cell should be the key function supporting multi-TA LTM. Via PDCCH, serving cell triggers PRACH toward candidate cell for TA acquisition, with detailed info such as associated SSB, retransmission indication, etc. As remained detail, RAN1 discuss whether the failure or dropping of candidate cell PRACH should be always recognized by serving cell.

Agreement - For PDCCH-order based PRACH for candidate cell, If UE capability does not support simultaneous/parallel transmissions, when the PRACH transmission to a candidate cell other than current serving cell(including any interruption due to processing time to build the PRACH transmission, carrier or/and BWP switching time if any, UL or DL RF retuning time if any, additional preparation time if any) happen to overlap over one or more symbols or have a time gap below a certain threshold (e.g., N symbols, FFS: the value of N) with following UL transmission to one of the serving cells - PRACH transmission - PUCCH/PUSCH transmission carrying HARQ-ACK, SR, P/SP CSI, aperiodic CSI - SRS transmission - Any other PUCCH/PUSCH transmission - Down-select the UE behavior in this case - Alt 1: Dropping rule is needed - Alt 2: up to UE implementation

As an example, when serving cell PUSCH and candidate cell PRACH overlapped and both are not received, it means one is dropped and the other one failed. If PRACH dropped, PRACH transmission to be triggered without increasing transmission power, i.e. PRACH triggering DCI would not indicate PRACH retransmission. But if serving cell PUSCH is dropped, serving cell may trigger candidate cell PRACH with indication retransmission to increase PRACH transmission power. Therefore, considering the optimization of performance, it may beneficial to define dropping rule as Alt 1 above.

But it should be noted that PRACH is always triggered by serving cell, and significant problems with UL overlapping can be avoided by serving cell’s scheduling. Therefore, we propose to accept Alt 2 above.

Observation 1: Dropping rule may assist serving cell for better understanding which UL transmission failed and which channel requests adjustment.

Observation 2: No functional failure to be observed without defining dropping rules between serving cell UL transmission and candidate cell PRACH transmission when those are overlapped.

Proposal 1: Do not define specification based dropping rule between overlapped serving cell UL transmission and candidate cell PRACH transmission.

Remaining details on RAR

It was agreed that RAR for candidate cell TA to be received via serving cell to avoid additional UE complexity at receiving RAR. As remained detail, RAN1 started discussion how to schedule RAR containing PDSCH for candidate cell. As a first issue, RAN1 needs to decide whether UE to be configured with additional RAR window or candidate cell PDSCH scheduled by PDCCH in UE specific search space:

Agreement On the determination of the PRACH transmission power when reception of RAR is not configured, a [1-bit] field in PDCCH order explicitly indicating initial transmission or retransmission of PRACH is supported ordered. Note: From RAN1 perspective, when reception of RAR is configured, there may have 4 alternatives to determine the randomaccess response window - Alt1: Postpone the starting point of the random access response window - Alt2: Extend the length of the random access response window - Alt3: Length and offset of the starting point of RAR window can be configured by RRC - Alt4: if MAC CE is used to carry TA and PDCCH is scrambled by C-RNTI in USS, RAR window is not needed [Note: the random access response window for candidate cell(s) is separately configured from the normal RAR window.]

In legacy RACH process, PDCCH scrambled by RA-RNTI supports both of idle and RRC connected UE for the initial access toward a cell UE specific scrambling is not configured. In case of candidate cell TA acquisition, since RAR is received via serving cell which already provided UE specific scrambling sequence, no critical reason exist to schedule RAR PDSCH via cell specific search space.

At UE’s perspective, additional RAR window possibly per candidate cell should bring additional complexity which unnecessary to support candidate cell TA acquisition, also at gNB’s perspective, candidate cell RAR window may bring unnecessary restriction at scheduling candidate cell RAR. Therefore, we propose not to define candidate cell RAR window but relying on C-RNTI to schedule PDSCH containing candidate cell RAR.

Observation 3: Candidate cell RAR can be scheduled by PDCCH scrambled with UE specific sequence.

Observation 4: Candidate cell RAR brings additional UE complexity and serving cell’s scheduling restriction.

Proposal 2: Serving cell schedules PDSCH containing candidate cell RAR via PDCCH scrambled by C-RNTI (Alt 4 above).

- RAN1 does not define candidate cell specific RAR window

As other details on candidate cell RAR, RAN1 started discussion whether multiple of candidate cell TA simultaneously supported:

Note: In addition to TA, when reception of RAR is configured, RAN1 discussed the following alternatives: - Alt 1: when there is only one ongoing RACH procedure at each time, the identification of candidate cell is not needed - Alt 2: when more than one RACH procedures are allowed at each time, the identification of candidate cell is contained in RAR - As aligned with the discussion above, if RAR window is not defined, may no UE complexity issue at supporting multiple PRACH process.

If RAR window is not defined, in RAN1’s perspective, support of multiple simultaneous candidate cell PRACH would not bring any further UE complexity. Therefore, RAN1 may not need to discuss and decide whether only single or multiple of candidate cell PRACH process simultaneously supported.

Observation 5: RAN1 may not have any issue at supporting multiple of simultaneous candidate cell PRACH, if PDSCH containing candidate cell RAR is scheduled by PDCCH scrambled with C-RNTI.

Therefore, down selection between two alternative above should be decided by upper layer considering various RAN2 or RAN3 aspects.

Proposal 3: It is up to high layer discussion whether simultaneous multiple candidate cell PRACH to be supported

- Detailed design of candidate cell RAR is also up to high layer decision

As remained small details, RAN1 discuss whether/how to reset the counter of candidate cell PRACH:

Agreement On the determination of the PRACH transmission power when reception of RAR is not configured, a 1-bit field in PDCCH order explicitly indicating initial transmission or retransmission of PRACH, FFS - UE will increase the power with the value of power ramping configuration if it is indicated as re-transmission, unless the max allowed power is achieved - whether/how to reset the counter

UE ramps up transmission power of PRACH when trigged with retransmission indicator. Thus, UE should buff PRACH transmission information till the process is completed. Since candidate cell RAR is configurable, UE should be configured with candidate cell PRACH timer to understand the completion of candidate cell PRACH process. As an example, when candidate cell RAR is not configured, UE may assume candidate cell PRACH completed unless retransmission request is received before the timer expired. With the completion of candidate cell PRACH transmission, retransmission counter reset.

Observation 6: When candidate cell RAR is not configured, timer is required to confirm the completion of candidate cell PRACH process

Proposal 4: UE reset candidate cell PRACH retransmission counter when RAR is received or timer expired

Other issues

A small discussion on UE capability was open in the previous RAN1 meeting:

[FL note] For TA acquisition of candidate cell(s) when reception of RAR is configured, whether the number of candidate cells can exceed the maximum number of TAs that can be maintained by UE capability

For the simplified UE operation, we propose not to configured more of candidate cell than UE supporting for TA acquisition.

Proposal 5: UE is not configured with candidate cells more than supported for separated TA acquisition

When multiple of simultaneous process is supported for candidate cell PRACH, UE should understand which of indicated TA/TAG is associated to which candidate cell. As discussed above, it should be up to upper layer discussion whether multiple of simultaneous PRACH process is supported for candidate cell TA. But considering the time plan of Rel-18 discussion, RAN1 may better to support the association between indicated TA/TAG and candidate cell.

[FL note] Association between TA and candidate cell - Alt2: Associate TA/TAG and candidate cell explicitly - Alt3: Do not support associating the TA with a candidate cell

Proposal 6: Support explicit association between TA/TAG and candidate cell explicitly.

Conclusions

In this contribution, we discuss remained issues at acquiring and managing candidate cell TA value before the handover decided. We provide following observations and suggest following proposals.

Observation 1: Dropping rule may assist serving cell for better understanding which UL transmission failed and which channel requests adjustment.

Observation 2: No functional failure to be observed without defining dropping rules between serving cell UL transmission and candidate cell PRACH transmission when those are overlapped.

Proposal 1: Do not define specification based dropping rule between overlapped serving cell UL transmission and candidate cell PRACH transmission.

Observation 3: Candidate cell RAR can be scheduled by PDCCH scrambled with UE specific sequence.

Observation 4: Candidate cell RAR brings additional UE complexity and serving cell’s scheduling restriction.

Proposal 2: Serving cell schedules PDSCH containing candidate cell RAR via PDCCH scrambled by C-RNTI (Alt 4 above).

- RAN1 does not define candidate cell specific RAR window

Observation 5: RAN1 may not have any issue at supporting multiple of simultaneous candidate cell PRACH, if PDSCH containing candidate cell RAR is scheduled by PDCCH scrambled with C-RNTI.

Proposal 3: It is up to high layer discussion whether simultaneous multiple candidate cell PRACH to be supported

- Detailed design of candidate cell RAR is also up to high layer decision

Observation 6: When candidate cell RAR is not configured, timer is required to confirm the completion of candidate cell PRACH process

Proposal 4: UE reset candidate cell PRACH retransmission counter when RAR is received or timer expired

Proposal 5: UE is not configured with candidate cells more than supported for separated TA acquisition

Proposal 6: Support explicit association between TA/TAG and candidate cell explicitly.

- References

[1] RP-222332 “Revised WID on Further NR mobility enhancements”, 3GPP RAN#97e, e-Meeting, September, 2022.

[2] R1-2208331 (R2-2209257) “LS on L1/L2-based inter-cell mobility”, 3GPP RAN2#119, e-Meeting, October, 2022

[3] R1-2209755, “Non-serving cell TA management for NR L1/L2 mobility enhancement”, 3GPP RAN1 #110bis-e, October, 2022

[4] R1-2212069, “Candidate cell TA acquisition for NR L1/L2 mobility enhancement”, 3GPP RAN1 #111, November, 2022

[5] R1-2305534, “Candidate cell TA acquisition for NR L1/L2 mobility enhancement”, 3GPP RAN1 #113, May, 2023

[6] Chair note, 3GPP RAN1 #112, March 2023

[7] Chair note, 3GPP RAN1 #113, May 2023