3GPP TSG RAN WG1 Meeting #114	R1-2308058

Toulouse, France, August 21st – August 25th, 2023

Agenda Item:	9.9.3

Source:	MediaTek Inc.

Title:	Disabling of HARQ for IoT NTN

Document for:	Discussion and Decision

- 1 Introduction

In RAN#98e [1], the revised WID on IoT NTN enhancements was endorsed for Release 18. Rel-17 IoT-NTN is considered as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objective for disabling of HARQ feedback.

According to the WID, disabling of HARQ feedback for IoT NTN has been agreed in RAN meetings.

This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below: - Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1, RAN2] - Study and specify needed improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1, RAN2] NOTE: The need for RAN4 Core requirements for this objective will be identified after the conclusion on the need for improvements.

This contribution aims to discuss aspects related to disabling of HARQ feedback for IoT NTN.

- 2 Discussion on disabling of HARQ feedback

- Configure enabling/disabling on HARQ feedback in IoT NTN

Disabling of HARQ feedback was discussed in last few meetings and agreements are made in RAN1 and RAN2 for options of disabling HARQ feedback in Rel-18 IoT NTN.

Agreement (RAN1 112) Confirm the following working assumption with the following update: Working assumption For NB-IoT NTN and eMTC NTN for CE Mode B, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission: - Support Option 1 in case only per-HARQ process bitmap signaling is configured - Support Option 3 DCI direct indication of HARQ feedback enable/disable in case only DCI solution enabling/disabling signaling is configured - Support Option 3 DCI indication to override Option 1 configuration for corresponding transmission in case both per-HARQ process bitmap and DCI solution enabling/disabling signaling are configured - FFS #1: Option 3 DCI-based overridden mechanism is applied to both semi-statically HARQ feedback enabled and disabled processes or only applied to semi-statically HARQ feedback disabled processes or only applied to semi-statically HARQ feedback enabled processes. - FFS #2: whether/how to support Option 3 overriding Option 1 configuration for corresponding transmission for multiple TBs scheduled by single DCI - FFS#3：Option 3 DCI-based overridden mechanism is DCI signaling to reverse the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration or DCI signaling to directly indicate the HARQ feedback enable/disable for the corresponding transmission regardless of per-HARQ process RRC configuration. RAN1 strives to have a common design (in terms of DCI design, PDCCH monitoring, etc.) for “Option 3” and “Option 3 + Option 1”. For eMTC NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, take Option 1 for CE Mode A. Working assumption(RAN1 113) For DCI-based direct indication in single TB scheduled by DCI, - Indication by reusing/reinterpreting HARQ-ACK related field in DCI - For eMTC CE mode B, one state of “HARQ-ACK resource offset” field in DCI format 6-1B is used for indication of HARQ feedback disabled, other states are used for indication of HARQ feedback enabled and corresponding HARQ-ACK resource. - FFS: detailed state, and whether this state is different across different UEs - For NBIoT, one state of “HARQ-ACK resource” field in DCI format N1 is used for indication of HARQ feedback disabled, other states are used for indication of HARQ feedback enabled and corresponding HARQ-ACK resource. - FFS: detailed state, and whether this state is different across different UEs - If reusing/reinterpreting HARQ-ACK related field in DCI is also used for DCI overriding scheme, the interpretation of the state can be different than for DCI-based direct indication. Agreement (RAN1 113) For single TB scheduled by DCI, Working assumption 1 DCI based overridden indication is applied to both semi-statically HARQ disabled and enabled processes - For DCI based overridden indication, adopt indication by reusing/reinterpreting HARQ-ACK related field in DCI - For eMTC CE mode B, “HARQ-ACK resource offset” field in DCI format 6-1B is used for indication of maintaining/reversing the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration and corresponding HARQ-ACK resource in case of indication of HARQ feedback enabled. - HARQ feedback disabled is reversed to enabled in case of any states other than state A in “HARQ-ACK resource offset”, otherwise is maintained as disabled. - HARQ feedback enabled is maintained in case of any states other than state A in “HARQ-ACK resource offset”, otherwise is reversed to disabled. - FFS: detailed state A, and whether this state A is different across different UEs - For NBIoT, “HARQ-ACK resource” field in DCI format N1 is used for indication of maintaining/reversing the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration and corresponding HARQ-ACK resource in case of indication of HARQ feedback enabled. - The same DCI indication functionality as eMTC is adopted.

In RAN1 112, RAN1 made agreement on how to configure/indicate enabling/disabling of HARQ feedback for downlink transmission for NB-IoT NTN, two new indication alternatives were involved, RRC per-HARQ process bitmap and DCI solution.

As illustrated in Figure 1, there are four cases on how to configure/indicate enabling/disabling of HARQ feedback for downlink transmission for NB-IoT NTN with the two new indication alternatives:

- when both RRC per-HARQ process bitmap and DCI solution are configured, support DCI indication to override RRC bitmap configuration

- when RRC per-HARQ process bitmap solution is configured and DCI solution is not configured, support RRC bitmap configuration

- when RRC per-HARQ process bitmap solution is not configured and DCI solution is configured, support DCI solution

- when both RRC per-HARQ process bitmap solution and DCI solution are not configured, the legacy procedure should be supported where all HARQ processes with feedback enabled

Figure 1: enabling/disabling of HARQ feedback for downlink transmission for NB-IoT NTN

Observation 1: Both RRC per-HARQ process bitmap solution and DCI solution are new indication alternatives.

Observation 2: When both RRC per-HARQ process bitmap solution and DCI solution are not configured, the legacy procedure where all HARQ processes with feedback enabled should be supported.

For disabling of HARQ feedback in NB-IoT NTN with DCI, it has been agreed that “HARQ-ACK resource” field in DCI format N1 is used for DCI directly indication and DCI overridden indication as summarized in following table 1(a) and 1(b). Based on the Table 1, if DCI solution for disabling of HARQ feedback is configured, no matter whether there is RRC bitmap or not, for the HARQ process scheduled by the DCI format N1:

- the HARQ feedback is disabled if “HARQ-ACK resource” field in DCI is state A;

- the HARQ feedback is enabled if “HARQ-ACK resource” field in DCI are other states.

Hence, the same state A should be utilized in both DCI directly indication and DCI overridden indication. To simplify, state A of “HARQ-ACK resource” field can be ‘0000’ in NB-IoT without additional network configuration.

Table 1: disabling of HARQ feedback in IoT NTN with “HARQ-ACK resource” field in DCI format N1

(a) DCI based overridden indication

| “HARQ-ACK resource” field in DCI | RRC configuration |  |
| --- | --- | --- |
|  | HARQ feedback enabled | HARQ feedback disabled |
| State A | Reverse to HARQ feedback disabled | Maintain HARQ feedback disabled |
| Other states | Maintain HARQ feedback enabled and indicate corresponding HARQ resource | Reverse to HARQ feedback enabled and indicate corresponding HARQ resource |

(b) DCI directly indication

| “HARQ-ACK resource” field in DCI | HARQ feedback enabled/disabled |
| --- | --- |
| State A | HARQ feedback disabled |
| Other states | HARQ feedback enabled and indicate corresponding HARQ resource |

Observation 3: If DCI-based indication for HARQ feedback is configured, no matter whether there is RRC bitmap or not, for the HARQ process scheduled by the DCI format N1:

- the HARQ feedback is disabled if “HARQ-ACK resource” field in DCI is state A;

- the HARQ feedback is enabled if “HARQ-ACK resource” field in DCI are other states.

Proposal 1: If DCI-based indication in NB-IoT is configured, state A of “HARQ-ACK resource” field is ‘0000’ in NB-IoT without additional network configuration.

- NPDCCH monitoring restriction for NB-IoT NTN with HARQ feedback enabled

NPDCCH monitoring restriction for NB-IoT NTN with HARQ feedback enabled was discussed in last meeting and agreement is made in RAN1.

Agreement (RAN1 113) For single TB scheduled by DCI, Working assumption 2 For Option 1 + Option 3 DCI based overridden mechanism, for a HARQ process configured as HARQ feedback disabled by per-HARQ process bitmap signaling and further reversed to HARQ feedback enabled by DCI, the NBIoT UE does not wait for an RTT+3ms (i.e., till subframe n+Kmac+3 in TS36.213 section 16.6) before monitoring NPDCCH for the same HARQ process (or monitoring any NPDCCH for the case of single HARQ process configuration). - Send an LS to RAN2 with the following contents: - RAN1 respectfully ask RAN2 for the feasibility of Working assumption 2 (taking into account potential RAN2 spec impact). Agreement (RAN1 113) For a NB-IoT UE operating with two HARQ processes, for an UL HARQ process with HARQ mode B, the minimum time between the end of NPUSCH transmission and the start of NPDCCH monitoring for the same HARQ process is 1 ms. - Note: this implies a RAN1 specification change in Rel-18 Agreement (RAN1 113) For a NB-IoT UE operating with one HARQ process, for an UL HARQ process with HARQ mode B, the minimum time between the end of NPUSCH transmission and the start of NPDCCH monitoring is 1 ms. - Note: this implies a RAN1 specification change in Rel-18

In RAN1 113, to distinguish DCI directly HARQ feedback indication and DCI overridden HARQ feedback indication, RAN1 has agreed the working assumption that  For Option 1 + Option 3 DCI based overridden mechanism, for a HARQ process configured as HARQ feedback disabled by per-HARQ process bitmap signalling and further reversed to HARQ feedback enabled by DCI, the NBIoT UE does not wait for an RTT+3ms (i.e., till subframe n+Kmac+3 in TS36.213 section 16.6) before monitoring NPDCCH for the same HARQ process (or monitoring any NPDCCH for the case of single HARQ process configuration), which means for DCI overridden procedure,  blind retransmission is allowed. For the NPDCCH monitoring restriction in this blind retransmission mechanism, the minimum monitoring restriction can be considered as 1ms, similar to the minimum time between the end of NPUSCH transmission and the start of NPDCCH monitoring for an UL HARQ process with HARQ mode B.

Proposal 2: If the NB-IoT UE has a NPUSCH transmission ending in subframe n scheduled by NPDCCH with DCI format N1, where the HARQ process is configured as HARQ feedback disabled by per-HARQ process bitmap signaling and further reversed to HARQ feedback enabled by DCI, the UE is not required to receive transmissions in the Type B half-duplex guard periods (1ms) for FDD.

From the no NPDCCH monitoring restriction for HARQ mode B and HARQ feedback enabled configured as HARQ feedback disabled by per-HARQ process bitmap signalling and further reversed to HARQ feedback enabled by DCI, 1ms should be captured in TS 36.213 at Clause 16.6. So, we have the following proposal:

Proposal 3: Capture TP#1 in clause 16.6 in TS 36.213.

========================= Start of TP #1 for TS 36.213 ========================= ## 16.6 Narrowband physical downlink control channel related procedures Throughout this clause, if a NB-IoT UE is configured with higher layer parameter k-Mac, Kmac = k-Mac otherwise, Kmac = 0. A UE shall monitor a set of NPDCCH candidates (described in Clause 10.2.5.1 of [3]) as configured by higher layer signalling for control information, where monitoring implies attempting to decode each of the NPDCCHs in the set according to all the monitored DCI formats. The set of NPDCCH candidates to monitor are defined in terms of NPDCCH search spaces. <Unchanged parts are omitted> If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig - and if the UE has a NPUSCH transmission ending in subframe n, - the UE is not required to receive transmissions in the Type B half-duplex guard periods as specified in [3]for FDD ; and - in a non NTN serving cell, the UE is not expected to receive an NPDCCH with DCI format N0/N1 for the same HARQ process ID as the NPUSCH transmission in any subframe starting from subframe n+1 to subframe n+3 or - in a NTN serving cell, the UE is not expected to receive an NPDCCH with DCI format N0/N1 for the same HARQ process ID as the NPUSCH transmission in any downlink subframe that overlaps with uplink subframe n+1 to subframe n+Kmac+3, - if the corresponding NPUSCH transmission is scheduled by NPDCCH with DCI format N0 and not configured with higher layer parameter uplinkHARQ-mode set to ‘HARQModeB’; or - if the corresponding NPUSCH transmission is scheduled by NPDCCH with DCI format N1 and if the NPUSCH transmission carries ACK/NACK response, as determined in clause 16.4.2 without configured with higher layer parameter downlinkHARQ-FeedbackDisabled-NB indicating disabled HARQ-ACK information for the NPUSCH transmission; else if the UE is not using higher layer parameter edt-Parameters or if the UE is using higher layer parameter edt-Parameters and - if the NB-IoT UE has a NPUSCH transmission ending in subframe n, - the UE is not required to receive transmissions in the Type B half-duplex guard periods as specified in [3]for FDD ; and - in a non NTN serving cell, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+3 or - in a NTN serving cell, the UE is not required to monitor NPDCCH in any downlink subframe that overlaps with uplink subframe n+1 to subframe n+Kmac+3 - if the corresponding NPUSCH transmission is scheduled by NPDCCH with DCI format N0 and not configured with higher layer parameter uplinkHARQ-mode set to ‘HARQModeB’; or - if the corresponding NPUSCH transmission is scheduled by NPDCCH with DCI format N1 and if the NPUSCH transmission carries ACK/NACK response, as determined in clause 16.4.2 without configured with higher layer parameter downlinkHARQ-FeedbackDisabled-NB indicating disabled HARQ-ACK information for the NPUSCH transmission; otherwise, - If the NB-IoT UE has a NPUSCH transmission for Msg3 ending in subframe n'with transport block size , whereas if would have been selected the NPUSCH transmission would have ended in subframe n, the UE is not required to monitor NPDCCH in any subframe starting from subframe n'+1 to subframe n+3 or in a NTN serving cell, in any downlink subframe that overlaps with uplink subframe n'+1 to subframe n+Kmac+3. If a NB-IoT UE receives a NPDSCH transmission ending in subframe n, and if the UE is not required to transmit a corresponding NPUSCH format 2, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+12. If a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig - the UE is not required to monitor an NPDCCH candidate of an NPDCCH search space if the candidate ends in subframe n, and if the UE is configured to monitor NPDCCH candidates of another NPDCCH search space having starting subframe k0 before subframe n+5 otherwise - the UE is not required to monitor NPDCCH candidates of an NPDCCH search space if an NPDCCH candidate of the NPDCCH search space ends in subframe n, and if the UE is configured to monitor NPDCCH candidates of another NPDCCH search space having starting subframe k0 before subframe n+5. <Unchanged parts are omitted> ========================= End of TP #1 for TS 36.213 =========================

2.3 HARQ feedback for scheduling multiple TB

Agreement (RAN1 113) for NB-IoT and LTE-MTC in CE Mode B, if multiple TBs is configured, for DCI-based HARQ enabling/disabling direct indication in multiple TBs scheduled by single DCI, the same indication is applied to all scheduled TBs, i.e. HARQ is enabled or disabled for all TBs.

Multiple TB is one optional feature considering unicast and multicast for Rel-16 NB-IoT. There is no multi-TB HARQ feedback in SC-MTCH. There is multi-TB HARQ feedback in unicast, with consideration of HARQ ACK bundling.

If higher layer parameter npdsch-MultiTB-Config is enabled and the corresponding DCI is mapped onto the UE specific search space given by the C-RNTI, the Number of scheduled TB for Unicast is 1 bit, where value 0 indicates a single TB is scheduled and value 1 indicates multiple TB are scheduled.

For NB-IoT, if Number of scheduled TB for Unicast is present and multiple TB are scheduled, the multi-TB (two TBs) are scheduled together without “HARQ process number” field in DCI. However, as specified in TS 36.213 of Section 16.4.1.5, If the UE is configured with higher layer parameter npdsch-MultiTB-Config and multiple TB are scheduled in the corresponding DCI, the HARQ process ID of 0 is for the first TB and HARQ process ID of 1 shall be assumed for the second TB, otherwise, HARQ process ID of 0 shall be assumed.

TS 36.213 Section 16.4.1.5 If the UE is configured with higher layer parameter npdsch-MultiTB-Config and multiple TB are scheduled in the corresponding DCI, the HARQ process ID of 0 is for the first TB and HARQ process ID of 1 shall be assumed for the second TB, otherwise, HARQ process ID of 0 shall be assumed.

There are two ways to facilitate enabling or disabling HARQ feedback of multi-TB based on RRC per-HARQ process bitmap solution:

- HARQ feedback of multi-TB are both enabled or both disabled.

- enabling or disabling HARQ feedback of multi-TB respectively on the HARQ process ID of 0 for the first TB and HARQ process ID of 1 for the second TB.

Observation 4: There are two ways to facilitate enabling or disabling HARQ feedback of multi-TB based on RRC per-HARQ process bitmap solution:

- HARQ feedback of multi-TB are both enabled or both disabled.

- enabling or disabling HARQ feedback of multi-TB respectively on the HARQ process ID of 0 for the first TB and HARQ process ID of 1 for the second TB.

It has been agreed in RAN1 113, that for NB-IoT and LTE-MTC in CE Mode B, if multiple TBs is configured, for DCI-based HARQ enabling/disabling direct indication in multiple TBs scheduled by single DCI, the same indication is applied to all scheduled TBs, i.e. HARQ is enabled or disabled for all TBs. If way (2) is utilized for multi-TB case for RRC per-HARQ process bitmap solution, there will involve more discussion at least on the following cases:

- when one TB is enabled and the other TB is disabled, and how DCI overridden solution can be done for different TBs.

- when one TB is enabled and the other TB is disabled, and how No NPDCCH monitoring restriction can be set.

- when one TB is enabled and the other TB is disabled, how TBS calculation for NPUSCH can be impacted.

Consider limited TU for Rel-18 and to simplify in NB-IoT, way (1) can be a better way for progress.

Proposal 4: For RRC per-HARQ process bitmap solution, HARQ feedback for multiple TBs are both enabled or both disabled at least in NB-IoT.

Proposal 5: For RRC per-HARQ process bitmap solution, HARQ feedback enabling/disabling for multiple TBs case is only based on the configuration of HARQ process ID of 0 for all TBs.

3 Conclusion

In this contribution, the following proposals were made

Observation 1: Both RRC per-HARQ process bitmap solution and DCI solution are new indication alternatives.

Observation 2: When both RRC per-HARQ process bitmap solution and DCI solution are not configured, the legacy procedure where all HARQ processes with feedback enabled should be supported.

Observation 3: If DCI-based indication for HARQ feedback is configured, no matter whether there is RRC bitmap or not, for the HARQ process scheduled by the DCI format N1:

- the HARQ feedback is disabled if “HARQ-ACK resource” field in DCI is state A;

- the HARQ feedback is enabled if “HARQ-ACK resource” field in DCI are other states.

Proposal 1: If DCI-based indication in NB-IoT is configured, state A of “HARQ-ACK resource” field is ‘0000’ in NB-IoT without additional network configuration.

Proposal 2: If the NB-IoT UE has a NPUSCH transmission ending in subframe n scheduled by NPDCCH with DCI format N1, where the HARQ process is configured as HARQ feedback disabled by per-HARQ process bitmap signaling and further reversed to HARQ feedback enabled by DCI, the UE is not required to receive transmissions in the Type B half-duplex guard periods (1ms) for FDD.

Proposal 3: Capture TP#1 in clause 16.6 in TS 36.213.

Observation 4: There are two ways to facilitate enabling or disabling HARQ feedback of multi-TB based on RRC per-HARQ process bitmap solution:

- HARQ feedback of multi-TB are both enabled or both disabled.

- enabling or disabling HARQ feedback of multi-TB respectively on the HARQ process ID of 0 for the first TB and HARQ process ID of 1 for the second TB.

Proposal 4: For RRC per-HARQ process bitmap solution, HARQ feedback for multiple TBs are both enabled or both disabled at least in NB-IoT.

Proposal 5: For RRC per-HARQ process bitmap solution, HARQ feedback enabling/disabling for multiple TBs case is only based on the configuration of HARQ process ID of 0 for all TBs.

4 References

- RP-223519, Moderator (MediaTek), Revised WID on IoT NTN enhancements, 12-16 December, 2022

- TR 36.763, Release 17

- R1-2303999, Lenovo, FLS#2 on disabling of HARQ feedback for IoT NTN

- R1-1912125, MediaTek, Delay-tolerant re-transmission mechanisms in NR-NTN, RAN1#99, November 2019

- R2-2304274, LS on HARQ Enhancements