3GPP TSG RAN WG1 #114								   R1-2308070

Toulouse, France, August 21st – August 25th, 2023

Agenda Item:	9.1.1.1

Source: 	MediaTek Inc.

Title:	Remaining issues on unified TCI framework extension for multi-TRP

Document for: 	Discussion and Decision

Introduction

In this contribution, we will share our views on following issues for unified TCI extension:

- General issue for unified TCI extension

- TCI selection from the indicated joint/DL/UL TCI states

- UL power control for UL MTRP

Discussion

General issue for unified TCI extension

In RAN1#113, there was intensive discussion on how to configure/determine that a CC/BWP is operated in Rel-18 unified TCI framework for S-DCI based MTRP operation or Rel-17 unified TCI framework.

For M-DCI based MTRP operation, there is no such issue since it is clear that if the UE is provided with dl-OrJointTCI-StateList or TCI-UL-State and if two coresetPoolIndex values are configured in a BWP, then the BWP is operated in Rel-18 unified TCI framework for M-DCI based MTRP operation.

However, there is no explicit RRC configuration for S-DCI based MTRP operation. For the case if the UE is provided with dl-OrJointTCI-StateList or TCI-UL-State, whether it is operated in Rel-18 unified TCI framework for S-DCI based MTRP operation or Rel-17 unified TCI framework is unclear. Based on the draft CR for Rel-18 unified TCI framework extension [1][2], the new behaviors of Rel-18 unified TCI framework for S-DCI based MTRP operation are applied “if a UE is provided dl-OrJointTCI-StateList or TCI-UL-State and is indicated first and second TCI-State or TCI-UL-State” or “when a UE is configured with dl-OrJointTCI-StateList or TCI-UL-State and is having two indicated TCI-States or TCI-UL-States”. In our view, current specification in the draft CR [1][2] should be clear for differentiation between Rel-18 unified TCI framework for S-DCI based MTRP operation and Rel-17 unified TCI framework.

Table 1. Configuration/determination for different modes of unified TCI framework

| NW Configuration | dl-OrJointTCI-StateList or TCI-UL-State is provided | Two different coresetPoolIndex values of are provided to a BWP | Having two indicated TCI-States or TCI-UL-States |
| --- | --- | --- | --- |
| Rel-17 unified TCI framework | V |  |  |
| Rel-18 unified TCI framework for S-DCI based MTRP operation |  |  | V |
| Rel-18 unified TCI framework for M-DCI based MTRP operation |  | V |  |

Proposal 1: When dl-OrJointTCI-StateList or TCI-UL-State is provided to a CC/BWP:

- If two different coresetPoolIndex values of are provided to the CC/BWP, the CC/BWP is operated in Rel-18 unified TCI framework for M-DCI based MTRP operation

- Else if the UE is indicated first and second TCI-States or TCI-UL-States to the CC/BWP, the CC/BWP is operated in Rel-18 unified TCI framework for S-DCI based MTRP operation

- Else, the CC/BWP is operated in Rel-17 unified TCI framework

TCI selection from the indicated joint/DL/UL TCI states

TCI selection for the following channels/signals are discussed in this session:

- (S-DCI) PDSCH

- (S-DCI/M-DCI) AP-CSI-RS for CSI/BM

(S-DCI) PDSCH

Based on the agreements made before RAN1#114, for S-DCI based MTRP operation, there are different UE behaviors for PDSCH reception scheduled before and after a threshold when the UE doesn’t report its capability of [two default beams for S-DCI based MTRP] in FR2. Regarding the threshold, we see there are three potential schemes as follows:

- Alt1: The threshold is a UE capability, legacy UE capability (timeDurationForQCL) is reused

- Alt2: The threshold is a UE capability, a new UE capability is used

- Alt3: The threshold is configured by NW through RRC, and the threshold should be equal or larger than a new UE capability

Legacy capability timeDurationForQCL includes UE processing time for DCI decoding and beam switching according to “TCI field”. However, the UE behavior for PDSCH reception in Rel-18 unified TCI framework is different from that based on “TCI field” in legacy TCI framework. In Rel-18 unified TCI framework, the one or both indicated joint/DL TCI states to be used for PDSCH reception are already applicable, i.e., it is just a selection from the currently applied TCI states and the beam switching time doesn’t have to considered in the threshold. Therefore, the threshold should depend on a new UE capability. On the other hand, since different UEs may have different capabilities regarding the threshold, if the threshold can be configured by NW through RRC, it would be better for NW scheduling. In summary, we prefer Alt3 for the threshold.

Proposal 2: On unified TCI framework extension for S-DCI based MTRP, the threshold used for PDSCH reception scheduled/activated by DCI format 1_0/1_1/1_2 is configured by NW through RRC signaling, and the threshold should be equal or larger than a new UE capability

(S-DCI/M-DCI) AP-CSI-RS for CSI/BM

In Rel-18 unified TCI framework for both S-DCI and M-DCI based MTRP, a resource or resource set level configuration of TCI selection (applying the fist or the second indicated joint/DL TCI state) can be provided for AP-CSI-RS for CSI/BM.

There is remaining issue for AP-CSI-RS in S-DCI case is whether to support a fixed rule is needed if the configuration of TCI selection is not provided for AP-CSI-RS for NCJT CSI. If our view, since applying different indicated joint/DL TCI states within a same resource set for NCJT CSI can be achieved by resource level configuration, the fixed rule is just an optimization for RRC overhead reduction. Therefore, we don’t see the necessitate to support the fixed rule for NCJT CSI.

Proposal 3: On unified TCI framework extension for S-DCI based MTRP, not support the following:

- For an AP CSI-RS resource set configured with two Resource Groups for NCJT CSI and configured to follow unified TCI state, if applyIndicatedTCIState is not provided to the aperiodic CSI-RS resource set, the UE shall apply the first indicated joint/DL TCI state to the CSI-RS resource(s) in Group 1 and the second indicated joint/DL TCI state to the CSI-RS resource(s) in Group 2.

Another remaining issue for both S-DCI and M-DCI cases is the UE behavior if AP-CSI-RS is triggered before a threshold and the detail of the threshold. For Rel-17 unified TCI framework, if the indicated TCI state is associated with the PCI of the serving cell, the indicated TCI state is applied to the aperiodic CSI-RS before the threshold (beamSwitchTiming) regardless of configuration of followUnifiedTCI-State. If the indicated TCI state is associated with a PCI different from the serving cell, a default QCL assumption is applied to the aperiodic CSI-RS before the threshold (beamSwitchTiming) regardless of configuration of followUnifiedTCI-State.

For Rel-18 unified TCI framework, for the aperiodic CSI-RS before the threshold, we think the behavior would depend on whether the UE support “default beam per coresetPoolIndex” or “two default beams for S-DCI based MTRP”. When the UE supports “default beam per coresetPoolIndex” or “two default beams for S-DCI based MTRP”, the UE should be able to buffer OFDM symbols before the threshold by two indicated TCI states, and the UE can apply the first or second indicated TCI state to the AP CSI-RS according to the corresponding RRC configuration after the DCI is decoded. Otherwise, a simple rule can be adopted, e.g., applying the first indicated TCI state to the AP CSI-RS.

Regarding the corresponding UE capabilities for “default beam per coresetPoolIndex” and “two default beams”, we think legacy capabilities FG 16-2a-6/16-2b-0 and corresponding RRC configurations can be reused for Rel-18 unified TCI framework, respectively. Regarding the threshold, legacy UE capability beamSwitchTiming/beamSwitchTiming-r16 can be reused for Rel-18 unified TCI framework as well.

Proposal 4: On unified TCI framework extension, if the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of an aperiodic CSI-RS resources in an aperiodic CSI-RS resource set for BM/CSI is smaller than a threshold:

- If the UE is configured with enableDefaultTCI-StatePerCoresetPoolIndex for M-DCI based MTRP or configured with enableTwoDefaultTCI-States for S-DCI based MTRP and if the aperiodic CSI-RS resource set is configured with followUnifiedTCI-State, the UE shall apply the first or the second indicated TCI state to the aperiodic CSI-RS resource set or to the aperiodic CSI-RS resource in the aperiodic CSI-RS resource set according to the RRC parameter provided to the aperiodic CSI-RS resource set or to the aperiodic CSI-RS resource in the aperiodic CSI-RS resource set. Otherwise, the UE shall apply the first indicated TCI state to the aperiodic CSI-RS resource set.

- Legacy UE capability beamSwitchTiming/beamSwitchTiming-r16 is reused as the threshold

UL Power control for UL MTRP schemes

The following issues are discussed in this session:

- Per-panel configured transmitted power for STxMP

- Type 1 power headroom report enhancement for Rel-17 TDM repetition

- Type 1 power headroom report enhancement for Rel-18 STxMP

- Per-TRP pathloss variation monitoring for PHR triggering

Per-panel configured transmitted power for STxMP

In RAN4#107, RAN4 has agreed to define ‘per-panel’ configured transmitted power for STxMP power control, and how to replace ‘per-panel’ to a spec terminology is still FFS. In our view, how to define per-panel ‘k (k=0,1)’ for PCMAXf,c,k should be decided in RAN1 group or at least RAN1 should provide suggestion to RAN4.

In RAN1#113, RAN1 has agreed that the UE shall determine a first Tx power for PUSCH/PUCCH transmission occasion i based on the UL PC parameter settings for PUSCH/PUCCH, if any, and the PL-RS included in the first indicated joint/UL TCI state, and a second Tx power for the same PUSCH/PUCCH transmission occasion i based on the UL PC parameter settings for PUSCH/PUCCH, if any, and the PL-RS included in the second indicated joint/UL TCI state. Following the same principle, we think PCMAXf,c,k can be also defined as per indicated TCI state, where k= 0,1 corresponds to the first and second indicated joint/UL TCI states, respectively. Regarding association k=0,1 with SRS resource sets, it is only applicable to PUSCH transmission instead of PUCCH transmission.

Proposal 5: On unified TCI framework extension, support PCMAXf,c,k for PUSCH/PUCCH STxMP, where k=0,1 corresponds to the first and second indicated joint/UL TCI states, respectively.

Type 1 power headroom report enhancement for Rel-17 TDM repetition

In Rel-17, two PHRs are introduced for PUSCH TDM repetition scheme. When Rel-17 two PHR mode is configured, one report in slot n contains two PHRs based on followings:

- If there is actual repetition(s) among M-TRP PUSCH repetitions in slot n, the UE provides the 1st PHR for the PUSCH repetition starting earliest in slot n, where the PUSCH repetition is associated with one SRS resource set

  - If there is another PUSCH repetition(s) associated with the other SRS resource set in slot n, the UE provides the 2nd PHR for the first PUSCH repetition associated with the other SRS resource set in slot n

  - If there is no PUSCH repetition associated with the other SRS resource set in slot n, the UE provides the 2nd PHR for a reference PUSCH transmission

- If there is no actual repetition among M-TRP PUSCH repetitions in slot n, the UE provides the two PHRs for two reference PUSCH transmissions associated with two SRS resource sets, respectively.

In Rel-18 unified TCI framework extension for S-DCI based MTRP, two PHRs can be also configured for PUSCH TDM repetition scheme. However, if the UE determines that any PHR is based on a reference PUSCH transmission, the UL PC parameter setting and PL-RS should be obtained from one of the indicated joint/UL TCI states as well.

Proposal 6: On unified TCI framework extension for S-DCI based MTRP, if two PHRs are configured, and two SRS resource sets are configured with usage set to 'codebook' or 'nonCodebook':

- If a UE provides a Type 1 PHR for a reference PUSCH transmission associated with the first SRS resource set, the UL PC parameter setting and PL-RS are obtained from the first indicated joint/UL TCI state.

- If a UE provides a Type 1 PHR for a reference PUSCH transmission associated with the second SRS resource set, the UL PC parameter setting and PL-RS are obtained from the second indicated joint/UL TCI state

Type 1 power headroom report enhancement for Rel-18 STxMP

In Rel-17, when unified TCI framework is configured, if the UE determines that a PHR is based on a reference PUSCH transmission, the UL PC parameter setting (including P0, alpha, and closed loop index) and PL-RS are obtained from the indicated joint/UL TCI state. However, in Rel-18, there are two indicated joint/UL TCI states, which one should be used for determining virtual PHR is unclear, and we prefer a simple rule for this case, e.g., based on the 1st indicated joint/UL TCI state.

Proposal 7: On unified TCI framework extension, if two PHRs are not configured to the UE and the UE determines a Type 1 PHR based on a reference PUSCH transmission, the UL PC parameter setting and PL-RS are obtained from the first indicated joint/UL TCI state.

- For M-DCI based MTRP operation, the first and the second indicated joint/UL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively.

For the case if multi-panel PUSCH Tx scheme (SDM/SFN) is configured, we think two PHRs can be reused as baseline, but some enhancements would be necessary. For PUSCH SDM/SFN scheme, there is only one PUSCH Tx occasion and the PUSCH Tx occasion may be associated with one or two SRS resource sets. Therefore, determination rule for two PHRs should be modified. Meanwhile, if any PHR is determined from a reference PUSCH transmission, the UL PC parameter setting and PL-RS should be obtained from one of the indicated joint/UL TCI states as well. In our view, the determination rule for two PHRs can be specified as follows:

- If the UE determines that one or two PHRs are based on an actual PUSCH transmission

  - If the actual PUSCH transmission is associated with two SRS resource sets, the UE provides the first PHR for the actual PUSCH transmission based on the UL PC parameter setting and PL-RS included in the 1st indicated joint/UL TCI state, and the second PHR for the actual PUSCH transmission based on the UL PC parameter setting and PL-RS included in the 2nd indicated joint/UL TCI state

  - If the actual PUSCH transmission is only associated with the first SRS resource set, the UE provides the first PHR for the actual PUSCH transmission based on the UL PC parameter setting and PL-RS included in the first indicated joint/UL TCI state, and the second PHR for a reference PUSCH transmission based on the UL PC parameter setting and PL-RS included in the second indicated joint/UL TCI state

  - If the actual PUSCH transmission is only associated with the second SRS resource set, the UE provides the first PHR for the actual PUSCH transmission based on the UL PC parameter setting and PL-RS included in the second indicated joint/UL TCI state, and the second PHR for a reference PUSCH transmission based on the UL PC parameter setting and PL-RS included in the first indicated joint/UL TCI state

- If the UE determines that two PHRs are based on a reference PUSCH transmission, the UE provides the first PHR for a reference PUSCH transmission based on the UL PC parameter setting and PL-RS included in the first indicated joint/UL TCI state, and the second PHR for a reference PUSCH transmission based on the UL PC parameter setting and PL-RS included in the second indicated joint/UL TCI stat

Proposal 8: On unified TCI framework extension for S-DCI based MTRP, if two PHRs are configured, multipanelScheme is set to ‘SDMScheme’ or‘SFNscheme’, and two SRS resource sets are configured with usage set to 'codebook' or 'nonCodebook':

- If the UE determines that one or two PHRs are based on an actual PUSCH transmission

  - If the actual PUSCH transmission is associated with two SRS resource sets, the UE provides the first PHR for the actual PUSCH transmission based on the UL PC parameter setting and PL-RS included in the 1st indicated joint/UL TCI state, and the second PHR for the actual PUSCH transmission based on the UL PC parameter setting and PL-RS included in the 2nd indicated joint/UL TCI state

  - If the actual PUSCH transmission is only associated with the first SRS resource set, the UE provides the first PHR for the actual PUSCH transmission based on the UL PC parameter setting and PL-RS included in the first indicated joint/UL TCI state, and the second PHR for a reference PUSCH transmission based on the UL PC parameter setting and PL-RS included in the second indicated joint/UL TCI state

  - If the actual PUSCH transmission is only associated with the second SRS resource set, the UE provides the first PHR for the actual PUSCH transmission based on the UL PC parameter setting and PL-RS included in the second indicated joint/UL TCI state, and the second PHR for a reference PUSCH transmission based on the UL PC parameter setting and PL-RS included in the first indicated joint/UL TCI state

- If the UE determines that two PHRs are based on a reference PUSCH transmission, the UE provides the first PHR for a reference PUSCH transmission based on the UL PC parameter setting and PL-RS included in the first indicated joint/UL TCI state, and the second PHR for a reference PUSCH transmission based on the UL PC parameter setting and PL-RS included in the second indicated joint/UL TCI state

Figure 1. Examples of two PHRs are not configured and configured for M-DCI based MTRP

For M-DCI based MTRP, we think two PHRs are also beneficial. For example, if a PHR is triggered due to path loss variation for a first TPR, however, NW may schedule an actual PUSCH transmission to a second TRP. Based on current specification, the UE shall report power headroom based on the actual PUSCH transmission to the second TRP rather than the first TRP. Therefore, NW cannot obtain the power headroom for the first TRP even the report is triggered for the = first TRP. To resolve this problem, two PHRs can be supported for M-DCI based MTRP. For above example, when two PHRs are configured, the UE provides a first PHR for the actual PUSCH transmission scheduled to a second TRP and a second PHR for a reference PUSCH transmission for the first TRP.

Proposal 9: On unified TCI framework extension for M-DCI based MTRP, support two PHRs when two SRS resource sets are configured with usage set to 'codebook' or 'nonCodebook'

- If the UE determines that one of the PHRs is based on an actual PUSCH transmission

  - If the actual PUSCH transmission is associated with the first SRS resource set, the UE provides the first PHR for the actual PUSCH transmission based on the UL PC parameter setting and PL-RS included in the first indicated joint/UL TCI state, and the second PHR for a reference PUSCH transmission based on the UL PC parameter setting and PL-RS included in the second indicated joint/UL TCI state

  - If the actual PUSCH transmission is associated with the second SRS resource set, the UE provides the first PHR for the actual PUSCH transmission based on the UL PC parameter setting and PL-RS included in the second indicated joint/UL TCI state, and the second PHR for a reference PUSCH transmission based on the UL PC parameter setting and PL-RS included in the first indicated joint/UL TCI state

- If the UE determines that two PHRs are based on reference PUSCH transmissions, the UE provides the first PHR for a reference PUSCH transmission based on the UL PC parameter setting and PL-RS included in the first indicated joint/UL TCI state, and the second PHR for a reference PUSCH transmission based on the UL PC parameter setting and PL-RS included in the second indicated joint/UL TCI state

For above, the first and the second indicated TCI states correspond to the indicated states specific to coresetPoolIndex value 0 and value 1, respectively.

Per-TRP pathloss variation monitoring for PHR triggering

A power headroom report could be triggered by the event which are according to the pathloss variation on monitoring resource(s) (e.g., pathloss variation):

- Pathloss variation over time since the last transmission of a PHR has changed more than a phr-Tx-PowerFactorChange dB, irrespective of whether the pathloss reference has changed in between.

Under S-DCI based or M-DCI based M-TRP operation, one triggering issue may be raised. In one example of path-loss variation triggered PHR, since pathloss variation monitoring is based on pathloss RSs in a same cell instead of a specific pathloss RS, a UE may determine pathloss variation according to two pathloss RSs associated with different TRPs. Hence, a false alarm on pathloss variation may happen if cross comparing the two pathloss values associated with two different TRPs.

In our view, the links to two different TRPs should be considered as independent for pathloss variation monitoring, such that the pathloss variation monitoring should be TRP-specific. In addition, regardless of single PHR mode or two PHR mode, the TRP-specific pathloss variation monitoring should be applied if M-TRP operation is configured. Under unified TCI framework, TRP-specific pathloss variation monitoring can be determined based on the pathloss RS(s) included in the TCI-state(s) corresponding the same first/second indicated TCI state.

Proposal 10: For power headroom reporting enhancement, if a UE is configured with two SRS resource sets for CB/NCB, support per-TRP pathloss variation monitoring for event-triggered PHR

- The pathloss reference used for determining the path loss variation of one cell at different time should be derived from the same first or second indicated TCI state

- Note: The pathloss reference can be changed according to TCI state update to the first or second indicated TCI state

Conclusion

In this contribution, extension of unified TCI framework for MTRP operation was discussed. Based on the discussion in the previous sections, we made the following proposals and observations:

General issue for unified TCI extension

Proposal 1: When dl-OrJointTCI-StateList or TCI-UL-State is provided to a CC/BWP:

- If two different coresetPoolIndex values of are provided to the CC/BWP, the CC/BWP is operated in Rel-18 unified TCI framework for M-DCI based MTRP operation

- Else if the UE is indicated first and second TCI-States or TCI-UL-States to the CC/BWP, the CC/BWP is operated in Rel-18 unified TCI framework for S-DCI based MTRP operation

- Else, the CC/BWP is operated in Rel-17 unified TCI framework

TCI selection from the indicated joint/DL/UL TCI states

Proposal 2: On unified TCI framework extension for S-DCI based MTRP, the threshold used for PDSCH reception scheduled/activated by DCI format 1_0/1_1/1_2 is configured by NW through RRC signaling, and the threshold should be equal or larger than a new UE capability

Proposal 3: On unified TCI framework extension for S-DCI based MTRP, not support the following:

- For an AP CSI-RS resource set configured with two Resource Groups for NCJT CSI and configured to follow unified TCI state, if applyIndicatedTCIState is not provided to the aperiodic CSI-RS resource set, the UE shall apply the first indicated joint/DL TCI state to the CSI-RS resource(s) in Group 1 and the second indicated joint/DL TCI state to the CSI-RS resource(s) in Group 2.

Proposal 4: On unified TCI framework extension, if the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of an aperiodic CSI-RS resources in an aperiodic CSI-RS resource set for BM/CSI is smaller than a threshold:

- If the UE is configured with enableDefaultTCI-StatePerCoresetPoolIndex for M-DCI based MTRP or configured with enableTwoDefaultTCI-States for S-DCI based MTRP and if the aperiodic CSI-RS resource set is configured with followUnifiedTCI-State, the UE shall apply the first or the second indicated TCI state to the aperiodic CSI-RS resource set or to the aperiodic CSI-RS resource in the aperiodic CSI-RS resource set according to the RRC parameter provided to the aperiodic CSI-RS resource set or to the aperiodic CSI-RS resource in the aperiodic CSI-RS resource set. Otherwise, the UE shall apply the first indicated TCI state to the aperiodic CSI-RS resource set.

- Legacy UE capability beamSwitchTiming/beamSwitchTiming-r16 is reused as the threshold

UL Power control for UL MTRP schemes

Proposal 5: On unified TCI framework extension, support PCMAXf,c,k for PUSCH/PUCCH STxMP, where k=0,1 corresponds to the first and second indicated joint/UL TCI states, respectively.

Proposal 6: On unified TCI framework extension for S-DCI based MTRP, if two PHRs are configured, and two SRS resource sets are configured with usage set to 'codebook' or 'nonCodebook':

- If a UE provides a Type 1 PHR for a reference PUSCH transmission associated with the first SRS resource set, the UL PC parameter setting and PL-RS are obtained from the first indicated joint/UL TCI state.

- If a UE provides a Type 1 PHR for a reference PUSCH transmission associated with the second SRS resource set, the UL PC parameter setting and PL-RS are obtained from the second indicated joint/UL TCI state

Proposal 7: On unified TCI framework extension, if two PHRs are not configured to the UE and the UE determines a Type 1 PHR based on a reference PUSCH transmission, the UL PC parameter setting and PL-RS are obtained from the first indicated joint/UL TCI state.

- For M-DCI based MTRP operation, the first and the second indicated joint/UL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively.

Proposal 8: On unified TCI framework extension for S-DCI based MTRP, if two PHRs are configured, multipanelScheme is set to ‘SDMScheme’ or‘SFNscheme’, and two SRS resource sets are configured with usage set to 'codebook' or 'nonCodebook':

- If the UE determines that one or two PHRs are based on an actual PUSCH transmission

  - If the actual PUSCH transmission is associated with two SRS resource sets, the UE provides the first PHR for the actual PUSCH transmission based on the UL PC parameter setting and PL-RS included in the 1st indicated joint/UL TCI state, and the second PHR for the actual PUSCH transmission based on the UL PC parameter setting and PL-RS included in the 2nd indicated joint/UL TCI state

  - If the actual PUSCH transmission is only associated with the first SRS resource set, the UE provides the first PHR for the actual PUSCH transmission based on the UL PC parameter setting and PL-RS included in the first indicated joint/UL TCI state, and the second PHR for a reference PUSCH transmission based on the UL PC parameter setting and PL-RS included in the second indicated joint/UL TCI state

  - If the actual PUSCH transmission is only associated with the second SRS resource set, the UE provides the first PHR for the actual PUSCH transmission based on the UL PC parameter setting and PL-RS included in the second indicated joint/UL TCI state, and the second PHR for a reference PUSCH transmission based on the UL PC parameter setting and PL-RS included in the first indicated joint/UL TCI state

- If the UE determines that two PHRs are based on a reference PUSCH transmission, the UE provides the first PHR for a reference PUSCH transmission based on the UL PC parameter setting and PL-RS included in the first indicated joint/UL TCI state, and the second PHR for a reference PUSCH transmission based on the UL PC parameter setting and PL-RS included in the second indicated joint/UL TCI state

Proposal 9: On unified TCI framework extension for M-DCI based MTRP, support two PHRs when two SRS resource sets are configured with usage set to 'codebook' or 'nonCodebook'

- If the UE determines that one of the PHRs is based on an actual PUSCH transmission

  - If the actual PUSCH transmission is associated with the first SRS resource set, the UE provides the first PHR for the actual PUSCH transmission based on the UL PC parameter setting and PL-RS included in the first indicated joint/UL TCI state, and the second PHR for a reference PUSCH transmission based on the UL PC parameter setting and PL-RS included in the second indicated joint/UL TCI state

  - If the actual PUSCH transmission is associated with the second SRS resource set, the UE provides the first PHR for the actual PUSCH transmission based on the UL PC parameter setting and PL-RS included in the second indicated joint/UL TCI state, and the second PHR for a reference PUSCH transmission based on the UL PC parameter setting and PL-RS included in the first indicated joint/UL TCI state

- If the UE determines that two PHRs are based on reference PUSCH transmissions, the UE provides the first PHR for a reference PUSCH transmission based on the UL PC parameter setting and PL-RS included in the first indicated joint/UL TCI state, and the second PHR for a reference PUSCH transmission based on the UL PC parameter setting and PL-RS included in the second indicated joint/UL TCI state

For above, the first and the second indicated TCI states correspond to the indicated states specific to coresetPoolIndex value 0 and value 1, respectively.

Proposal 10: For power headroom reporting enhancement, if a UE is configured with two SRS resource sets for CB/NCB, support per-TRP pathloss variation monitoring for event-triggered PHR

- The pathloss reference used for determining the path loss variation of one cell at different time should be derived from the same first or second indicated TCI state

- Note: The pathloss reference can be changed according to TCI state update to the first or second indicated TCI state

Reference

- R1-2306292, “Introduction of MIMO Evolution for Downlink and Uplink”, Samsung, June 2023

- R1-2306332, “Introduction of specification support for MIMO enhancements on uTCI_STxMP_DMRS_SRS_8Tx_2TA, Nokia, June 2023

- R4-2310268, “WF on UE RF requirements for NR MIMO evolution”, Samsung, May 2023

- 3GPP RAN1 #113, “Chairman’s Notes RAN1 #113”, May 2023

- 3GPP RAN1 #112bis-e, “Chairman’s Notes RAN1 #112bis-e”, April 2023

- 3GPP RAN1 #112, “Chairman’s Notes RAN1 #112”, February 2023

- 3GPP RAN1 #111, “Chairman’s Notes RAN1 #111”, November 2022

- 3GPP RAN1 #110bis-e, “Chairman’s Notes RAN1 #110bis-e”, October 2022

- 3GPP RAN1 #110, “Chairman’s Notes RAN1 #110”, August 2022

- 3GPP RAN1 #109e, “Chairman’s Notes RAN1 #109-e”, May 2022