3GPP TSG RAN WG1 #114			R1-2308233

Toulouse, France, August 21st – August 25th, 2023

Agenda item:	9.14.1

Source: 	Moderator (Qualcomm Incorporated)

Title: 	Feature lead summary on Evaluation Methodology for IMT-2020 satellite

Document for:	Discussion and Decision

# Proposals for online discussion

The following proposals have been stable for several rounds of discussion (except for the proposal 7.1, which has been stable for less than 24h).

Proposal 1.1: For DL and UL overhead:

- DL overhead is assumed to be 0.14

- UL overhead is assumed to be 0.08

Proposal 2.1: For the evaluation metrics that require usage of beam area, a beam area of 1415 km2 is assumed.

Proposal 3.1: The modeling of delay / RTT for SLS is up to companies to report.

Proposal 5.1.1: All the beams are considered for computing the area of connection density.

Proposal 5.1.2: For SLS to LLS metric of connection density, “pre-processing SINR” should be used instead of “pre-processing SNR”.

Proposal 5.1.3a: Elevation angle of 90° is used for determining mean and standard deviation values of K-factor and delay spread for NTN TDL-C Rural channel model.

Proposal 5.2.2: For mMTC LLS (NR and eMTC), 16-QAM can be used in addition to QPSK to derive the SNR to SE mapping.

Proposal 5.2.3a: For connection density evaluation with full buffer system-level simulation followed by link-level simulation, for SINR CDF distribution derivation, all the beams are assumed to be fully loaded.

Proposal 6.1a: Observation:

Based on RAN1 evaluations, for peak data rate and spectral efficiency the max code rate and modulation order is:

- For DL, the modulation scheme is Qm=6, and Rmax between 666/1024 and 822/1024

- For UL, the modulation scheme is Qm=4, and Rmax between 434/1024 and 553/1024

3GPP will report peak data rate and spectral efficiency based on

- Rmax=822/1024 and Qm=6 for DL

- Rmax=553/1024 for and Qm=4 for UL

Proposal 7.1: For the link budget template:

- The attached xls to R1-2308233 is endorsed as the starting point for the link budget template.

- The target elevation angle is [30] degrees, and the link budget template includes the link margin (in dB) at that elevation angle.

- The LLS to derive the “required SNR” is performed under an NTN TDL-C rural channel model with the K-factor corresponding to the target elevation angle.

- The rest of the parameters (bits / #reps / others) and whether different values are used for different scenarios (eMBB-s, HRC-s, mMTC-s) are to be discussed via email discussion.

# Issue #1: Assumed overhead

The following proposal is stable from the 1st round. No more discussion is needed

Proposal 1.1: For DL and UL overhead:

- DL overhead is assumed to be 0.14

- UL overhead is assumed to be 0.08

# Issue #6: Peak data rate and spectral efficiency

Given the input in the 1st round, the following proposal is updated:

Proposal 6.1a:

Observation:

Based on RAN1 evaluations, for peak data rate and spectral efficiency the max code rate and modulation order is:

- For DL, the modulation scheme is Qm=6, and Rmax between 666/1024 and 772822/1024

- For UL, the modulation scheme is Qm=4, and Rmax between 434/1024 and 553/1024

FL to prepare a TP for the TR based on the agreement above.

3GPP will report peak data rate and spectral efficiency based on

- Rmax=822/1024 and Qm=6 for DL

- Rmax=553/1024 for and Qm=4 for UL

## Q6.1: Please provide comments on proposal 6.1a

| Company | Comment |
| --- | --- |
| ZTE | OK |
| Ericsson | OK |

# Issue #2: Calculation of area

Based on the input of the 1st round, and some offline discussion, we propose to use the “average beam area” for the calculations that requires the area of a beam.

The proposed scheme is as follows:

- Calculate the area of the ellipsoid of each of the beams in the surface of the earth Abeam,i

- Calculate the average area across all the ellipsoids AbeamAve=mean(Abeam,i)

- The area used for calculations is the area of the hexagon within a circular beam of area AbeamAve, i.e., A=AbeamAve×3√232π

Following this calculation, the area is approximately 1415 km2

Proposal 2.1: For the evaluation metrics that require usage of beam area, a beam area of 1415 km2 is assumed.

## Q2.1: Please provide comments on proposal 2.1

| Company | Comment |
| --- | --- |
| Huawei/HiSilicon | Agree with proposal 2.1 as it is. Minor comments: there is a typo in the text of proposed scheme: it should be sqrt(3) instead of sqrt(2) A=AbeamAve×3√32π |
| ZTE | Agree with the changes above. |
| Ericsson | Agree with Proposal 2.1 and with the change introduced for computing A in the third sub-bullet above. |

# Others

The following proposal are stable from the 1st round and no more discussion is needed

Proposal 5.1.1: All the beams are considered for computing the area of connection density.

Proposal 5.1.2: For SLS to LLS metric of connection density, “pre-processing SINR” should be used instead of “pre-processing SNR”.

Proposal 3.1: The modeling of delay / RTT for SLS is up to companies to report.

Proposal 5.2.2: For mMTC LLS (NR and eMTC), 16-QAM can be used in addition to QPSK to derive the SNR to SE mapping.

The following proposal has been slightly modified in line with the comments but overall seems to be stable:

Proposal 5.1.3a: Elevation angle of 80° or 90° is used for determining mean and standard deviation values of K-factor and delay spread for NTN TDL-C Rural channel model.

The following two proposals are not stable and need further discussion. For Proposal 5.2.3, it is FL’s understanding that for the SINR derivation in full buffer, it should be assumed that all the cells are fully loaded. If we assume 10UEs per cell, the interference seen will be reduced.

Proposal 5.2.3a (HW): For connection density evaluation with full buffer system-level simulation followed by link-level simulation, UE number of 10 UEs per beam is used for SINR CDF distribution derivation, all the beams are assumed to be fully loaded.

Proposal 5.2.4 (ZTE): Frequency reuse factor = 4 with combination of Polarization and frequency can also be considered for evaluation.

## Q6.1: Please provide comments on proposals 5.1.3a, 5.2.3a and 5.2.4

| Company | Comment |
| --- | --- |
| ZTE | We are supportive on these two. Also, for 5.2.3(a), agree with FL on the assessment of interference. |
| Ericsson | Proposal 5.1.3a: OK Proposal 5.2.3a (HW): OK Proposal 5.2.4 (ZTE): We still don’t see the need for the evaluations with FRF=4. |

# Link budget template

The link budget template is uploaded in the drafts folder for NR and IOT NTN.

For LLS for link budget:

- NTN TDL-C rural with 30 degree elevation angle is used.

- Target bitrate (i.e. TBS / #repetitions) is to be discussed by email

For the link budget, a target elevation angle of [30] degrees is assumed, and the link margin (in dB) at that elevation angle is reported.

Therefore, we make the following proposal:

Proposal 7.1: For the link budget template:

- The attached xls is endorsed as the starting point for the link budget template.

- The target elevation angle is [30] degrees, and the link budget template includes the link margin (in dB) at that elevation angle.

- The LLS to derive the “required SNR” is performed under an NTN TDL-C rural channel model with the K-factor corresponding to the target elevation angle.

- The rest of the parameters (bits / #reps / others) and whether different values are used for different scenarios (eMBB-s, HRC-s, mMTC-s) are to be discussed via email discussion.

| Company | Comment |
| --- | --- |

# [Previous round]

## Issue #1: Assumed overhead

The following input was received in this meeting:

| Company | Downlink overhead | Uplink overhead |
| --- | --- | --- |
| Huawei | 0.1115 | 0.0875 |
| ZTE | 0.14 | 0.08 |
| Samsung | 0.14 | 0.08 |
| Qualcomm | 0.14 | 0 |

Based on the majority view, the FL makes the following proposal:

Proposal 1.1: For DL and UL overhead:

- DL overhead is assumed to be 0.14

- UL overhead is assumed to be 0.08

### Q1.1: Please provide comments on proposal 1.1:

| Company | Comment |
| --- | --- |
| Huawei/HiSilicon | OK with the proposal |
| Nokia, Nokia Shanghai Bell | OK |
| Ericsson | OK |
| ZTE | Ok |
| Thales | Ok |
| Panasonic | OK |

## Issue #2: Calculation of area

Regarding the area calculation for the different metric, the following proposals are made to this meeting:

- ZTE: The area of each beam can be calculated as

- Thales: For a LEO600km based NTN with satellite beam diameter of 50km, ρ is calculated based on the area of the hexagonal cell circumscribed by the circular satellite beam footprint as follows: ρ = 1/ 1623,80 km-1.

Given that several evaluation results have been already submitted to this meeting, and that different method to calculate the area are expected to differ in a very small amount, it is the FL’s view that RAN1 may not need to make additional agreements regarding how to calculate the area

### Q2.1: Please provide comments on the proposals above. Specifically, whether RAN1 should make further agreements or whether it can be left to the different companies:

| Company | Comment |
| --- | --- |
| Huawei/HiSilicon | The difference among company results does not seem to be small (17% difference between 1623 and 1386), we suggest to make agreement on the calculation of area as otherwise this difference in area calculation will lead to difference in final results. For calculation of area, we suggest to follow system simulation configuration as defined in Section 6.1.1.1-4 of TR38.821. |
| Nokia, Nokia Shanghai Bell | We are a bit indifferent to the way the area is calculated – the main thing is that the consistent definition of area is defined such that results for density can be compared in a fair manner. |
| Ericsson | Agree with FL’s view that no need to make further agreements. |
| MediaTek | Agree something common between companies should be agreed. |
| ZTE | The calculation is somehow approximation of coverage. Since we don’t touch the details of beam design in satellite, e.g., beam shaping, something with relative accuracy is sufficient. |
| Thales | An agreement is needed In our view: for a LEO600km based NTN with satellite beam diameter of 50km, ρ is calculated based on the area of the hexagonal cell circumscribed by the circular satellite beam footprint as follows: ρ = 1/ 1623,80 km-1. |
| Panasonic | Agree with FL. |

## Issue #3: Delay for SLS

Several inputs were received regarding how to calculate the delay for SLS:

- ZTE: A minimum 10 degree elevation angle can be considered for feeder link.

- Nokia: Model variable feeder link delay

Given the situation, FL proposes that companies report how they model the delay (including feeder link delay).

Proposal 3.1: The modeling of delay / RTT for SLS is up to companies to report.

### Q3.1: Please provide comments on proposal 3.1:

| Company | Comment |
| --- | --- |
| Nokia, Nokia Shanghai Bell | OK |
| Ericsson | Agree with FL proposal. |
| ZTE | Ok. |
| Thales | Ok |
| Panasonic | OK |

## Issue #4: UP and CP latency

Several companies (Thales, Oppo, Samsung) provided input on UP and CP latency. It is FL’s observation that RAN2 is currently discussing these metrics, so the FL recommends to postpone this discussion until RAN2 requests some input..

## Others

Several proposals were made on different topics by multiple companies.

From FL perspective, the following two proposals from ZTE should not be controversial and may be directly agreed:

Proposal 5.1.1 (ZTE): All the beams are considered for computing the area of connection density.

Proposal 5.1.2 (ZTE): For SLS to LLS metric of connection density, “pre-processing SINR” should be used instead of “pre-processing SNR”.

Proposal 5.1.3 (HW): Elevation angle of 80° or 90° is used for determining mean and standard deviation values of K-factor and delay spread for NTN TDL-C Rural channel model.

### Q5.1: Please provide comments on proposals 5.1.1 to 5.1.3:

| Company | Comment |
| --- | --- |
| Huawei/HiSilicon | OK with 5.1.1 and 5.1.2. Suggest to agree on proposal 5.1.3, or something equivalent (e.g. 90° elevation angel if agreeable by companies). It is important to agree on this parameter as it impacts performance of link-level simulation as shown in our contribution. We observed few other companies also use 90° elevation angel for link-level simulation. |
| Nokia, Nokia Shanghai Bell | OK with proposals 5.1.1, 5.1.2 and 5.1.3. |
| Ericsson | Agree with all three proposals. |
| MediaTek | Ok with all 3 proposals. |
| ZTE | Ok for all |
| Thales | Ok |

The following proposals are not trivial and may require additional discussions:

Proposal 5.2.1 (ZTE): For reliability calculations, The maximum delay time should be discussed, 34 ms can be the starting point for discussion.

Proposal 5.2.2 (QC): For mMTC LLS (NR and eMTC), 16-QAM can be used in addition to QPSK to derive the SNR to SE mapping.

Proposal 5.2.3 (HW): For connection density evaluation with full buffer system-level simulation followed by link-level simulation, UE number of 10 UEs per beam is used for SINR CDF distribution derivation.

Proposal 5.2.4 (ZTE): Frequency reuse factor = 4 with combination of Polarization and frequency can also be considered for evaluation.

### Q5.2: Please provide comments on proposals 5.2.1 to 5.2.4:

| Company | Comment |
| --- | --- |
| Huawei/HiSilicon | Suggest to further discuss and agree on proposal 5.2.3. This is important for SINR CDF derivation. |
| Nokia, Nokia Shanghai Bell | OK with proposal 5.2.1 Not OK with Proposla 5.2.4, as “reuse factor” of 4 would in principle require mandatory support from UE side of both RHCP and LHCP in order to facilitate such operation. Current scenarios for frequency reuse should be sufficient. |
| Ericsson | Proposal 5.2.1: ITU-R Report M.2514 states “The satellite component of IMT-2020 shall be able to support reliability as high as 1-10−3. The connection time duration required to meet the reliability target should be provided by the proponent of the submitted RIT/SRIT.” So in our view we only need to show that we can reach a reliability of 1-10−3, and then simply report the time involved in achieving such reliability. Proposal 5.2.2: Ok, if warranted by the SINR. Proposal 5.2.3: No strong opinion since we are not planning to do those kinds of simulations. Proposal 5.2.4: We have not yet done these evaluations, so we cannot judge if this is really needed. |
| MediaTek | Proposal 5.2.3: Given that for NB-IoT, the resource is per subcarrier, we wonder if 12 UEs per beam in that case is not more appropriate than 10 UEs. |
| ZTE | Proposal 5.2.1: Ok Proposal 5.2.2: Yes, it’s up to link budget. For Proposal 5.2.3: actually, more UEs, e.g., 100 is better to achieve the smooth CDF. Proposal 5.2.4: Company is encouraged to check it since current the results can not satisfy the requirement without FR4 |

## Issue #6: Candidate values for peak data rate and spectral efficiency

In the previous meeting, the following was agreed:

For uplink peak spectral efficiency and peak data rate:

- A bandwidth of 8 RBs is used.

- The peak data rate is [2.22-2.65]Mpbs

- The peak spectral efficiency is [1.54-1.84]bps/Hz

- This value is achieved with an uplink SNR of [7.05-7.08]dB

- FFS: Other assumptions to be included in the TR.

For downlink peak spectral efficiency and peak data rate:

- A transmission bandwidth of 160 PRBs out of a channel bandwidth of 30 MHz is used. Channel bandwidth of 30 MHz is used as denominator for calculation of spectrum efficiency.

- The peak data rate is [104.58-118.9]Mbps

- The peak spectral efficiency is [3.67-4.13]bps/Hz

- This value is achieved with an uplink SNR of [16.91-16.95]dB

- FFS: Other assumptions to be included in the TR.

In view of the above, and taking into account the different overhead assumed by different companies, the FL makes the following proposal

Proposal 6.1: For peak data rate and spectral efficiency:

- For DL, the modulation scheme is Qm=6, and Rmax between 666/1024 and 772/1024

- For UL, the modulation scheme is Qm=4, and Rmax between 434/1024 and 553/1024

FL to prepare a TP for the TR based on the agreement above.

### Q6.1: Please provide comments on proposal 6.1

| Company | Comment |
| --- | --- |
| Ericsson | OK. |
| ZTE | Ok |
| Thales | If only one value should be captured in the TR, We may consider only one value for the max coding rate for uplink and one value for the max coding rate for downlink. A Text proposal is being discussed under AI 9.14.2. This proposal when agreed can be captured in the below TP: https://www.3gpp.org/ftp/Meetings_3GPP_SYNC/RAN1/Inbox/drafts/9.14(FS_IMT2020_SAT_eval)/9.14.2/1_Peak%20SE%20and%20peak%20data%20rate/R1-230641x%20%20%20-%20Att%20%20TP%20for%20TR37.911-Peak%20spectral%20efficiency%20and%20peak%20data%20rate-v00.docx |
| Panasonic | Because several companies assumes R=822/1024 (MCS25) for DL, we suggest to update as follows. For DL, the modulation scheme is Qm=6, and Rmax between 666/1024 and 772822/1024 |

# Summary of contributions

| Tdoc# | Company | Proposals |
| --- | --- | --- |
| R1-2306507 | Huawei, HiSilicon | Proposal 1: For peak spectral efficiency, peak data rate and SLS eMBB-s evaluations of NR NTN, the overhead assumptions listed in Table 1 are used，which leads to overhead of 0.1115 and 0.0875 for downlink and uplink respectively Proposal 2: Elevation angle of 80° or 90° is used for determining mean and standard deviation values of K-factor and delay spread for NTN TDL-C Rural channel model. Proposal 3: For connection density evaluation with full buffer system-level simulation followed by link-level simulation, UE number of 10 UEs per beam is used for SINR CDF distribution derivation. |
| R1-2306567 | ZTE | Proposal 1: For the purpose of calibration, the recommended overhead values in 38.306 [2] can be used to align the evaluation results among companies. Proposal 2: A minimum 10 degree elevation angle can be considered for feeder link. Proposal 3: Frequency reuse factor = 4 with combination of Polarization and frequency can also be considered for evaluation. Proposal 4: The area of each beam can be calculated as Proposal 5: All the beams are considered for computing the area of connection density. Proposal 6: For SLS to LLS metric of connection density, “pre-processing SINR” should be used instead of “pre-processing SNR”. Proposal 7: The maximum delay time should be discussed, 34 ms can be the starting point for discussion. Proposal 8: The baseline scenario can be earth moving cell using a single NTN gateway without feeder link switching involved. |
| R1-2307247 | Nokia, Nokia Shanghai Bell | Proposal 1: To ensure realistic evaluations of the experienced RTT, the variable feeder link delay should be taken into account when reporting the results for the IMT-2020 satellite evaluations. |
| R1-2307551 | OPPO | Proposal 1: Consider Table 1 and Table 2 for evaluation of user plane latency for NR-NTN downlink and uplink respectively. Proposal 2: Consider Table 3 for evaluation of control plane latency for NR-NTN. |
| R1-2307706 | Samsung | Proposal 1: Consider 0.14 for DL and 0.08 for UL as overhead values, respectively, to calculate peak data rate and peak spectral efficiency. Proposal 2: As for User/Control plane latency, reuse the existing assumption of UE and gNB processing time that is defined in TR 37.910. Proposal 3: As for User/Control plane latency, assume that satellite processing time is zero. |
| R1-2307729 | THALES | Proposal 1: Peak spectral efficiency is evaluated in NR NTN using the analytical method with: - For DL: a modulation order of 6 with a maximum coding rate of 0.7021 - For UL: a modulation order of 4 with a maximum coding rate of 0,4785 Proposal 2: Using the analytical method, area traffic capacity can be derived from the average spectral efficiency SEavg as follows: Carea=ρ×BW×SEavg Wherein: BW denote the channel bandwidth And ρ is the TRxP (transmission and reception point) density (TRxP/m2). For a LEO600km based NTN with satellite beam diameter of 50km, ρ is calculated based on the area of the hexagonal cell circumscribed by the circular satellite beam footprint as follows: ρ = 1/ 1623,80 km-1. Proposal 3: The Table 7 is used for NR NTN Control plane latency analysis Proposal 4: Send LS to RAN2 to provide RAN1 inputs on Control plane procedure latency Proposal 5: The components in Table 8 are considered for DL user plane procedure latency analysis for NR NTN with HARQ enabled. FFS: HARQ is disabled and radio retransmission handled by RLC ARQ. Proposal 6: The components in Table 9 are considered for UL user plane procedure latency analysis for NR NTN. FFS HARQ is disabled and radio retransmission handled by RLC ARQ. Proposal 7: Send LS to RAN2 to provide RAN1 inputs on User plane procedure latency Proposal 8: Beam level mobility in same cell with no RRC involvement should be considered for mobility interruption time evaluation. |
| R1-2307955 | Qualcomm Incorporated | Proposal 1: RAN1 to discuss whether to increase the number of users per cell to increase the achievable spectral efficiency. Proposal 2: For mMTC LLS (NR and eMTC), 16-QAM can be used in addition to QPSK to derive the SNR to SE mapping. Proposal 3: For uplink peak data rate and spectral efficiency, the assumed overhead is 0. Proposal 4: For downlink peak data rate and spectral efficiency, the assumed overhead is 0.14. |

# References

[1] R1-230xxxx Self-evaluation results for NR NTN, RAN1#114, Qualcomm Incorporated