3GPP TSG RAN WG1 Meeting #115	R1-2310985

Chicago, USA, November 13th – November 17th, 2023

Agenda item:		8.5.1

Source:	Nokia, Nokia Shanghai Bell

Title:		Remaining issues for Techniques in Spatial and Power Domains

Document for:		Discussion and Decision

# Introduction

In this contribution, we provide our views on the remaining issues for the techniques in spatial and power domains. And in our companion contribution, we discuss the remaining issues for the enhancements on cell DTX/DRX mechanism [1].

# Techniques in Spatial and Power domains

In the previous RAN1 meetings, several agreements were reached regarding the Techniques in Spatial domain. In the following, we discuss some of the remaining open points for such techniques.

## On CRI reporting

On which CSI quantities to report, the following baseline was supported according to the RAN1#113 agreement below:

Agreement For both spatial domain NES, when UE reports CSIs corresponding to one or more sub-configurations provided in a CSI report configuration, - At least support baseline: Report CSI for each indicated sub-configuration, according to reportQuantity configuration - FFS: details on how to map CSI(s) in a CSI report - Further enhancement on CSI payload reduction is not precluded

In RAN1#114, as can be seen in the conclusion copied below, there will be no enhancements on PMI reduction or RI reduction in Rel-18 NES.

Conclusion - No further enhancements for PMI reduction in R18 NES. - No further enhancements for RI reduction in R18 NES. - No support of UE reporting PDSCH power reduction tolerance in R18 NES.

In the following, we discuss some clarification with regard to reporting CRI (CSI-RS resource indicator).

For report quantities including CRI, similar logic as applied for CRI reporting in legacy could be considered but now per CSI report sub-configuration instead of configuration. Specifically, if a sub-configuration includes or is associated with more than one CSI-RS resource (for channel measurements), then CRI selection and thus reporting is needed for that sub-configuration. Otherwise, if a sub-configuration includes or is associated with a single CSI-RS resource, then CRI reporting is not needed for that sub-configuration.

Proposal 1: For CRI reporting for Type 1/Type 2 SD adaptation, support the following legacy behavior but now per CSI report sub-configuration as follows:

- If a sub-configuration includes or is associated with more than one CSI-RS resource (for channel measurements), then CRI selection and thus reporting is needed for that sub-configuration.

- Otherwise, if a sub-configuration includes or is associated with a single CSI-RS resource (for channel measurements), then CRI reporting is not needed for that sub-configuration.

## Report quantities 'cri-RI-CQI', 'cri-RI-i1-CQI', and 'cri-RI-i1'

In RAN1#114bis, the support of report quantities 'cri-RI-i1-CQI', 'cri-RI-CQI', and 'cri-RI-i1' was agreed, as can be seen from the agreement copied below:

Agreement Support gNB can configure report quantities of 'cri-RI-i1-CQI', 'cri-RI-CQI', or 'cri-RI-i1'.

For 'cri-RI-i1', it seems that no special consideration is needed.

On the other side, for 'cri-RI-i1-CQI', it should be discussed whether the CBSR parameter typeISinglePanel-codebookSubsetRestriction-i2 needs to be configured on per sub-configuration level for Type 1 SD adaptation with or without PD adaptation. Since RAN1 already agreed to configure CBSR in each sub-configuration for Type 1 SD adaptation, it’s expected that typeISinglePanel-codebookSubsetRestriction-i2 would need to be configured in each sub-configuration in this case.

Proposal 2: For Type 1 SD adaptation (with or without PD adaptation) with a CSI report configuration configured with a list of sub-configurations, if the UE is configured with the report quantity 'cri-RI-i1-CQI', typeISinglePanel-codebookSubsetRestriction-i2 should be configured in each sub-configuration.

On the other hand, for 'cri-RI-CQI' (also known as non-PMI feedback), in legacy operation, which CSI-RS port should be used for a certain rank hypothesis can be indicated in the CSI report configuration through the high layer parameter non-PMI-PortIndication field. Specifically, if the UE is configured with non-PMI-PortIndication, for each CSI-RS resource in the linked ResourceConfig for channel measurement, a port indication for each rank R, indicating which R ports to use, is provided – where this is indicated in form of a sequence as described in TS 38.214. Note this indication is used e.g., for CQI calculation, and the non-PMI operation is restricted to the case of up to 8 CSI-RS ports.

It's also worth noting that, if the UE is not configured with non-PMI-PortIndication, the UE assumes for rank R the first R ports in the increasing order of port index.

If non-PMI-PortIndication is to be configured for the UE, and given that under Rel-18 NES, specifically Type 1 SD adaptation with or without PD adaptation, a CSI report configuration can be configured with a list of sub-configurations each of which corresponding to an antenna port subset, then a single non-PMI-PortIndication under the CSI report configuration would not work. Actually, in this case, non-PMI-PortIndication would be required for each sub-configuration.

Proposal 3: For Type 1 SD adaptation (with or without PD adaptation) with a CSI report configuration configured with a list of sub-configurations, if the UE is configured with the report quantity 'cri-RI-CQI', for the case where non-PMI port indication is to be provided/configured, non-PMI-PortIndication should be configured for each sub-configuration.

## On dropping the CSI report containing multiple CSIs

In RAN1#114bis, the following aspect has been on the table for further discussion.

For RAN1#115: Further discuss in the next meeting for the following case: A UE configured with CSI report configuration with e.g. two sub-configurations (sub-config#1 and sub-config#2), each with e.g. two CSI-RS resources, none of the CSI-RS Tx occasions of sub-config#1 meet the CSI reference resource, i.e. they are later than CSI ref. resource, the UE shall - Alt 1: report the CSI report, according to current spec - Alt 2: report the CSI sub-report#2 only and drop the CSI sub-report#1 only - Alt 3: drop the entire CSI report

In our view, as a simple approach, a UE reports a CSI report containing one or more CSIs for different sub-configurations only after receiving at least one CSI-RS transmission occasion for channel measurement corresponding to each of the sub-configurations no later than CSI reference resource. Otherwise, the UE drops the (whole) CSI report. Hence, Alt.3 would be preferrable.

Proposal 4: For a CSI report corresponding to a CSI configuration including a list of sub-configurations:

- The UE reports a CSI report containing one or more CSIs for different sub-configurations only after receiving at least one CSI-RS transmission occasion for channel measurement corresponding to each of the sub-configurations no later than CSI reference resource.

- Otherwise, support Alt.3, i.e., the UE drops the (whole) CSI report.

## Counting of active CSI-RS resources and ports

On the counting of active CSI-RS resources and antenna ports, there is the following discussion point from RAN1#114bis:

For RAN1#115 (Companies are encouraged to study and be ready to make decision in the next meeting) For a CSI report configuration containing sub-configuration(s), if a CSI-RS resource is referred by M sub-configurations among X sub-configurations, the CSI-RS resource is counted M times and CSI-RS ports within the CSI-RS resource are counted by agreed in previous meeting, and - X=N for AP-CSI-RS resources - [X=L for P-CSI-RS resources] - FFS: X= N or L for SP-CSI-RS resources - FFS: X= N or L for SP-CSI report - Support following UE capability parameters for NES: - simultaneous ports at least for per CC - simultaneous resources at least for per CC

In our view, it’s reasonable to consider the dependency of X on the CSI reporting type rather than the CSI-RS resource type. This is because, what really counts for the counting of active CSI-RS resources and ports is the actual sub-configurations the UE is considering for measurements.

Based on the above observation, for aperiodic and semi-persistent CSI reporting, X should be equal to the number (N) of triggered/active sub-configurations, given that the UE is only measuring the CSI-RS resources and ports corresponding to those sub-configurations. On the other side, for periodic CSI reporting, X=L since the L configured sub-configurations are considered for measurements.

We think that the above approach provides a good trade-off so that the capabilities (e.g., per CC) with respect to active CSI-RS resources and ports are not quickly exhausted – and thus to make the Rel-18 NES feature (on SD and PD adaptations) more useful and less restrictive.

Proposal 5: For the purpose of counting of active CSI-RS resource and ports, for a CSI report configuration containing one or more sub-configurations and where a CSI-RS resource is referred by M sub-configurations among X sub-configurations, adopt the following:

- X = N for aperiodic and semi-persistent CSI reporting.

- X = L for periodic CSI reporting.

## On dropping of Part 1 CSIs and Part 2 WB CSIs

In RAN1#114bis, the following aspect has been on the table for further discussion.

For RAN1#115 - Further check whether there is any issue according to the current specification that, for the CSI mapping of a CSI report configuration having L sub-configurations, Part 2 wideband CSIs have the same priority and are dropped per sub-configuration level in the ascending order of sub-configuration index. - Further check whether part 1 CSI can already be dropped in legacy for a CSI report configuration, and if so, then Part 1 CSIs are dropped per sub-configuration level, in the ascending order of sub-configuration index. - Companies are encouraged to provide TPs for necessary changes

Similar to Part 2 WB CSIs, Part 1 CSIs for different CSI reports are jointly multiplexed (and could be seen as corresponding to a same priority). In legacy, for CSI reporting configured on PUCCH, the UE may drop a number of Part 1 CSIs if the PUCCH resource cannot carry the entire multiplexed/scheduled UCI(s), where the UE selects a number of Part 1 CSIs to drop/ report.

Considering a CSI report with multiple CSIs for different sub-configurations, the question is whether to support and introduce dropping of Part 1 CSI on a sub-configuration level. In our view, we don’t see any strong reason to support such dropping, especially that dropping is rather a corner/emergency case and no need to optimize for such a case particularly since we are in the maintenance phase of Rel-18. Put differently, dropping of Part 1 CSI, if needed, should be kept on a CSI report level as in legacy, i.e. all Part 1 CSIs for such report shall be dropped.

Proposal 6: For a CSI report corresponding to a CSI configuration including a list of sub-configurations, do not support Part 1 CSI dropping on per sub-configuration level.

Regarding the issue on whether or not Part 2 WB CSI dropping can be performed on a per sub-configuration level in the ascending order of sub-configuration index, in legacy the Part 2 WB CSIs of all CSI reports correspond to the same priority level and are jointly multiplexed, where it is different from Part 2 even/odd SB CSIs that each CSI report correspond to a given priority level, and the Part 2 even/odd SB CSI reports with different priority level are mapped to the UCI bit sequence one by one based on the ascending order of priority index. For Rel18 NES, it has been agreed that multiple CSIs (of multiple sub-configurations) can be contained within a CSI report. An illustration of Part 2 WB CSIs and Part 2 (even/odd) SB CSIs for such CSI report along with other CSI reports is shown in Figure 1 below.

Figure 1.

Moreover, about the omission/dropping issue of Part 2 CSI report, the following agreement was made in RAN1#114 meeting:

Agreement​: For CSIs across multiple sub-configurations in one CSI reportConfig map different sub-configurations based on RAN1#114 agreement in 9.7.1​ - For Part 2 priority reporting level​ - Option 1: for a given band type from {wideband, even subband, odd subband}, the omission order follows the priority order determined by sub-configuration index

To our view, the early agreement agreed above is not very clear on how the omission/dropping should be handled especially for Part 2 WB CSI report containing sub-configurations. In legacy approach, the omission/dropping is performed per priority level for both Part 2 WB CSI report, meaning that all the CSI reports with Part 2 WB CSI report in Priority 0 are dropped simultaneously, and there were no specific rules specified relate to the Part 2 WB CSI report dropping on how a given CSI reports within Priority 0 should be dropped.

Observation 1: In legacy, the omission/dropping for Part 2 WB CSI report is performed per priority level, meaning that all Part 2 WB CSIs of the CSI reports, which are of Priority 0, are dropped simultaneously.

Observation 2: In legacy, there were no specific rules specified related to the Part 2 WB CSI report dropping on how a given CSI reports within Priority 0 should be dropped.

Now considering the Part 2 WB CSI report dropping for Rel18 NES containing sub-configurations, to our view, at this late stage of the maintenance phase, the omission/dropping per priority level for Part 2 WB CSI report should be maintained as legacy approach, where as shown in Figure 1, the contents of Priority 0 is multiplexed of legacy CSI reports and new NES CSI reports containing sub-configurations, and there can be new open issues if the omission/dropping for Part 2 WB CSI report is performed per sub-configuration level.

Thus, based on the above thinking, we believe that we should not deviate from the legacy design by now allowing Part 2 WB CSI dropping on a sub-configuration level, especially that dropping of Part 2 WB is rather a corner/emergency case, and thus we should no optimize for it. This legacy behavior should be kept, even if this requires reverting a previous RAN1 agreement.

Proposal 7: For a CSI report corresponding to a CSI configuration including a list of sub-configurations, do not support Part 2 WB CSI dropping on a sub-configuration level, i.e., all Part 2 WB CSs are dropped simultaneously as in legacy.

# Conclusions

In this contribution, we have the following observations and proposals:

Proposal 1: For CRI reporting for Type 1/Type 2 SD adaptation, support the following legacy behavior but now per CSI report sub-configuration as follows:

- If a sub-configuration includes or is associated with more than one CSI-RS resource (for channel measurements), then CRI selection and thus reporting is needed for that sub-configuration.

- Otherwise, if a sub-configuration includes or is associated with a single CSI-RS resource (for channel measurements), then CRI reporting is not needed for that sub-configuration.

Proposal 2: For Type 1 SD adaptation (with or without PD adaptation) with a CSI report configuration configured with a list of sub-configurations, if the UE is configured with the report quantity 'cri-RI-i1-CQI', typeISinglePanel-codebookSubsetRestriction-i2 should be configured in each sub-configuration.

Proposal 3: For Type 1 SD adaptation (with or without PD adaptation) with a CSI report configuration configured with a list of sub-configurations, if the UE is configured with the report quantity 'cri-RI-CQI', for the case where non-PMI port indication is to be provided/configured, non-PMI-PortIndication should be configured for each sub-configuration.

Proposal 4: For a CSI report corresponding to a CSI configuration including a list of sub-configurations:

- The UE reports a CSI report containing one or more CSIs for different sub-configurations only after receiving at least one CSI-RS transmission occasion for channel measurement corresponding to each of the sub-configurations no later than CSI reference resource.

- Otherwise, support Alt.3, i.e., the UE drops the (whole) CSI report.

Proposal 5: For the purpose of counting of active CSI-RS resource and ports, for a CSI report configuration containing one or more sub-configurations and where a CSI-RS resource is referred by M sub-configurations among X sub-configurations, adopt the following:

- X = N for aperiodic and semi-persistent CSI reporting

- X = L for periodic CSI reporting

Proposal 6: For a CSI report corresponding to a CSI configuration including a list of sub-configurations, do not support Part 1 CSI dropping on per sub-configuration level.

Observation 1: In legacy, the omission/dropping for Part 2 WB CSI report is performed per priority level, meaning that all Part 2 WB CSIs of the CSI reports, which are of Priority 0, are dropped simultaneously.

Observation 2: In legacy, there were no specific rules specified related to the Part 2 WB CSI report dropping on how a given CSI reports within Priority 0 should be dropped.

Proposal 7: For a CSI report corresponding to a CSI configuration including a list of sub-configurations, do not support Part 2 WB CSI dropping on a sub-configuration level, i.e., all Part 2 WB CSs are dropped simultaneously as in legacy.

# References

- R1-2310986, “Remaining issues for enhancements on cell DTX/DRX mechanism”, Nokia, Nokia Shanghai Bell, November 13th – 17th, 2023.