3GPP TSG RAN WG1 #115          													                     R1-2311136

Chicago, USA, November 13th – November 17th, 2023

Source: 	Intel Corporation

Title:	Discussion on UE features for NR coverage enhancement

Agenda item:	8.16.8

Document for:	Discussion and Decision

# Introduction

At the RAN1#114bis meeting, the following agreements were made regarding UE features for NR coverage enhancement [1].

Agreement

- Component of FG 54-1 is confirmed as: Support of multiple PRACH transmissions with the same Tx spatial filter. Support {2, 4, 8} for the number of multiple PRACH transmissions with the same Tx spatial filter.

- The column of “Consequence if the feature is not supported by the UE” in FG 54-1 is confirmed as: UE doesn’t support multiple PRACH transmissions with the same Tx spatial filter

Agreement

- The column of “Need for the gNB to know if the feature is supported” for FG54-1 is confirmed as “Yes”

Agreement

- Add a note in FG 54-3: If UE supporting this FG supports FG 11-1, the UE supports FG 54-3 with DCI format 0_2

- Delete “FFS whether to separate this FG for DCI 0_1/0_2[/0_3]” from FG 54-3

Agreement

- No prerequisite FG is defined for FG 54-3

Agreement

- FG 54-3a is updated as follows

| 54. NR_cov_enh2 | 54-3a | PHR enhancement for dynamic waveform switching | Reporting of power headroom information for an assumed PUSCH using target waveform different from waveform of actual PUSCH FFS details | 54-3 | Yes | N/A |  |  |  |  |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

In the contribution, we discuss UE features for NR coverage enhancement. The updated UE features for Rel-18 NR coverage enhancement after RAN1#114bis meeting was captured in [2] and are listed in the Appendix.

# Discussion on UE features for NR coverage enhancement

## UE features for PRACH coverage enhancement

According to RAN2 agreement, RAN2 assumes that a separate UE capability for CFRA with MSG1 repetition is not needed. Following the decision from RAN2, Table 1 illustrates the suggested update for UE feature group for PRACH coverage enhancement.

Table 1. UE feature group for PRACH coverage enhancement

| Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- |
| 54-1 | PRACH coverage enhancements | Support of multiple PRACH transmissions with the same Tx spatial filter for CBRA and CFRA. Support {2, 4, 8} for the number of multiple PRACH transmissions with same Tx spatial filter. FFS whether to separate this FG for CBRA and CFRA |  |

Proposal 1

- For UE feature group of PRACH coverage enhancement,

  - Consider Table 1 for UE feature group of PRACH coverage enhancement.

  - UE feature for multiple PRACH transmissions with same Tx beam is defined per UE.

  - FDD/TDD and FR1/FR2 differentiation is not necessary.

## UE features for dynamic waveform switching

For dynamic waveform switching between DFT-s-OFDM and CP-OFDM waveform, it was agreed that this feature can be applied for the PUSCH which is scheduled by DCI format 0_1 or 0_2. Further, it was agreed that dynamic waveform switching can be applicable for the multi-PUSCH scheduling. However, RAN1 has not concluded whether dynamic waveform switching is applicable for PUSCH transmission which is scheduled by DCI format 0_3, i.e., multi-cell scheduling.

In our view, it is not clear the need to define separate FGs for all different cases, e.g., when dynamic waveform switching is applied for single-PUSCH or multi-PUSCH scheduling, or single-cell or multi-cell scheduling. In particular, a single FG for dynamic waveform switching would be sufficient.

Based on the discussions above, Table 2 illustrates suggested update for UE feature groups for dynamic waveform switching.

Table 2. UE feature groups for dynamic waveform switching

| Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- |
| 54-3 | Dynamic waveform switching | Support of dynamic waveform switching for DCI format 0_1/0_2[/0_3]. FFS whether to separate this FG for multi-PUSCH scheduling FFS whether/how to separate this FG for single-carrier case and multiple-carrier case |  |
| 54-3a | PHR enhancement for dynamic waveform switching | Reporting of power headroom information for an assumed PUSCH using target waveform different from waveform of actual PUSCH | 54-3 |

Proposal 2

- For UE feature groups of dynamic waveform switching

  - Consider Table 2 for UE feature groups of dynamic waveform switching.

  - UE features for dynamic waveform switching are defined per UE.

  - FDD/TDD and FR1/FR2 differentiation is not necessary.

# Conclusions

In this contribution, we discussed UE features for Re-18 NR coverage enhancement. Further, we summarize the proposals as follows:

Proposal 1

- For UE feature group of PRACH coverage enhancement,

  - Consider Table 1 for UE feature group of PRACH coverage enhancement.

  - UE feature for multiple PRACH transmissions with same Tx beam is defined per UE.

  - FDD/TDD and FR1/FR2 differentiation is not necessary.

Proposal 2

- For UE feature groups of dynamic waveform switching

  - Consider Table 2 for UE feature groups of dynamic waveform switching.

  - UE features for dynamic waveform switching are defined per UE.

  - FDD/TDD and FR1/FR2 differentiation is not necessary.

# References

- Chairman’s notes, RAN1 #114bis Meeting, October 2023

- R1-2310635, “Updated RAN1 UE features list for Rel-18 NR after RAN1#114bis”, October 2023

# Appendix:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 54. NR_cov_enh2 | 54-1 | PRACH coverage enhancements | Support of multiple PRACH transmissions with the same Tx spatial filter. Support {2, 4, 8} for the number of multiple PRACH transmissions with same Tx spatial filter. FFS whether to separate this FG for CBRA and CFRA |  | Yes |  | UE doesn’t support multiple PRACH transmissions with the same Tx spatial filter. |  | N/A | N/A | N/A |  | Optional with capability signalling. |
| 54. NR_cov_enh2 | 54-3 | Dynamic waveform switching | Support of dynamic waveform switching for DCI format 0_1/0_2[/0_3]. FFS whether to separate this FG for multi-PUSCH scheduling FFS whether/how to separate this FG for single-carrier case and multiple-carrier case |  | Yes |  | Dynamic waveform switching is not supported | FFS | N/A | N/A | N/A | If UE supporting this FG supports FG 11-1, the UE supports FG 54-3 with DCI format 0_2 | Optional with capability signaling. |
| 54. NR_cov_enh2 | 54-3a | PHR enhancement for dynamic waveform switching | Reporting of power headroom information for an assumed PUSCH using target waveform different from waveform of actual PUSCH | 54-3 | Yes | N/A |  |  |  |  |  |  | Optional with capability signaling. |