3GPP TSG RAN WG1 #115		R1-2311240

Chicago, USA, November 13th – November 17th, 2023

Source:	OPPO

Title:                     Remaining issues on SL positioning reference signal

Agenda Item:       8.3.1.1

Document for:	Discussion and Decision

Introduction

The study on expanded and improved NR positioning had been completed in RAN1#111, and the item was converted into work item in RAN#98 with following objectives on SL positioning[1]:

- Specify solutions for support of SL positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]:

  - Specify SL PRS for support of SL positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1].

    - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum.

    - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified.

  - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2].

  - Specify support of resource allocation for SL PRS:

    - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1].

      - For resource allocation mechanism for SL PRS in Scheme 2:

        - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1].

        - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1].

    - Support resource allocation for shared resource pool with Rel-16/17/18 SL communication and dedicated resource pool for SL PRS [RAN1].

      - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 SL communication, backward compatibility with legacy Rel-16/17 UEs should be ensured.

  - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1].

  - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2].

  - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]:

    - Specify the protocol and procedures for SL positioning between UEs (Protocol for SL positioning procedures (SLPP)).

    - Specify the protocol and procedures for SL positioning between UEs and LMF.

  - Specify signalling to NG-RAN for SL positioning and ranging service authorizations as needed. [RAN3, RAN2]

  - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

In RAN1#114 meeting the item of Expanded and Improved NR Positioning was completed, in this paper we discuss and give our views on some remaining issues on SL PRS.

Discussion

There is one issue for FFS in the following agreement achieved in RAN1#114[2]. Depending on the resource pool bandwidth and PSCCH resource configuration, it may happen that only one SL PRS resource is configured in the resource pool and the configured bandwidth of PSCCH is same as that of the resource pool, and also same as the SL PRS resource. In this case, as symbol-level transmission power of PSCCH is same as associated SL PRS, no AGC symbol is necessary in between the PSCCH and SL PRS.

In the last meeting this issue was discussed[3], and overwhelming majority of companies prefer not to introduce such a new rule in addition to the one agreed in RAN1#113 for this rare configuration. Given that, any SL PRS resource in Rel-18 is supposed to be immediately preceded by an AGC symbol. However, there is no clear agreement so far to prevent a configuration of allocating more than 1 symbols before a SL PRS resource, as shown in Figure 1 below, such an agreement is necessary to avoid potential ambiguity.

Agreement For a dedicated resource pool, explicit (pre-)configuration of SL PRS resources in a slot includes: - SL PRS Resource ID, (M, N) pattern, starting symbol, comb offset. - FFS: constraints to the (pre-)configuration to address potential AGC issues.

Proposal 1: For dedicated resource pool, the AGC symbol for a SL PRS resource is immediately preceded by the PSCCH symbol or another SL PRS resource.

Furthermore, if multiple FDM-ed resource pools are (pre-)configured on the SL BWP, (pre-)configuration should guarantee that starting symbol and number of contiguous OFDM symbols for any SL PRS resource should be aligned across the FDM-ed resource pools. Otherwise, additional AGC operation may be needed for a receiver in the middle of a SL PRS resource. This issue was discussed in the last meeting as well and majority companies supported to make a conclusion. But there were companies prefer not to make such a conclusion as the network can address the issue by proper configuration. From our perspective, as all companies believe the alignment should be there, a conclusion would be helpful for people to understand RAN1’s assumption.

Proposal 2: Make the following as a conclusion: It is RAN1’s understanding that, depending on (pre-)configuration, location(s) for AGC symbol(s) and starting symbol(s) and duration(s) for SL PRS resource(s) and any other SL physical signals or channels in a slot are aligned across all FDM-ed resource pools on a SL BWP.

In the last meeting it was also discussed to indicate in SCI 2-D whether the same antenna port is applied for SL-PRS and PSSCH DMRS for Rx UE joint processing. In general, bandwidth and sequence of PSSCH DMRS and SL PRS are the same in shared resource pool, PSSCH DMRS could be used as SL PRS to improve the accuracy, and even positioning related measurement can be based only on PSSCH DMRS. If companies prefer not to change SCI 2-D at this stage, SLPP layer signalling can be considered for the QCL indication.

Proposal 3: Support to indicate whether the same antenna port is applied for SL-PRS and PSSCH DMRS via SCI 2-D or SLPP signalling.

Conclusion

In this contribution, we discussed the remaining issues on SL PRS design for SL positioning, we have following proposals:

Proposal 1: For dedicated resource pool, the AGC symbol for a SL PRS resource is immediately preceded by the PSCCH symbol or another SL PRS resource.

Proposal 2: Make the following as a conclusion: It is RAN1’s understanding that, depending on (pre-)configuration, location(s) for AGC symbol(s) and starting symbol(s) and duration(s) for SL PRS resource(s) and any other SL physical signals or channels in a slot are aligned across all FDM-ed resource pools on a SL BWP.

Proposal 3: Support to indicate whether the same antenna port is applied for SL-PRS and PSSCH DMRS via SCI 2-D or SLPP signalling.

References

- RP-230328, Revised WID on Expanded and Improved NR Positioning.

- RAN1 Chairman’s Notes, RAN1#114, August 2023.

- R1-2310334, FL summary #2 on SL positioning reference signal.