3GPP TSG RAN WG1 #115		R1-2311244

Chicago, USA, November 13th - November 17th, 2023

Source:	OPPO

Title:	Discussion on remaining issue for enhancements on cell DTX/DRX mechanism

Agenda Item:	8.5.2

Document for:	Discussion and Decision

Introduction

In this contribution, we provide our views on remaining issue for enhancements on cell DTX/DRX mechanism for NR network energy saving.

Discussion

The following two alternatives of UE behaviour related to DCI format 2-9 monitoring during non-active periods of C-DRX were discussed:

- Alt 1) UE does not monitor DCI format 2-9 during non-active periods of C-DRX

- Alt 2) UE is expected to monitor DCI format 2-9 during non-active periods of C-DRX

In legacy UE C-DRX operation, UE is not expected to monitor a DCI format transmitted in Type-3 CSS during non-active periods of C-DRX for UE power saving purpose. If both cell DTX/DRX and UE C-DRX are configured, and the cell DTX/DRX configuration is activated/deactivated via DCI format 2_9, since DCI format 2_9 is also transmitted through Type-3 CSS, the UE shall not monitor DCI format 2_9 during non-active periods of C-DRX following legacy behaviour, to avoid increasing UE power consumption.

Proposal 1: UE does not monitor DCI format 2-9 during non-active periods of C-DRX.

In previous RAN2 meetings #121bis and #122, the following agreements were made.

Agreements:

- As baseline, UE does not transmit on CG occasions during Cell DRX non-active periods

- As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, e.g. SR transmissions are dropped during the non-active period

- When an DG grant is received, by the gNB during cell DRX/DTX, the UE follows the grant assignment (i.e. like in legacy).  This includes DL HARQ feedback.

In RAN1 meeting #114, the following conclusions regarding HARQ-ACK feedback were made.

Conclusion: - HARQ-ACK of SPS PDSCH transmitted is not impacted by non-active period of cell DRX. - Note: HARQ-ACK of SPS PDSCH not received due to non-active period of cell DTX is impacted. Conclusion - The following channels are not impacted by non-active period of cell DRX - HARQ-ACK of a DCI format without scheduling a PDSCH

Based on the above agreements and conclusions, for a connected mode UE during the non-active periods of a cell DRX, it is expected to transmit HARQ-ACK corresponding to a DCI format without scheduling a PDSCH, SPS PDSCH, or PDSCH scheduled by a DCI format, and it is not expected to transmit CG-PUSCH. If the resources allocated for HARQ-ACK and CG-PUSCH transmission are overlapped in a slot belongs to non-active period of cell DRX activated by RRC signaling, the UE is expected to transmit PUCCH carrying the HARQ-ACK and not transmit the CG-PUSCH.

However, if UE is prepared to multiplex HARQ-ACK information on CG-PUSCH on an uplink slot, and the uplink slot is expected to belong to a non-active period of a cell DRX which is activated by a DCI format 2_9 and the UE does not receive this DCI format 2_9 correctly, then the UE would transmit CG-PUSCH multiplexed with HARQ-ACK, while the gNB would be expected to receive PUCCH on the uplink slot. To avoid this misunderstanding, it is proposed to restrict UE behavior, i.e., UE is not expected to multiplex HARQ-ACK information on CG-PUSCH if the UE is configured with cell DRX activation/deactivation via DCI format 2_9.

Proposal 2: When cell DRX is configured only by RRC, UE is not expected to multiplex HARQ-ACK information on CG-PUSCH during non-active period of cell DRX; When cell DRX activation/deactivation by DCI format 2_9 is configured, UE shall always not multiplex HARQ-ACK information on CG-PUSCH.

The following draft TP is proposed to reflect proposal 2.

------------ start of TP for TS 38.213-----------------------

9	UE procedure for reporting control information

<unchanged parts are omitted>

A DCI format indicating a SPS PDSCH release, or SCell dormancy without scheduling a PDSCH reception, or indicating a TCI state update without scheduling PDSCH reception, is referred to as a DCI format having associated HARQ-ACK information without scheduling a PDSCH reception.

When a UE determines overlapping for PUCCH and/or PUSCH transmissions, the UE excludes CG PUSCH transmissions and PUSCH transmissions with SP-CSI if the UE is configured to monitor PDCCH for DCI format 2_9; otherwise, the UE excludes CG PUSCH transmissions and PUSCH transmissions with SP-CSI overlapping with non-active periods of cell DRX.

<unchanged parts are omitted>

------------ end of TP for TS 38.213 -----------------------

Another issue is that if slot aggregation/repetition transmission is configured, the UE behaviour is not clear when resource pre-configured for SPS-PDSCH/CG-PUSCH transmission and resource belongs to non-active period of cell DTX/DRX are partially overlapped. In our view, the UE at least shall not receive/transmit SPS-PDSCH/CG-PUSCH during non-active period of cell DTX/DRX for energy saving purpose, and considering that partial transmission may not satisfy the demodulation requirement, the UE shall not receive/transmit the other part of SPS-PDSCH/CG-PUSCH either. It is proposed to not receive/transmit SPS-PDSCH/CG-PUSCH when resource pre-configured for SPS-PDSCH/CG-PUSCH transmission and resource belongs to non-active period of cell DTX/DRX are partially overlapped.

Proposal 3: UE shall not receive/transmit SPS-PDSCH/CG-PUSCH when resource pre-configured for SPS-PDSCH/CG-PUSCH transmission and resource belongs to non-active period of cell DTX/DRX are partially overlapped.

Conclusion

In this contribution, we provide our views on the potential enhancements on cell DTX/DRX mechanism for NR network energy saving. The following proposals are provided.

Proposal 1: UE does not monitor DCI format 2-9 during non-active periods of C-DRX.

Proposal 2: When cell DRX is configured only by RRC, UE is not expected to multiplex HARQ-ACK information on CG-PUSCH during non-active period of cell DRX; When cell DRX activation/deactivation by DCI format 2_9 is configured, UE shall always not multiplex HARQ-ACK information on CG-PUSCH.

Proposal 3: UE shall not receive/transmit SPS-PDSCH/CG-PUSCH when resource pre-configured for SPS-PDSCH/CG-PUSCH transmission and resource belongs to non-active period of cell DTX/DRX are partially overlapped.

The following draft TP is proposed to reflect proposal 2.

------------ start of TP for TS 38.213-----------------------

9	UE procedure for reporting control information

<unchanged parts are omitted>

A DCI format indicating a SPS PDSCH release, or SCell dormancy without scheduling a PDSCH reception, or indicating a TCI state update without scheduling PDSCH reception, is referred to as a DCI format having associated HARQ-ACK information without scheduling a PDSCH reception.

When a UE determines overlapping for PUCCH and/or PUSCH transmissions, the UE excludes CG PUSCH transmissions and PUSCH transmissions with SP-CSI if the UE is configured to monitor PDCCH for DCI format 2_9; otherwise, the UE excludes CG PUSCH transmissions and PUSCH transmissions with SP-CSI overlapping with non-active periods of cell DRX.

<unchanged parts are omitted>

------------ end of TP for TS 38.213 -----------------------

Reference

- R1-2309597, Discussion on remaining issue for enhancements on cell DTX/DRX mechanism, OPPO.

- R1-2310454, Discussion summary #5 for enhancements on cell DTX/DRX mechanism, Moderator (Intel Corporation).