3GPP TSG RAN WG1 #115	R1-2311612

Chicago, USA, November 13th – November 17th, 2023

Source:	NTT DOCOMO, INC

Title:	Remaining issues on unified TCI framework extension for multi-TRP

Agenda Item:	8.1.1.1

Document for: 	Discussion and Decision

- Introduction

In RAN#94-e meeting, Rel-18 WID on MIMO [1] was agreed. From 7 objectives, there are three objectives for extension of unified TCI framework, as shown below.

- Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework. […] - Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. - Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed.

In this contribution, we discuss unified TCI framework related enhancements.

- Unified TCI framework extension for multi-TRP (M-TRP)

  - Beam Application Timing (BAT) for mDCI mTRP

In RAN1#114bis in AI 7.2, the following agreement was made for BAT for Rel.17 unified TCI framework. Based on the Rel.18 CR (R1-2310669), if multiple HARQ-ACK bits are transmitted on a PUCCH/PUSCH, the indicated TCI state associated with the last DCI with positive HARQ-ACK value is applied.

Agreement (RAN1#114bis in AI7.2) The following TP is endorsed in principle. Final CRs in R1-2310668 (Rel-17) and R1-2310669 (Rel-18). 5.1.5 Antenna ports quasi co-location <Unchanged part omitted> When a UE configured with dl-OrJointTCI-StateList would transmit a PUCCH with positive HARQ-ACK or a PUSCH with positive HARQ-ACK corresponding to the DCI carrying the TCI State indication and without DL assignment, or corresponding to the PDSCH scheduled by the DCI carrying the TCI State indication, and if the indicated TCI State is different from the previously indicated one, the indicated TCI-State and/or TCI-UL-State should be applied starting from the first slot that is at least symbols after the last symbol of the PUCCH or the PUSCH, and if the UE receives more than one indicated TCI state for a CC/BWP to be applied starting from the first slot that is at least symbols after the last symbol of the PUCCH or the PUSCH, the indicated TCI state carried in the latest DCI in time corresponding to positive HARQ-ACK value is applied. The first slot and the symbols are both determined on the active BWP with the smallest SCS among the BWP(s) from the CCs applying the indicated TCI-State or TCI-UL-State that are active at the end of the PUCCH or the PUSCH carrying the positive HARQ-ACK. <Unchanged part omitted>

However, the agreed Rel.18 CR (R1-2310669) has a problem for joint HARQ-ACK feedback for mDCI mTRP. For joint HARQ-ACK feedback for mDCI mTRP, one PUCCH/PUSCH can contain multiple HARQ-ACK bits associated with two different coresetPoolIndexes. Based on the CR, in case of joint HARQ-ACK feedback for mDCI mTRP, UE applies the indicated TCI state associated with the last DCI with positive HARQ-ACK value regardless of coresetPoolIndex. It makes cross TRP TCI indication which is not aligned with the previous agreement. For example, as shown in Figure 2-1, if one PUCCH/PUSCH contains positive HARQ-ACK for coresetPoolIndex = 1 and contains negative HARQ-ACK for coresetPoolIndex = 0, the agreed CR says TCI#2 is applied to both of coresetPoolIndexes. In this case, we believe there should be no need to update indicated TCI for coresetPoolIndex = 0, based on the previous agreement.

Figure 2-1: Issue of BAT for joint HARQ-ACK feedback for mDCI mTRP.

Hence, we suggest clarifying that specification of BAT is determined per each coresetPoolIndex for mDCI mTRP.

Proposal 2-1:

  - For mDCI mTRP, the BAT is determined per coresetPoolIndex for mDCI mTRP.

  - Adopt the following TP for TS 38.214 v18.0.0.

    - Reason for change: In Rel.18 CR (R1-2310669), if multiple HARQ-ACK bits are transmitted on a PUCCH/PUSCH, the indicated TCI state associated with the last DCI with positive HARQ-ACK value is applied regardless of coresetPoolIndex. However, for joint HARQ-ACK feedback for mDCI mTRP, one PUCCH/PUSCH contains multiple HARQ-ACK bits associated with two different coresetPoolIndexes. Hence, the current Rel.18 spec. results in cross TRP TCI indication in case of joint HARQ-ACK feedback for mDCI mTRP, which is not aligned with the previous agreement.

    - Summary of change: BAT is determined per each coresetPoolIndex for mDCI mTRP.

    - Consequence if not approved: The specification is not aligned with previous agreement.

5.1.5 Antenna ports quasi co-location ## *** Unchanged parts are omitted *** When a UE configured with dl-OrJointTCI-StateList and is not configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet, the UE would transmit a PUCCH with positive HARQ-ACK or a PUSCH with positive HARQ-ACK corresponding to the DCI carrying the TCI State indication and without DL assignment, or corresponding to the PDSCH scheduled by the DCI carrying the TCI State indication, and if the indicated TCI State(s) is/are different from the previously indicated one(s), the indicated TCI-State(s) and/or TCI-UL-State(s) should be applied starting from the first slot that is at least beamAppTime symbols after the last symbol of the PUCCH or the PUSCH, and if the UE receives more than one indicated TCI state for a CC/BWP to be applied starting from the first slot that is at least beamAppTime symbols after the last symbol of the PUCCH or the PUSCH, the indicated TCI state carried in the latest DCI in time corresponding to positive HARQ-ACK value is applied. The first slot and the beamAppTime symbols are both determined on the active BWP with the smallest SCS among the BWP(s) from the CCs applying the indicated TCI-State(s) or TCI-UL-State(s) that are active at the end of the PUCCH or the PUSCH carrying the positive HARQ-ACK. When a UE configured with dl-OrJointTCI-StateList and is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet, the UE would transmit a PUCCH with positive HARQ-ACK or a PUSCH with positive HARQ-ACK corresponding to the DCI carrying the TCI State indication specific to a coresetPoolIndex value and without DL assignment, or corresponding to the PDSCH scheduled by the DCI carrying the TCI State indication specific to a coresetPoolIndex value, and if the indicated TCI State(s) specific to the coresetPoolIndex value is/are different from the previously indicated one(s), the indicated TCI-State(s) and/or TCI-UL-State(s) specific to the coresetPoolIndex value should be applied starting from the first slot that is at least beamAppTime symbols after the last symbol of the PUCCH or the PUSCH, and if the UE receives more than one indicated TCI state specific to the coresetPoolIndex value for a CC/BWP to be applied starting from the first slot that is at least beamAppTime symbols after the last symbol of the PUCCH or the PUSCH, the indicated TCI state specific to the coresetPoolIndex value carried in the latest DCI in time corresponding to positive HARQ-ACK value is applied. The first slot and the beamAppTime symbols are both determined on the active BWP with the smallest SCS among the BWP(s) from the CCs applying the indicated TCI-State(s) or TCI-UL-State(s) specific to the coresetPoolIndex value that are active at the end of the PUCCH or the PUSCH carrying the positive HARQ-ACK. *** Unchanged parts are omitted ***

  - Single-DCI based M-TRP

- Switching between one indicated TCI state and two indicated TCI states

In RAN1#114, whether to support MAC CE based dynamic switching between sTRP and mTRP was discussed. Some companies claimed the MAC CE based dynamic switching is not necessary because DCI based switching by TCI state selection field is already supported. In our view, we assume DCI based switching by TCI state selection field will be optional UE feature, and MAC CE based beam switching is necessary when TCI state selection field is not configured in DCI format 1_1/1_2.

In RAN1#114, the following options were discussed how to enable switching between one indicated TCI state and two indicated TCI states.

- Alt1: - A CC/BWP is operated in Rel-18 unified TCI framework extension for S-DCI based MTRP if Rel-18 TCI state activation command (MAC-CE) where at least one activated TCI codepoint is mapped with both the first and second join/DL/UL TCI states is received and applied to the CC/BWP - A CC/BWP is operated in Rel-17 unified TCI framework if Rel-18 TCI state activation command (MAC-CE) where all activated TCI codepoint(s) is mapped with either only the first joint/DL/UL TCI state(s) or only the second joint/DL/UL TCI state(s) is received and applied to the CC/BWP - A CC/BWP is operated in Rel-17 unified TCI framework if Rel-17 TCI state activation command (MAC-CE) is received and applied to the CC/BWP - Alt2: - A CC/BWP is operated in Rel-18 unified TCI framework extension for S-DCI based MTRP if Rel-18 TCI state activation command (MAC-CE) is received and applied to the CC/BWP - A CC/BWP is operated in Rel-17 unified TCI framework if Rel-17 TCI state activation command (MAC-CE) is received and applied to the CC/BWP - Alt3: Introduce a field in Rel-18 TCI state activation command (MAC-CE) to explicitly indicate that a CC/BWP is operated in Rel-17 unified TCI framework or Rel-18 unified TCI framework extension for S-DCI based MTRP

To our understanding, the difference is summarized below.

- Alt.1:

- If at least one TCI codepoint mapped with more than one TCI states in Rel.18 MAC CE, it means mTRP operation.

- Else, it means sTRP operation.

- Alt.2:

- If Rel.18 MAC CE activates TCI states, it means mTRP operation.

- Else (Rel.17 MAC CE activates TCI states), it means sTRP operation.

- Alt.3:

- Introduce the explicit MAC CE field to indicate sTRP operation or mTRP operation.

In our view, any options work. We believe Alt.1 and Alt.2 have benefit that it does not require explicit MAC CE field to indicate sTRP or mTRP operation. Between Alt.1 or Alt.2, in previous RAN1 meetings, some companies pointed out if gNB wants to update two indicated TCI states to one indicated TCI state, that Alt.2 requires to send Rel.18 MAC CE to deactivate all TCI codepoint and send Rel.17 MAC CE to activate all TCI codepoint. On the other hand, Alt.1 requires one Rel.18 MAC CE. Hence, Alt.1 is more reasonable.

Proposal 2-2:

  - When a UE is configured with dl-OrJointTCI-StateList or TCI-UL-State, and if TCI state selection field is not configured in DCI format 1_1/1_2, support Alt.1:

    - A CC/BWP is operated in Rel-18 unified TCI framework extension for S-DCI based MTRP if Rel-18 TCI state activation command (MAC-CE) where at least one activated TCI codepoint is mapped with both the first and second join/DL/UL TCI states is received and applied to the CC/BWP

    - A CC/BWP is operated in Rel-17 unified TCI framework if Rel-18 TCI state activation command (MAC-CE) where all activated TCI codepoint(s) is mapped with either only the first joint/DL/UL TCI state(s) or only the second joint/DL/UL TCI state(s) is received and applied to the CC/BWP

    - A CC/BWP is operated in Rel-17 unified TCI framework if Rel-17 TCI state activation command (MAC-CE) is received and applied to the CC/BWP

- 2.3 TCI state indication for CJT

Based on the previous Rel.18 agreements, 1 or 2 joint/DL TCI state(s) is applied to CJT-PDSCH. Also, up to 4 CSI-RSs for CMR can be configured for CJT, and it means up to 4 different joint/DL TCI states can be configured to up to 4 CMRs for CJT. In our view, it is not proper to configure totally different joint/DL TCI state(s) for CJT-PDSCH and CMR(s) for CJT, thus, restriction of joint/DL TCI state(s) between CJT-PDSCH and CMR(s) for CJT is necessary, e.g., indicated joint/DL TCI state(s) for CJT-PDSCH should be QCLed with at least part of joint/DL TCI state(s) configured for CSI-RS(s) as CMR(s) for CJT.

Proposal 2-3:

  - Introduce restriction that indicated joint/DL TCI state(s) for CJT-PDSCH should be QCLed with at least part of joint/DL TCI state(s) configured for CSI-RS(s) as CMR(s) for CJT.

Figure 2-1 shows examples of the proposed restriction of joint/DL TCI state(s) between CJT-PDSCH and CMR(s) for CJT.

Case1 shows the case when one joint/DL TCI state is indicated to CJT-PDSCH.

- Example 1-A), the indicated joint/DL TCI state for PDSCH should be QCLed with joint/DL TCI state of all CMRs for CJT.

- Example 1-B), the indicated joint/DL TCI state for PDSCH should be QCLed with joint/DL TCI state of some CMRs for CJT, and other joint/DL TCI states can be configured to other CMRs for CJT.

Case2 shows the case when two joint/DL TCI states are indicated to CJT-PDSCH.

- Example 2-A), one of the two indicated joint/DL TCI states for PDSCH should be QCLed with joint/DL TCI state of all CMRs for CJT.

- Example 2-B), one of the two indicated joint/DL TCI states should be QCLed with joint/DL TCI state of some CMRs for CJT, and the other indicated joint/DL TCI state should be QCLed with joint/DL TCI state of other CMRs for CJT.

Case1: If one joint/DL TCI state is indicated to CJT-PDSCH.

Case2: If two joint/DL TCI states are indicated to CJT-PDSCH.

Figure 2-1: Examples of restriction of joint/DL TCI state between CJT-PDSCH and CMR(s) for CJT.

However, to meet the proposed joint/DL TCI state restriction between PDSCH-CJT and CMR(s) for CJT, there is a problem that the same joint/DL TCI state cannot be applied to PDSCH and CSI-RS for some cases, due to the following restriction of Rel.18 unified TCI state.

- Indicated joint/DL TCI state(s) is applied to PDSCH and A-CSI-RS.

- Indicated joint/DL TCI state(s) is NOT applied to P/SP-CSI-RS. Instead, RRC configures joint/DL TCI state to P/SP-CSI-RS.

It means if CMR(s) for CJT is P/SP-CSI-RS, the indicated joint/DL TCI state(s) is applied to PDSCH but not applied to CMR(s) for CJT. The following options can be considered.

- Opt. 1) Solved by gNB implementation (i.e. gNB indicates the same TCI state to CJT-PDSCH and CSI-RS for CMR for CJT. It means, if CMR for CJT is P/SP-CSI-RS, gNB cannot update indicated TCI by MAC CE/DCI).

- Opt. 2) Only A-CSI-RS is allowed for CSI-RS for CMR for CJT.

- Opt. 3) Apply MAC CE/DCI based indicated TCI state to P/SP-CSI-RS, in case of CJT-PDSCH.

In our view, Opt.1 and Opt.2 cause gNB restriction, which are not preferred. Opt.3 was discussed in R17 unified TCI. However, it was not agreed because P/SP-CSI-RS is usually monitored by many UEs, and it is not feasible to update QCL of P/SP-CSI-RS for one UE only. To relax companies’ concern, the following can be additionally considered:

- A) Opt.3 is applicable only when CJT is configured for PDSCH.

- B) Additional support of group common DCI to update indicated joint/DL TCI state(s) to group of UEs who monitor the same P/SP-CSI-RS resource.

Proposal 2-4:

  - To indicate the same joint/DL TCI state(s) between CJT-PDSCH and CMR(s) for CJT, the following options can be considered. Opt. 3 is preferred.

    - Opt. 1) Solved by gNB implementation (i.e. gNB indicates the same TCI state to CJT-PDSCH and CSI-RS for CMR for CJT. It means, if CMR for CJT is P/SP-CSI-RS, gNB cannot update indicated TCI by MAC CE/DCI).

    - Opt. 2) Only A-CSI-RS is allowed for CSI-RS for CMR for CJT.

    - Opt. 3) Apply MAC CE/DCI based indicated TCI state to P/SP-CSI-RS, in case of CJT-PDSCH.

- 2.4 unified TCI for Type1-CG PUSCH

For Type-1 CG PUSCH, it was agreed that RRC configuration can provide the information that UE shall apply first indicated TCI or second indicated TCI or both indicated TCI states.

In STxMP, following agreement was made in the last meeting.

Agreement For single TRP operation, when single-DCI STxMP SDM or SFN is configured and two SRS resource sets for CB/NCB are configured: - If one Type-1 CG PUSCH RRC configuration contains only one SRI field and/or one TPMI field, the PUSCH transmission of this CG PUSCH is associated with the first SRS resource set if the first indicated TCI state is applied, and the PUSCH transmission of this CG PUSCH is associated with the second SRS resource set if the second indicated TCI state is applied.

Similar issue exists in Rel-17 M-TRP TDM PUSCH repetition and the above enhancement should be also applied to Rel-17 M-TRP TDM repetition. Although in Rel-17 if Type-1 CG PUSCH is sTRP operation the PUSCH transmission is associated with first SRS resource set, given the indication of applyIndicatedTCIState is introduced in Rel-18 unified TCI, it is beneficial that Type-1 CG PUSCH can be associated with either the first or second TRP according to the applyIndicatedTCIState indication.

Proposal 2.5:

- For S-DCI based M-TRP TDM PUSCH repetition, when two SRS resource sets are configured, if Type-1 CG PUSCH configuration contains only one SRI field and/or one TPMI field, the PUSCH transmission of this CG PUSCH is associated with first SRS resource set if the first indicated TCI state is applied, and the PUSCH transmission of this CG PUSCH is associated with the second SRS resource set if the second indicated TCI state is applied.

- Adopt the following TP for TS 38.214.

## 6.1.2.3 Resource allocation for uplink transmission with configured grant ## *** Unchanged parts are omitted *** When a UE is configured with dl-OrJointTCI-StateList or TCI-UL-State and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or 'noncodebook', and the higher layer parameter multipanelScheme is set to ‘SDMscheme’ or ‘SFNscheme’, and the higher layer parameter rrc-ConfiguredUplinkGrant does not contain srs-ResourceIndicator2 or precodingAndNumberOfLayers2, the PUSCH transmission occasion(s) is associated with the first SRS resource set if the first indicated TCI-States or TCI-UL-States applies and is associated with the second SRS resource set if the second indicated TCI-States or TCI-UL-States applies. ## *** Unchanged parts are omitted ***

- Power control for M-TRP operation

Following agreement was made in the last meeting but has not been captured in the draft CR of TS 38.213.

Agreement On unified TCI framework extension for S-DCI based MTRP, if twoPHRMode is configured, and two SRS resource sets for CB/NCB and multipanelScheme for SDM/SFN are configured: - If the UE determines that only one Type 1 PHR is based on an actual PUSCH transmission - If the actual PUSCH transmission applies only the first indicated joint/UL TCI state, the UE provides the second {power headroom, configured max output power} associated with the second indicated joint/UL TCI state for a reference PUSCH transmission - If the actual PUSCH transmission applies only the second indicated joint/UL TCI state, the UE provides the first {power headroom, configured max output power} associated with the first indicated joint/UL TCI state for a reference PUSCH transmission - If the UE determines that both Type 1 PHRs are based on reference PUSCH transmissions, the UE provides the first {power headroom, configured max output power} associated with the first indicated joint/UL TCI state for a reference PUSCH transmission, and the second {power headroom, configured max output power} associated with the second indicated joint/UL TCI state for another reference PUSCH transmission - FFS: Whether the configured max output power reported in above cases is per UE or per panel or both

Proposal 3.1:

  - Adopt the following TP for TS 38.213.

## 7.7.1 Type 1 PH report ## *** Unchanged parts are omitted *** If a UE is provided, for active UL BWP b of carrier f of serving cell c, - twoPHRMode, - two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', - dl-OrJointTCI-StateList or TCI-UL-State and is indicated a first TCI-State or TCI-UL-State and a second TCI-State or TCI-UL-State, and - multipanelScheme the UE provides - a first Type 1 power headroom report and a first configured maximum output power associated with the first TCI-State or TCI-UL-State for an actual PUSCH transmission using a spatial domain filter corresponding only to the first TCI-State or TCI-UL-State and a second Type 1 power headroom report and a second configured maximum output power associated with the second TCI-State or TCI-UL-State for reference PUSCH transmission - a first Type 1 power headroom report and a first configured maximum output power associate with the first TCI-State or TCI-UL-State for reference PUSCH transmission and a Type 1 power headroom report and a configured maximum output power associated with the second TCI-State or TCI-UL-State for an actual PUSCH transmission using a spatial domain filter corresponding only to the second TCI-State or TCI-UL-State - a first Type 1 power headroom report and a first configured maximum output power associated with the first TCI-State or TCI-UL-State, and a second Type 1 power headroom report and a second configured maximum output power associated with the second TCI-State or TCI-UL-State, for an actual PUSCH transmission using a spatial domain filter corresponding to the first TCI-State or TCI-UL-State and using a spatial domain filter corresponding to the second TCI-State or TCI-UL-State - a first Type 1 power headroom report and a first configured maximum output power associated with the first TCI-State or TCI-UL-State for reference PUSCH transmission and a second Type 1 power headroom report and a second configured maximum output power associated with the second TCI-State or TCI-UL-State for reference PUSCH transmission if UE determines that both PHRs are based on reference PUSCH transmissions ## *** Unchanged parts are omitted ***

For PHR of STxMP, there is an FFS on whether the configured max output power reported in above cases is per UE or per panel or both. We think it should be per panel. RAN4 have concluded that the configured transmitted power during STxMP shall be defined per indicated TCI. Meanwhile, when the above agreement was made, the motivation to report two configured max output power for reference PUSCH transmission is to let gNB aware of per panel max output power. One argument for report per UE PCMAX may be related to the assumption of total power limitation per UE over all UE panels. However, in our understanding, to ensure total power limitation per UE over all panels does not imply UE would configure a per UE PCMAX during STxMP. In contrary, we think UE may not configure a per UE PCMAX in STxMP. According to RAN4 reply LS in R4-2306657 (related text copied as below), the total power limitation per UE over all panels is ensured by the condition 1 and 2. In our understanding, UE shall set per panel PCMAX to ensure the conditions are met, and may not set a per UE PCMAX.

RAN4 would like to confirm to RAN1 the conditions to be simultaneously met by a UE for STxMP: - The total EIRP over all panels cannot exceed the existing EIRP limitation. - The total TRP over all panels cannot exceed the existing TRP limitation. - The sum of per-panel EIRP power limitation for STxMP can be greater than the existing EIRP limitation. - See condition #1, which must always be obeyed. - In cases where the beams for each panel have low enough spatial overlap, this clause allows EIRP for each panel to individually approach the existing EIRP limitation. - RAN4 has not completed spec. definition work for ‘per panel EIRP power limitation’. - No explicit ‘per-panel’ TRP limit is deemed necessary.

Proposal 3.2:

  - The configured maximum output power in PHR of STxMP is per panel.

TP was agreed in the last meeting for virtual PHR for M-TRP TDM repetition that when UE is provided twoPHRMode power control parameters associated with first/second TCI states are used for virtual PHR of first/second SRS resource set. The enhancement should also be applied for twoPHRMode for STxMP SDM/SFN scheme.

Proposal 3.3:

  - Adopt the following TP for TS 38.213. Note that the text highlighted in blue is TP proposed in proposal 3.1.

## 7.7.1 Type 1 PH report ## *** Unchanged parts are omitted *** If a UE is provided, for active UL BWP b of carrier f of serving cell c, - twoPHRMode, - two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', - dl-OrJointTCI-StateList or TCI-UL-State and is indicated a first TCI-State or TCI-UL-State and a second TCI-State or TCI-UL-State, and - multipanelScheme the UE provides - a first Type 1 power headroom report and a first configured maximum output power associated with the first TCI-State or TCI-UL-State for an actual PUSCH transmission using a spatial domain filter corresponding only to the first TCI-State or TCI-UL-State and a second Type 1 power headroom report and a second configured maximum output power associated with the second TCI-State or TCI-UL-State for reference PUSCH transmission. The UE provides the second Type 1 power headroom report for reference PUSCH transmission using the p0AlphaSetforPUSCH and pathlossReferenceRS-Id-r17 values associated with the second TCI-State or TCI-UL-State. - a first Type 1 power headroom report and a first configured maximum output power associate with the first TCI-State or TCI-UL-State for reference PUSCH transmission and a Type 1 power headroom report and a configured maximum output power associated with the second TCI-State or TCI-UL-State for an actual PUSCH transmission using a spatial domain filter corresponding only to the second TCI-State or TCI-UL-State. The UE provides the first Type 1 power headroom report for reference PUSCH transmission using the p0AlphaSetforPUSCH and pathlossReferenceRS-Id-r17 values associated with the first TCI-State or TCI-UL-State. - a first Type 1 power headroom report and a first configured maximum output power associated with the first TCI-State or TCI-UL-State, and a second Type 1 power headroom report and a second configured maximum output power associated with the second TCI-State or TCI-UL-State, for an actual PUSCH transmission using a spatial domain filter corresponding to the first TCI-State or TCI-UL-State and using a spatial domain filter corresponding to the second TCI-State or TCI-UL-State - a first Type 1 power headroom report and a first configured maximum output power associated with the first TCI-State or TCI-UL-State for reference PUSCH transmission and a second Type 1 power headroom report and a second configured maximum output power associated with the second TCI-State or TCI-UL-State for reference PUSCH transmission if UE determines that both PHRs are based on reference PUSCH transmissions. The UE provides the first Type 1 power headroom report for reference PUSCH transmission using the p0AlphaSetforPUSCH and pathlossReferenceRS-Id-r17 values associated with the first TCI-State or TCI-UL-State. The UE provides the second Type 1 power headroom report for reference PUSCH transmission using the p0AlphaSetforPUSCH and pathlossReferenceRS-Id-r17 values associated with the second TCI-State or TCI-UL-State. ## *** Unchanged parts are omitted ***

In Rel-17 unified TCI for sTRP operation, power control parameters associated with indicated TCI state are used for virtual PHR. In Rel-18, if UE is indicated two TCI states, and if M-TRP/STxMP PUSCH scheme is not enabled, or if M-TRP/STxMP PUSCH scheme is enabled but twoPHRMode is not configured, clarification is needed on whether power control parameters associated with first or second indicated TCI are used for virtual PHR.

Proposal 3.4:

  - Adopt the following TP for TS 38.213.

## 7.7.1 Type 1 PH report ## *** Unchanged parts are omitted *** If the UE determines that a Type 1 power headroom report for an activated serving cell is based on a reference PUSCH transmission then, for PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c, the UE computes the Type 1 power headroom report as PHtype1,b,f,ci,j,qd,l= PCMAX,f,c(i)-PO_PUSCH,b,f,cj+αb,f,cj⋅PLb,f,cqd+fb,f,ci,l [dB] where PCMAX,f,c(i) is computed assuming MPR=0 dB, A-MPR=0 dB, P-MPR=0 dB. TC = 0 dB. MPR, A-MPR, P-MPR and TC are defined in [8-1, TS 38.101-1], [8-2, TS 38.101-2] and [8-3, TS 38.101-3]. The remaining parameters are defined in clause 7.1.1 and, if ul-powerControl is not provided, PO_PUSCH,b,f,c(j) and αb,f,cj are obtained using PO_NOMINAL,PUSCH,f,c0 and p0-PUSCH-AlphaSetId = 0, PLb,f,c(qd) is obtained using pusch-PathlossReferenceRS-Id = 0, and l=0. If ul-powerControl is provided, PO_PUSCH,b,f,c(j), αb,f,cj and l are obtained by p0AlphaSetforPUSCH associated with the indicated TCI-State or TCI-UL-State, PLb,f,c(qd) is obtained by PL-RS associated with the indicated TCI-State or TCI-UL-State. If ul-powerControl is provided and if UE is indicated a first TCI-State or TCI-UL-State and a second TCI-State or TCI-UL-State, PO_PUSCH,b,f,c(j), αb,f,cj and l are obtained by p0AlphaSetforPUSCH associated with the first indicated TCI-State or TCI-UL-State, PLb,f,c(qd) is obtained by PL-RS associated with the first indicated TCI-State or TCI-UL-State. ## *** Unchanged parts are omitted ***

In Rel-17 M-TRP PUSCH repetition, if UE is not provided twoPHRMode, the UE provides one PHR in slot n and the actual PHR is for the first PUSCH repetition associated with first SRS resource set or second SRS resource set in slot n. For STxMP SDM/SFN scheme, if UE is not provided twoPHRMode, whether UE provides actual PHR associated with first or second SRS resource set needs to be clarified.

Proposal 3.5:

  - Adopt the following TP for TS 38.213.

## 7.7.1 Type 1 PH report ## *** Unchanged parts are omitted *** If a UE is not provided twoPHRMode, and is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' on active UL BWP b of carrier f of serving cell c, the UE provides one Type 1 power headroom report in a slot n. If the Type 1 power headroom report is for an actual PUSCH repetition, the Type 1 power headroom report is for the first PUSCH repetition associated with the first SRS resource set or the second SRS resource set that overlaps with slot n. If a UE is not provided twoPHRMode, and is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook’, and is provided dl-OrJointTCI-StateList or TCI-UL-State and is indicated a first TCI-State or TCI-UL-State and a second TCI-State or TCI-UL-State, and is provided multipanelScheme on active UL BWP b of carrier f of serving cell c, the UE provides one Type 1 power headroom report in a slot n. If UE provides Type 1 power headroom report for an actual PUSCH transmission using both the first TCI-state or first TCI-UL-State and the second TCI-state or second TCI-UL-State, the Type 1 power headroom report is for the PUSCH transmission associated with the first TCI-state or TCI-UL-State. ## *** Unchanged parts are omitted ***

Similar issue exists in M-DCI STxMP PUSCH+PUSCH. If two PUSCH transmissions associated with two CORESETPoolIndex overlap in time in slot n, clarification is needed on whether UE provides actual PHR for PUSCH associated with CORESETPoolIndex value 0 or 1.

Proposal 3.6:

  - Adopt the following TP for TS 38.213.

## 7.7.1 Type 1 PH report ## *** Unchanged parts are omitted *** If a UE is not provided twoPHRMode, and is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' on active UL BWP b of carrier f of serving cell c, the UE provides one Type 1 power headroom report in a slot n. If the Type 1 power headroom report is for an actual PUSCH repetition, the Type 1 power headroom report is for the first PUSCH repetition associated with the first SRS resource set or the second SRS resource set that overlaps with slot n. If a UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook’, and is provided enableSTx2PofmDCI, and is provided dl-OrJointTCI-StateList or TCI-UL-State and is indicated a first TCI-State or TCI-UL-State and a second TCI-State or TCI-UL-State on active UL BWP b of carrier f of serving cell c, the UE provides one Type 1 power headroom report in a slot n. If the Type 1 power headroom report is for an actual PUSCH transmission and two PUSCH transmissions associated with different CORESETPoolIndex overlap in time in slot n, the Type 1 power headroom report is for the PUSCH transmission associated with CORESETPoolIndex with a value of 0. ## *** Unchanged parts are omitted ***

- Conclusion

In this contribution, we discussed unified TCI framework related enhancements. Based on the discussion, we made the following proposals.

Unified TCI framework extension for M-TRP

Proposal 2-1:

  - For mDCI mTRP, the BAT is determined per coresetPoolIndex for mDCI mTRP.

  - Adopt the following TP for TS 38.214 v18.0.0.

    - Reason for change: In Rel.18 CR (R1-2310669), if multiple HARQ-ACK bits are transmitted on a PUCCH/PUSCH, the indicated TCI state associated with the last DCI with positive HARQ-ACK value is applied regardless of coresetPoolIndex. However, for joint HARQ-ACK feedback for mDCI mTRP, one PUCCH/PUSCH contains multiple HARQ-ACK bits associated with two different coresetPoolIndexes. Hence, the current Rel.18 spec. results in cross TRP TCI indication in case of joint HARQ-ACK feedback for mDCI mTRP, which is not aligned with the previous agreement.

    - Summary of change: BAT is determined per each coresetPoolIndex for mDCI mTRP.

    - Consequence if not approved: The specification is not aligned with previous agreement.

5.1.5 Antenna ports quasi co-location ## *** Unchanged parts are omitted *** When a UE configured with dl-OrJointTCI-StateList and is not configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet, the UE would transmit a PUCCH with positive HARQ-ACK or a PUSCH with positive HARQ-ACK corresponding to the DCI carrying the TCI State indication and without DL assignment, or corresponding to the PDSCH scheduled by the DCI carrying the TCI State indication, and if the indicated TCI State(s) is/are different from the previously indicated one(s), the indicated TCI-State(s) and/or TCI-UL-State(s) should be applied starting from the first slot that is at least beamAppTime symbols after the last symbol of the PUCCH or the PUSCH, and if the UE receives more than one indicated TCI state for a CC/BWP to be applied starting from the first slot that is at least beamAppTime symbols after the last symbol of the PUCCH or the PUSCH, the indicated TCI state carried in the latest DCI in time corresponding to positive HARQ-ACK value is applied. The first slot and the beamAppTime symbols are both determined on the active BWP with the smallest SCS among the BWP(s) from the CCs applying the indicated TCI-State(s) or TCI-UL-State(s) that are active at the end of the PUCCH or the PUSCH carrying the positive HARQ-ACK. When a UE configured with dl-OrJointTCI-StateList and is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet, the UE would transmit a PUCCH with positive HARQ-ACK or a PUSCH with positive HARQ-ACK corresponding to the DCI carrying the TCI State indication specific to a coresetPoolIndex value and without DL assignment, or corresponding to the PDSCH scheduled by the DCI carrying the TCI State indication specific to a coresetPoolIndex value, and if the indicated TCI State(s) specific to the coresetPoolIndex value is/are different from the previously indicated one(s), the indicated TCI-State(s) and/or TCI-UL-State(s) specific to the coresetPoolIndex value should be applied starting from the first slot that is at least beamAppTime symbols after the last symbol of the PUCCH or the PUSCH, and if the UE receives more than one indicated TCI state specific to the coresetPoolIndex value for a CC/BWP to be applied starting from the first slot that is at least beamAppTime symbols after the last symbol of the PUCCH or the PUSCH, the indicated TCI state specific to the coresetPoolIndex value carried in the latest DCI in time corresponding to positive HARQ-ACK value is applied. The first slot and the beamAppTime symbols are both determined on the active BWP with the smallest SCS among the BWP(s) from the CCs applying the indicated TCI-State(s) or TCI-UL-State(s) specific to the coresetPoolIndex value that are active at the end of the PUCCH or the PUSCH carrying the positive HARQ-ACK. *** Unchanged parts are omitted ***

Proposal 2-2:

  - When a UE is configured with dl-OrJointTCI-StateList or TCI-UL-State, and if TCI state selection field is not configured in DCI format 1_1/1_2, support Alt.1:

    - A CC/BWP is operated in Rel-18 unified TCI framework extension for S-DCI based MTRP if Rel-18 TCI state activation command (MAC-CE) where at least one activated TCI codepoint is mapped with both the first and second join/DL/UL TCI states is received and applied to the CC/BWP

    - A CC/BWP is operated in Rel-17 unified TCI framework if Rel-18 TCI state activation command (MAC-CE) where all activated TCI codepoint(s) is mapped with either only the first joint/DL/UL TCI state(s) or only the second joint/DL/UL TCI state(s) is received and applied to the CC/BWP

    - A CC/BWP is operated in Rel-17 unified TCI framework if Rel-17 TCI state activation command (MAC-CE) is received and applied to the CC/BWP

Proposal 2-3:

  - Introduce restriction that indicated joint/DL TCI state(s) for CJT-PDSCH should be QCLed with at least part of joint/DL TCI state(s) configured for CSI-RS(s) as CMR(s) for CJT.

Proposal 2-4:

  - To indicate the same joint/DL TCI state(s) between CJT-PDSCH and CMR(s) for CJT, the following options can be considered. Opt. 3 is preferred.

    - Opt. 1) Solved by gNB implementation (i.e. gNB indicates the same TCI state to CJT-PDSCH and CSI-RS for CMR for CJT. It means, if CMR for CJT is P/SP-CSI-RS, gNB cannot update indicated TCI by MAC CE/DCI).

    - Opt. 2) Only A-CSI-RS is allowed for CSI-RS for CMR for CJT.

    - Opt. 3) Apply MAC CE/DCI based indicated TCI state to P/SP-CSI-RS, in case of CJT-PDSCH.

Proposal 2.5:

- For S-DCI based M-TRP TDM PUSCH repetition, when two SRS resource sets are configured, if Type-1 CG PUSCH configuration contains only one SRI field and/or one TPMI field, the PUSCH transmission of this CG PUSCH is associated with first SRS resource set if the first indicated TCI state is applied, and the PUSCH transmission of this CG PUSCH is associated with the second SRS resource set if the second indicated TCI state is applied.

- Adopt the following TP for TS 38.214.

## 6.1.2.3 Resource allocation for uplink transmission with configured grant ## *** Unchanged parts are omitted *** When a UE is configured with dl-OrJointTCI-StateList or TCI-UL-State and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook' or 'noncodebook', and the higher layer parameter multipanelScheme is set to ‘SDMscheme’ or ‘SFNscheme’, and the higher layer parameter rrc-ConfiguredUplinkGrant does not contain srs-ResourceIndicator2 or precodingAndNumberOfLayers2, the PUSCH transmission occasion(s) is associated with the first SRS resource set if the first indicated TCI-States or TCI-UL-States applies and is associated with the second SRS resource set if the second indicated TCI-States or TCI-UL-States applies. ## *** Unchanged parts are omitted ***

Power control for M-TRP operation

Proposal 3.1:

  - Adopt the following TP for TS 38.213.

## 7.7.1 Type 1 PH report ## *** Unchanged parts are omitted *** If a UE is provided, for active UL BWP b of carrier f of serving cell c, - twoPHRMode, - two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', - dl-OrJointTCI-StateList or TCI-UL-State and is indicated a first TCI-State or TCI-UL-State and a second TCI-State or TCI-UL-State, and - multipanelScheme the UE provides - a first Type 1 power headroom report and a first configured maximum output power associated with the first TCI-State or TCI-UL-State for an actual PUSCH transmission using a spatial domain filter corresponding only to the first TCI-State or TCI-UL-State and a second Type 1 power headroom report and a second configured maximum output power associated with the second TCI-State or TCI-UL-State for reference PUSCH transmission - a first Type 1 power headroom report and a first configured maximum output power associate with the first TCI-State or TCI-UL-State for reference PUSCH transmission and a Type 1 power headroom report and a configured maximum output power associated with the second TCI-State or TCI-UL-State for an actual PUSCH transmission using a spatial domain filter corresponding only to the second TCI-State or TCI-UL-State - a first Type 1 power headroom report and a first configured maximum output power associated with the first TCI-State or TCI-UL-State, and a second Type 1 power headroom report and a second configured maximum output power associated with the second TCI-State or TCI-UL-State, for an actual PUSCH transmission using a spatial domain filter corresponding to the first TCI-State or TCI-UL-State and using a spatial domain filter corresponding to the second TCI-State or TCI-UL-State - a first Type 1 power headroom report and a first configured maximum output power associated with the first TCI-State or TCI-UL-State for reference PUSCH transmission and a second Type 1 power headroom report and a second configured maximum output power associated with the second TCI-State or TCI-UL-State for reference PUSCH transmission if UE determines that both PHRs are based on reference PUSCH transmissions ## *** Unchanged parts are omitted ***

Proposal 3.2:

  - The configured maximum output power in PHR of STxMP is per panel.

Proposal 3.3:

  - Adopt the following TP for TS 38.213. Note that the text highlighted in blue is TP proposed in proposal 3.1.

## 7.7.1 Type 1 PH report ## *** Unchanged parts are omitted *** If a UE is provided, for active UL BWP b of carrier f of serving cell c, - twoPHRMode, - two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', - dl-OrJointTCI-StateList or TCI-UL-State and is indicated a first TCI-State or TCI-UL-State and a second TCI-State or TCI-UL-State, and - multipanelScheme the UE provides - a first Type 1 power headroom report and a first configured maximum output power associated with the first TCI-State or TCI-UL-State for an actual PUSCH transmission using a spatial domain filter corresponding only to the first TCI-State or TCI-UL-State and a second Type 1 power headroom report and a second configured maximum output power associated with the second TCI-State or TCI-UL-State for reference PUSCH transmission. The UE provides the second Type 1 power headroom report for reference PUSCH transmission using the p0AlphaSetforPUSCH and pathlossReferenceRS-Id-r17 values associated with the second TCI-State or TCI-UL-State. - a first Type 1 power headroom report and a first configured maximum output power associate with the first TCI-State or TCI-UL-State for reference PUSCH transmission and a Type 1 power headroom report and a configured maximum output power associated with the second TCI-State or TCI-UL-State for an actual PUSCH transmission using a spatial domain filter corresponding only to the second TCI-State or TCI-UL-State. The UE provides the first Type 1 power headroom report for reference PUSCH transmission using the p0AlphaSetforPUSCH and pathlossReferenceRS-Id-r17 values associated with the first TCI-State or TCI-UL-State. - a first Type 1 power headroom report and a first configured maximum output power associated with the first TCI-State or TCI-UL-State, and a second Type 1 power headroom report and a second configured maximum output power associated with the second TCI-State or TCI-UL-State, for an actual PUSCH transmission using a spatial domain filter corresponding to the first TCI-State or TCI-UL-State and using a spatial domain filter corresponding to the second TCI-State or TCI-UL-State - a first Type 1 power headroom report and a first configured maximum output power associated with the first TCI-State or TCI-UL-State for reference PUSCH transmission and a second Type 1 power headroom report and a second configured maximum output power associated with the second TCI-State or TCI-UL-State for reference PUSCH transmission if UE determines that both PHRs are based on reference PUSCH transmissions. The UE provides the first Type 1 power headroom report for reference PUSCH transmission using the p0AlphaSetforPUSCH and pathlossReferenceRS-Id-r17 values associated with the first TCI-State or TCI-UL-State. The UE provides the second Type 1 power headroom report for reference PUSCH transmission using the p0AlphaSetforPUSCH and pathlossReferenceRS-Id-r17 values associated with the second TCI-State or TCI-UL-State. ## *** Unchanged parts are omitted ***

Proposal 3.4:

  - Adopt the following TP for TS 38.213.

## 7.7.1 Type 1 PH report ## *** Unchanged parts are omitted *** If the UE determines that a Type 1 power headroom report for an activated serving cell is based on a reference PUSCH transmission then, for PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c, the UE computes the Type 1 power headroom report as PHtype1,b,f,ci,j,qd,l= PCMAX,f,c(i)-PO_PUSCH,b,f,cj+αb,f,cj⋅PLb,f,cqd+fb,f,ci,l [dB] where PCMAX,f,c(i) is computed assuming MPR=0 dB, A-MPR=0 dB, P-MPR=0 dB. TC = 0 dB. MPR, A-MPR, P-MPR and TC are defined in [8-1, TS 38.101-1], [8-2, TS 38.101-2] and [8-3, TS 38.101-3]. The remaining parameters are defined in clause 7.1.1 and, if ul-powerControl is not provided, PO_PUSCH,b,f,c(j) and αb,f,cj are obtained using PO_NOMINAL,PUSCH,f,c0 and p0-PUSCH-AlphaSetId = 0, PLb,f,c(qd) is obtained using pusch-PathlossReferenceRS-Id = 0, and l=0. If ul-powerControl is provided, PO_PUSCH,b,f,c(j), αb,f,cj and l are obtained by p0AlphaSetforPUSCH associated with the indicated TCI-State or TCI-UL-State, PLb,f,c(qd) is obtained by PL-RS associated with the indicated TCI-State or TCI-UL-State. If ul-powerControl is provided and if UE is indicated a first TCI-State or TCI-UL-State and a second TCI-State or TCI-UL-State, PO_PUSCH,b,f,c(j), αb,f,cj and l are obtained by p0AlphaSetforPUSCH associated with the first indicated TCI-State or TCI-UL-State, PLb,f,c(qd) is obtained by PL-RS associated with the first indicated TCI-State or TCI-UL-State. ## *** Unchanged parts are omitted ***

Proposal 3.5:

  - Adopt the following TP for TS 38.213.

## 7.7.1 Type 1 PH report ## *** Unchanged parts are omitted *** If a UE is not provided twoPHRMode, and is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' on active UL BWP b of carrier f of serving cell c, the UE provides one Type 1 power headroom report in a slot n. If the Type 1 power headroom report is for an actual PUSCH repetition, the Type 1 power headroom report is for the first PUSCH repetition associated with the first SRS resource set or the second SRS resource set that overlaps with slot n. If a UE is not provided twoPHRMode, and is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook’, and is provided dl-OrJointTCI-StateList or TCI-UL-State and is indicated a first TCI-State or TCI-UL-State and a second TCI-State or TCI-UL-State, and is provided multipanelScheme on active UL BWP b of carrier f of serving cell c, the UE provides one Type 1 power headroom report in a slot n. If UE provides Type 1 power headroom report for an actual PUSCH transmission using both the first TCI-state or first TCI-UL-State and the second TCI-state or second TCI-UL-State, the Type 1 power headroom report is for the PUSCH transmission associated with the first TCI-state or TCI-UL-State. ## *** Unchanged parts are omitted ***

Proposal 3.6:

  - Adopt the following TP for TS 38.213.

## 7.7.1 Type 1 PH report ## *** Unchanged parts are omitted *** If a UE is not provided twoPHRMode, and is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook' on active UL BWP b of carrier f of serving cell c, the UE provides one Type 1 power headroom report in a slot n. If the Type 1 power headroom report is for an actual PUSCH repetition, the Type 1 power headroom report is for the first PUSCH repetition associated with the first SRS resource set or the second SRS resource set that overlaps with slot n. If a UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook’, and is provided enableSTx2PofmDCI, and is provided dl-OrJointTCI-StateList or TCI-UL-State and is indicated a first TCI-State or TCI-UL-State and a second TCI-State or TCI-UL-State on active UL BWP b of carrier f of serving cell c, the UE provides one Type 1 power headroom report in a slot n. If the Type 1 power headroom report is for an actual PUSCH transmission and two PUSCH transmissions associated with different CORESETPoolIndex overlap in time in slot n, the Type 1 power headroom report is for the PUSCH transmission associated with CORESETPoolIndex with a value of 0. ## *** Unchanged parts are omitted ***

- Reference

[1] 3GPP RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, RAN#94-e, Dec. 2021.

[2] 3GPP R4-2303494, “Reply LS on UE power limitation for STxMP in FR2, RAN4, Mar. 2023