3GPP TSG RAN WG1 #115	           R1-2311642

Chicago, USA, Nov 13th – 17th, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on UE features for NR MIMO evolution

Agenda Item:	8.16.1

Document for: 	Discussion and Decision

Introduction

Updated RAN1 UE features list for Rel-18 NR after RAN1#115 is captured in R1-2310635 [1]. In this contribution, we provide our views on UE features for NR MIMO evolution.

Discussions

Unified TCI framework for multi-TRP

| 40. NR_MIMO_evo_DL_UL | 40-1-1 | Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC | FFS: maximum number of configured joint TCI states per CC per BWP FFS: maximum number of activated joint TCI states across all CCs FFS: what to do/signal about additional support of applying different TCI state of PDSCH, PUCCH or PUSCH from that of the scheduled/activated PDCCH when the UE applies one indicated joint/DL/UL TCI state to the PDSCH, PUCCH or PUSCH | FFS | yes | n/a | Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported | Per band | n/a | n/a | n/a | Note: FG 16-2b-0 can be used to indicate support of two default beams | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-1-1a | Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC | FFS: maximum number of activated joint TCI states per CC | FFS | yes | n/a | Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC is not supported | Per band | n/a | n/a | n/a | Note: FG 16-2b-0 can be used to indicate support of two default beams | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-1b | TRP-specific BFR with unified TCI framework with Unified TCI | Support for TRP-specific BFR with unified TCI framework with Unified TCI | FFS | yes | n/a |  | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-1c | DCI format 1_1 and if supported 1_2 configured with TCI selection field | Support of DCI format 1_1 and if supported 1_2 configured with TCI selection field | At least one of {40-1-1/1a/2/2a} | yes | n/a | DCI format 1_1 and if supported 1_2 configured with TCI selection field are not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-2 | Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC | FFS: maximum number of configured DL TCI states per CC per BWP FFS: maximum number of configured UL TCI states per CC per BWP FFS: maximum number of activated DL TCI states across all CCs FFS: maximum number of activated UL TCI states across all CCs | FFS | yes | n/a | Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported | Per band | n/a | n/a | n/a | Note: FG 16-2b-0 can be used to indicate support of two default beams | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-2a | Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC | FFS: maximum number of activated DL TCI states across all CCs FFS: maximum number of activated UL TCI states across all CCs | FFS | yes | n/a | Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC is not supported | Per band | n/a | n/a | n/a | Note: FG 16-2b-0 can be used to indicate support of two default beams | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-3 | Per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP | Support of per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP |  | yes | n/a |  | Per band | n/a | n/a | n/a | Component candidate values: {per resource, per resource set, both} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-4 | Two TCI states for CJT Tx scheme for PDSCH | Support of two TCI states for CJT Tx scheme for PDSCH [FFS: for CSI-RS] |  | yes | n/a | Two TCI states for CJT Tx scheme for PDSCH are not supported | Per band | n/a | n/a | n/a | Component candidate values: {CJT Scheme-A, CJT scheme-B, both} CJT Scheme-A: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA CJT Scheme-B: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint/DL TCI state | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-7 | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC | 1. Support of [intra-cell] mTRP operation for M-DCI [2. Support of a single default beam for PDSCH with scheduling offset less than a threshold unless UE indicates the support of two default beams] 3. Maximum number of configured joint TCI states per BWP per CC 4. Maximum number of activated joint TCI states across all CCs per ‘coresetPoolIndex’ value 5. One MAC-CE activates one joint TCI-states per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value | [16-2a, 23-4, 40-6-1, 40-6-2, 40-6-3a, 40-6-3b, 40-6-4] | yes | n/a | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported | [Per band] | n/a | n/a | n/a | Component 3 candidate values: {8, 12, 16, 24, 32, 48, 64, 128} Component 4 candidate values: {2, 4, 8, 16} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH transmissions | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-7a | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC | 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 without DL assignment) 2. Maximum number of MAC-CE activated joint TCI states per BWP per CC per coresetpoolindex [3. The minimum beam application time in symbols] | [FFS] | yes | n/a | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported | [Per band] | n/a | n/a | n/a | Component 2 candidate values: {2,3,4,5,6,7,8) Component 3 candidate values: {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336 } only can be applicable in FR2 | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-9 | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC | 1. Maximum number of configured DL TCI states per BWP per CC 2. Maximum number of configured UL TCI states per BWP per CC 3. Maximum number of activated DL TCI states across all CC 4. Maximum number of activated UL TCI states across all CC 5. One MAC-CE activated DL TCI-state per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value. 6. One MAC-CE activated UL TCI-state per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value. | [23-10-1] | yes | n/a | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported | [Per band] | n/a | n/a | n/a | Component 1 candidate value {8, 12, 16, 24, 32, 48, 64, 128} Component 2 candidate value {8, 12, 16, 24, 32, 48, 64} Component 3 candidate values: {1, 2, 4, 8, 16} Component 4 candidate values: {1, 2, 4, 8, 16} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-9a | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC | 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 without DL assignment) 2. maximum number of activated DL TCI states per CORESETPoolIndex per BWP per CC 3. maximum number of activated UL TCI states per CORESETPoolIndex per BWP per CC | [23-10-1b, 40-1-9] | yes | n/a | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported | [Per band] | n/a | n/a | n/a | Component 2 candidate values: {1, 2, 3, 4, 5, 6, 7, 8} Component 3 candidate values: {1, 2, 3, 4, 5, 6, 7, 8,} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-12 | Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP | 1. Support of common multi-CC TCI state ID update and activation for single-DCI based multi-TRP 2. Maximum number of CC list(s) | [40-1-1, 40-1-1, 40-1-2] | yes | n/a | Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP is not supported | [Per band] | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-13 | Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP | 1. Support of common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP 2. Maximum number of CC list(s) | [40-1-7] | yes | n/a | Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP is not supported | [Per band] | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4} | Optional with capability signaling |

40-1-1

- Components:

  - We think the first two FFS can be supported. We think “per CC per BWP” should be “per BWP per CC”.

  - Regarding to the last FFS, we are not ok with the current text. If the last FFS part is supported, when UE is indicated with two TCI states (TCI#1, TCI#2), gNB must use the same TCI between the scheduling DCI and the scheduled PDSCH. It looks M-TRP PDSCH can be only scheduled by SFN-PDCCH. Even for S-TRP PDSCH, we think DCI with TCI#1 schedules PDSCH with TCI#2 should be automatically supported if UE supports TCI state selection field. We think the motivation of the FFS is to ensure that one UE can assume that TCI state of the scheduling DCI can be the same as one of the TCI states of the scheduled PDSCH. We suggest updating the FFS to have the following separate two FGs. Or, another possible option is just remove the last FFS.

    - FG1 (optional): Support TCI state selection field. UE assumes TCI state of the scheduling PDCCH is the same as one of the TCI states of the scheduled PDSCH.

    - FG2 (optional): Support TCI state selection field. UE assumes TCI state of the scheduling PDCCH can be the same or different as one of the TCI states of the scheduled PDSCH.

40-1-1a/40-1-2/40-1-2a

- Components:

  - We think all the FFS can be supported.

40-1-1b

- Prerequisite FG:

  - It should be FG23-5-2 (MTRP BFR based on two BFD-RS sets) and at least of one of FG40-1-1, FG40-1-2, FG40-1-7, or FG40-1-9.

40-1-7

- Component 1:

  - It is fine to agree with “intra-cell” if there is another row for “inter-cell”.

- Component 2:

  - It should be removed to be consistent with single DCI case (e.g. FG40-1-1). Then, we can add Note: FG 16-2a-6 can be used to indicate support of two default beams.

MTRP TA

| 40. NR_MIMO_evo_DL_UL | 40-2-1 | Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement | FFS: Maximum number of n-TimingAdvanceOffset value per serving cell | 16-2a | yes | n/a |  | [Per band] | n/a | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-2-2 | Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement | FFS: Maximum number of n-TimingAdvanceOffset value per serving cell | 16-2a | yes | n/a |  | [Per band] | n/a | n/a | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-2-3 | TAG ID indication via absolute TA command MAC CE | Support indicating one of two TAG IDs configured in the SpCell via absolute TA command MAC CE. |  | yes | n/a |  | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-2-4 | Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA | Support of cross-TRP PDCCH order based on CFRA for inter-cell [WA: and intra-cell] multi-DCI based mTRP |  | yes | N/A |  | [Per band] | No | No | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-2-5 | TCI states of one coresetPoolIndex associated to both TAGs | Support of UL/joint TCI states of UL signals/channels associated to one coresetPoolIndex correspond to both TAGs This is a WA | FG 40-2-1 or 40-2-2 | yes | n/a |  | [Per band] | n/a | n/a | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-2-6 | Rx timing difference larger than CP length |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-2-7 | Two TAs for multi-DCI STxMP PUSCH+PUSCH | Support of two TAs for multi-DCI STxMP PUSCH+PUSCH | [40-2-1 or 40-2-2, FFS more] | yes | n/a |  | [Per band] | n/a | n/a | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-2-8 | The maximum number of TAGs all CCs |  |  |  |  |  |  |  |  |  |  |  |

40-2-1

- Components: Regarding the FFS, it was agreed in last meeting that for intra-cell multi-DCI based Multi-TRP operation, only a single NTA,offset is configured. No component or separate row is needed.

- Prerequisite FG: In RAN1#113, it was concluded that there is no consensus on support two TA for Rel-15/16 spatial relation framework. Two TA is applied only when unified TCI framework is applied, thus, FG40-1-7 or 40-1-9 (Rel-18 unified TCI for M-DCI M-TRP) should be its prerequisites.

40-2-2

- Components: Regarding the FFS, support up to two n-TimingAdvanceOffset value per serving cell can be basic feature. No separate row is needed.

- Prerequisite FG: Similar as 40-2-1, FG40-1-7 or 40-1-9 (Rel-18 unified TCI for M-DCI M-TRP) should be its prerequisite. Furthermore, 23-4 (inter-cell mTRP) should be its prerequisite.

FG 40-2-3

- Prerequisite FG: FG 40-2-1 or 40-2-2 should be its prerequisite. In addition, it is used only for 2-step RACH, thus FG 9-1 (2-step RACH) should be its prerequisite.

FG 40-2-4

- Component: WA of cross TRP PDCCH order RACH for intra-cell M-TRP was confirmed. “[WA: and intra-cell]” should be replaced by “and intra-cell”.

- Prerequisite FG: FG 40-2-1 or 40-2-2 should be added to prerequisite FG.

FG 40-2-5

- Component: This WA has not been confirmed in RAN1.

FG 40-2-6

- Prerequisite FG: FG 40-2-1 or 40-2-2 should be added to prerequisite FG.

FG 40-2-7

- Prerequisite FG: FG 40-6-3a or 40-6-3b (CB or NCB M-DCI STxMP) should be added to prerequisite FG.

FG 40-2-8

- Component: maximum number of TAGs across all CCs.

- Candidate value: {2,3,4}

- Prerequisite: 40-2-1 or 40-2-2

CSI

    - CJT CSI

| 40. NR_MIMO_evo_DL_UL | 40-3-1-1 | Support of mode 2 for Rel-16-based CJT type-II codebook | Support of N=N_TRP only Support of N_L=1 only 1. Support of mode 2 for Rel-16 eType-II codebook refinement for multi-TRP CJT 2. Support for PMI subband R=1. 3. Support of parameter combinations with L=2,4 4. Support of rank 1,2 5. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT] 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs | [2-35] | Yes | N/A |  | Per band and Per BC | No | N/A | N/A | FFS: CPU occupation Component 5 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel. 16 based type-II CJT must support this FG | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-1a | Support of mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset | [1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with PMI subband R=1. 2. Support of parameter combinations with L=2,4 3. Support of rank 1,2] 4. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT] 5. Supported frequency basis selection mode 1, i.e., common frequency basis selection among different TRPs with FD basis selection integer frequency offset | 40-3-1-1 | Yes | N/A | Mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset is not supported | Per band and Per BC | N/A | N/A | N/A | Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-2 | Support for FD basis selection fractional offset mode for Rel-16-based CJT codebook with mode1 | Supported frequency basis selection mode 1 with FD basis selection fractional frequency offset for Rel-16 eType-II based CJT codebook | 40-3-1-1a | Yes | N/A | FD basis selection fractional offset mode for Rel-16-based CJT codebook with mode 1 is not supported | Per band and Per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-3 | Support R=2 for Rel-16-based CJT codebook | 1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with PMI subbands R=2 2. {Max # of Tx ports in one resource set, Max # of resource sets, total # of Tx ports}, [across all CCs] simultaneously, with R=2 | 40-3-1-1 | Yes | N/A | R=2 for Rel-16-based CJT codebook is not supported | Per-band Per-BC | N/A | N/A | N/A | Component 2 candidate values: a) {4,8,12,16,24,32} b) {2 to 64} c) {4 to 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-4 | Support pv={1/2,1/2,1/2,1/2} and beta=1/2 for Rel-16-based CJT codebook | 1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with parameter combination pv={1/2,1/2,1/2,1/2} and beta=1/2 | 40-3-1-1 | Yes | N/A | pv={1/2,1/2,1/2,1/2} and beta=1/2 for Rel-16-based CJT codebook are not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-5 | Support of mode 2 for Rel-17-based CJT type-II codebook | Support of N=N_TRP only Support of N_L=1 only 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT 2. Support of PMI subband R=1. 3. Support of parameter combinations with M=1 4. Support of rank 1,2 5. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs | [2-35] | Yes | N/A |  | Per-band and Per-BC | No | N/A | N/A | FFS: CPU occupation Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel 17 based type-II CJT must support this FG | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-5a | Support of mode 1 for Rel-17-based CJT type-II codebook with FD basis selection integer frequency offset | 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with PMI subband R=1. 2. Support of parameter combinations with M=1 3. Support of rank 1,2 4. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT 5. Supported frequency basis selection mode 1, i.e., common frequency basis selection among different TRPs with FD basis selection integer frequency offset | 40-3-1-5 | Yes | N/A | Mode 1 for Rel-17-based CJT type-II codebook with FD basis selection integer frequency offset is not supported | Per band and per BC | N/A | N/A | N/A | Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-6 | Support for FD basis selection fractional offset mode for Rel-17-based CJT codebook with mode1 | 1. Supported frequency basis selection mode 1 with FD basis selection fractional frequency offset for Rel-17 FeType-II port selection based CJT codebook | 40-3-1-5a | Yes | N/A | FD basis selection fractional offset mode for Rel-17-based CJT codebook with mode1 is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-7 | Support of M=2 and R=1 for Rel-17-based CJT codebook | 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with M=2 and PMI subband R=1 2. {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, [across all CCs] simultaneously, with M=2 and R=1 | 40-3-1-5 or 40-3-1-5a | Yes | N/A | M=2 and R=1 for Rel-17-based CJT codebook are not supported | Per band and per BC | N/A | N/A | N/A | Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-8 | Support R=2 for Rel-17-based CJT codebook | 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with PMI subband R=2. 2. {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, [across all CCs] simultaneously, with R=2 | 40-3-1-5 or 40-3-1-5a | Yes | N/A | R=2 for Rel-17-based CJT codebook is not supported | Per band and Per BC | N/A | N/A | N/A | Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-9 | Support for 2NN1N2 >32 for Rel-16 based CJT codebook | Maximum number of ports across all TRPs for one CJT CSI measurement | 40-3-1-1 | Yes | N/A | 2NN1N2 >32 is not supported for Rel-16 based CJT codebook | Per band and Per BC | N/A | N/A | N/A | Component 1 candidate values: {64, 96, 128} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-9a | Support for 2NN1N2 >32 for Rel-17 based CJT codebook | Maximum number of ports across all TRPs for one CJT CSI measurement | 40-3-1-5 | Yes | N/A | 2NN1N2 >32 is not supported for Rel-17 based CJT codebook | Per band and Per BC | N/A | N/A | N/A | Component 1 candidate values: {64, 96, 128} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-11 | Active CSI-RS resources and ports for mixed codebook types including Type-II-CJT in any slot | 1. List of codebook combinations 2. List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | 40-3-1-1,40-3-1-5,2-36 | Yes | N/A | Active CSI-RS resources and ports for mixed Type-II-CJT codebook types in any slot is not supported | [Per-band Per-BC/Per FS] | N/A | N/A | N/A | [Codebook 1 = {Type I SP, Type I MP} (Codebook 2, Codebook 3) = {(eType-II-CJT R=1, NULL), (eType-II-CJT R=2, NULL), (FeType-II-CJT PS R=1 M=1, NULL), (FeType-II-CJT PS R=1 M=2, NULL), (FeType-II-CJT PS R=2 M=2, NULL), (FFS CB combos…)}] | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-12 | Support of Rank 3 and 4 for Rel-16-based CJT type-II codebook | 1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with rank 3,4 | 40-3-1-1 | Yes | N/A | Rank 3 and 4 for Rel-16-based CJT type-II codebook is not supported | Per-band and Per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-13 | Support of Rank 3 and 4 for Rel-17-based CJT type-II codebook | 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with rank 3,4 | 40-3-1-5 | Yes | N/A | Rank 3 and 4 for Rel-17-based CJT type-II codebook is not supported | Per-band and Per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-14 | Support of Support of L=6 for Rel-16-based CJT type-II codebook | 1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with parameter combination with L=6 | 40-3-1-1 | Yes | N/A | Support of L=6 for Rel-16-based CJT type-II codebook is not supported | Per-band and Per BC | N/A | N/A | N/A | Note: Only for N_TRP=1 | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-15 | dynamic selection of N<=N_TRP for Rel-16-based CJT type-II codebook | 1. Support of selection of N <= N_TRP CSI-RS resource by UE for multi-TRP CJT based on Rel-16 eType-II codebook | 40-3-1-1 | Yes | N/A | Dynamic selection of N<N_TRP for Rel-16-based CJT type-II codebook is not supported | Per-band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-16 | dynamic selection of N<=N_TRP for Rel-17-based CJT type-II codebook | 1. Support of selection of N <= N_TRP CSI-RS resource by UE for multi-TRP CJT based on Rel-17 FeType-II port selection codebook | 40-3-1-5 | Yes | N/A | Dynamic selection of N<N_TRP for Rel-17-based CJT type-II codebook is not supported | Per-band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-17 | Support for N_L>1 combinations of number of SD basis across CSI-RS resources for Rel-16-based CJT type-II codebook | 1. Maximum number of lists for spatial basis selection, i.e., N_L, for multi-TRP CJT based on Rel-16 eType-II codebook | 40-3-1-1 | Yes | N/A | N_L>1 combinations across for Rel-16-based CJT type-II codebook is not supported | Per band and Per BC | N/A | N/A | N/A | Candidate values for component 1: N_L = {2,4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-18 | Support for N_L>1 combinations of number of ports across CSI-RS resources for Rel-17-based CJT type-II codebook | 1. Maximum number of lists for ports selection, i.e., NL, for multi-TRP CJT based on Rel-17 FeType-II port selection codebook | 40-3-1-5 | Yes | N/A | N_L>1 combinations across for Rel-17-based CJT type-II codebook is not supported | Per band and Per BC | N/A | N/A | N/A | Candidate values for component 1: N_L = {2,4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-23 | Unequal number of spatial basis selection configuration for multi-TRP CJT | 1. Support of unequal number of spatial basis selection configuration across CSI-RS resources for multi-TRP CJT including Rel-16 eType-II codebook refinement | 40-3-1-1 | Yes | N/A | Unequal number of spatial basis selection configuration for multi-TRP CJT is not supported | Per-band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-23a | Unequal number of port selection configuration for multi-TRP CJT | 1. Support of unequal number of port selection configuration across CSI-RS resources for multi-TRP CJT including Rel-17 FeType-II port selection codebook refinement | 40-3-1-5 | Yes | N/A | Unequal number of port selection configuration for multi-TRP CJT is not supported | Per-band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |

40-3-1-1

- Prerequisite FG: OK with 2-35.

- Component:

  - OK with ‘across all CCs’.

  - Support to add CPU occupation with the max value as a component of the basic feature. And a note should be added to clarify that when NTRP=1 is configured, legacy OCPU applies, i.e. OCPU =1.

40-3-1-1a

- Component: support the yellow text, to align with 40-3-1-1.

40-3-1-3/40-3-1-7/40-3-1-8

- Component:

  - OK with ‘across all CCs’.

40-3-1-5

- Prerequisite FG: OK with 2-35.

- Component:

  - Support to add CPU occupation with the max value as a component of the basic feature.

40-3-1-11

- Type: OK with Per-band and per BC.

In addition, We also support to add new FGs for following. And each of following can have separate rows for Rel-16 based enhancement and Rel-17 based enhancement respectively.

- CPU occupation for N_TRP>1 CMRs for CJT CSI.

- UE CSI processing time for N_TRP>1 CMRs for CJT CSI.

| 40. NR_MIMO_evo_DL_UL | 40-3-1-19 | CPU counting for NTRP>1 for Rel-16-based CJT type-II codebook | - Value of X for CPU occupation (OCPU = X.NTRP) |  | Yes | N/A |  | Per-band and per BC | No | N/A | N/A | Candidate values: {1,1.5,2} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-20 | CPU counting for NTRP>1 for Rel-17-based CJT type-II codebook | - Value of X for CPU occupation (OCPU = X.NTRP) |  | Yes | N/A |  | Per-band and per BC | No | N/A | N/A | Candidate values: {1,1.5,2} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-21 | Minimum CSI calculation delay requirements for NTRP>1 for AP-CSI reporting for Rel-16-based CJT type-II codebook | - Supported minimum delay requirements |  | Yes | N/A |  | Per-band and per BC | No | N/A | N/A | Candidate values: {(Z2, Z'2), Z2+Z'2, 2Z'2)} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-22 | Minimum CSI calculation delay requirements for NTRP>1 for AP-CSI reporting for Rel-16-based CJT type-II codebook | - Supported minimum delay requirements |  | Yes | N/A |  | Per-band and per BC | No | N/A | N/A | Candidate values: {(Z2, Z'2), Z2+Z'2, 2Z'2)} | Optional with capability signaling |

    - Doppler CSI

| 40. NR_MIMO_evo_DL_UL | 40-3-2-1 | Support of Rel-16-based doppler CSI | 1. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 2. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 3. Support parameter combinations with L=2,4 4. Support for rank = 1,2 5. A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously [6. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting] 7. Value of Y for CPU occupation (OCPU = Y.N4), when P/SP-CSI-RS is configured for CMR 8. Value of Y for CPU occupation (OCPU = Y.K), when A-CSI-RS is configured for CMR 9. Support for the size of DD-basis, N4=1 10. Scaling factor for active resource counting Kp | 2-35 | Yes | N/A | Rel-16 based Type II doppler codebook is not supported | Per-band and Per-BC | N/A | N/A | N/A | Component 7 candidate values: {1, 2, 3} Component 8 candidate values: {1, 2, 3} Component 10 candidate values: {1, 2, 4} Note: When N4=1, OCPU =4 Note: OCPU ≥ 4 when P/SP-CSI-RS is configured for CMR Note: when K=12, OCPU =8 Note: A UE that supports CSI enhancement for Rel. 16 based type-II doppler must support this FG | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-1a | Support of Rel-16-based doppler measurement with N4>1 | 1. Support for the size of DD-basis, N4>1 [2. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously] [3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting] | 40-3-2-1 | Yes | N/A | Rel-16 based Type II doppler codebook with N4>1 is not supported | Per-band and Per-BC | N/A | N/A | N/A |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-2 | Support R=2 for Rel-16-based doppler codebook | A list of supported combinations {[Max N4,] Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}, across all CCs simultaneously, with R=2 | 40-3-2-1 | Yes | N/A | R=2 for Rel-16-based doppler codebook is not supported | Per band and per BC | N/A | N/A | N/A | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-3 | Support X=1 based on first and last slot of WCSI, for Rel-16-based doppler codebook | Support of TDCQI = ‘1-2’ | 40-3-2-1 | Yes | N/A | X=1 based on first and last slot of WCSI, for Rel-16-based doppler codebook is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-3a | Support X=2 CQI based on 2 slots for Rel-16-based doppler codebook | Support of TDCQI = ‘2’ | 40-3-2-1 | Yes | N/A | X=2 CQI based on 2 slots for Rel-16-based doppler codebook is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-4 | Support of Rel-17-based doppler measurement | 1. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI 2. Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with PMI subband R=1 3. Support of parameter combinations with M = 1 4. Support for rank = 1,2 5: A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one doppler CSI measurement [6. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting] 7. Value of Y for CPU occupation (OCPU = Y.K), when A-CSI-RS is configured for CMR 8. Support for N4=1 9. Scaling factor for active resource counting Kp | 40-3-2-1, 2-35 | Yes | N/A | Rel-17 based Type II doppler codebook is not supported | Per-band and Per-BC | N/A | N/A | N/A | Component 7 candidate values: {1, 2, 3} Component 9 candidate values: {1, 2, 4} Note: OCPU = 4 when P/SP-CSI-RS is configured for CMR Note: when K=12, OCPU =8 Note: A UE that supports CSI enhancement for Rel. 17-based type-2 doppler must support this FG | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-5 | Support of M=2 and R=1 for Rel-17-based doppler codebook | 1. Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with M=2 and PMI subband R=1 2. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource b) Maximum total number of NZP CSI-RS resource c) Maximum total number of Tx ports of NZP CSI-RS resources | 40-3-2-4 | Yes | N/A | M=2 and R=1 for Rel-17-based doppler codebook is not supported | Per-band and Per-BC | N/A | N/A | N/A | Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-6 | Support R=2 for Rel-17-based doppler codebook | 1. Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with PMI subbands R=2 2. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource b) Maximum total number of NZP CSI-RS resource c) Maximum total number of Tx ports of NZP CSI-RS resources | 40-3-2-4 | Yes | N/A | R=2 for Rel-17-based doppler codebook is not supported | Per band and per BC | N/A | N/A | N/A | Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} d) {2,3,4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-7 | support of l = (n – nCSI,ref ) for CSI reference slot for Rel-16 based doppler codebook | 1. Support of l = (n – nCSI,ref ) for CSI reference slot when N4=1 and d>1 2. Support of l = (n – nCSI,ref ) for CSI reference slot when N4>1 | 40-3-2-1 | Yes | N/A | l = (n – nCSI,ref ) for CSI reference slot for Rel-16 based doppler codebook is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-7a | Support of l = (n – nCSI,ref ) for CSI reference slot for Rel-17 based doppler codebook | 1. Support of l = (n – nCSI,ref ) for CSI reference slot when N4=1 and d>1 | 40-3-2-4 | Yes | N/A | l = (n – nCSI,ref ) for CSI reference slot for Rel-17 based doppler codebook is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |

40-3-2-1:

- Components:

  - For Component 6, it was argued that this is for UE’s memory limitation, while component 5 is for UE’s per-slot limitation. We are ok to remove the bracket (i.e., confirm to include this component).

  - Capture default Z’=2*Z2’ (i.e., r=Z2’)

Agreement (RAN1#113) For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z/Z’ - For N4=1: reuse legacy Z’ values - For N4>1, introduce two UE capabilities: - Capability 1: Reuse legacy Z’ values - Capability 2: Legacy Z’ values + r - The value(s) of r>0 can depend on the configured N4 value - FFS: exact value(s) of r Note: Since this pertains Type-II, the relevant values are Z2/Z2’ Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z/Z’ for Capability 2, r=legacy Z2’

  - Capture default Z:

    - Z=Z2+w+r for P/SP CSI-RS, where w values need to be discussed further in UE feature session and r value is equal to legacy Z2’

    - Z=Z2+14.(K–1).m + r

Agreement For the Rel-18 Type-II codebook refinement for high/medium velocities, regarding Z - Based on the two UE capabilities agreed for Z’: - Capability 1: - For AP CSI-RS: Z=legacy Z+14.(K–1).m - For P/SP CSI-RS: Z= legacy Z+w where w>0 - TBD: Value of w - Capability 2: - For AP CSI-RS: Z= legacy Z+14.(K–1).m + r - For P/SP CSI-RS: Z= legacy Z+w+r - Note: r corresponds to the agreed value for Z’ relaxation in previous agreement. Note: Since this pertains Type-II, the relevant values are Z2/Z2’

40-3-2-1a:

- Component 2: Ok to confirm

- Component 3: Ok to confirm

40-3-2-2:

- Component: Ok to confirm the yellow part

40-3-2-4:

- Components: Ok to confirm the yellow part

    - TDCP

| 40. NR_MIMO_evo_DL_UL | 40-3-3-1 | TDCP (Time Domain Channel Properties) report | 1. Support of Y=1 delay value for TDCP report 2. Basic delay value, component candidate value <= D_basic = 1 slot 3. Support of amplitude report 4. Value of X for CPU occupation (OCPU=(Y+1).X) 5. Support to configure KTRS = 1 TRS resource set 6. Maximum number of simultaneously active CSI-RS resources for TDCP across all CCs Note: counting of simultaneously active CSI-RS resources follows existing specification TS38.214 | [2-35] | Yes | N/A | Time Domain Channel Properties report is not supported | [Per band] | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-2 | Number of delay values | Number Y>1 of delay values for which TDCP is reported | 40-3-3-1 | Yes | N/A | TDCP is not reported for more than 1 delay value | [Per-band Per-BC] | N/A | N/A | N/A | Candidate values: {2,3,4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-1a | Supported maximum delay value larger than D_basic | Support of maximum delay value larger than D_basic =1 slot | 40-3-3-1 | Yes | N/A | delay value(s) larger than D_basic are not supported | [Per-band Per-BC] | N/A | N/A | N/A | Candidate values: {2 slots, 3 slots, 4 slots, 5 slots, 6 slots, 10 slots} [Note: 10 slots is only applicable for SCS >= 30 kHz, and 6 slots is maximum for SCS = 15 kHz] | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-4 | Phase report | Support of phase report for Y>=1 | 40-3-3-1 | Yes | N/A | Phase report for Y>=1 is not supported | [Per-band Per-BC] | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-5 | Number of CSI-RS resources for TDCP | 1. Maximum number of configured CSI-RS resources for TDCP per CC 2. Maximum number of configured CSI-RS resources for TDCP across all CCs 3. Maximum number of simultaneously active CSI-RS resources for TDCP per CC Note: counting of simultaneously active CSI-RS resources follows existing specification TS38.214 | 40-3-3-1 | Yes | N/A | Number of CSI-RS resources for TDCP is not reported | [Per-band Per-BC] | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-6 | Maximum number of TRS resource sets in a report configuration | Max number of TRS resource sets in a single CSI-RS resource setting | 40-3-3-1 | Yes | N/A | More than 1 TRS resource set in a report configuration is not supported | [Per-band Per-BC] | N/A | N/A | N/A | Candidate values: {2,3} | Optional with capability signaling |

40-3-3-1:

- Prerequisite FG: Ok to confirm 2-35

- Type: per-band. In our view, this feature does not require “per-BC” granularity

40-3-3-1a:

- Note: We believe the Note should be confirmed as it is the intention of the agreement

- Type: per-band, following 40-3-3-1

All the other FGs:

- Type: Per band, following 40-3-3-1

DMRS

| 40. NR_MIMO_evo_DL_UL | 40-4-1 | Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type A | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol | [2-5] | Yes | n/a |  | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-4-1a | Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type B | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol | [2-5] | Yes | n/a | UE does not support basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type B | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-1b | 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH | Support of 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH | 40-4-1 | Yes | n/a | UE does not support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-1c | Alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH | Support of alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH | 40-4-1, 5-28 | Yes | n/a | UE does not support alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-1d | 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH | Support of 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH | 40-4-1 | Yes | n/a | UE does not support 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-1e | 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH | Support of 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH | 40-4-1 | Yes | n/a | UE does not support 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-1f | 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH | Support of 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH | 40-4-1 | Yes | n/a | UE does not support 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-1g | DMRS type for Rel.18 enhanced DMRS ports for PUSCH | Support of DMRS type for Rel.18 enhanced DMRS ports for PUSCH | 40-4-1 | Yes | n/a | UE does not support DMRS type for Rel.18 enhanced DMRS ports for PUSCH | [Per FS] | No | No | n/a | Component 1 candidate values: {etype 1, both etype 1 and etype 2} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-1h | 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 | Support of 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 | 40-4-1 | Yes | n/a | 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 is not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-1i | 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 | Support of 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 | 40-4-1, FFS more | Yes | n/a | 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 is not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-1j | Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port for mapping type A | Support of Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port for mapping type A | 40-4-1 | Yes | n/a | Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port is not supported for mapping type A | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-4 | Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports | Support reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports | 40-4-1 | Yes | n/a | Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports is not supported | [Per FS] | No | No | n/a | Note: If this feature is not supported, UE expects that gNB shall apply at least the following scheduling restriction for PDSCH for FD-OCC 4 in Rel.18 eType 1 DMRS 1) The number of consecutively scheduled PRBs for PDSCH is even 2) The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-5 | Rel-18 DL DMRS with single DCI based M-TRP | Support of Rel-18 DL DMRS with single DCI based M-TRP | 40-4-1, FFS more | Yes | n/a | Rel-18 DL DMRS with single DCI based M-TRP is not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-5a | Additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP | Support of additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP | 40-4-5 | Yes | n/a | Additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP are not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-7 | Rel-18 DL DMRS with M-DCI based M-TRP | Support of Rel-18 DL DMRS with multi- DCI based M-TRP PDSCH operation | FFS | Yes | n/a | Rel-18 DL DMRS with M-DCI based M-TRP is not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-6 | Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbols 3) Support 1 symbol FL DMRS and 2 additional DMRS symbols | [2-16] | Yes | n/a | Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports is not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-6a | Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol | [2-16a] | Yes | n/a | Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports is not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-6c | DMRS type for Rel.18 enhanced DMRS ports for PUSCH | Support of DMRS type for Rel.18 enhanced DMRS ports for PUSCH | 40-4-6 | Yes | n/a | DMRS type for Rel.18 enhanced DMRS ports for PUSCH is not supported | [Per FS] | No | No | n/a | Component candidate values: {eType 1, both eType 1 and eType 2} [Note: A UE supporting one of FG 40-4-6 or FG 40-4-6a must support this FG] | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-6d | 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH | Support of 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH | 40-4-6 | Yes | n/a | 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH is not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-6e | 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH | Support of 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH | 40-4-6 | Yes | n/a | 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH is not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-6f | 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH | Support of 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH | 40-4-6 | Yes | n/a | 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH is not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-6g | 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 | Support of 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 | 40-4-6, FFS more | Yes | n/a | 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 is not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-6h | 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 | Support of 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 | 40-4-6, FFS more | Yes | n/a | 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 is not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-6i | 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 | Support of 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 | 40-4-6, FFS more | Yes | n/a | 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 is not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-6j | 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 | Support of 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 | 40-4-6, FFS more | Yes | n/a | 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 is not supported | [Per FS] | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-10 | DMRS port configuration for PUSCH [with rank 5-8/ with 8Tx] | DMRS port configuration for PUSCH [with rank 5-8 / with 8Tx] for Rel 15 and Rel. 18 | [40-4-6] | Yes | n/a | Port configuration for PUSCH [with rank 5-8 / with 8Tx] is not supported | [Per FS] | No | No | n/a | Candidate values: {Rel. 15 DMRS, Rel. 15 DMRS and Rel. 18 DMRS} [Note: A UE supporting 8 Tx must support this FG] | Optional with capability signaling |

40-4-1a

- Pre-requisite FG: It should be FG2-6 (TypeB), not FG2-5 (TypeA).

40-4-1g

- Note: If UE supports Rel.18 DMRS ports but not report this FG, it is not clear which DMRS type UE supports. Hence, we suggest to add the following note, to avoid the case UE does not report this FG:

  - Note: A UE supporting at least one of FG 40-4-1 or FG 40-4-1a must support this FG.

- Pre-requisite FG: FG2-5 is fine.

40-4-1a

- Pre-requisite FG: FG2-5 should be removed. Instead, FG2-6 (Basic DL DMRS for scheduling type B) should be noted.

40-4-1h

- Note: Since 1-port DL PTRS (FG2-46) is mandatory in FR2 in Rel.15, we suggest to add the following note, to avoid the situation UE does not support this FG in FR2:

  - Note: A UE supporting FG2-46 and at least one of FG 40-4-1 or FG 40-4-1a must support this FG.

40-4-5

- Pre-requisite FG: We suggest the following update to add FGs for single DCI multi-TRP (16-2b-1/16-2b-2/16-2b-3/16-2b-4/16-2b-5):

  - At least one of 40-4-1/40-4-1a, and at least one of 16-2b-1/16-2b-2/16-2b-3/16-2b-4/16-2b-5.

40-4-7

- Pre-requisite FG: We suggest the following update to add FGs for multi DCI multi-TRP (16-2a):

  - At least one of 40-4-1/40-4-1a, and 16-2a.

40-4-6c

- Note: If UE supports Rel.18 DMRS ports but not report this FG, it is not clear which DMRS type UE supports. Hence, we suggest to support the following note:

  - [Note: A UE supporting one of FG 40-4-6 or FG 40-4-6a must support this FG]

40-4-6g

- Note: Since 1-port UL PTRS (FG2-47) is mandatory in FR2 in Rel.15, we suggest to add the following note, to avoid the situation UE does not support this FG in FR2:

  - Note: A UE supporting FG2-47 and at least one of FG 40-4-6 or FG 40-4-6a must support this FG.

40-4-10

- Component: For the yellow highlighted part, we think it should be “with 8Tx”. Even if 8Tx is configured, UE can be dynamically indicated with rank 1-4. In this case, it is not clear which FG is used if it is noted as “with rank 5-8”. For clarification, we suggest to updated as:

  - DMRS port configuration for PUSCH [with rank 5-8/ configured with 8Tx].

- Pre-requisite FG: FG40-4-6 should be removed. It is possible that UE only supports Rel.15 DMRS ports but UE supports 8Tx. Instead, FG40-7-1 (Basic FG for 8Tx) can be pre-requisite FG.

- Note: We support to have the note.

SRS

| 40. NR_MIMO_evo_DL_UL | 40-5-1 | SRS comb offset hopping | Support of SRS comb offset hopping | FFS | Yes | n/a | SRS comb offset hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-5-2 | SRS cyclic shift hopping | Support of SRS cyclic shift hopping | [2-53] | Yes | n/a | SRS cyclic shift hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-1b | SRS comb offset hopping combined with legacy group/sequence hopping | Support of SRS comb offset hopping combined with legacy group/sequence hopping | 40-5-1 | Yes | n/a | SRS comb offset hopping combined with legacy group/sequence hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-2b | SRS cyclic shift hopping combined with legacy group/sequence hopping | Support of SRS cyclic shift hopping combined with legacy group/sequence hopping | 40-5-2 | Yes | n/a | SRS cyclic shift hopping combined with legacy group/sequence hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-3 | SRS cyclic shift hopping combined with SRS comb offset hopping | Support of SRS cyclic shift hopping combined SRS comb offset hopping | 40-5-1, 40-5-2 | Yes | n/a | SRS cyclic shift hopping combined with SRS comb offset hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-1a | Comb offset hopping time-domain behavior when repetition factor R>1 | Supported comb offset hopping granularity in time when repetition factor R>1 is configured | 40-5-1 | Yes | n/a |  | Per band | n/a | n/a | n/a | Component candidate values: {‘per SRS symbol’,’per R SRS symbols’, ‘both’} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-4 | SRS without TDMed 8 Tx ports—antenna switching | Support of SRS with 8 Tx ports—antenna switching FFS: detailed components FFS: separate row for comb8 |  | Yes | n/a | SRS with 8 Tx ports—antenna switching is not supported | [Per band] | n/a | n/a | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-4a | SRS without TDMed 8 Tx ports—codebook | Support of SRS with 8 Tx ports—codebook FFS: detailed components FFS: separate row for comb8 |  | Yes | n/a | SRS with 8 Tx ports—codebook is not supported | [Per band] | n/a | n/a | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-6 | SRS with TDMed 8 Tx ports—antenna switching | Support of SRS with TDMed 8 Tx ports—antenna switching FFS: detailed components FFS: separate row for comb8 |  | Yes | n/a | SRS with TDMed 8 Tx ports—antenna switching is not supported | [Per band] | n/a | n/a | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-7 | SRS with TDMed 8 Tx ports—codebook | Support of SRS with TDMed 8 Tx ports—codebook FFS: detailed components FFS: merge with antenna switching |  | Yes | n/a | SRS with TDMed 8 Tx ports—codebook is not supported | [Per band] | n/a | n/a | n/a |  | Optional with capability signaling |

40-5-1:

- Prerequisite FG: Not essential, but if it is needed, we think it should be 2-53

40-5-2:

- Prerequisite FG: Ok to confirm 2-53

40-5-4/6:

- Components:

  - The following components should be captured:

    - 1) Support SRS Tx port switch,

    - 2) Report whether the uplink TX switching impact to downlink receiving in a band,

    - 3) Report whether the UL Tx is switched together with UL Tx in another band

  - Separate row for comb8 isn’t necessary. Comb8 is per-band capability, thus, once this FG is supported for a band combination in which all the corresponding bands support comb8 (FG 23-8-8), the support of FG40-5-4/-5 for comb8 is implied.

- Type: Per-BC

40-5-4a/7:

- Components: Other than “Support of SRS with TDMed 8 Tx ports—codebook”, no component is needed

- Type: Per-band

Others:

- Add a new FG for cyclic shift offset hopping with finer time-delay-domain granularity

  - Prerequisite: FG 40-5-2

  - Type: Per band

- Add a new FG for hopping subset configuration for each of comb offset hopping and cyclic shift hopping (a FG for each)

  - Prerequisite: FG 40-5-1 and 40-5-2

  - Type: per band

STxMP

| 40. NR_MIMO_evo_DL_UL | 40-6-1 | Single-DCI based STx2P SDM scheme for PUSCH—codebook | 1. Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—codebook 2. 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—codebook 3. Support of two SRS resource sets with usage set to 'codebook' 4. Maximum number of SRS resources in one SRS resource set 5. Maximum number of layers of each panel for Single-DCI STx2P with SDM [6. Maximum total number of layers across both panels for Single-DCI STx2P with SDM] [7. Max number of PUSCH ports associated with on SRS resource set] | 2-14 | Yes | N/A | Single-DCI based STx2P SDM scheme for PUSCH—codebook is not supported | [Per FSPC] | No | FR2 only | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-6-1-1 | 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook | Support of 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook | 40-6-1 | Yes | N/A | 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook is not supported | [Per FSPC] | No | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-1a | Single-DCI based STx2P SDM scheme for PUSCH—noncodebook | 1. Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—noncodebook 2. 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—noncodebook 3. Support of two SRS resource sets with usage set to 'noncodebook' 4. Maximum number of SRS resources in one SRS resource set 5. Maximum number of layers of each panel for Single-DCI STx2P with SDM [6. Maximum total number of layers across both panels for Single-DCI STx2P with SDM] [7. Maximum number of simultaneous transmitted SRS resources at one symbol] [8. Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol] | FFS | Yes | N/A | Single-DCI based STx2P SDM scheme for PUSCH—noncodebook is not supported | [Per FSPC] | No | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-1a-1 | 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook | Support of 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook | 40-6-1a | Yes | N/A | 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook is not supported | [Per FSPC] | No | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-1b | Association between CSI-RS and SRS for noncodebook single-DCI based STx2P SDM scheme for PUSCH | 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STxMP SDM scheme for PUSCH 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously | [2-15a, 40-6-1a | Yes | N/A | Associated CSI-RS resources for non-codebook single-DCI based STxMP SDM scheme for PUSCH is not supported | [Per Band] | n/a | FR2 only | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-6-1-2 | New DMRS port entry for single-DCI based SDM scheme | Support of new DMRS port entry {0, 2, 3} | 40-6-1,40-6-1a | Yes | N/A | New DMRS port entry for single-DCI based SDM scheme is not supported | [Per Band] | n/a | FR2 only | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-6-2 | Single-DCI based STx2P SFN scheme for PUSCH—codebook | 1. Support of single-DCI based STx2P SFN scheme for codebook based PUSCH 2.Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP 3. 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—codebook 4. Support of two SRS resource sets with usage set to 'codebook' 5. Supported number of SRS resources in one SRS resource set 6. Supported maximal number of MIMO layers of each SRS resource set for CB PUSCH with SFN scheme [7. Maximum number of SRS antenna ports for each SRS resource in each SRS resource set] [8. Maximum total number of layers across both panels for Single-DCI STx2P with SFN] [9. Max number of PUSCH ports associated with SRS resource set] | 2-14 | Yes | n/a | Single-DCI based STx2P SFN scheme for PUSCH—codebook is not supported | [Per FS] | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-2-1 | 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook | Support of 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook | 40-6-2 | Yes | n/a | 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook is not supported | [Per FS] | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-2a | Single-DCI based STx2P SFN scheme for PUSCH—noncodebook | 1. Support of single-DCI based STx2P SFN scheme for noncodebook based PUSCH 2.Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP 3. 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—noncodebook 4. Support of two SRS resource sets with usage set to 'noncodebook' 5. Supported number of SRS resources in one SRS resource set 6. Supported maximal number of MIMO layers of each SRS resource set for NCB PUSCH with SFN scheme [7. Maximum number of simultaneous transmitted SRS resources at one symbol] [8. Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol] | FFS | Yes | n/a | Single-DCI based STx2P SFN scheme for PUSCH—noncodebook is not supported | [Per FS] | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-2a-1 | 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook | Support of 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook | 40-6-2a | Yes | n/a | 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook is not supported | [Per FS] | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-2b | Association between CSI-RS and SRS for noncodebook single-DCI based STx2P SFN scheme for PUSCH | 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STxMP SFN scheme for PUSCH 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously | 2-15a | Yes | n/a | Associated CSI-RS resources for non-codebook single-DCI based STxMP SFN scheme for PUSCH is not supported | [Per Band] | n/a | FR2 only | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3a | codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG | 1. Support of multi-DCI based STx2P PUSCH+PUSCH for codebook-based PUSCH with fully[/partial] overlapping PUSCHs in time and [non-overlapping in frequency] 2. Support of two SRS resource sets with usage set to 'codebook' associated with two coresetPoolIndex values 3. Maximum number of SRS resources in one SRS resource set 4. Maximum number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain [5. Supported number of NZP PUSCH ports for each PUSCH of PUSCH+PUSCH overlapping in time domain] 6. Maximum number of PUSCHs per CORESETPoolIndex per slot 7. Maximum total number of layers across two overlapping PUSCH - 8. Maximum number of SRS antenna ports for each SRS resource in each SRS resource set | 2-14 | Yes | N/A | Codebook multi-DCI based STx2P PUSCH+PUSCH is not supported | [Per Band] | n/a | FR2 only | n/a | Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex. Component 3 candidate values: {1,2,4} Note: If value 4 is reported for component 3, UE also reports value 4 in FG 16-5c. Component 4 candidate values: {1,2} [Component 5 candidate values: {1,2,4} Note: If a row of the TPMI consists of all 0’s, the corresponding PUSCH port is not counted.] Component 6 candidate values: {1,2,3,4,7} Note: per SCS, similar with Rel-15 Component 7 candidate values: {2 ,3, 4} Component 8 candidate values: {1, 2 ,4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3c | [Codebook multi-DCI based STx2P PUSCH+PUSCH – Partial-overlapping PUSCHs in time and non-overlapping in frequency] | [Support of partial-overlapping PUSCHs in time and non-overlapping in frequency] | [40-6-3a] | [Yes] | [N/A] | [Partial-overlapping PUSCHs in time and non-overlapping in frequency for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a] |  | [Optional with capability signaling] |
| 40. NR_MIMO_evo_DL_UL | 40-6-3d | [Codebook multi-DCI based STx2P PUSCH+PUSCH – Overlapping PUSCHs in time and partially overlapping in frequency] | [Support of overlapping PUSCHs in time and partially overlapping in frequency] | [40-6-3a] | [Yes] | [N/A] | [Overlapping PUSCHs in time and partially overlapping in frequency for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a] |  | [Optional with capability signaling] |
| 40. NR_MIMO_evo_DL_UL | 40-6-3e | [Codebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in both frequency and time] | [Support of fully overlapping in both frequency and time] | [40-6-3a] | [Yes] | [N/A] | [Fully overlapping in both frequency and time for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a] |  | [Optional with capability signaling] |
| 40. NR_MIMO_evo_DL_UL | 40-6-3f | [Codebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in frequency, partially overlapping in time] | [Support of fully overlapping in frequency, partially overlapping in time] | [40-6-3a] | [Yes] | [N/A] | [Fully overlapping in frequency, partially overlapping in time for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a] |  | [Optional with capability signaling] |
| 40. NR_MIMO_evo_DL_UL | 40-6-3g | [Codebook multi-DCI based STx2P PUSCH+PUSCH – Partially or non-overlapping in frequency, partially or fully overlapping in time] | [Support of partially or non-overlapping in frequency, partially or fully overlapping in time] | [40-6-3a] | [Yes] | [N/A] | [Partially or non-overlapping in frequency, partially or fully overlapping in time for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a[ |  | [Optional with capability signaling] |
| 40. NR_MIMO_evo_DL_UL | 40-6-3h | Codebook multi-DCI based STx2P PUSCH+PUSCH for CG+CG | Support of multi-DCI based STxMP CG-PUSCH+CG-PUSCH [FFS: other components] | [40-6-3a] | Yes | N/A | Multi-DCI based STx2P for CG+CG is not supported for codebook | [Per Band] | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3i | Codebook multi-DCI based STx2P PUSCH+PUSCH for DG+CG | Support of multi-DCI based STxMP DG-PUSCH+CG-PUSCH [FFS: other components] | [40-6-3a] | Yes | N/A | Multi-DCI based STx2P for DG+CG is not supported for codebook | [Per Band] | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3b | Noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG | 1. Support of multi-DCI based STxMP PUSCH+PUSCH for noncodebook-based PUSCH with fully[/partial] overlapping PUSCHs in time and [non-overlapping in frequency] 2. Support of two SRS resource sets with usage set to 'noncodebook' associated with two coresetPoolInde values 3. Maximum number of SRS resources in one SRS resource set 4. Maximum number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain [5. Maximum number of simultaneously transmitted SRS resources in one symbol per SRS resource set] 6. Maximum number of PUSCHs per CORESETPoolIndex per slot 7. Maximum total number of layers across two overlapping PUSCH | 2-14 | Yes | N/A | Noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported | [Per Band] | n/a | FR2 only | n/a | Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex. Component 3 candidate values: {1,2,[3,]4} Component 4 candidate values: {1,2} [Component 5 candidate values: {1,2,[3,4]}] Component 6 candidate values: {1,2,3,4,7} Note: per SCS, similar with Rel-15 Component 7 candidate values: {2 ,3, 4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3j | [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Partial-overlapping PUSCHs in time and non-overlapping in frequency] | [Support of partial-overlapping PUSCHs in time and non-overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH] | [40-6-3a] | [Yes] | [N/A] | [Partial-overlapping PUSCHs in time and non-overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a] |  | [Optional with capability signaling] |
| 40. NR_MIMO_evo_DL_UL | 40-6-3k | [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Overlapping PUSCHs in time and partially overlapping in frequency] | [Support of overlapping PUSCHs in time and partially overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH] | [40-6-3a] | [Yes] | [N/A] | [Overlapping PUSCHs in time and partially overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a] |  | [Optional with capability signaling] |
| 40. NR_MIMO_evo_DL_UL | 40-6-3l | [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in both frequency and time] | [Support of fully overlapping in both frequency and time for noncodebook multi-DCI based STx2P PUSCH+PUSCH] | [40-6-3a] | [Yes] | [N/A] | [Fully overlapping in both frequency and time for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a] |  | [Optional with capability signaling] |
| 40. NR_MIMO_evo_DL_UL | 40-6-3m | [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in frequency, partially overlapping in time] | [Support of fully overlapping in frequency, partially overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH] | [40-6-3a] | [Yes] | [N/A] | [Fully overlapping in frequency, partially overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a] |  | [Optional with capability signaling] |
| 40. NR_MIMO_evo_DL_UL | 40-6-3n | [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Partially or non-overlapping in frequency, partially or fully overlapping in time] | [Support of partially or non-overlapping in frequency, partially or fully overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH] | [40-6-3a] | [Yes] | [N/A] | [Partially or non-overlapping in frequency, partially or fully overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a] |  | [Optional with capability signaling] |
| 40. NR_MIMO_evo_DL_UL | 40-6-3o | Noncodebook multi-DCI based STx2P PUSCH+PUSCH for CG+CG | Support of multi-DCI based STxMP CG-PUSCH+CG-PUSCH for noncodebook [FFS: other components] | [40-6-3a] | Yes | N/A | Multi-DCI based STx2P for CG+CG is not supported for noncodebook is not supported | [Per Band] | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3p | Noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+CG | Support of multi-DCI based STxMP DG-PUSCH+CG-PUSCH for noncodebook [FFS: other components] | [40-6-3a] | Yes | N/A | Multi-DCI based STx2P for DG+CG is not supported for noncodebook is not supported | [Per Band] | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3b-1 | Associated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH |  |  |  |  |  |  |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-6-4 | Single-DCI based STx2P SFN scheme for PUCCH | 1. Support of single-DCI based STx2P SFN scheme for PUCCH 2. Supported PUCCH formats for STxMP SFN scheme | 23-3-2d | Yes | n/a | Single-DCI based STx2P SFN scheme for PUCCH is not supported | [Per FS] | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-5 | Support grouped-based beam reporting for STx2P |  |  |  |  |  |  |  |  |  |  |  |

40-6-1

- Component

  - Candidate value of component 4: {1,2,4}

  - Candidate value of component 5: {1,2}

  - Component 6 seems duplicated with component 5 and should be removed.

  - Support component 7 with candidate value as {1,2,4}

  - In addition, another component max number of SRS ports of each SRS resource should be included with candidate value as {1,2,4}

40-6-1a

- Component:

  - Candidate value of component 4: {1,2,3,4}

  - Candidate value of component 5: {1,2}

  - Component 6 seems duplicated with component 5 and should be removed.

  - Support component 8 with candidate value as {1,2,3,4}

  - For component 7, we have not agreed on simultaneous transmission of two SRS resource sets. It can be discussed later.

- Prerequisite FG: 2-15 (NCB based PUSCH).

40-6-1b:

- Component:

  - Candidate value of component 2 can be {1 to 8}.

  - Candidate value of component 3 can be {1 to 8}.

  - Candidate value of component 4 can be {0 to 8}.

  - Candidate value of component 5 can be {1 to 16}.

  - Candidate value of component 6 can be {1, 2}.

40-6-2

- Component:

  - Candidate value of component 5: {1,2,4}

  - Candidate value of component 6: {1,2}

  - Support component 7 with candidate value as {1,2,4}

  - Component 8 seems duplicate with component 6 and should be removed.

  - Support component 9 with candidate value as {1,2,4}

40-6-2a

- Component:

  - Candidate value of component 5: {1,2,3,4}

  - Candidate value of component 6: {1,2}

  - Support component 8 with candidate value as {1,2,3,4}

  - For component 7, we have not agreed on simultaneous transmission of two SRS resource sets. It can be discussed later.

- Prerequisite FG: 2-15.

40-6-2b

- Component:

  - Candidate value of component 2 can be {1 to 8}.

  - Candidate value of component 3 can be {1 to 8}.

  - Candidate value of component 4 can be {0 to 8}.

  - Candidate value of component 5 can be {1 to 16}.

  - Candidate value of component 6 can be {1, 2}.

40-6-3a

- Component:

  - Support full/partial overlapping in time and non-overlapping in frequency in 40-6-3a.

  - Support component 5.

40-6-3c/3d/3e/3f/3g.

- Support one separate row for full/partial overlapping in time and partial overlapping in frequency, and one separate row for full/partial overlapping in time and full overlapping in frequency, i.e., keep 40-6-3d, merge 40-6-3e and 40-6-3f, remove 40-6-3c and 40-6-3g.

40-6-3b

- Component:

  - Support full/partial overlapping in time and non-overlapping in frequency in 40-6-3b.

  - Support component 5.

- Prerequisite FG: 2-14 should be replaced by 2-15.

40-6-3j/3k/3l/3h/3n

- Support one separate row for full/partial overlapping in time and partial overlapping in frequency, and one separate row for full/partial overlapping in time and full overlapping in frequency, i.e., keep 40-6-3k, merge 40-6-l and 40-6-3m, remove 40-6-3j and 40-6-3n.

40-6-3b-1

- Component: same components as 40-6-1b and 40-6-2b should be included.

  - 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STxMP SFN scheme for PUSCH

  - 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP. Candidate value {1 to 8}.

  - 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP. Candidate value {1 to 8}

  - 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP. Candidate value {0 to 8}

  - 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS. Candidate value {1 to 16}

  - 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously. Candidate value {1,2}

- Prerequisite FG: 40-6-3b, 2-15a

40-6-5

- Component: similar components as Rel-17 group based beam reporting can be included. Following components can be included.

  - Max number of beam groups in a beam reporting. Candidate values: {1,2,3,4}.

  - Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs. Candidate values: {2,3,4,8,16,32,64}

  - Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs. Candidate values: {8,16,32,64,128}

8TX

| 40. NR_MIMO_evo_DL_UL | 40-7-1 | Basic features for Codebook-based 8Tx PUSCH | 1. Supported maximal PUSCH MIMO layers for codebook based PUSCH 2. Supported maximum number of 8 port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 8Tx PUSCH | FFS | Yes | n/a | Codebook-based 8Tx PUSCH is not supported | [Per FSPC] | No | No | No | A UE that supprts FG 40-7-1 must support at least one of FGs 40-7-1a/b/c/d | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-7-1a | Codebook-based 8Tx PUSCH—[full-coherent codebook] | Support of codebook-based 8Tx PUSCH—[full-coherent codebook] | 40-7-1 | Yes | n/a | Codebook-based 8Tx PUSCH—[full-coherent] codebook is not supported | [Per FSPC] | No | No | No | FFS: whether to (N1,N2) as component or new row | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1b | (N1,N2) for coherent codebook Codebook-based 8Tx PUSCH—[partial-coherent codebook], Ng=2 | Support of codebook-based 8Tx PUSCH—[partial-coherent codebook], Ng=2 | 40-7-1 | Yes | n/a | Codebook-based 8Tx PUSCH—[partial-coherent codebook], Ng=2 is not supported | [Per FSPC] | No | No | No |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1c | Codebook-based 8Tx PUSCH— [partial-coherent codebook], Ng=4 | Support of codebook-based 8Tx PUSCH— [partial-coherent codebook], Ng=4 | 40-7-1 | Yes | n/a | Codebook-based 8Tx PUSCH— [partial-coherent codebook], Ng=4 is not supported | [Per FSPC] | No | No | No |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1d | Codebook-based 8Tx PUSCH— [non-coherent codebook] | Support of codebook-based 8Tx PUSCH— [non-coherent codebook] | 40-7-1 | Yes | n/a | Codebook-based 8Tx PUSCH— [non-coherent codebook] is not supported | [Per FSPC] | No | No | No |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1e | UL full power transmission mode 0 | Support of UL full power transmission mode of fullpower when UE is capable of 8 Tx codebook based PUSCH operation | FFS | yes | n/a | FFS | [Per FSPC] | n/a | n/a | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1f | UL full power transmission mode 1 | Support of UL full power transmission mode of fullpowerMode1 when UE is capable of 8 Tx codebook based PUSCH operation This is a WA | FFS | yes | n/a | FFS | [Per FSPC] | n/a | n/a | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1g | UL full power transmission mode 2 | 1. Support of UL full power transmission mode of fullpowerMode2 when UE is capable of 8 Tx codebook based PUSCH operation [2. Maximum number of SRS resources in one SRS resource set with usage set to 'codebook' for 8Tx codebook based PUSCH for Mode 2] This is a WA | FFS | yes | n/a | FFS | [Per FSPC] | n/a | n/a | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-7-2 | Basic features for Non-Codebook-based 8Tx PUSCH | 1. Supported maximal PUSCH MIMO layers for non-codebook based PUSCH 2. Supported maximum number of SRS resources per SRS resource set with usage set to 'nonCodebook’ 3. Maximum number of simultaneous transmitted SRS resources at one symbol | FFS | Yes | n/a | Non-codebook based 8Tx PUSCH is not supported | [Per FSPC] | No | No | No |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-2a | Association between CSI-RS and SRS for non-codebook case | 1. Support association between NZP-CSI-RS and SRS resource set via RRC parameter "SRS-ResourceSet" for noncodebook 8Tx PUSCH operation [2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources, and total # of Tx ports} across all CCs simultaneously.] | FFS | yes | n/a | Association between CSI-RS and SRS for non-codebook case is not supported | [Per FSPC] | No | No | No |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-7-3 | CBG based 2 CWs PUSCH with rank >4 | Support CBG based transmission for 2 CWs PUSCH | FFS | yes | n/a | CBG based transmission for 2 CWs PUSCH is not supported | [Per FSPC] | No | No | No |  | Optional with capability signalling |

40-7-1

- Prerequisite FG: 2-14 (CB based PUSCH).

- Component

  - For component 1, candidate values are {1,2,3,4,5,6,7,8}.

40-7-1a/1b/1c/1d

- Component

  - Based on current CR, ‘[full-coherent codebook]’ can be replaced by ‘Codebook1’, ‘[partial-coherent codebook], Ng=2’ can be replaced by ‘Codebook2’, ‘[partial-coherent codebook], Ng=4’ can be replaced by ‘Codebook3’, ‘[non-coherent codebook]’ can be replaced by ‘Codebook4’.

  - For the FFS for 40-7-1a, we prefer to make (N1,N2) as component, instead of new row. For the candidate values for (N1, N2), (4,1) or (2,2) can be reported by UE. There is no need to support ‘both’ in our views.

40-7-1e

- Prerequisite FG: 40-7-1b, 40-7-1c, or 40-7-1d.

- Consequence if the feature is not supported: UL full power transmission mode 0 is not supported.

40-7-1f

- Prerequisite FG: 40-7-1b, 40-7-1c, or 40-7-1d.

- Component: remove ‘This is a WA’.

- Consequence if the feature is not supported: UL full power transmission mode 1 is not supported.

40-7-1g

- Prerequisite FG: 40-7-1b.

- Component:

  - Remove ‘This is a WA’.

  - Support the 2nd component, and its candidate values include {1,2,4}.

- Consequence if the feature is not supported: UL full power transmission mode 2 is not supported for Codebook2.

40-7-2

- Prerequisite FG: 2-15 (NCB based PUSCH).

- Component

  - For component 1, candidate values are {1,2,3,4,5,6,7,8}.

  - For component 2, candidate values are {1,2,3,4,5,6,7,8}.

  - For component 3, candidate values are {1,2,3,4,5,6,7,8}.

40-7-2a

- Prerequisite FG: 40-7-2, 2-15a.

- Component:

  - OK to keep component 2. The candidate value of component 2 refers to FG 2-15a: Maximum size of the list is 16. The candidate values for the max # of Tx port in one resource is {2, 4, 8, 12, 16, 24, 32}. The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports is: {from 2 to 256}.

40-7-3

- Prerequisite FG: 40-7-1 or 40-7-2.

In addition, for all the FGs above, the granularity could be per FSPC.

We also support to add new FGs for SRS resource for full power mode 2, and full power TPMI group indication for codebook2.

| 40. NR_MIMO_evo_DL_UL | 40-7-1g-1 | UL full power transmission mode of fullpowerMode2 for 8 Tx – SRS resource | 1. The supported SRS configurations with different number of antenna ports per SRS resource for full power Mode 2 for 8 Tx codebook based PUSCH operation | 40-7-1g | yes | n/a |  | [Per FSPC] |  |  |  | Component 1 candidate values: {(1, 8), (2, 8), (4, 8)} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-7-1g-2 | UL full power transmission mode of fullpowerMode2 – full power TPMI groups | - TPMI group(s) which delivers full power | 40-7-1g | yes | n/a | TPMI group(s) for UL full power transmission mode 2 for 8 Tx codebook based PUSCH operation is not supported | [Per FSPC] | n/a | n/a | n/a | Component 1 candidate values: {first antenna group, second antenna group} | Optional with capability signalling |

Summary

N/A

Reference

[1]	R1-2310635, Updated RAN1 UE features list for Rel-18 NR after RAN1#114bis, Moderators (AT&T, NTT DOCOMO, INC.), Oct. 2023