3GPP TSG RAN WG1 #115			R1-2311706

Chicago, USA, November 13th – November 17th, 2023

Agenda item:	8.16.1

Source: 	Apple Inc.

Title: 	Views on UE features for Rel-18 NR MIMO evolution

Document for:	Discussion/Decision

- Views on UE features for Rel-18 NR MIMO evolution

In the latest RAN1#114bis, the UE feature list for Rel-18 NR MIMO evolution was updated [1]. In this contribution, we present our views on UE features for Rel-18 NR MIMO evolution focusing on the additional FGs and components definition. At high level, the FGs are organized in the following way to handle different objectives/agendas in Rel-18 NR MIMO evolution.

- FG40-1x covers the extension of unified TCI framework for multi-TRP.

- FG40-2x covers the two TAs for multi-DCI multi-TRP.

- FG40-3x covers the CSI enhancement.

- FG40-4x covers the increased number of DMRS ports.

- FG40-5x covers the SRS enhancement.

- FG40-6x covers the STxMP (Simultaneous Transmission across Multiple Panels) enhancement.

- FG40-7x covers 8Tx UL.

## Extension of unified TCI framework for multi-TRP

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-1-1 | Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC | - The maximum number of configured joint TCI state per BWP per CC in a band. - Two MAC-CE activated joint TCI-states per CC in a band. - MAC-CE based TCI-state indication for two active TCI-states. - The maximum number of MAC-CE activated joint TCI-states across all CC(s) in a band. | FFS | yes | n/a | Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values {8, 12, 16, 24, 32, 48, 64, 128} Component 4 candidate value {2, 4, 8, 16, 32} Note: FG 16-2b-0 can be used to indicate support of two default beams | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-1a | Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC | - TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) - The maximum number of MAC-CE activated joint TCI states per CC in a band. - The minimum beam application time in Y symbols per SCS | FFS | yes | n/a | Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC is not supported | Per band | n/a | n/a | n/a | Component 2 candidate value {2, 4, 8, 10, 12, 14, 16} Component 3 candidate value {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336} only can be indicated in FR2 Note: FG 16-2b-0 can be used to indicate support of two default beams | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-1b | TRP-specific BFR with unified TCI framework with Unified TCI | Support for TRP-specific BFR with unified TCI framework with Unified TCI | FFS | yes | n/a |  | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-1c | DCI format 1_1 and if supported 1_2 configured with TCI selection field | Support of DCI format 1_1 and if supported 1_2 configured with TCI selection field | At least one of {40-1-1/1a/2/2a} | yes | n/a | DCI format 1_1 and if supported 1_2 configured with TCI selection field are not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-2 | Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC | - The maximum number of configured separate DL TCI state per BWP per CC in a band. - The maximum number of configured separate UL TCI state per BWP per CC in a band. - Two MAC-CE activated DL TCI-states per CC in a band. - Two MAC-CE activated UL TCI-states per CC in a band. - MAC-CE based TCI-state indication. - The maximum number of MAC-CE activated DL TCI states across all CC(s) in a band. - The maximum number of MAC-CE activated UL TCI states across all CC(s) in a band. | FFS | yes | n/a | Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values {8, 12, 16, 24, 32, 48, 64, 128} Component 2 candidate values {8, 12, 16, 24, 32, 48, 64, 128} Component 6 candidate value {2, 4, 8, 16, 32} Component 7 candidate value {2, 4, 8, 16, 32} Note: FG 16-2b-0 can be used to indicate support of two default beams | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-2a | Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC | - TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) - The maximum number of MAC-CE activated DL TCI states per CC in a band. - The maximum number of MAC-CE activated UL TCI states per CC in a band. - The minimum beam application time in Y symbols per SCS | FFS | yes | n/a | Unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with multiple activated TCI codepoints per CC is not supported | Per band | n/a | n/a | n/a | Component 2 candidate value {2, 4, 8, 10, 12, 14, 16} Component 3 candidate value {2, 4, 8, 10, 12, 14, 16} Component 4 candidate value {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336} only can be indicated in FR2. Note: FG 16-2b-0 can be used to indicate support of two default beams | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-3 | Per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP | Support of per aperiodic CSI-RS resource/resource set configuration for TCI selection in S-DCI based MTRP |  | yes | n/a |  | Per band | n/a | n/a | n/a | Component candidate values: {per resource, per resource set, both} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-4 | Two TCI states for CJT Tx scheme for PDSCH | Support of two TCI states for CJT Tx scheme for PDSCH [FFS: for CSI-RS] |  | yes | n/a | Two TCI states for CJT Tx scheme for PDSCH are not supported | Per band | n/a | n/a | n/a | Component candidate values: {CJT Scheme-A, CJT scheme-B, both} CJT Scheme-A: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA CJT Scheme-B: PDSCH DMRS port(s) is QCLed with the DL RSs of both indicated joint/DL TCI states with respect to QCL-TypeA except for QCL parameters {Doppler shift, Doppler spread} of the second indicated joint/DL TCI state | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-7 | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC | 1. Support of mTRP operation for M-DCI 2. Support of a single default beam for PDSCH with scheduling offset less than a threshold unless UE indicates the support of two default beams 3. Maximum number of configured joint TCI states per BWP per CC 4. Maximum number of activated joint TCI states across all CCs per ‘coresetPoolIndex’ value 5. One MAC-CE activates one joint TCI-states per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value | [16-2a, 23-4, 40-6-1, 40-6-2, 40-6-3a, 40-6-3b, 40-6-4] | yes | n/a | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported | Per ban] | n/a | n/a | n/a | Component 1 candidate values {Intra-cell, intra-cell and inter-cell} Component 3 candidate values: {8, 12, 16, 24, 32, 48, 64, 128} Component 4 candidate values: {2, 4, 8, 16} Note: activated joint TCI state(s) include all PDCCH/PDSCH receptions and PUSCH/PUCCH transmissions | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-7a | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC | 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of monitored DCI formats 1_1 and if supported 1_2 without DL assignment) 2. Maximum number of MAC-CE activated joint TCI states per BWP per CC per coresetpoolindex 3. The minimum beam application time in symbols | [FFS] | yes | n/a | Unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported | Per band | n/a | n/a | n/a | Component 2 candidate values: {2,3,4,5,6,7,8) Component 3 candidate values: {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336 } only can be applicable in FR2 | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-9 | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC | 1. Maximum number of configured DL TCI states per BWP per CC 2. Maximum number of configured UL TCI states per BWP per CC 3. Maximum number of activated DL TCI states across all CC 4. Maximum number of activated UL TCI states across all CC 5. One MAC-CE activated DL TCI-state per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value. 6. One MAC-CE activated UL TCI-state per CC in a band for a TRP associated with a ‘coresetPoolIndex’ value. 7. Support unified TCI with separate DL/UL TCI update for multi-DCI intra-cell and inter-cell multi-TRP operation | [23-10-1] | yes | n/a | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC is not supported | Per band | n/a | n/a | n/a | Component 1 candidate value {8, 12, 16, 24, 32, 48, 64, 128} Component 2 candidate value {8, 12, 16, 24, 32, 48, 64} Component 3 candidate values: {1, 2, 4, 8, 16} Component 4 candidate values: {1, 2, 4, 8, 16} Component 7 candidate values {Intra-cell, intra-cell and inter-cell} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-9a | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC | 1. TCI state indication for update and activation a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 with DL assignment) b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1 and if supported 1_2 without DL assignment) 2. maximum number of activated DL TCI states per CORESETPoolIndex per BWP per CC 3. maximum number of activated UL TCI states per CORESETPoolIndex per BWP per CC 4. The minimum beam application time in Y symbols per SCS | [23-10-1b, 40-1-9] | yes | n/a | Unified TCI with separate DL/UL TCI update for multi-DCI based multi-TRP with multiple activated TCI codepoints per CORESETPoolIndex per CC is not supported | Per band | n/a | n/a | n/a | Component 2 candidate values: {1, 2, 3, 4, 5, 6, 7, 8} Component 3 candidate values: {1, 2, 3, 4, 5, 6, 7, 8,} Component 4 candidate value {1, 2, 4, 7, 14, 28, 42, 56, 70, 84, 98, 112, 224, 336}, where {84, 98, 112, 224, 336} only can be indicated in FR2. | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-1-12 | Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP | 1. Support of common multi-CC TCI state ID update and activation for single-DCI based multi-TRP 2. Maximum number of CC list(s) | [40-1-1, 40-1-1, 40-1-2] | yes | n/a | Common multi-CC TCI state ID update and activation for single-DCI based multi-TRP is not supported | Per band | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-13 | Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP | 1. Support of common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP 2. Maximum number of CC list(s) | [40-1-7] | yes | n/a | Common multi-CC TCI state ID update and activation for multi-DCI based multi-TRP is not supported | Per band | n/a | n/a | n/a | Component 2 candidate values: {1,2,3,4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-1-5 | For unified TCI framework for single-DCI multi-TRP, spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission | - For unified TCI framework for single-DCI multi-TRP, support of spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission that is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission. Note: This is still an FFS since RAN1#112. Depends on whether this is supported by Rel-18, needs further discussion |  |  |  |  | Per band |  |  |  |  | Optional with capability signalling |

## Two TAs for multi-DCI multi-TRP

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-2-1 | Basic feature for multi-DCI based intra-cell Multi-TRP operation with two TA enhancement | - Support of 2 TA/TAG within a serving cell for intra-cell multi-DCI multi-TRP - Rx timing difference between different TRP is no larger than CP length. - The UL/joint TCI states of UL signals/channels associated to one coresetPoolIndex correspond to one TAG | 16-2a | yes | n/a |  | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-2-2 | Basic feature for multi-DCI based inter-cell Multi-TRP operation with two TA enhancement | - serving cell for inter-cell multi-DCI multi-TRP - Rx timing difference between different TRP is no larger than CP length. - The UL/joint TCI states of UL signals/channels associated to one coresetPoolIndex correspond to one TAG. | 16-2a | yes | n/a |  | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-2-3 | TAG ID indication via absolute TA command MAC CE | Support indicating one of two TAG IDs configured in the SpCell via absolute TA command MAC CE. |  | yes | n/a |  | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-2-4 | Support of a PDCCH order sent by one TRP triggers RACH procedure (specifically PRACH) towards a different TRP based on CFRA | Support of cross-TRP PDCCH order based on CFRA for inter-cell [WA: and intra-cell] multi-DCI based mTRP |  | yes | N/A |  | Per band | No | No | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-2-5 | TCI states of one coresetPoolIndex associated to both TAGs | Support of UL/joint TCI states of UL signals/channels associated to one coresetPoolIndex correspond to both TAGs This is a WA | FG 40-2-1 or 40-2-2 | yes | n/a |  | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-2-6 | Rx timing difference larger than CP length | - Support of maximum Rx timing difference between the two DL reference timings larger than CP |  |  |  |  | Per FS |  |  |  |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-2-7 | Two TAs for multi-DCI STxMP PUSCH+PUSCH | Support of two TAs for multi-DCI STxMP PUSCH+PUSCH | [40-2-1 or 40-2-2, FFS more] | yes | n/a |  | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-2-8 | The maximum number of TAGs all CCs | - Maximum number of TAGs that can be supported across all intra-frequency and inter-frequency CCs. |  |  |  |  | Per BC |  |  |  | Component 1 candidate value: {2, 3, 4[, 5, 6, 7, 8]} Note: the UE only supports the configuration where all UL CCs of the same frequency band are configured with up to 2 Timing Advance Group ID. | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-2-9 | Overlapping UL transmission reduction | - Support of reducing the overlapping duration of the later of the two time-domain overlapping UL transmissions, when the UE does not support UL STx2P (Simultaneous Transmission cross 2 Panels), for multi-DCI based multi-TRP operation with two TA enhancement |  |  |  |  | Per band |  |  |  | Note: If UE does not support this feature, UE does not expect the two UL transmissions to overlap (i.e., scheduling restriction is applied to avoid overlap between the two UL transmissions) | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-2-10 | Maximum number of n-TimingAdvanceOffset value per serving cell | - Maximum number of n-TimingAdvanceOffset value supported per serving cell |  |  |  |  | Per band |  |  |  | Component 1 candidate value: {1, 2} | Optional with capability signalling |

## CSI enhancement

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-1 | Support of mode 2 for Rel-16-based CJT type-II codebook | Support of N=N_TRP only Support of N_L=1 only 1. Support of mode 2 for Rel-16 eType-II codebook refinement for multi-TRP CJT 2. Support for PMI subband R=1. 3. Support of parameter combinations with L=2,4 4. Support of rank 1,2 5. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT] 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs | [2-35] | Yes | N/A |  | Per band and Per BC | No | N/A | N/A | FFS: CPU occupation Component 5 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel. 16 based type-II CJT must support this FG | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-1a | Support of mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset | 1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with PMI subband R=1. 2. Support of parameter combinations with L=2,4 3. Support of rank 1,2 4. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT] 5. Supported frequency basis selection mode 1, i.e., common frequency basis selection among different TRPs with FD basis selection integer frequency offset | 40-3-1-1 | Yes | N/A | Mode 1 for Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset is not supported | Per band and Per BC | N/A | N/A | N/A | Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-1b | Maximum number of TRPs supported for Rel-16-based CJT Type-II codebook | 1, Maximum number of TRPs supported for Rel-16-based CJT Type-II codebook | 40-3-1-1 | Yes | N/A | Rel-16-based CJT type-II codebook with FD basis selection integer frequency offset is not supported | Per band and Per BC | N/A | N/A | N/A | Component 1 candidate values: {2,3,4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-2 | Support for FD basis selection fractional offset mode for Rel-16-based CJT codebook with mode1 | Supported frequency basis selection mode 1 with FD basis selection fractional frequency offset for Rel-16 eType-II based CJT codebook | 40-3-1-1a | Yes | N/A | FD basis selection fractional offset mode for Rel-16-based CJT codebook with mode 1 is not supported | Per band and Per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-3 | Support R=2 for Rel-16-based CJT codebook | 1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with PMI subbands R=2 2. {Max # of Tx ports in one resource set, Max # of resource sets, total # of Tx ports}, across all CCs simultaneously, with R=2 | 40-3-1-1 | Yes | N/A | R=2 for Rel-16-based CJT codebook is not supported | Per-band Per-BC | N/A | N/A | N/A | Component 2 candidate values: a) {4,8,12,16,24,32} b) {2 to 64} c) {4 to 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-4 | Support pv={1/2,1/2,1/2,1/2} and beta=1/2 for Rel-16-based CJT codebook | 1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with parameter combination pv={1/2,1/2,1/2,1/2} and beta=1/2 | 40-3-1-1 | Yes | N/A | pv={1/2,1/2,1/2,1/2} and beta=1/2 for Rel-16-based CJT codebook are not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-5 | Support of mode 2 for Rel-17-based CJT type-II codebook | Support of N=N_TRP only Support of N_L=1 only 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT 2. Support of PMI subband R=1. 3. Support of parameter combinations with M=1 4. Support of rank 1,2 5. A list of supported combinations, up to 16, [across all CCs] simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT 6. Supported frequency basis selection mode 2, i.e., common frequency basis selection among different TRPs | [2-35] | Yes | N/A |  | Per-band and Per-BC | No | N/A | N/A | FFS: CPU occupation Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Note: A-CSI is supported, and whether UE supports SP-CSI on PUSCH is dependent on FG2-32b Note: A UE that supports CSI enhancement for Rel 17 based type-II CJT must support this FG | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-5a | Support of mode 1 for Rel-17-based CJT type-II codebook with FD basis selection integer frequency offset | 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with PMI subband R=1. 2. Support of parameter combinations with M=1 3. Support of rank 1,2 4. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource associated with multi-TRP CJT b) Maximum total number of NZP CSI-RS resource associated with multi-TRP CJT c) Maximum total number of Tx ports of NZP CSI-RS resources associated with multi-TRP CJT 5. Supported frequency basis selection mode 1, i.e., common frequency basis selection among different TRPs with FD basis selection integer frequency offset | 40-3-1-5 | Yes | N/A | Mode 1 for Rel-17-based CJT type-II codebook with FD basis selection integer frequency offset is not supported | Per band and per BC | N/A | N/A | N/A | Component 4 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-5b | Maximum number of TRPs supported for Rel-17-based CJT Type-II codebook | 1, Maximum number of TRPs supported for Rel-17-based CJT Type-II codebook | 40-3-1-1 | Yes | N/A | Rel-17-based CJT type-II codebook with FD basis selection integer frequency offset is not supported | Per band and Per BC | N/A | N/A | N/A | Component 1 candidate values: {2,3,4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-6 | Support for FD basis selection fractional offset mode for Rel-17-based CJT codebook with mode1 | 1. Supported frequency basis selection mode 1 with FD basis selection fractional frequency offset for Rel-17 FeType-II port selection based CJT codebook | 40-3-1-5a | Yes | N/A | FD basis selection fractional offset mode for Rel-17-based CJT codebook with mode1 is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-7 | Support of M=2 and R=1 for Rel-17-based CJT codebook | 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with M=2 and PMI subband R=1 2. {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, across all CCs simultaneously, with M=2 and R=1 | 40-3-1-5 or 40-3-1-5a | Yes | N/A | M=2 and R=1 for Rel-17-based CJT codebook are not supported | Per band and per BC | N/A | N/A | N/A | Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-8 | Support R=2 for Rel-17-based CJT codebook | 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with PMI subband R=2. 2. {Max # of Tx ports in one resource set, Max # of resources and total # of Tx ports}, [across all CCs simultaneously, with R=2 | 40-3-1-5 or 40-3-1-5a | Yes | N/A | R=2 for Rel-17-based CJT codebook is not supported | Per band and Per BC | N/A | N/A | N/A | Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-9 | Support for 2NN1N2 >32 for Rel-16 based CJT codebook | Maximum number of ports across all TRPs for one CJT CSI measurement | 40-3-1-1 | Yes | N/A | 2NN1N2 >32 is not supported for Rel-16 based CJT codebook | Per band and Per BC | N/A | N/A | N/A | Component 1 candidate values: {64, 96, 128} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-9a | Support for 2NN1N2 >32 for Rel-17 based CJT codebook | Maximum number of ports across all TRPs for one CJT CSI measurement | 40-3-1-5 | Yes | N/A | 2NN1N2 >32 is not supported for Rel-17 based CJT codebook | Per band and Per BC | N/A | N/A | N/A | Component 1 candidate values: {64, 96, 128} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-11 | Active CSI-RS resources and ports for mixed codebook types including Type-II-CJT in any slot | 1. List of codebook combinations 2. List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | 40-3-1-1,40-3-1-5,2-36 | Yes | N/A | Active CSI-RS resources and ports for mixed Type-II-CJT codebook types in any slot is not supported | Per-band and Per-BC | N/A | N/A | N/A | [Codebook 1 = {Type I SP, Type I MP} (Codebook 2, Codebook 3) = {(eType-II-CJT R=1, NULL), (eType-II-CJT R=2, NULL), (FeType-II-CJT PS R=1 M=1, NULL), (FeType-II-CJT PS R=1 M=2, NULL), (FeType-II-CJT PS R=2 M=2, NULL), (FFS CB combos…)}] | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-12 | Support of Rank 3 and 4 for Rel-16-based CJT type-II codebook | 1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with rank 3,4 | 40-3-1-1 | Yes | N/A | Rank 3 and 4 for Rel-16-based CJT type-II codebook is not supported | Per-band and Per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-13 | Support of Rank 3 and 4 for Rel-17-based CJT type-II codebook | 1. Support of Rel-17 FeType-II port selection codebook refinement for multi-TRP CJT with rank 3,4 | 40-3-1-5 | Yes | N/A | Rank 3 and 4 for Rel-17-based CJT type-II codebook is not supported | Per-band and Per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-14 | Support of Support of L=6 for Rel-16-based CJT type-II codebook | 1. Support of Rel-16 eType-II codebook refinement for multi-TRP CJT with parameter combination with L=6 | 40-3-1-1 | Yes | N/A | Support of L=6 for Rel-16-based CJT type-II codebook is not supported | Per-band and Per BC | N/A | N/A | N/A | Note: Only for N_TRP=1 | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-15 | dynamic selection of N<=N_TRP for Rel-16-based CJT type-II codebook | 1. Support of selection of N <= N_TRP CSI-RS resource by UE for multi-TRP CJT based on Rel-16 eType-II codebook | 40-3-1-1 | Yes | N/A | Dynamic selection of N<N_TRP for Rel-16-based CJT type-II codebook is not supported | Per-band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-16 | dynamic selection of N<=N_TRP for Rel-17-based CJT type-II codebook | 1. Support of selection of N <= N_TRP CSI-RS resource by UE for multi-TRP CJT based on Rel-17 FeType-II port selection codebook | 40-3-1-5 | Yes | N/A | Dynamic selection of N<N_TRP for Rel-17-based CJT type-II codebook is not supported | Per-band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-17 | Support for N_L>1 combinations of number of SD basis across CSI-RS resources for Rel-16-based CJT type-II codebook | 1. Maximum number of lists for spatial basis selection, i.e., N_L, for multi-TRP CJT based on Rel-16 eType-II codebook | 40-3-1-1 | Yes | N/A | N_L>1 combinations across for Rel-16-based CJT type-II codebook is not supported | Per band and Per BC | N/A | N/A | N/A | Candidate values for component 1: N_L = {2,4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-18 | Support for N_L>1 combinations of number of ports across CSI-RS resources for Rel-17-based CJT type-II codebook | 1. Maximum number of lists for ports selection, i.e., NL, for multi-TRP CJT based on Rel-17 FeType-II port selection codebook | 40-3-1-5 | Yes | N/A | N_L>1 combinations across for Rel-17-based CJT type-II codebook is not supported | Per band and Per BC | N/A | N/A | N/A | Candidate values for component 1: N_L = {2,4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-23 | Unequal number of spatial basis selection configuration for multi-TRP CJT | 1. Support of unequal number of spatial basis selection configuration across CSI-RS resources for multi-TRP CJT including Rel-16 eType-II codebook refinement | 40-3-1-1 | Yes | N/A | Unequal number of spatial basis selection configuration for multi-TRP CJT is not supported | Per-band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-23a | Unequal number of port selection configuration for multi-TRP CJT | 1. Support of unequal number of port selection configuration across CSI-RS resources for multi-TRP CJT including Rel-17 FeType-II port selection codebook refinement | 40-3-1-5 | Yes | N/A | Unequal number of port selection configuration for multi-TRP CJT is not supported | Per-band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-1-24 | CPU counting for multi-TRP CJT | Scaling factor X for CPU occupation of multi-TRP CJT CSI based on Type-II codebook |  |  |  |  | Per FS |  |  |  | Component 1 candidate values: {1, 1.5, 2} Note: When NTRP=1 TRP is configured, OCPU =1. When NTRP>1 TRPS are configured, OCPU =X * NTRP | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-1 | Support of Rel-16-based doppler CSI | 1. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 2. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 3. Support parameter combinations with L=2,4 4. Support for rank = 1,2 5. A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously [6. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting] 7. Value of Y for CPU occupation (OCPU = Y.N4), when P/SP-CSI-RS is configured for CMR 8. Value of Y for CPU occupation (OCPU = Y.K), when A-CSI-RS is configured for CMR 9. Support for the size of DD-basis, N4=1 10. Scaling factor for active resource counting Kp | 2-35 | Yes | N/A | Rel-16 based Type II doppler codebook is not supported | Per-band and Per-BC | N/A | N/A | N/A | Component 5 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} Component 7 candidate values: {1, 2, 3} Component 8 candidate values: {1, 2, 3} Component 10 candidate values: {1, 2, 4} Note: When N4=1, OCPU =4 Note: OCPU ≥ 4 when P/SP-CSI-RS is configured for CMR Note: when K=12, OCPU =8 Note: A UE that supports CSI enhancement for Rel. 16 based type-II doppler must support this FG | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-1a | Support of Rel-16-based doppler measurement with N4>1 | 1. Support for the size of DD-basis, N4>1 2. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource - Maximum total number of NZP CSI-RS resource - Maximum total number of Tx ports of NZP CSI-RS resources - Number of predicted CSI (N4) | 40-3-2-1 | Yes | N/A | Rel-16 based Type II doppler codebook with N4>1 is not supported | Per-band and Per-BC | N/A | N/A | N/A | Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} d) {1, 2, 4, 8} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-1b | Maximum number of aperiodic CSI-RS resources that can be configured in the same CSI report setting for Rel-16-based doppler measurement | 1. Maximum number of aperiodic CSI-RS resources that can be configured in the same CSI report setting for Rel-16-based doppler measurement | 40-3-2-1 | Yes | N/A |  | Per-band and Per-BC | N/A | N/A | N/A | Component 1 candidate values: {4, 8, 12} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-2 | Support R=2 for Rel-16-based doppler codebook | A list of supported combinations {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}, across all CCs simultaneously, with R=2 | 40-3-2-1 | Yes | N/A | R=2 for Rel-16-based doppler codebook is not supported | Per band and per BC | N/A | N/A | N/A | Candidate values for component 1: - Maximum 16 triplets - Max N4 {1,2,4,8} - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-2a | Support Rel-16-based predicted PMI codebook with parameter combination with L=6 | Support of Rel-16 eType-II regular codebook refinement for predicted PMI with parameter combination with L=6 |  |  |  |  | Per band and per BC |  |  |  |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-2b | Support Rel-16-based predicted PMI codebook with rank 3,4 | Support of Rel-16 eType-II regular codebook refinement for predicted PMI with rank 3,4 |  |  |  |  | Per band and per BC |  |  |  |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-3 | Support X=1 based on first and last slot of WCSI, for Rel-16-based doppler codebook | Support of TDCQI = ‘1-2’ | 40-3-2-1 | Yes | N/A | X=1 based on first and last slot of WCSI, for Rel-16-based doppler codebook is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-3a | Support X=2 CQI based on 2 slots for Rel-16-based doppler codebook | Support of TDCQI = ‘2’ | 40-3-2-1 | Yes | N/A | X=2 CQI based on 2 slots for Rel-16-based doppler codebook is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-4 | Support of Rel-17-based doppler measurement | 1. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI 2. Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with PMI subband R=1 3. Support of parameter combinations with M = 1 4. Support for rank = 1,2 5: A list of supported combinations, each combination is {Max # of time unit, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one doppler CSI measurement [6. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting] 7. Value of Y for CPU occupation (OCPU = Y.K), when A-CSI-RS is configured for CMR 8. Support for N4=1 9. Scaling factor for active resource counting Kp | 40-3-2-1, 2-35 | Yes | N/A | Rel-17 based Type II doppler codebook is not supported | Per-band and Per-BC | N/A | N/A | N/A | Component 5 candidate values: a) {4, 8, 12, 16, 24, 32} b) {1, 2,3,4 … 64} c) {4, …, 256} Component 7 candidate values: {1, 2, 3} Component 9 candidate values: {1, 2, 4} Note: OCPU = 4 when P/SP-CSI-RS is configured for CMR Note: when K=12, OCPU =8 Note: A UE that supports CSI enhancement for Rel. 17-based type-2 doppler must support this FG | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-4b | Maximum number of aperiodic CSI-RS resources that can be configured in the same CSI report setting for Rel-17-based doppler measurement | 1. Maximum number of aperiodic CSI-RS resources that can be configured in the same CSI report setting for Rel-17-based doppler measurement | 40-3-2-4 | Yes | N/A |  | Per-band and Per-BC | N/A | N/A | N/A | Component 1 candidate values: {4, 8, 12} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-5 | Support of M=2 and R=1 for Rel-17-based doppler codebook | 1. Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with M=2 and PMI subband R=1 2. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource b) Maximum total number of NZP CSI-RS resource c) Maximum total number of Tx ports of NZP CSI-RS resources | 40-3-2-4 | Yes | N/A | M=2 and R=1 for Rel-17-based doppler codebook is not supported | Per-band and Per-BC | N/A | N/A | N/A | Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-6 | Support R=2 for Rel-17-based doppler codebook | 1. Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with PMI subbands R=2 2. A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is a) Maximum number of Tx ports in one NZP CSI-RS resource b) Maximum total number of NZP CSI-RS resource c) Maximum total number of Tx ports of NZP CSI-RS resources | 40-3-2-4 | Yes | N/A | R=2 for Rel-17-based doppler codebook is not supported | Per band and per BC | N/A | N/A | N/A | Component 2 candidate values: a) {4, 8, 12, 16, 24, 32} b) {2,3,4 … 64} c) {4, …, 256} d) {2,3,4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-7 | support of l = (n – nCSI,ref ) for CSI reference slot for Rel-16 based doppler codebook | 1. Support of l = (n – nCSI,ref ) for CSI reference slot when N4=1 and d>1 2. Support of l = (n – nCSI,ref ) for CSI reference slot when N4>1 | 40-3-2-1 | Yes | N/A | l = (n – nCSI,ref ) for CSI reference slot for Rel-16 based doppler codebook is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-7a | Support of l = (n – nCSI,ref ) for CSI reference slot for Rel-17 based doppler codebook | 1. Support of l = (n – nCSI,ref ) for CSI reference slot when N4=1 and d>1 | 40-3-2-4 | Yes | N/A | l = (n – nCSI,ref ) for CSI reference slot for Rel-17 based doppler codebook is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-8 | Support Rel-17-based predicted PMI codebook with rank 3,4 | Support of Rel-17 FeType-II port selection codebook refinement for predicted PMI with rank 3,4 |  |  |  |  | Per band and per BC |  |  |  |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-2-9 | Aperiodic CSI report timing relaxation for predicted PMI codebook based on Type-II codebook | - Aperiodic CSI report timing relaxation, w, for predicted PMI codebook based on Type-II codebook. - Aperiodic CSI report timing relaxation, Z’, for predicted PMI codebook based on Type-II codebook |  |  |  |  | Per band and per BC |  |  |  | Component 1 candidate values: UE reports candidate value, w, independently for each SCS in unit of symbols For 15kHz SCS: {0, 4, 8, 16, 32} For 30kHz SCS: {0, 8, 16, 32, 64} For 60kHz SCS: {0, 16, 32, 64, 128} For 120kHz SCS: {0, 16, 32, 64, 128} For 480kHz SCS: {0, 64, 128, 256, 512} For 960kHz SCS: {0, 128, 256, 512, 1024} Component 2 candidate values: {CAP1, CAP2} For CAP1 in component 2 1) For AP CSI-RS: Z=legacy Z+14*(K–1)*m 2) For P/SP CSI-RS: Z= legacy Z+w For CAP2 in component 3 1) For AP CSI-RS: Z= legacy Z+14*(K–1)*m + r 2) For P/SP CSI-RS: Z= legacy Z+w+r where Legacy Z/Z’ values are defined as Z2/Z'2 in Table 5.4-2 in TS38.214 K = {4,8,12}, is the number of AP CSI-RS resources for the CMR in a CSI report setting M = {1,2}, is the offset between two adjacent AP CSI-RS resources for the CMR in slots | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-1 | TDCP (Time Domain Channel Properties) report | 1. Support of Y=1 delay value for TDCP report 2. Basic delay value, component candidate value <= D_basic = 1 slot 3. Support of amplitude report 4. Value of X for CPU occupation (OCPU=(Y+1).X) 5. Support to configure KTRS = 1 TRS resource set 6. Maximum number of simultaneously active CSI-RS resources for TDCP across all CCs Note: counting of simultaneously active CSI-RS resources follows existing specification TS38.214 | [2-35] | Yes | N/A | Time Domain Channel Properties report is not supported | per band and per BC | N/A | N/A | N/A | Component 4 candidate values: {1,2} Component 6 candidate values: {2,4,6,8,12,16,20, 24,28,32,40,48,56,64} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-2 | Number of delay values | Number Y>1 of delay values for which TDCP is reported | 40-3-3-1 | Yes | N/A | TDCP is not reported for more than 1 delay value | per band and per BC | N/A | N/A | N/A | Candidate values: {2,3,4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-1a | Supported maximum delay value larger than D_basic | Support of maximum delay value larger than D_basic =1 slot | 40-3-3-1 | Yes | N/A | delay value(s) larger than D_basic are not supported | per band and per BC | N/A | N/A | N/A | Candidate values: {2 slots, 3 slots, 4 slots, 5 slots, 6 slots, 10 slots} Note: 10 slots is only applicable for SCS >= 30 kHz, and 6 slots is maximum for SCS = 15 kHz | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-4 | Phase report | Support of phase report for Y>=1 | 40-3-3-1 | Yes | N/A | Phase report for Y>=1 is not supported | per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-5 | Number of CSI-RS resources for TDCP | 1. Maximum number of configured CSI-RS resources for TDCP per CC 2. Maximum number of configured CSI-RS resources for TDCP across all CCs 3. Maximum number of simultaneously active CSI-RS resources for TDCP per CC Note: counting of simultaneously active CSI-RS resources follows existing specification TS38.214 | 40-3-3-1 | Yes | N/A | Number of CSI-RS resources for TDCP is not reported | per band and per BC | N/A | N/A | N/A | Component 1 candidate values: {2,4,6,8} Component 2 candidate values: {2,4,6,8,12,16} Component 3 candidate values: {2,4,6,8,12,16,20,24,28,32} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-3-3-6 | Maximum number of TRS resource sets in a report configuration | Max number of TRS resource sets in a single CSI-RS resource setting | 40-3-3-1 | Yes | N/A | More than 1 TRS resource set in a report configuration is not supported | per band and per BC | N/A | N/A | N/A | Candidate values: {2,3} | Optional with capability signaling |

## Increased number of orthogonal DMRS ports

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-4-1 | Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type A | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol | [2-5] | Yes | n/a |  | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-1a | Basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type B | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol | [2-5] | Yes | n/a | UE does not support basic feature of Rel.18 enhanced DMRS ports for PDSCH for mapping type B | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-1b | 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH | Support of 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH | 40-4-1 | Yes | n/a | UE does not support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PDSCH | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-1c | Alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH | Support of alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH | 40-4-1, 5-28 | Yes | n/a | UE does not support alternative additional DMRS position for co-existence with LTE CRS for Rel.18 enhanced DMRS ports for PDSCH | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-1d | 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH | Support of 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH | 40-4-1 | Yes | n/a | UE does not support 2 symbols FL-DMRS for Rel.18 enhanced DMRS ports for PDSCH | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-1e | 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH | Support of 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH | 40-4-1 | Yes | n/a | UE does not support 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PDSCH | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-1f | 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH | Support of 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH | 40-4-1 | Yes | n/a | UE does not support 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PDSCH | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-1g | DMRS type for Rel.18 enhanced DMRS ports for PUSCH | Support of DMRS type for Rel.18 enhanced DMRS ports for PUSCH | 40-4-1 | Yes | n/a | UE does not support DMRS type for Rel.18 enhanced DMRS ports for PUSCH | Per FS | No | No | n/a | Component 1 candidate values: {etype 1, both etype 1 and etype 2} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-1h | 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 | Support of 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 | 40-4-1 | Yes | n/a | 1 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 is not supported | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-1i | 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 | Support of 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 | 40-4-1, FFS more | Yes | n/a | 2 port DL PTRS for Rel.18 enhanced DMRS ports for PDSCH with rank 1-8 is not supported | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-1j | Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port for mapping type A | Support of Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port for mapping type A | 40-4-1 | Yes | n/a | Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port is not supported for mapping type A | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-4 | Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports | Support reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports | 40-4-1 | Yes | n/a | Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports is not supported | Per FS | No | No | n/a | Note: If this feature is not supported, UE expects that gNB shall apply at least the following scheduling restriction for PDSCH for FD-OCC 4 in Rel.18 eType 1 DMRS 1) The number of consecutively scheduled PRBs for PDSCH is even 2) The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-4a | Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports for PDSCH with fdmSchemeA | Support reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports for PDSCH with fdmSchemeA | 40-4-1, 23-6-1/1b | Yes | n/a | Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports for PDSCH with fdmSchemeA is not supported | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-4b | Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports for PDSCH with fdmSchemeB | Support reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports for PDSCH with fdmSchemeB | 40-4-1, 23-6-2/2b | Yes | n/a | Reception of PDSCH without the scheduling restriction for Rel.18 eType1 DMRS ports for PDSCH with fdmSchemeB is not supported | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-5 | Rel-18 DL DMRS with single DCI based M-TRP | Support of Rel-18 DL DMRS with single DCI based M-TRP | 40-4-1, FFS more | Yes | n/a | Rel-18 DL DMRS with single DCI based M-TRP is not supported | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-5a | Additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP | Support of additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP | 40-4-5 | Yes | n/a | Additional row(s) for antenna ports (0,2,3) for Rel.18 DMRS ports for single-DCI based M-TRP are not supported | [Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-7 | Rel-18 DL DMRS with M-DCI based M-TRP | Support of Rel-18 DL DMRS with multi- DCI based M-TRP PDSCH operation | FFS | Yes | n/a | Rel-18 DL DMRS with M-DCI based M-TRP is not supported | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-8 | Support of Rel-18 DMRS and PDSCH processing capability 2 simultaneously | Support Rel-18 DMRS and PDSCH processing capability 2 simultaneously | 40-4-1/1a, 5-5a/5b | Yes | n/a | Rel-18 DMRS and PDSCH processing capability 2 are not support simultaneously | Per FS | No | No | n/a | Note: PDSCH processing capability #2 related UE capability follows legacy FG5-5a, FG5-5b, FG5-13, FG13-a, FG5-13c | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-6 | Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbols 3) Support 1 symbol FL DMRS and 2 additional DMRS symbols | [2-16] | Yes | n/a | Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type A for Rel.18 enhanced DMRS ports is not supported | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-6a | Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol | [2-16a] | Yes | n/a | Basic feature of Rel.18 enhanced DMRS ports for PUSCH for scheduling type B for Rel.18 enhanced DMRS ports is not supported | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-6b | Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PUSCH | 1. Support of 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port for Rel.18 enhanced DMRS ports for PUSCH |  |  |  |  | Per FS |  |  |  |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-4-6c | DMRS type for Rel.18 enhanced DMRS ports for PUSCH | Support of DMRS type for Rel.18 enhanced DMRS ports for PUSCH | 40-4-6 | Yes | n/a | DMRS type for Rel.18 enhanced DMRS ports for PUSCH is not supported | Per FS | No | No | n/a | Component candidate values: {eType 1, both eType 1 and eType 2} [Note: A UE supporting one of FG 40-4-6 or FG 40-4-6a must support this FG] | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-6d | 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH | Support of 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH | 40-4-6 | Yes | n/a | 2 symbols front-loaded DMRS (uplink) for Rel.18 enhanced DMRS ports for PUSCH is not supported | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-6e | 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH | Support of 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH | 40-4-6 | Yes | n/a | 2-symbol FL DMRS + one additional 2-symbols DMRS for Rel.18 enhanced DMRS ports for PUSCH is not supported | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-6f | 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH | Support of 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH | 40-4-6 | Yes | n/a | 1 symbol FL DMRS and 3 additional DMRS symbols for Rel.18 enhanced DMRS ports for PUSCH is not supported | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-6g | 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 | Support of 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 | 40-4-6, FFS more | Yes | n/a | 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 is not supported | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-6h | 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 | Support of 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 | 40-4-6, FFS more | Yes | n/a | 1 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 is not supported | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-6i | 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 | Support of 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 | 40-4-6, FFS more | Yes | n/a | 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 1-4 is not supported | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-6j | 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 | Support of 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 | 40-4-6, FFS more | Yes | n/a | 2 port UL PTRS for Rel.18 enhanced DMRS ports for PUSCH with rank 5-8 is not supported | Per FS | No | No | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-4-10 | DMRS port configuration for PUSCH [with rank 5-8/ with 8Tx] | DMRS port configuration for PUSCH [with rank 5-8 / with 8Tx] for Rel 15 and Rel. 18 | [40-4-6] | Yes | n/a | Port configuration for PUSCH [with rank 5-8 / with 8Tx] is not supported | Per FS | No | No | n/a | Candidate values: {Rel. 15 DMRS, Rel. 15 DMRS and Rel. 18 DMRS} [Note: A UE supporting 8 Tx must support this FG] | Optional with capability signaling |

## SRS enhancement

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-5-1 | SRS comb offset hopping | Support of SRS comb offset hopping | FFS | Yes | n/a | SRS comb offset hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-2 | SRS cyclic shift hopping | Support of SRS cyclic shift hopping | [2-53] | Yes | n/a | SRS cyclic shift hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-1b | SRS comb offset hopping combined with legacy group/sequence hopping | Support of SRS comb offset hopping combined with legacy group/sequence hopping | 40-5-1 | Yes | n/a | SRS comb offset hopping combined with legacy group/sequence hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-2b | SRS cyclic shift hopping combined with legacy group/sequence hopping | Support of SRS cyclic shift hopping combined with legacy group/sequence hopping | 40-5-2 | Yes | n/a | SRS cyclic shift hopping combined with legacy group/sequence hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-3 | SRS cyclic shift hopping combined with SRS comb offset hopping | Support of SRS cyclic shift hopping combined SRS comb offset hopping | 40-5-1, 40-5-2 | Yes | n/a | SRS cyclic shift hopping combined with SRS comb offset hopping is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-1a | Comb offset hopping time-domain behavior when repetition factor R>1 | Supported comb offset hopping granularity in time when repetition factor R>1 is configured | 40-5-1 | Yes | n/a |  | Per band | n/a | n/a | n/a | Component candidate values: {‘per SRS symbol’,’per R SRS symbols’, ‘both’} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-1c | Configuration of subset of comb offsets for comb offset hopping | Support of configuration of a subset of comb offsets when comb offset hopping is configured |  |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-5-2c | Configuration of subset of cyclic shifts for cyclic shift hopping | Support of configuration of a subset of cyclic shifts when cyclic shift hopping is configured |  |  |  |  | Per band |  |  |  |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-5-2d | Finer time-delay-domain granularity for SRS cyclic shift hopping | Support of finer time-delay-domain granularity for SRS cyclic shift hopping |  |  |  |  | Per band |  |  |  | Note: The finer granularity above only applies to the cyclic shift offsets when cyclic shift hopping is enabled. If a subset for cyclic shifts is configured, this feature cannot be configured | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-5-4 | SRS without TDMed 8 Tx ports—antenna switching | - Support of 8T8R SRS antenna switching. - Report the entry number of the first-listed band with UL in the band combination that affects this DL. - Report the entry number of the first-listed band with UL in the band combination that switches together with this UL. FFS: detailed components FFS: separate row for comb8 |  | Yes | n/a | SRS with 8 Tx ports—antenna switching is not supported | Per FS | n/a | n/a | n/a | Component 1 candidate values: a combination from the set {t1r1, t2r2, t1r2, t4r4, t2r4, t1r4, t2r6, t1r6, t4r8, t2r8, t1r8} Note: Component 1 indicates the supported xTyR in addition to 8T8R. Component 2 candidate values: {1 to 32} Component 3 candidate values: {1 to 32} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-4a | SRS without TDMed 8 Tx ports—codebook | Support of SRS with 8 Tx ports—codebook FFS: detailed components FFS: separate row for comb8 |  | Yes | n/a | SRS with 8 Tx ports—codebook is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-6 | SRS with TDMed 8 Tx ports—antenna switching | Support of SRS with TDMed 8 Tx ports—antenna switching FFS: detailed components FFS: separate row for comb8 | 40-5-4 | Yes | n/a | SRS with TDMed 8 Tx ports—antenna switching is not supported | Per FS | n/a | n/a | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-7 | SRS with TDMed 8 Tx ports—codebook | Support of SRS with TDMed 8 Tx ports—codebook FFS: detailed components FFS: merge with antenna switching |  | Yes | n/a | SRS with TDMed 8 Tx ports—codebook is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-5-8 | Maximum 2 SP and 1 periodic SRS sets for 8T8R antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for 8T8R antenna switching | 40-5-4 | Yes | n/a | Maximum 2 SP and 1 periodic SRS sets for 8T8R antenna switching is not supported | Per FS | n/a | n/a | n/a |  | Optional with capability signalling |

## STxMP (Simultaneous Transmission across Multiple Panels) enhancement

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-6-1 | Single-DCI based STx2P SDM scheme for PUSCH—codebook | 1. Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—codebook 2. 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—codebook 3. Support of two SRS resource sets with usage set to 'codebook' 4. Maximum number of SRS resources in one SRS resource set 5. Maximum number of layers of each panel for Single-DCI STx2P with SDM 6. Maximum total number of layers across both panels for Single-DCI STx2P with SDM 7. Maximum number of SRS antenna ports for each SRS resource in each SRS resource set [7. Max number of PUSCH ports associated with on SRS resource set] | 2-14 | Yes | N/A | Single-DCI based STx2P SDM scheme for PUSCH—codebook is not supported | Per FSPC | No | FR2 only | n/a | Component 4 candidate values: {1, 2 ,4} Component 5 candidate values: {1, 2} Component 6 candidate values: {2 ,3, 4} Component 7 candidate values: {1, 2 [,4]} Note: If value 4 is reported for component 4, UE also reports value 4 in FG 16-5c. | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-1-1 | 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook | Support of 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook | 40-6-1 | Yes | N/A | 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—codebook is not supported | Per FS | No | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-1a | Single-DCI based STx2P SDM scheme for PUSCH—noncodebook | 1. Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SDM and sTRP for PUSCH—noncodebook 2. 1 PTRS port for single-DCI based STx2P SDM scheme for PUSCH—noncodebook 3. Support of two SRS resource sets with usage set to 'noncodebook' 4. Maximum number of SRS resources in one SRS resource set 5. Maximum number of layers of each panel for Single-DCI STx2P with SDM 6. Maximum total number of layers across both panels for Single-DCI STx2P with SDM 7. Maximum number of simultaneous transmitted SRS resources at one symbol 8. Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol | FFS | Yes | N/A | Single-DCI based STx2P SDM scheme for PUSCH—noncodebook is not supported | Per FSPC | No | FR2 only | n/a | Component 4 candidate values: {1, 2 ,3, 4} Component 5 candidate values: {1, 2} Component 6 candidate values: {2 ,3, 4} Component 7 candidate values: {1, 2, 3, 4, 5, ,6, 7, 8} Component 8 candidate values: {1, 2, 3, 4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-1a-1 | 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook | Support of 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook | 40-6-1a | Yes | N/A | 2 PTRS ports for single-DCI based STx2P SDM scheme for PUSCH—noncodebook is not supported | Per FS | No | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-1b | Association between CSI-RS and SRS for noncodebook single-DCI based STx2P SDM scheme for PUSCH | 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STxMP SDM scheme for PUSCH 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously | [2-15a, 40-6-1a | Yes | N/A | Associated CSI-RS resources for non-codebook single-DCI based STxMP SDM scheme for PUSCH is not supported | Per band | n/a | FR2 only | n/a | Component 2 candidate values: {1 to 8} Component 3 candidate values: {1 to 8} Component 4 candidate values: {0 to 8} Component 5 candidate values: {1 to 16} Component 6 candidate values: {1 to 2} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-6-1-2 | New DMRS port entry for single-DCI based SDM scheme | Support of new DMRS port entry {0, 2, 3} | 40-6-1,40-6-1a | Yes | N/A | New DMRS port entry for single-DCI based SDM scheme is not supported | Per band | n/a | FR2 only | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-6-2 | Single-DCI based STx2P SFN scheme for PUSCH—codebook | 1. Support of single-DCI based STx2P SFN scheme for codebook based PUSCH 2.Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP 3. 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—codebook 4. Support of two SRS resource sets with usage set to 'codebook' 5. Supported number of SRS resources in one SRS resource set 6. Supported maximal number of MIMO layers of each SRS resource set for CB PUSCH with SFN scheme 7. Maximum number of SRS antenna ports for each SRS resource in each SRS resource set [8. Maximum total number of layers across both panels for Single-DCI STx2P with SFN] [9. Max number of PUSCH ports associated with SRS resource set] | 2-14 | Yes | n/a | Single-DCI based STx2P SFN scheme for PUSCH—codebook is not supported | Per FS | n/a | FR2 only | n/a | Component 5 candidate values: {1, 2 ,4} Component 6 candidate values: {1, 2} Component 7 candidate values: {1, 2 [,4]} Note: If value 4 is reported for component 5, UE also reports value 4 in FG 16-5c. | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-2-1 | 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook | Support of 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook | 40-6-2 | Yes | n/a | 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—codebook is not supported | Per FS | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-2a | Single-DCI based STx2P SFN scheme for PUSCH—noncodebook | 1. Support of single-DCI based STx2P SFN scheme for noncodebook based PUSCH 2.Dynamic switching by DCI 0_1/0_2 between single-DCI STxMP SFN and sTRP 3. 1 PTRS port for single-DCI based STx2P SFN scheme for PUSCH—noncodebook 4. Support of two SRS resource sets with usage set to 'noncodebook' 5. Supported number of SRS resources in one SRS resource set 6. Supported maximal number of MIMO layers of each SRS resource set for NCB PUSCH with SFN scheme [7. Maximum number of simultaneous transmitted SRS resources at one symbol] 8. Maximum number of simultaneous transmitted SRS resources from one SRS resource set at one symbol | FFS | Yes | n/a | Single-DCI based STx2P SFN scheme for PUSCH—noncodebook is not supported | Per FS | n/a | FR2 only | n/a | Component 5 candidate values: {1, 2 ,3, 4} Component 6 candidate values: {1, 2} Component 8 candidate values: {1, 2, 3, 4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-2a-1 | 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook | Support of 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook | 40-6-2a | Yes | n/a | 2 PTRS ports for single-DCI based STx2P SFN scheme for PUSCH—noncodebook is not supported | Per FS | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-2b | Association between CSI-RS and SRS for noncodebook single-DCI based STx2P SFN scheme for PUSCH | 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook based STxMP SFN scheme for PUSCH 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously | 2-15a | Yes | n/a | Associated CSI-RS resources for non-codebook single-DCI based STxMP SFN scheme for PUSCH is not supported | Per FS | n/a | FR2 only | n/a | Component 2 candidate values: {1 to 8} Component 3 candidate values: {1 to 8} Component 4 candidate values: {0 to 8} Component 5 candidate values: {1 to 16} Component 6 candidate values: {1 to 2} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3a | codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG | 1. Support of multi-DCI based STx2P PUSCH+PUSCH for codebook-based PUSCH with fully overlapping PUSCHs in time and frequency 2. Support of two SRS resource sets with usage set to 'codebook' associated with two coresetPoolIndex values 3. Maximum number of SRS resources in one SRS resource set 4. Maximum number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain [5. Supported number of NZP PUSCH ports for each PUSCH of PUSCH+PUSCH overlapping in time domain] 6. Maximum number of PUSCHs per CORESETPoolIndex per slot 7. Maximum total number of layers across two overlapping PUSCH 8. Maximum number of SRS antenna ports for each SRS resource in each SRS resource set | 2-14 | Yes | N/A | Codebook multi-DCI based STx2P PUSCH+PUSCH is not supported | per FS | n/a | FR2 only | n/a | Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex. Component 3 candidate values: {1,2,4} Note: If value 4 is reported for component 3, UE also reports value 4 in FG 16-5c. Component 4 candidate values: {1,2} [Component 5 candidate values: {1,2,4} Note: If a row of the TPMI consists of all 0’s, the corresponding PUSCH port is not counted.] Component 6 candidate values: {1,2,3,4,7} Note: per SCS, similar with Rel-15 Component 7 candidate values: {2 ,3, 4} Component 8 candidate values: {1, 2 ,4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3c | Multi-DCI STx2P scheme – fully overlapping in frequency, partially overlapping in time for codebook based PUSCH | - Support of Multi-DCI STx2P schedule with two PUSCHs fully overlapping in frequency and partially overlapping in time for codebook based PUSCH. |  |  |  |  | Per band |  | FR2 only |  |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3d | Multi-DCI STx2P scheme – partially or non-overlapping in frequency, fully overlapping in time for codebook based PUSCH | Support of Multi-DCI STx2P schedule with two PUSCHs partially or non-overlapping in frequency and fully overlapping in time for codebook based PUSCH |  |  |  |  | Per band |  | FR2 only |  |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3e | Multi-DCI STx2P scheme – partially or non-overlapping in frequency, partially overlapping in time for codebook based PUSCH | - Support of Multi-DCI STx2P schedule with two PUSCHs partially or non-overlapping in frequency and partially overlapping in time for codebook based PUSCH. |  |  |  |  | Per band |  | FR2 only |  |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3e | [Codebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in both frequency and time] | [Support of fully overlapping in both frequency and time] | [40-6-3a] | [Yes] | [N/A] | [Fully overlapping in both frequency and time for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a] |  | [Optional with capability signaling] |
| 40. NR_MIMO_evo_DL_UL | 40-6-3f | [Codebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in frequency, partially overlapping in time] | [Support of fully overlapping in frequency, partially overlapping in time] | [40-6-3a] | [Yes] | [N/A] | [Fully overlapping in frequency, partially overlapping in time for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a] |  | [Optional with capability signaling] |
| 40. NR_MIMO_evo_DL_UL | 40-6-3g | [Codebook multi-DCI based STx2P PUSCH+PUSCH – Partially or non-overlapping in frequency, partially or fully overlapping in time] | [Support of partially or non-overlapping in frequency, partially or fully overlapping in time] | [40-6-3a] | [Yes] | [N/A] | [Partially or non-overlapping in frequency, partially or fully overlapping in time for codebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a[ |  | [Optional with capability signaling] |
| 40. NR_MIMO_evo_DL_UL | 40-6-3h | Codebook multi-DCI based STx2P PUSCH+PUSCH for CG+CG | Support of multi-DCI based STxMP CG-PUSCH+CG-PUSCH [FFS: other components] | [40-6-3a] | Yes | N/A | Multi-DCI based STx2P for CG+CG is not supported for codebook | Per band | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3i | Codebook multi-DCI based STx2P PUSCH+PUSCH for DG+CG | Support of multi-DCI based STxMP DG-PUSCH+CG-PUSCH [FFS: other components] | [40-6-3a] | Yes | N/A | Multi-DCI based STx2P for DG+CG is not supported for codebook | Per band | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3b | Noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG | 1. Support of multi-DCI based STxMP PUSCH+PUSCH for noncodebook-based PUSCH with fully overlapping PUSCHs in time and frequency 2. Support of two SRS resource sets with usage set to 'noncodebook' associated with two coresetPoolIndex values 3. Maximum number of SRS resources in one SRS resource set 4. Maximum number of layers of each PUSCH of PUSCH+PUSCH overlapping in time domain 5. Maximum number of simultaneously transmitted SRS resources in one symbol per SRS resource set 6. Maximum number of PUSCHs per CORESETPoolIndex per slot 7. Maximum total number of layers across two overlapping PUSCH | 2-14 | Yes | N/A | Noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported | Per FS | n/a | FR2 only | n/a | Note: Processing capability 2 is not supported in any CC if at least one CC is configured with two values of CORESETPoolIndex. Component 3 candidate values: {1,2,3,4} Component 4 candidate values: {1,2} Component 5 candidate values: {1,2,3,4} Component 6 candidate values: {1,2,3,4,7} Note: per SCS, similar with Rel-15 Component 7 candidate values: {2 ,3, 4} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3j | Multi-DCI STx2P scheme – fully overlapping in frequency, partially overlapping in time for noncodebook based PUSCH | - Support of Multi-DCI STx2P schedule with two PUSCHs fully overlapping in frequency and partially overlapping in time for noncodebook based PUSCH. |  |  |  |  | Per band |  | FR2 only |  |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3k | Multi-DCI STx2P scheme – partially or non-overlapping in frequency, fully overlapping in time for noncodebook based PUSCH | - Support of Multi-DCI STx2P schedule with two PUSCHs partially or non-overlapping in frequency and fully overlapping in time for noncodebook based PUSCH. |  |  |  |  | Per band |  | FR2 only |  |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3l | Multi-DCI STx2P scheme – partially or non-overlapping in frequency, partially overlapping in time for noncodebook based PUSCH | - Support of Multi-DCI STx2P schedule with two PUSCHs partially or non-overlapping in frequency and partially overlapping in time for noncodebook based PUSCH. |  |  |  |  | Per band |  | FR2 only |  |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3j | [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Partial-overlapping PUSCHs in time and non-overlapping in frequency] | [Support of partial-overlapping PUSCHs in time and non-overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH] | [40-6-3a] | [Yes] | [N/A] | [Partial-overlapping PUSCHs in time and non-overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a] |  | [Optional with capability signaling] |
| 40. NR_MIMO_evo_DL_UL | 40-6-3k | [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Overlapping PUSCHs in time and partially overlapping in frequency] | [Support of overlapping PUSCHs in time and partially overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH] | [40-6-3a] | [Yes] | [N/A] | [Overlapping PUSCHs in time and partially overlapping in frequency for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a] |  | [Optional with capability signaling] |
| 40. NR_MIMO_evo_DL_UL | 40-6-3l | [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in both frequency and time] | [Support of fully overlapping in both frequency and time for noncodebook multi-DCI based STx2P PUSCH+PUSCH] | [40-6-3a] | [Yes] | [N/A] | [Fully overlapping in both frequency and time for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a] |  | [Optional with capability signaling] |
| 40. NR_MIMO_evo_DL_UL | 40-6-3m | [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Fully overlapping in frequency, partially overlapping in time] | [Support of fully overlapping in frequency, partially overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH] | [40-6-3a] | [Yes] | [N/A] | [Fully overlapping in frequency, partially overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a] |  | [Optional with capability signaling] |
| 40. NR_MIMO_evo_DL_UL | 40-6-3n | [Noncodebook multi-DCI based STx2P PUSCH+PUSCH – Partially or non-overlapping in frequency, partially or fully overlapping in time] | [Support of partially or non-overlapping in frequency, partially or fully overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH] | [40-6-3a] | [Yes] | [N/A] | [Partially or non-overlapping in frequency, partially or fully overlapping in time for noncodebook multi-DCI based STx2P PUSCH+PUSCH is not supported] | [Per Band] | [n/a] | [FR2 only] | [n/a] |  | [Optional with capability signaling] |
| 40. NR_MIMO_evo_DL_UL | 40-6-3o | Noncodebook multi-DCI based STx2P PUSCH+PUSCH for CG+CG | Support of multi-DCI based STxMP CG-PUSCH+CG-PUSCH for noncodebook [FFS: other components] | [40-6-3a] | Yes | N/A | Multi-DCI based STx2P for CG+CG is not supported for noncodebook is not supported | Per band | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3p | Noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+CG | Support of multi-DCI based STxMP DG-PUSCH+CG-PUSCH for noncodebook [FFS: other components] | [40-6-3a] | Yes | N/A | Multi-DCI based STx2P for DG+CG is not supported for noncodebook is not supported | Per band | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3r-1 | Single-DCI based STx2P SDM scheme for PUSCH and repetition in time | 1. single-DCI based STx2P and semi-static indication of PUSCH repetitions over multiple slots 2. single-DCI based STx2P and dynamic indication of repetition Type-A 3. single-DCI based STx2P SDM and dynamic indication of repetition Type-B | 40-6-1, or 40-6-1a | Yes | N/A | UE cannot be indicated to perform STx2P SDM scheme over R15/16 PUSCH repetitions in time | Per FSPC | No |  | n/a | Notes: For component 1, UE also reports 5-17, and/or 5-16, and/or 5-14. For component 2, UE also reports 11-6. For component 3, UE also reports 11-5. | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3r-2 | Single-DCI based STx2P SFN scheme for PUSCH and repetition in time | 1. single-DCI based STx2P and semi-static indication of PUSCH repetitions over multiple slots 2. single-DCI based STx2P and dynamic indication of repetition Type-A 3. single-DCI based STx2P SDM and dynamic indication of repetition Type-B | 40-6-2, or 40-6-2a | Yes | N/A | UE cannot be indicated to perform STx2P SFN scheme over R15/16 PUSCH repetitions in time | Per FSPC | No |  | n/a | Notes: For component 1, UE also reports 5-17, and/or 5-16, and/or 5-14. For component 2, UE also reports 11-6. For component 3, UE also reports 11-5. | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3r-3 | multi-DCI based STx2P for PUSCH+PUSCH and repetition in time for at least one of the PUSCHs | 1. multi-DCI based STx2P for PUSCH+PUSCH and semi-static indication of PUSCH repetitions over multiple slots 2. multi-DCI based STx2P for PUSCH+PUSCH and dynamic indication of repetition Type-A 3. multi-DCI based STx2P for PUSCH+PUSCH and dynamic indication of repetition Type-B | 40-6-3a, or 40-6-3b | Yes | N/A | UE cannot be indicated to perform STx2P SDM scheme over R15/16 PUSCH repetitions in time | Per FSPC | No |  | n/a | Notes: For component 1, UE also reports 5-17, and/or 5-16, and/or 5-14. For component 2, UE also reports 11-6. For component 3, UE also reports 11-5. | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3q | Multi-DCI STx2P with different PHY priorities | Support of Multi-DCI STx2P with different PHY priorities | FG12-1 |  |  |  | Per FS |  | FR2 only |  |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-3b-1 | Associated CSI-RS resources for noncodebook multi-DCI based STx2P PUSCH+PUSCH | 1. Support of up to two NZP CSI-RS resources associated with the two SRS resource sets for non-codebook multi-DCI based STx2P PUSCH+PUSCH 2. Maximum number of periodic SRS resources associated with first and second CSI-RS per BWP 3. Maximum number of aperiodic SRS resources associated with first and second CSI-RS per BWP 4. Maximum number of semi-persistent SRS resources associated with first and second CSI-RS per BWP 5. UE can process Y SRS resources associated with first and second CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS 6. UE can process up to X CSI-RS resources associated with SRS for non-codebook-based transmission simultaneously |  |  |  |  | Per band |  |  |  | Component 2 candidate values: {1 to 8} Component 3 candidate values: {1 to 8} Component 4 candidate values: {0 to 8} Component 5 candidate values: {1 to 16} Component 6 candidate values: {1 to 2} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-6-4 | Single-DCI based STx2P SFN scheme for PUCCH | 1. Support of single-DCI based STx2P SFN scheme for PUCCH 2. Supported PUCCH formats for STxMP SFN scheme | 23-3-2d | Yes | n/a | Single-DCI based STx2P SFN scheme for PUCCH is not supported | Per FS | n/a | FR2 only | n/a |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-6-4a | Rel-18 STxMP SFN scheme and the Rel-17 repetition number parameter pucch-RepetitionNrofSlots in a same PUCCH resource at the same time | Support of enabling Rel-18 STxMP SFN scheme and the Rel-17 repetition number parameter pucch-RepetitionNrofSlots in a same PUCCH resource at the same time |  |  |  |  | Per band |  |  |  |  |  |
| 40. NR_MIMO_evo_DL_UL | 40-6-5 | Support grouped-based beam reporting for STx2P | - Support grouped-based beam reporting for STx2P. |  |  |  |  | Per band |  | FR2 only |  | Component 1 candidate value: {mode 1, mode 2, both} Note: Mode 1: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously, and the reported pair of CRIs or SSBRIs can be received simultaneously. Mode 2: For each reported pair of CRIs or SSBRIs, the UL Tx spatial filters determined from the reported pair of CRIs or SSBRIs can be applied simultaneously. | Optional with capability signalling |

## 8Tx UL operation

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40. NR_MIMO_evo_DL_UL | 40-7-1 | Basic features for Codebook-based 8Tx PUSCH | 1. Supported maximal PUSCH MIMO layers for codebook based PUSCH 2. Supported maximum number of 8 port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 8Tx PUSCH | FFS | Yes | n/a | Codebook-based 8Tx PUSCH is not supported | Per FSPC | No | No | No | Component 1 candidate values: {1, 2, [3,] 4, [5, 6, 7,] 8} Component 2 candidate values: {1, 2} A UE that supports FG 40-7-1 must support at least one of FGs 40-7-1a/b/c/d | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1a | Codebook-based 8Tx PUSCH—[full-coherent codebook] Higher-layer parameter CodebookType = “codebook1” | Support of codebook-based 8Tx PUSCH— Higher-layer parameter CodebookType = “codebook1” | 40-7-1 | Yes | n/a | Codebook-based 8Tx PUSCH— Higher-layer parameter CodebookType = “codebook1” is not supported | Per FS | No | No | No | Candidate values: {N1=4 and N1=1, N1=2 and N2=2} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1b | (N1,N2) for coherent codebook Codebook-based 8Tx PUSCH—[partial-coherent codebook], Higher-layer parameter CodebookType = “codebook2” | Support of codebook-based 8Tx PUSCH—[partial-coherent codebook], Higher-layer parameter CodebookType = “codebook2” | 40-7-1 | Yes | n/a | Codebook-based 8Tx PUSCH - Higher-layer parameter CodebookType = “codebook2” is not supported | Per FS | No | No | No |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1c | Codebook-based 8Tx PUSCH— [partial-coherent codebook], Higher-layer parameter CodebookType = “codebook3” | Support of codebook-based 8Tx PUSCH— [partial-coherent codebook], Higher-layer parameter CodebookType = “codebook3” | 40-7-1 | Yes | n/a | Codebook-based 8Tx PUSCH— Higher-layer parameter CodebookType = “codebook3” is not supported | Per FS | No | No | No |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1d | Codebook-based 8Tx PUSCH, Higher-layer parameter CodebookType = “codebook4” | Support of codebook-based 8Tx PUSCH— Higher-layer parameter CodebookType = “codebook4” | 40-7-1 | Yes | n/a | Codebook-based 8Tx PUSCH— Higher-layer parameter CodebookType = “codebook4” is not supported | Per FS | No | No | No |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1e | UL full power transmission mode 0 | Support of UL full power transmission mode of fullpower when UE is capable of 8 Tx codebook based PUSCH operation | FFS | yes | n/a | FFS | Per FS | n/a | n/a | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1f | UL full power transmission mode 1 | Support of UL full power transmission mode of fullpowerMode1 when UE is capable of 8 Tx codebook based PUSCH operation | FFS | yes | n/a | FFS | Per FS | n/a | n/a | n/a |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1g | UL full power transmission mode 2 | 1. Support of UL full power transmission mode of fullpowerMode2 when UE is capable of 8 Tx codebook based PUSCH operation 2. Maximum number of SRS resources in one SRS resource set with usage set to 'codebook' for 8Tx codebook based PUSCH for Mode 2 | FFS | yes | n/a | FFS | Per FS | n/a | n/a | n/a | Component 2 candidate values: {1, 2, 4} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1h | UL full power transmission mode of fullpowerMode2 for 8 Tx – SRS resource | The supported SRS configurations with different number of antenna ports per SRS resource for full power Mode 2 when UE is capable of and configured with 8 Tx codebook based PUSCH operation |  |  |  |  | Per FS |  |  |  | Component 1 candidate values: {1_8, 1_2_8, 1_4_8, 1_2_4_8} 1st state (1_8): each SRS resource can be configured with 1 port or 8 ports. 2nd state (1_2_8): each SRS resource can be configured with 1 port or 2 ports or 8 ports. 3rd state (1_4_8): each SRS resource can be configured with 1 port or 4 ports or 8 ports. 4th state (1_2_4_8): each SRS resource can be configured with 1 port or 2 port or 4 ports or 8 ports. | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1i | UL full power transmission mode of fullpowerMode2 for 8 Tx – full power TPMI groups, Higher-layer parameter CodebookType = “codebook2” | - TPMI group(s) which delivers full power when UE is capable of and configured with 8 Tx codebook based PUSCH operation, Higher-layer parameter CodebookType = “codebook2” |  |  |  |  | Per FS |  |  |  | Component 1 candidate values: {first coherent antenna port group, second coherent antenna port group} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-2 | Basic features for Non-Codebook-based 8Tx PUSCH | 1. Supported maximal PUSCH MIMO layers for non-codebook based PUSCH 2. Supported maximum number of SRS resources per SRS resource set with usage set to 'nonCodebook’ 3. Maximum number of simultaneous transmitted SRS resources at one symbol | FFS | Yes | n/a | Non-codebook based 8Tx PUSCH is not supported | Per FSPC | No | No | No | Component 1 candidate values: {1, 2, [3, ]4, [5, 6, 7, ]8} Component 2 candidate values: {5, 6, 7, 8} Component 3 candidate values: {1, 2, 3, 4, 5, 6, 7, 8} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-2a | Association between CSI-RS and SRS for non-codebook case | 1. Support association between NZP-CSI-RS and SRS resource set via RRC parameter "SRS-ResourceSet" for noncodebook 8Tx PUSCH operation 2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources, and total # of Tx ports} across all CCs simultaneously. | FFS | yes | n/a | Association between CSI-RS and SRS for non-codebook case is not supported | Per band | No | No | No | Component 2 candidate value: Maximum size of the list is 16. The candidate values for the max # of Tx port in one resource is {2, 4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {1 to 64} The candidate value set of total # of ports is: {2 to 256} | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-7-3 | CBG based 2 CWs PUSCH with rank >4 | Support CBG based transmission for 2 CWs PUSCH | FFS | yes | n/a | CBG based transmission for 2 CWs PUSCH is not supported | Per FS | No | No | No |  | Optional with capability signalling |
| 40. NR_MIMO_evo_DL_UL | 40-7-4e | UL full power transmission mode of fullpowerMode2 | FFS How to support downgrade configuration, i.e., UE is configured with 2/4 Tx while UE supports 8Tx. Whether and how to use the legacy FG16-5c, FG16-5c-2, FG16-5c-3 |  |  |  |  | Per FS |  |  |  |  | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1a-1 | PUSCH codebook coherency subset when an 8 Tx capable UE is configured with 2 Tx codebook based PUSCH operation | 1.Supported codebook coherency subset type when an 8 Tx capable UE is configured with 2 Tx codebook based PUSCH. |  |  |  |  | Per band |  |  |  | Component 1 candidate values: {non-coherent, non-coherent and full-coherent} | Optional with capability signaling |
| 40. NR_MIMO_evo_DL_UL | 40-7-1a-2 | PUSCH codebook coherency subset when an 8 Tx capable UE is configured with 4 Tx codebook based PUSCH operation | Supported codebook coherency subset type when an 8 Tx capable UE is configured with 4 Tx codebook based PUSCH operation. |  |  |  |  | Per band |  |  |  | Component 1 candidate values: {non-coherent, non-coherent and partial coherent, non-coherent and partial coherent and full-coherent} FFS: whether to reuse legacy FG2-13 assuming UE reports that UE supports 4Tx in legacy | Optional with capability signaling |

# 2. Reference

[1] R1-2310635, “RAN1 UE features list for Rel-18 NR after RAN1_114bis”, 3GPP TSG RAN WG1 #114bis, Xiamen, China, October 9th – October 13th, 2023.