3GPP TSG RAN WG1 #115	    			            R1-2311830

Chicago, IL, USA, November 13th – November 17th, 2023

Agenda item:	8.1.1.2

Source: 	Samsung

Title: 	Views on two TAs for m-DCI

Document for:	Discussion and Decision

# Introduction

A work item has been approved to further evolve MIMO in Rel-18 [1]. Rel-18 improvements include identifying and specifying necessary enhancements for uplink MIMO. In particular, one of the focus areas of Rel-18 is multi-TRP operation, where the timing advance can be different for UL transmission to each TRP. To this end, one of the objectives of the Rel-18 MIMO WI is to study and if justified specify support for two TAs:

7. Study, and if justified, specify the following

- Two TAs for UL multi-DCI for multi-TRP operation

In RAN#114, the work item was complete. The specifications have been updated to include support of 2TA for multi-DCI multi-TRP. In RAN1#114b, additional updates where made to specifications. In this document, we discuss a few remaining maintenance issues.

# Discussion

In this document, we consider the following remaining details for the PDCCH order design:

- Indication for cross-TRP PDCCH order PRACH triggering

- PRACH power control

In addition, we consider:

- The association of DL reference timings to TAG ID.

## Remaining details of PDCCH order design

An initial TA value is an absolute TA, that determines the time of uplink transmission from a UE relative to a reference time. As discussed above, it has been agreed to use CFRA-based PDCCH order to acquire the TA of the second TRP. However, there are some remaining design aspects that need to be further considered.

## Intra-cell cross-TRP PDCCH order PRACH triggering

In RAN1#115, it has been agreed to support intra-cell cross-TRP triggering of PRACH by a PDCCH order for multi-DCI based multi-TRP operation with two TA.

Agreement

The following working assumption is confirmed

- “For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, support the case where a PDCCH order sent by TRPX triggers RACH procedure towards either TRPX or TRPY.”

Above confirmation does not change power control for the same TRP PDCCH order.

Furthermore, for inter-cell cross-TRP triggering of PRACH by a PDCCH order for multi-DCI based multi-TRP operation with two TA it has been agreed that 1-bit flag is used for indicating active additionalPCI in the PDCCH order that is separate from the field used for LTM.

Agreement

For inter-cell multi-DCI based Multi-TRP operation with two TA enhancement, 1 bit is supported for indicating active additionalPCI in the PDCCH order

- the single bit in the PDCCH order indicates if the PRACH triggering is towards servingCell PCI or active additional PCI

Note: This has no impact on whether common or separate field with cell indication in LTM is used

Agreement

When a UE is configured with both the inter-cell multi-DCI based Multi-TRP operation with two TAs and Rel-18 LTM features,

- Alt 1:  separate fields are used to indicate additionalPCI (for inter-cell mTRP) and to indicate cell indicator field (for Rel-18 LTM)

There are two remaining issues; the first is how to indicate an activated additionalPCI in the PDCCH order using a one-bit flag. For this, we suggest that when the one-bit flag is “1”, the PDCCH order indicates the cell associated with the activated additionalPCI. When the one-bit flag is “0” the PDCCH order indicates the serving cell. Therefore, we suggest the following TP for TS 38.212 for the PCI indicator field:

Text proposal 1: (for TS 38.212 for PCI indicator field)

If the value of “PCI indicator” is 0, the PRACH transmission is associated with a serving cell PCI, if the value of “PCI indicator” is 1, the PRACH transmission is associated with an additionalPCI with active TCI states.

The second remaining issue is how to indicate cross-TRP triggering for the intra-cell case. For this scenario, we suggest to re-use the same field (PCI indicator) for the intra-cell case, such that when the PCI indicator is “0”, the PRACH is towards the same TRP as the PDCCH order, when PCI indicator is “1”, the PRACH is towards the other TRP. The name of the field can be updated to cover inter-cell and intra-cell use cases.

Proposal 1:

Re-use the “PCI indicator” field for intra-cell cross-TRP triggering of PRACH by a PDCCH order for multi-DCI based multi-TRP operation with two TA.

- If PCI indicator is “0”, the PRACH is towards the same TRP as the PDCCH order.

- If PCI indicator is “0”, the PRACH is towards the other.

Text Proposal 2: (for TS 38.212 (R1-2310748) for PCI indicator field)

-

PCI PRACH transmission indicator - 0 or 1 bit

-	1bit indicating the PCI or reference signal associated with the PRACH transmission if the UE is provided with tag-Id2 and SSB-MTC-AddtionalPCI, and the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs. If UE is provided SSB-MTC-AdditinalPCI, if the value of “PRACH transmission indicator” is 0, the PRACH transmission is associated with a serving cell PCI, if the value of “PRACH transmission indicator” is 1, the PRACH transmission is associated with an additionalPCI with active TCI states. If UE is not provided SSB-MTC-AdditinalPCI, if the value of “PRACH transmission indicator” is 0, the PRACH transmission depends on the DL RS that the DM-RS of the PDCCH order is quasi-collocated with as described in clause 10.1 of TS 38.213, if the value of “PRACH transmission indicator” is 1, the PRACH transmission depends on the SS/PBCH index provided by the PDCCH order.

-	0 bit otherwise.

In RAN1#111, the following agreement was made for inter-cell cross-TRP PDCCH order PRACH triggering:

Agreement

For multi-DCI based Multi-TRP operation with two TA enhancement, support the case where a PDCCH order sent by one TRP triggers RACH procedure towards either the same TRP or a different TRP at least for inter-cell Multi-DCI.

- FFS: for intra-cell Multi-DCI

- FFS: whether there are any restrictions needed

- FFS: if cross TRP RACH triggering is an optional feature

An open point in this agreement is whether any restrictions are needed. Cross-TRP triggering of PRACH is useful when the serving cell is sending the PDCCH order and the PRACH is transmitted towards a cell other than the serving cell. There is no identifiable reason for the scenario where a PDCCH order is sent from a non-serving cell and PRACH is towards a serving cell or towards another non-serving cell to be beneficial. Therefore, we suggest to limit inter-cell cross-TRP PDCCH order PRACH triggering to the case when the PDCCH order, from a serving cell, triggers a PRACH transmission towards a non-serving cell.

Proposal 2: For inter-cell cross-TRP PDCCH order PRACH triggering, only the serving cell can trigger a PRACH transmission towards another cell.

## PRACH power control

In RAN1#112b-e, it was agreed to further study the power control of the PRACH transmission in case of cross-TRP PRACH triggering:

Agreement

For multi-DCI based inter-cell multi-TRP and intra-cell multi-TRP operation with two TAGs configured in a CC, for a CFRA based PDCCH order from one TRP triggering PRACH towards another TRP, study whether and, if needed, how to determine the transmit power of the triggered PRACH preamble

In RAN1#114b, the following two FL alternatives were made, but not agreed:

There are two different Alternative approaches being proposed in the TDocs.  The following is a summary of views expressed:

Atl 1:

SSB indicated in the CFRA based PDCCH order is used as the PL-RS for determining the transmit power of the triggered PRACH transmission.

- Note:  the UE expects the SSB index to satisfy the "Known conditions for pathloss reference signal" of Section 8.14.2 of 38.133.

Alt 2:

- when a PDCCH order sent by TRPX triggers RACH procedure towards the same TRPX, the legacy rule applies for PRACH power control;

- when a PDCCH order sent by TRPX triggers RACH procedure towards TRPY (X≠Y) associated with an active additionalPCI, the PL-RS for determining the transmit power of the triggered PRACH transmission is the DL-RS of either the first indicated TCI state or the second indicated TCI state that is associated with the active additionalPCI;

In LTM, it has already been agreed in RAN1#113 that SSB indicated in the PDCCH order can be used for determining the PRACH transmission power:

Agreement

For PDCCH order based PRACH to candidate cell, the candidate cell SSB indicated in the PDCCH order serves as the path loss RS for PRACH Tx power determination.

Furthermore, RAN4 in LS (R4-2314454) has indicated scenarios where additional latency will be required or will not be required between the PDCCH order and the PRACH transmission. It seems natural to reuse the LTM approach for determining the PL RS for PRACH transmission. This minimize the spec impact and the implementation complexity by leveraging the same solution for 2TA and LTM

Observation 1:

When PRACH is transmitted towards a TRP that is different from the TRP that transmits PDCCH order, leveraging the LTM solution for determining the PL-RS for PRACH reduces implementation complexity and specification impact.

If the SS/PBCH block used as PL-RS for PRACH transmission is not known and maintained by the UE additional latency is incurred for the UE to measure the SSB and determine the PL. Therefore, it will be beneficial to have the SSB indicated in the PDCCH order and used as a PL-RS be associated with the active TCI states.

Observation 2:

Having the SS/PBCH index indicated in the PDCCH order, used as PL-RS for PRACH, associated with activated TCI states mitigates the need for additional latency between PDCCH order and PRACH transmission.

The UE can be indicated any activated TCI state with the activated TCI state list and is expected to start receiving and/or transmitting using the indicted TCI state during a beam application time, during which there may be no SSB to perform additional measurements. Hence, using an SS/PBCH block associated with indicated TCI states rather than activated TCI states provides no additional latency benefit for the latency between the PDCCH order and PRACH transmission.

Observation 3:

Having an SS/PBCH block associated with an indicated TCI state, provides no additional latency benefit over having an SSB/PBCH block associated with an activated TCI state.

Based on the previous discussion, it seems natural that for 2TA, with cross-TRP PRACH triggering, to use the SS/PBCH index indicated in the PDCCH order as a PL-RS (same as LTM solution). From a latency perspective, it is beneficial to have the SS/PBCH index associated with an activated TCI state.

Proposal 3:

When PRACH is transmitted towards a TRP that is different from the TRP that transmits PDCCH order,

- For multi-DCI based inter-cell multi-TRP and intra-cell multi-TRP operation with two TAGs configured in a CC, SSB indicated in the CFRA based PDCCH order is used as the PL-RS for determining the transmit power of the triggered PRACH transmission.

- UE expects the indicated SSB in PDCCH order to be associated with an activated TCI state.

This follows closely PRACH power control for mobility. Based on this proposal, we suggest the following TP for clause 7.4 of TS 38.213 (v18.0.0)

Text Proposal 3: TS 38.213 (v18.0.0) clause 7.4

If a PRACH transmission from a UE is in response to a detection of a PDCCH order by the UE that triggers a contention-free random access procedure and depending on the DL RS that the DM-RS of the PDCCH order is quasi-collocated with as described in clause 10.1 when the PRACH transmission is on a serving cell or when PCI Indicator in the PDCCH order [5, TS 38.212] is 0, or depending on an indicated SS/PBCH block when the PRACH transmission is on a non-serving cell or when PCI Indicator in the PDCCH order [5, TS 38.212] is 1, referenceSignalPower is provided by a corresponding ss-PBCH-BlockPower. If the UE is configured resources for a periodic CSI-RS reception or the PRACH transmission is associated with a link recovery procedure where a corresponding index qnew is associated with a periodic CSI-RS configuration as described in clause 6, referenceSignalPower is obtained by ss-PBCH-BlockPower and powerControlOffsetSS where powerControlOffsetSS provides an offset of CSI-RS transmission power relative to SS/PBCH block transmission power [6, TS 38.214]. If powerControlOffsetSS is not provided to the UE, the UE assumes an offset of 0 dB. If the active TCI state for the PDCCH that provides the PDCCH order includes two RS, the UE expects that one RS is configured with qcl-Type set to 'typeD' and the UE uses the one RS when applying a value provided by powerControlOffsetSS.

A variant of the last proposal can be that the UE selects, as PL-RS, any SS/PBCH block associated with an active TCI state that has the same TAG ID as the TCI state associated with the SS/PBCH block indicated in the PDCCH order. This gives the UE flexibility in choosing a SS/PBCH block to the same TRP for PL estimation.

## DL reference timing for two TA

In RAN#114, the issue of DL reference timing for two TAs was discussed and captured in the FL summary. When the UE maintains two TAGs for uplink transmission, the UE can maintain two DL reference timings for DL reception, with a reference timing associated with each TAG ID. When transmitting in UL, the UE uses the DL reference timing corresponding to the TAG ID used for UL transmission.

Proposal 4: For an uplink transmission using a TAG ID, the UE uses a DL reference timing corresponding to the TAG ID used to determine the timing of the UL transmission.

TS 38.213 already describes the association of two DL reference timings with two TAG IDs in the following:

If a UE operates with two TAGs on an active UL BWP of a serving cell, the UE expects that a difference between a first downlink timing associated with a first TAG and a second downlink timing associated with a second TAG is not larger than the CP length for the active UL BWP unless the UE indicates larger-thanCP-capability.

Therefore, there is no need for a spec update.

To assist the UE in determining which DL signals are associated with each TAG ID, we suggest dividing the SSBs into two groups, with each group associated with a TAG ID as illustrated in Figure 1. When the UE determines a DL reference time based on an SSB reception, the UE would be able to determine which DL reference time group the DL reference time is for. The UE would then be able to directly group SSBs associated with the same TAG ID in the same DL reference time group.

Figure 1: Association of SSBs to TAG IDs.

Proposal 5: For multi-DCI based multi-TRP operation with two TA enhancement, the SSBs are divided into two groups, each group has a same DL reference time and is associated with a corresponding TAG ID.

# Conclusions

The following proposals, text proposals and observation have been made regarding support of 2 TAs:

Text proposal 1: (for TS 38.212 for PCI indicator field)

If the value of “PCI indicator” is 0, the PRACH transmission is associated with a serving cell PCI, if the value of “PCI indicator” is 1, the PRACH transmission is associated with an additionalPCI with active TCI states.

Proposal 1:

Re-use the “PCI indicator” field for intra-cell cross-TRP triggering of PRACH by a PDCCH order for multi-DCI based multi-TRP operation with two TA.

- If PCI indicator is “0”, the PRACH is towards the same TRP as the PDCCH order.

- If PCI indicator is “0”, the PRACH is towards the other.

Text Proposal 2: (for TS 38.212 (R1-2310748) for PCI indicator field)

-

PCI PRACH transmission indicator - 0 or 1 bit

-	1bit indicating the PCI or reference signal associated with the PRACH transmission if the UE is provided with tag-Id2 and SSB-MTC-AddtionalPCI, and the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for the first CORESETs, and is provided coresetPoolIndex with value 1 for the second CORESETs. If UE is provided SSB-MTC-AdditinalPCI, if the value of “PRACH transmission indicator” is 0, the PRACH transmission is associated with a serving cell PCI, if the value of “PRACH transmission indicator” is 1, the PRACH transmission is associated with an additionalPCI with active TCI states. If UE is not provided SSB-MTC-AdditinalPCI, if the value of “PRACH transmission indicator” is 0, the PRACH transmission depends on the DL RS that the DM-RS of the PDCCH order is quasi-collocated with as described in clause 10.1 of TS 38.213, if the value of “PRACH transmission indicator” is 1, the PRACH transmission depends on the SS/PBCH index provided by the PDCCH order.

-	0 bit otherwise.

Proposal 2: For inter-cell cross-TRP PDCCH order PRACH triggering, only the serving cell can trigger a PRACH transmission towards another cell.

Observation 1:

When PRACH is transmitted towards a TRP that is different from the TRP that transmits PDCCH order, leveraging the LTM solution for determining the PL-RS for PRACH reduces implementation complexity and specification impact.

Observation 2:

Having the SS/PBCH index indicated in the PDCCH order, used as PL-RS for PRACH, associated with activated TCI states mitigates the need for additional latency between PDCCH order and PRACH transmission.

Observation 3:

Having an SS/PBCH block associated with an indicated TCI state, provides no additional latency benefit over having an SSB/PBCH block associated with an activated TCI state.

Proposal 3:

When PRACH is transmitted towards a TRP that is different from the TRP that transmits PDCCH order,

- For multi-DCI based inter-cell multi-TRP and intra-cell multi-TRP operation with two TAGs configured in a CC, SSB indicated in the CFRA based PDCCH order is used as the PL-RS for determining the transmit power of the triggered PRACH transmission.

- UE expects the indicated SSB in PDCCH order to be associated with an activated TCI state.

Text Proposal 3: TS 38.213 (v18.0.0) clause 7.4

If a PRACH transmission from a UE is in response to a detection of a PDCCH order by the UE that triggers a contention-free random access procedure and depending on the DL RS that the DM-RS of the PDCCH order is quasi-collocated with as described in clause 10.1 when the PRACH transmission is on a serving cell or when PCI Indicator in the PDCCH order [5, TS 38.212] is 0, or depending on an indicated SS/PBCH block when the PRACH transmission is on a non-serving cell or when PCI Indicator in the PDCCH order [5, TS 38.212] is 1, referenceSignalPower is provided by a corresponding ss-PBCH-BlockPower. If the UE is configured resources for a periodic CSI-RS reception or the PRACH transmission is associated with a link recovery procedure where a corresponding index qnew is associated with a periodic CSI-RS configuration as described in clause 6, referenceSignalPower is obtained by ss-PBCH-BlockPower and powerControlOffsetSS where powerControlOffsetSS provides an offset of CSI-RS transmission power relative to SS/PBCH block transmission power [6, TS 38.214]. If powerControlOffsetSS is not provided to the UE, the UE assumes an offset of 0 dB. If the active TCI state for the PDCCH that provides the PDCCH order includes two RS, the UE expects that one RS is configured with qcl-Type set to 'typeD' and the UE uses the one RS when applying a value provided by powerControlOffsetSS.

Proposal 4: For an uplink transmission using a TAG ID, the UE uses a DL reference timing corresponding to the TAG ID used to determine the timing of the UL transmission.

Proposal 5: For multi-DCI based multi-TRP operation with two TA enhancement, the SSBs are divided into two groups, each group has a same DL reference time and is associated with a corresponding TAG ID.

# References

- RP-213598, “MIMO Evolution for Downlink and Uplink”, 3GPP RAN#94e, e-Meeting, December, 2021.