3GPP TSG RAN WG1 #115			R1-2311870

Chicago, USA, November 13th – November 17th, 2023

Agenda Item:		8.16.4

Source:			Samsung

Title:			UE features for eRedCap

Document for:		Discussion and decision

1 Introduction

In RAN1#113 meeting, following UE feature draft was agreed [1]:

| 48. NR_redcap_enh | 48-1 | RedCap UE with reduced peak data rate and reduced baseband bandwidth in FR1 | The following components are the same as for supportOfRedCap-r17 (28-1): 1. Maximum FR1 RedCap UE bandwidth is 20 MHz. 3. Early indication of RedCap UE in Msg.1 for 4-step RACH 4. Separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources 5. Separate initial DL BWP for RedCap UEs - It includes CSS/CORESET for random access - For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included - For separate initial DL BWP used in connected mode as BWP#0 configuration option 1, CD-SSB is included 6. 1 UE-specific RRC configured DL BWP per carrier 7. 1 UE-specific RRC configured UL BWP per carrier 8. RRC reconfiguration of any parameters related to BWP 9. UE-specific RRC configured DL BWP with CD-SSB or NCD-SSB 10. NCD-SSB based measurements in RRC-configured DL BWP The following components are new compared to supportOfRedCap-r17 (28-1): [11. DL/UL peak data rate target of 10 Mbps] 12. Maximum number of PDSCH/PUSCH PRBs that can be scheduled for unicast per slot of 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS 13. Relaxed RAR-PDSCH processing timeline FFS whether to add additional components |  | Y |  | Network assumes the UE is not a RedCap UE with reduced peak data rate and reduced baseband bandwidth in FR1. | [Per UE] | [No] |  | [No] (FR1 only) |  |  | A UE supporting this FG is not required to support FG 6-1. A UE supporting this FG is not allowed to support FG 28-1. The specifications for a UE supporting FG 28-1 (‘RedCap UE’) also apply for a UE supporting this FG (FG 48-1) unless stated otherwise. It is up to RAN2 whether/how to capture the capabilities for early indication of RedCap UE in Msg 3 and Msg A. |  | Optional with capability signaling UEs supporting Rel-18 eRedCap UE complexity reduction feature(s) indicate support of this FG instead of FG 28-1 (supportOfRedCap-r17). |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 48. NR_redcap_enh | 48-2 | RedCap UE with reduced peak data rate without reduced baseband bandwidth in FR1 | The capabilities of FG 48-2 are the same as for FG 48-1 except that the following restriction does not apply: 12. Maximum number of PDSCH/PUSCH PRBs that can be scheduled for unicast per slot of 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS [13. Relaxed RAR-PDSCH processing timeline] | 48-1 | Y |  | Network assumes the UE is not a RedCap UE with reduced peak data rate without reduced baseband bandwidth in FR1. | [Per UE] | [No] | [No] (FR1 only) |  |  |  |  |  |  | Optional with capability signaling |  |

And, in RAN1#114 meeting and RAN1#114bis meeting, following agreements on FG 48 were made [2-3]:

Agreement

- Reporting type of FG 48-1/48-2 is “per UE” without FDD/TDD differentiation and without FR1/FR2 differentiation

  - The column of “Need of FR1/FR2 differentiation” is updated to “FR1 only”

Agreement

- Component 11 in FG 48-1 is confirmed as “DL/UL peak data rate target of 10 Mbps corresponding to vLayers·Qm·f = 3.2”

- Add following in the component column of FG 48-2

  - Component 11 in FG 48-1 does not apply and DL/UL peak data rate target of 10 Mbps corresponding to vLayers·Qm·f = 0.75 when vLayers = 1 and vLayers·Qm·f = 0.8 when vLayers = 2

Agreement

- Component 13 in FG 48-1 is revised as follows

  - Relaxed processing timeline of 1/0.5 ms for 15/30 kHz SCS when the RAR PDSCH and MsgB PDSCH (if supported) is larger than 25/12 PRBs for 15/30 kHz SCS

Agreement

- Add Component 14 in FG 48-1 as “Network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs”

- Add a note in FG 48-1 as “It is up to RAN2 whether/how to capture the capabilities for additional separate early indication of Rel-18 eRedCap UE in Msg 3 and Msg A PUSCH.”

Agreement

- Component 13 in FG 48-1 is supported by FG 48-2 during initial access. Revisit component 13 for FG 48-2 if RAN2 agrees on differentiation of barring for Rel-18 eRedCap UEs

Agreement

- Component 12 in FG48-1 is revised as follows:

  - Maximum number of PDSCH/PUSCH PRBs that can be scheduled/configured for unicast per slot is of 25 PRBs for 15 kHz SCS and is 12 PRBs for 30 kHz SCS

Agreement

- Add following component in FG48-1

  - Maximum number of Msg4 PDSCH PRBs that can be [decoded/scheduled] and maximum number of Msg 3 PUSCH PRBs and Msg A PUSCH PRBs (if supported) that can be [transmitted/scheduled] is 25 PRBs for 15 kHz SCS and is 12 PRBs for 30 kHz SCS

In this contribution, we discuss the UE features for eRedcap.

2 Discussion

In the last RAN1 meeting, one agreement was made for RAR PDSCH timeline relaxation for FG 48-2 UE as following:

Agreement - The following does not apply to FG 48-2 UEs for CFRA: - RAR PDSCH timeline relaxation

This means component 13 in FG 48-1 is not supported by FG 48-2 UE for CFRA.

Proposal #1: Component 13 is not supported by FG 48-2 UE for CFRA.

In the last RAN1 meeting, one agreement was made to capture BB bandwidth reduction for Msg4 PDSCH, Msg3 PUSCH and MsgA PUSCH as following:

Agreement - Add following component in FG48-1 - Maximum number of Msg4 PDSCH PRBs that can be [decoded/scheduled] and maximum number of Msg 3 PUSCH PRBs and Msg A PUSCH PRBs (if supported) that can be [transmitted/scheduled] is 25 PRBs for 15 kHz SCS and is 12 PRBs for 30 kHz SCS

The new component can be applied for FG 48-2 UE for CBRA and not applied for FG 48-2 UE for CFRA.

Proposal #2: Following component is not supported by FG 48-2 UE for CFRA.

- Maximum number of Msg4 PDSCH PRBs that can be [decoded/scheduled] and maximum number of Msg 3 PUSCH PRBs and Msg A PUSCH PRBs (if supported) that can be [transmitted/scheduled] is 25 PRBs for 15 kHz SCS and is 12 PRBs for 30 kHz SCS

3 Conclusion

This paper discussed UE features for eRedcap. Based on the discussions, we have the following proposals:

Proposal #1: Component 13 is not supported by FG 48-2 UE for CFRA.

Proposal #2: Following component is not supported by FG 48-2 UE for CFRA.

- Maximum number of Msg4 PDSCH PRBs that can be [decoded/scheduled] and maximum number of Msg 3 PUSCH PRBs and Msg A PUSCH PRBs (if supported) that can be [transmitted/scheduled] is 25 PRBs for 15 kHz SCS and is 12 PRBs for 30 kHz SCS

Reference

Draft Minutes Report for RAN1#113 Meeting

Draft Minutes Report for RAN1#114 Meeting

Draft Minutes Report for RAN1#114bis Meeting