3GPP TSG RAN WG1 #115			R1- 2311911

Chicago, USA, November 13th – 17th, 2023

Agenda Item:	8.3.2

Source: 	LG Electronics

Title: 	Remaining issues on carrier phase positioning in NR

Document for:	Discussion and decision

- Introduction

In RAN1#114 meeting, positioning item is officially finished and related CR is provided [1] – [5] based on the agreements had been made. However, there are some of remaining topics to fully reflect agreements to specification and to remove ambiguity of UE and/or gNB behaviour. In this contribution, we discuss remaining issues for supporting NR DL and UL carrier phase positioning.

- Discussion

In this section, phase continuity to derive RSCPD and forwarding information for UE-based positioning is discussed in each section.

2.1 Phase continuity of the RSCPD

In the latest version of draft TS 38.215, definition of DL RSCP and DL RSCPD is described in section 5.1.42 and 5.1.43, respectively. To this end, there is no requirements at all to ensure phase continuity of RSCPD. It should be noted that the original intention to introduce the RSCPD is to remove the initial Rx phase error by differential of two different PRS measurements, which can only be realized by same initial Rx phase error is guaranteed between them. If it is not guaranteed, i.e., if the phase continuity is not guaranteed between two different measurements to derive RSCPD, the consequence of differential between them is only adding up more ambiguity to them.

Observation 1. If the phase continuity is not maintained for RSCPD, the RSCPD will only become difference between two different RSCPs without ensuring removal of initial Rx phase error.

Regarding ensuring initial Rx phase error elimination for RSCPD measurement, some of the companies thinks that this should be handled by RAN4 therefore it is not needed to be discussed in RAN1. What they imagined was there would be some accuracy requirement for the RSCPD is defined by RAN4, therefore phase continuity is not revealed directly but can be realized by ensuring certain range of error margin to derive RSCPD. However, the ongoing situation is bit different when looking into the RAN4’s progress as described in [6]. The way they are heading for is as follows; absolute measurement accuracy requirements is not going to be defined for the DL RSCP and the ‘relative’ measurement accuracy requirements is going to be defined for DL RSCPD. The reason why the accuracy requirement is defined only for the RSCPD is based on the following understanding. The absolute measurement accuracy, which is the difference between real value and the measured one, cannot be defined due to the lack of understanding on initial Rx phase error. On the other hand, the measurement accuracy for the RSCPD is agreed to be introduced with assumption that there is no remained initial phase error by differencing operation. In our understanding, that is based on the phase continuity between two different RSCPs to derive RSCPD is maintained.

Observation 2. The discussion in RAN4 on accuracy requirements for DL RSCPD measurement assumes no initial Rx phase error is remained for two different measurements to derive RSCPD.

Besides, ‘phase continuity’ is not a new one but had been discussed in Rel-17 CE for DMRS bundling. From that, UE behaviours those disables UE to maintain phase continuity, i.e., events, are revealed by RAN4, and those are notified to RAN1 and are not allowed for the UE to maintain phase continuity of transmission during the certain period of time by specification. In TS 38.214 section 6.1.7, events that cause phase continuity not to be maintained are described and clearly UE shall maintain phase continuity during certain period of time is described as follows:

Events which cause power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or PUCCH transmissions of PUCCH repetition, within the nominal TDW, are: […] The UE shall maintain power consistency and phase continuity within an actual TDW, across PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or across PUCCH transmissions of PUCCH repetition, in case the actual TDW is created in response to frequency hopping, or in response to the use of a different SRS resource set association for the two PUSCH transmissions of PUSCH repetition type A, or PUSCH repetition type B, or in response to the use of different spatial relations or different power control parameters for the two PUCCH transmissions of PUCCH repetition, or in response to any event not triggered by DCI or MAC-CE. The UE maintains power consistency and phase continuity within an actual TDW, across PUSCH transmissions of PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or across PUCCH transmissions of PUCCH repetition, in case the actual TDW is created in response to an event triggered by DCI other than frequency hopping or by MAC-CE, subject to UE capability.

The only difference between phase continuity in Rel-17 CE and that for RSCPD is whether it is phase continuity for transmission or reception. To ensure same initial phase error between two different measured PRS resources to derive RSCPD, it is clear that UE shall maintain phase continuity during that time, and it could be captured in RAN1 spec.

Proposal 1. UE shall maintain phase continuity during two different measured PRS resources to derive RSCPD.

- Capture the phase continuity condition for RSCPD measurement in RAN1 spec.

The next thing should be considered is how long time duration UE shall maintain phase continuity. For the time duration that UE shall maintain phase continuity, two alternatives can be considered; the first one is, introducing another time domain window which divides measurement gap. This one may look simple solution, but then at least two different PRS resources should be configured for that window and UE behaviour when one is missing or one of them cannot be measured. At the same time, it is quite restrictive for network scheduling PRS resources considering the measurement gap is UE-specific configuration, which means the boundary of it may not be aligned across UEs.

Alternatively, indicated time window can be reused for phase continuity. Although the original intention of it is to enable simultaneous measurement between PRU and UE, there is no harm to use it for UE to maintain phase continuity. Considering the duration of it is maximum 16 slots and the number of indicated DL PRS resource set(s) per TRP within a time domain window is maximum 2, the parameters of it seems quite sufficient for UE to measure two different PRS resource within a window to derive RSCPD. And since it is not configured across measurement gap, uplink transmission or uplink slot is not included within a indicated time window, which is one of the necessary conditions for UE to maintain phase continuity. Moreover considering carrier phase measurement of UEs without measurement gap is supported, this approach seems appropriated. In short, reusing indicated time window for the UE to maintain phase continuity is feasible and will minimize introducing additional specification work.

Proposal 2. For the time duration that UE shall maintain phase continuity, indicated time window is reused.

2.2 Forwarding information for UE-based positioning

In the previous RAN1#114bis meeting, there was discussion regarding on forwarding PRU information for UE-based positioning. Unfortunately, due to the different understanding among group on agreement we have, it cannot be concluded. Agreement relevant to it is shown below.

Agreement For UE-based carrier phase positioning, support enabling LMF to forward the DL carrier phase measurement reported by a PRU, with additional information of the same PRU to a target UE for UE-based carrier phase positioning in the positioning assistance data. - Note: Whether the forwarded DL carrier phase measurement is DL RSCP and/or DL RSCPD depends at least on which of them is (are) supported by UE capability. - additional information of the same PRU includes at least PRU location. FFS: additional PRU information, e.g. the AoD of PRU to each TRP, etc.

The most controversial part of interpretation on the agreement was, is the “DL carrier phase measurement reported by a PRU” is RSCP or RSCPD or both of them. It should be noted that if the phase continuity is not maintained to derive RSCPD, UE can derive RSCPD of PRU from the forwarded two RSCPs since there is no restriction at all. That means, UE can obtain or derive both of RSCP and RSCPD even the DL carrier phase measurement in the agreement is interpreted as RSCP only. In this case, forwarding RSCP measurement effectively equivalent to forwarding RSCPD measurement. However, as we discussed above, ensuring phase continuity condition for RSCPD measurements would be necessary for reliable carrier phase positioning accuracy.

On the other hand, if the RSCPD measured by a PRU is obtained within a certain period of time by maintaining phase continuity, forwarding RSCP measurement cannot be equivalent to forwarding RSCPD measurement. In such case, forwarded RSCPD measurement is useful to UE since the initial Rx phase error is eliminated, meanwhile the UE cannot ensure same initial phase error condition between RSCP measurements. From this perspective, it seems reasonable to forward PRU RSCPD measurement to a target UE for UE based positioning.

Proposal 3.  Support for providing DL RSCPD measurements performed by the PRU to the UE

- DL RSCP measurements performed by the PRU is not provided to UE.

- Conclusion

In this contribution, we discussed on remaining topics for carrier phase measurement in NR. From the discussion, we obtained following proposals and observations:

Observation 1. If the phase continuity is not maintained for RSCPD, the RSCPD will only become difference between two different RSCPs without ensuring removal of initial Rx phase error.

Observation 2. The discussion in RAN4 on accuracy requirements for DL RSCPD measurement assumes no initial Rx phase error is remained for two different measurements to derive RSCPD.

Proposal 1. UE shall maintain phase continuity during two different measured PRS resources to derive RSCPD.

- Capture the phase continuity condition for RSCPD measurement in RAN1 spec.

Proposal 2. For the time duration that UE shall maintain phase continuity, indicated time window is reused.

Proposal 3.  Support for providing DL RSCPD measurements performed by the PRU to the UE

- DL RSCP measurements performed by the PRU is not provided to UE.

- Reference

- R1-2308691, "Introduction of Rel-18 Positioning Enhancements for TS38.215", Intel Corporation

- R1-2308702, "Introduction of expanded and improved NR positioning", Samsung

- R1-2308706, "Introduction of expanded and improved NR positioning", Ericsson

- R1-2308713, "Introduction of NR positioning enhancement in Rel-18", Huawei

- R1-2308719, "Introduction of specification support for Expanded and Improved NR Positioning", Nokia

- R4-2317380, “WF on R18 positioning – SL positioning and Carrier phase Positioning”, CATT, RAN4#108bis