3GPP TSG RAN WG1 #115			R1-231nnnn

Chicago, USA, November 13th – November 17th, 2023

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes for 8.16.12

Agenda Item:		8.16.12

Document for: 	Endorsement

### 8.16.12	UE features for MC enhancements

Agreement

For DCI format 1_3 with same SCS between scheduling cell and cells in the set:

- [At least for the case if UE does not support [FG6-10 (legacy R15 x-CC scheduling)] for some bands in the BC where the UE supports multi-cell scheduling by DCI format 1_3 for a set of cells;]

  - If the scheduling cell is in the set but is NOT the reference cell;

    - Same number of DCIs as the case when the scheduling cell is in the set and is the reference cell

  - FFS whether to include this case in FG 49-1 or advanced FG

Agreement

- Add following component in FG 49-1

  - Monitoring SS set(s) for DCI format 1_3 for a set of cells for the following cases

    - 1) Search space set configuration for DCI format 1_3 for the set of cells is provided only on the scheduling cell, or;

    - 2) Search space set configurations for DCI format 1_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being NOT in the set of cells

    - UE supporting FG 49-1 can additionally report whether the UE support following case

      - 3) Search space set configurations for DCI format 1_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells

- Delete FGs 49-1a

- Update the sentence in FG 49-1 as follows:

  - FFS whether this FG is separated for the case when scheduling cell is not included in a set of cells and/or when scheduling cell is not the reference cell for the set, and FFS for the case when same SCS but different carrier types between scheduling cell and set of cells

Agreement

For DCI format 1_3 with same SCS between scheduling cell and cells in the set:

- [For the case if UE support [FG6-10 (legacy R15 x-CC scheduling)] for some bands in the BC where the UE supports multi-cell scheduling by DCI format 1_3 for a set of cells;]

  - One DCI 1_3 for the set of cells and one unicast DL SC-DCI for each of the cells that are not scheduled by DCI 1_3 per slot of scheduling cell

Agreement

For 49-1:

- Component 10 is confirmed as:

  - The number of unicast DL DCIs to process per slot of scheduling cell for a set of cells configured for multi-cell PDSCH scheduling by DCI format 1_3

    - One DCI format 1_3 for the set of cells and,

    - One unicast DL DCI formats 1_0/1_1/1_2 (if supported) for each of the cells that are not scheduled by DCI 1_3

- Add following note in FG 49-1:

  - Support of CCS with DL DCI formats 1_1/1_2 is according to FG 6-10

Agreement

For 49-2:

- Component 9 is confirmed as:

  - The number of unicast UL DCIs to process per slot of scheduling cell for a set of cells configured for multi-cell PUSCH scheduling by DCI format 0_3

    - For FDD scheduling cell

      - [Up to] one DCI format 0_3 for the set of cells and,

      - [Up to] one unicast UL DCI formats 0_0/0_1/0_2 (if supported) for each of the cells

      - For a cell in a set of cells, no more than one DCI scheduling PUSCH for the cell

    - For TDD scheduling cell

      - [Up to] two DCI format 0_3 for the set of cells and,

      - [Up to] two unicast UL DCI formats 0_0/0_1/0_2 (if supported) for each of the cells

      - For a cell in a set of cells, no more than two DCI scheduling PUSCH for the cell

- Add following note in FG 49-2:

  - Support of CCS with UL DCI formats 0_1/0_2 is according to FG 6-10

R1-2310886	Discussion on UE features for MC enhancements	Huawei, HiSilicon

R1-2310900	On UE features for MC enhancements	Nokia, Nokia Shanghai Bell

R1-2311028	Discussion on UE feature for MC enhancements	ZTE

R1-2311125	Discussion on Rel-18 Multi-carrier enhancements UE features	vivo

R1-2311191	Discussion on UE features for multi-carrier enhancement	Spreadtrum Communications

R1-2311279	Discussion on UE features for multi-carrier enhancement	OPPO

R1-2311393	Discussion on UE features for MC enhancements	xiaomi

R1-2311651	Discussion on UE features for Multi-carrier enhancements	NTT DOCOMO, INC.

R1-2311714	Views on UE features for Rel-18 MC enhancements	Apple

R1-2311877	UE features for multi-carrier enhancements	Samsung

R1-2311903	Discussion on UE features for MC enhancements	LG Electronics

R1-2311991	On UE feature discussion for Rel-18 multi-carrier enhancement	MediaTek Inc.

R1-2312070	UE features for MC enhancements	Qualcomm Incorporated

R1-2312104	UE features for Rel-18 multi-carrier enhancements	Ericsson