3GPP TSG RAN WG1 #115			R1-231nnnn

Chicago, USA, November 13th – November 17th, 2023

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes for 8.16.4

Agenda Item:		8.16.4

Document for: 	Endorsement

### 8.16.4	UE features for eRedCap

Agreement

- Component 15 in FG 48-1 is revised as: Maximum number of Msg4 PDSCH PRBs, which is scheduled by DCI scrambled by a TC-RNTI, that can be [decoded/scheduled] and maximum number of Msg 3 PUSCH PRBs and Msg A PUSCH PRBs (if supported), which is scheduled by RAR UL grant or by a DCI scrambled by a TC-RNTI, or is configured for a Type-2 random access procedure, that can be [transmitted/scheduled] is 25 PRBs for 15 kHz SCS and is 12 PRBs for 30 kHz SCS

- Component 15 in FG 48-1 is applicable to FG 48-2

Agreement

- Remove “FFS whether to add additional components” in FG48-1

Agreement

- “Component 13 in FG 48-1 is supported by FG 48-2 during initial access” in FG 48-2 is revised to “Component 13 in FG 48-1 is supported by FG 48-2 for CBRA during initial access”

Agreement

- Component column of FG 48-2 is updated as follows

Component 13 in FG 48-1 is supported by FG 48-2 during initial access. The capabilities of FG 48-2 are the same as for FG 48-1 with following exceptions that the following restriction does not apply: 12. Maximum number of PDSCH/PUSCH PRBs that can be scheduled/configured for unicast is 25 PRBs for 15 kHz SCS and is 12 PRBs for 30 kHz SCS Component 11 in FG 48-1 does not apply for FG 48-2, and instead FG 48-2 has a DL/UL peak data rate target of 10 Mbps corresponding to vLayers·Qm·f = 0.75 when vLayers = 1 and vLayers·Qm·f = 0.8 when vLayers = 2 Component 12 in FG 48-1 does not apply for FG 48-2. Component 13 in FG 48-1 is supported by FG 48-2 only for CBRA.

R1-2310821	R18 RedCap UE features	FUTUREWEI

R1-2310858	UE features for Rel-18 RedCap	Huawei, HiSilicon

R1-2310892	On UE features for eRedCap	Nokia, Nokia Shanghai Bell

R1-2310996	Discussion on UE features for eRedcap	ZTE, Sanechips

R1-2311120	Discussion on Rel-18 eRedCap UE features	vivo

R1-2311186	Discussion on UE features for eRedCap	Spreadtrum Communications

R1-2311266	Rel-18 eRedCap UE features	OPPO

R1-2311331	Discussion on UE features for R18 RedCap	CATT

R1-2311396	Discussion on UE features for eRedCap	xiaomi

R1-2311428	Discussion on the UE features for NR NTN enhancements	NEC

R1-2311556	Discussion on UE features for eRedCap	China Telecom

R1-2311645	Discussion on UE features for eRedCap	NTT DOCOMO, INC.

R1-2311709	UE features for eRedCap	Apple

R1-2311870	UE features for eRedCap	Samsung

R1-2311979	UE features for eRedCap	MediaTek Inc.

R1-2312064	UE features for eRedcap	Qualcomm Incorporated

R1-2312168	UE features for eRedCap	Ericsson