3GPP TSG RAN WG1 #115			R1-231nnnn

Chicago, USA, November 13th – November 17th, 2023

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes for 8.16.6

Agenda Item:		8.16.6

Document for: 	Endorsement

### 8.16.6	UE features for XR enhancements

Agreement

- Reporting type of FGs 50-1, 50-1a, and 50-2 is per band

Agreement

- FG 11-9 is not included as a prerequisite FG of FG 50-1a

- Add a note: When UE supports both FG 11-9 and 50-1a, the total number which can be configured for legacy CG and multi-PUSCH CG should not exceed the value reported by FG 11-9

Agreement

- Candidate values for component 1 in FG 50-1a is at least {1, 2, 4, 8, 12}, FFS other values

Agreement

- Confirm following notes in FG 50-1a

  - For all the reported bands in FR1, a same X1 value is reported for component 2. For all the reported bands in FR2, a same X2 value is reported for component 2

  - The total number of configured/active configured grant configurations across all serving cells in FR1 is no greater than X1.

  - The total number of configured/active configured grant configurations across all serving cells in FR2 is no greater than X2.

  - If there are some serving cell(s) in FR1 and some serving cell(s) in FR2, the total number of configured/active configured grant configurations across all serving cells is no greater than max(X1, X2).

  - Regarding the interpretation of UE capabilities in case of cross-carrier operation, support of FG50-91a is based on the support of this capability for the band of the scheduled/triggered/indicated cell only

Agreement

- No additional values are introduced as candidate values for component 1 in FG 50-1a

Agreement

- Introduce following FG

| 50. NR_XR_Enh | 50-1b | Joint release in a DCI for two or more configured grant Type 2 configurations, including multi-PUSCH CG configuration(s), for a given BWP of a serving cell | M<=4 bits indication in the Release DCI is used for indicating which CG configuration(s) is/are released, where the association between each state indicated by the indication and the CG configuration(s) is - Up to 2^M states are higher layer configurable, where each of the state can be mapped to a single or multiple CG configurations to be released - In case of no higher layer configured state(s), separate release is used where the release corresponds to the CG configuration index indicated by the indication FFS whether/how to capture the relationship with 11-9a | One of {50-1, 50-1a} | Yes | N/A |  | Per band | N/A | N/A | N/A | Regarding the interpretation of UE capabilities in case of cross-carrier operation, support of FG50-1b is based on the support of this capability for the band of the scheduled/triggered/indicated cell only Note: For the case of joint release in a DCI for two or more configured grant Type 2 configurations, including multi-PUSCH CG configuration(s), for a given BWP of a serving cell, the reporting of this FG applies, i.e., ignore FG 11-9a If UE supports 11-9a but does not support this FG, the UE does not expect to be indicated for joint release including multi-PUSCH CG configuration(s) | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

R1-2310847	UE features for XR enhancements	Huawei, HiSilicon

R1-2310894	On UE features for XR enhancements	Nokia, Nokia Shanghai Bell

R1-2310998	Discussion on UE features for XR enhancements	ZTE, Sanechips

R1-2311122	Discussion on Rel-18 XR UE features	vivo

R1-2311187	Discussion on UE features for XR Enhancements	Spreadtrum Communications

R1-2311280	Discussion on UE features for XR	OPPO

R1-2311333	Discussion on Rel-18 UE features for XR in RAN1	CATT

R1-2311398	Discussion on UE features for XR-specific capacity enhancements	xiaomi

R1-2311534	Discussion on UE features for XR	InterDigital, Inc.

R1-2311647	Discussion on UE features for Rel-18 XR enhancements	NTT DOCOMO, INC.

R1-2311711	Views on UE features for XR enhancements	Apple

R1-2311872	UE features for XR	Samsung

R1-2311902	Discussion on UE features for XR enhancements	LG Electronics

R1-2311906	On UE features for XR Enhancements	Ericsson

R1-2311983	Discussion on UE features for XR enhancements	MediaTek Inc.

R1-2312066	UE features for XR enhancements	Qualcomm Incorporated