3GPP TSG RAN WG1 #116		R1-2400596

Athens, Greece, February 26th – March 1st, 2024

Source:	 OPPO

Title:          Remaining issues in multi-carrier enhancement for NR

Agenda Item:   8.8

Document for:  Discussion and Decision

# Introduction

In this document, we provide our views on the remaining issues for both aspects in Rel-18 multi-carrier enhancement:

- Multi-cell PUSCH/PDSCH scheduling with a single DCI.

- Uplink Tx switching.

# Remaining issues in multi-cell scheduling

The following TP is proposed to handle the case where the OLPC parameter set indicator, as a Type-1a field, indicates an invalid value for a co-scheduled cell.

| Reason for change: | For the OLPC parameter set indicator, defined as Type 1a field, the bit-width of the field can be up to maxr∈{1,2,…,NcellUL,2}Mor, while the acceptable value for a specific co-scheduled cell r fits within Mor bits. Then, the current specification misses a case where the DCI field indicates a value requiring more than Mor bits for the co-scheduled cell r that satisfies Mor>0. Such an indicated value that is legit for the Type-1a field but larger than what a co-scheduled cell can accept is an invalid code-point. When an invalid code-point indication occurs, the indication should be ignored, and P0 can be determined by p0-PUSCH-Alpha or p0-PUSCH-Alpha2 but not P0-PUSCH-Set, which is the existing UE behavior under either of the following conditions (ref. section 7.1.1 in TS38.213): - Condition-1: p0-PUSCH-SetList is not configured for the cell. - Condition-2: The corresponding DCI does not include an OLPC parameter set indication field. It is preferable to mimic Condition-2 for the UE behavior in case the invalid code-point occurs. |
| --- | --- |
| Summary of change: | Add a text description in TS38.212 for the case an invalid code-point is indicated in OLPC parameter set indication field for a co-scheduled cell: In such a case, the existing UE behavior is followed as if the DCI does not include OLPC parameter set indication field for the scheduled cell. |
| Consequences if not approved: | UE behavior is undefined in case the OLPC parameter set indication field indicates an invalid code-point for a co-scheduled cell. |
| ### 7.3.1.1.4 Format 0_3 ************** Unchanged parts omitted************** - Open-loop power control parameter set indication - maxr∈{1,2,…,NcellUL,2}Mor bits applying to the scheduled cells with Mor>0 independently, where r is mapped to the cells according to an ascending order of a serving cell index with r=1 corresponding to the cell with the smallest serving cell index, and Mor is defined by the following: - 0 bit if the higher layer parameter p0-PUSCH-SetList is not configured; - 1 or 2 bits otherwise, - 1 bit if SRS resource indicator is present in the DCI format 0_3; - 1 or 2 bits as determined by higher layer parameter olpc-ParameterSetDCI-0-1 if SRS resource indicator is not present in the DCI format 0_3. If a DCI format 0_3 indicates a code point in this field that does not correspond to a configured p0 in p0-PUSCH-SetList of a scheduled cell, the uplink power control defined in Clause 7.1.1 of [5, TS 38.213] for the scheduled cell assumes the DCI format does not include an open-loop power control parameter set indication field. ************** Unchanged parts omitted************** |  |

Another issue relating to invalid code-point of Type-1a field comes from the following RAN1 #115 agreement, which was commented after RAN1 #115 to be implemented in RAN2 specification:

Agreement - When Antenna port(s) field in DCI format 1_3 is configured as type1a, UE expects to be configured with a common table from Tables 7.3.1.2.2-1/2/3/4 in TS38.212 is used for all cells in set of cells. - The DMRS mapping type should be the same across the cells in set of cells - When Antenna port(s) field in DCI format 0_3 is configured as type1a, UE expects to be configured with a common table from Tables 7.3.1.1.2-6, 7.3.1.1.2-6A, 7.3.1.1.2-7, 7.3.1.1.2-7A, 7.3.1.1.2-8, 7.3.1.1.2-9, 7.3.1.1.2-10, 7.3.1.1.2-11, 7.3.1.1.2-12, 7.3.1.1.2-13, 7.3.1.1.2-14, 7.3.1.1.2-15, 7.3.1.1.2-16, 7.3.1.1.2-17, 7.3.1.1.2-18, 7.3.1.1.2-19, 7.3.1.1.2-20, 7.3.1.1.2-21, 7.3.1.1.2-22, 7.3.1.1.2-23, 7.3.1.1.2-24, and 7.3.1.1.2-25 in TS38.212 is used for all cells in set of cells. - The DMRS mapping type should be the same across the cells in set of cells - When TPMI field in DCI format 0_3 is configured as type1a, UE expects to be configured with a common table from Tables 7.3.1.1.2-2, 7.3.1.1.2-2A, 7.3.1.1.2-B, 7.3.1.1.2-3, 7.3.1.1.2-3A, 7.3.1.1.2-4, 7.3.1.1.2-4A, 7.3.1.1.2-5, and 7.3.1.1.2-5A in TS38.212 is used for all cells in set of cells. - When SRI field in DCI format 0_3 is configured as type1a, UE expects to be configured with a common table from Tables 7.3.1.1.2-28, 7.3.1.1.2-29, 7.3.1.1.2-30, 7.3.1.1.2-31, 7.3.1.1.2-32, 7.3.1.1.2-32A, and 7.3.1.1.2-32B in TS38.212 is used for all cells in set of cells.

The intention of the above agreement is to avoid invalid code-points in the indication of these DCI fields (when indicated as Type-1a) for any co-scheduled cell. However, making all co-scheduled cells to use the common table only removes the invalid code-points due to “out of indication value range for a cell”, but is not able to avoid the invalid code-points due to “pointing to a reserved or undefined table entry” for a co-scheduled cell. Such “reserved/undefined table entry” are quite populated among the tables listed in above RAN1 agreement, for example:

- For antenna port in DL DCI: At least Tables 7.3.1.2.2-2/3/4;

- For TPMI: At least Tables 7.3.1.1.2-2/2A/2B/3/3A/4/5;

- For SRI: At least Tables 7.3.1.1.2-28(copied below)/29/30/31.

Table 7.3.1.1.2-28 in TS38.212 for SRI indication

| Bit field mapped to index | SRI(s), NSRS=2 | Bit field mapped to index | SRI(s), NSRS=3 | Bit field mapped to index | SRI(s), NSRS=4 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 1 | 1 |
|  |  | 2 | 2 | 2 | 2 |
|  |  | 3 | reserved | 3 | 3 |

It should be noted that RAN1 should avoid re-defining UE behaviors upon these invalid code-points, because the specification is not supposed to do so on “reserved” table entries. Then, the simple fix to the RAN1 #115 agreement is to add a further restriction to avoid the DCI field indication corresponding to a reserved/undefined table entry for any co-scheduled cell.

Proposal 1: For RAN1 #115 agreement requesting “common table” configurations for DCI fields including antenna port, TPMI and SRI, update the agreement by appending the following sub-bullet:

- UE does not expect the corresponding DCI field to indicate a reserved or undefined table entry for a co-scheduled cell.

# Remaining issues in uplink-Tx switching

This discussion follows up the RAN1 #115 progress on ambiguity between one Tx switching and two Tx switching.

| Reason for change: | RAN1 #115 made the following agreement, with pending FFS on spec impacts: Agreement - For UL Tx switching, if an UL transmission with starting symbol at T0 is scheduled to a UE, the UE is not expected to be scheduled with another UL transmission overlapping with the first UL transmission in time and resulting in interruption on the first UL transmission by a DCI arriving later than T0-Toffset. - FFS: whether any specification impact is needed considering the existing cut-off time specified in S6.1.6 of TS 38.214 - If the two Tx chains are triggered to switch between two different band pairs (e.g., band A + band B->band C+ band D), and when the two UL transmissions on band C and band D are at least partially overlapped in time domain, the Toffset of timeline T0-Toffset for switching is determined based on the switching gap defined for a single Tx switching in [8, TS 38.101-1] - T0 is the starting point of the earlier transmission among the two uplink transmissions (band C + band D). This does not change the definition of T0 in current specifications. - FFS: whether any specification impact is needed For the first bullet above, new spec impact is not needed given the existing specification already says the following (ref. section 6.1.6 in TS38.214): If an uplink switching is triggered for an uplink transmission starting at T0, after T0-Toffset, the UE is not expected to cancel the uplink switching, or to trigger any other new uplink switching occurring before T0 for any other uplink transmission that is scheduled after T0-Toffset, where Toffset is the UE processing procedure time defined for the uplink transmission triggering the switch given in clause 5.3, clause 5.4, clause 6.2.1, clause 6.4 and in clause 9 of [6, TS 38.213]. For the second bullet above, - For duration Toffset, it has been discussed clearly in RAN4. RAN1 spec could safely refer to TS38.101 directly. - For reference point T0, according to current specification, the 2nd DCI should not be later than T0-Toffset, where T0 is the starting point of the 1st PUSCH triggering Tx switching. In addition, to ensure enough processing time, the gap between the 2nd DCI and the 2nd PUSCH should not be smaller than Toffset. So although current specification does not indicate earlier PUSCH transmission explicitly, the DCI triggering Tx switching is in fact no later than T0-Toffset, where T0 is the starting point of the earlier transmission among the two uplink transmissions. Therefore no new spec change is needed here. |
| --- | --- |
| Summary of change: | Only add a reference to TS38.101 in TS38.214 for RAN4 definition of Toffset. |
| Consequences if not approved: | The definition of duration Toffset for Rel-18 UL Tx switching is missing from RAN1 spec perspective. |
| ### 6.1.6 Uplink switching ************** Unchanged parts omitted************** If an uplink switching is triggered for an uplink transmission starting at T0, after T0-Toffset, the UE is not expected to cancel the uplink switching, or to trigger any other new uplink switching occurring before T0 for any other uplink transmission that is scheduled after T0-Toffset, where Toffset is the UE processing procedure time defined for the uplink transmission triggering the switch given in clause 5.3, clause 5.4, clause 6.2.1, clause 6.4 and in, clause 9 of [6, TS 38.213] and clause 6.3A of [8, TS 38.101]. ************** Unchanged parts omitted************** |  |

# Conclusion

Proposal 1: For RAN1 #115 agreement requesting “common table” configurations for DCI fields including antenna port, TPMI and SRI, update the agreement by appending the following sub-bullet:

- UE does not expect the corresponding DCI field to indicate a reserved or undefined table entry for a co-scheduled cell.

Proposal 2: RAN1 adopts the two TPs given in this contribution.

# Reference

[1] R1-2312514, Feature lead summary #5 on multi-cell PUSCH/PDSCH scheduling with a single DCI, Lenovo