3GPP TSG RAN WG1 #116			R1-2401051

Athens, Greece, February 26th – March 1st, 2024

Source:	Panasonic

Title: 	Maintenance of NR Sidelink unlicensed spectrum

Agenda Item:	8.2

Item for:	Discussion

Introduction

This document provides our view on maintenance of sidelink unlicensed spectrum.

Discussion

Power control for PSFCH in 38.213

NPSFCH,one for sl-PSFCH-Type = ‘type2 (alt 1-1b) is described follows [1].

- if Nsch,Tx,PSFCH≤Nmax,PSFCH - if PPSFCH,one+10log10NPSFCH,one≤PCMAX, where PCMAX is determined for Nsch,Tx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1] and - NPSFCH,one=Nsch,Tx,PSFCH for operation without shared spectrum channel access - NTx,PSFCH=Nsch,Tx,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm] - NPSFCH,one=k=1Nsch,Tx,PSFCHNPSFCH,one,kinterlace for operation with shared spectrum channel access and sl-PSFCH-Type = ‘type1’, where NPSFCH,one,kinterlace is the number of PRBs in the interlace for PSFCH transmission k - NTx,PSFCH=Nsch,Tx,PSFCH and PPSFCH,ki=PPSFCH,one+10log10NPSFCH,one,kinterlace [dBm], where the power on one PRB in the interlace for PSFCH transmission is PPSFCH,PRB,k(i)=PPSFCH,one - NPSFCH,one=Nsch,Tx,PSFCH⋅NPSFCH,oneinterlace2+NPSFCH,oneinterlace1⋅10(-PPSFCH,offset/10) for operation with shared spectrum channel access and sl-PSFCH-Type = ‘type2’, where NPSFCH,oneinterlace2 is provided by sl-PSFCH-Type2-DedicatedPRB, PPSFCH,offset is provided by sl-PSFCH-Type2-PowerOffset, and NPSFCH,oneinterlace1 is the number of PRBs in the first interlace for all Nsch,Tx,PSFCH PSFCH transmissions after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0 - NTx,PSFCH=Nsch,Tx,PSFCH and PPSFCH,ki=PPSFCH,one+10log10NPSFCH,oneinterlace2+NPSFCH,oneinterlace1⋅10(-PPSFCH,offset/10) [dBm], where the power on one PRB in the first interlace for PSFCH transmission is PPSFCH,PRB,first,k(i)=PPSFCH,PRB,second,ki-PPSFCH,offset and the power on one PRB in the subset of PRBs in the second interlace for PSFCH transmission is PPSFCH,PRB,second,k(i)=PPSFCH,one, where PPSFCH,offset is provided by sl-PSFCH-Type2-PowerOffset

Editor’s note mentioned two discussion points.

- The text related to the use of power offset for PSFCH type 2 does not reflect common RAN1 understanding of working assumptions/agreements. It is subject to deletion and substitution if it is not confirmed by RAN1 (see also the Tdoc with the summary of email discussions). RAN1 is requested to discuss and conclude on that text in RAN1#116. - In all instances of this statement, RAN1 is to conclude on whether or not dropped PRBs are considered in determining the PSFCH transmission power.

For power offset between P_common and P_dedicated, related RAN1 working assumption and agreement are follows.

Working assumption in RAN1#114bis - In “Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)”: - Assume the UE transmits N PSFCH - Denote the final Tx power on one common PRB is P_common - Denote the final Tx power on one dedicated PRB is P_dedicated - P_common <= P_dedicated - (pre-)configure an offset between P_common and P_dedicated - Send an LS to RAN4 asking whether there is any difficulty for supporting the following cases - P_common < P_dedicated - P_common = P_dedicated Agreement in RAN1#115 For SL-U PHY, following higher layer parameters are endorsed. index; Parameter name in the text; Description; Value range; Default value aspect; Per (UE, cell, TRP, …). 23; PSFCHPower -offset; Indicates the power offset between Tx power on one common PRB (P_common) and Tx power on one dedicated PRB (P_dedicated) when transmissionStructureForPSFCH is (pre-)configured as common interlace, i.e., P_common = P_dedicated - offset. UE expects the same (pre-)configured value of PSFCHPowerOffset across all resource pools. The unit is dB.; INTEGER (0,1,2,…,10) Note: this value range may need to be updated based on RAN4 reply LS; N/A; Per resource Pool.

From the working assumption, the offset value is offset between the final Tx power on one common PRB(P_common) and the final Tx power on one dedicated PRB(P_dedicated). There are two different views on the meaning of "NPSFCH,oneinterlace1 is the number of PRBs in the first interlace for all Nsch,Tx,PSFCH PSFCH transmissions"

- The power on common PRB is accumulated if multiple PSFCH transmissions are performed in an RB set.

- The power on common PRB is shared for multiple PSFCH transmissions in an RB set.

Our view is second one as common PRB is shared by multiple PSFCH transmissions in an RB set. Even if multiple PSFCHs are transmitted in an RB set, NPSFCH,oneinterlace1 is not increased. The final Tx power on one common PRB (P_common) is always smaller than the final Tx power on one dedicated PRB(P_dedicated). The current specification has no issue and not necessary to be modified.

Whether or not dropped PRBs are considered in determining the PSFCH transmission power was discussed. This dropped PRBs are common PRB for sl-PSFCH-Type = ‘type2 (alt 1-1b) in section 16.3.0 as follows.

For operation with shared spectrum channel access, when sl-PSFCH-Type = 'type2', a PRB s1 in the first interlace is excluded from the resources for a PSFCH transmission, if ¦s1-s2¦≤5 for μ=0 or ¦s1-s2¦≤2 for μ=1 for any PRB s2 in the PRB subset when the PRB subset is selected for PSFCH transmission, and shigh-slow≥88 for μ=0 or shigh-slow≥44 for μ=1, where PRB shigh and PRB slow are the largest and smallest PRB indexes, respectively, in the resources for the PSFCH transmission assuming PRB s1 is excluded.

This dropping of common PRBs can be decided before to calculate transmission power. If this dropping is not achieved before to calculate transmission power, in some case, PPSFCH,one+10log10NPSFCH,one≤PCMAX, is not satisfied and one more PSFCH is need to be dropped unnecessary. Therefore, we propose to keep the "after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0".

Proposal 1: For power control of PSFCH in 38.213, NPSFCH,oneinterlace1 is the number of PRBs in the first interlace is shared by multiple PSFCH transmissions in an RB set. The specification change is not necessary.

Proposal 2: For power control of PSFCH in 38.213, "after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0" is necessary. The specification change is not necessary.

TBS in 38.214

In 38.212[2], following was captured for SCI format 2-A. Depending on the "COT sharing flag", the size of SCI format 2-A is changed.

If the 'COT sharing flag' field in SCI format 1-A is present and set to '1', all the remaining fields are set as follows: - CAPC – 2 bits. Values '00', '01', '10' and '11' correspond to CAPC values '1', '2', '3' and '4' as defined in Table 4.5-1 of [14, TS 37.213], respectively. - COT sharing cast type – 2 bits as defined in Table 8.4.1.1-1. - COT sharing additional ID – 24 bits. The 16 LSBs provide layer 1 destination ID and the 8 MSBs provide layer 1 source ID, as defined in [6, TS 38.214]. The 8 MSBs are reserved when the COT sharing cast type field is set to '00' or '01'. - Remaining COT duration – log2(10∙2μ) bits as defined in clause 4.5.3 of [14, TS 37.213], where μ is defined in Table 4.2-1 of Clause 4.2 of [4, TS 38.211]. Table 8.4.1.1-1: Cast type indicator or COT sharing cast type Value of Cast type indicator or COT sharing cast type; Cast type. 00; Broadcast. 01; Groupcast when HARQ-ACK information includes ACK or NACK. 10; Unicast. 11; Groupcast when HARQ-ACK information includes only NACK; or reserved, if higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured.

In transport block size determination in 38,214[3] section 8.1.3.2, NRESCI,2 is used.

- A UE determines the total number of REs allocated for PSSCH ()NRE) by NRE=NRE'∙nPRB-NRESCI,1-NRESCI,2NRE= NRE'* nPRB, where - nPRB is the total number of allocated PRBs for the PSSCH. If the higher layer parameter transmissionStructureForPSCCHandPSSCH is set to 'interlaceRB', a reference number of PRBs (nref) per interlace within 1 RB set, numRefPRBOfInterlace, is provided by higher layers for determination of total number of PRBs for PSSCH, that is nPRB = nref * ninter,subCH * nsubCH * nRB-set, where ninter,subCH is given by the higher layer parameter numInterlacePerSubchannel, nsubCH is the number of occupied sub-channels within one RB set for the PSSCH, and nRB-set is the number of occupied RB sets for the PSSCH. If the higher layer parameter transmissionStructureForPSCCHandPSSCH is set to ‘contiguousRB’, nPRB = nsubCHsize * nsubCH , where nsubCHsize is provided by higher layer parameter sl-SubchannelSize, and nsubCH is the number of occupied sub-channels for the PSSCH. - NRESCI,1 is the total number of REs occupied by the PSCCH and PSCCH DM-RS. - NRESCI,2 is the number of coded modulation symbols generated for 2nd-stage SCI transmission (prior to duplication for the 2nd layer, if present) according to Clause 8.4.4 of [5, TS 38.212], with the assumption of γ=0.

TBS should be same for initial transmission and retransmission. Therefore, for TBS determination, UE needs to assume same value for NRESCI,2 regardless of 'COT sharing flag'. The actual size of SCI format 2-A can be different. There are two options as follows.

Option 1: NRESCI,2 is assumed the size with the 'COT sharing flag'=0.

Option 2: NRESCI,2 is (pre-)configured.

We think option 1 is simpler than option 2. For TBS determination, there is another parameter NohPRB. NohPRBis the overhead given by higher layer parameter sl-X-Overhead. NohPRB can take into account the additional overhead value. Additional pre-configured value is not necessary.

Proposal 3: For TBS determination NRESCI,2 is assumed the size with the 'COT sharing flag’=0.

| Reason for change: | TBS should be same for initial transmission and retransmission. For TBS determination, UE needs to assume same value for NRESCI,2 regardless of 'COT sharing flag' |
| --- | --- |
| Summary of change: | For TBS determination NRESCI,2 is assumed the size with the 'COT sharing flag'=0. |
| Consequences if not approved: | For initial transmission and retransmission, TBS is different size depending the 'COT sharing flag'. |

Adopt the following TP for TS 38.214.

---------------------------- Start of Text Proposal for TS 38.214-----------------------------

- 8.1.3.2	Transport block size determination

< Unchanged parts are omitted >

-	A UE determines the total number of REs allocated for PSSCH ()NRE) by NRE=NRE'∙nPRB-NRESCI,1-NRESCI,2NRE= NRE'* nPRB, where

-	nPRB is the total number of allocated PRBs for the PSSCH. If the higher layer parameter transmissionStructureForPSCCHandPSSCH is set to 'interlaceRB', a reference number of PRBs (nref) per interlace within 1 RB set, numRefPRBOfInterlace, is provided by higher layers for determination of total number of PRBs for PSSCH, that is nPRB = nref * ninter,subCH * nsubCH * nRB-set, where ninter,subCH is given by the higher layer parameter numInterlacePerSubchannel, nsubCH is the number of occupied sub-channels within one RB set  for the PSSCH, and  nRB-set is the number of occupied RB sets  for the PSSCH. If the higher layer parameter transmissionStructureForPSCCHandPSSCH is set to ‘contiguousRB’, nPRB = nsubCHsize * nsubCH , where nsubCHsize is provided by higher layer parameter sl-SubchannelSize, and nsubCH is the number of occupied sub-channels for the PSSCH.

-	NRESCI,1 is the total number of REs occupied by the PSCCH and PSCCH DM-RS.

-	NRESCI,2 is the number of coded modulation symbols generated for 2nd-stage SCI transmission (prior to duplication for the 2nd layer, if present) according to Clause 8.4.4 of [5, TS 38.212], with the assumption of γ=0 and 'COT sharing flag'=0 if the 'COT sharing flag' field in SCI format 1-A is present .

< Unchanged parts are omitted >

--------------------------------------- End of Text Proposal ----------------------------------

Chanel access in 37.213

- Resume SL transmission(s)

For resume SL transmission in section 4.5.2[4], the condition of CAPC is not described in current specification as follows.

When a UE initiates a channel occupancy on a channel to transmit SL transmission(s) within the channel occupancy, if the UE stops transmitting on the channel, the UE can resume SL transmission(s) within the channel occupancy on the channel after performing Type 2A SL channel access procedures as described in clause 4.5.2.1 if the UE continuously senses the channel to be idle before resuming transmission.

Whether to add "and the CAPC value corresponding the SL transmission(s) is at most equal to the CAPC value used to access the channel" on the end of sentence was discussed.  On the other hand, in section 4.5.3[4], followings are described for the case that COT is initiated by another UE.

For the case when a UE transmits SL transmission(s) in a shared channel occupancy initiated by another UE, the channel access priority class value corresponding to the SL transmission(s) is at most equal to the channel access priority class value provided by the channel access priority class in the channel occupancy sharing information.

We think it is not necessary to be restricted to COT is initiated by another UE for SL transmission(s) in a shared channel occupancy. This sentence can include behaviour for COT initiating UE resume the own COT.

Proposal 4: The channel access priority class value for SL transmission(s) in a shared channel occupancy is specified for both COT initiating UE and another UE.

| Reason for change: | For resuming SL transmission, the CAPC value corresponding the SL transmission(s) is at most equal to the CAPC value used to access the channel should be described. |
| --- | --- |
| Summary of change: | The channel access priority class value for SL transmission(s) in a shared channel occupancy is specified for both COT initiating UE and another UE. |
| Consequences if not approved: | When COT initiating UE resume SL transmission, the allowed CAPC value is unclear. |

Adopt the following TP for TS 37.213.

---------------------------- Start of Text Proposal for TS 37.213-----------------------------

- 4.5.3	SL channel access procedures in a shared channel occupancy

When a UE initiates a channel occupancy using the channel access procedures described in clause 4.5.1 or clause 4.5.6.3 on a channel(s) to transmit SL transmission(s) including PSCCH/PSSCH(s), the UE can provide a channel occupancy sharing information in SL control information that includes at least the Layer 1 source and destination IDs, the corresponding channel access priority class, the remaining channel occupancy duration, and the frequency domain information for the applicable RB set(s) of the channel occupancy. The channel occupancy sharing information can also include additional IDs and associated cast type. The additional IDs includes one pair of Layer 1 source and destination IDs for all cast types, where the source ID is set to the source ID of the UE initiating channel occupancy for unicast and to the reserved bits for groupcast and broadcast. The channel occupancy sharing information transmitted in slot n indicates the remaining channel occupancy duration in a number of slot(s) K. If  K=0, the initiated channel occupancy by the UE shall not be shared for SL transmission(s) by other UE(s). Otherwise, the initiated channel occupancy by the UE can be shared for SL transmission(s) by other UE(s) within a duration starting from the end of slot n and ending at slot n+K.

For the case when a UE transmits SL transmission(s) in a shared channel occupancy, the channel access priority class value corresponding to the SL transmission(s) is at most equal to the channel access priority class value provided by the channel access priority class in the channel occupancy sharing information.

< Unchanged parts are omitted >

--------------------------------------- End of Text Proposal ----------------------------------

Conclusion

This document provided our view on maintenance on NR Sidelink unlicensed spectrum. Based on the discussions, we have following proposals.

Proposal 1: For power control of PSFCH in 38.213, NPSFCH,oneinterlace1 is the number of PRBs in the first interlace is shared by multiple PSFCH transmissions in an RB set. The specification change is not necessary.

Proposal 2: For power control of PSFCH in 38.213, "after excluding PRBs for PSFCH transmissions as described in Clause 16.3.0" is necessary. The specification change is not necessary.

Proposal 3: For TBS determination NRESCI,2 is assumed the size with the 'COT sharing flag'=0.

Proposal 4: The channel access priority class value for SL transmission(s) in a shared channel occupancy is specified for both COT initiating UE and another UE.

Reference

[1] 3GPP TS 38.213 V18.1.0, "Physical layer procedures for control"

[2] 3GPP TS 38.212 V18.1.0, "Multiplexing and channel coding"

[3] 3GPP TS 38.214 V18.1.0, "Physical layer procedures for data"

[4] 3GPP TS 37.213 V18.1.0, "Physical layer procedures for shared spectrum channel access"