3GPP TSG RAN WG1 #116			R1-2301713

Athens, Greece, February 26th – March 1st, 2024

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes for 8.12.4

Agenda Item:		8.12.4

Document for: 	Endorsement

### 8.12.4	UE features for MC enhancements

R1-2401512	Summary of discussion on UE features for MC enhancements	Moderator (NTT DOCOMO, INC.)

Agreement

- For FG49-2/2b, remove square brackets of “[Up to]” in component 9.

Agreement

- Remove following from FG49-1/49-2

  - [Note: When scheduling cell is outside the set of cells, UE is not expected to be configured with another cell to monitor PDCCH candidates for the scheduling cell]

Agreement

- For 49-3x,

  - add the following component

    - Processing up to X unicast DCI scheduling PDSCH per scheduled cell in a set of cells configured for multi-cell PDSCH scheduling by DCI format 1_3:

      - X is based on pair of (scheduling CC SCS, scheduled CC SCS):

        - Candidate value(s) of X

          - X={2,4} for (15,120), (15,60), (30,120) and X={2} for (15,30), (30,60), (60,120 kHz)

        - X applies per slot of scheduling CC

  - prerequisite FG is FG49-1b

  - reporting granularity is per BC

Agreement

- For 49-3y,

  - add the following component

    - Processing up to X unicast DCI scheduling PUSCH per scheduled CC in a set of cells configured for multi-cell PUSCH scheduling by DCI format 0_3

      - X is based on pair of (scheduling CC SCS, scheduled CC SCS):

        - Candidate value(s) of X

          - X={2,4} for (15,120), (15,60), (30,120) and X={2} for (15,30), (30,60), (60,120 kHz)

        - X applies per slot of scheduling CC

  - prerequisite FG is FG49-2b

  - reporting granularity is per BC

Agreement

- For FG49-5a,

  - Confirm component description i.e., remove yellow highlight, with following updates

    - Component 1 is updated to “Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_3 scheduling at least a PDSCH”

  - Confirm note description i.e., remove yellow highlight

Agreement

- For FG49-5b,

  - Confirm component description i.e., remove yellow highlight, with following update

    - Component 4 is updated to “4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in [11-4])”

  - Confirm note description i.e., remove yellow highlight, with following update

    - Add “For component 2 and 5, same values as for FG25-6 are reported (if the UE also report FG25-6)”

R1-2400044	Remaining issues on UE features for multi-carrier enhancement	Spreadtrum Communications

R1-2400084	Remaining issues of MC-enhancements UE features	Nokia, Nokia Shanghai Bell

R1-2400137	Discussion on UE features for MC enhancements	Huawei, HiSilicon

R1-2400229	Discussion on Rel-18 Multi-carrier enhancements UE features	vivo

R1-2400298	Discussion on UE feature for MC enhancements	ZTE

R1-2400451	Discussion on UE features for MC enhancements	CATT

R1-2400537	Discussion on UE features for MC enhancements	xiaomi

R1-2400597	Discussion on UE features for multi-carrier enhancement	OPPO

R1-2400718	UE features for multi-carrier enhancements	Samsung

R1-2400998	On remaining issues for UE features for Rel-18 MC enhancements	Apple

R1-2401104	Discussion on UE features for multi-carrier enhancements	NTT DOCOMO, INC.

R1-2401141	UE features for Rel-18 multi-carrier enhancements	Ericsson

R1-2401290	On UE feature discussion for Rel-18 multi-carrier enhancement	MediaTek Inc.

R1-2401324	Discussion on UE features for MC enhancements	LG Electronics

R1-2401427	UE features for MC enhancements	Qualcomm Incorporated