3GPP TSG RAN WG1 #116			R1-2301714

Athens, Greece, February 26th – March 1st, 2024

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes for 8.12.5

Agenda Item:		8.12.5

Document for: 	Endorsement

- 8.12.5	UE features for other Rel-18 work items (Topics A)

Including UE features for eRedCap, XR, CovEnh, dedicated spectrum less than 5MHz, eDSS, and endorsed TEI proposals.

CovEnh

R1-2401513	Summary of discussion on UE features for coverage enhancements	Moderator (NTT DOCOMO, INC.)

Agreement:

- Introduce FG 54-1a for PRACH repetitions with less than N symbols gap.

  - Component: Support of transmitting two PRACH repetitions when a gap between the last symbol of a PRACH repetition in the first slot and the first symbol of a PRACH repetition in the second slot is less than N symbols, where N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, N=16 for μ=5, N=32 for μ=6, and μ is the SCS configuration for the UL BWP with the PRACH

  - Prerequisite FG: FG 54-1

  - Need for the gNB to know if the feature is supported: YES

  - Type: per band

Agreement:

- Remove the whole [/0_3] from the component in FG 54-3 and 54-3b

Agreement:

- The reporting type of FG 54-3a is per band

- Add N/A to the remaining FFS columns

Agreement:

- Add the following in “consequence if not supported” for FG 54-3b

  - “Dynamic waveform switching for intra-band UL CA is not supported for the band”

Agreement:

- Candidate values for X in FG54-3b are {2,3,4,5,6,7,8}

R1-2401642	Summary#2 of discussion on UE features for coverage enhancements	Moderator (NTT DOCOMO, INC.)

Agreement:

- Add following note for FG54-3a

  - “Note: A UE can be configured to use either the single entry PHR with assumed PUSCH MAC CE or the multiple entry PHR with assumed PUSCH MAC CE for a cell group if the UE indicates support for FG54-3a in any one cell of the cell group”

- Add the following in “consequence if not supported” for FG 54-3a

  - “UE does not report PCMAX_FC for the assumed PUSCH for the band”

Agreement:

- Update the component description for FG54-3 as below

  - Support of dynamic waveform switching for DCI format 0_1/0_2 in a band when configured with only 1 UL carrier in the band.

Agreement:

- Remove the following note for FG54-3

  - “If UE supporting this FG supports FG 10-17, the UE supports FG 54-3 with multi-PUSCH scheduling”

eRedCap

R1-2401514	Summary of discussion on UE features for eRedCap	Moderator (NTT DOCOMO, INC.)

Agreement:

Modify last component for FG 48-2 as following:

- “Component 13 and Component 15 in FG 48-1 is supported by FG 48-2 only for CBRA.”

R1-2401643	Summary#2 of discussion on UE features for eRedCap	Moderator (NTT DOCOMO, INC.)

XR

R1-2401515	Summary of discussion on UE features for XR enhancements	Moderator (NTT DOCOMO, INC.)

Agreement:

- Delete “FFS whether/how to capture the relationship with 11-9a” from component column in FG 50-1b

- Confirm the note column in FG 50-1b with updating following note

  - “Note: For the case of joint release in a DCI for two or more configured grant Type 2 configurations, including multi-PUSCH CG configuration(s), for a given BWP of a serving cell, the reporting of this FG applies, i.e., irrespective of FG 11-9a”

R1-2401644	Summary#2 of discussion on UE features for XR enhancements	Moderator (NTT DOCOMO, INC.)

Agreement:

- Add following note for FG50-1a

  - Note: separate release of different multi-PUSCHs configuration grant Type 2 configuration, i.e., one DCI release one multi-PUSCHs configured grant Type 2 configuration is supported with this FG

Agreement:

- Introduce following new FGs

  - 50-1c: Multi-PUSCHs Type 2 configured grant release by DCI format 0_1

    - Component: Support of Multi-PUSCHs type 2 configured grant release by DCI format 0_1

    - All others are copied from 11-10

  - 50-1d: Multi-PUSCHs Type 2 configured grant release by DCI format 0_2

    - Component: Support of Multi-PUSCHs type 2 configured grant release by DCI format 0_2

    - All others are copied from 11-11

dedicated spectrum less than 5MHz

R1-2401516	Summary of discussion on UE features for dedicated spectrum less than 5MHz	Moderator (NTT DOCOMO, INC.)

R1-2401645	Summary#2 of discussion on UE features for dedicated spectrum less than 5MHz	Moderator (NTT DOCOMO, INC.)

Agreement:

- Add following notes in FG 51-1/2/3

  - This FG is only applicable to single-carrier operation.

  - This FG is not applicable to UEs indicating supportOfRedCap-r17 (i.e., FG 28-1) or supportOfERedCap-r18 (i.e., FG 48-1).

TEIs

R1-2401517	Summary of discussion on UE features for TEIs	Moderator (NTT DOCOMO, INC.)

Agreement:

- Revise “PUCCH resource” to “PUCCH time domain resource” in FGs 55-4a/b/c/d

- Confirm the column of “Consequence if the feature is not supported by the UE” in FGs 55-4d/e with the above revision

Agreement:

- Reporting type of FGs 55-4a/4b/4c/4d/4e is per band and per BC

R1-2401646	Summary#2 of discussion on UE features for TEIs	Moderator (NTT DOCOMO, INC.)

Agreement:

- Change reporting granularity from per FS to per BC for FG55-6a/c/d/e/f/g

- Remove following notes for FG55-6a/c/d/e/f/g

  - A UE shall indicate the same value for the same position in all FeatureSetsPerBands in the indicated FeatureSetCombination

Agreement:

- Add followings in note for FG55-6h

  - Component 3: {4, 8, 16, 32, 44, 64, no limit}

  - Component 4: {4, 8, 16, 32, 44, 64, 128, 256, 512, no limit}

R1-2400085	Remaining issues of UE Features for Other Topics A (eRedCap, XR, CovEnh, sub-5MHz, eDSS, TEI)	Nokia, Nokia Shanghai Bell

R1-2400148	UE features for other Rel-18 WIs (Topics A)	Huawei, HiSilicon

R1-2400230	Discussion on Rel-18 UE features for eRedCap, XR and CovEnh	vivo

R1-2400299	Discussion on UE feature topics A	ZTE

R1-2400315	Discussion on UE feature for coverage enhancement	CMCC

R1-2400416	UE features for other Rel-18 work items (Topics A)	CATT

R1-2400598	Discussion on UE features under Topic A	OPPO

R1-2400676	Rel-18 UE features topics set A	Ericsson

R1-2400716	UE features for other Rel-18 work items (Topics A)	Samsung

R1-2400825	Remaining issues on UE features for coverage enhancement	Spreadtrum Communications

R1-2400974	Discussion on UE features for eRedCap	xiaomi

R1-2400999	On UE features for Rel-18 work items in Topic A	Apple

R1-2401035	Discussion on UE features for further NR coverage enhancement in Rel.18	Panasonic

R1-2401105	Discussion on UE features for Topic A	NTT DOCOMO, INC.

R1-2401169	UE features for Further NR Coverage Enhancements	Sharp

R1-2401305	UE features for other Rel-18 work items (Topics A)	MediaTek Inc.

R1-2401428	UE features for other Rel-18 work items (Topics A)	Qualcomm Incorporated