3GPP TSG-RAN WG1 Meeting #116 	R1-240XXX

# Athens, Greece, February 26th – March 1st, 2023

Title:	LS on improved GNSS operations in Rel-18 IoT NTN

Response to: 	N/A

Release:	Rel-18

Work Item:	IoT_NTN_enh-Core

Source:	RAN WG1

To:	RAN WG2

CC:

Contact Person:

- Name:				Gilles Charbit

- E-mail Address:	gilles.charbit@mediatek.com

Send any reply LS to:	3GPP Liaisons Coordinator:  mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall description

RAN1 has discussed the end of duration X when timeAlignmentTimer is not infinity for improved GNSS operations and made the conclusion on the following:

Agreement Send an LS to RAN2 with the following: From RAN1 perspective, when timeAlignmentTimer is not infinity, the following alternatives were considered, and it is up to RAN2 to specify: - Alt-1: the end of X should be at the point where new timer ULTransmissionExtentionTimer expires and ULTransmissionExtentionTimer is reset with length equal to remaining timeAlignmentTimer every time when a MAC CE (to be defined by RAN2) is received - Alt-1a: the end of X should be at the point where new timer ULTransmissionExtentionTimer expires and ULTransmissionExtentionTimer is set to remaining timeAlignmentTimer at the start point of X and ULTransmissionExtentionTimer is reset with length equal to configured timeAlignmentTimer value every time when a MAC CE (to be defined by RAN2) is received - Alt-2: the end of X should be at the point where timeAlignmentTimer expires and timeAlignmentTimer is reset every time when a legacy MAC TAC is received Note 1: It is up to RAN2 to decide whether the MAC CE is the legacy TAC or a new TAC or a new MAC CE. Note 2: It is up to RAN2 to implement the above behaviour based on new timer, existing timer, or by extending GNSS validity. Note 3: For Alt-1a, from RAN1 perspective, eNB should be able to update TA without extending X

2. Actions:

RAN1 respectfully asks RAN2 to take the above RAN1 agreement into account.

3. Dates of next TSG-RAN WG1 meetings:

TSG-RAN WG1 Meeting #116-bis 	May 15 – April 19, 2024			Changsha, China

TSG-RAN WG1 Meeting #117 	May 20 – May 24, 2024			Fukuoka, Japan