3GPP TSG-RAN WG1 Meeting #116-bis	R1-2402078

Changsha, China, April 15th – April 19th, 2024

Agenda Item:	9.11.1

Source:	Ericsson

Title:	On NR-NTN downlink coverage enhancement

Document for:	Discussion

# Introduction

The first objective of the WID entitled “Non-Terrestrial Networks (NTN) for NR Phase 3” describes the scopes of downlink coverage enhancements [1]. RAN1 made multiple agreements related to this objective during the RAN1#116 meeting [2], including additional reference satellite parameters for LEO 600 km. In this contribution, we further provide our views on evaluation scenarios, assumptions and methodology, and some initial results based on the assumption made so far.

# Discussion

## 2.1 System-level evaluation scenarios, methodology and results

RAN1 defined three scenarios for Set1 in FR1 for the DL coverage study, which referred to as Set1-1 FR1, Set1-2 FR1 and Set1-3 FR1 [2] and the additional reference satellite parameters defined under each set are summarized in the following Tables.

Table 1. Additional reference satellite parameters for LEO 600 km Set1-1 FR1.

| LEO600km Set1-1 FR1 (i.e., S-band) |
| --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size (Note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 34 |
| Payload Total DL power level (dBW) | 31.24 |
| Aggregated EIRP (Total) (dBW) | 61.24* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 41 |
| Total number of beam footprints*** | 1058 |
| Total number of simultaneously active beams ** | 106 |
| % simultaneously active beams** | 10.02 % |
| *Note: EIRP limit is 61.24 dBm for the reference configuration. **Assuming 100 % Resource Block utilization within the same beam at max power. Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) *** For a constellation design at 600km with low elevation angle with 30° and selected (i.e Set 1 parameters) beam size Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies. |  |

Table 2. Additional reference satellite parameters for LEO 600 km Set1-2 FR1.

| LEO600km Set1-2 FR1 (i.e., S-band) |
| --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size (note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 34 |
| Payload Total DL power level (dBW) | 23 |
| Aggregated EIRP (Total) (dBW) | 53* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 41 |
| Total number of beam footprints | 1058 |
| Total number of simultaneously active beams** | 16 |
| % simultaneously active beams** | 1.5 % |
| *Note: EIRP limit is 53 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 16 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies |  |

Table 3. Additional reference satellite parameters for LEO 600 km Set1-3 FR1.

| LEO600km Set 1-3 FR1 (i.e., S-band) |
| --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size (note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 26 |
| Payload Total DL power level (dBW) | 23.24 |
| Aggregated EIRP (Total) (dBW) | 53.24* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 33 |
| Total number of beam footprints | 1058 |
| Total number of simultaneously active beams** | 106 |
| % simultaneously active beams** | 10.02 % |
| *Note: EIRP limit is 53.24 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies |  |

### Beam layout

Agreement For NR NTN Rel-19 DL coverage evaluation, Beam layout defined in Table 6.1.1.1-4 in TR 38.821 can be reused. - Using other beam layouts is not precluded, and should be reported by companies

Additionally, RAN1 also made an agreement to reuse the beam layout defined in Table 6.1.1.1-4 in TR 38.821 while the using of other beam layouts is not precluded and reported by companies for the Rel-19 DL coverage evaluations. Specifically, hexagonal mapping of the beam bore sight directions on UV plane are defined and based on the beam layout definition in UV plane and the satellite location, one can obtain the beam layout plot on earth by applying the right transfer functions (cf. Annex A). The straight line being orthogonal to UV plane is pointing towards the Earth centre and UV coordinates of the nadir of the reference satellite is (0,0). The 3-dB contour radius in UV plane rUV=sinθ3dB with θ3dB=HPBW/2. The baseline inter-beam distance or adjacent beam spacing (ABS) on UV plane is computed based on the 3-dB beam width of the satellite antenna pattern as ABS= dUV=3×rUV= 3×sin(0.5 × HPBWrad).  The hexagonal cell area as function of rUV is  332rUV2, which can be expressed in terms of ABS as 32 ABS2.

At this point we will make some comments with respect to the above-mentioned baseline beam layout based on the UV plane, which is then projecting onto the earth. While the distance between adjacent beams is constant in the UV plane, this is not true in the X-Y-Z coordinate system. In the 3D cartesian system, beams will keep increasing their spacing when moving outwards from the nadir point, which leads to the following issues:

- There is a limit on the number of hexagonal tiers that can be deployed based on the UV plane. When increasing the number of spot beam tiers, and depending on the configured ABS, the UV coordinates of the deployed cells will be assigned values beyond the circle that represents the set of directions that intersects the Earth. For instance, with ABS = 0.0668  (i.e., beam size of 50 km), satellite footprint within 30° elevation angle corresponds to 1058 cells. When the cell centres in the UV plane are projected onto the earth, only 627 out of 1058 cell centres are intersecting with the Earth’s surface, which is illustrated in Figure 1. However, only 627 cells will be intersecting with the Earth’s surface. This means that the complete coverage area of the satellite footprint may not be deployed in the simulations.  Additionally, though equal beam spacing is considered in the UV plane, the cell spacing projected onto the Earth’s surface becomes larger as beyond nadir point. Hence, inter beam interference may not accurately reproduced as in the case of equidistance cell placement.

The adopted UV plane is not a fitting methodology to allow an accurate inter-beam interference calculation, nor a placement of high number of spot beam tier layouts.

Figure 1. Visualizations of the cell center placement on the Earth’s surface illustrating the issues mentioned in Observation 1.

Next, we discuss the issues by coupling the beam spacing to the configured beam width in the definition of ABS, i.e.,  3×sin(0.5 × HPBWrad):

- The number of cells within the coverage area of the satellite is pre-determined. This is because there is no freedom of increasing or decreasing the ABS without modifying the HPBW. Since HPBW is a fixed configuration parameter (e.g., see Table 1), so is ABS. This is problematic because e.g., a configuration of a small beam width will directly imply a large number of cells within the coverage area of the satellite.

- The interference level of two adjacent cells at the cell edge is pre-determined. This means that any system optimization does not have the freedom to optimize the SNR/SIR relationship by modifying the spacing between spot beams as this is a fixed parameter coupled to the HPBW.

The adopted ABS definition couples the satellite antenna beam width, which is a fixed simulation configuration, with the ABS. As an effect, the system cannot be optimized in terms of number of beam footprints within the satellite coverage area and SNR/SIR balancing.

Alternatively, one can also increase ABS by keeping the same HPBW, which is the approach considered in our evaluations. Note in this case, the antenna gain from both intended and unintended satellite beams at the cell-edge is decreased.

If one increases the ABS by n  then the hexagonal cell area increases by n. Therefore, by increasing the ABS, one can reduce the number of cells falling inside the region of interest and hence reduces the number of satellite beam footprints to cover the same area.

If one increases the ABS by n, then the hexagonal cell area increases by n in the UV plane. Then, the number of satellite beam footprints required to provide the coverage to the specific region of interest can be decreased by increasing the ABS in the UV plane.

The ABS can be increased either by increasing the HPBW or directly increasing the factor of rUV. We consider the latter option in the following, specifically,

ABS'= 3n×sin0.5 × HPBW rad, 	(Eq. 1)

which corresponds to the increased contour radius in the UV plane.

For Set-1 LEO-600, 3dB beamwidth (HPBW) of 4.4127 deg gives rUV=0.0386 and dUV=ABS=0.0668 in the UV plane. The corresponding central hexagonal cell area is 0.0039 in the UV plane. This translates to the hexagonal cell radius of 25 km (equivalently, satellite beam diameter of 50 km) on the earth.

For instance, if we set n=2  and n=3 in (Eq. 1), then ABS'=0.0945 and ABS'=0.1157, respectively, which translate to the hexagonal cell radius of 35.35 km and 43.3 km, respectively, (equivalently, satellite beam diameter of 70.7 km and 86.6 km) on the earth. Due to this increased ABS', the number of hexagonal cells falling inside the specific region of interest is reduced by 2 and 3, respectively, compared to the baseline ABS. It is worth to mention that in this case the antenna gain from both intended and unintended satellite beams at the cell-edge is decreased and the corresponding impact on the coverage is studied by means of the CDFs of geometry SNR, SIR, and SINR.

RAN1 to consider beam layout with at least one increased value of ABS (e.g., 0.1157, equivalently, beam size of 86.6 km) in addition to the baseline ABS of 0.0668 (equivalently beam size of 50 km) with the same HPBW.

### Geometry SNRs, SIRs and SINRs

In this section, we consider the baseline schemes such as frequency reuse factor (FRF) = 1 and FRF = 3, which are widely studied throughout the previous releases of NR-NTN. Additionally, the “beam hopping technique” resembles a scheme that turns ON-and-OFF beams strategically within the satellite footprint, i.e., at a specific time, beams in one beam hopping pattern are active whereas the beams associated with other beam hopping patterns are inactive. There can be different “beam hopping” variants/strategies:

- In one variant of this technique, time is equally allocated for each beam hopping pattern agnostic e.g., to the traffic demands, and assumes that the transmission of common channels and signals associated with mandatory system information is respected.

- In another variant of this technique, time is nonuniformly allocated across beam hopping patterns, subject e.g., to the traffic demands. A beam hopping pattern corresponding to the larger traffic demands gets more active time and again subject to the transmission of common channels and signals associated with mandatory system information.

Specifically, we consider the former one, i.e., beam hopping (BH) with equal time reuse factor (TRF) = 3.

All these schemes are studied with baseline ABS and as well as with ABS = 0.0668,  ABS'=0.0945 and ABS'=0.1157.

Figure 2. Illustration of beam hooping technique (TRF =3) and frequency reuse technique (FRF = 3)

The channel bandwidth and maximum transmission bandwidth play an important role on the coverage enhancements techniques. When assuming a 5 MHz channel bandwidth (CBW), the maximum transmission bandwidth is 25 PRBs, which in our understanding has different implications on the “Frequency Reuse” and “Beam Hopping” techniques, for example:

- FRF-3: In this case there is a number of beams associated to “Frequencies 1, 2, and 3” respectively, meaning the 25 PRBs corresponding to the maximum transmission bandwidth would be split into three spectrum portions of e.g., 8 PRBs for each of the “Frequencies 1, 2, and 3”. The splitting of the maximum transmission bandwidth is foreseen to impose some limitations on the “Frequency Reuse” technique at least for initial access, since at least 20-PRBs are needed to transmit the SS/PBCH block. This means that for FRF-3, likely a channel bandwidth of 15 MHz would be required as to split the maximum transmission bandwidth of 79 PRBs into three spectrum portions of e.g., 25 PRBs where the SS/PBCH block can fit. Also in Table 1 the maximum bandwidth indicated is per beam and that does not mean the total channel bandwidth available 5 MHz and it is reasonable to consider CBW of 15 MHz, which provides the maximum transmission bandwidth of 5 MHz for the beams operating on each spectrum portion. Therefore, In the evaluations that follow, we consider the maximum transmission bandwidth of 25 PRBs of 5 MHz.

- BH with TRF-3: In this case there is a number of beams associated to “beam hopping patterns A, B, and C” respectively, where 25 PRBs become available for transmission at a specific time for each of the beams in “beam hopping pattern A” whereas beams associated to “beam hopping patterns B and C” are inactive, thereafter at a second specific time the 25 PRBs become available for transmission for each of the beams in “beam hopping pattern B” whereas beams associated to “beam hopping pattern C and A” are inactive, and so on in a cyclic fashion. Overall, the full maximum transmission bandwidth is available for each of the active beam. In the evaluations that follow, we consider the transmission bandwidth of 5 MHz.

Shown in Figure 3  are the CDFs of geometry SNR for FRF = 1, FRF = 3 as well as for beam hopping (BH) with time reuse factor (TRF) = 3. We adopt beam layout with different values of ABS = 0.0668 , ABS'=0.0945 and ABS'=0.1157. As can be seen in the Figure, the dynamic range of SNR increases with the increase of beam size as the serving link range increases compared to that of ABS = 0.0668.

Next, The CDFs of geometry SIR for FRF = 1, FRF = 3 and BH with TRF = 3, with different values of ABS = 0.0668 , ABS'=0.0945 and ABS'=0.1157 are presented in Figure 4 while the corresponding SINRs are presented in Figure 5. For FRF=1, ABS'=0.0945 or ABS'=0.1157 provides reduced interference level and hence SIR improves. If we compare the curves of ABS = 0.0668, ABS'=0.0945 and ABS'=0.1157 for FRF=3/TRF=3, the user locations in the cell interior part experience reduced interference and hence the corresponding SIR increases while the user locations in the cell exterior part experience reduced signal level due to the large link range, and hence marginal decrease in the SIR (though the aggregate interference level also reduces as the interfering link distances increase). Overall, the SIR improves with increased ABS'=0.0945.

Figure 3. CDFs of the downlink geometry SNR for FRF = 1, FRF = 3 and BH with TRF = 3. Beam layouts with ABS = 0.0668 , ABS'=0.0945 and with ABS'=0.1157 are considered.

For FRF = 1, larger ABS with the same HPBW reduces the aggregate interference and hence improves the downlink SINR in majority UE locations (excluding the cell-edge users), while marginally decreasing the SINR for the cell-edge users.

For FRF = 3 and BH with TRF = 3, larger ABS with the same HPBW reduces interference and hence improves the downlink SINR in a limited number of UE locations (i.e., around the cell center), while reducing the SINR for majority UE locations, including the cell-edge users.

RAN1 to discuss the baseline techniques such as FRF = 1 and FRF = 3 while comparing the performance of beam hopping technique in the downlink coverage enhancements and the specification impact point of view.

Figure 4. CDFs of the downlink geometry SIR for FRF = 1, FRF = 3 and BH with TRF = 3. Beam layouts with ABS = 0.0668 , ABS'=0.0945 and with ABS'=0.1157 are considered.

Figure 5. CDFs of the downlink geometry SINR for FRF = 1, FRF = 3 and BH with TRF = 3. Beam layouts with ABS = 0.0668 , ABS'=0.0945 and with ABS'=0.1157 are considered.

### Considerations for beam hopping

As part of the WID objective, i.e., DL coverage enhancement, system-level or link-level enhancement that is considered shall aim to improve the coverage across the satellite footprint. If we inspect the scenarios considered in Sets 1-1, Set 1-2 and Set 1-3, simultaneous active beams are 10.02%, 1.5% and 10.02%, respectively, which determines the fraction of area that can be served under the satellite footprint. One way to increase the fraction of coverage area is to increase the number of simultaneous active beams. However, due to the power budget and number of RF chains available at the satellite payload, the number of active beams can’t be increased beyond certain limit. This eventually leads to the utilization of beam hopping, which is discussed in the previous sections.

For instance, consider Set 1-1. If a beam pattern consists of distinct 10% of beams is activated each time instance to provide the coverage on the earth, then it requires total 10-time instances to cover the whole satellite footprint, i.e., TRF = 10. After completing one such round, the first beam pattern gets its turn to serve again. The time involved between two instances a beam in a specific beam pattern being active is called revisit time, Trevisit, which is limited by the timeline constraints imposed by the 5G NR common signals (e.g., SSB, SIB1, SIB19, etc.) that facilitate the UE to access the network.  For instance, the UEs in IDLE mode are expected to receive SSB once every 20 msec.  If we base this as a revisit time interval, each beam in a beam pattern can only get 2 msec interval to be active, called dwell time, Tdwell. Such a short interval of dwell time will not be sufficient to meet the traffic demands in each cell illuminated by a beam. One can propose to relax the periodicity of SSB in this context of Rel-19 coverage enhancement so that Trevisit can be increased and hence Tdwell. Additionally, there are timelines associated with the other common signals need to be accounted here. Altogether, this is not straightforward and may involve significant specification impact. Furthermore, RAN1 need to study first the impact on the legacy UEs by such changes to the periodicity of common signals.

Alternatively, one can combine the beam hopping with the judicious selection of the beam size such that required TRF can be reduced to cover the satellite footprint. This allows to increase the Tdwell for a given Trevisit. For instance, if we consider a beam layout with n=2 and n=3 in ABS', then the total number beam footprints are about 529 and 353 and the required TRFs are 5 and 3 (excluding 36 beam footprints). The corresponding Tdwell are about 4 msec and 7 msec (for one pattern 6 msec). Therefore, larger ABS allows to reduce the required TRF which in turn yields increased Tdwell for a given Trevisit in the context of beam hopping.

Larger ABS allows to reduce the required time reuse factor which in turn yields increased beam illumination ON-time for a given Trevisit in the context of beam hopping.

RAN1 to consider beam layout with larger ABS without increasing the HPBW in the system-level evaluations of the downlink coverage enhancements study.

RAN1 aim to consider the low or moderate time reuse factors (that allows reasonable Tdwell) in the context of beam hopping to limit the specification impact.

The system-level evaluations, including the simulations for calibration, in the previous releases consider a certain number of UEs are uniformly distributed within each NTN cell [5]. Compared to terrestrial cells, NTN cells are much larger and the UE placement in the cells follows the population distribution of the corresponding service area, which can change within/across cells. Also, the uniform density of the UEs across cells may not be able to represent the traffic imbalance situations among NTN cells, e.g., cell with large population may have large traffic demands. To obtain the relevant numbers for Trevist and Tdwell and consequently assess the benefits of beam hopping in terms of the other KPIs such as traffic satisfaction, it is important to revest the UE distribution to account for abovementioned aspects in the system-level evaluations.

The existing UE distribution model does not reflect the expected NTN deployment in terms of cell user density, cell population/traffic imbalance.

RAN1 to revisit the UE distribution model to reflect the relevant NTN deployment aspects, including the non-uniformity of the traffic distribution in system-level evaluation of beam hopping.

### Phased array antenna model

Agreement (cf. RAN1#116) Adopt the following phased array antenna parameters for LEO 600km in FR1: Satellite phased array antenna Characteristics; LEO-600. Orbit; LEO-600km. Frequency range/band; FR1/S-Band. Antenna element pattern; Table7.3-1 in TR 38.901. Horizontal/vertical 3 dB beam width of single element (degree); [65] for H; [65] for V. Antenna polarization; Circular (RHCP or LHCP). Number of antenna elements; [400 elements (20 x 20)]. Equivalent satellite antenna aperture; 2m. Element maximum gain; 4 dBi. Antenna maximum gain; 30 dBi. Steering loss at 30° elevation angle; [4dB].

To provide the reasonable values for the parameters in square brackets, we utilize the guidelines provided in Section 7.2.4 Array antenna model in [3]. According to this, the peak element directivity (assuming no losses for a given antenna aperture area) can be expressed as:

DE,max≤10log104πdhdvλ2		(Eq. 7.2.4-1)

where dh and dv  represent horizontal element separation and vertical element separation, respectively. Based on this, one can compute the lossless peak element directivity as in the range of 4.97 dB to 10.07 dB when the element spacing is set in the range of  0.5 λ  to 0.9 λ. Therefore, it is essential to understand the typical element spacing employed in the Satellite antennas to obtain the limit on the element maximum gain. Based on that, one can then compute the number of elements fit within a certain antenna aperture as function element spacing, which determines the antenna maximum gain.

RAN1 to discuss the typical element spacing employed in the Satellite antennas, which is necessary to set the element peak gain and hence the number of elements fit within a certain antenna aperture as function element spacing.

## Link budget analysis

The WID objective for DL coverage enhancement includes the following guidelines to be considered:

- Notes for this objective: - SSB channel enhancement is not considered - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

Since as per the WID there is no enhancement considered for SSB and given detecting SSB is essential for initial access, we first consider evaluating the link budget of SSB, which can serve as a reference point for other physical channels and signals that are subsequently transmitted in the DL.

Agreement (cf. RAN1#116) For link-level study, downlink coverage performance in NR NTN is evaluated according to the following steps. Step 1: CNR is calculated as defined in 6.1.3.1 of TR 38.821 Step 2: Required SNR of target service is evaluated by LLS Step 3: The CNR and the required SNR are compared Agreement (cf. RAN1#116) For link budget calculation, parameters in the following table are assumed: Parameters. Carrier frequency; 2 GHz for DL (S-band). Satellite altitude; 600 km. Target elevation angle; 30° (LEO). Atmospheric loss; Equation (6.6-8) in [38.811]. Shadowing margin; 3 dB. Scintillation loss; Section 6.6.6 in [38.811] Ionospheric loss: = 2.2 dB Tropospheric loss: Table 6.6.6.2.1-1 of [38.811]. Additional loss; 0 dB. Clear sky conditions; Yes. Satellite antenna polarization; Circular polarization. Terminal type; [S band: (M, N, P) = (1,1,2)]. UE antenna gain; -5.5dBi. Free space path loss; Equation (6.6-2) in [38.811]. Polarization loss; 3dB. Outcome; CNR.

By considering the satellite EIRP density per beam of 34 dBW/MHz as defined for LEO 600 km set1-1 FR1 and applying the parameters as agreed above, the target CNR is found to be -1.88 dB (cf. Table 2).

Table 4. Link budget summary for the scenario LEO 600 km Set1-1 FR1.

| Scenario | LEO-600 Set1-1/Set1-2 FR1 | LEO-600 Set1-3 FR1 |
| --- | --- | --- |
| Terminal Type | Handheld | Handheld |
| Elevation Angle | 30° | 30° |
| Direction | DL | DL |
| Satellite EIRP density/beam [dBW/MHz] | 34.0 | 63.0 |
| Bandwidth [MHz] | 5 | 5 |
| TX: Satellite EIRP/beam [dBm] | 71 | 33 |
| UE antenna gain | -5.5 dBi | -5.5 dBi |
| RX: G/T [dB/K] | -37.12 | -37.12 |
| Boltzmann constant k [dBW/K/Hz] | -228.6 | -228.6 |
| Free space path loss (PL) [dB] | 159.1 | 159.1 |
| Atmospheric loss (LA) | 0.0660 | 0.0660 |
| Shadow fading margin (SF) [dB] | 3 | 3 |
| Scintillation loss (SL) [dB] | 2.2 | 2.2 |
| Polarization loss [dB] | 3 | 3 |
| Additional losses (AD) [dB] | 0 | 0 |
| Target CNR [dB] | -1.88 | -9.88 |

The CNRs for Set1-1/Set1-2 and Set 1-3 are computed as -1.88 dB and -9.88 dB, respectively, which translates to -5.88 dB and -13.88 dB, respectively, when the 4 dB beam steering loss is applied.

Due to the relatively lower satellite EIRP density per beam, Set 1-3 provides lower CNR value. This leads to the reduction in the link margin and hence the downlink coverage performance.

Given that the CNR value is relatively lower for Set 1-3, RAN1 to revisit the setting of Set 1-3 or to prioritize Set 1-1/1-2 in the study.

## Assumptions for link level simulations

The settings applicable to the link-level simulations of SSB are summarized in Table 3 of Annex B.

In relation with the assumptions for PDCCH in CORESET#0, when a 5 MHz channel bandwidth and both SSB and PDCCH are assumed to use a 15 kHz subcarrier spacing, then the CORESET’s #0 configuration is given by Table 13-1 in TS 38.213. Based on that configuration table, we selected a CORESET#0 size consisting of 24-PRBs along with 2-OFDM symbols, which allow us to use an aggregation level equal to 8 (i.e., AL = 8), summarized in Table 5.

Table 5. Assumptions for PDCCH link-level simulations.

| Parameter | Value |
| --- | --- |
| Target BLER | 1% |
| Channel model | NTN-TDL-C |
| Payloads of PDCCH | 40 bits |
| CORESET size | 24 PRBs, 2 OFDM symbols |
| Aggregation level | 8 |
| Interleaving | Non-interleave |

RAN1 to consider the settings in Table 5 for PDCCH link-level simulation in the study of downlink coverage enhancements.

The settings to the PDSCH link level simulations are explicitly listed in subsection 6.2 in Annex B, which are based on the Release-18 coverage enhancement discussion.

## Link margins for downlink channels

To determine the “Link Margin” of SSB, the link-level simulations are conducted based on settings summarized the previous subsection. The link budget evaluations of SSB resulted in a “Link Margin” of +7.22 dB. Table 6 provides an overview of the obtained results from the link-budget analysis including the link margins of SSB, PDCCH, Msg2 PDSCH, Msg4 PDSCH, and “low data rate service” PDSCH, including how those margins are with respect to SSB. We have not considered the paging channel in the link budget analysis yet as it was not part of the Realease-18 coverage enhancement discussions.

From the link budget evaluations, we can observe that the link margin difference between SSB and PDCCH in CORESET0 is ΔSSB-PDCCH ~ +2.56 dB, whereas the link margin difference between SSB and Msg2 PDSCH is ΔSSB-PDSCH ~ -1.7 dB (meaning Msg2 PDSCH outperforms SSB), while for Msg4 PDSCH and for “low data services” PDSCH the resulting link margin differences are ΔSSB-PDSCH_Msg4 ~ +2.2 dB and ΔSSB-PDSCH_low_data_rate ~ + 2.1 dB. Thus, from the above evaluations the largest the link margin difference was found between SSB and PDCCH in CORESET0 (ΔSSB-PDCCH ~ +2.56 dB). Thus, PDCCH in CORESET0 would require to be enhanced by ~ +2.56 dB to match the link margin of SSB, however other DL physical channels will require an enhancement of ~ +2 dB to be on-par of the link margin of SSB. One important aspect to highlight is that overall DL is not a bottle neck since typically there is a significant coverage margin between SSB and the uplink physical channels (e.g., PUSCH), thus the coverage enhancement target in dB can be moderate aiming at enhancing the DL channels or the most imbalanced DL channel with respect to SSB (Otherwise, DL and UL might become even more imbalanced).

Table 6. Link-budget evaluations and “Link Margins” comparison of SSB, PDCCH in CORESET0, Msg2 PDSCH, Msg4 PDSCH, and PDSCH (Low data rate service) for LEO Set 1-1 settings under TDL-C channel.

| Physical Channel/signal | Target CNR (dB) | SNR (dB) | Downlink Margin (dB) | Delta (dB) with respect SSB |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSB (BLER Target 1%) | -1.88 | -9.1 | +7.22 | ΔSSB-PDCCH | +4.3 | ΔSSB-PDSCH_Msg2 | Outperforms SSB by 1.7dB. | ΔSSB-PDSCH_Msg4 | +2.2 | ΔSSB-PDSCH_Low_data_rate | +2.1 |
| PDCCH (BLER Target 1%) | -1.88 | -4.8 | +2.92 |  |  |  |  |  |  |  |  |
| PDSCH (Msg2) (BLER Target 10%) | -1.88 | -10.8 | +8.92 |  |  |  |  |  |  |  |  |
| PDSCH (Msg4) (BLER Target 10%) | -1.88 | -6.9 | +5.02 |  |  |  |  |  |  |  |  |
| PDSCH (Low data rate service) (BLER Target 10%) | -1.88 | -7 | +5.12 |  |  |  |  |  |  |  |  |

From preliminary link budget evaluations on DL physical channels, the link margin of SSB (which is a DL physical channel that shall be detected during initial access) was used as reference, and then link margin difference between SSB and other DL physical channels was estimated as follows: ΔSSB-PDCCH ~ +2.56 dB , ΔSSB-PDSCH ~ -1.7 dB (meaning Msg2 PDSCH outperforms SSB), ΔSSB-PDSCH_Msg4 ~ +2.2 dB, and ΔSSB-PDSCH_low_data_rate ~ +2.1 dB.

Overall DL is not a bottleneck since typically there is a large coverage margin between SSB and the UL physical channels (e.g., PUSCH), thus the coverage enhancement target in dB can be moderate aiming at enhancing the DL channels or the most imbalanced DL channel with respect SSB (otherwise DL and UL might become even more imbalanced).

### Impact of increased periodicity of SSB

During the last meeting, there was a discussion related to the increased periodicity of SSB, including the 1 SSB used for processing instead of 4 SSBs. It is necessary to consider its impact on the link margin by means of link-level evaluations as well as the impact on the legacy UEs.

RAN1 to discuss the impact on the legacy UEs due to the increased periodicity of SSB.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The adopted UV plane is not a fitting methodology to allow an accurate inter-beam interference calculation, nor a placement of high number of spot beam tier layouts.

Observation 2	The adopted ABS definition couples the satellite antenna beam width, which is a fixed simulation configuration, with the ABS. As an effect, the system cannot be optimized in terms of number of beam footprints within the satellite coverage area and SNR/SIR balancing.

Observation 3	If one increases the ABS by n, then the hexagonal cell area increases by n in the UV plane. Then, the number of satellite beam footprints required to provide the coverage to the specific region of interest can be decreased by increasing the ABS in the UV plane.

Observation 4	For FRF = 1, larger ABS with the same HPBW reduces the aggregate interference and hence improves the downlink SINR in majority UE locations (excluding the cell-edge users), while marginally decreasing the SINR for the cell-edge users.

Observation 5	For FRF = 3 and BH with TRF = 3, larger ABS with the same HPBW reduces interference and hence improves the downlink SINR in a limited number of UE locations (i.e., around the cell center), while reducing the SINR for majority UE locations, including the cell-edge users.

Observation 6	Larger ABS allows to reduce the required time reuse factor which in turn yields increased beam illumination ON-time for a given Trevisit in the context of beam hopping.

Observation 7	The existing UE distribution model does not reflect the expected NTN deployment in terms of cell user density, cell population/traffic imbalance.

Observation 8	The CNRs for Set1-1/Set1-2 and Set 1-3 are computed as -1.88 dB and -9.88 dB, respectively, which translates to -5.88 dB and -13.88 dB, respectively, when the 4 dB beam steering loss is applied.

Observation 9	Due to the relatively lower satellite EIRP density per beam, Set 1-3 provides lower CNR value. This leads to the reduction in the link margin and hence the downlink coverage performance.

Observation 10	From preliminary link budget evaluations on DL physical channels, the link margin of SSB (which is a DL physical channel that shall be detected during initial access) was used as reference, and then link margin difference between SSB and other DL physical channels was estimated as follows: ΔSSB-PDCCH ~ +2.56 dB , ΔSSB-PDSCH ~ -1.7 dB (meaning Msg2 PDSCH outperforms SSB), ΔSSB-PDSCH_Msg4 ~ +2.2 dB, and ΔSSB-PDSCH_low_data_rate ~ +2.1 dB.

Observation 11	Overall DL is not a bottleneck since typically there is a large coverage margin between SSB and the UL physical channels (e.g., PUSCH), thus the coverage enhancement target in dB can be moderate aiming at enhancing the DL channels or the most imbalanced DL channel with respect SSB (otherwise DL and UL might become even more imbalanced).

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN1 to consider beam layout with at least one increased value of ABS (e.g., 0.1157, equivalently, beam size of 86.6 km) in addition to the baseline ABS of 0.0668 (equivalently beam size of 50 km) with the same HPBW.

Proposal 2	RAN1 to discuss the baseline techniques such as FRF = 1 and FRF = 3 while comparing the performance of beam hopping technique in the downlink coverage enhancements and the specification impact point of view.

Proposal 3	RAN1 to consider beam layout with larger ABS without increasing the HPBW in the system-level evaluations of the downlink coverage enhancements study.

Proposal 4	RAN1 aim to consider the low or moderate time reuse factors (that allows reasonable Tdwell) in the context of beam hopping to limit the specification impact.

Proposal 5	RAN1 to revisit the UE distribution model to reflect the relevant NTN deployment aspects, including the non-uniformity of the traffic distribution in system-level evaluation of beam hopping.

Proposal 6	RAN1 to discuss the typical element spacing employed in the Satellite antennas, which is necessary to set the element peak gain and hence the number of elements fit within a certain antenna aperture as function element spacing.

Proposal 7	Given that the CNR value is relatively lower for Set 1-3, RAN1 to revisit the setting of Set 1-3 or to prioritize Set 1-1/1-2 in the study.

Proposal 8	RAN1 to consider the settings in Table 5 for PDCCH link-level simulation in the study of downlink coverage enhancements.

Proposal 9	RAN1 to discuss the impact on the legacy UEs due to the increased periodicity of SSB.

# References

RP-240775, “New WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, 3GPP TSG RAN Meeting #103, Maastricht, The Netherlands, March 18-22, 2024.

R1-2401769, Session notes for 9.11 (Non-Terrestrial Networks for NR Phase 3 and Internet of Things Phase 3), Ad-Hoc Chair (Huawei), RAN1#116, March 2024.

3GPP TR38.820, “Study on the 7 to 24 GHz frequency range for NR”, v16.1.0, March 2021.

3GPP TR 38.811, "Study on New Radio (NR) to support non-terrestrial networks", v15.4.0, September 2020.

3GPP TR 38.821, "Solutions for NR to support Non-Terrestrial Networks (NTN)", v16.1.0 June 2021.

# Annex A

Table 6.1.1.1-4: Beam layout definition for single satellite simulation [5]

| Scenario | Scenario A, C2 and D2 |
| --- | --- |
| Beam layout definition | Baseline: Hexagonal mapping of the beam bore sight directions on UV plane defined in the satellite reference frame. Only the 3dB beam width parameters should be used. The beam diameter and beam spacing values can be computed directly from the 3 dB beam width assumptions and should be considered as informative. |
| Number of beams | Baseline: 19-beam layout considering wrap-around mechanism (i.e. 18 beams surrounding the central beam and allocated on 2 distinct "tiers") |
| UV plane illustration (extracted from [19]) |  |
| UV plane convention | U axis is defined as the perpendicular line to the satellite-earth line on the orbital plane as illustrated here after: The straight line being orthogonal to UV plane is pointing towards the Earth centre. UV coordinates of the nadir of the reference satellite is (0,0) |
| Adjacent beam spacing on UV plane | Baseline: Adjacent beam spacing computation based on 3dB beam width of the satellite antenna pattern: ABS = sqrt(3) x sin(HPBW/2 [rad]) |
| Central beam bore sight direction definition | Baseline: Case 1: Central beam center is considered at nadir point Case 2: Central beam boresight direction computed based on elevation angle target |

# Annex B

Table 8. Link level simulation parameters common to all simulations.

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2 GHz (S-band) |
| Waveform | DL: CP-OFDM |
| SCS | 15 kHz |
| Max channel bandwidth | 30 MHz (as in TR 38.821 [3] ) |
| Channel model | NTN-TDL-A (NLOS) and NTN-TDL-C (LOS) (As in Table 6.1.2-4 of TR 38.821 [3] ) Evaluation scenario: Rural Elevation angle: 30 deg |
| Channel estimation | Realistic |
| UE speed | 3 km/h |
| Frequency offset/drift | 0.1 ppm offset, no drift assumed |
| UE antenna type and configuration | DL: 2RX, cross-polarized linear polarization omnidirectional antenna elements |
| Satellite antenna type and configuration | DL: 1TX circular polarization |

The link-level simulations assumptions for performing a link-level evaluations of SSB, PDCCH in CORESET0, PDSCH for low data rate service, PDSCH for Msg2, and PDSCH for Msg4 can be found below:

## SSB

Table 9. Assumptions for SSB simulations.

| Parameter | Value |
| --- | --- |
| BLER target | 1% |
| Channel model | NTN-TDL-C |
| Periodicity | 20 ms |
| Allocated RBs | 20 PRBs |
| Performance metric | Combination of 4 SSBs in 80ms. |

## PDSCH

### Low data rate service

Table 11. Assumptions for PDSCH simulations of low data rate service.

| Parameter | Value |
| --- | --- |
| BLER target | 10% iBLER |
| HARQ configuration | Enabled for LEO scenarios |
| DMRS configuration | Type I, 2 DMRS symbols, no multiplexing with data. |
| PDSCH duration | 12 OS |
| PRBs/TBS/MCS | 160 PRBs (30 MHz) MCS-0 from Table 5.1.3.1-2 and Table 5.1.3.1-3 of 38.214 [2] are used. |
| Aggregation factor | 1,2,3,4,5,6,7,8,16 |
| Simulation length | 20000 slots |

#### MSG2

Table 12. Assumptions for PDSCH simulations of MSG2 transmission.

| Parameter | Value |
| --- | --- |
| BLER target | 10 % |
| Channel model | NTN-TDL-A and NTN-TDL-C |
| HARQ configuration | Disabled |
| DMRS configuration | 3 DMRS symbols |
| PDSCH duration | 12 OS (of which 3 are used for DMRS) |
| PRBs/TBS/MCS | As in TR 38.830 [5]: 12 PRBs Payload is 8 bytes MCS-0 from Table 5.1.3.1-1 of 38.214 [2] with TB scale factor 0.25 |
| Aggregation factor | 1 |
| Simulation length | 20000 slots |

#### MSG4

Table 13. Assumptions for PDSCH simulations of MSG4 transmission.

| Parameter | Value |
| --- | --- |
| BLER target | 10% |
| Channel model | NTN-TDL-C |
| Payload | 1040 bits |
| TBS | >=1040 bits |
| DMRS configuration | 1+2 |
| PRBs num per slot | 42 PRBs |
| OSs num per slot | 12 OSs |
| MCS index | 0 |
| MCS table | Table 5.1.3.1-2: MCS index table 2 for PDSCH |
| Aggregation factor | 1 |
| HARQ | No |