3GPP TSG RAN WG1#116bis			R1-24xxxxx

Changsha, China, April 15th – 29th, 2024

Source:	Moderator (Nokia)

Title:	[116bis-Pre-R18-NR] Summary of maxRank configuration restriction with fullpowerMode1 and transform precoding ‘enabled’

Agenda Item:	7

Release:	Release 16

WI code:	NR_eMIMO-core

Document for:	Discussion and Decision

Introduction

A question on contradicting specification in the Rel-16 TS 38.212 table headings 7.3.1.1.2-3A/5A and the corresponding Precoding information and number of layers -bitfields in DCI formats 0_1 and 0_2 are in contradiction.

- Table headings condition the usage of the tables to fullpowerMode1, and [{CP-OFDM with maxRank = 1}, or {transform precoding with any maxRank}]

- DCI format 0_1/0_2 bitfields condition the usage of the tables to fullpowerMode1, and maxRank=1

The maxRank configuration is common for CG-PUSCH and DG-PUSCH, while the waveform is configured separately for the two.

Furthermore, the table headings 7.3.1.1.2-3A/5A do not refer to the maxRankDCI-0-2 that is the condition to be applied with DCI format 0_2.

| Ref | Proposals |
| --- | --- |
| [1, 2] | Proposal 1: When the PUSCH to be transmitted in response to a DCI format 0_1 is transform precoded, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, the tables 7.3.1.1.2-3A and 7.3.1.1.2-5A apply regardless of the configured value of maxRank. Proposal 2: When the PUSCH to be transmitted in response to a DCI format 0_2 is transform precoded, txConfig = codebook, ul-FullPowerTransmission = fullpowerMode1, the tables 7.3.1.1.2-3A and 7.3.1.1.2-5A apply regardless of the configured value of maxRank or maxRankDCI-0-2. Proposal 3: Agree to the TP according to the accompanying draft CR [2] |

References

- R1-2403313, On maxRank configuration restriction with fullpowerMode1 and transform precoding ‘enabled’, Nokia

- R1-2403314	Correction to maxRank configuration restriction with fullpowerMode1 and transform precoding ‘enabled’, Nokia

Discussion

Based on the Monday morning online discussion, several companies saw the discrepancy as something to be fixed, while some concerns were raised on whether there is any unclarity with the specification.

Question 1: Are you fine with the general direction proposed in [1]. If you have a concern, please explain.

Please provide your comments on the proposals 1 and 2 to the table below

| Company | Comments |
| --- | --- |
| Apple | We are fine with the proposed CR |
| MTK | We support the proposed CR. As discussed during online session, considering DG and CG PUSCH, it is not reasonable to fix maxRank = 1 for transform precoding. |
| Nokia | As the proponent we obviously support the correction and the CR |
| OPPO | We are fine with the proposed CR. |
| Ericsson | We support the CR. This seems to be a wording error, and as Nokia points out, the text describing the table is inconsistent with the table heading itself. The use case of DFT-S-OFDM CG + CP -OFDM DG is quite reasonable, and the proposed correction avoids confusion that it would not be supported. |
| ZTE | Fine with the proposed CR for the wording error modification. |

Question 2: Do you have specific comments or revision proposals to the draft CR in [2]?

Please provide your comments on the proposals 1 and 2 to the table below

| Company | Comments |
| --- | --- |