3GPP TSG RAN WG1 #117	R1-2404527

Fukuoka, Japan, May 20th – 24th, 2024

Source:	Lenovo

Title:	On AI/ML for CSI prediction

Agenda Item:	9.1.3.1

Document for:	Discussion and Decision

# Introduction

In Rel-18, AI/ML-based CSI feedback enhancements for CSI prediction were discussed. It was agreed to extend the study of AI/ML-based CSI prediction to Rel-19, with the following scope of the study agreed in RAN#102 [1]:

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CSI feedback enhancement [RAN1]: - For CSI prediction (one-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843 (e.g., cell/site specific model could be considered to improve performance gain).

Furthermore, in RAN1#116bis [2], the following agreements and conclusions were reached:

Agreement - For the AI/ML based CSI prediction, adopt following assumptions as a baseline for evaluation purpose - UE speed: 30km/h, 60km/h - Others can be additionally submitted, e.g., 10km/h, 120km/h - Observation window (number/distance): 5/5ms,10/5ms - Others can be additionally submitted, e.g., 4/5ms, 15/5ms - Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms, 4/5ms/5ms - Others can be additionally submitted, e.g., 2/5ms/5ms, 3/5ms/5ms, 1/5ms/10ms - For other assumptions, reuse Rel-18 baseline Agreement - For the AI/ML based CSI prediction, for CSI report, adopt following as a baseline for evaluation purpose - N4 value: 1, 4 - Others can be additionally submitted, e.g., 2, 8 - paramCombination-Doppler-r18: 6,7 or paramCombination -r16 = 5,6 (for Benchmark 1) - Others can be additionally submitted. - Note: The same selected parameter combination shall be applied for benchmarks. - CSI report periodicity: 5ms, 20ms (encouraged) - Others can be additionally submitted, e.g., 10ms Conclusion Consider error modelling in TR36.897 Table A.1-2 as a baseline if channel estimation error is modeled. - Other modelling is not precluded, and companies should report how to model channel estimation error if other modelling is considered. Conclusion If phase discontinuity is modeled, it is modelled as a uniform distribution between -ϕmax,ϕmax within a time window of 20msTwindow, where ϕmax=40 degrees can be a baseline. - Other modelling is not precluded, and companies should report how to model phase discontinuity if other modelling is considered, and additional Twindow.，if adopted Conclusion For the phase discontinuity modelling, it is clarified that - A fixed phase for all CSI-RS observations within the time window, and another fixed phase for the next time window. The phases are according to uniform distribution. Conclusion - For evaluation of the UE-sided model based CSI prediction, UE distribution of (80% indoor, 20% outdoor) can be optionally simulated. Note: Indoor speed is 3 km/h, outdoor speed is chosen from the following options: 30 km/h, 60 km/h. Assumption on O2I car penetration loss and spatial consistency follow the Rel-18 AI/ML based CSI prediction Agreement For the results template used to collect evaluation results for UE -sided model based CSI prediction, adopt Table 6 used in Rel-18 as starting point with the following addition: - Assumption - UE distribution (Baseline: 100% outdoor, Optional: 80% indoor, 20% outdoor) - Whether/how channel estimation error is modelled - Whether/how phase discontinuity is modelled - Methods used to handle the phase discontinuity (if applied) - Benchmark 2 - FLOPs/M - Details of complexity calculation, e.g., complexity of prediction and complexity of filter update Agreement - For the results template used to collect evaluation results for UE-sided model based CSI prediction using localized models, adopt Table 6 used in Rel-18 as starting point, capturing the generalized model result and the localized model result as separate columns, with the following additions for the localized model: - Dataset description - Local region modelling: e.g., Option 1 or Option 2, and further details - Temporal modelling: e.g., how temporal variation is modelled in train and test sets - Dataset description for generalized model Agreement For the UE-sided model based CSI prediction, for optional evaluation using AP CSI-RS, consider following assumption on observation window (number/distance) - Observation window: 12/2ms, 8/2ms, 4/2ms - Others can be additionally submitted Agreement For AI/ML based CSI prediction, at least for inference, legacy CSI-RS configuration can be a starting point. Further study on whether there is a need for specification enhancement. Agreement - At least for inference, for UE-sided model based CSI prediction, legacy feedback mechanism using codebook type set to “typeII-Doppler-r18” is a starting point of discussion. Study the necessity and potential specification impacts including at least following aspects: - CSI processing criteria and timeline Agreement For performance monitoring for functionality-based LCM, further study on details of type 1,2 and 3, e.g., potential specification impact, pros/cons aspects. - To clarify the boundary between type 1 and type 3 - To clarify definition of monitoring output and performance metric

In this contribution, we discuss further aspects of AI/ML-based CSI prediction via UE-sided models, as well as provide preliminary evaluation results for AI/ML-based CSI prediction compared with legacy non-AI/ML schemes.

# Scope of AI/ML-based CSI prediction

## 2.1 Observation/Prediction windows for CSI prediction

One important aspect of CSI prediction is the characterization of the observation and prediction windows associated with CSI prediction, where the observation window corresponds to the time duration for a CSI measurement occasion based on DL CSI-RS transmission, and the prediction window corresponds to the time duration for which the predicted CSI feedback is reported, corresponding to a future time with respect to the CSI measurements in the observation window. In Rel-18 MIMO WI for predicted CSI CB, both the observation window and prediction window for CSI prediction were specified for CSI enhancements for high-speed UEs. The design outline is as follows:

- Observation window: The time interval in which the UE is expected to receive a burst of CSI-RS transmissions. Two alternatives were proposed:

  - Alt1. A group of  p CSI-RS transmission occasions of a periodic/semi-persistent CSI-RS resource.

  - Alt2. A group of κ aperiodic CSI-RS resources with a spacing of m slots between two consecutive CSI-RS transmissions, where m=1,2, based on RRC configuration.

- Prediction window: The time interval comprising WCSI slots for which the UE feeds back the CSI report quantities, e.g., PMI corresponding to the precoding vectors, where WCSI=N4∙d, such that N4 is the number of precoding vectors fed back in one CSI report across time domain, and d is a number of slots over which the same precoding vector is valid. The value of d is configurable and upper bounded by the value of m.

Note that for UE-based CSI prediction, both the observation window and the prediction window may be configured by the network. Given that, we propose studying potential configurations of the observation and prediction windows for UE-based CSI prediction, if applicable.

For UE-based CSI prediction, both the observation window and the prediction window are network configured

Study potential configurations of the observation window and the prediction window for UE-based CSI prediction

## 2.2 CSI feedback format of CSI prediction

- Based on the Rel-18 study on AI/ML-based CSI framework, the input of the AI/ML model for CSI prediction can be in the form of a channel matrix, or the eigenvectors of the channel matrix. In this section, we discuss the format of the CSI feedback for CSI prediction. In legacy CSI feedback schemes, the CSI feedback is mainly reported in a format corresponding to a quantized set of coefficients of the precoding matrix, which is associated with a configured number of subbands, wherein for the special case of wideband reporting, the entire BWP corresponds to a single subband. To ensure a fair comparison between the proposed AI/ML-based CSI prediction scheme and legacy CSI feedback, the CSI feedback format under the proposed CSI prediction framework should match that of the legacy CSI feedback, i.e., the predicted CSI is reported in a form of a precoding matrix associated with a pre-configured number of sub-bands of a given bandwidth. Moreover, the PMI format over the spatial domain and the frequency domain should be similar to that of legacy PMI, i.e., reusing the DFT-based spatial-domain compression and frequency-domain compression, if applicable. Reusing the legacy CSI feedback format with respect to the CSI fields, and the space and frequency domain transformations of the PMI, is important to ensure that gains from AI-based CSI prediction stem from the time-domain processing of the channel, which is the main objective of the study of this sub-use case scenario.

The legacy Type-II CSI feedback format is used as the baseline for CSI feedback for AI-based CSI prediction, at least with respect to the configured CSI fields, the spatial domain and frequency domain representation within the PMI

For UE-based CSI prediction, the UE feeds back information corresponding to the RI(s), PMI(s) and CQI(s) corresponding to the prediction window duration. This approach resembles the Rel-18 framework for codebook-based CSI reporting for predicted PMI, where the PMI coefficients are DFT-transformed in time domain to reduce the CSI payload. Note that this time-domain DFT transformation is a suboptimal solution that was supported in Rel-18 codebook design to maintain the linearity of the codebook with respect to spatial/frequency/time domain transformation. On the other hand, AI/ML models for CSI prediction may be able to derive a more efficient transformation of the predicted CSI with concise CSI payload via non-linear transformation of the PMI coefficients in time domain. Given that, the CSI feedback format for CSI prediction needs to be further studied, at least with respect to time-domain representation of the CSI feedback corresponding to PMI. The Rel-18 codebook design for predicted PMI with time-domain DFT transformation can be considered a baseline for the study.

For UE-based CSI prediction, study potential enhancements of the CSI feedback format for predicted CSI, at least with respect to time-domain representation

Additionally, further design details regarding the CSI feedback for UE-based CSI prediction needs to be addressed, e.g., a computational complexity metric, e.g., number of CPUs, that quantifies measurements and/or computations corresponding to the CSI report, as well as the number of AI-based CSI reports that can be computed by the UE simultaneously across one (or all) CCs.

Study the CPU calculation for AI/ML-based CSI report(s) for UE-based CSI prediction

## 2.3 Performance monitoring of AI/ML-based CSI prediction

- As mentioned above, the CSI feedback corresponding to CSI prediction comprises a set of CSI parameters associated with multiple slots in time, where the CSI parameters correspond to the calculated CSI corresponding to the prediction window. For efficient performance monitoring, a comparison of the ground truth CSI and the AI/ML model output is needed. Note that model monitoring enables model adaptation based on the predicted CSI quality, where the performance monitoring outcome may include the following levels:

- Level-0: No AI model change. This applies when the performance based on the same AI/ML model is stable.

- Level-1: CSI parameters update. Under this level, the AI/ML model is unchanged, but a few parameter changes are applied, e.g., modifying the quantization resolution.

- Level-2: Model parameters update. Under this level, the structure of AI/ML model is unchanged, but some weights or parameters of the AI/ML model are updated.

- Level-3: AI model switching. Switching from one AI/ML model to another from a set of pre-configured AI/ML models to track changes in channel, e.g., change in channel conditions from LoS to NLoS and vice versa.

- Level-4: Fallback to non-AI scheme. This is the most extreme scheme adaptation level possible, in which the UE is switched to a legacy non-AI/ML CSI feedback scheme, e.g., Rel-18 Type-II Doppler codebook.

Study the specification impact corresponding to AI/ML model monitoring, considering the following monitoring decisions: (i) No model change, (ii) CSI parameters update, (iii) Model parameter update, (iv) Model switching, and (v) Fallback to non-AI/ML scheme

- For UE-based prediction, the UE possesses both the ground truth CSI, e.g., CSI measured without prediction based on observation window at time unit t + t0, and the AI/ML model output, e.g., predicted CSI for time unit t + t0 based on an earlier observation window at time t, and is hence expected to be pursued at the UE. For CSI prediction performance evaluation, intermediate KPI, e.g., SGCS between the precoding vectors under comparison, can be used. For performance monitoring evaluation, three reference time instants can be considered at: (i) the first slot, (ii) the median slot, and (iii) the last slot of the prediction window.

For performance monitoring under UE-based CSI prediction, three reference time instants are considered: (i) at the first slot of the prediction window, (ii) at the median slot of the prediction window, and (iii) at the last slot of the prediction window

While the CSI feedback overhead can be reduced with explicit performance monitoring feedback, i.e., where the UE feeds back the model monitoring recommendation to the network, this would lead to restricting the network flexibility with respect to analyzing the performance and further assessing the monitoring decision. One solution can be configuring the UE to monitor a set of configured events defined by the network, where the UE reports side information corresponding to the configured event with different level of detail based on configurable CSI feedback resolution. Further discussion is needed on how such events are defined, in addition to the corresponding CSI feedback format for performance monitoring. Given that, we have the following proposal.

For performance monitoring under UE-based CSI prediction, study and evaluate the pros and cons of the following alternatives:

- Implicit performance monitoring: the UE feeds back CSI measurements based on CSI report quantities or intermediate KPIs that enable the network to derive performance monitoring decisions

- Explicit performance monitoring: the monitoring feedback includes performance monitoring recommendation based on a set of network-configured performance monitoring metrics

## 2.4 Monitoring types for CSI prediction

For CSI prediction using UE side model use case, the following monitoring types for functionality-based LCM have been captured in TR 38.843 [3], as follows:

- Type 1: - UE calculates the performance metric(s) - UE reports performance monitoring output that facilitates functionality fallback decision at the network - Performance monitoring output details can be further defined - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed). - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 2: - UE reports predicted CSI and/or the corresponding ground-truth - NW calculates the performance metrics. - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 3: - UE calculates the performance metric(s) - UE reports performance metric(s) to the NW - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting).

- In RAN1#116bis, it was discussed that clarifying the boundary between Type 1 and Type 3 monitoring is needed. In our understanding, in Type 1 monitoring, the UE is configured with reporting a recommendation or proposed action to the network with respect to model adaptation, where the recommendation decision is based on a network configuration, e.g., event-triggered monitoring feedback with the event(s) configured by the network. On the other hand, Type 3 monitoring implies that the UE reports a pre-configured performance metric, e.g., CQI, GCS, that the network can analyze to make a decision on model adaptation. In other words, Type 3 implies that the processing of the performance metric for a model adaptation decision is pursued at the network side, whereas in Type 1 the UE computes an auxiliary recommendation based on the measured performance metric to assist the network with the model adaptation decision.

Type 3 monitoring implies that the processing of the performance metric for a model adaptation decision is pursued at the network side, whereas for Type 1 monitoring the UE computes an auxiliary recommendation based on the measured performance metric to assist the network with the model adaptation decision

- In order to elaborate more on the differences between Type 1 and Type 3 monitoring according to the description provided in TR 38.843, we provide our views on the proposed definitions of the monitoring output and performance metrics, as follows:

- Performance metric: a parameter computed at the UE based on channel measurements at the UE side, where the performance metric is implicit, i.e., not reported, for Type 1 monitoring, and is explicit, i.e., reported by the UE, for Type 3 monitoring.

- Monitoring output: a parameter comprising a decision (generated at the NW side) or a feedback of a recommended decision (generated at the UE side), based on performance metric(s) computed at the UE side.

For CSI prediction using a UE side model, adopt the following definition for ‘performance metric’: a parameter computed at the UE based on channel measurements at the UE side, where the performance metric is implicit, i.e., not reported, for Type 1 monitoring, and is explicit, i.e., reported by the UE, for Type 3 monitoring

For CSI prediction using a UE side model, adopt the following definition for ‘monitoring output’: a parameter comprising a decision (generated at the NW side) or a feedback of a recommended decision (generated at the UE side), based on performance metric(s) computed at the UE side

# Performance Evaluation for CSI Prediction

## Assumption of AI/ML-based CSI prediction

AI/ML model

In the evaluations, Conv-LSTM framework is adopted to pursue AI/ML based CSI prediction, where the input data is in the form of a raw channel matrix with dimensions of Nobs×Nport×Nresource, where Nobs denotes the observation window length in time, Nport is the number of ports and Nresource represents the number of frequency resource units. The framework consists of multiple ConvLSTM2D layers and a Conv2D layer as shown in Figure 1, which are used to learn spatial-temporal features of the input data to enable deriving the output predictions. The model uses the Adam optimizer and is trained using MSE as the loss function. Total number of parameters of our AI/ML model are ~6.8K.

Figure 1. Conv-LSTM framework for CSI prediction

Dataset for model training and testing

For AI/ML-based CSI prediction, system level simulations are used to generate channel matrix data, where 570 UEs are dropped randomly over 57 cells, with 10 UEs per cell, and the channel matrices corresponding to each UE are captured with an interval of 5ms within 300ms, resulting in a total of 34200 channel matrix samples. Based on the channel matrix samples, N sequences of CSI input and CSI output can be constructed with a certain CSI observation window length, where 80% of the sequences are used for model training and the remaining 20% of the sequences are used for model testing. Assumptions related to the system level simulation setup are provided in the Appendix.

Benchmark/reference for CSI prediction

To evaluate performance of AI/ML-based CSI prediction, two baseline schemes corresponding to non-AI/ML based CSI prediction methods are considered, i.e., sample-and-hold and auto-regression (AR) based CSI prediction. For auto-regression (AR) based CSI prediction, Yule-Walker equation is used to estimate parameters of AR model based on channel information of multiple past time instances.

## Performance evaluation results

For AI/ML-based CSI prediction, 5 CSI measurements with interval of 5ms are taken as model input to predict CSI in the next/future 5-10 ms. The dataset for training and testing is generated from 570 UEs of 57 cells, i.e., the trained model is a cell-common model. For AR based CSI prediction, we consider a measurement window size corresponding to the past 25 CSI measurements.

Performance evaluation based on intermediate KPIs

First, we compare the performance of the AI/ML-based CSI prediction with the two non-AI/ML baseline schemes for CSI prediction via two intermediate KPIs, (i) NMSE for both 5ms and 10ms CSI prediction, as shown in Table 1, and (ii) SGCS corresponding to up to 4 MIMO layers, as shown in Table 2. The SGCS of each MIMO layer is calculated separately based on the eigenvector of the predicted channel matrix.

|  | NMSE (dB) for CSI prediction |  |
| --- | --- | --- |
|  | 5ms prediction | 10ms prediction |
| AI/ML-based CSI prediction | -12.59 | -4.74 |
| AR-based CSI prediction | -11.30 | 0.56 |
| Sample-and-hold | 0.52 | 4.17 |

Table 1. NMSE performance results for 5-10 ms CSI prediction

|  | SGCS for 5ms prediction for up to 4 MIMO layers |  |  |  |
| --- | --- | --- | --- | --- |
|  | MIMO Layer 1 | MIMO Layer 2 | MIMO Layer 3 | MIMO Layer 4 |
| AI/ML-based CSI prediction | 0.93 | 0.89 | 0.83 | 0.79 |
| AR-based CSI prediction | 0.92 | 0.84 | 0.50 | 0.32 |
| Sample-and-hold | 0.80 | 0.72 | 0.67 | 0.54 |

Table 2. SGCS performance results for 5 ms CSI prediction for up to 4 MIMO layers

Based on results shown above, compared to sample-and hold method, NMSE can be improved by up to 13.11 dB using AI/ML-based CSI prediction scheme, whereas 16.25% gain for the SGCS for MIMO Layer 1 is achieved via the AI/ML-based CSI prediction scheme over a 5ms duration. The gains are reduced to 8.91 dB in NMSE for CSI prediction over a 10ms duration. On the other hand, AI/ML-based CSI prediction achieves limited performance gain over AR-based CSI prediction over a 5ms prediction window, i.e., 1.29 dB gain in NMSE and 1% gain for the SGCS for MIMO Layer 1, whereas the NMSE gain is improved to 5.3 dB for a 10ms prediction window. When more prior CSI information of past time instances are available for parameter estimation of AR model, its performance can be improved, even comparable with AI/ML method. Note that the parameters of AR model need to be updated in each prediction step and the number of parameters increases as the window size increases which leads to higher complexity. It is also clear from the intermediate KPI analysis that the performance of the AI/ML-based CSI prediction scheme for a 10ms prediction window degrades significantly compared with the same scheme for a 5ms prediction window. The same observation can be made on AR-based CSI prediction over 5ms and 10ms prediction windows.

Compared with sample-and hold CSI prediction scheme, NMSE for AI/ML-based CSI prediction scheme achieves a 13.11 dB improvement in NMSE and 16.25% improvement in MIMO Layer-1 SGCS gain for a 5ms CSI prediction window

AI/ML-based CSI prediction achieves marginal gains over auto-regression based CSI prediction scheme with respect to NMSE and MIMO Layer-1 SGCS for a 5ms CSI prediction window

The performance of the AI/ML-based CSI prediction scheme for a 10ms prediction window, with respect to NMSE and SGCS KPIs, degrades significantly compared with the same scheme for a 5ms prediction window

Performance evaluation based on UPT

To compare the system performance of AI/ML-based CSI prediction scheme with that of the non-AI/ML CSI prediction schemes, mean UPT as well as 5%-ile UPT of the AI/ML-based CSI prediction and the two non-AI/ML-based CSI predictions mentioned above have been evaluated and compared. The relative performance gains of AI/ML method over these non-AI/ML methods are shown in Figure 1. For system performance evaluations with respect to mean UPT, the AI/ML-based CSI prediction achieves 24% and 51% gains over AR-based and sample-and-hold CSI prediction schemes, respectively, over a 5ms CSI prediction window. On the other hand, the AI/ML-based CSI prediction scheme and AR-based CSI prediction scheme achieve similar performance with respect to for 5%-ile UPT, whereas AI/ML-based CSI prediction scheme achieves 10.5% gain over sample-and-hold CSI prediction scheme, over a 5ms CSI prediction window.

Figure 2. System performance gain of AI/ML CSI prediction over non-AI CSI prediction

System performance evaluations show that with respect to mean UPT, the AI/ML-based CSI prediction achieves 24% and 51% gains over AR-based and sample-and-hold CSI prediction schemes, respectively, over a 5ms CSI prediction window

System performance evaluations show that with respect to 5%-ile UPT over a 5ms CSI prediction window, the AI/ML based CSI prediction achieves:

- Similar performance to AR-based CSI prediction scheme

- 10.5% gain over sample-and-hold CSI prediction scheme

# Conclusion

This contribution addressed AI/ML-based CSI feedback enhancements. We have the following observations:

- For UE-based CSI prediction, both the observation window and the prediction window are network configured

- Type 3 monitoring implies that the processing of the performance metric for a model adaptation decision is pursued at the network side, whereas for Type 1 monitoring the UE computes an auxiliary recommendation based on the measured performance metric to assist the network with the model adaptation decision

- Compared with sample-and hold CSI prediction scheme, NMSE for AI/ML-based CSI prediction scheme achieves a 13.11 dB improvement in NMSE and 16.25% improvement in MIMO Layer-1 SGCS gain for a 5ms CSI prediction window

- AI/ML-based CSI prediction achieves marginal gains over auto-regression based CSI prediction scheme with respect to NMSE and MIMO Layer-1 SGCS for a 5ms CSI prediction window

- The performance of the AI/ML-based CSI prediction scheme for a 10ms prediction window, with respect to NMSE and SGCS KPIs, degrades significantly compared with the same scheme for a 5ms prediction window

- System performance evaluations show that with respect to mean UPT, the AI/ML-based CSI prediction achieves 24% and 51% gains over AR-based and sample-and-hold CSI prediction schemes, respectively, over a 5ms CSI prediction window

System performance evaluations show that with respect to 5%-ile UPT over a 5ms CSI prediction window, the AI/ML based CSI prediction achieves:

- Similar performance to AR-based CSI prediction scheme

- 10.5% gain over sample-and-hold CSI prediction scheme

Furthermore, we have the following proposals:

- Study potential configurations of the observation window and the prediction window for UE-based CSI prediction

- The legacy Type-II CSI feedback format is used as the baseline for CSI feedback for AI-based CSI prediction, at least with respect to the configured CSI fields, the spatial domain and frequency domain representation within the PMI

- For UE-based CSI prediction, study potential enhancements of the CSI feedback format for predicted CSI, at least with respect to time-domain representation

- Study the CPU calculation for AI/ML-based CSI report(s) for UE-based CSI prediction

- Study the specification impact corresponding to AI/ML model monitoring, considering the following monitoring decisions: (i) No model change, (ii) CSI parameters update, (iii) Model parameter update, (iv) Model switching, and (v) Fallback to non-AI/ML scheme

- For performance monitoring under UE-based CSI prediction, three reference time instants are considered: (i) at the first slot of the prediction window, (ii) at the median slot of the prediction window, and (iii) at the last slot of the prediction window

- For performance monitoring under UE-based CSI prediction, study and evaluate the pros and cons of the following alternatives:

- Implicit performance monitoring: the UE feeds back CSI measurements based on CSI report quantities or intermediate KPIs that enable the network to derive performance monitoring decisions

- Explicit performance monitoring: the monitoring feedback includes performance monitoring recommendation based on a set of network-configured performance monitoring metrics

- For CSI prediction using a UE side model, adopt the following definition for ‘performance metric’: a parameter computed at the UE based on channel measurements at the UE side, where the performance metric is implicit, i.e., not reported, for Type 1 monitoring, and is explicit, i.e., reported by the UE, for Type 3 monitoring

- For CSI prediction using a UE side model, adopt the following definition for ‘monitoring output’: a parameter comprising a decision (generated at the NW side) or a feedback of a recommended decision (generated at the UE side), based on performance metric(s) computed at the UE side

# References

| [1] | RP-234039, "New WID on AI/ML for NR air interface," Edinburgh, Scotland, Dec. 11-15, 2023. |
| --- | --- |
| [2] | 3GPP TSG RAN WG1#116bis, "Draft Meeting Report," Changsha, China, Apr. 15-19, 2024. |
| [3] | 3GPP TR 38.843, "Study on AI/ML for NR air interface," Dec. 2023. |

# Appendix

| Parameter | Value |
| --- | --- |
| Duplexing scheme | FDD |
| Scenario | Dense Urban |
| Frequency Range | FR1 2GHz |
| Inter-BS distance | 200m |
| Channel model | According to TR 38.901 |
| gNB port layout | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| UE port layout | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power | 41 dBm for 10MHz |
| BS antenna height | 25m |
| UE antenna height & gain | According to TR 36.873 |
| UE receiver noise figure | 9dB |
| Modulation | Up to 256QAM |
| Slot numerology | 14 OFDM symbol slot |
| SCS | 15kHz for 2GHz |
| Simulation bandwidth | 10 MHz |
| MIMO layers | maximum MU layers:12 |
| CSI feedback | Rel-16 codebook - CSI feedback periodicity: 5 ms - CSI feedback delay: 4 ms |
| Traffic model | Full-buffer |
| UE distribution | 100% outdoor (30km/h) |
| UE receiver | MMSE-IRC as the baseline receiver |
| Feedback assumption | Realistic |
| Channel estimation | Ideal channel estimation for dataset construction Realistic channel estimation for system performance evaluation |