3GPP TSG-RAN WG1 Meeting #117	R1-2404996

# Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for UE behavior on monitoring DCI format 3_2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on the agreement made in RAN1#116bis, a UE can be indicated to monitor DCI format 3_2, or DCI format 3_0 and DCI format 3_2, or DCI format 3_0, DCI format 3_1 and DCI format 3_2. But this is not captured in RAN1’s spec yet. Agreement in RAN1#116bis With regards to the dci-FormatsSL and in relation to DCI format 3_2: - Add the following 3 new codepoints to the existing dci-FormatsSL - formats3-2 - formats3-0 and formats3-2 - formats3-0-And-3-1 and formats3-2 - Send an LS to RAN2 with the above agreement - RAN1 is still discussing whether to add a 4th codepoint for formats3-1 and formats3-2 Also, based on our understanding, UE may also be indicated to monitor DCI formats3-1 and formats3-2. Moreover, the IE “dci-FormatsSL” is NOT expandable. we think a new IE (e.g., dci-FormatsSLExt) should be introduced for the newly added codepoints for DCI monitoring: formats3-2, formats3-0 and formats3-2, formats3-1 and formats3-2, formats3-0-And-3-1 and formats3-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture in the spec for UE monitoring DCI format 3_2, or DCI format 3_0 and DCI format 3_2, or DCI format 3_1 and DCI format 3_2, or DCI format 3_0, DCI format 3_1 and DCI format 3_2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The agreement is not captured in the spec and UE’s behavior is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Impact Analysis: No backward compatible issue is expected from the CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

10.1	UE procedure for determining physical downlink control channel assignment

<omitted text>

-	if search space set s is a USS set,

-	an indication by dci-Formats to monitor PDCCH candidates either for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or

-	an indication by dci-FormatsExt to monitor PDCCH candidates for DCI format 0_2 and DCI format 1_2, or for DCI format 0_1, DCI format 1_1, DCI format 0_2, and DCI format 1_2, or

-	an indication by dci-FormatsMC to monitor PDCCH candidates for one or both of DCI format 0_3 and DCI format 1_3, or

-	an indication by dci-FormatsSL to monitor PDCCH candidates for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or for DCI format 3_0, or for DCI format 3_1, or for DCI format 3_0 and DCI format 3_1, on an indication by dci-Format-NCR to monitor PDCCH candidates for DCI format 2_8

-  an indication by [dci-FormatsSLext] to monitor PDCCH candidates for DCI format3_2, or for DCI format 3_0 and DCI format 3_2, or for DCI format 3_1 and DCI format 3_2, or for DCI format 3_0, DCI format 3_1 and DCI format 3_2