3GPP TSG-RAN WG1 Meeting #117 	R1-2405751

Fukuoka, Japan, 20-24 May, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0196 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Rel-18 Multi-carrier enhancements in 38.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MC_enh-Core |  |  |  |  |  | Date: |  |  | 2024-05-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current description in Table 7.3.1.1.4-1 may result in different understanding on the mapping between bit field indexes and the cell sets configured by the higher layer parameter MC-DCI-SetofCellsToAddModList. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update Table 7.3.1.1.4-1 to avoid the potential misunderstanding by using the cell set ID to determine the mapping between bit field indexes and the cell sets configured by the higher layer parameter MC-DCI-SetofCellsToAddModList. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 7.3.1.1.4	Format 0_3

DCI format 0_3 is used for the scheduling of one PUSCH in one cell, or multiple PUSCHs in multiple cells with one PUSCH per cell.

The following information is transmitted by means of the DCI format 0_3 with CRC scrambled by C-RNTI or MCS-C-RNTI:

-	Identifier for DCI formats - 1 bit

-	The value of this bit field is always set to 0, indicating an UL DCI format

-	Scheduled cell set indicator - log2Nset bits, where Nset is the number of cell sets which are configured by higher layer parameter MC-DCI-SetofCellsToAddModList to be respectively scheduled by DCI format 0_3/1_3 from the cell on which this format is carried by PDCCH. If present, this field is used to indicate the scheduled cell set according to Table 7.3.1.1.4-1; otherwise, the scheduled cell set is the cell set configured to be scheduled by DCI format 0_3/1_3 from the cell by higher layer parameter MC-DCI-SetofCellsToAddModList.

-	Scheduled cells indicator - number of bits determined by the following:

-	0 bit if the higher layer parameter scheduledCellComboListDCI-0-3 for the scheduled cell set is not configured;

-	otherwise log2IUL bits indicating the scheduled cells in the scheduled cell set according to Table 7.3.1.1.4-2, where IUL is the number of entries in the higher layer parameter scheduledCellComboListDCI-0-3. If only one entry is configured in the higher layer parameter scheduledCellComboListDCI-0-3, the scheduled cells are the cells configured by higher layer parameter scheduledCellComboListDCI-0-3.

< Unchanged parts are omitted >

Table 7.3.1.1.4-1: Scheduled cell set indicator in DCI format 0_3 and DCI format 1_3

| Bit field mapped to index | Scheduled cell set |
| --- | --- |
| 0 | The cell set with the smallest set ID configured by MC-DCI-SetofCellsToAddModList |
| 1 | The cell set with the 2nd smallest set ID configured by MC-DCI-SetofCellsToAddModList |
| 2 | The cell set with the 3rd smallest set ID configured by MC-DCI-SetofCellsToAddModList, if any |
| 3 | The cell set with the 4th smallest set ID configured by MC-DCI-SetofCellsToAddModList, if any |

Table 7.3.1.1.4-2: Scheduled cells indicator in DCI format 0_3

| Bit field mapped to index | Scheduled cells |
| --- | --- |
| 0 | The cells configured by the 1st entry in scheduledCellComboListDCI-0-3 |
| 1 | The cells configured by the 2nd entry in scheduledCellComboListDCI-0-3 |
| 2 | The cells configured by the 3rd entry in scheduledCellComboListDCI-0-3, if any |
| 3 | The cells configured by the 4th entry in scheduledCellComboListDCI-0-3, if any |
| 4 | The cells configured by the 5th entry in scheduledCellComboListDCI-0-3, if any |
| 5 | The cells configured by the 6th entry in scheduledCellComboListDCI-0-3, if any |
| 6 | The cells configured by the 7th entry in scheduledCellComboListDCI-0-3, if any |
| 7 | The cells configured by the 8th entry in scheduledCellComboListDCI-0-3, if any |
| 8 | The cells configured by the 9th entry in scheduledCellComboListDCI-0-3, if any |
| 9 | The cells configured by the 10th entry in scheduledCellComboListDCI-0-3, if any |
| 10 | The cells configured by the 11th entry in scheduledCellComboListDCI-0-3, if any |
| 11 | The cells configured by the 12th entry in scheduledCellComboListDCI-0-3, if any |
| 12 | The cells configured by the 13th entry in scheduledCellComboListDCI-0-3, if any |
| 13 | The cells configured by the 14th entry in scheduledCellComboListDCI-0-3, if any |
| 14 | The cells configured by the 15th entry in scheduledCellComboListDCI-0-3, if any |
| 15 | The cells configured by the 16th entry in scheduledCellComboListDCI-0-3, if any |

< Unchanged parts are omitted >