3GPP TSG-RAN WG1 Meeting #118	R1-2405850

Maastricht, Netherlands, August 19-August 23, 2024

Agenda Item:	9.4.1.1

Source:	Huawei, HiSilicon

Title:	Evaluation methodology and assumptions for Ambient IoT

Document for:	Discussion and Decision

# Introduction

In the RAN1#116bis and RAN1#117 meeting, the framework of deployment scenarios, LLS assumptions and link budget template have been discussed and agreed.

This contribution firstly addresses the remaining aspects of the RAN design targets for coverage and latency. The rest parts focus on the remaining details of deployment scenarios, and detailed assumptions for link budget calculation and link-level simulations to be used in our evaluation. Based on the proposed evaluation assumptions, we give coverage evaluation results for D1T1-device 1, D1T1-device 2a, D2T2-device 1 and D2T2-device 2a in this paper. Coverage evaluation results for D1T1-device 2b and D2T2-device 2b are expected to be added later.

# RAN design targets on maximum distance and latency

## Maximum distance

In the RAN1#116bis meeting, it is agreed to set different maximum distance targets for different devices, within the range of 10 m to 50 m. The exact values of the maximum distance targets can be determined according to the link budget evaluations for different devices.

(Copied from R1-2403663 [1]) Agreement The maximum distance targets are set separately for device 1, device 2a, device 2b, respectively - FFS detailed values and RAN1 can further decide the target within in the range of 10m to 50m after link budget study. - FFS whether to set different values for different scenarios

In Section 5.7, the coverage evaluation results are summarized for different devices in different scenarios. Based on the results, it is seen the potentially achievable maximum distance obviously varies in different scenarios for the same device. For the sake of appropriate air interface design, different values of the maximum distance target can be set for different scenarios.

Proposal 1: Different values of the maximum distance target are set for different scenarios, which are decided according to the corresponding link budget study.

## Latency

In TR 38.848, the target latency is defined as “one-way end-to-end maximum latency”, which “also includes the query/triggering time”. It should be noted that “the time for charging the Ambient IoT device storage (if present) is not included in the latency”.

(Copied from TR 38.848 [2]) The one-way end-to-end maximum latency targets, as defined in TR 22.840, are: - Longer latency target: 10 seconds - Shorter latency target: 1 second A use case is assigned to a latency target according to TR 22.840. RAN WGs can refine a definition of latency suitable for their work within the above. NOTE: The time for charging the Ambient IoT device storage (if present) is not included in the latency defined above. Time for energy harvesting, charging, etc. is regarded as an implementation issue only. NOTE: the one-way end-to-end maximum latency is assumed to also include query/triggering time.

The Rel-19 study is restricted to the traffic types of indoor inventory (DO-DTT) and command (DT).

- The one-way end-to-end maximum latency of DO-DTT traffic can be regarded as the time from the beginning of the query/triggering message transmission from the base station (BS) or intermediate node to the device, to the end of the successfully received reported message transmission from the device to the basestation or intermediate node. DO-DTT is intended for inventory where the specific device ID is unknown to the basestation or intermediate UE before query/triggering, thus the contention-based access procedure should be included in the latency of DO-DTT inventory.

- The one-way end-to-end maximum latency of DT traffic refers to command use cases, where the command message is transmitted to specific device(s) already known by basestation or intermediate UE. There may or may not be triggering message for the case. Different from DO-DTT inventory, contention-based access is not required for DT command.

Regarding the definition of “successfully received” for the purpose of evaluations of the latency for a single A-IoT device, the corresponding practice in the latency evaluation of NR/LTE can be referred. In the study on self-evaluation towards IMT-2020 submission [3], two types of latency are included, which are the control plane latency and user plane latency. The former “refers to the transition time from a most “battery efficient” state (e.g. Idle state) to the start of continuous data transfer”, which includes the access procedure of NR/LTE. Meanwhile, the user plane latency is “the contribution of the radio network to the time from when the source sends a packet to when the destination receives it”, which considers the one-way latency of a single DL or UL message, including the HARQ retransmission time. As the access procedure has to be taken into account in both the inventory and command latency calculations, it is more reasonable to refer to the principle of the control plane latency evaluation of NR/LTE, which accounts for the time taken for the transmission of the different access messages and the respective processing delay at the UE and gNB. Furthermore, since the control plane latency considers only a single attempt to complete the access procedure [3], it would make sense to carry forward the same concept to consider only the initial transmission for each message during the whole procedure. Hence, for the calculation of latency for a single device, we propose to consider only the first attempt for each R2D or D2R transmission.

Proposal 2: Refine the definition of latency as

- For inventory use case (for DO-DTT traffic type):

  - The time interval between the time that the inventory request is sent from BS/intermediate UE to a A-IoT device and the time that the inventory report is successfully received at BS/intermediate UE from the A-IoT device.

- For command use case (for DT traffic type):

  - The time interval between the time that the DL command is sent from BS/intermediate UE and the time that the command is successfully received at A-IoT device.

- Note: the successfully received is considered as follows

  - Successfully received is associated with a target BLER for the reception of one attempt at inventory for the A-IoT device. Target BLER is 1% or 10%

  - The first attempt is taken into account

- Note: The latency is evaluated for a single A-IoT device.

- Note: Time for energy harvesting is not included in the definition of latency.

For inventory of multiple devices, it is not included in the RAN design targets defined in TR 38.848 and hence, can be used only for optional evaluation purposes. Consequently, no performance assessment is needed on this parameter. Companies can report their respective evaluation method and results, and hence, there is no need to discuss this in further detail. An example of the numerical methodology used to calculate this is provided in Annex C.

Proposal 3: The inventory completion time of multiple Ambient IoT devices is defined as:

- The time a reader successfully completed the reception of inventory information / inventory process for [Z]% of A-IoT devices for a given number of A-IoT devices within corresponding coverage by the reader

- Z = {99%(Mandatory), 90%(Optional)}

# Deployment scenarios for coverage and coexistence evaluation

In the RAN1#116bis meeting, 8 deployment scenarios were discussed and agreed for evaluations, with the detailed spectrum for each scenario to be further studied.

(Copied from R1-2403663 [1]) Agreement The following scenarios are defined, - FFS: which of these scenarios will be evaluated. Scenario; CW Inside/outside topology; Diagram of the scenario; Description of the scenario; Device 1/2a/2b; CW spectrum; D2R spectrum; R2D spectrum. D1T1-A1; CW inside topology; - CW node inside topology 1 - ‘CW’ in CW2D and ‘R2’ in D2R are different - ‘CW’ in CW2D and ‘R1’ in R2D are same - ‘R1’ in R2D and ‘R2’ in D2R are different; Device 1, 2a; Case 1-1 (inside topology, DL) Case 1-2 (inside topology, UL); Same as CW. D1T1-A2; - CW node inside topology 1 - same ‘CW’ and ‘R’ node for CW2D, D2R and R2D; Same as D1T1-A1; Same as CW. D1T1-B; CW outside topology; - CW node outside topology 1 - ‘CW’ in CW2D and ‘R’ in D2R are different - ‘CW’ in CW2D and ‘R’ in R2D are different - ‘R’ in R2D and ‘R’ in D2R are same; Case 1-4 (outside topology, UL); Same as CW. D1T1-C; No CW; - No CW Node.; Device 2b; N/A; UL. D2T2-A1; CW inside topology; - CW node inside topology 2 - ‘CW’ in CW2D and ‘R2’ in D2R are different - ‘CW’ in CW2D and ‘R1’ in R2D are same - ‘R1’ in R2D and ‘R2’ in D2R are different - BS communicates with R1 and R2; Device 1, 2a; Case 2-2 (inside topology, UL); Same as CW. D2T2-A2; - CW node inside topology 2 - same ‘CW’ and ‘R’ node for CW2D, D2R and R2D - BS communicates with R; Same as D2T2-A1; Same as CW. D2T2-B; CW outside topology; - CW node outside topology 2 - ‘CW’ in CW2D and ‘R’ in D2R are different - ‘CW’ in CW2D and ‘R’ in R2D are different - ‘R’ in R2D and ‘R’ in D2R are same - BS communicates with R; Case 2-3 (outside topology, DL) Case 2-4 (outside topology, UL); Same as CW. D2T2-C; No CW; - No CW Node. - BS communicates with R; Device 2b; N/A; FFS. Note: this table is for the case where D2R is in the same spectrum as CW2D..

The layout of basestations / intermediate UEs and devices were also discussed. For D2T2, the distribution of intermediate UEs and devices needs to be further clarified. The CW distribution in both D1T1 and D2T2 also needs further study.

(Copied from R1-2403663 [1]) Agreement The following layout is used for evaluation purpose, - FFS: CW distribution for D1T1-B and D2T2-B Parameter; Assumptions for D1T1; Assumptions for D2T2. Scenario; InF-DH; InH-office; InF-DL. Hall size; 120x60 m; 120 x50 m; 300x150 m. Room height; 10 m; 3m; 10 m. Sectorization; None. BS deployment / Intermediate UE dropping; 18 BSs on a square lattice with spacing D, located D/2 from the walls. - L=120m x W=60m; D=20m - BS height = 8 m; - L=120m x W=50m; - Intermediate UE height = 1.5 m FFS: Intermediate UE dropping; - L=300m x W=150m; - Intermediate UE height = 1.5 m FFS: Intermediate UE dropping. Device distribution; Device Height= 1.5 m AIoT devices drop uniformly distributed over the horizontal area; Device Height= 1.5 m AIoT devices drop uniformly distributed over the horizontal area FFS: which devices are involved in the evaluations; Device Height= 1.5m AIoT devices drop uniformly distributed over the horizontal area FFS: which devices are involved in the evaluations. Device mobility (horizontal plane only); 3 kph; 3 kph; 3 kph.

## Deployment scenario 1 with Topology 1

R2D spectrum

Regarding the spectrum for R2D transmissions in D1T1, downlink spectrum is more reasonable considering the following two aspects.

- From the view of regulation, it is safer for basestation to transmit in FDD downlink spectrum. In the discussions during the RAN1#116 meeting, IAB was mentioned as an example of basestation transmitting in uplink spectrum. As discussed in our paper on carrier-wave characteristics [4], IAB node is regarded as a mobile terminal (MT) when it transmits in the uplink slot of TDD spectrum for the backhaul link, while only TDD band is defined for IAB. Consequently, IAB cannot be regarded as an example to prove the legality of the transmissions from basestation in FDD uplink spectrum.

- Higher permitted transmit power in FDD downlink spectrum is important to improve the link budget of R2D transmission, especially considering the limited receive sensitivity of Device 1. Considering the maximum transmit power of the UHF RFID reader can reach 35 dBm EIRP, basestation transmitting in FDD downlink spectrum is the only way to achieve similar or higher transmit power for R2D transmissions.

Proposal 4: The study assumes FDD downlink spectrum for R2D transmissions in D1T1.

CW2D and D2R spectrum

Regarding the spectrum for CW2D transmission in the case of CW inside Topology 1 (Case 1-1/Case 1-2), the external carrier-wave is transmitted by a basestation due to the constraint of being inside Topology 1. Similar to the R2D transmission from basestation, considering both the potential regulation and the higher permitted transmit power, Ambient IoT basestation is recommended to transmit carrier-wave in FDD downlink spectrum, which is Case 1-1.

Regarding the spectrum for CW2D transmission in the case of CW outside Topology 1 (Case 1-4), carrier-wave node outside Topology 1 is introduced to transmit carrier-wave in uplink spectrum. Correspondingly, the basestation also receives backscattered signal in the uplink spectrum, which benefits its implementation from the perspective of hardware being reused for the uplink receiver.

For Device 1 and Device 2a without a large frequency shifter, the D2R transmission is assumed be in the same carrier as CW2D. With this assumption, the D2R transmission in Case 1-1 is transmitted in the DL spectrum, which, although is not covered in existing regulations, would not cause interference to the other transmissions in the DL spectrum due to the low transmit power of the D2R backscattered transmissions. As discussed in [5], it is suggested to de-prioritized the evaluations for Device 2a with a large frequency shifter, due to the uncertain applicability of the frequency shifter.

Proposal 5: In D1T1, the study assumes the following spectrum for both CW2D and D2R transmission.

- D1T1-A: DL spectrum (Case 1-1)

- D1T1-B: UL spectrum (Case 1-4)

CW distribution

In D1T1-B, the distribution of CW node can be more flexible, and depending on the practical solutions for dedicated scenarios. For example, co-site deployment can be applied between CW node and basestation in the case of sufficient link budget for D2R, while denser deployment can be applied for CW node to achieve higher “CW received power [1E5]” at device in the case of compensating insufficient link budget or achieving higher data rate for D2R. Consequently, it would be better to report the corresponding assumption by companies according to the corresponding preference on detailed deployment.

Proposal 6: In D1T1-B, the CW distribution is reported by companies.

## Deployment scenario 2 with Topology 2

R2D spectrum

Regarding the spectrum for R2D transmission in D2T2, it is safer to assume that the intermediate UE transmits in uplink spectrum from the view of regulations.  To support a R2D receiver both in downlink spectrum for D1T1 and in uplink spectrum for D2T2, the multi-band capability of Ambient IoT devices has to be addressed. In our paper on the characteristics of external carrier-wave [4], it has been observed that there can be a modest impact on device power consumption and complexity to support a broadband or dual-band antenna even by Device 1. It is hence our view that an Ambient IoT device, even device 1, is capable of handling R2D reception in both the downlink and uplink spectrum to cater to both D1T1 and D2T2 respectively.

Proposal 7: The study assumes uplink spectrum for the R2D transmission in D2T2.

CW2D and D2R spectrum

Regarding the spectrum for CW2D transmissions in the case of the CW node being inside Topology 2 (Case 2-2), it is natural for the intermediate UE to transmit carrier-wave in uplink spectrum, considering the existing spectrum regulations.

Regarding the spectrum for CW2D transmissions in the case of the CW node being outside Topology 2 (Case 2-3/Case 2-4), an additional node outside Topology 2 is introduced to transmit the carrier-wave. Considering that the carrier-wave is transmitted in uplink spectrum for Case 2-2, it is reasonable and beneficial to reuse the uplink receiver implementation of the intermediate UE for both cases where the carrier-wave node is inside and outside the topology. Consequently, the carrier-wave node is recommended to also transmit carrier-wave in uplink spectrum. If the external carrier-wave is assumed to be transmitted in downlink spectrum for CW outside Topology 2, as is in Case 2-3, the intermediate UE has to support a different D2R receiver implementation in uplink spectrum for Case 2-2 and in downlink spectrum for Case 2-3, respectively. This would increase the complexity of the intermediate UE, which is not favorable.

Proposal 8: The study assumes UL spectrum for both CW2D and D2R transmission in both D2T2-A and D2T2-B.

Regarding the spectrum for D2R transmissions in D2T2-C, from the perspective of regulations, it is unsuitable to assume the Ambient IoT device to transmit in the FDD DL spectrum. As UL spectrum is assumed for the D2R transmission in D2T2-A and D2T2-B, the D2R transmission in UL spectrum is also beneficial for the harmonized design of the D2R receiver for all the devices.

Proposal 9: The study assumes UL spectrum for the D2R transmission in D2T2-C.

Intermediate UE dropping

Regarding the intermediate UE dropping in D2T2, it is recommended to assume a grid with e.g. 10 m or 20 m between two adjacent intermediate UEs. For a small distance of 10 m between two adjacent intermediate UEs, continuous coverage can be achieved. For a relatively large distance of 20 m, discontinuous coverage is expected, where each intermediate UE only needs to communicates with the devices within its effective coverage.

Proposal 10: The intermediate UEs are assumed to be deployed in a grid with e.g. 10 m /20 m distance between every two adjacent intermediate UEs.

Devices involved in the evaluations

To determine which devices are involved in the evaluations for D2T2, the distance supported by an intermediate UE can be obtained by the link budget calculation firstly. Then the devices within the calculated distance from each intermediate UE will be involved in the evaluations. In other words, the devices out of the coverage of all the intermediate UEs will not be involved in the evaluations.

Proposal 11: The devices within the calculated distance, which is obtained by the corresponding link budget calculation, from each intermediate UE will be involved in the evaluations.

CW distribution

In D2T2-B, the CW distribution can be flexible, which may or may not share the same layout of intermediate UE. It can highly depend on the CW cancellation capability and receiver sensitivity of the intermediate UE. For example, for the intermediate UE with lower CW cancellation capability, larger distance between the UE and the CW node can obtain more spatial isolation, which correspondingly requires less CW interference suppression at the UE. It is recommended to report the CW distribution by companies according to the corresponding understanding on the capability and deployment of the intermediate UE.

Proposal 12: In D2T2-B, the CW distribution is reported by companies.

# Link level simulations

In the post-meeting email discussion after the RAN1#117 meeting, a template of coverage evaluation assumptions in link level simulation was agreed, which is copied in Annex A. This section describes the detailed link-level simulation assumptions for both R2D and D2R link, and the corresponding simulation results.

## LLS assumptions

In the link-level simulations, the assumptions in Table 1 are used to obtain the CNR/SNR thresholds for link budget calculations. Detail LLS assumption for each evaluation case would be reported to the excel template file provided by feature lead.

Table 1: Assumptions for link-level simulation

|  | Parameters |  | Assumptions |
| --- | --- | --- | --- |
|  | R2D/D2R common parameters |  |  |
| [0a] | Carrier frequency |  | 900 MHz (M), 2 GHz (O) |
| [0b] | SCS |  | 15 kHz |
| [0c] | Block structure |  | Blocks as agreed in 9.4.2.3 |
| [0d] | Channel model |  | TDL-A, TDL-D |
| [0e] | Delay spread |  | 30 ns |
| [0f] | Device velocity |  | 3 km/h |
| [0g] | Number of Tx/Rx chains for Ambient IoT device |  | 1 |
| [0h1] | BS | Number of antenna elements | 2 |
| [0h2] |  | Number of TXRUs | 2 |
| [0j1] | Intermediate UE | Number of antenna elements | 1 |
| [0j2] |  | Number of TXRUs | 1 |
| [0m] | Reference data rate |  | R2D: ~30-40 kbps (M=6, only for Budget-Alt2 if used) D2R: ~1 kbps, ~5-7 kbps |
| [0n] | Message size |  | 20 bits, 96 bits, 400 bits |
| [0p] | BLER target |  | 10%, 1% |
| [0q] | Sampling frequency |  | Initial SFO (Sampling Frequency Offset) (Fe): - (M) Randomly select a value from the range of [0.1 ~ 1] * 105 ppm for device 1, - (M) Randomly select a value from the range of [0.1 ~ 1] *104 ppm for device 2, CFO for device 2b: - [100 ppm/1000 ppm, 0.1 ppm/s] |
| [0r] | Device 1/2a/2b |  | Device 1, RF-ED Device 2a, RF-ED Device 2b, RF-ED/IF-ED/ZIF |
|  | R2D specific parameters |  |  |
| [1a] | Transmission bandwidth |  | 180 kHz |
| [1b] | ED bandwidth |  | For evaluations, the value(s) of ED bandwidth is 20 MHz for RF-ED, 1.92MHz for IF and 180kHz for ZIF receiver. |
| [1c] | BB LPF |  | 5-order Butterworth/RC filter with cutoff frequency at half of R2D transmission bandwidth |
| [1d] | Waveform |  | OOK waveform generated by OFDM modulator |
| [1e] | Modulation |  | OOK-4 with M = 6 |
| [1f] | Line code |  | Manchester |
| [1g] | FEC |  | No FEC |
| [1h] | ADC bit width |  | 1-bit for device 1 4-bit for device 2 |
| [1j] | Detection/decoding method for Line code |  | Transition edge detection |
|  | D2R specific parameters |  |  |
| [2a1] | Transmission bandwidth (w.r.t. D2R data rate) |  | DSB 15 kHz with ~1 kbps DSB 150 kHz with ~5-7 kbps |
| [2a2] | [OOK/BPSK/BFSK chip rate] |  | 7.5 kcps for DSB 15 kHz Transmission bandwidth 75 kcps for DSB 150 kHz Transmission bandwidth |
| [2a3] | Receiver bandwidth |  | 18 kHz for DSB 15 kHz transmission bandwidth 180 kHz for DSB 150 kHz transmission bandwidth |
| [2b] | Waveform (CW) |  | Unmodulated single tone |
| [2d] | Modulation |  | OOK |
| [2e] | Line code |  | Manchester encoding |
| [2g] | FEC |  | CC with 1/3 code rate |
| [2h] | ADC bit width |  | 12-bit |
| [2j] | D2R receiver |  | Coherent receiver |
|  | Other assumptions |  |  |
| [3a] | Other assumptions |  | To be reported by company |
| [3b] | Note: Companies to report required SINR/SNR/CINR/CNR according to BLER target. |  |  |

## R2D link simulation results

This section shows the link-level simulation results of the R2D link (See Table 2). Note that for coverage evaluation, these R2D link level simulation results would be only used when Budget-Alt2 is applied (e.g. for Device 2b with IF-ED/ZIF architecture). At this moment, we provide the following results for IF-ED receiver with 1.92MHz ED bandwidth. For other cases devices with RF-ED receiver where Budget-Alt1 is applied, a pre-defined threshold would be used for R2D coverage evaluation and thus no need to refer R2D link simulation results in this section. Since the D2R link simulation results of Device 2b are expected to be added later, the coverage evaluation for Device 2b will be provided later accordingly. Here we provide the following results for information at the time.

Table 2 R2D LLS results for Budget-Alt2: Device 2b with IF-ED receiver

| R2D LLS Case | Scenario | Device | Channel model | Data rate | Chip rate | Message size | BLER target | CNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | D1T1 | 2b | TDL-A | 27.4kbps | 90kcps (M=6) | 20bits | 10% | 14.0 |
| 2 | D1T1 | 2b | TDL-A | 34.6kbps | 90kcps (M=6) | 96bits | 10% | 14.6 |
| 3 | D1T1 | 2b | TDL-A | 39.9kbps | 90kcps (M=6) | 400bits | 10% | 16.0 |
| 4 | D1T1 | 2b | TDL-A | 27.4kbps | 90kcps (M=6) | 20bits | 1% | 26.0 |
| 5 | D1T1 | 2b | TDL-A | 34.6kbps | 90kcps (M=6) | 96bits | 1% | 26.0 |
| 6 | D1T1 | 2b | TDL-A | 39.9kbps | 90kcps (M=6) | 400bits | 1% | 27.0 |
| 7 | D2T2 | 2b | TDL-D | 27.4kbps | 90kcps (M=6) | 20bits | 10% | 8.7 |
| 8 | D2T2 | 2b | TDL-D | 34.6kbps | 90kcps (M=6) | 96bits | 10% | 10.0 |
| 9 | D2T2 | 2b | TDL-D | 39.9kbps | 90kcps (M=6) | 400bits | 10% | 10.2 |
| 10 | D2T2 | 2b | TDL-D | 27.4kbps | 90kcps (M=6) | 20bits | 1% | 13.0 |
| 11 | D2T2 | 2b | TDL-D | 34.6kbps | 90kcps (M=6) | 96bits | 1% | 14.2 |
| 12 | D2T2 | 2b | TDL-D | 39.9kbps | 90kcps (M=6) | 400bits | 1% | 14.2 |

Observation 1: The R2D link performance (M=6, OOK-4) of different message sizes and different data rates for device 2b IF-ED receiver are as follows,

- For BLER 10%

  - From 14.0dB to 16.0dB for D1T1 in TDL-A channel

  - From 8.7dB to 10.2dB for D2T2 in TDL-D channel

- For BLER 1%

  - From 26.0dB to 27.0dB for D1T1 in TDL-A channel

  - From 13.0dB to 14.2dB for D2T2 in TDL-D channel

## D2R link simulation results

This section shows the link-level simulation results of the D2R link (See Table 3 to Table 8). At the time, we provide our simulation results for 900MHz carrier frequency. There are total 72 D2R LLS cases by combination of {2 scenarios, 3 devices, 2 reference data rates, 3 message size, 2 BLER target}. Each D2R LLS case has a unique case number which would be referred by link budget calculations in section 5.

At the time, we provide D2R link simulation results for device 1 and device 2a with coherent receiver (from D2R LLS Case 1 to Case 48). The simulation results for device 2b (D2R LLS Case 49-72) are expected to be provided later.

Table 3 D2R LLS results: D1T1-Device 1, 900MHz

| D2R LLS Case | Scenario | Device | Channel model | Reference data rate | Chip rate | Message size | BLER target | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | D1T1 | 1 | TDL-A | ~1kpbs | 7.5kcps | 20bits | 10% | -5.9 |
| 2 | D1T1 | 1 | TDL-A | ~1kpbs | 7.5kcps | 96bits | 10% | -4.3 |
| 3 | D1T1 | 1 | TDL-A | ~1kpbs | 7.5kcps | 400bits | 10% | -0.3 |
| 4 | D1T1 | 1 | TDL-A | ~5-7 kpbs | 75 kcps | 20bits | 10% | -6.4 |
| 5 | D1T1 | 1 | TDL-A | ~5-7 kpbs | 75 kcps | 96bits | 10% | -6 |
| 6 | D1T1 | 1 | TDL-A | ~5-7 kpbs | 75 kcps | 400bits | 10% | -5.3 |
| 7 | D1T1 | 1 | TDL-A | ~1kpbs | 7.5kcps | 20bits | 1% | -0.4 |
| 8 | D1T1 | 1 | TDL-A | ~1kpbs | 7.5kcps | 96bits | 1% | 3.7 |
| 9 | D1T1 | 1 | TDL-A | ~1kpbs | 7.5kcps | 400bits | 1% | 8.2 |
| 10 | D1T1 | 1 | TDL-A | ~5-7 kpbs | 75 kcps | 20bits | 1% | -1.1 |
| 11 | D1T1 | 1 | TDL-A | ~5-7 kpbs | 75 kcps | 96bits | 1% | -0.2 |
| 12 | D1T1 | 1 | TDL-A | ~5-7 kpbs | 75 kcps | 400bits | 1% | 0.4 |

Table 4 D2R LLS results: D2T2-Device 1, 900MHz

| D2R LLS Case | Scenario | Device | Channel model | Reference data rate | Chip rate | Message size | BLER target | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | D2T2 | 1 | TDL-D | ~1kpbs | 7.5kcps | 20bits | 10% | -5.7 |
| 14 | D2T2 | 1 | TDL-D | ~1kpbs | 7.5kcps | 96bits | 10% | -5 |
| 15 | D2T2 | 1 | TDL-D | ~1kpbs | 7.5kcps | 400bits | 10% | -3.5 |
| 16 | D2T2 | 1 | TDL-D | ~5-7 kpbs | 75 kcps | 20bits | 10% | -6 |
| 17 | D2T2 | 1 | TDL-D | ~5-7 kpbs | 75 kcps | 96bits | 10% | -5.9 |
| 18 | D2T2 | 1 | TDL-D | ~5-7 kpbs | 75 kcps | 400bits | 10% | -5.4 |
| 19 | D2T2 | 1 | TDL-D | ~1kpbs | 7.5kcps | 20bits | 1% | -1.7 |
| 20 | D2T2 | 1 | TDL-D | ~1kpbs | 7.5kcps | 96bits | 1% | -0.9 |
| 21 | D2T2 | 1 | TDL-D | ~1kpbs | 7.5kcps | 400bits | 1% | 1.1 |
| 22 | D2T2 | 1 | TDL-D | ~5-7 kpbs | 75 kcps | 20bits | 1% | -1.8 |
| 23 | D2T2 | 1 | TDL-D | ~5-7 kpbs | 75 kcps | 96bits | 1% | -1.9 |
| 24 | D2T2 | 1 | TDL-D | ~5-7 kpbs | 75 kcps | 400bits | 1% | -1 |

Table 5 D2R LLS results: D1T1-Device 2a, 900MHz

| D2R LLS Case | Scenario | Device | Channel model | Reference data rate | Chip rate | Message size | BLER target | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25 | D1T1 | 2a | TDL-A | ~1kpbs | 7.5kcps | 20bits | 10% | -7.1 |
| 26 | D1T1 | 2a | TDL-A | ~1kpbs | 7.5kcps | 96bits | 10% | -4.7 |
| 27 | D1T1 | 2a | TDL-A | ~1kpbs | 7.5kcps | 400bits | 10% | -0.5 |
| 28 | D1T1 | 2a | TDL-A | ~5-7 kpbs | 75 kcps | 20bits | 10% | -8 |
| 29 | D1T1 | 2a | TDL-A | ~5-7 kpbs | 75 kcps | 96bits | 10% | -7.5 |
| 30 | D1T1 | 2a | TDL-A | ~5-7 kpbs | 75 kcps | 400bits | 10% | -6.3 |
| 31 | D1T1 | 2a | TDL-A | ~1kpbs | 7.5kcps | 20bits | 1% | -1.3 |
| 32 | D1T1 | 2a | TDL-A | ~1kpbs | 7.5kcps | 96bits | 1% | 4.6 |
| 33 | D1T1 | 2a | TDL-A | ~1kpbs | 7.5kcps | 400bits | 1% | 13 |
| 34 | D1T1 | 2a | TDL-A | ~5-7 kpbs | 75 kcps | 20bits | 1% | -2.5 |
| 35 | D1T1 | 2a | TDL-A | ~5-7 kpbs | 75 kcps | 96bits | 1% | -1.8 |
| 36 | D1T1 | 2a | TDL-A | ~5-7 kpbs | 75 kcps | 400bits | 1% | -1 |

Table 6 D2R LLS results: D2T2-Device 2a, 900MHz

| D2R LLS Case | Scenario | Device | Channel model | Reference data rate | Chip rate | Message size | BLER target | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 37 | D2T2 | 2a | TDL-D | ~1kpbs | 7.5kcps | 20bits | 10% | -6.8 |
| 38 | D2T2 | 2a | TDL-D | ~1kpbs | 7.5kcps | 96bits | 10% | -5.7 |
| 39 | D2T2 | 2a | TDL-D | ~1kpbs | 7.5kcps | 400bits | 10% | -3.8 |
| 40 | D2T2 | 2a | TDL-D | ~5-7 kpbs | 75 kcps | 20bits | 10% | -7.7 |
| 41 | D2T2 | 2a | TDL-D | ~5-7 kpbs | 75 kcps | 96bits | 10% | -7.2 |
| 42 | D2T2 | 2a | TDL-D | ~5-7 kpbs | 75 kcps | 400bits | 10% | -6.2 |
| 43 | D2T2 | 2a | TDL-D | ~1kpbs | 7.5kcps | 20bits | 1% | -2.6 |
| 44 | D2T2 | 2a | TDL-D | ~1kpbs | 7.5kcps | 96bits | 1% | -1.5 |
| 45 | D2T2 | 2a | TDL-D | ~1kpbs | 7.5kcps | 400bits | 1% | 0.9 |
| 46 | D2T2 | 2a | TDL-D | ~5-7 kpbs | 75 kcps | 20bits | 1% | -3.7 |
| 47 | D2T2 | 2a | TDL-D | ~5-7 kpbs | 75 kcps | 96bits | 1% | -3.3 |
| 48 | D2T2 | 2a | TDL-D | ~5-7 kpbs | 75 kcps | 400bits | 1% | -1.9 |

Table 7 D2R LLS results: D1T1-Device 2b, 900MHz

| D2R LLS Case | Scenario | Device | Channel model | Reference data rate | Chip rate | Message size | BLER target | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49 | D1T1 | 2b | TDL-A | ~1kpbs | 7.5kcps | 20bits | 10% |  |
| 50 | D1T1 | 2b | TDL-A | ~1kpbs | 7.5kcps | 96bits | 10% |  |
| 51 | D1T1 | 2b | TDL-A | ~1kpbs | 7.5kcps | 400bits | 10% |  |
| 52 | D1T1 | 2b | TDL-A | ~5-7 kpbs | 75 kcps | 20bits | 10% |  |
| 53 | D1T1 | 2b | TDL-A | ~5-7 kpbs | 75 kcps | 96bits | 10% |  |
| 54 | D1T1 | 2b | TDL-A | ~5-7 kpbs | 75 kcps | 400bits | 10% |  |
| 55 | D1T1 | 2b | TDL-A | ~1kpbs | 7.5kcps | 20bits | 1% |  |
| 56 | D1T1 | 2b | TDL-A | ~1kpbs | 7.5kcps | 96bits | 1% |  |
| 57 | D1T1 | 2b | TDL-A | ~1kpbs | 7.5kcps | 400bits | 1% |  |
| 58 | D1T1 | 2b | TDL-A | ~5-7 kpbs | 75 kcps | 20bits | 1% |  |
| 59 | D1T1 | 2b | TDL-A | ~5-7 kpbs | 75 kcps | 96bits | 1% |  |
| 60 | D1T1 | 2b | TDL-A | ~5-7 kpbs | 75 kcps | 400bits | 1% |  |

Table 8 D2R LLS results: D2T2-Device 2b, 900MHz

| D2R LLS Case | Scenario | Device | Channel model | Data rate | Chip rate | Message size | BLER target | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 61 | D2T2 | 2b | TDL-D | ~1kpbs | 7.5kcps | 20bits | 10% |  |
| 62 | D2T2 | 2b | TDL-D | ~1kpbs | 7.5kcps | 96bits | 10% |  |
| 63 | D2T2 | 2b | TDL-D | ~1kpbs | 7.5kcps | 400bits | 10% |  |
| 64 | D2T2 | 2b | TDL-D | ~5-7 kpbs | 75 kcps | 20bits | 10% |  |
| 65 | D2T2 | 2b | TDL-D | ~5-7 kpbs | 75 kcps | 96bits | 10% |  |
| 66 | D2T2 | 2b | TDL-D | ~5-7 kpbs | 75 kcps | 400bits | 10% |  |
| 67 | D2T2 | 2b | TDL-D | ~1kpbs | 7.5kcps | 20bits | 1% |  |
| 68 | D2T2 | 2b | TDL-D | ~1kpbs | 7.5kcps | 96bits | 1% |  |
| 69 | D2T2 | 2b | TDL-D | ~1kpbs | 7.5kcps | 400bits | 1% |  |
| 70 | D2T2 | 2b | TDL-D | ~5-7 kpbs | 75 kcps | 20bits | 1% |  |
| 71 | D2T2 | 2b | TDL-D | ~5-7 kpbs | 75 kcps | 96bits | 1% |  |
| 72 | D2T2 | 2b | TDL-D | ~5-7 kpbs | 75 kcps | 400bits | 1% |  |

Observation 2: The D2R link performance of different message sizes and different data rates for device 1 are as follows,

- For BLER 10%

  - From -6.4dB to 0.3dB for D1T1 in TDL-A channel

  - From -6dB to – 3.5dB for D2T2 in TDL-D channel

- For BLER 1%

  - From -1.1dB to 8.2dB for D1T1 in TDL-A channel

  - From -1.9dB to 1.1dB for D2T2 in TDL-D channel

Observation 3: The D2R link performance of different message sizes and different data rates for device 2a are as follows,

- For BLER 10%

  - From -8dB to -0.5dB for D1T1 in TDL-A channel

  - From -7.7dB to -3.8dB for D2T2 in TDL-D channel

- For BLER 1%

  - From -2.5dB to 13dB for D1T1 in TDL-A channel

  - From -3.7dB to 0.9dB for D2T2 in TDL-D channel

# Link budget calculations

Based on the assumptions for link-level simulations and the LLS results in section 4, this section 5 summarize the link budget results and the corresponding maximum distance for different devices in different deployment scenarios based on the agreed template.

## Link budget for Device 1 in D1T1

The link budgets and the corresponding maximum distances for the R2D and D2R link of Device 1 in the D1T1-A1/A2/B scenario are summarized in Table 9. Detail assumptions and calculations for each evaluation case would be reported to the excel template file provided by feature lead.

Table 9: MPL and maximum distance: D1T1-Device 1, 900MHz

| Case | Scenario | CW case | Device | R2D EIRP | D2R LLS case | R2D MPL (dB) | D2R MPL (dB) | Min MPL (dB) | Min Distance (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | D1T1-A1 | 1-1 | 1 | 35 | 1 | 63.1 | 66.95 | 63.10 | 24.40 |
| 2 | D1T1-A2 | 1-1 | 1 | 35 | 1 | 63.1 | 62.04 | 62.04 | 21.83 |
| 3 | D1T1-B | 1-4 | 1 | 35 | 1 | 63.1 | 66.24 | 63.10 | 24.40 |
| 4 | D1T1-A1 | 1-1 | 1 | 35 | 2 | 63.1 | 66.15 | 63.10 | 24.40 |
| 5 | D1T1-A2 | 1-1 | 1 | 35 | 2 | 63.1 | 61.24 | 61.24 | 20.07 |
| 6 | D1T1-B | 1-4 | 1 | 35 | 2 | 63.1 | 65.44 | 63.10 | 24.40 |
| 7 | D1T1-A1 | 1-1 | 1 | 35 | 3 | 63.1 | 64.15 | 63.10 | 24.40 |
| 8 | D1T1-A2 | 1-1 | 1 | 35 | 3 | 63.1 | 59.24 | 59.24 | 16.26 |
| 9 | D1T1-B | 1-4 | 1 | 35 | 3 | 63.1 | 63.44 | 63.10 | 24.40 |
| 10 | D1T1-A1 | 1-1 | 1 | 35 | 4 | 63.1 | 66.49 | 63.10 | 24.40 |
| 11 | D1T1-A2 | 1-1 | 1 | 35 | 4 | 63.1 | 62.20 | 62.20 | 22.21 |
| 12 | D1T1-B | 1-4 | 1 | 35 | 4 | 63.1 | 63.59 | 63.10 | 24.40 |
| 13 | D1T1-A1 | 1-1 | 1 | 35 | 5 | 63.1 | 66.29 | 63.10 | 24.40 |
| 14 | D1T1-A2 | 1-1 | 1 | 35 | 5 | 63.1 | 62.00 | 62.00 | 21.75 |
| 15 | D1T1-B | 1-4 | 1 | 35 | 5 | 63.1 | 63.39 | 63.10 | 24.40 |
| 16 | D1T1-A1 | 1-1 | 1 | 35 | 6 | 63.1 | 65.94 | 63.10 | 24.40 |
| 17 | D1T1-A2 | 1-1 | 1 | 35 | 6 | 63.1 | 61.65 | 61.65 | 20.96 |
| 18 | D1T1-B | 1-4 | 1 | 35 | 6 | 63.1 | 63.04 | 63.04 | 24.25 |
| 19 | D1T1-A1 | 1-1 | 1 | 35 | 7 | 63.1 | 64.20 | 63.10 | 24.40 |
| 20 | D1T1-A2 | 1-1 | 1 | 35 | 7 | 63.1 | 59.29 | 59.29 | 16.35 |
| 21 | D1T1-B | 1-4 | 1 | 35 | 7 | 63.1 | 63.49 | 63.10 | 24.40 |
| 22 | D1T1-A1 | 1-1 | 1 | 35 | 8 | 63.1 | 62.15 | 62.15 | 22.09 |
| 23 | D1T1-A2 | 1-1 | 1 | 35 | 8 | 63.1 | 57.24 | 57.24 | 13.18 |
| 24 | D1T1-B | 1-4 | 1 | 35 | 8 | 63.1 | 61.44 | 61.44 | 20.50 |
| 25 | D1T1-A1 | 1-1 | 1 | 35 | 9 | 63.1 | 59.90 | 59.90 | 17.44 |
| 26 | D1T1-A2 | 1-1 | 1 | 35 | 9 | 63.1 | 54.99 | 54.99 | 10.40 |
| 27 | D1T1-B | 1-4 | 1 | 35 | 9 | 63.1 | 59.19 | 59.19 | 16.18 |
| 28 | D1T1-A1 | 1-1 | 1 | 35 | 10 | 63.1 | 63.84 | 63.10 | 24.40 |
| 29 | D1T1-A2 | 1-1 | 1 | 35 | 10 | 63.1 | 59.55 | 59.55 | 16.81 |
| 30 | D1T1-B | 1-4 | 1 | 35 | 10 | 63.1 | 60.94 | 60.94 | 19.45 |
| 31 | D1T1-A1 | 1-1 | 1 | 35 | 11 | 63.1 | 63.39 | 63.10 | 24.40 |
| 32 | D1T1-A2 | 1-1 | 1 | 35 | 11 | 63.1 | 59.10 | 59.10 | 16.03 |
| 33 | D1T1-B | 1-4 | 1 | 35 | 11 | 63.1 | 60.49 | 60.49 | 18.55 |
| 34 | D1T1-A1 | 1-1 | 1 | 35 | 12 | 63.1 | 63.09 | 63.09 | 24.38 |
| 35 | D1T1-A2 | 1-1 | 1 | 35 | 12 | 63.1 | 58.80 | 58.80 | 15.53 |
| 36 | D1T1-B | 1-4 | 1 | 35 | 12 | 63.1 | 60.19 | 60.19 | 17.97 |
| 37 | D1T1-A1 | 1-1 | 1 | 30 | 1 | 58.1 | 66.93 | 58.10 | 14.43 |
| 38 | D1T1-A2 | 1-1 | 1 | 30 | 1 | 58.1 | 62.04 | 58.10 | 14.43 |
| 39 | D1T1-B | 1-4 | 1 | 30 | 1 | 58.1 | 66.90 | 58.10 | 14.43 |
| 40 | D1T1-A1 | 1-1 | 1 | 30 | 2 | 58.1 | 66.13 | 58.10 | 14.43 |
| 41 | D1T1-A2 | 1-1 | 1 | 30 | 2 | 58.1 | 61.24 | 58.10 | 14.43 |
| 42 | D1T1-B | 1-4 | 1 | 30 | 2 | 58.1 | 66.10 | 58.10 | 14.43 |
| 43 | D1T1-A1 | 1-1 | 1 | 30 | 3 | 58.1 | 64.13 | 58.10 | 14.43 |
| 44 | D1T1-A2 | 1-1 | 1 | 30 | 3 | 58.1 | 59.24 | 58.10 | 14.43 |
| 45 | D1T1-B | 1-4 | 1 | 30 | 3 | 58.1 | 64.10 | 58.10 | 14.43 |
| 46 | D1T1-A1 | 1-1 | 1 | 30 | 4 | 58.1 | 66.32 | 58.10 | 14.43 |
| 47 | D1T1-A2 | 1-1 | 1 | 30 | 4 | 58.1 | 62.18 | 58.10 | 14.43 |
| 48 | D1T1-B | 1-4 | 1 | 30 | 4 | 58.1 | 66.13 | 58.10 | 14.43 |
| 49 | D1T1-A1 | 1-1 | 1 | 30 | 5 | 58.1 | 66.12 | 58.10 | 14.43 |
| 50 | D1T1-A2 | 1-1 | 1 | 30 | 5 | 58.1 | 61.98 | 58.10 | 14.43 |
| 51 | D1T1-B | 1-4 | 1 | 30 | 5 | 58.1 | 65.93 | 58.10 | 14.43 |
| 52 | D1T1-A1 | 1-1 | 1 | 30 | 6 | 58.1 | 65.77 | 58.10 | 14.43 |
| 53 | D1T1-A2 | 1-1 | 1 | 30 | 6 | 58.1 | 61.63 | 58.10 | 14.43 |
| 54 | D1T1-B | 1-4 | 1 | 30 | 6 | 58.1 | 65.58 | 58.10 | 14.43 |
| 55 | D1T1-A1 | 1-1 | 1 | 30 | 7 | 58.1 | 64.18 | 58.10 | 14.43 |
| 56 | D1T1-A2 | 1-1 | 1 | 30 | 7 | 58.1 | 59.29 | 58.10 | 14.43 |
| 57 | D1T1-B | 1-4 | 1 | 30 | 7 | 58.1 | 64.15 | 58.10 | 14.43 |
| 58 | D1T1-A1 | 1-1 | 1 | 30 | 8 | 58.1 | 62.13 | 58.10 | 14.43 |
| 59 | D1T1-A2 | 1-1 | 1 | 30 | 8 | 58.1 | 57.24 | 57.24 | 13.18 |
| 60 | D1T1-B | 1-4 | 1 | 30 | 8 | 58.1 | 62.10 | 58.10 | 14.43 |
| 61 | D1T1-A1 | 1-1 | 1 | 30 | 9 | 58.1 | 59.88 | 58.10 | 14.43 |
| 62 | D1T1-A2 | 1-1 | 1 | 30 | 9 | 58.1 | 54.99 | 54.99 | 10.40 |
| 63 | D1T1-B | 1-4 | 1 | 30 | 9 | 58.1 | 59.85 | 58.10 | 14.43 |
| 64 | D1T1-A1 | 1-1 | 1 | 30 | 10 | 58.1 | 63.67 | 58.10 | 14.43 |
| 65 | D1T1-A2 | 1-1 | 1 | 30 | 10 | 58.1 | 59.53 | 58.10 | 14.43 |
| 66 | D1T1-B | 1-4 | 1 | 30 | 10 | 58.1 | 63.48 | 58.10 | 14.43 |
| 67 | D1T1-A1 | 1-1 | 1 | 30 | 11 | 58.1 | 63.22 | 58.10 | 14.43 |
| 68 | D1T1-A2 | 1-1 | 1 | 30 | 11 | 58.1 | 59.08 | 58.10 | 14.43 |
| 69 | D1T1-B | 1-4 | 1 | 30 | 11 | 58.1 | 63.03 | 58.10 | 14.43 |
| 70 | D1T1-A1 | 1-1 | 1 | 30 | 12 | 58.1 | 62.92 | 58.10 | 14.43 |
| 71 | D1T1-A2 | 1-1 | 1 | 30 | 12 | 58.1 | 58.78 | 58.10 | 14.43 |
| 72 | D1T1-B | 1-4 | 1 | 30 | 12 | 58.1 | 62.73 | 58.10 | 14.43 |

## Link budget for Device 2a in D1T1

The link budgets and the corresponding maximum distances for the R2D and D2R link of Device 2a in the D1T1-A1/A2/B scenario are summarized in Table 10. Detail assumptions and calculations for each evaluation case would be reported to the excel template file provided by feature lead.

Table 10: MPL and maximum distance: D1T1-Device 2a, 900MHz

| Case | Scenario | CW case | Device | R2D EIRP | D2R LLS case | R2D MPL (dB) | D2R MPL (dB) | Min MPL (dB) | Min Distance (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | D1T1-A1 | 1-1 | 2a | 35 | 25 | 72.1 | 76.84 | 72.10 | 62.86 |
| 2 | D1T1-A2 | 1-1 | 2a | 35 | 25 | 72.1 | 67.64 | 67.64 | 39.33 |
| 3 | D1T1-B | 1-4 | 2a | 35 | 25 | 72.1 | 73.94 | 72.10 | 62.86 |
| 4 | D1T1-A1 | 1-1 | 2a | 35 | 26 | 72.1 | 75.64 | 72.10 | 62.86 |
| 5 | D1T1-A2 | 1-1 | 2a | 35 | 26 | 72.1 | 66.44 | 66.44 | 34.67 |
| 6 | D1T1-B | 1-4 | 2a | 35 | 26 | 72.1 | 72.74 | 72.10 | 62.86 |
| 7 | D1T1-A1 | 1-1 | 2a | 35 | 27 | 72.1 | 73.54 | 72.10 | 62.86 |
| 8 | D1T1-A2 | 1-1 | 2a | 35 | 27 | 72.1 | 64.34 | 64.34 | 27.80 |
| 9 | D1T1-B | 1-4 | 2a | 35 | 27 | 72.1 | 70.64 | 70.64 | 53.92 |
| 10 | D1T1-A1 | 1-1 | 2a | 35 | 28 | 72.1 | 74.39 | 72.10 | 62.86 |
| 11 | D1T1-A2 | 1-1 | 2a | 35 | 28 | 72.1 | 68.00 | 68.00 | 40.87 |
| 12 | D1T1-B | 1-4 | 2a | 35 | 28 | 72.1 | 69.78 | 69.78 | 49.24 |
| 13 | D1T1-A1 | 1-1 | 2a | 35 | 29 | 72.1 | 74.14 | 72.10 | 62.86 |
| 14 | D1T1-A2 | 1-1 | 2a | 35 | 29 | 72.1 | 67.75 | 67.75 | 39.81 |
| 15 | D1T1-B | 1-4 | 2a | 35 | 29 | 72.1 | 69.53 | 69.53 | 47.96 |
| 16 | D1T1-A1 | 1-1 | 2a | 35 | 30 | 72.1 | 73.54 | 72.10 | 62.86 |
| 17 | D1T1-A2 | 1-1 | 2a | 35 | 30 | 72.1 | 67.15 | 67.15 | 37.37 |
| 18 | D1T1-B | 1-4 | 2a | 35 | 30 | 72.1 | 68.93 | 68.93 | 45.03 |
| 19 | D1T1-A1 | 1-1 | 2a | 35 | 31 | 72.1 | 73.94 | 72.10 | 62.86 |
| 20 | D1T1-A2 | 1-1 | 2a | 35 | 31 | 72.1 | 64.74 | 64.74 | 29.00 |
| 21 | D1T1-B | 1-4 | 2a | 35 | 31 | 72.1 | 71.04 | 71.04 | 56.23 |
| 22 | D1T1-A1 | 1-1 | 2a | 35 | 32 | 72.1 | 70.99 | 70.99 | 55.94 |
| 23 | D1T1-A2 | 1-1 | 2a | 35 | 32 | 72.1 | 61.79 | 61.79 | 21.26 |
| 24 | D1T1-B | 1-4 | 2a | 35 | 32 | 72.1 | 68.09 | 68.09 | 41.24 |
| 25 | D1T1-A1 | 1-1 | 2a | 35 | 33 | 72.1 | 66.79 | 66.79 | 35.97 |
| 26 | D1T1-A2 | 1-1 | 2a | 35 | 33 | 72.1 | 57.59 | 57.59 | 13.67 |
| 27 | D1T1-B | 1-4 | 2a | 35 | 33 | 72.1 | 63.89 | 63.89 | 26.52 |
| 28 | D1T1-A1 | 1-1 | 2a | 35 | 34 | 72.1 | 71.64 | 71.64 | 59.90 |
| 29 | D1T1-A2 | 1-1 | 2a | 35 | 34 | 72.1 | 65.25 | 65.25 | 30.61 |
| 30 | D1T1-B | 1-4 | 2a | 35 | 34 | 72.1 | 67.03 | 67.03 | 36.87 |
| 31 | D1T1-A1 | 1-1 | 2a | 35 | 35 | 72.1 | 71.29 | 71.29 | 57.73 |
| 32 | D1T1-A2 | 1-1 | 2a | 35 | 35 | 72.1 | 64.90 | 64.90 | 29.50 |
| 33 | D1T1-B | 1-4 | 2a | 35 | 35 | 72.1 | 66.68 | 66.68 | 35.54 |
| 34 | D1T1-A1 | 1-1 | 2a | 35 | 36 | 72.1 | 70.89 | 70.89 | 55.35 |
| 35 | D1T1-A2 | 1-1 | 2a | 35 | 36 | 72.1 | 64.50 | 64.50 | 28.29 |
| 36 | D1T1-B | 1-4 | 2a | 35 | 36 | 72.1 | 66.28 | 66.28 | 34.08 |
| 37 | D1T1-A1 | 1-1 | 2a | 30 | 25 | 67.1 | 76.67 | 67.10 | 37.16 |
| 38 | D1T1-A2 | 1-1 | 2a | 30 | 25 | 67.1 | 67.64 | 67.10 | 37.16 |
| 39 | D1T1-B | 1-4 | 2a | 30 | 25 | 67.1 | 76.48 | 67.10 | 37.16 |
| 40 | D1T1-A1 | 1-1 | 2a | 30 | 26 | 67.1 | 75.47 | 67.10 | 37.16 |
| 41 | D1T1-A2 | 1-1 | 2a | 30 | 26 | 67.1 | 66.44 | 66.44 | 34.66 |
| 42 | D1T1-B | 1-4 | 2a | 30 | 26 | 67.1 | 75.28 | 67.10 | 37.16 |
| 43 | D1T1-A1 | 1-1 | 2a | 30 | 27 | 67.1 | 73.37 | 67.10 | 37.16 |
| 44 | D1T1-A2 | 1-1 | 2a | 30 | 27 | 67.1 | 64.34 | 64.34 | 27.79 |
| 45 | D1T1-B | 1-4 | 2a | 30 | 27 | 67.1 | 73.18 | 67.10 | 37.16 |
| 46 | D1T1-A1 | 1-1 | 2a | 30 | 28 | 67.1 | 73.97 | 67.10 | 37.16 |
| 47 | D1T1-A2 | 1-1 | 2a | 30 | 28 | 67.1 | 67.98 | 67.10 | 37.16 |
| 48 | D1T1-B | 1-4 | 2a | 30 | 28 | 67.1 | 73.54 | 67.10 | 37.16 |
| 49 | D1T1-A1 | 1-1 | 2a | 30 | 29 | 67.1 | 73.72 | 67.10 | 37.16 |
| 50 | D1T1-A2 | 1-1 | 2a | 30 | 29 | 67.1 | 67.73 | 67.10 | 37.16 |
| 51 | D1T1-B | 1-4 | 2a | 30 | 29 | 67.1 | 73.29 | 67.10 | 37.16 |
| 52 | D1T1-A1 | 1-1 | 2a | 30 | 30 | 67.1 | 73.12 | 67.10 | 37.16 |
| 53 | D1T1-A2 | 1-1 | 2a | 30 | 30 | 67.1 | 67.13 | 67.10 | 37.16 |
| 54 | D1T1-B | 1-4 | 2a | 30 | 30 | 67.1 | 72.69 | 67.10 | 37.16 |
| 55 | D1T1-A1 | 1-1 | 2a | 30 | 31 | 67.1 | 73.77 | 67.10 | 37.16 |
| 56 | D1T1-A2 | 1-1 | 2a | 30 | 31 | 67.1 | 64.74 | 64.74 | 28.99 |
| 57 | D1T1-B | 1-4 | 2a | 30 | 31 | 67.1 | 73.58 | 67.10 | 37.16 |
| 58 | D1T1-A1 | 1-1 | 2a | 30 | 32 | 67.1 | 70.82 | 67.10 | 37.16 |
| 59 | D1T1-A2 | 1-1 | 2a | 30 | 32 | 67.1 | 61.79 | 61.79 | 21.26 |
| 60 | D1T1-B | 1-4 | 2a | 30 | 32 | 67.1 | 70.63 | 67.10 | 37.16 |
| 61 | D1T1-A1 | 1-1 | 2a | 30 | 33 | 67.1 | 66.62 | 66.62 | 35.34 |
| 62 | D1T1-A2 | 1-1 | 2a | 30 | 33 | 67.1 | 57.59 | 57.59 | 13.67 |
| 63 | D1T1-B | 1-4 | 2a | 30 | 33 | 67.1 | 66.43 | 66.43 | 34.62 |
| 64 | D1T1-A1 | 1-1 | 2a | 30 | 34 | 67.1 | 71.22 | 67.10 | 37.16 |
| 65 | D1T1-A2 | 1-1 | 2a | 30 | 34 | 67.1 | 65.23 | 65.23 | 30.53 |
| 66 | D1T1-B | 1-4 | 2a | 30 | 34 | 67.1 | 70.79 | 67.10 | 37.16 |
| 67 | D1T1-A1 | 1-1 | 2a | 30 | 35 | 67.1 | 70.87 | 67.10 | 37.16 |
| 68 | D1T1-A2 | 1-1 | 2a | 30 | 35 | 67.1 | 64.88 | 64.88 | 29.43 |
| 69 | D1T1-B | 1-4 | 2a | 30 | 35 | 67.1 | 70.44 | 67.10 | 37.16 |
| 70 | D1T1-A1 | 1-1 | 2a | 30 | 36 | 67.1 | 70.47 | 67.10 | 37.16 |
| 71 | D1T1-A2 | 1-1 | 2a | 30 | 36 | 67.1 | 64.48 | 64.48 | 28.21 |
| 72 | D1T1-B | 1-4 | 2a | 30 | 36 | 67.1 | 70.04 | 67.10 | 37.16 |

## Link budget for Device 2b in D1T1

The link budgets and the corresponding maximum distances for the R2D and D2R link of Device 2b in the D1T1-C scenario are expected to be summarized in Table 11 later.

Table 11: MPL and maximum distance: D1T1-Device 2b, 900MHz

| Case | Scenario | CW case | Device | R2D EIRP | D2R LLS case | R2D MPL (dB) | D2R MPL (dB) | Min MPL (dB) | Min Distance (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | D1T1-C | N/A | 2b | 35 | 49 |  |  |  |  |
| 2 | D1T1-C | N/A | 2b | 35 | 50 |  |  |  |  |
| 3 | D1T1-C | N/A | 2b | 35 | 51 |  |  |  |  |
| 4 | D1T1-C | N/A | 2b | 35 | 52 |  |  |  |  |
| 5 | D1T1-C | N/A | 2b | 35 | 53 |  |  |  |  |
| 6 | D1T1-C | N/A | 2b | 35 | 54 |  |  |  |  |
| 7 | D1T1-C | N/A | 2b | 35 | 55 |  |  |  |  |
| 8 | D1T1-C | N/A | 2b | 35 | 56 |  |  |  |  |
| 9 | D1T1-C | N/A | 2b | 35 | 57 |  |  |  |  |
| 10 | D1T1-C | N/A | 2b | 35 | 58 |  |  |  |  |
| 11 | D1T1-C | N/A | 2b | 35 | 59 |  |  |  |  |
| 12 | D1T1-C | N/A | 2b | 35 | 60 |  |  |  |  |
| 13 | D1T1-C | N/A | 2b | 30 | 49 |  |  |  |  |
| 14 | D1T1-C | N/A | 2b | 30 | 50 |  |  |  |  |
| 15 | D1T1-C | N/A | 2b | 30 | 51 |  |  |  |  |
| 16 | D1T1-C | N/A | 2b | 30 | 52 |  |  |  |  |
| 17 | D1T1-C | N/A | 2b | 30 | 53 |  |  |  |  |
| 18 | D1T1-C | N/A | 2b | 30 | 54 |  |  |  |  |
| 19 | D1T1-C | N/A | 2b | 30 | 55 |  |  |  |  |
| 20 | D1T1-C | N/A | 2b | 30 | 56 |  |  |  |  |
| 21 | D1T1-C | N/A | 2b | 30 | 57 |  |  |  |  |
| 22 | D1T1-C | N/A | 2b | 30 | 58 |  |  |  |  |
| 23 | D1T1-C | N/A | 2b | 30 | 59 |  |  |  |  |
| 24 | D1T1-C | N/A | 2b | 30 | 60 |  |  |  |  |

## Link budget for Device 1 in D2T2

The link budgets and the corresponding maximum distances for the R2D and D2R link of Device 1 in the D2T2-A1/A2/B scenario are summarized in Table 12. Detail assumptions and calculations for each evaluation case would be reported to the excel template file provided by feature lead.

Table 12: MPL and maximum distance: D2T2-Device 1, 900MHz

| Case | Scenario | CW case | Device | R2D EIRP | D2R LLS case | R2D MPL (dB) | D2R MPL (dB) | Min MPL (dB) | Min Distance (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | D2T2-A1 | 2-2 | 1 | 23 | 13 | 51.1 | 62.41 | 51.10 | 13.61 |
| 2 | D2T2-A2 | 2-2 | 1 | 23 | 13 | 51.1 | 57.89 | 51.10 | 13.61 |
| 3 | D2T2-B | 2-4 | 1 | 23 | 13 | 51.1 | 76.03 | 51.10 | 13.61 |
| 4 | D2T2-A1 | 2-2 | 1 | 23 | 14 | 51.1 | 62.06 | 51.10 | 13.61 |
| 5 | D2T2-A2 | 2-2 | 1 | 23 | 14 | 51.1 | 57.54 | 51.10 | 13.61 |
| 6 | D2T2-B | 2-4 | 1 | 23 | 14 | 51.1 | 75.33 | 51.10 | 13.61 |
| 7 | D2T2-A1 | 2-2 | 1 | 23 | 15 | 51.1 | 61.31 | 51.10 | 13.61 |
| 8 | D2T2-A2 | 2-2 | 1 | 23 | 15 | 51.1 | 56.79 | 51.10 | 13.61 |
| 9 | D2T2-B | 2-4 | 1 | 23 | 15 | 51.1 | 73.83 | 51.10 | 13.61 |
| 10 | D2T2-A1 | 2-2 | 1 | 23 | 16 | 51.1 | 60.17 | 51.10 | 13.61 |
| 11 | D2T2-A2 | 2-2 | 1 | 23 | 16 | 51.1 | 57.56 | 51.10 | 13.61 |
| 12 | D2T2-B | 2-4 | 1 | 23 | 16 | 51.1 | 71.56 | 51.10 | 13.61 |
| 13 | D2T2-A1 | 2-2 | 1 | 23 | 17 | 51.1 | 60.12 | 51.10 | 13.61 |
| 14 | D2T2-A2 | 2-2 | 1 | 23 | 17 | 51.1 | 57.51 | 51.10 | 13.61 |
| 15 | D2T2-B | 2-4 | 1 | 23 | 17 | 51.1 | 71.46 | 51.10 | 13.61 |
| 16 | D2T2-A1 | 2-2 | 1 | 23 | 18 | 51.1 | 59.87 | 51.10 | 13.61 |
| 17 | D2T2-A2 | 2-2 | 1 | 23 | 18 | 51.1 | 57.26 | 51.10 | 13.61 |
| 18 | D2T2-B | 2-4 | 1 | 23 | 18 | 51.1 | 70.96 | 51.10 | 13.61 |
| 19 | D2T2-A1 | 2-2 | 1 | 23 | 19 | 51.1 | 60.41 | 51.10 | 13.61 |
| 20 | D2T2-A2 | 2-2 | 1 | 23 | 19 | 51.1 | 55.89 | 51.10 | 13.61 |
| 21 | D2T2-B | 2-4 | 1 | 23 | 19 | 51.1 | 72.03 | 51.10 | 13.61 |
| 22 | D2T2-A1 | 2-2 | 1 | 23 | 20 | 51.1 | 60.01 | 51.10 | 13.61 |
| 23 | D2T2-A2 | 2-2 | 1 | 23 | 20 | 51.1 | 55.49 | 51.10 | 13.61 |
| 24 | D2T2-B | 2-4 | 1 | 23 | 20 | 51.1 | 71.23 | 51.10 | 13.61 |
| 25 | D2T2-A1 | 2-2 | 1 | 23 | 21 | 51.1 | 59.01 | 51.10 | 13.61 |
| 26 | D2T2-A2 | 2-2 | 1 | 23 | 21 | 51.1 | 54.49 | 51.10 | 13.61 |
| 27 | D2T2-B | 2-4 | 1 | 23 | 21 | 51.1 | 69.23 | 51.10 | 13.61 |
| 28 | D2T2-A1 | 2-2 | 1 | 23 | 22 | 51.1 | 58.07 | 51.10 | 13.61 |
| 29 | D2T2-A2 | 2-2 | 1 | 23 | 22 | 51.1 | 55.46 | 51.10 | 13.61 |
| 30 | D2T2-B | 2-4 | 1 | 23 | 22 | 51.1 | 67.36 | 51.10 | 13.61 |
| 31 | D2T2-A1 | 2-2 | 1 | 23 | 23 | 51.1 | 58.12 | 51.10 | 13.61 |
| 32 | D2T2-A2 | 2-2 | 1 | 23 | 23 | 51.1 | 55.51 | 51.10 | 13.61 |
| 33 | D2T2-B | 2-4 | 1 | 23 | 23 | 51.1 | 67.46 | 51.10 | 13.61 |
| 34 | D2T2-A1 | 2-2 | 1 | 23 | 24 | 51.1 | 57.67 | 51.10 | 13.61 |
| 35 | D2T2-A2 | 2-2 | 1 | 23 | 24 | 51.1 | 55.06 | 51.10 | 13.61 |
| 36 | D2T2-B | 2-4 | 1 | 23 | 24 | 51.1 | 66.56 | 51.10 | 13.61 |

## Link budget for Device 2a in D2T2

The link budgets and the corresponding maximum distances for the R2D and D2R link of Device 2a in the D2T2-A1/A2/B scenario are summarized in Table 13. Detail assumptions and calculations for each evaluation case would be reported to the excel template file provided by feature lead.

Table 13: MPL and maximum distance: D2T2-Device 2a, 900MHz

| Case | Scenario | CW case | Device | R2D EIRP | D2R LLS case | R2D MPL (dB) | D2R MPL (dB) | Min MPL (dB) | Min Distance (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | D2T2-A1 | 2-2 | 2a | 23 | 37 | 60.1 | 67.96 | 60.10 | 45.09 |
| 2 | D2T2-A2 | 2-2 | 2a | 23 | 37 | 60.1 | 63.44 | 60.10 | 45.09 |
| 3 | D2T2-B | 2-4 | 2a | 23 | 37 | 60.1 | 87.13 | 60.10 | 45.09 |
| 4 | D2T2-A1 | 2-2 | 2a | 23 | 38 | 60.1 | 67.41 | 60.10 | 45.09 |
| 5 | D2T2-A2 | 2-2 | 2a | 23 | 38 | 60.1 | 62.89 | 60.10 | 45.09 |
| 6 | D2T2-B | 2-4 | 2a | 23 | 38 | 60.1 | 86.03 | 60.10 | 45.09 |
| 7 | D2T2-A1 | 2-2 | 2a | 23 | 39 | 60.1 | 66.46 | 60.10 | 45.09 |
| 8 | D2T2-A2 | 2-2 | 2a | 23 | 39 | 60.1 | 61.94 | 60.10 | 45.09 |
| 9 | D2T2-B | 2-4 | 2a | 23 | 39 | 60.1 | 84.13 | 60.10 | 45.09 |
| 10 | D2T2-A1 | 2-2 | 2a | 23 | 40 | 60.1 | 66.02 | 60.10 | 45.09 |
| 11 | D2T2-A2 | 2-2 | 2a | 23 | 40 | 60.1 | 63.41 | 60.10 | 45.09 |
| 12 | D2T2-B | 2-4 | 2a | 23 | 40 | 60.1 | 83.26 | 60.10 | 45.09 |
| 13 | D2T2-A1 | 2-2 | 2a | 23 | 41 | 60.1 | 65.77 | 60.10 | 45.09 |
| 14 | D2T2-A2 | 2-2 | 2a | 23 | 41 | 60.1 | 63.16 | 60.10 | 45.09 |
| 15 | D2T2-B | 2-4 | 2a | 23 | 41 | 60.1 | 82.76 | 60.10 | 45.09 |
| 16 | D2T2-A1 | 2-2 | 2a | 23 | 42 | 60.1 | 65.27 | 60.10 | 45.09 |
| 17 | D2T2-A2 | 2-2 | 2a | 23 | 42 | 60.1 | 62.66 | 60.10 | 45.09 |
| 18 | D2T2-B | 2-4 | 2a | 23 | 42 | 60.1 | 81.76 | 60.10 | 45.09 |
| 19 | D2T2-A1 | 2-2 | 2a | 23 | 43 | 60.1 | 65.86 | 60.10 | 45.09 |
| 20 | D2T2-A2 | 2-2 | 2a | 23 | 43 | 60.1 | 61.34 | 60.10 | 45.09 |
| 21 | D2T2-B | 2-4 | 2a | 23 | 43 | 60.1 | 82.93 | 60.10 | 45.09 |
| 22 | D2T2-A1 | 2-2 | 2a | 23 | 44 | 60.1 | 65.31 | 60.10 | 45.09 |
| 23 | D2T2-A2 | 2-2 | 2a | 23 | 44 | 60.1 | 60.79 | 60.10 | 45.09 |
| 24 | D2T2-B | 2-4 | 2a | 23 | 44 | 60.1 | 81.83 | 60.10 | 45.09 |
| 25 | D2T2-A1 | 2-2 | 2a | 23 | 45 | 60.1 | 64.11 | 60.10 | 45.09 |
| 26 | D2T2-A2 | 2-2 | 2a | 23 | 45 | 60.1 | 59.59 | 59.59 | 42.12 |
| 27 | D2T2-B | 2-4 | 2a | 23 | 45 | 60.1 | 79.43 | 60.10 | 45.09 |
| 28 | D2T2-A1 | 2-2 | 2a | 23 | 46 | 60.1 | 64.02 | 60.10 | 45.09 |
| 29 | D2T2-A2 | 2-2 | 2a | 23 | 46 | 60.1 | 61.41 | 60.10 | 45.09 |
| 30 | D2T2-B | 2-4 | 2a | 23 | 46 | 60.1 | 79.26 | 60.10 | 45.09 |
| 31 | D2T2-A1 | 2-2 | 2a | 23 | 47 | 60.1 | 63.82 | 60.10 | 45.09 |
| 32 | D2T2-A2 | 2-2 | 2a | 23 | 47 | 60.1 | 61.21 | 60.10 | 45.09 |
| 33 | D2T2-B | 2-4 | 2a | 23 | 47 | 60.1 | 78.86 | 60.10 | 45.09 |
| 34 | D2T2-A1 | 2-2 | 2a | 23 | 48 | 60.1 | 63.12 | 60.10 | 45.09 |
| 35 | D2T2-A2 | 2-2 | 2a | 23 | 48 | 60.1 | 60.51 | 60.10 | 45.09 |
| 36 | D2T2-B | 2-4 | 2a | 23 | 48 | 60.1 | 77.46 | 60.10 | 45.09 |

## Link budget for Device 2b in D2T2

The link budgets and the corresponding maximum distances for the R2D and D2R link of Device 2b in the D2T2-C scenario are expected to be summarized in Table 14 later.

Table 14: MPL and maximum distance: D2T2-Device 2b, 900MHz

| Case | Scenario | CW case | Device | R2D EIRP | D2R LLS case | R2D MPL (dB) | D2R MPL (dB) | Min MPL (dB) | Min Distance (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | D2T2-C | N/A | 2b | 23 | 61 |  |  |  |  |
| 2 | D2T2-C | N/A | 2b | 23 | 62 |  |  |  |  |
| 3 | D2T2-C | N/A | 2b | 23 | 63 |  |  |  |  |
| 4 | D2T2-C | N/A | 2b | 23 | 64 |  |  |  |  |
| 5 | D2T2-C | N/A | 2b | 23 | 65 |  |  |  |  |
| 6 | D2T2-C | N/A | 2b | 23 | 66 |  |  |  |  |
| 7 | D2T2-C | N/A | 2b | 23 | 67 |  |  |  |  |
| 8 | D2T2-C | N/A | 2b | 23 | 68 |  |  |  |  |
| 9 | D2T2-C | N/A | 2b | 23 | 69 |  |  |  |  |
| 10 | D2T2-C | N/A | 2b | 23 | 70 |  |  |  |  |
| 11 | D2T2-C | N/A | 2b | 23 | 71 |  |  |  |  |
| 12 | D2T2-C | N/A | 2b | 23 | 72 |  |  |  |  |

## Summary

Based on the coverage evaluation results above in section 5, we have the following observations:

Observation 4: For D1T1 device 1, all 3 scenarios A1/A2/B can achieve maximum distance within 10-50m under InF-DH NLOS channel model at 900MHz

- For A1, the coverage is from 14.43m to 24.40m

- For A2, the coverage is from 10.40m to 22.21m

- For B, the coverage is from 14.43m to 24.40m

Observation 5: For D1T1 device 2a, all 3 scenarios A1/A2/B can achieve maximum distance within 10-50m under InF-DH NLOS channel model at 900MHz while some of A1/B cases can achieve maximum distance longer than 50m

- For A1, the coverage is from 35.34m to 62.86m

- For A2, the coverage is from 13.67m to 40.87m

- For B, the coverage is from 26.52m to 62.86m

Observation 6: For D2T2 device 1, all 3 scenarios A1/A2/B can achieve maximum distance within 10-50m under InH-Office LOS channel model at 900MHz

- For A1, the coverage is 13.61m

- For A2, the coverage is 13.61m

- For B, the coverage is 13.61m

Observation 7: For D2T2 device 2a, all 3 scenarios A1/A2/B can achieve maximum distance within 10-50m under InH-Office LOS channel model at 900MHz

- For A1, the coverage is 45.09m

- For A2, the coverage is from 42.12m to 45.09m

- For B, the coverage is 45.09m

# Conclusions

Based on the analysis in this contribution, we have the following observations and proposals:

Observation 1: The R2D link performance (M=6, OOK-4) of different message sizes and different data rates for device 2b IF-ED receiver are as follows,

- For BLER 10%

  - From 14.0dB to 16.0dB for D1T1 in TDL-A channel

  - From 8.7dB to 10.2dB for D2T2 in TDL-D channel

- For BLER 1%

  - From 26.0dB to 27.0dB for D1T1 in TDL-A channel

  - From 13.0dB to 14.2dB for D2T2 in TDL-D channel

Observation 2: The D2R link performance of different message sizes and different data rates for device 1 are as follows,

- For BLER 10%

  - From -6.4dB to 0.3dB for D1T1 in TDL-A channel

  - From -6dB to – 3.5dB for D2T2 in TDL-D channel

- For BLER 1%

  - From -1.1dB to 8.2dB for D1T1 in TDL-A channel

  - From -1.9dB to 1.1dB for D2T2 in TDL-D channel

Observation 3: The D2R link performance of different message sizes and different data rates for device 2a are as follows,

- For BLER 10%

  - From -8dB to -0.5dB for D1T1 in TDL-A channel

  - From -7.7dB to -3.8dB for D2T2 in TDL-D channel

- For BLER 1%

  - From -2.5dB to 13dB for D1T1 in TDL-A channel

  - From -3.7dB to 0.9dB for D2T2 in TDL-D channel

Observation 4: For D1T1 device 1, all 3 scenarios A1/A2/B can achieve maximum distance within 10-50m under InF-DH NLOS channel model at 900MHz

- For A1, the coverage is from 14.43m to 24.40m

- For A2, the coverage is from 10.40m to 22.21m

- For B, the coverage is from 14.43m to 24.40m

Observation 5: For D1T1 device 2a, all 3 scenarios A1/A2/B can achieve maximum distance within 10-50m under InF-DH NLOS channel model at 900MHz while some of A1/B cases can achieve maximum distance longer than 50m

- For A1, the coverage is from 35.34m to 62.86m

- For A2, the coverage is from 13.67m to 40.87m

- For B, the coverage is from 26.52m to 62.86m

Observation 6: For D2T2 device 1, all 3 scenarios A1/A2/B can achieve maximum distance within 10-50m under InH-Office LOS channel model at 900MHz

- For A1, the coverage is 13.61m

- For A2, the coverage is 13.61m

- For B, the coverage is 13.61m

Observation 7: For D2T2 device 2a, all 3 scenarios A1/A2/B can achieve maximum distance within 10-50m under InH-Office LOS channel model at 900MHz

- For A1, the coverage is 45.09m

- For A2, the coverage is from 42.12m to 45.09m

- For B, the coverage is 45.09m

Proposal 1:  Different values of the maximum distance target is set for different scenarios, which are decided according to the corresponding link budget study.

Proposal 2: Refine the definition of latency as

- For inventory use case (for DO-DTT traffic type):

  - The time interval between the time that the inventory request is sent from BS/intermediate UE to a A-IoT device and the time that the inventory report is successfully received at BS/intermediate UE from the A-IoT device.

- For command use case (for DT traffic type):

  - The time interval between the time that the DL command is sent from BS/intermediate UE and the time that the command is successfully received at A-IoT device.

- Note: the successfully received is considered as follows

  - Successfully received is associated with a target BLER for the reception of one attempt at inventory for the A-IoT device. Target BLER is 1% or 10%

  - The first attempt is taken into account

- Note: The latency is evaluated for a single A-IoT device.

Note: Time for energy harvesting is not included in the definition of latency.

Proposal 3: The inventory completion time of multiple Ambient IoT devices is defined as:

- The time a reader successfully completed the reception of inventory information / inventory process for [Z]% of A-IoT devices for a given number of A-IoT devices within corresponding coverage by the reader

- Z = {99%(Mandatory), 90%(Optional)}

Proposal 4: The study assumes FDD downlink spectrum for R2D transmissions in D1T1.

Proposal 5: In D1T1, the study assumes the following spectrum for both CW2D and D2R transmission.

- D1T1-A: DL spectrum (Case 1-1)

- D1T1-B: UL spectrum (Case 1-4)

Proposal 6: In D1T1-B, the CW distribution is reported by companies.

Proposal 7: The study assumes uplink spectrum for the R2D transmission in D2T2.

Proposal 8: The study assumes UL spectrum for both CW2D and D2R transmission in both D2T2-A and D2T2-B.

Proposal 9: The study assumes UL spectrum for the D2R transmission in D2T2-C.

Proposal 10: The intermediate UEs are assumed to be deployed in a grid with e.g. 10 m /20 m distance between every two adjacent intermediate UEs.

Proposal 11: The devices within the calculated distance, which is obtained by the corresponding link budget calculation, from each intermediate UE will be involved in the evaluations.

Proposal 12: In D2T2-B, the CW distribution is reported by companies.

# References

- R1-2403663, “Session notes for 9.4 Study on solutions for Ambient IoT (Internet of Things)”, Ad-Hoc Chair (Huawei).

- 3GPP TR 38.848, “Study on Ambient IoT (Internet of Things) in RAN”.

- 3GPP TR 37.910, “Study on self-evaluation towards IMT-2020 submission”.

- R1-2405772, “Email discussion on Ambient IoT evaluation assumptions”, Moderator (CMCC)

- Modeling and Estimation of One-Shot Random Access for Finite-User Multichannel Slotted ALOHA Systems, IEEE Communications Letters (Vol. 16, Iss. 8), Aug. 2012

- Fast and Accurate Estimation of RFID Tags, IEEE/ACM Transactions on Networking (Vol. 23, Iss. 1), Feb. 2015

- PLACE: Physical Layer Cardinality Estimation for Large-Scale RFID Systems, IEEE/ACM Transactions on Networking (Vol. 24, Iss. 5), Oct. 2016

- On Number of Tags Estimation in RFID Systems, IEEE Systems Journal (Vol. 11, Iss. 3), Sep. 2017

# Annex A: Agreed link-level simulation assumptions

(Copied from R1-2405772 [4]) The following table of coverage evaluation assumptions in link level simulation is considered. Parameters; Assumptions; Company result1; Company result 2. R2D/D2R common parameters. [0a]; Carrier frequency; Refer to link budget template. [0b]; SCS; 15 kHz as baseline. [0c]; Block structure; Blocks as agreed in 9.4.2.3, or other blocks reported by companies. [0d]; Channel model; <Editor’s Note: will be updated according to the agreements made for channel model>. [0e]; Delay spread; [30, 150] ns - An RMS delay spread of 30 ns (M) and [150] ns (O) is considered for TDL-A channel model. - An RMS delay spread of 30 ns is considered for TDL-D channel model.. [0f]; Device velocity; 3 km/h. [0g]; Number of Tx/Rx chains for Ambient IoT device; 1. [0h1]; BS; Number of antenna elements; 2 or 4. [0h2]; Number of TXRUs; 2 or 4. [0j1]; Intermediate UE; Number of antenna elements; 1 or 2. [0j2]; Number of TXRUs; 1 or 2. [0m]; Reference data rate; [0.1, 1, 5] kbps [0.1] kbps (M), [1] kbps (M), [2] kbps (O), [5 ~ 7] kbps (O M), [large value] (O) 1 kbps (M), 5 ~ 7 kbps (M), [large value] (O) FFS:0.1 kbps (M), - Note1: companies to report the exact data rate. - Note 2: the exact data rate is close to the values listed above. - Note 3: The exact data rate is calculated by dividing the total message size (excluding CRC) by the total transmission time including applicable overheads (e.g., CRC, pre/mid/post-ambles if present). - Note 4: the exact data rate may be related to coding scheme, repetition and etc. - Note 5: all data rates considered are for evaluation purpose only. [0n]; Message size; {20 bits, 96 bits, 400 bits} are considered for message size. - Note 1: companies to report the M value and chip length used for each message size - Note 2: CRC is not included for the message size. [0p]; BLER target; 1%, 10%. [0q]; Sampling frequency; Working assumption on [0q] (sampling frequency) in link level simulation table Companies to report the Sampling frequency (e.g., 1.92Msps or other feasible values if any) Initial SFO (Sampling Frequency Offset) (Fe): - (M) Randomly select a value from the range of [0.1 ~ 1] *10^4 ppm for device 2, - (M) Randomly select a value from the range of [0.1 ~ 1] * 10^5 ppm for device 1, - (O) Randomly select a value from the range of [0.1 ~ 1] *10^5 ppm for device 2, - FFS: Optionally evaluate a fixed value SFO for device 1 and 2 - Note: For random selection, the value is randomly selected per simulation drop, according to a uniform distribution - Note: Above values are only for sampling purpose. - FFS other values - Note: Above assumptions are only for LLS evaluation purpose only for R2D and [D2R]. The timing drift ΔT over a time T is modelled as ΔT = ±Fe * T. - Note: Accuracy can be improved after clock calibration for at least device 2. FFS applicable for device 1 - Note: SFO after clock calibration can be applied to Fe. - FFS other models CFO for device 2b. - [100ppm/200ppm/1000ppm, 0.1ppm/s]” Note: Above assumptions are for LLS evaluation purpose only. [0r]; Device 1/2a/2b; Options are as follows, - Device 1, RF-ED - Device 2a, RF-ED - Device 2b, RF-ED/IF-ED/ZIF <Editor’s Note: will be updated according to agreements from 9.4.1.2>. R2D specific parameters. [1a]; Transmission bandwidth; 180 kHz as baseline. Other larger values are not precluded.. [1b]; FFS: ED bandwidth; The ED bandwidth is the bandwidth for calculating the noise/interference (if any) power: For evaluations, the value(s) of ED bandwidth is 20 MHz for RF-ED, [180] kHz for IF/ZIF receiver. Note: this does not imply that a A-IoT device supports sampling clock rate as large as RF ED bandwidth.. [1c]; FFS: BB LPF; [X]-order Butterworth/RC filter with cutoff frequency at [Y] kHz, half of R2D transmission bandwidth. Companies to report X = {3, 5}.. [1d]; Waveform; OOK waveform generated by OFDM modulator. [1e]; Modulation; OOK Companies to report, e.g., OOK-1, OOK-4 with M chips per OFDM symbol. [1f]; Line code; Companies to report, e.g., Manchester, PIE. [1g]; FEC; No FEC as baseline. [1h]; ADC bit width; 1-bit for device 1 4-bit for device 2. [1j]; Detection/decoding method for Line code; Companies to report. D2R specific parameters. [2a1]; Transmission bandwidth (w.r.t. D2R data rate); - [2a1]-Alt1 (M): - DSB - X kHz (M) and Y kHz (O) is considered for D2R transmission bandwidth. - The value is for two sidebands, i.e., the total transmission bandwidth for DSB is X kHz (M) and Y kHz (O). - [2a1]-Alt2: - SSB - X kHz (M) and Y kHz (O) is considered for D2R transmission bandwidth. - The value is for one sideband, i.e., the total transmission bandwidth for DSBSSB is X kHz (M) and Y kHz (O). - For device 2b only, FFS for device 2a. - The value of X and Y is as follows, to be down-select from alternative 1 and 2 - Alternative 1: - X = {15 (M), 180 (O)} - Y =180 - Alternative 2: - X and Y reported by companies, - the value may be related to, e.g., - Reference data rate - Coding scheme - Repetition - With or without SFS - SSB or DSB - X = {[15 (M)], [180 (O)]}, other values are not precluded and reported by companies. [2a2]; [OOK/BPSK/BFSK chip rate]; Companies to report. [2a3]; Receiver bandwidth; D2R receiver bandwidth is the bandwidth used at the reader side to filter out the D2R signals for calculating noise and interference (if any) power. - Assume the receiver matches the transmitter's modulation, i.e., to receiver uses SSB when transmitter uses SSB, receiver uses DSB when transmitter uses DSB. Companies to report the value, and further down-selection of the values and DSB/SSB is not precluded.. [2b]; Waveform (CW); Companies to report waveform, e.g., unmodulated single tone, multi-tone(multiple unmodulated single tone). [2d]; Modulation; Companies to report modulation, e.g., OOK, BPSK, BFSK. [2e]; Line code; Companies to report, e.g., Manchester encoding, FM0 encoding, Miller encoding, no line coding. [2g]; FEC; Companies to report, e.g., CC, No FEC. [2h]; ADC bit width; Companies to report, e.g., 11-bit. [2j]; D2R receiver; FFS: Reader receiver, e.g., coherent receiver / non-coherent receiver Companies to report, e.g., coherent receiver / non-coherent receiver. Other assumptions. [3a]; Other assumptions; To be reported by company. [3b]; Note: Companies to report required SINR/SNR/CINR/CNR according to BLER target.. Note 1: -These values are only for evaluation purpose and any differences among device types (if any) are not intended for harmonized design approach.Any differences among devicestypes (if any) are for evaluation purpose only. Note 2: (M) denotes the value is mandatory to be evaluated. (O) denotes the value can be optionally evaluated..

# Annex B: Agreed link budget template

(Copied from R1-2405772 [4]) No.; Item; Reader-to-Device; Device-to-Reader. [0A]; Scenarios; D1T1-A1/A2/B/C D2T2-A1/A2/B/C; D1T1-A1/A2/B/C D2T2-A1/A2/B/C. [0A1]; CW case; N/A; 1-1/1-2/1-4/2-2/2-3/2-4. [0B]; Device 1/2a/2b; Device 1/2a/2b; Device 1/2a/2b. [0C]; Center frequency (MHz); 900MHz (M), 2GHz (O); 900MHz (M), 2GHz (O). [0D]; Topology/Pathloss model; For D2T2: - [0D]-Alt1: InF-DL NLOS - [0D]-Alt2: InH-Office LOS For D1T1: - InF-DH NLOS; For D2T2: - [0D]-Alt1: InF-DL NLOS - [0D]-Alt2: InH-Office LOS For D1T1: - InF-DH NLOS. [1D]; Number of Tx antenna elements / TxRU/ Tx chains modelled in LLS; For BS: - 2(M) or 4(O) antenna elements for 0.9 GHz For Intermediate UE: - 1(M) or 2(O); 1. [1E]; Total Tx Power (dBm); - For BS in DL spectrum for indoor - [1E]-R2D-Alt1: 33dBm(M), - [1E]-R2D-Alt2: 38dBm(O), - [1E]-R2D-Alt3: 24dBm(M) - Companies to report if PSD constraints are imposed (company to report the condition for applying PSD constraints in Row [5A]: Other notes) - For UL spectrum for indoor, - [1E]-R2D-Alt4:23dBm (M) - [1E]-R2D-Alt5:26dBm(O); - For device 1/2a: (see note 1) - [1E]-D2R-Alt1: (For scenarios ‘B’) - The Device Tx Power is calculated by CW received power which can be derived by at least CW2D distance (m) value and other related factors. - [1E]-D2R-Alt2: (For scenarios ‘A1’ and ‘A2’) - The Device Tx Power is calculated by assuming CW2D pathloss = D2R pathloss. - For device 2b: (For scenarios ‘C’) - [1E]-D2R-Alt3: -20 dBm(M) - [1E]-D2R-Alt4: -10 dBm(O). [1E1]; CW Tx power (dBm); N/A; For scenario ‘A1’, ‘A2’ and ‘B’ - Report a value from the candidate values [1E]-R2D-Alt1/[1E]-R2D-Alt2/[1E]-R2D-Alt3 from [1E]-R2D if CW in DL spectrum - Report a value from the candidate values [1E]-R2D-Alt4/[1E]-R2D-Alt5 from [1E]-R2D if CW in UL spectrum. Note: only applicable for device 1/2a. [1E2]; CW Tx antenna gain (dBi); N/A; - Company to report, the value equals to - UE Tx ant gain, or - BS Tx ant gain Note: only applicable for device 1/2a. [1E3]; CW2D distance (m); N/A; For scenarios ‘B’ - D1T1-B: - 5m, - 10m, - 20m - CW2D distance is derived assuming CW node is located with the same position as ‘R1’ in ‘A1’ scenario. (see note 1) - D2T2-B: - 5m, - 10m, - FFS other values For scenarios ‘A1’ and ‘A2’ - Calculated (see note 1), (i.e., CW2D distance is calculated by assuming CW2D pathloss = D2R pathloss) Note: only applicable for device 1/2a Note: companies to report which value(s) are evaluated.. [1E4]; CW2D pathloss (dB); N/A; Calculated (see note1) Note: only applicable for device 1/2a. [1E5]; CW received power (dBm); N/A; Calculated (see note1) Note: only applicable for device 1/2a. [1F]; Transmission Bandwidth used for the evaluated channel (Hz); 180kHz(M), 360kHz(O), 1.08MHz(O); Refer to LLS table [1a]. [1G]; Tx antenna gain (dBi); - For BS for indoor, 6 dBi(M), 2dBi(M) - For intermediate UE, 0 dBi; - For A-IoT device, 0dBi. [1H]; Ambient IoT backscatter loss (dB) due to Modulation factor; N/A; - OOK: 6 dB - PSK: 0 dB - FSK: Y dB It is applicable for device 1 and 2a Companies to report and justify their assumptions for Y. Companies to report in row 3D if they assume any additional related loss.. [1J]; Ambient IoT on-object antenna penalty; Not applicable; 0.9dB or 4.7dB. [1K]; Ambient IoT backscatter amplifier gain (dB); N/A; - 10 dB (M) - 15 dB (O) Note: Only for device 2a. [1N]; Cable, connector, combiner, body losses, etc. (dB); - For BS, X dB, X <=3 to be reported by companies with justification provided in row 5A - For intermediate UE, 1 dB; N/A. [1M]; EIRP (dBm); Calculated (see Note 1) FFS: any limitation of the EIRP subject to future discussion; Calculated (see Note 1). [2A]; Number of receive antenna elements / TxRU / chains modelled in LLS; Same as [1D]-D2R; Same as [1D]-R2D. [2B]; Bandwidth used for the evaluated channel (Hz); Refer to LLS table [1b] ED bandwidth; Refer to LLS table [2a] [receiver bandwidth?]. [2C]; Receiver antenna gain (dBi); same as [1G]-D2R; Same as [1G]-R2D. [2X]; Cable, connector, combiner, body losses, etc. (dB); N/A; Same as [1N]-R2D. [2D]; Receiver Noise Figure (dB); For RF-ED receiver - 20dB, Device 2 - FFS other values For IF/ZIF receiver - 15dB, Device 2; For BS as reader - 5dB For intermediate UE as reader - 7dB. [2E]; Thermal Noise power spectrum density (dBm/Hz); -174; -174. [2F]; Noise Power (dBm); Calculated (see Note 1); Calculated (see Note 1). [2G]; Required SNR/CNR; Reported by companies for Budget-Alt2; Reported by companies for Budget-Alt2. [2H]; Ambient IoT on-object antenna penalty; 0.9dB or 4.7dB; Not applicable. [2J]; Budget-Alt1/ Budget-Alt2; Budget-Alt1/ Budget-Alt2 (see note1); Budget-Alt2. [2K]; CW cancellation (dB); N/A; Companies to report for scenario A2/A1/B for BS and intermediate UE. Note: - Only applicable for device 1/2a - The value provided is for the unmodulated single-tone CW. The impact of a multi-tone CW, e.g., assuming an [X] dB difference, is FFS. [2K1]; Remaining CW interference (dB); N/A; Calculated (see Note 1) Note: only applicable for device 1/2a. [2K2]; Receiver sensitivity loss(dB); N/A; Calculated (see Note 1) Note: only applicable for device 1/2a. [2L]; Receiver Sensitivity (dBm); For Budget-Alt1, - For device 1 (RF-ED), for example: - {-30dBm, -36dBm, -40dBm, etc} - For device 2 (RF-ED), for example: - {-40dBm, -45dBm, etc} For Budget-Alt2, - Calculated (see note1); Calculated (see Note 1) Note: the receiver sensitivity includes the receiver sensitivity loss [2K2], i.e. after CW cancellation at least if ‘A2’ scenario is used. [3A]; Shadow fading margin (dB); For D1T1: 4 dB For D2T2: 3dB for InH-LOS 7.2dB for InF-DL-NLOS; For D1T1: 4 dB For D2T2: 3dB for InH-LOS 7.2dB for InF-DL-NLOS. [3B]; polarization mismatching loss (dB); 3 dB; 3 dB. [3C]; BS selection/macro-diversity gain (dB); 0 dB FFS: other values are not precluded Note: only applicable for D1T1; 0 dB FFS: other values are not precluded Note: only applicable for D1T1. [3D]; Other gains (dB) (if any please specify); Reported by companies with justification; Reported by companies with justification. [4A]; MPL (dB); Calculated (see Note 1); Calculated (see Note 1). [4B]; Distance (m); Calculated (see Note 1); Calculated (see Note 1). [5A]; Other notes; Companies to report; Companies to report. Note1: calculated values in the Table XXXX are derived according to the followings, [1E3] - For scenarios where CW2D distance is calculated by assuming CW2D pathloss = D2R pathloss ‘A1’ and ‘A2’, [1E3] is derived by assuming pathloss is [1E4] and use the pathloss formula as agreed. [1E4] - For scenarios where CW2D distance is calculated by assuming CW2D pathloss = D2R pathloss - [1E4] = 0.5* ( [1E1] + [1E2] - [1N](R2D) + [2C] (R2D) – [2H](R2D) – 2*[3A] – 2*[3B] +[3D](R2D) + [1K] – [1H] + [1G] – [1J] + [2C] – [2X] – [2L] + [3C] + [3D] ) - [1K] is only for device 2a - Otherwise - [1E4] is derived according to pathloss formula by assume distance is [1E3] [1E5] - [1E5] = [1E1] + [1E2] - [1N](R2D) - [1E4] + [2C] (R2D) – [2H](R2D) – [3A] – [3B] + [3D](R2D) [1E] - [1E] = [1E5]+ [1K] – [1H] - [1K] is only for device 2a [1M]: - For R2D, - [1M] = [1E] + [1G] - [1N] - For D2R - [1M] = [1E] + [1G] - [1J] [2F]: - [2F] = [2D] + [2E] +lin2dB([2B]) [2G] - For the R2D LLS for ED, CINR/CNR is reported, where CINR/CNR is defined as the ratio of signal power spectral density in the transmission bandwidth to the noise and interference (if any) power spectral density in the device ED channel bandwidth. - For R2D ZIF receiver, report the same metrics (i.e., CNR/CINR, signal transmission bandwidth, ED bandwidth) as agreed for RF-ED/IF receiver. - For the D2R LLS, the SINR/SNR is reported and it is defined as the ratio of signal power to noise and interference (if any) power in the receiver bandwidth. - On/off keying backscatter loss (including DC removal loss) is not taken into account in the LLS and is included in link budget table [1H]. [2J] - For R2D link in the coverage evaluation, for device 1 - Budget-Alt1 is used (note: receiver architecture is RF ED) - For R2D link in the coverage evaluation for device 2, - Budget-Alt1 is used if receiver architecture is RF ED - Budget-Alt2 is used if receiver architecture is IF/ZIF ED - Note1a: this does not preclude to have LLS for device 1 and 2 R2D link with RF-ED if needed. - Note1b: For device 2 R2D link with RF-ED, Budget-Alt1 is mandatory, Budget-Alt2 is optional. - Note1c: this does not imply all M values are achievable with the sensitivity given by Budget-Alt1 for RF ED - Note1d: For device 2 with an RF ED-based receiver on the R2D link, if the receiver sensitivity derived from Budget-Alt2, assuming a noise figure of [X dB], exceeds the receiver sensitivity based on Budget-Alt1, then Budget-Alt2 is applied. [2K1]: - [2K1] = [1E1] + [1E2] -[1N](R2D) + [2C] - [2X] - [2K] [2K2]: - 2K2=lin2dB1+dB2lin([2K1])dB2lin([2F]) [2L]: - For R2D and Budget-Alt2, - [2L] = [2G] - lin2dB([2B] / [1F]) + [2F] - Note 1e: the term ‘lin2dB([2B] / [1F])’ is applied due to scaling from CNR/CINR to SNR/SINR. - For D2R, - [2L] = [2G] + [2F] + [2K2], device 1/2a - [2L] = [2G] + [2F], device 2b [4A] - For R2D, - [4A]=[1M]+[2C] -[2H]-[2L]-[3A]-[3B]+[3C]+[3D] - For D2R, - [4A]=[1M]+[2C] -[2X]-[2L]-[3A]-[3B]+[3C]+[3D] [4B] - [4B] is derived by assuming pathloss is [4A] and use the pathloss formula as agreed. Note2: (M) denotes the value is mandatory to be evaluated. (O) denotes the value can be optionally evaluated.

# Annex C: Numerical methodology for the evaluation of the inventory completion time of multiple devices

Referring to the ISO-180006C based UHF RFID, an access scheme called “dynamic framed slotted ALOHA (DFSA)” is assumed for the inventory by Ambient IoT. As shown in Figure D-1, there can be assumed three types of “ALOHA-slot” in the inventory procedure, which are successful ALOHA-slot, collided ALOHA-slot, and idle ALOHA-slot. The inventory completion time can be calculated from the duration and the total number of each type of “ALOHA-slot”.

(a) The duration of the successful ALOHA-slot

(b) The duration of the collided ALOHA-slot

(c) The duration of the idle ALOHA-slot

Figure D-1: Successful/Collided/Idle ALOHA-slots in the DFSA access procedure

For notational convenience in the following, where we say slot we always mean an “ALOHA-slot”.

The first step is to derive the duration of each type of “slot”. From Figure D-1,

In Step 1a, the duration of the transmission of each message appearing in the access procedure is calculated as follows.

- Firstly, the size of each message used in the whole access procedure need to be determined, including MSTrigger, MSQuery, MSRN, MSReply, MSID, and MSQueryRep.

- Secondly, the transmission parameters, such as modulation, coding, repetitions, and chip length, of each R2D or D2R transmission need to be determined. The number of chips and chip length of the preamble, midamble (if present), and postamble of each transmission also need to be determined

- Thirdly, the length of each message can be calculated according to the message size and the corresponding R2D or D2R transmission parameters, including TTrigger, TQuery, TRN, TReply, TID, and TQueryRep. It should be noted that the preamble, midamble (if present), and postamble are taken into account in the length of each transmission.

In Step 1b, the duration of each type of “slot” is calculated as follows.

- In an idle “slot”, there is no response from any device. The reader can detect the preamble of the potential D2R transmission(s) to determine if there is response from a device or multiple devices. If no D2R preamble is detected within TR2D_R2D, the reader will send a QueryRep message to indicate the end of the slot. The duration of the idle slot consists of the length of the detection window and the R2D transmission of the “QueryRep” message. Consequently, the duration of an idle slot is calculated by the following formula.

TIdle = TR2D_R2D + TQueryRep

- In a collided “slot”, the reader detects that there does exist preamble(s) from one or more devices. Then the reader will send a QueryRep message to indicate the end of the slot. The duration of a collided slot consists of the length of the D2R transmission of a “RN” message, the R2D transmission of a “QueryRep” message, and the gap between consecutive R2D/D2R transmissions for the Rx processing and Tx preparation.

TCollided = TR2D + TRN + TD2R + TQueryRep

- In a successful “slot”, the reader successfully detects the response from a device, and replies to the device with the random number received in the RN message from the device. After the device detects the matched random number, it will send its ID to the reader. The reader will send a QueryRep message to end the slot after it receives the Device ID. The duration of a successful slot consists of the length of the D2R transmissions of a RN message and the Device ID, the R2D transmissions of a Reply message and QueryRep message, and the gap between consecutive R2D/D2R transmissions for the Rx processing and Tx preparation of reader and device.

TSuccess = TR2D+TRN+TD2R+Treply+TR2D+TID+TD2R+TQueryRep

In the above, each gap between consecutive R2D/D2R transmissions can be calculated as the average of the pre-defined minimum and maximum processing time of the corresponding timing relationship parameter.

The second step is to derive the total number of each type of “slot”.

- Referring to the theoretical derivations of the slotted ALOHA scheme [5], the probability of the three types of “slot” in the ith round can be approximately calculated according to the following formulas.

PIdle i = e-Mi/Ni

PSuccessi =e-Mi/Ni

PCollidedi=1-PIdlei-PSuccessi

where N(i) is the total number of “slots” in the i-th round of inventory, M(i) is the number of devices participating in the corresponding round of inventory. Then the number of each type of “slot” in each round is calculated as

NIdlei = Ni∙e-Mi/Ni

NSuccessi =Ni∙e-Mi/Ni

NCollidedi=Ni - NIdlei - NSuccessi

At the beginning of the first inventory round, N(1) is optimally set according to M(1). In theory, the optimal case is N(1) = M(1). If the candidate values of N(i) do not include M(1), N(1) is set to the candidate value closest to M(1). In practice, the value of M(1) can be obtained from the application server or estimated by a certain cardinality estimation method [6]-[8]. In the whole procedure, the number of unread devices after the i-th round is Ni-NSuccessi, which is regarded as the M(i) for the (i+1)th round. The allocated number of slots in the i-th round N(i+1) is set to M(i+1) or the candidate value closest to M(i+1).

- Assuming I rounds of inventory is needed to read Z% of all the devices, the overall number of each type of “slot” in the whole inventory procedure is calculated as

NIdle = i=1INIdlei

NSuccess = i=1INSuccessi

NCollided = i=1INCollidedi

The third step is to calculate the latency of the whole inventory procedure, according to the duration and total number of each type of “slot” derived in the first and second step, respectively. It should be noted that the initial triggering message (like the Select message in the UHF RFID) and the configuration message at the beginning of each inventory round (like the Query message in the UHF RFID) have to be taken into account.

TInventory=TTrigger+I∙TQuery+TIdle∙NIdle+TCollided∙NCollided+TSuccess∙NSuccess

In general, the devices with different pathlosses from the reader will use different data rate to satisfy the target BLER. In other words, a higher data rate can be assumed for the R2D/D2R transmissions of the devices with smaller pathloss. The inventory completion time corresponding to different data rate can be calculated according to the above methodology. To simplify the evaluation, limited number of R2D/D2R data rates can be assumed. The number of devices with one of the chosen data rates can be determined by the following steps.

- Firstly, the link budget corresponding to each data rate (DR) is determined. For example, DR1 corresponds to the worst coverage, whose link budget should support the required maximum distance. Based on DR1, the link budget corresponding to the other two data rates DR2 and DR3 can be assumed, for modeling purposes, to be e.g. approximately 10 dB and 20 dB better than that for DR1, respectively.

- According to the above link budgets and the pathloss model, the supported maximum distance corresponding to each data rate is then calculated, which are denoted as DMax,DR1, DMax,DR2, and DMax,DR3.

- Centered at the reader, the devices within the range of the DMax,DR3 correspond to DR3, while the remaining devices within the range of DMax,DR2 correspond to DR2. The other devices correspond to DR1.

Figure D-2: The example of the device distributions corresponding to different data rates

Independent inventory can be applied to devices with DR3 firstly. By setting the transmission parameters appropriately, the devices corresponding to DR2 and DR1 will not be able to correctly decode the control messages, and thus not try to access during the procedure. After the inventory of devices with DR3 being finished, the devices with DR2 will be inventoried subsequently. At last, the whole inventory is finished by reading the devices with DR3 after the DR2 inventory being done. The overall inventory completion time is calculated as the sum of the durations corresponding to the three data rates.