3GPP TSG-RAN WG1 Meeting #118	R1-2405855

Maastricht, Netherlands, August 19-August 23, 2024

Agenda Item:	9.4.2.4

Source:	Huawei, HiSilicon

Title:	On external carrier wave for backscattering based Ambient IoT device

Document for:	Discussion and Decision

# Introduction

To support backscatter modulation, external carrier-wave has to be provided to Ambient IoT device. The external carrier-wave will also be received by the Ambient IoT uplink receiver, which causes interference. Consequently, the agreed waveform characteristics of the carrier-wave are studied for both appropriate link performance and modest implementation complexity of interference suppression.

(Copied from RP-240826 [1]) …. - RAN1-led: For the Ambient IoT DL and UL: … - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. …

In the RAN1#116 and RAN1#116bis meeting, the single-tone unmodulated sinusoid waveform and the multiple unmodulated single-tone waveform were agreed to be studied. The contiguous multi-tone OFDM waveform has been precluded for the Rel-19 study. In the RAN1#117 meeting, it is agreed to deprioritize N > 2 for the N multiple unmodulated single-tone waveform.

(Copied from R1-2401767 [2]) Agreement For R19 A-IoT study item, at least single-tone unmodulated sinusoid waveform is a candidate waveform for carrier wave for D2R backscattering. Agreement For CW waveform for D2R backscattering, contiguous multi-tone OFDM signal is not studied in R19 SI. (Copied from R1-2403663 [3]) Agreement For CW waveform for D2R backscattering, multiple unmodulated single-tone is studied compared to single-tone in R19 SI. - Two unmodulated single-tones as a starting point - FFS: Other number of tones - FFS: how large gap is needed between tones (Copied from R1-2405696 [4]) Agreement For multiple unmodulated single-tone transmitted by one CW node, other number of tones (i.e. >2) is deprioritized. - Note: other number of tones (i.e. >2) is studied only when obvious gains are provided.

This contribution analyzes and compares the characteristics of the two candidate carrier-wave waveforms, which are described in frequency domain. The considerations on the time and power domain characteristics of external carrier-wave are briefly discussed. To prioritize the more practical cases for coverage and coexistence evaluations, the spectrum for carrier-wave in different topologies is also discussed.

# Discussions on the two candidate waveforms for carrier-wave

The unmodulated multiple single-tone waveform can be viewed as the combination of a limited number (e.g., 2 tones) of unmodulated single-tones, where the same baseband signal will be modulated onto each tone in the backscatter modulation. To avoid overlapping between the signals on adjacent tones, the frequency gap should be no smaller than the transmission bandwidth of the D2R transmission.

In addition, the harmonics in the backscattered signal has to be considered. In principle, the power attenuation of each harmonic component follows the inverse square law. For example, the power of the 3rd harmonic is 10*log10(1/32) = -9.5 dB lower than the target signal in the main lobe. If the gap between the two single-tones is set to the triple of the single sideband bandwidth of the D2R signal, the SIR could be 9.5 dB. The harmonics may or may not produce non-negligible impact to the link performance, depending on the SNR threshold of the corresponding D2R transmissions.

Proposal 1: The frequency gap between the adjacent tones in the multiple unmodulated single-tone waveform should be no smaller than the transmission bandwidth of the corresponding D2R transmission. The impact of the harmonics in the backscattered signal has to be additionally considered, which can require larger frequency gap.

Figure 1: Spectrum of the unmodulated single-tone and two unmodulated single-tone waveform

According to the agreement of the RAN1#116bis meeting, the D2R reception performance, spectrum utilization of backscattered signal, CW interference suppression, and complexity of CW generation are to be studied for the two candidate carrier-wave waveforms, and the corresponding observations and/or comparisons are agreed to be summarized in the table agreed in the RAN1#117 meeting.

(Copied from R1-24036636 [3]) Agreement Study at least the following characteristics of unmodulated single-tone and multiple unmodulated single-tone CW waveforms for backscattering: - For D2R - Reception performance - Spectrum utilization of backscattered signal corresponding to the CW waveforms - CW interference suppression at D2R receiver - Including complexity and CW cancellation capability value/range (if any) - For scenarios ’A1’, ’A2’ and ’B’ - Relative complexity of CW generation (Copied from R1-2405696 [4]) Agreement For the study of characteristics of CW waveforms, the following table is adopted as a template for capturing observations. Note 1: Further row(s) can be added, if other CW waveform characteristic is agreed. CW waveform characteristics; Observations and/or comparisons of single-tone unmodulated sinusoid waveform without frequency hopping and two unmodulated single-tones waveform for backscattering; … (if any). D2R reception performance. Spectrum utilization of backscattered signal corresponding to the CW waveforms. CW interference suppression at D2R receiver. Relative complexity of CW generation. Note: For two unmodulated single-tones waveform, the two tones are transmitted from the same CW node..

According to the above, the following subsections compare the two candidate waveforms item by item, with a summary in the last subsection. The technical issues of the unmodulated single-tone waveform with frequency hopping are also discussed.

## Reception performance

In the RAN1#117 meeting, the following observation on the reception performance corresponding to the unmodulated single-tone and two unmodulated single-tones waveform is agreed.

(Copied from R1-2405696 [4]) Observation For D2R reception performance, - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform provides [X Y] dB frequency diversity gain in a fading channel, at least depending on the gap between the two tones and the channel’s coherence bandwidth. - Note: The total transmission power is assumed the same for single-tone unmodulated sinusoid waveform without frequency hopping and two unmodulated single-tones waveform. - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

Considering the constrained sampling clock frequency (e.g., ≤ 1.92 MHz) of Ambient IoT device, the maximum D2R transmission bandwidth is expected to be a few MHz (e.g., DSB bandwidth of 2.4 MHz), while the typical D2R transmission bandwidth could be several 10 or 100 kHz. Meanwhile, the RMS delay spread of a typical indoor multi-path channel is from a few 10 ns to few 100 ns, which corresponds to a coherence bandwidth at 10 MHz level. As the coherence bandwidth is significantly larger than the transmission bandwidth, the indoor multi-path channel is frequency-selective for D2R transmissions.

For unmodulated single-tone waveform

With the link-level simulation assumptions in Table 1, the D2R link performance of the carrier-wave with unmodulated single-tone waveform is shown in Figure 2 and Figure 3, which corresponds to the D2R receiver with 1 and 2 Rx antennas, respectively.

Table 1: Overall comparison of different types of waveforms

| Parameters | Assumptions |
| --- | --- |
| Channel model | TDL-A 30 ns for D1T1 TDL-D 30 ns for D2T2 |
| Rx antenna | 2 for the basestation in D1T1 1 for the intermediate UE D2T2 |
| SFO | 105 ppm |
| Data size | 96 bits |
| Device moving speed | 3 km/h |
| Transmission bandwidth | 15 kHz |
| Modulation | BPSK (the performance gap between OOK and BPSK is included in the link budget calculation) |
| FEC | Convolutional code with 1/3 code rate |
| Line code | Manchester |
| Repetitions | No |

For two unmodulated single-tone waveform

In addition to the link-level simulation assumptions in Table 1, the following two assumptions are considered for the carrier-wave with two unmodulated single-tone waveform.

- The two single-tones are assumed to be allocated at the two edges of a licensed FDD band, to maximumly obtain the frequency diversity gain. The NR channel bandwidth is usually 10 MHz or 20 MHz in licensed FDD bands. In the simulations, the frequency gap between the two single-tones is assumed to be 10 MHz, which is typical for sub-GHz bands.

- Assuming fixed transmit power for the carrier-wave transmitter, the two single-tones are expected to equally share the total transmit power, which means the power allocated to each single-tone should be half of the total transmit power.

The link-level simulation results corresponding to the D2R receiver with 1 and 2 Rx antennas is shown in Figure 2 and Figure 3, respectively.

Figure 2: Simulated D2R link performance for D2R receiver with 1 Rx antennas

Figure 3: Simulated D2R link performance for D2R receiver with 2 Rx antennas

For the case of 1 Rx antenna at the D2R receiver in the LOS (TDL-D) channel, Figure 2 shows that the two unmodulated single-tone waveform only obtains -0.2 dB and 0.7 dB gain at 10% and 1% BLER, respectively. The small frequency diversity gain is caused by the following two aspects.

- In the LOS (TDL-D) channel, the frequency selectivity is not as obvious as the NLOS (TDL-A) channel. Accordingly, the frequency diversity gain is limited in the case.

- Only non-coherent combining can be applied to the preamble and postamble of each D2R transmission. As the total transmit power of the CW node is equally divided between the two single-tones, the average SNR is degraded by 3 dB for the backscattered signal on each single-tone carrier, which further leads to performance degradation of timing acquisition and SFO estimation.

For the case of 2 Rx antenna at the D2R receiver in the NLOS (TDL-A) channel, 0.1 dB frequency diversity gain at 10% BLER is observed, as seen in Figure 3. Though the frequency selectivity of TDL-A channel becomes obvious, the spatial diversity gain obtained through the 2 Rx antenna largely compensates the loss of frequency diversity for the unmodulated single-tone carrier-wave. However, with both spatial and frequency diversity gain, the link performance of the two unmodulated single-tone waveform enjoys a steeper roll-off with SNR. It achieves 4.5 dB frequency diversity gain at 1% BLER, when compared to the unmodulated single-tone waveform.

Observation 1: Assuming a frequency gap of 10 MHz between the two tones of the carrier-wave with two unmodulated single-tone waveform, the observed frequency diversity gain in the InF-DH NLOS channel with an RMS delay spread of 30 ns is

- -0.2 dB and 0.7 dB at 10% and 1% BLER, respectively, for a D2R receiver with 1 Rx antennas in the TDL-D channel with an RMS delay spread of 30 ns

- 0.1 dB and 4.5 dB at 10% and 1% BLER, respectively, for a D2R receiver with 2 Rx antennas in the TDL-A channel with an RMS delay spread of 30 ns

when compared to the carrier-wave with unmodulated single-tone waveform.

It should be noted that the limited available bandwidth of 10 MHz or 20 MHz for the licensed FDD bands could limit the frequency diversity gain for the carrier-wave with multiple unmodulated single-tone waveform. In the indoor multi-path channel with shorter delay spread (e.g., 30 ns), the maximum frequency gap (10 MHz or 20 MHz) between the two tones is smaller than the theoretical coherence bandwidth (e.g., 33.3 MHz for 30 ns delay spread).

Observation 2: In the indoor multi-path channel with short delay spread (e.g., 10 ns-level), the frequency diversity gain for multiple unmodulated single-tone waveform is expected to be not fully obtained, as the maximum frequency gap (e.g., 10 MHz or 20 MHz) between the two tones in an FDD band could be smaller than the coherence bandwidth.

Table 2 summarizes the analysis of the two candidate waveforms and their respective reception performance of the backscattered signal.

Table 2: Comparison of CW waveforms w.r.t reception performance

| Type of waveform | Frequency diversity |
| --- | --- |
| Unmodulated single-tone | None |
| Two unmodulated single-tone | - -0.2 / 0.7 dB gain at 10% / 1% BLER for a reader with 1 Rx antenna in TDL-D channel - 0.1 / 4.5 dB gain at 10% / 1% BLER for a reader with 2 Rx antennas in TDL-A channel Note: assuming the frequency gap between the two tones is 10 MHz in a multipath channel with RMS delay spread of 30 ns. |

Proposal 2: Capture the following updates to the observation on CW waveform characteristics w.r.t. the D2R reception performance into the agreed template:

For D2R reception performance,

- Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform provides {-0.2, 0.7} dB / {0.1, 4.5} dB gain at {10%, 1%} BLER for a reader with 1 Rx / 2 Rx antenna in the TDL-D / TDL-A channel with 30 ns RMS delay spread[X Y] dB frequency diversity gain in a fading channel, with a gap of 10 MHz  between the two unmodulated single-tones at least depending on the gap between the two tones and the channel’s coherence bandwidth.

  - Note: The total transmission power is assumed the same for single-tone unmodulated sinusoid waveform without frequency hopping and two unmodulated single-tones waveform.

  - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

  - Note: For two unmodulated single-tones waveform, the frequency gap between the two single-tones is recommended to approach the coherence bandwidth (e.g., usually at 10 MHz level) of the indoor multi-path channel to obtain appropriate frequency diversity gain.

## Spectral utilization of backscattered signal

For unmodulated single-tone waveform

Since the product of two signals in the time domain corresponds to the convolution of them in the frequency domain, the bandwidth of the backscattered signal is equal to the sum of the bandwidth of the carrier-wave and the transmission bandwidth of the D2R transmission. As the bandwidth of a single-tone is negligible, the bandwidth of the backscattered signal approximately equals the transmission bandwidth of the D2R transmission.

Observation 3: For unmodulated single-tone carrier-wave waveform, the bandwidth of the backscattered signal approximately equals the transmission bandwidth of the D2R transmission.

For two unmodulated single-tone waveform

The baseband signal will be mapped onto both the two single-tones, as shown in Figure 4. The bandwidth of the backscattered signal is 2 times the transmission bandwidth of the D2R transmission.

Observation 4: For the two unmodulated single-tone carrier-wave waveform, the backscattered signal occupies at least 2 times the transmission bandwidth of the D2R transmission.

Figure 4: Spectrum of backscattered signal based on different carrier-wave waveform

It should be noted that the above assumes the unoccupied spectrum between each two adjacent single-tones can still be schedulable for e.g. NR. Otherwise, the spectrum efficiency will be further reduced. Table 3 summarizes the analysis of the two candidate waveforms and their respective spectrum utilization of the backscattered signal.

Table 3: Comparison of CW waveforms w.r.t spectral utilization of backscatter modulation

| Type of waveform | Utilized Spectrum |
| --- | --- |
| Unmodulated single-tone | Transmission bandwidth of the D2R transmission |
| Two unmodulated single-tone | 2 times the transmission bandwidth of the D2R transmission (assuming unoccupied gap can be scheduled for other purposes) |

Proposal 3: Capture the following observation on CW waveform characteristics w.r.t. the spectral utilization of backscatter modulation into the agreed template:

- For the two unmodulated single-tone waveform, the frequency gap between the adjacent tones should at least be equal to the coherence bandwidth (e.g., usually at 10 MHz level) of the indoor multi-path channel, which is usually much larger than the transmission bandwidth of the D2R transmissions, to obtain appropriate frequency diversity gain.

- The backscattered signal corresponding to the external carrier-wave with the two unmodulated single-tone waveform utilizes 2 times the frequency resources when compared to a single-tone waveform.

## Carrier-wave interference suppression at D2R receiver

In the RAN1#117 meeting, the following observation on the CW interference suppression is agreed.

(Copied from R1-2405696 [4]) Observation For CW interference suppression at D2R receiver, - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform: - Requires additional complexity if RF interference cancellation is used at least with CW waveform reconstruction. - Note: RF interference cancellation is needed when the received CW interference power exceeds the blocking threshold of the receiver - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

In the case that D2R backscattering is transmitted in the same carrier as the carrier-wave, the backscattered signal from an Ambient IoT device will be interfered by the simultaneously received carrier-wave from the carrier-wave transmitter. To achieve optimal D2R receiver sensitivity, a significant suppression to the received carrier-wave interference is needed. For example, assuming the carrier-wave signal is transmitted with 23 dBm power, an overall suppression of about 143 dB is required to achieve the residual interference power of -120 dBm.

### General interference suppression

The following interference suppression scheme can be applied to suppress the carrier-wave interference at the D2R receiver.

- If the carrier-wave source is separately deployed from the D2R receiver (e.g., in the ‘A1’ and ‘B’ scenarios), the spatial isolation between them can reduce the received interference power. For example, the path loss is 57 dB for an isolation distance of 20 meters, assuming free-space propagation. Considering the antenna gain of both the carrier-wave transmitter and the D2R receiver is usually negative in the direction directly connecting them, the overall equivalent path loss for the carrier-wave interference will be even higher.

If the carrier-wave source is co-deployed or integrated with the D2R receiver (e.g., in the ‘A2’ scenarios), spatial isolation can no longer be relied on for interference suppression. To compensate the loss, an additional circulator or directional coupler is usually used, which can provide an interference suppression of 40-60 dB [5][6].

- Interference cancellation in the RF front-end can be applied especially when the interference power exceeds the blocking threshold of receiver. For example, assuming 23 dBm transmit power for external carrier-wave and at least 57 dB spatial isolation between the carrier-wave transmitter and D2R receiver, the received carrier-wave interference power is (23-57) = -34 dBm, which is around the general blocking requirement of -35 dBm for micro-BSs [7]. In this case, RF interference cancellation may be applied, depending on the actual blocking performance of the BS. If transmit power higher than 23 dBm is used for carrier-wave, RF interference cancellation will probably be needed. In principle, it can be implemented by reconstructing a local carrier-wave according to the received interference, and subtracting it from the received signal. The performance heavily depends on the similarity between the reconstructed and actual received interference. Consequently, the optimal waveform for carrier-wave is the one which can be easily reconstructed according to the received carrier-wave interference.

- Digital baseband interference suppression usually provides the largest portion of suppression at the D2R receiver. In general, it can be done by filtering and/or reconstructing-then-subtracting the interference from the received signal. Filtering can be done in frequency domain or spatial domain. Frequency domain filtering depends on the characteristics of the carrier-wave waveform, while spatial domain filtering can be fulfilled by e.g. the classic MMSE-IRC method. The implementation of reconstructing-then-subtracting interference is usually more complicated, as it requires accurate modeling and estimation of both the carrier-wave waveform and the propagation channel.

Figure 5: Carrier-wave interference suppression at Ambient IoT uplink receiver

For both the interference cancellation in RF front-end and interference suppression in digital baseband, the characteristics of carrier-wave waveform obviously impact the performance and implementation complexity of D2R receiver.

### Unmodulated single-tone waveform

Unmodulated single-tone waveform has the advantage of both optimal performance and simple implementation.

- For RF interference cancellation, the implementation is similar to the analog self-interference cancellation circuit for full-duplex radios, which can reach higher than 20 dB interference suppression [8]. However, the implementation towards the simple unmodulated single-tone CW is expected to be much simpler comparing to conventional full-duplex radios. The local unmodulated single-tone carrier-wave signal can be conveniently reconstructed based on a short duration (e.g., ≤10 ms) of the received single-tone carrier-wave. Thanks to the simple mathematical expression of unmodulated single-tone waveform, the hypothesis testing can be done only with two parameters of amplitude and initial phase. By subtracting the reconstructed local single-tone carrier-wave from the received carrier-wave, the optimum values for the two parameters can be detected when the residual interference power is minimized or below a certain threshold.

A suppression of 10-30 dB can usually be expected for RF interference cancellation. Assuming different reference clock between the carrier-wave transmitter and D2R receiver (e.g., in D1T1-B scenario without wired connection between the basestation and the carrier-wave node), 10 dB or higher suppression can be achieved after an initial frequency calibration by e.g. Hz-level CFO estimation based on the received carrier-wave interference. Assuming the same reference clock between the carrier-wave transmitter and D2R receiver (e.g., in D1T1-A1/A2 scenario, or D1T1-B scenario with wired connection between the basestation and the carrier-wave node), higher interference suppression can be reached, such as 20 dB or higher.

- In the digital baseband, a significant suppression (e.g., >80 dBc) can be simply achieved by high-pass filtering, as the received single-tone carrier-wave is shifted to direct current (DC) at baseband. Figure 6 shows the amplitude-frequency response of a 33-order FIR high-pass filter. Regarding the bitwidth of 11-bits or 12-bits for the ADC of the D2R receiver, oversampling is an effective way to increase the resolution or equivalent bitwidth of ADC. As a general guideline, oversampling the ADC by a factor of four provides one additional bit of resolution, or a 6 dB increase in dynamic range [9][10]. For example, the sampling rate of the ADC usually exceeds 100 Msps. As the sampling rate is far beyond the transmission bandwidth of the D2R signal, the equivalent dynamic range is able to cover the interference suppression of 80 dBc for the digital high-pass filter.

Figure 6: Carrier-wave interference suppression at Ambient IoT uplink receiver

Observation 5: Unmodulated single-tone waveform based carrier-wave has the advantage of optimal performance and simple implementation for the interference suppression at the D2R receiver, as it can be simply done by baseband high-pass filtering and RF interference cancellation based on amplitude and initial phase hypothesis testing.

In the case of separated carrier-wave transmitter and D2R receiver (e.g., ~60 dB @ 25 m) in D1T1-A1 or D1T1-B, through spatial isolation, RF interference cancellation and high-pass filtering, the overall suppression to the carrier-wave interference can reach about (60+10+80) = 150 dB. For larger spatial isolation corresponding to longer distance between the carrier-wave transmitter and D2R receiver (e.g., ~70 dB @ 70 m), the carrier-wave interference can be even higher (e.g., 70+10+80 = 160 dB).

In the case of integrated carrier-wave transmitter and D2R receiver in D1T1-A2, with ~40 dB interference suppression provided by a circulator/directional coupler and/or the antenna with high isolation between Tx and Rx, the carrier-wave interference suppression can reach about (40+20+80) = 140 dB.

Considering the relatively lower capability of intermediate UE in D2T2, its carrier-wave interference suppression in various scenarios could be assumed to be 10 dB lower than a basestation.

Observation 6: The carrier-wave interference suppression capability of a D2R receiver can be

- For separated carrier-wave transmitter and D2R receiver in D1T1-A1 or D1T1-B, 150~160 dB for a basestation, depending on the distance between the carrier-wave transmitter and the D2R receiver.

- For integrated carrier-wave transmitter and D2R receiver D1T1-A2, 140 dB for a basestation.

- For an intermediate UE in D2T2, the interference suppression in various scenarios is assumed to be 10 dB lower than the basestation discussed above.

### Multiple unmodulated single-tone waveform

Regarding the multiple unmodulated single-tone waveform, more complicated RF interference cancellation is required, when compared to the unmodulated single-tone carrier-wave. The following implementations can be considered.

- One potential implementation is to apply RF narrowband band-pass filtering to the received RF signal, so as to separate the backscatter signals on each “single-tone carrier”. Then the reconstructing-then-subtracting processing for the unmodulated single-tone waveform can be reused for the filtered signal on each “single tone carrier”. In general, the overall implementation complexity is N times of that for the unmodulated single tone waveform. However, the filtering capability of the RF narrowband band-pass filter has to be considered. For example, given a fixed available RF bandwidth, more tones lead to smaller pass-band bandwidth (higher Q-value) and smaller guard band (steep roll-off) for each RF band-pass filter, both of which significantly increase the implementation complexity. In other words, RF band-pass filter is difficult to be implemented for the case of small frequency gap (e.g., 1 MHz @ 900 MHz band) between adjacent tones in the multiple unmodulated single-tone waveform.

- Another potential implementation is to perform hypothesis testing based on the combination of the amplitude and initial phase of the two single-tone carriers for local carrier-wave reconstruction. The complexity of hypothesis testing increases exponentially with the number of variables, which becomes 4 for the two unmodulated single-tone waveform. Comparing to the unmodulated single-tone waveform, the carrier-wave interference cancellation capability is also probably reduced due to the residual error of more variables. In general, this implementation is unsuitable for the carrier-wave with more than 2 tones.

At digital baseband, a similar or the same processing for unmodulated single-tone carrier-wave can be reused to the received signal on each tone in the carrier-wave. In general, a 2-cycle operation of the baseband receiver processing for the unmodulated two single-tone carrier-wave is required.

Observation 7: For the N unmodulated single-tone waveform, there are the following two optional implementations for the CW interference cancellation in the RF front-end:

- 2 RF chains supporting the reconstructing-then-subtracting interference cancellation, with the corresponding narrowband RF band-pass filters to subtract the RF signal and interference on each “single-tone” of the carrier-wave waveform

- Hypothesis testing based reconstructing-then-subtracting interference cancellation, which is too complicated for N > 2

- Meanwhile, an 2-cycle operation of the receiver baseband processing, where each cycle is for the processing of one “single-tone” in the carrier-wave waveform, is required.

Regarding the use of an intermediate UE in D2T2, the increased hardware and software complexity for the support of the multiple unmodulated single-tone waveform carrier-wave could be too high. In other words, it is recommended to assume only unmodulated single-tone waveform for the external carrier-wave in Topology 2.

Table 4 summarizes the analysis of the two candidate waveforms and their respective complexity of the carrier-wave interference suppression at the D2R receiver.

Table 4: Comparison of CW waveforms w.r.t performing CW interference suppression at D2R receiver

| Type of waveform | RF front-end interference cancellation complexity | Digital baseband interference suppression complexity |
| --- | --- | --- |
| Single-tone unmodulated sinusoid waveform | Low For reconstruction of local single-tone carrier-wave, hypothesis testing for only amplitude and initial phase can be used. | Low High-pass filtering can be used |
| Two unmodulated single-tone waveforms | Medium For reconstruction of local carrier-wave Option 1: Separate backscattered signal on each single-tone using RF narrowband band-pass filtering, requiring 2 RF chains. Option 2: Hypothesis testing with complexity exponentially increased with two single-tones in the waveform. | Medium Similar to unmodulated single-tone waveform for the backscattered signal on each “single-tone” in the waveform, but requires a I-cycle operation. |

Proposal 4: Capture the following updates to the observation on CW waveform characteristics w.r.t. the carrier-wave interference suppression at D2R receiver into the agreed template:

For CW interference suppression at D2R receiver,

- Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform:

  - Requires additional complexity if RF interference cancellation is used at least with CW waveform reconstruction.

    - Note: RF interference cancellation is needed when the received CW interference power exceeds the blocking threshold of the receiver

  - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

  - Note: The two unmodulated single-tones waveform can be too complicated to be supported by an intermediate UE in D2T2.

## CW generation complexity

In the RAN1#117 meeting, the following observation on the CW generation complexity is agreed.

(Copied from R1-2405696 [4]) Observation For relative complexity of CW generation - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform: - Leads to higher PAPR of the generated CW, which impacts the implementation of the power amplifier in the CW node. - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

From the perspective of baseband processing for the carrier-wave generation, both unmodulated single-tone and multiple unmodulated single-tone waveform are not complicated. One issue to be considered is the PAPR of the generated carrier-wave, which impacts the implementation of the power amplifier in the carrier-wave transmitter.

For unmodulated single-tone waveform

The unmodulated single-tone waveform enjoys the characteristic of constant envelope, which is beneficial to both simple implementation and high power-added efficiency (PAE) for a power amplifier.

For multiple unmodulated single-tone waveform

Assuming equal frequency gap between adjacent tones, the PAPRs of the multiple unmodulated single-tone waveform with different number of tones are simulated and listed in Table 5. In the simulations, the frequency gap between the leftmost and the rightmost tones is set to 10 MHz. More tones in the waveform leads to higher PAPR, which could cause power back-off especially for the case of reusing the existing hardware of e.g. basestation or UE to transmit the external carrier-wave.

Table 5: Simulated PAPRs of the multiple unmodulated single-tone waveform with different number of tones

| Number of tones | 2 | 3 | 4 | 6 | 8 |
| --- | --- | --- | --- | --- | --- |
| PAPR (dB) | 3.0 | 4.8 | 6.0 | 7.8 | 9.0 |

Observation 8: More tones in the CW waveform lead to higher PAPR, which may cause power back-off especially for the case of reusing existing hardware of e.g. basestation or UE to transmit the external carrier-wave.

Another potential issue with the two unmodulated single-tone carrier-wave transmitter is the interference generated by the intermodulation between the two unmodulated single-tones. As shown in Figure 7, the signal power is concentrated on the two single-tones in frequency domain, which could lead to IM3 interference with high PSD. To achieve sufficient frequency diversity gain, the gap between the two single-tones is expected to be several MHz or few 10s of MHz. In this case, the corresponding third-order intermodulation (IM3) interference could be out of the target band of Ambient IoT. For example, assuming the two single-tones with a gap of 9 MHz are located at the two sides of an NR band for in-band operation, there will be IM3 interference 9 MHz away from each single-tone, which are located at the adjacent bands on the two sides of the target NR band.

Figure 7: Potential intermodulation between the two unmodulated single-tone

Observation 9: The two unmodulated single-tone carrier-wave transmitter may generate out-of-band intermodulation interference, which requires additional interference suppression to meet the out-of-band emission requirements.

Table 6 summarizes the analysis of the two candidate waveforms and their respective CW generation complexity.

Table 6: Comparison of CW waveforms w.r.t CW generation complexity

| Type of waveform | PAPR |
| --- | --- |
| Unmodulated single-tone | Constant envelope Convenient for the CW transmitter to reach high PAE |
| Two unmodulated single-tone | Higher PAPR for more tones in the waveform, and 3dB PAPR for two single-tones Additional out-of-band interference suppression is needed to deal with the intermodulation interference generated by the two unmodulated single-tones More complicated and lower power efficiency for the PA implementation of CW transmitter |

Proposal 5: Capture the following update to the observation on CW waveform characteristics w.r.t. the CW generation complexity into the agreed template:

- Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform:

  - Leads to higher PAPR of the generated CW, which impacts the implementation of the power amplifier in the CW node.

  - Leads to additional out-of-band interference suppression to deal with the intermodulation interference generated by the two unmodulated single-tones.

  - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

## Unmodulated single-tone waveform with frequency hopping

To obtain frequency diversity for backscattering based D2R transmission, another theoretical option, besides the multiple unmodulated single-tone waveform for the external carrier-wave, is to apply frequency hopping to the external carrier-wave with unmodulated single-tone waveform. As discussed in our paper on device architecture [11], a buffer exceeding a certain size (e.g., around or larger than 100 bits) is too complicated and power consuming for Device 1. Consequently, interleaving is assumed not to be supported by Ambient IoT devices. Without interleaving, frequency hopping is expected to be used together with repetition to obtain frequency diversity, by carrying the different repetitions of each bit or block on different carrier frequency.

However, the following two aspects need to be considered to justify the applicability and benefit of applying frequency hopping to the unmodulated single-tone waveform.

- The frequency of the external carrier-wave is transparent to Ambient IoT devices, as a device only needs to control the switching of its impedance matching circuit according to the D2R baseband signal. In other words, the frequency hopping of the external carrier-wave is unperceivable to devices.

- Considering the inaccurate frequency of the extreme-low power clock in Ambient IoT devices, the timing is not synchronized between the reader and the device. The overall timing unalignment includes both the unpredictable processing latency of each device and the accumulated timing offset during the D2R transmission.

The above two aspects determine that it is difficult to align the boundary of each hop of the external carrier-wave with its corresponding bit/block repetition of the D2R transmission.

Observation 10: The unknown timing offset of each D2R transmission makes it hard to align the boundary of each hop of the external carrier-wave with the corresponding segment of the D2R transmission.

From the perspective of D2R receiver, the RF interference cancellation usually requires a certain duration to detect the amplitude and phase of the received carrier-wave interference, which could be at e.g. ms-level. When the frequency of the external carrier-wave changes, the amplitude and phase of the received carrier-wave interference would probably change as well. Hence, the receiver has to complete the detection of the single tone CW in the first frequency before hopping to the second frequency, otherwise, RF interference cancellation cannot work appropriately. It will obviously impact the receiver performance, and may even lead to blocking of the CW transmitted in the second frequency by the D2R receiver [7].

Observation 11: For the RF interference cancellation at the D2R receiver, the frequency hopping of external carrier-wave requires the re-detection of the parameters of the received carrier-wave interference.

Proposal 6: The unmodulated single-tone waveform with frequency hopping is not studied in Rel-19.

## Summary

Based on the analysis in the previous subsections, the comparisons between the unmodulated single-tone waveform without frequency hopping and the two unmodulated single-tones waveform are summarized in Table 7.

Table 7: Comparisons between the unmodulated single-tone waveform without frequency hopping and the two unmodulated single-tones waveform

| CW waveform characteristics | Observations and/or comparisons of single-tone unmodulated sinusoid waveform without frequency hopping and two unmodulated single-tones waveform for backscattering |
| --- | --- |
| D2R reception performance | For D2R reception performance, - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform provides {-0.2, 0.7} dB / {0.1, 4.5} dB gain at {10%, 1%} BLER for a reader with 1 Rx / 2 Rx antenna in the TDL-D / TDL-A channel with 30 ns RMS delay spread[X Y] dB frequency diversity gain in a fading channel, with a gap of 10 MHz between the two unmodulated single-tones at least depending on the gap between the two tones and the channel’s coherence bandwidth. - Note: The total transmission power is assumed the same for single-tone unmodulated sinusoid waveform without frequency hopping and two unmodulated single-tones waveform. - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node. - Note: For two unmodulated single-tones waveform, the frequency gap between the two single-tones is recommended to approach the coherence bandwidth (e.g., usually at 10 MHz level) of the indoor multi-path channel to obtain appropriate frequency diversity gain. |
| Spectrum utilization of backscattered signal corresponding to the CW waveforms | - For the two unmodulated single-tone waveform, the frequency gap between the adjacent tones should at least be equal to the coherence bandwidth (e.g., usually at 10 MHz level) of the indoor multi-path channel, which is usually much larger than the transmission bandwidth of the D2R transmissions, to obtain appropriate frequency diversity gain. - The backscattered signal corresponding to the external carrier-wave with the two unmodulated single-tone waveform utilizes 2 times the frequency resources when compared to a single-tone waveform. |
| CW interference suppression at D2R receiver | For CW interference suppression at D2R receiver, - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform: - Requires additional complexity if RF interference cancellation is used at least with CW waveform reconstruction. - Note: RF interference cancellation is needed when the received CW interference power exceeds the blocking threshold of the receiver - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node. - Note: The two unmodulated single-tones waveform can be too complicated to be supported by an intermediate UE in D2T2. |
| Relative complexity of CW generation | - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform: - Leads to higher PAPR of the generated CW, which impacts the implementation of the power amplifier in the CW node. - Leads to additional out-of-band interference suppression to deal with the intermodulation interference generated by the two unmodulated single-tones. - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node. |
| Note: For two unmodulated single-tones waveform, the two tones are transmitted from the same CW node. |  |

Proposal 7: Capture the comparisons between the unmodulated single-tone waveform without frequency hopping and the two unmodulated single-tones waveform in Table 7 into the TR.

From the above, it is seen that the link performance gain potentially provided by the two unmodulated single-tones waveform for external carrier-wave also lead to doubled frequency resource utilization and carrier-wave interference suppression complexity. The corresponding complicated RF interference cancellation may even not be supported by an intermediate UE. Consequently, unmodulated single-tone waveform should be the baseline for the external carrier-wave.

Proposal 8: The unmodulated single-tone waveform without frequency hopping is regarded as the baseline for the external carrier-wave.

# Discussions on the time, frequency and power domain characteristics of carrier-wave

One set of characteristics that needs to be studied with respect to the CW waveform are those that can be controlled by the CW node. This includes the timing related aspects of when the CW would be transmitted, the resources used in the frequency domain to transmit the CW and at what power. It has to be noted that while RAN1 is tasked with studying the characteristics that would need control of the CW node, the exact procedures or methods of how the CW node controls these characteristics, such as signaling details, are not in the scope of the study.

Observation 12: While RAN1 is tasked with studying the characteristics that would need control of the CW node, the exact procedures or methods of how the CW node controls these characteristics, such as signaling details, are beyond the scope of the study.

## Time domain characteristics

Regarding the question of when the external carrier-wave will be provided to devices, there can be two options described as follows.

- Dynamic CW on-off: carrier-wave is assumed to be present before the start of each D2R transmission, and stops after the end of each D2R transmission.

- Semi-static CW on-off: carrier-wave is assumed to be present before the start of an inventory procedure, and stops after the end of the inventory procedure.

For the case of R2D and D2R in the same band (e.g., both in the FDD DL or UL band), the dynamic CW on-off option has to be used, as the semi-static CW on-off option will cause CW interference to the RF-ED receiver of the devices.

For the case of R2D and D2R in different bands (e.g., R2D/D2R in the FDD DL/UL band, respectively), the semi-static CW on-off can also be considered. More efficient and convenient implementation is expected for this option, as no precise “On-Off” switching is needed for the carrier-wave during each inventory.

Proposal 9: The carrier-wave time domain characteristics consider the following two options:

- Dynamic CW on-off: carrier-wave is assumed to be present before the start of each D2R transmission, and stopped after the end of each D2R transmission.

- Semi-static CW on-off: carrier-wave is assumed to be present before the start of an inventory procedure, and stopped after the end of the inventory procedure.

## Frequency domain characteristics

For the frequency resources that are to be used to transmit the carrier-wave, it would be irrelevant to the device since the frequency of the carrier-wave is transparent to the device itself. It has to be understood that details such as how this information is provided to the CW node would also fall outside the scope of the study. However, the frequency to be used for the CW transmission is a characteristic that could be controlled by the CW node.

For a single tone waveform, the only relevant aspect here to be controlled would be the center frequency used by the carrier-wave. For the two single tones waveform, the center frequencies for each of the two tones as well as the frequency gap between the two single-tones could also be controlled, where the frequency gap is ensured to be no smaller than the transmission bandwidth of the corresponding D2R transmission, as discussed in Section 2, and close to the coherence bandwidth of the indoor multi-path channel, as discussed in Section 2.1.

Proposal 10: The carrier-wave frequency domain characteristics consider the following aspects:

- For the unmodulated single-tone carrier-wave waveform, center frequency.

- For the two unmodulated single-tone waveform, center frequencies of the two tones and frequency gap between them.

## Power domain characteristics

Regarding the transmit power of carrier-wave, both the carrier-wave interference to D2R receiver and to NR basestation or UE have to be considered. For the D2R receiver, the received carrier-wave interference with too high power may cause blocking, if the residual interference power after RF carrier-wave interference cancellation still exceeds the blocking threshold. For NR basestation or UE, in the case of subcarrier orthogonality between received carrier-wave and NR signal being maintained, the carrier-wave interference power should be kept below the blocking threshold.

From the perspective of spectrum regulations, the transmit power of carrier-wave is assumed not to exceed the allowed transmit power in the corresponding spectrum. The existing regulations and specifications of the target spectrum(s) can be referred as the start point.

As the external carrier-wave may be provided by an intermediate UE, the transmit power of the carrier-wave is assumed not to exceed the supported maximum transmit power of the UE in the corresponding spectrum, so as to achieve better hardware reusage.

Proposal 11: The transmit power of carrier-wave considers the following aspects:

- Interference to D2R receiver (RAN1-led)

- Interference to NR basestation or UE (RAN4-led)

- Maximum permitted transmit power in the corresponding spectrum (RAN4-led)

- CW transmitter capability (RAN1-led)

# Prioritized spectrum for carrier-wave

To understand the impact of carrier-wave interference to the receiver of some network element(s) of Ambient IoT or NR, the spectrum for external carrier-wave needs to be identified first. In the RAN1 #116 meeting, the following agreements have been reached on external carrier wave for backscatter modulation in the D2R link.

(Copied from R1-2401767 [2]) Agreement For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 1, the following cases for CW transmission are studied. - Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum - Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum - Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum Agreement For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for topology 2, the following cases for CW transmission are studied. - Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum - Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum - Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum

## Topology 1

For Topology 1, the following 3 cases are to be further studied according to the corresponding agreement.

- Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum

- Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum

- Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum

CW is transmitted from inside the topology (Case 1-1 and 1-2)

Regarding Case 1-1 and 1-2, the external carrier-wave is transmitted by the BS due to the constraint of the carrier-wave being transmitted inside Topology 1. Considering both the existing regulations and the higher legal transmit power, an Ambient IoT BS is recommended to transmit carrier-wave in downlink spectrum as the relevant case for study in Rel-19.

Figure 8: Backscattering without FDD frequency shifting, CW transmitted from inside Topology 1

In the discussions during the previous RAN1 meetings, while existing regulatory restrictions would prevent the BS from transmitting in the DL spectrum, IAB was mentioned as an example of BS transmitting in uplink spectrum. However, the example is not precise and it has to be noted that the procedure and circumstances in which the IAB node operates is quite unique. Firstly, IAB works only in TDD bands, as defined in the section 5.2 of TS 38.174 [12]. Secondly, the mobile terminal (MT) part of the IAB node transmits to the NR BS or parent node in the uplink slots, for which timing advance and the same requirement on frequency error as that of an NR UE are applied [12]. In other words, the IAB node is regarded as a MT when it transmits to BS or parent node in the uplink slot of TDD spectrum for the backhaul link [13], which is quite different from the case of Ambient IoT BS transmitting carrier-wave in the uplink band of FDD spectrum. From the above analysis, Case 1-1 is preferred, where the CW is transmitted in the DL spectrum.

(Copied from TS 38.174 [12]) 5.2 Operating bands NR IAB is designed to operate in the operating bands in FR1 defined in table 5.2-1 and operating bands in FR2-1 defined in 38.104 [2]. Table 5.2-1 NR IAB operating bands in FR1 NR operating band; Uplink (UL) operating band BS receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band BS transmit / UE receive FDL,low – FDL,high; Duplex Mode. n41; 2496 MHz – 2690 MHz; 2496 MHz – 2690 MHz; TDD. n77; 3300 MHz – 4200 MHz; 3300 MHz – 4200 MHz; TDD. n78; 3300 MHz – 3800 MHz; 3300 MHz – 3800 MHz; TDD. n79; 4400 MHz – 5000 MHz; 4400 MHz – 5000 MHz; TDD. (Copied from TS 38.300 [13]) 4.7.2 Protocol Stacks … Protocol stacks for an IAB-donor with split gNB architecture are specified in TS 38.401 [4]. Fig. 4.7.2-1: Protocol stack for the support of F1-U protocol Fig. 4.7.2-2: Protocol stack for the support of F1-C protocol

Another observation made by companies is that in cases 1-1 and 1-2, the BS would require full-duplex capabilities. This would depend on the deployment scenario in which the devices are used, since in D1T1-A1, the BS that provides the carrier-wave is different from the reader receiving the D2R transmissions, and hence would not require full duplex support, immaterial of whether the transmission is in the uplink or downlink spectrum. At the same time, the D1T1-A2 deployment scenario would require the BS to support full duplex capabilities for either of the cases where the CW is transmitted in uplink or downlink spectrum. Hence, in our opinion, this aspect should not be used to compare different CW transmission cases to determine the transmission spectrum.

Yet another discussion point was the need for a separate RF chain at the BS in order to support case 1-1 and D2R transmissions being received in the DL spectrum. It has to be noted that at the BS, the most components in the RF chain can support reception from both the DL and UL spectrum, and would require only a few filters to be changed for the BS to support reception on the UL spectrum. On the other hand, while case 1-2 would not require any changes to the receive RF chain on the UL spectrum, the BS would require additional hardware for transmitting the CW in the UL spectrum. It is possible that the hardware impact of the transmitter in the UL band can be even larger than of the receiver in the DL band.

For the transmission of D2R in the DL spectrum, while existing regulations do not cover such a scenario, it is our understanding that the D2R backscattered transmissions would be of a much lower, and hence would not cause interference to the other transmissions in the DL spectrum. Since regulatory issues like this pertain to the interference to other RATs, such consistence evaluations are expected to be handled by RAN4.

CW is transmitted from outside the topology (Case 1-4)

Regarding Case 1-4, a carrier-wave node outside Topology 1 is introduced to transmit carrier-wave in the uplink spectrum. Correspondingly, the BS also receives the backscattered signal in the uplink spectrum, which benefits its implementation from the perspective of hardware reusing for uplink receiver.

Figure 9: Backscattering without FDD frequency shifting, CW transmitted outside Topology 1

Another advantage of having the carrier-wave being transmitted by an external carrier-wave node is that spatial isolation can be achieved at the BS receiver, reducing the received interference power, where, as mentioned earlier, the path loss can be 57 dB for an isolation distance of 20 meters. For the inside topology cases, this can be achieved for certain deployment scenarios where the BS transmitting the CW is different from the reader.

Observation 13: For the case of CW inside D1T1, BS transmitting the CW in DL spectrum is more suitable from the perspective of higher compliance with spectrum regulations and higher permitted transmit power.

Proposal 12: The study on case 1-2 is concluded by capturing the following in the TR:

- Existing regulatory restrictions prevent the BS from transmitting in UL spectrum.

Proposal 13: For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for Topology 1, the following cases for CW transmission are prioritized for evaluation purposes.

- Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum

- Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum

Table 8 summarizes the pros and cons of the different CW transmission cases for topology 1.

Table 8: Comparison of different CW transmission cases for Topology 1

| CW Transmission case | Advantages | Disadvantages |
| --- | --- | --- |
| Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum | - CW transmission by BS in DL spectrum is in-line with existing spectrum regulations. - Low power D2R backscattered transmissions would not interfere with other R2D transmissions in the DL spectrum. | - Spatial isolation is only possible when the BS transmitting the CW is different from the reader. |
| Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum | - No changes required at BS to receive D2R transmissions in UL spectrum. | - CW transmission by BS in UL spectrum is not allowed as per existing spectrum regulations. - Additional hardware required for transmitting CW in UL spectrum. - Spatial isolation is only possible when the BS transmitting the CW is different from the reader. |
| Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum | - D2R transmissions by device in UL spectrum is in-line with existing spectrum regulations. - No changes required at BS to receive D2R transmissions in UL spectrum. - Spatial isolation is possible, reducing the received interference power. |  |

Proposal 14: Capture the comparison of the different CW transmission cases for Topology 1 in Table 8 into the TR.

## Topology 2

For Topology 2, the following 3 cases are to be further studied according to the corresponding agreement.

- Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum

- Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum

- Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum

CW is transmitted from inside the topology (Case 2-2)

Regarding Case 2-2, it is natural for the intermediate UE to transmit the carrier-wave in uplink spectrum, considering the existing spectrum regulations. From the perspective of D2R receiver implementation at the intermediate UE, it could be difficult to reuse the existing receiver in legacy NR UEs even without considering the spectrum issue, due to the carrier-wave interference suppression and the processing of highly probable non-OFDM signals for D2R transmissions. An independent D2R receiver implementation in the uplink spectrum is expected to be optimal for the coverage performance of Ambient IoT in D2T2.

Figure 10: Backscattering without FDD frequency shifting, CW transmitted from inside Topology 2

CW is transmitted from inside the topology (Case 2-3 and 2-4)

Regarding Case 2-3 and 2-4, an additional node outside Topology 2 is introduced to transmit the carrier-wave. Considering carrier-wave is transmitted in the uplink spectrum for Case 2-2, it is reasonable and beneficial to reuse the uplink receiver of intermediate UE for both cases where the carrier-wave node is inside and outside D2T2. Consequently, the carrier-wave node is recommended to also transmit carrier-wave in uplink spectrum. If Case 2-3 is assumed, the intermediate UE has to support a D2R receiver implementation in downlink spectrum for Case 2-3 as well as uplink spectrum to support case 2-2, increasing the complexity of the intermediate UE. Clearly, Case 2-4 is preferable.

Another advantage of using an external carrier-wave node is the spatial isolation that it offers, as previously stated in Topology 1 in the previous section and in Section 2.3.1, while it is applicable to the inside topology case only for certain deployment scenarios where the BS transmitting the CW is different from the reader.

Figure 11: Backscattering without FDD frequency shifting, CW transmitted from outside Topology 2

Observation 14: For the case of CW outside D2T2, carrier-wave node transmitting the CW in UL spectrum is beneficial for reusing the D2R receiver implementation at the intermediate UE for the case of CW inside D2T2.

Proposal 15: The study on case 2-3 is concluded by capturing the following in the TR:

- Intermediate UE has to support different D2R receiver implementations in uplink and downlink spectrum, increasing UE complexity.

Proposal 16: For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for Topology 2, the following cases for CW transmission are prioritized for evaluation purposes.

- Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum

- Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum

Table 9 summarizes the pros and cons of the different CW transmission cases for topology 2.

Table 9: Comparison of different CW transmission cases for Topology 2

| CW Transmission case | Advantages | Disadvantages |
| --- | --- | --- |
| Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum | - CW transmission by intermediate UE in UL spectrum is in-line with existing spectrum regulations. - D2R transmissions by device in UL spectrum is in-line with existing spectrum regulations. | - Spatial isolation is only possible when the intermediate UE transmitting the CW is different from the reader. |
| Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum | - Spatial isolation is possible, reducing the received interference power. | - Increased complexity of the intermediate UE since it would be required to support reception on the DL spectrum and UL spectrum, to support the CW being transmitted from inside the topology. |
| Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum | - D2R transmissions by device in UL spectrum is in-line with existing spectrum regulations. - Spatial isolation is possible, reducing the received interference power. |  |

Proposal 17: Capture the comparison of the different CW transmission cases for Topology 2 in Table 9 into the TR.

## Device capability of supporting multiple bands for backscattering

For the case of D2R backscattered signal being in the same carrier as the carrier-wave for D2R backscattering, Ambient IoT devices perform backscattering either in downlink spectrum e.g. in Case 1-1, or in uplink spectrum e.g. in Case 1-4. From the view of device implementation, the support of backscattering in different bands by the same device is technically feasible and mature.

- One way is to support broadband operations for Ambient IoT devices, similar to the existing UHF RFID tags. For example, the spectrum for UHF RFID is usually allocated within two main frequency ranges all over the world, which are 902-928 MHz and 865-868 MHz [14]. Most of the existing UHF RFID tags support the entire operating frequency range of 860-960 MHz, as specified in [15]. The key point is to properly design a broadband tag antenna with desired performance over the worldwide operating frequency range [16][17].

- The second way is to support multi-frequency characteristics for Ambient IoT devices. Referring to the related researches on the dual-band antenna for UHF RFID tag, the antenna can operate in the two main frequency ranges simultaneously, by “increasing the parasitic branch, coupling resonance and the load reactance as well as other technologies” [17][18]. With proper design, the tag antenna can even support two bands with much larger frequency interval, such as the UHF RFID band of 855-935 MHz and the ISM band of 2.4-2.5 GHz [19].

- Except the above broadband or dual-band antenna design, there can also be other solutions to support multiple bands for device. For example, the RFID tag with dual-frequency IC is manufactured with a single chip but two independent antennas [20]. The overhead of such implementation can be small, as only one additional antenna is needed. In general, this kind of solution is more suitable to support two FDD bands with a large frequency gap between each other (e.g., 900 MHz and 1.8 GHz).

From the above, the support of working in both downlink and uplink spectrum by a single Ambient IoT device is feasible, with little impact on device power consumption and cost.

Observation 15: It is feasible for the same Ambient IoT device to support backscattering on both the downlink and uplink spectrum, with modest impact on device power consumption and complexity.

# Conclusions

In this contribution, the suitable spectrum and waveform characteristics for the external carrier-wave are discussed. It is recommended to capture the following observations, together with the proposals, in the TR.

Observation 1: Assuming a frequency gap of 10 MHz between the two tones of the carrier-wave with two unmodulated single-tone waveform, the observed frequency diversity gain in the InF-DH NLOS channel with an RMS delay spread of 30 ns is

- -0.2 dB and 0.7 dB at 10% and 1% BLER, respectively, for a D2R receiver with 1 Rx antennas in the TDL-D channel with an RMS delay spread of 30 ns

- 0.1 dB and 4.5 dB at 10% and 1% BLER, respectively, for a D2R receiver with 2 Rx antennas in the TDL-A channel with an RMS delay spread of 30 ns

when compared to the carrier-wave with unmodulated single-tone waveform.

Observation 2: In the indoor multi-path channel with short delay spread (e.g., 10 ns-level), the frequency diversity gain for multiple unmodulated single-tone waveform is expected to be not fully obtained, as the maximum frequency gap (e.g., 10 MHz or 20 MHz) between the two tones in an FDD band could be smaller than the coherence bandwidth.

Observation 3: For unmodulated single-tone carrier-wave waveform, the bandwidth of the backscattered signal approximately equals the transmission bandwidth of the D2R transmission.

Observation 4: For the two unmodulated single-tone carrier-wave waveform, the backscattered signal occupies at least 2 times the transmission bandwidth of the D2R transmission.

Observation 5: Unmodulated single-tone waveform based carrier-wave has the advantage of optimal performance and simple implementation for the interference suppression at the D2R receiver, as it can be simply done by baseband high-pass filtering and RF interference cancellation based on amplitude and initial phase hypothesis testing.

Observation 6: The carrier-wave interference suppression capability of a D2R receiver can be

- For separated carrier-wave transmitter and D2R receiver in D1T1-A1 or D1T1-B, 150~160 dB for a basestation, depending on the distance between the carrier-wave transmitter and the D2R receiver.

- For integrated carrier-wave transmitter and D2R receiver D1T1-A2, 140 dB for a basestation.

- For an intermediate UE in D2T2, the interference suppression in various scenarios is assumed to be 10 dB lower than the basestation discussed above.

Observation 7: For the N unmodulated single-tone waveform, there are the following two optional implementations for the CW interference cancellation in the RF front-end:

- 2 RF chains supporting the reconstructing-then-subtracting interference cancellation, with the corresponding narrowband RF band-pass filters to subtract the RF signal and interference on each “single-tone” of the carrier-wave waveform

- Hypothesis testing based reconstructing-then-subtracting interference cancellation, which is too complicated for N > 2

- Meanwhile, an 2-cycle operation of the receiver baseband processing, where each cycle is for the processing of one “single-tone” in the carrier-wave waveform, is required.

Observation 8: More tones in the CW waveform lead to higher PAPR, which may cause power back-off especially for the case of reusing existing hardware of e.g. basestation or UE to transmit the external carrier-wave.

Observation 9: The two unmodulated single-tone carrier-wave transmitter may generate out-of-band intermodulation interference, which requires additional interference suppression to meet the out-of-band emission requirements.

Observation 10: The unknown timing offset of each D2R transmission makes it hard to align the boundary of each hop of the external carrier-wave with the corresponding segment of the D2R transmission.

Observation 11: For the RF interference cancellation at the D2R receiver, the frequency hopping of external carrier-wave requires the re-detection of the parameters of the received carrier-wave interference.

Observation 12: While RAN1 is tasked with studying the characteristics that would need control of the CW node, the exact procedures or methods of how the CW node controls these characteristics, such as signaling details, are beyond the scope of the study.

Observation 13: For the case of CW inside D1T1, BS transmitting the CW in DL spectrum is more suitable from the perspective of higher compliance with spectrum regulations and higher permitted transmit power.

Observation 14: For the case of CW outside D2T2, carrier-wave node transmitting the CW in UL spectrum is beneficial for reusing the D2R receiver implementation at the intermediate UE for the case of CW inside D2T2.

Observation 15: It is feasible for the same Ambient IoT device to support backscattering on both the downlink and uplink spectrum, with modest impact on device power consumption and complexity.

Proposal 1: The frequency gap between the adjacent tones in the multiple unmodulated single-tone waveform should be no smaller than the transmission bandwidth of the corresponding D2R transmission. The impact of the harmonics in the backscattered signal has to be additionally considered, which can require larger frequency gap.

Proposal 2: Capture the following updates to the observation on CW waveform characteristics w.r.t. the D2R reception performance into the agreed template:

For D2R reception performance,

- Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform provides {-0.2, 0.7} dB / {0.1, 4.5} dB gain at {10%, 1%} BLER for a reader with 1 Rx / 2 Rx antenna in the TDL-D / TDL-A channel with 30 ns RMS delay spread[X Y] dB frequency diversity gain in a fading channel, with a gap of 10 MHz  between the two unmodulated single-tones at least depending on the gap between the two tones and the channel’s coherence bandwidth.

  - Note: The total transmission power is assumed the same for single-tone unmodulated sinusoid waveform without frequency hopping and two unmodulated single-tones waveform.

  - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

  - Note: For two unmodulated single-tones waveform, the frequency gap between the two single-tones is recommended to approach the coherence bandwidth (e.g., usually at 10 MHz level) of the indoor multi-path channel to obtain appropriate frequency diversity gain.

Proposal 3: Capture the following observation on CW waveform characteristics w.r.t. the spectral utilization of backscatter modulation into the agreed template:

- For the two unmodulated single-tone waveform, the frequency gap between the adjacent tones should at least be equal to the coherence bandwidth (e.g., usually at 10 MHz level) of the indoor multi-path channel, which is usually much larger than the transmission bandwidth of the D2R transmissions, to obtain appropriate frequency diversity gain.

- The backscattered signal corresponding to the external carrier-wave with the two unmodulated single-tone waveform utilizes 2 times the frequency resources when compared to a single-tone waveform.

Proposal 4: Capture the following updates to the observation on CW waveform characteristics w.r.t. the carrier-wave interference suppression at D2R receiver into the agreed template:

For CW interference suppression at D2R receiver,

- Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform:

  - Requires additional complexity if RF interference cancellation is used at least with CW waveform reconstruction.

    - Note: RF interference cancellation is needed when the received CW interference power exceeds the blocking threshold of the receiver

  - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

  - Note: The two unmodulated single-tones waveform can be too complicated to be supported by an intermediate UE in D2T2.

Proposal 5: Capture the following update to the observation on CW waveform characteristics w.r.t. the CW generation complexity into the agreed template:

- Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform:

  - Leads to higher PAPR of the generated CW, which impacts the implementation of the power amplifier in the CW node.

  - Leads to additional out-of-band interference suppression to deal with the intermodulation interference generated by the two unmodulated single-tones.

  - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node.

Proposal 6: The unmodulated single-tone waveform with frequency hopping is not studied in Rel-19.

Proposal 7: Capture the comparisons between the unmodulated single-tone waveform without frequency hopping and the two unmodulated single-tones waveform in Table 7 into the TR.

Table 7: Comparisons between the unmodulated single-tone waveform without frequency hopping and the two unmodulated single-tones waveform

| CW waveform characteristics | Observations and/or comparisons of single-tone unmodulated sinusoid waveform without frequency hopping and two unmodulated single-tones waveform for backscattering |
| --- | --- |
| D2R reception performance | For D2R reception performance, - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform provides {-0.2, 0.7} dB / {0.1, 4.5} dB gain at {10%, 1%} BLER for a reader with 1 Rx / 2 Rx antenna in the TDL-D / TDL-A channel with 30 ns RMS delay spread[X Y] dB frequency diversity gain in a fading channel, with a gap of 10 MHz between the two unmodulated single-tones at least depending on the gap between the two tones and the channel’s coherence bandwidth. - Note: The total transmission power is assumed the same for single-tone unmodulated sinusoid waveform without frequency hopping and two unmodulated single-tones waveform. - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node. - Note: For two unmodulated single-tones waveform, the frequency gap between the two single-tones is recommended to approach the coherence bandwidth (e.g., usually at 10 MHz level) of the indoor multi-path channel to obtain appropriate frequency diversity gain. |
| Spectrum utilization of backscattered signal corresponding to the CW waveforms | - For the two unmodulated single-tone waveform, the frequency gap between the adjacent tones should at least be equal to the coherence bandwidth (e.g., usually at 10 MHz level) of the indoor multi-path channel, which is usually much larger than the transmission bandwidth of the D2R transmissions, to obtain appropriate frequency diversity gain. - The backscattered signal corresponding to the external carrier-wave with the two unmodulated single-tone waveform utilizes 2 times the frequency resources when compared to a single-tone waveform. |
| CW interference suppression at D2R receiver | For CW interference suppression at D2R receiver, - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform: - Requires additional complexity if RF interference cancellation is used at least with CW waveform reconstruction. - Note: RF interference cancellation is needed when the received CW interference power exceeds the blocking threshold of the receiver - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node. - Note: The two unmodulated single-tones waveform can be too complicated to be supported by an intermediate UE in D2T2. |
| Relative complexity of CW generation | - Compared to single-tone unmodulated sinusoid waveform without frequency hopping, two unmodulated single-tones waveform: - Leads to higher PAPR of the generated CW, which impacts the implementation of the power amplifier in the CW node. - Leads to additional out-of-band interference suppression to deal with the intermodulation interference generated by the two unmodulated single-tones. - Note: For two unmodulated single-tones waveform, assume the two tones are transmitted from the same CW node. |
| Note: For two unmodulated single-tones waveform, the two tones are transmitted from the same CW node. |  |

Proposal 8: The unmodulated single-tone waveform without frequency hopping is regarded as the baseline for the external carrier-wave.

Proposal 9: The carrier-wave time domain characteristics consider the following two options:

- Dynamic CW on-off: carrier-wave is assumed to be present before the start of each D2R transmission, and stopped after the end of each D2R transmission.

- Semi-static CW on-off: carrier-wave is assumed to be present before the start of an inventory procedure, and stopped after the end of the inventory procedure.

Proposal 10: The carrier-wave frequency domain characteristics consider the following aspects:

- For the unmodulated single-tone carrier-wave waveform, center frequency.

- For the two unmodulated single-tone waveform, center frequencies of the two tones and frequency gap between them.

Proposal 11: The transmit power of carrier-wave considers the following aspects:

- Interference to D2R receiver (RAN1-led)

- Interference to NR basestation or UE (RAN4-led)

- Maximum permitted transmit power in the corresponding spectrum (RAN4-led)

- CW transmitter capability (RAN1-led)

Proposal 12: The study on case 1-2 is concluded by capturing the following in the TR:

- Existing regulatory restrictions prevent the BS from transmitting in UL spectrum.

Proposal 13: For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for Topology 1, the following cases for CW transmission are prioritized for evaluation purposes.

- Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum

- Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum

Proposal 14: Capture the comparison of the different CW transmission cases for Topology 1 in Table 8 into the TR.

Table 8: Comparison of different CW transmission cases for Topology 1

| CW Transmission case | Advantages | Disadvantages |
| --- | --- | --- |
| Case 1-1: CW is transmitted from inside the topology, transmitted in DL spectrum | - CW transmission by BS in DL spectrum is in-line with existing spectrum regulations. - Low power D2R backscattered transmissions would not interfere with other R2D transmissions in the DL spectrum. | - Spatial isolation is only possible when the BS transmitting the CW is different from the reader. |
| Case 1-2: CW is transmitted from inside the topology, transmitted in UL spectrum | - No changes required at BS to receive D2R transmissions in UL spectrum. | - CW transmission by BS in UL spectrum is not allowed as per existing spectrum regulations. - Additional hardware required for transmitting CW in UL spectrum. - Spatial isolation is only possible when the BS transmitting the CW is different from the reader. |
| Case 1-4: CW is transmitted from outside the topology, transmitted in UL spectrum | - D2R transmissions by device in UL spectrum is in-line with existing spectrum regulations. - No changes required at BS to receive D2R transmissions in UL spectrum. - Spatial isolation is possible, reducing the received interference power. |  |

Proposal 15: The study on case 2-3 is concluded by capturing the following in the TR:

- Intermediate UE has to support different D2R receiver implementations in uplink and downlink spectrum, increasing UE complexity.

Proposal 16: For the case that D2R backscattering is transmitted in the same carrier as CW for D2R backscattering, and for Topology 2, the following cases for CW transmission are prioritized for evaluation purposes.

- Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum

- Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum

Proposal 17: Capture the comparison of the different CW transmission cases for Topology 2 in Table 9 into the TR.

Table 9: Comparison of different CW transmission cases for Topology 2

| CW Transmission case | Advantages | Disadvantages |
| --- | --- | --- |
| Case 2-2: CW is transmitted from inside the topology (i.e., intermediate UE), transmitted in UL spectrum | - CW transmission by intermediate UE in UL spectrum is in-line with existing spectrum regulations. - D2R transmissions by device in UL spectrum is in-line with existing spectrum regulations. | - Spatial isolation is only possible when the intermediate UE transmitting the CW is different from the reader. |
| Case 2-3: CW is transmitted from outside the topology, transmitted in DL spectrum | - Spatial isolation is possible, reducing the received interference power. | - Increased complexity of the intermediate UE since it would be required to support reception on the DL spectrum and UL spectrum, to support the CW being transmitted from inside the topology. |
| Case 2-4: CW is transmitted from outside the topology, transmitted in UL spectrum | - D2R transmissions by device in UL spectrum is in-line with existing spectrum regulations. - Spatial isolation is possible, reducing the received interference power. |  |

# References

- RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA.

- R1-2401767, “Session notes for 9.4 Study on solutions for Ambient IoT (Internet of Things) in NR”, Ad-Hoc Chair (Huawei).

- R1-2403663, “Session notes for 9.4 Study on solutions for Ambient IoT (Internet of Things)”, Ad-Hoc Chair (Huawei).

- R1-2405696, “Session notes for 9.4 Study on solutions for Ambient IoT (Internet of Things)”, Ad-Hoc Chair (Huawei).

- “Transmit/receive isolator for UHF RFID reader with wideband balanced directional coupler”, Asia Pacific Microwave Conference, December 2009.

- “A tunable transformer-based CMOS directional coupler for UHF RFID readers”, IEICE Electronics Express (vol. 14, no. 6), February 2017.

- 3GPP TS 38.104, “Base Station (BS) radio transmission and reception”.

- Analog Self-Interference Cancellation With Practical RF Components for Full-Duplex Radios, IEEE Transactions on Wireless Communications (Vol. 22, Iss. 7), July 2023

- Application Report: General Oversampling of MSP ADCs for Higher Resolution, Texas Instruments, Apr. 2016

- Technical article: Increase Dynamic Range of SAR ADCs Using Oversampling, Analog Devices, June 2015

- R1-2405851, “Ultra low power device architectures for Ambient IoT”, RAN1#118, Aug. 2024, Huawei, HiSilicon

- 3GPP TS 38.174, “Integrated Access and Backhaul (IAB) radio transmission and reception”.

- 3GPP TS 38.300, “NR and NG-RAN Overall description; Stage-2”.

- “Overview of UHF frequency allocations (860 to 960 MHz) for RAIN RFID”, GS1 Global, November 2022.

- EPC™ Radio-Frequency Identity Protocols Class-1 Generation-2 UHF RFID Protocol for Communications at 860 MHz – 960 MHz.

- “Planar Broadband Tag Antenna Mounted on the Metallic Material for UHF RFID System”, IEEE Antennas and Wireless Propagation Letters (Vol. 10), December 2011.

- “Broadband/Dual-band Metal-Mountable UHF RFID Tag Antennas: A Systematic Review, Taxonomy Analysis, Standards of Seamless RFID System Operation, Supporting IoT Implementations, Recommendations, and Future Directions”, IEEE Internet of Things Journal (Vol. 10, No. 16), August 2023.

- “Dual-band RFID tags based on folded dipole antennas loaded with spiral resonators”, IEEE International Workshop on Antenna Technology (iWAT), March 2012.

- “Dual-Band Compact Rectenna for UHF and ISM Wireless Power Transfer Systems”, IEEE Transactions on Antennas and Propagation (Vol. 69, Iss. 4), April 2021.

- The dual-frequency advantage in item-level tagging, Avery Dennison Corporation (https://rfid.averydennison.com/en/home/product-finder/midas-flagtag-df.html).