3GPP TSG RAN WG1 #118			R1-2406644

Maastricht, The Netherlands, August 19th – 23rd, 2024

Agenda item:	9.2.2

Source: 	Moderator (Samsung)

Title: 	Moderator Summary on Rel-19 CSI enhancements

Document for:	Discussion and Decision

## Introduction

The scope given in the Rel-19 NR MIMO Phase 5 WID pertaining to CSI enhancement is as follows:

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

## Summary of companies’ proposals and views

| 3.2 | Conclusion 3.B.1: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the number of configured associated SRS resource(s) (=Q) for antenna switching xTyR and the number of SRS ports (=PSRS) corresponding to the ‘reference UE antenna port’, there is no consensus to support the following: - Q>1 - PSRS >1 Note: This implies no support for configuring >1 SRS resource sets for associated SRS when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset) Proposal 3.B.2: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), - The supported value(s) of x follows the supported configuration(s) of SRS resource for antenna switching xTyR used for reciprocity-based DL CSI acquisition - Note: This doesn’t have any spec impact (no new RRC parameter is needed) - For the selection of PSRS=1 SRS port corresponding to the ‘reference UE antenna port’ (out of the available port(s)), decide, by RAN1#118, from the following schemes: - Scheme1: NW-configured via higher-layer (RRC) signalling - Scheme1B: NW dynamic selection via MAC CE or A-CSI triggering (DCI) - Scheme2: UE-selected, the selection included in the phase offset report - Scheme3: Fixed rule, e.g. the first out of the available port(s) FFS: - Whether/how to identify the transmission occasion of the Q=1 associated SRS resource to determine the reference UE antenna port in relation to the CSI request and/or SRS triggering - The supported time-domain behaviour(s) for the associated SRS resource (periodic, semi-persistent, aperiodic) - In case the NW configures the Q=1 associated SRS resource from an existing SP and/or AP SRS antenna switching resource configuration for DL CSI acquisition (which utilizes dynamic signalling), whether/how to identify the Q=1 associated SRS resource FL assessment: This issue was discussed OFFLINE [2]. Summary re 3.B.1/2: Support Q>1? - Yes: Qualcomm, - No (can accept no): Samsung, NTT DOCOMO, Ericsson, vivo, OPPO, NEC, Lenovo/MotM, CATT, Intel, ZTE, Apple, Huawei/HiSi, TCL, Xiaomi, Sony, Sharp, Google (ok), KDDI, IDC, CMCC, ETRI, New H3C, Nokia/NSB, Support PSRS>1? - Yes: Qualcomm, - No (can accept): Samsung, NTT DOCOMO, Ericsson, vivo, OPPO, NEC, Lenovo/MotM, CATT, Intel, ZTE, Apple, Huawei/HiSi, TCL, Xiaomi, Sony, IDC, Sharp, Google (ok), KDDI, IDC, CMCC, ETRI, New H3C, Nokia/NSB, x following xTyR? - Yes: NTT DOCOMO (open), Ericsson, vivo, OPPO, CATT, ZTE, Apple, Qualcomm, Huawei/HiSi, TCL, Google, Xiaomi, Sony, Sharp, Samsung (ok), KDDI, IDC, CMCC, New H3C, NEC, Nokia/NSB, - No (only x=1): Intel, | Conclusion 3.B.1 is the inevitable logical outcome whether one likes/supports it or not. Note that this was already discussed since RAN1#117 3.B.2: Support/fine: NTT DOCOMO, Ericsson, vivo, OPPO, CATT, ZTE, Apple, Huawei/HiSi, TCL, Xiaomi, Lenovo/MotM, Sharp, Google, KDDI, IDC, Samsung, Qualcomm, New H3C, NEC, Nokia/NSB, Intel, Sony, CMCC, Not support: |
| --- | --- | --- |
| 3.3 | Proposal 3.C.1: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, to facilitate UE-specific delay offset pre-compensation on PDSCH by the NW, decide, by RAN1#118, whether to support configuring a UE (via RRC signaling) to perform PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported delay offset (when ReportQuantity is ‘cjtc-Dd’). And if supported, whether any of the following is additionally supported or not: - NW indicates the delay offset value to be compensated for the Rel-18 eType-II CJT CSI report, and/or - The two separately configured reports (i.e. Rel-18 eType-II CJT CSI report and the CJTC delay offset report) are always jointly triggered and carried on a same PUSCH (hence on a same slot) - The delay offset value to be compensated is the latest reported DO before the DCI triggering the CJT CSI reporting Only AP-CSI-RS can be configured for the Rel-18 eType-II CJT report The above only applies when the CMRs do not share common QCL source for average delay indication Proposal 3.C.2: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, to facilitate UE-specific frequency offset pre-compensation on PDSCH by the NW, decide, by RAN1#118, whether to support configuring a UE (via RRC signaling) to perform PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported frequency offset (when ReportQuantity is ‘cjtc-F’). And if supported, whether any of the following is additionally supported or not: - NW indicates the frequency offset value to be compensated for the Rel-18 eType-II CJT CSI report, and/or - The two separately configured reports (i.e. Rel-18 eType-II CJT CSI report and the CJTC frequency offset report) are always jointly triggered and carried on a same PUSCH (hence on a same slot) Only AP-CSI-RS can be configured for the Rel-18 eType-II CJT report The above only applies when the CMRs do not share common QCL source for average delay indication | 3.C.1: proposal for discussion, not support - Support/fine: vivo, ZTE, Qualcomm, Ericsson, CATT, Nokia/NSB, IDC, Huawei/HiSi, MediaTek, Samsung, OPPO, Intel, Apple, Xiaomi, Sony, Fujitsu, Google, Sharp, Lenovo/MotM, CMCC, TCL, - Not support: 3.C.2: proposal for discussion, not support - Support/fine: vivo, ZTE, Qualcomm, Ericsson, MediaTek, Samsung, CATT, IDC, OPPO, Nokia/NSB, Intel, Apple, Xiaomi, Sony, Huawei/HiSi, Fujitsu, Google, Sharp, Lenovo/MotM, TCL, Not support: |
| 3.4 | Conclusion 3.D.1: For the Rel-19 aperiodic standalone CJT calibration reporting, there is no consensus to support the following reporting formats: - Joint FO + wideband PO reporting - Joint Dd + FO + wideband PO reporting Conclusion 3.D.1 is the inevitable logical outcome whether one likes/supports it or not. Note that this was already discussed since RAN1#117 and the situation doesn’t change much (in fact the support is even weaker now) FL assessment: This issue was discussed OFFLINE [2] and since RAN1#117. The situation hasn’t changed (even more opposing companies) Summary: Joint FO + wideband PO reporting: - Support/fine: - Not support: Samsung, OPPO, Huawei/HiSi, vivo, Lenovo/MotM, Sony, MediaTek, NTT DOCOMO, Ericsson, NEC, Intel, Apple, Fujitsu, TCL, Xiaomi, IDC, Sharp, KDDI, CMCC, ETRI, Joint Dd + FO + wideband PO reporting: - Support/fine: - Not support: Samsung, OPPO, Huawei/HiSi, vivo, Lenovo/MotM, Sony, MediaTek, NTT DOCOMO, Ericsson, NEC, Intel, Apple, Fujitsu, TCL, Xiaomi, IDC, Sharp, KDDI, CMCC, ETRI, |  |
| 3.5.1 | Proposal 3.E.1: For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the resolution for frequency offset reporting FOn, additionally support MFO = 256 FL assessment: The above proposal is motivated by additional results showing that the agreed 5 bits isn’t sufficient in some important deployment scenarios | Support/fine: vivo, Ericsson, Samsung, NTT DOCOMO, Qualcomm (UE cap), Nokia/NSB, Intel, Apple, Sony, Huawei/HiSi, CATT, ZTE, Google, Sharp, Lenovo/MotM, CMCC, KDDI, TCL, Not support: |
| 3.5.2 | Conclusion 3.E.2: For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the dynamic range for delay offset reporting Dn,offset and frequency offset reporting FOn, there is no consensus on supporting additional values of dynamic range FL assessment: This issue has been discussed since RAN1#116bis Parameter; Value; Company view. AD; 0.75CP; Support/fine: Intel, Ericsson, NTT DOCOMO, Xiaomi, Google, NEC, Sharp, KDDI, IDC, OPPO Not support: ZTE, Nokia/NSB, Samsung. 1.5CP; Support/fine: Samsung, Ericsson, NTT DOCOMO, Fujitsu, NEC, KDDI, Intel, Qualcomm (optional), Sony Not support: ZTE, Nokia/NSB, Xiaomi, Google, Sharp, IDC, OPPO,. 14∆f; Support/fine: Huawei/HiSi, Nokia/NSB, CATT Not support: Samsung, OPPO, Apple, Intel, Xiaomi, Google, Fujitsu, NEC, Sharp, KDDI, IDC, Kyocera,. 112∆f; Support/fine: ZTE, Huawei/HiSi, Nokia/NSB, CATT Not support: Samsung, OPPO, Apple, Intel, Xiaomi, Google, Fujitsu, NEC, Sharp, KDDI, IDC, Kyocera,. 124∆f; Support/fine: ZTE, Huawei/HiSi, Qualcomm, vivo, CATT Not support: Samsung, OPPO, Apple, Intel, Xiaomi, Google, Fujitsu, NEC, Sharp, KDDI, IDC, Kyocera,. AFO; 0.025ppm; Support/fine: Qualcomm, Sony Not support: Samsung, OPPO,. 0.05ppm; Support/fine: Qualcomm, Sony Not support: Samsung, OPPO,. 1/(8t); Support/fine: ZTE, Huawei/HiSi, CATT Not support: Samsung, Apple, Intel, Ericsson, Xiaomi, Fujitsu, NEC, IDC, Kyocera,. 1/(16t); Support/fine: ZTE, Huawei/HiSi, Nokia/NSB, CATT Not support: Samsung, Apple, Intel, Ericsson, Xiaomi, Fujitsu, IDC, NEC, Kyocera,. 1/(32t); Support/fine: Huawei/HiSi, Nokia/NSB, CATT Not support: Samsung, Apple, Intel, Ericsson, Xiaomi, Fujitsu, IDC, Kyocera,. |
| 3.6 | Conclusion 3.F: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), there is no consensus on supporting the following optimization schemes (in addition to 1 ‘CSI-RS for CSI’ resource set with NTRP resources): - Optimization 1: 1 CSI-RS resource set with J⋅NTRP single-port CSI-RS resources where J>1 is the number of CSI-RS resources per TRP - Optimization 2: 1 or NTRP TRS resource sets can also be used when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset) Question 3.F: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), please share your views on the need for supporting the following optimization schemes (in addition to 1 ‘CSI-RS for CSI’ resource set with NTRP resources): - Optimization 1: 1 CSI-RS resource set with J⋅NTRP single-port CSI-RS resources where J>1 is the number of CSI-RS resources per TRP - Support/fine: CATT, Qualcomm, NTT DOCOMO (open), Google (open), - Not support: Samsung, OPPO, Intel, Sony, Huawei/HiSi, ZTE, Sharp, CMCC, - Optimization 2: 1 or NTRP TRS resource sets can also be used when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset) - Support/fine: Nokia/NSB, Apple, NTT DOCOMO, ZTE, - Not support: Samsung, OPPO, Intel, Sony, Huawei/HiSi, Google, Sharp, CMCC, FL assessment: This FFS needs resolution. |  |
| 3.7.1 | Conclusion 3.G.1: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-Dd’ (Doffset+d), there is no consensus on supporting the following optimization schemes: - Optimization 1: Additional configuration for not reporting {dn} (to save up to NTRP –1 bits) - Optimization 2: Additional configuration for using dn to either extend the dynamic range (by 2x) or increase the resolution (by 2x) of Dn,offset Question 3.G.1: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-Dd’ (Doffset+d), please share your view on the following optimization schemes: - Optimization 1: Additional configuration for not reporting {dn} (to save up to NTRP –1 bits) - Support/fine: Qualcomm, TCL, Sharp, NEC, OPPO, - Not support: NTT DOCOMO, Samsung, Nokia/NSB, Intel, Apple, Xiaomi, Huawei/HiSi, ZTE, Fujitsu, Google, CMCC, KDDI, - Optimization 2: Additional configuration for using dn to either extend the dynamic range (by 2x) or increase the resolution (by 2x) of Dn,offset - Support/fine: CATT, NTT DOCOMO (open), Qualcomm, - Not support: Samsung, OPPO, Nokia/NSB, Intel, Apple, Xiaomi, Huawei/HiSi, ZTE, Fujitsu, Google, CMCC, KDDI, FL assessment: This FFS needs resolution. Optimization 1 was initially proposed by Qualcomm and 2 by CATT. |  |
| 3.7.2 | Proposal 3.G.2: For the Rel-19 aperiodic standalone CJT calibration reporting, the UCI mapping order is as follows: - When ReportQuantity is ‘cjtc-Dd’ (Doffset+d): - nref, - {Dn,offset, n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID, - {dn, n=0, 1, …, N TRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID - When ReportQuantity is ‘cjtc-F’ (FO): - nref, - {FOn, n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID, - When ReportQuantity is ‘cjtc-Dd-F’ (joint Doffset+d and FO): - nref1, - nref2, - {Dn,offset, n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID, - {dn, n=0, 1, …, N TRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID - {FOn, n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID, FL assessment: The UCI table and ordering for PO-related reports will have to wait until the associated SRS port selection as well as sub-band PO issues are resolved. | Support/fine: Huawei/HiSi, ZTE, Ericsson, Google, Sharp, Lenovo/MotM, TCL, Not support: |
| 3.9.1 | Conclusion 3.I.1: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-Dd’ (Doffset+d) or ‘cjtc-F’ (frequency offset), there is no consensus on additionally supporting the following optimization schemes: - Optimization 1: Aperiodic TRS resource set can also be used - Optimization 2: Different RE locations (FDM) are supported for the RSs Question 3.I.1: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-Dd’ (Doffset+d) or ‘cjtc-F’ (frequency offset), please share your view on the need for additionally supporting the following optimization schemes: - Optimization 1: Whether aperiodic TRS resource set can also be used - Optimization 2: Whether different RE locations (FDM) are supported for the RSs Optimization 1: - Yes: IDC, Intel, - No: Spreadtrum, CMCC, CATT, Samsung, OPPO, Qualcomm, Apple, Xiaomi, Huawei/HiSi, Fujitsu, Google, Lenovo/MotM, KDDI, Optimization 2: - Yes: CATT, ZTE, - No: CMCC, IDC, Samsung, OPPO, Intel, Apple, Xiaomi, Huawei/HiSi, Fujitsu, Google, Lenovo/MotM, KDDI, |  |
| 3.9.2 | Conclusion 3.I.2: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), there is no consensus on additionally supporting the following optimization scheme: - Different RE locations (FDM) are supported for the RSs Question 3.I.2: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), please share your view on the need for additionally supporting the following optimization scheme: - Whether different RE locations (FDM) are supported for the RSs - Yes: ZTE, Nokia/NSB, CATT, - No: Samsung, OPPO, Fujitsu, Lenovo/MotM, IDC, Nokia/NSB, Intel, Apple, Xiaomi, Sony, Huawei/HiSi, Google, Lenovo/MotM, CMCC, KDDI, |  |
| 1.1 | Conclusion 1.A.1: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports with RI=5-8, regarding Scheme-B, there is no consensus in supporting any additional number of SD basis vectors for RI=5-6 in addition to 3 (already agreed in RAN1#117), as well as for RI=7-8 in addition to 4 (already agreed in RAN1#117) Proposal 1.A.2: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports with RI=5-8, regarding Scheme-B, reuse the legacy Rel-15 Type-I layer pairing scheme, and fixed mapping between SD basis vectors and layers: - The k-th SD basis vector is associated with the k-th layer-group. - Note: This should be the baseline based on joint SD basis selection/indication using the agreed combinatorial indication (logCLGN1N2 bits). LG denotes the number of layer groups per previous agreement - Note: For RI=5-6, LG=3 was agreed, and the additional support for LG=4 is still FFS FL assessment: This was discussed OFFLINE [2] and since RAN1#117. It is time to conclude. Summary re 1.A.1: FFS2: additional support for 4 selected SD basis vectors for RI=5-6 (note that 3 is already agreed) - Support/fine: Huawei/HiSi, Xiaomi, NEC, CEWiT, CATT (optional), - Not support: ZTE, Intel, Samsung, OPPO, NTT DOCOMO, MediaTek, Fraunhofer IIS/HHI, Ericsson, Nokia/NSB, Lenovo/MotM, Tejas, Apple, Fujitsu, Spreadtrum, New H3C, TCL, CMCC, IDC, Sharp, KDDI, HONOR, Google (ok), FFS3: additional support of 'x' selected SD basis vectors for ranks 5-8, x not equal to ceil(v/2) (note that 3 is already agreed) - Support/fine: CEWiT, Tejas (open), - Not support: OPPO, NTT DOCOMO, ZTE, MediaTek, Ericsson, Lenovo/MotM, Nokia/NSB, Intel, Xiaomi, Apple, Fujitsu, Spreadtrum, New H3C, TCL, Huawei/HiSi, CMCC, IDC, Sharp, KDDI, HONOR, Google (ok), | Conclusion 1.A.1 is the inevitable logical outcome whether one likes/supports it or not. Note that this was already discussed since RAN1#117 and the situation doesn’t change 1.A.2: Support/fine: vivo, Intel, Samsung, MediaTek, Ericsson, OPPO, NEC, Lenovo/MotM, CATT, Fujitsu, Apple, NTT DOCOMO, Spreadtrum, New H3C, TCL, Huawei/HiSi, Google, CMCC, KDDI, Tejas, Kyocera, ZTE (ok), IDC (ok), Fraunhofer IIS/HHI, Xiaomi, Sharp, CEWiT, Qualcomm (ok), Nokia/NSB, HONOR, Not support: |
| 1.2.2 | Proposal 1.B.2: For the Rel-19 Type-II codebook refinement based on Rel-18 Type-II Doppler for 48, 64, and 128 CSI-RS ports, - For Capability 1 timeline: - OCPU is the legacy Rel-18 Type-II Doppler OCPU multiplied by ceil(P/32) and replace K with KDOPP for AP-CSI-RS, while still not exceeding 8 - Active resource counting follows the legacy Rel-18 Type-II Doppler - For Capability 2 timeline: - OCPU is the legacy Rel-18 Type-II Doppler OCPU and replace K with KDOPP for AP-CSI-RS - Active resource counting follows the legacy Rel-18 Type-II Doppler FL assessment: The timeline (Capability 1 and 2) has been agreed in RAN1#116bis. | Support/fine: Huawei/HiSi, MediaTek, ZTE, vivo, Xiaomi, CATT, Lenovo/MotM, NEC, NTT DOCOMO, IDC, New H3C, Samsung, OPPO (for Cap2), Spreadtrum, Nokia/NSB, Intel, CMCC, Apple, Ericsson, HONOR, Fujitsu, Google, Sharp, Fraunhofer IIS/HHI, Not support: OPPO (Cap1 use same solution as Cap2) |
| 1.2.3 | Proposal 1.B.3: For the Rel-19 Type-I MP codebook refinements for 48, 64, and 128 CSI-RS ports, regarding OCPU, timeline, active resource counting, and CMR restriction (FDM/TDM, EPRE offset), fully reuse those for the Rel-19 Type-I SP codebook FL assessment: This proposal is technically sound since Rel-19 Type-I MP is analogous to a Type-I version of Rel-18 Type-II CJT codebook – seems to be aligned with MediaTek’s proposal | Support/fine: MediaTek, Intel, vivo, Lenovo/MotM, NTT DOCOMO, IDC, Samsung, OPPO, Spreadtrum, Nokia/NSB, CMCC, Apple, Ericsson, Xiaomi, HONOR, Huawei/HiSi, CATT, ZTE, Fujitsu, Google, Fraunhofer IIS/HHI, Not support: |
| 1.3.2 | Conclusion 1.C.2: For the Rel-19 Type-I SP and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, on CBSR, there is no consensus on supporting any of the following additional (X1, X2) values: (1,2), (1,4), (2,4) Question 1.C.2: For the Rel-19 Type-I SP and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, on CBSR, please share your view whether any of the following (X1, X2) values should be supported: (1,2), (1,4), (2,4) (1,2): Tejas, Fraunhofer IIS/HHI (16,2), Xiaomi, NEC, NTT DOCOMO, Apple, Huawei/HiSi, (1,4): IDC (8,8), Tejas, Fraunhofer IIS/HHI (6,4)(8,4), Xiaomi, NEC, NTT DOCOMO, Apple, Huawei/HiSi, (2,4): IDC (8,8), Tejas, Fraunhofer IIS/HHI (16,4)(8,4),(8,8), Xiaomi, NEC, NTT DOCOMO, Apple, Huawei/HiSi, None: TCL, Nokia/NSB, HONOR, ZTE, Google, Lenovo/MotM, |  |
| 1.3.3 | Conclusion 1.C.3: For the 3-bit scaling scheme for Type-I SP codebook, there is no consensus on supporting any of the following additional (X1, X2) values: (1,2), (1,4), (2,4) Question 1.C.3: For the 3-bit scaling scheme for Type-I SP codebook, please share your view whether any of the following (X1, X2) values should be supported: (1,2), (1,4), (2,4), (8,1) (1,2): Apple, (1,4): Apple, (2,4): Apple (8,1): Ericsson, Samsung, Apple, ZTE (ok), Google, None: TCL, Nokia/NSB, HONOR, |  |
| 1.4 | Conclusion 1.D: For the Rel-19 Type-I single-panel (SP) codebook refinement for 48, 64, and 128 CSI-RS ports, for Scheme-A RI=3-4 only, there is no consensus in further extending the i1,3 table (Table 5.2.2.2.1-4 of TS 38.214) Question 1.D: For the Rel-19 Type-I single-panel (SP) codebook refinement for 48, 64, and 128 CSI-RS ports, for Scheme-A RI=3-4 only, please share your view whether the i1,3 table (Table 5.2.2.2.1-4 of TS 38.214) needs to be further extended. And if so, please be specific No need: Samsung, vivo, Nokia/NSB, CMCC, Fujitsu, Google, MediaTek, Need: - Scale by K: Huawei/HiSi, - Other: Xiaomi, CATT, NTT DOCOMO, - Open: ZTE |  |
| 1.5.1 | Proposal 1.E.1: For the Rel-19 Type-I MP codebook refinement for 48, 64, and 128 CSI-RS ports, the UCI parameters are captured in the tables below: - Note: The second column includes the location of the parameters when reported with two-part UCI Parameter; UCI; Details/description. RI; Part 1; Same as Rel-15 Type-I SP: RI=v={1,2,3,4}. Wideband CQI for the first TB; Part 1; Same as Rel-15 Type-I SP. Subband differential CQI for the first TB (*); Part 1; Same as Rel-15 Type-I SP. First SD basis vector selection indicator; Part 2 Wideband; One indicator per each of the Ng resources: - Each of the Ng indicators is the same as Rel-15 Type-I SP with the scheme following < 16-port design of Rel-15 Type-I SP codebookMode=1. Second SD basis vector selection indicator; Part 2 Wideband; One indicator per each of the Ng resources: - Each of the Ng indicators is the same as Rel-15 Type-I SP with the scheme following < 16-port design of R15 Type-I SP codebookMode=1. Inter-pol co-phase selection indicator; Part 2 Wideband or Subband (**); One (set of) indicator per each of the Ng resources: - Same as Rel-15 Type-I SP with the scheme following < 16-port design of R15 Type-I codebookMode=1. Inter-resource co-phasing; Part 2 Wideband; QPSK (2-bit) co-phasing on resource k=2,…,K relative to the first resource, layer-common. (*): Not included when CQI reporting granularity is set to ‘wideband’ (**): Wideband when PMI reporting is set to ‘wideband’, Subband when PMI reporting granularity is set to ‘subband’ FL assessment: This is a natural outcome of the previous agreement | Support/fine: MediaTek, NEC, NTT DOCOMO, Samsung, vivo, Qualcomm, Nokia/NSB, CMCC, Apple, Ericsson, Xiaomi, HONOR, Huawei/HiSi, CATT, ZTE, Fujitsu, Google, Sharp, Lenovo/MotM, Fraunhofer IIS/HHI, Tejas, Not support: |
| 1.5.2 | Proposal 1.E.2: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, the UCI parameters are captured in the tables below for Scheme-A for RI=5-8: - Note: The second column includes the location of the parameters when reported with two-part UCI Scheme-A Parameter; UCI; Details/description. First SD basis vector selection indicator; Part 2 Wideband; v=5-8: Decide (by RAN1#118) Alt1 vs Alt2 Alt1. Same as legacy Rel-15 Type-I SP, separate (i1, i2) indicators of log2N1O1 bits and log2N2O2 bits, respectively Alt2: Separate (i1, i2) indicators of log2N1 bits and log2N2 bits, respectively; and separate (q1, q2) indicators of log2O1 bits and log2O2 bits, respectively.. Second SD basis vector selection indicator; Part 2 Wideband; v=5-8: Freely select the other nSDBV=2 (v=5-6) or 3 (v=7-8) SD basis vectors such that they are orthogonal in at least one dimension (horizontal or vertical). Decide (by RAN1#118) the indication scheme.. Inter-pol co-phase selection indicator; Part 2 Wideband or Subband (**); v=5-8: Same as legacy Rel-15 Type-I SP. (*): Not included when CQI reporting granularity is set to ‘wideband’ (**): Wideband when PMI reporting is set to ‘wideband’, Subband when PMI reporting granularity is set to ‘subband’ FL assessment: This list is to complete UCI design for Type-I SP –Scheme-A RI=5-8. | Support/fine: MediaTek, ZTE, Xiaomi, IDC, NEC, NTT DOCOMO, vivo, Qualcomm, Nokia/NSB, CMCC, Samsung (ok), Apple, Ericsson, HONOR, Huawei/HiSi, CATT, Fujitsu, Google, Sharp, Lenovo/MotM, Tejas, Not support: |
| 1.6 | Proposal 1.F: For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding NZP CSI-RS resource aggregation to attain 32 < P (or PCSI-RS) ≤ 128, for AP-CSI-RS where the K NZP CSI-RS resources are located in two consecutive slots, - Except for codebook refinement based on Rel-18 Type-II Doppler, introduce per-resource higher-layer (RRC) configuration to indicate (via 1-bit per resource) whether 1-slot offset relative to the legacy resource-set-level slot offset configuration should be assumed or not - For codebook refinement based on Rel-18 Type-II Doppler, introduce per-resource higher-layer (RRC) configuration to indicate (via 1-bit per resource) whether 1-slot offset relative to the resource group slot offset should be assumed or not FL assessment: This FFS pertains to the detailed configuration mechanism. The above seems to be the simplest scheme while still keeping the legacy mechanism to configure resource-set-level slot offset. Note that either 0 (no offset) or 1-slot offset relative to the set-level offset is possible. | Support/fine: Samsung, IDC, OPPO, vivo, Qualcomm, Nokia/NSB, Intel, Apple, NTT DOCOMO (ok), Ericsson, Xiaomi, HONOR, Huawei/HiSi, CATT, ZTE (open), Google, MediaTek, Lenovo/MotM, Tejas, Not support: Spreadtrum, |
| 1.7 | Proposal 1.G: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding the port subset indication (from the Rel-18 SD NES Type-1), extend the bitmap (i.e., port-subsetIndicator) by adding p48, p64, and p128 along with their appropriate lengths - Note: Per previous agreements, K is the number of aggregated NZP CSI-RS resources to attain 32 < P (or PCSI-RS) ≤ 128 FFS: For SD NES Type-2 FL assessment: This seems to be the simplest method to extend Rel-18 SD NES port subset selection for Rel-19 Type-I SP codebook | Support/fine: Google, Samsung, IDC, OPPO, vivo, Spreadtrum, Qualcomm, Nokia/NSB, Intel, CMCC, Apple, Ericsson, Xiaomi, HONOR, Huawei/HiSi, ZTE (open), [Fujitsu], MediaTek, Tejas (open), Not support: NTT DOCOMO, Lenovo/MotM, |
| 1.8 | Proposal 1.H: For the Rel-19 Type-I multi-panel (MP) codebook refinement for 48, 64, and 128 CSI-RS ports, regarding the W2 structure, clarify that: - Inter-polarization co-phasing is based on legacy Rel-15 Type-I SP codebookMode=1 for PCSI-RS<16 - QPSK inter-resource co-phasing alphabet is {+1,+j,-1,-j} FL assessment: This clarification is useful to avoid future ambiguities | Support/fine: MediaTek, IDC, NTT DOCOMO, Samsung, OPPO, vivo, Spreadtrum, Qualcomm, Nokia/NSB, CMCC, Apple, Ericsson, Xiaomi, HONOR, Huawei/HiSi, CATT, ZTE, Fujitsu, Google, Lenovo/MotM, Not support: |
| 2.1.1 | Proposal 2.A.1: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding UCI parameters when two-part UCI/CSI is used: - Part 1: M CRI(s), M RI(s), M sets of CQI values for 1st CW - When MR is configured, the CRI(s) associated with the MR resource(s) are not reported - Part 2: M sets of {PMI, LI (if applicable), CQI values for 2nd CW (if applicable)} FL assessment: This issue was discussed OFFLINE [2] | Support/fine: Huawei/HiSi, MediaTek, Ericsson, vivo, Tejas, OPPO, NEC, Lenovo/MotM, CATT, Xiaomi, Intel, ZTE, Apple, Fujitsu, NTT DOCOMO, Spreadtrum, Qualcomm, New H3C, TCL, Google, CMCC, IDC, Sharp, KDDI, Samsung (ok), Tejas, Kyocera, HONOR, LG, Nokia/NSB, Not support: |
| 2.1.2 | Proposal 2.A.2: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, if MR is configured, the supported value(s) of MR are as follows: - Rel-15 Type-I SP: {1, 2} - Rel-16 eType-II: {1} FL assessment: This FFS needs resolution. | Support/fine: Huawei/HiSi, IDC, MediaTek, ZTE, CATT, NTT DOCOMO, LG, IDC, OPPO, vivo, Spreadtrum, Qualcomm, Nokia/NSB, Intel, CMCC, Samsung, Apple, Ericsson, Xiaomi (ok), HONOR, Fujitsu, Google, Sharp, Lenovo/MotM, Tejas (ok) Not support: TCL (only 1), |
| 2.2.1 | Proposal 2.B.1: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding CBSR and RI restriction, support resource-specific specific CBSR - FFS (by RAN1#118): Whether RI restriction is resource-common or resource-specific FL assessment: This issue was discussed OFFLINE [2] | Support/fine: Lenovo/MotM, CATT, Xiaomi, Qualcomm, Apple, New H3C, TCL, Samsung, MediaTek, Ericsson, NEC, Fujitsu, ZTE, NTT DOCOMO, Spreadtrum, Huawei/HiSi, Google, CMCC, IDC, Sharp, KDDI, Tejas, Kyocera, HONOR, LG, vivo (ok), Nokia/NSB, Intel, OPPO (ok), Not support: |
| 2.5 | Conclusion 2.E: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, for M>1, there is no consensus to support resource-common RI indication (in addition to the agreed resource-specific RI) Question 2.E: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, for M>1, please share your view whether resource-common RI indication should also be supported (in addition to the agreed resource-specific RI) Yes: Xiaomi, TCL, Huawei/HiSi, CATT, Tejas, NTT DOCOMO, LG, New H3C, No: vivo, Samsung, NEC, Qualcomm, Lenovo/MotM, Ericsson, Nokia/NSB, Google, Intel, CMCC, MediaTek, Fujitsu, Sharp, OPPO, Spreadtrum, Apple, ZTE, FL assessment: This FFS needs resolution. If the situation hasn’t changed from RAN1#116bis and 117, we will conclude no consensus in Round-1 |  |

| 3.1 | Proposal 3.A.1: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the support of sub-band reporting (>1): - Denoting the number of reported sub-band phase-offset values as NSB-P, and the sub-bands are indexed as {0, 1, …, NSB-P –1}, support, as a separate UE capability from wideband (=1) phase offset reporting, reporting, in one CSI reporting instance, {(n,, n,, n,NSB-P), n=0, 1, …, NTRP – 1, n≠nref} - The alphabet for n, follows the previously agreed alphabet for =1, including the ‘invalid’ state - FFS (by RAN1#118): - Supported sub-band size(s), e.g. following the legacy CSI sub-band definition, vs {4, 8, 16 PRBs} - Whether the UE performs measurement over the entire configured CSI reporting band WCSI - If needed, mechanism to limit CSI reporting overhead (e.g. maximum NSB-P) Note: UE is not required to perform DO/FO compensation for sub-band phase offset calculation FL assessment: This issue was discussed OFFLINE [2]. Supporting BOTH Opt1 and Opt2 have been precluded in RAN1#117 agreement – see above (therefore I will not accommodate such proposal – kindly please don’t re-propose this again 😊). | Support/fine: CATT, Samsung, NTT DOCOMO, Ericsson, OPPO (ok), Lenovo/MotM, Intel (ok), Apple, Fujitsu, TCL, Huawei/HiSi, Google, Sharp, KDDI, CMCC, NEC (ok), NICT, Nokia/NSB (ok), New H3C, Xiaomi (ok), ZTE (ok), Not support: Qualcomm (Opt1), Sony (Opt1), IDC (no sub-band), |
| --- | --- | --- |
| 1.5.3 | Proposal 1.E.3: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, for Scheme-A RI=5-8, by RAN1#118, decide, from the following alternatives, the second SD basis vector selection scheme (in Part 2 UCI, wideband): - Alt1: log2N1-bit (i1) and log2N2-bit (i2) indicators for each of the nSDBV other selected SD basis vectors, and the (q1,q2) for the nSDBV other selected SD basis vectors is indicated via a 1-bit flag f∈{0,1}, and a q2 (if f=0) or q1 (if f=1) - f=0 indicates q1 is same as the first SD basis vector - f=1 indicates q2 is same as the first SD basis vector - Alt2: log2N1-1N2O2+N2-1N1O1-(N1-1)(N2-1))-bit indicator for each of the nSDBV other selected SD basis vectors - Alt3: log2N1-1N2O2+N2-1N1O1-N1-1N2-1nSDBV-bit indicator, selecting nSDBV SD basis vectors from the beams orthogonal to first beam - Alt4: log2N1-bit (i1) and log2N2-bit (i2) indicators for each of the nSDBV other selected SD basis vectors, and One-bit orthogonal beam group indicator, i3 and separate (i1,i2) indicator per SD basis vector, with - i1∈{0,1,…,N1-1}, i2∈{0,1,…,N2O2-1}, if i3=0, or - i1∈{0,1,…,N1O1-1}, i2∈{0,1,…,N2-1}, if i3=1 Other alternatives are not precluded Note (from previous agreement): nSDBV=2 (v=5-6) or 3 (v=7-8) FL assessment: This needs to be resolved. The previous agreement leaves the details on indication open. While there are other slightly different alternatives, the above represent the ones that are aligned with the previous agreement and specific enough not to leave any open issue. Alt1: ZTE, Qualcomm, Ericsson, MediaTek, Fujitsu, Google, Lenovo/MotM, Fraunhofer IIS/HHI, Tejas, Intel, Alt2: Xiaomi, vivo Alt3: Huawei/HiSi, Samsung, vivo - Concern (combinatorial memory): Qualcomm, MediaTek, Fraunhofer IIS/HHI, Alt4: Nokia/NSB, | Support/fine: Huawei/HiSi, vivo, Xiaomi, ZTE, Nokia/NSB, Qualcomm, Samsung, Ericsson, MediaTek, Fujitsu, Google, Lenovo/MotM, Fraunhofer IIS/HHI, Tejas, Not support: |

Ground rules in sharing your inputs:

- Please do NOT input anything in Tables 1A, 2A, and 3A

  - Including company names - appreciate your trying to save me some work, but …

  - For some reason, most likely due to poor MS Word inter-platform/version compatibility support (if any), the formatting of the FL proposals will change (for the worse) if you do so. This has happened several times in Athens and Changsha ☹

- Please input your comments ONLY in Tables 1C, 2C, and 3C, thanks! 😊

### Issue 1 (WID objective 2a and 2b): Type-I and Type-II codebook refinement for up to 128 CSI-RS ports

Table 1A Summary: issue 1

| # | Issue/proposal | Companies’ views |
| --- | --- | --- |
| 1.1 | [117] Agreement For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports with RI=5-8, support the following schemes: - The same O1=O2 value(s) as RI=1-4 are supported - Scheme-A (based on Scheme3 described in RAN1#116bis): - W1 structure: - The 1st SD basis vector is freely selected and subsequent 2 (RI=5-6) or 3 SD basis vectors (RI=7-8) are freely selected such that they are orthogonal in at least one dimension (horizontal or vertical). - The v layers are mapped to the selected SD basis vectors following legacy Rel-15 Type-I for RI=5-8. - W2 structure: - Following legacy Rel-15 Type-I RI=5-8 - Scheme-B (based on Scheme2 described in RAN1#116bis): - W1 structure: - Independent selection of different ceil(v/2) SD basis vectors for RI = v, where each SD basis vector is applied to two respective layers following legacy Rel-15 Type-I for RI=5-8, except that, if v is odd, the last SD basis vector is applied to the orphan layer. - FFS: mapping between the orphan layer and its selected SD basis vector and, if needed, UE reporting of the selection - FFS: support of 4 selected SD basis vectors for RI=5-6 - The SD basis vectors are freely selected from a group of N1N2 orthogonal SD DFT basis vectors via combinatorial indication, as well as a layer-common (q1,q2) - W2 structure: - For the orphan layer, the inter-polarization co-phasing is selected from {1, j, -1, -j} - For two layers sharing a same SD basis vector, the inter-polarization co-phasing between two layers is selected from the following pairs {(1, -1), (j, -j)} to achieve inter-layer orthogonality. - Only Scheme-A (RI=1-4+RI=5-8) and Scheme-B (RI=1-4+RI=5-8) are supported in Rel-19 Conclusion 1.A.1: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports with RI=5-8, regarding Scheme-B, there is no consensus in supporting any additional number of SD basis vectors for RI=5-6 in addition to 3 (already agreed in RAN1#117), as well as for RI=7-8 in addition to 4 (already agreed in RAN1#117) Proposal 1.A.2: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports with RI=5-8, regarding Scheme-B, reuse the legacy Rel-15 Type-I layer pairing scheme, and fixed mapping between SD basis vectors and layers: - The k-th SD basis vector is associated with the k-th layer-group. - Note: This should be the baseline based on joint SD basis selection/indication using the agreed combinatorial indication (logCLGN1N2 bits). LG denotes the number of layer groups per previous agreement - Note: For RI=5-6, LG=3 was agreed, and the additional support for LG=4 is still FFS FL assessment: This was discussed OFFLINE [2] and since RAN1#117. It is time to conclude. Summary re 1.A.1: FFS2: additional support for 4 selected SD basis vectors for RI=5-6 (note that 3 is already agreed) - Support/fine: Huawei/HiSi, Xiaomi, NEC, CEWiT, CATT (optional), - Not support: ZTE, Intel, Samsung, OPPO, NTT DOCOMO, MediaTek, Fraunhofer IIS/HHI, Ericsson, Nokia/NSB, Lenovo/MotM, Tejas, Apple, Fujitsu, Spreadtrum, New H3C, TCL, CMCC, IDC, Sharp, KDDI, HONOR, Google (ok), FFS3: additional support of 'x' selected SD basis vectors for ranks 5-8, x not equal to ceil(v/2) (note that 3 is already agreed) - Support/fine: CEWiT, Tejas (open), - Not support: OPPO, NTT DOCOMO, ZTE, MediaTek, Ericsson, Lenovo/MotM, Nokia/NSB, Intel, Xiaomi, Apple, Fujitsu, Spreadtrum, New H3C, TCL, Huawei/HiSi, CMCC, IDC, Sharp, KDDI, HONOR, Google (ok), | Conclusion 1.A.1 is the inevitable logical outcome whether one likes/supports it or not. Note that this was already discussed since RAN1#117 and the situation doesn’t change 1.A.2: Support/fine: vivo, Intel, Samsung, MediaTek, Ericsson, OPPO, NEC, Lenovo/MotM, CATT, Fujitsu, Apple, NTT DOCOMO, Spreadtrum, New H3C, TCL, Huawei/HiSi, Google, CMCC, KDDI, Tejas, Kyocera, ZTE (ok), IDC (ok), Fraunhofer IIS/HHI, Xiaomi, Sharp, CEWiT, Qualcomm (ok), Nokia/NSB, HONOR, Not support: |
| 1.2.1 | Proposal 1.B.1: For the Rel-19 Type-I SP and Type-II codebook refinements (except based on Rel-18 Type-II Doppler) for 48, 64, and 128 CSI-RS ports, active resource counting is: - FFS: For Capability 1 timeline: 1 vs K - For Capability 2 timeline: 1 FL assessment: This was discussed OFFLINE [2]. Since Capability 2 is quite (too) relaxed, there is no reason to further relax both OCPU and ARC for Capability 2. For ARC, since the increase in the total # antenna ports (to up to 128) will be addressed in the ‘triplet’, there doesn’t seem any need to double-book this (mostly relevant to measurement buffering) in ARC (hence 1 should be more fitting, and K is excessive). Hence legacy in FG 2-33 can be interpreted as “Ks=1” (post aggregation) rather than “Ks=K”(pre-aggregation) | 1: Support/fine: Ericsson, Nokia/NSB, ZTE, Fraunhofer IIS/HHI, Intel, TCL, Samsung, vivo, Google, CATT, Qualcomm, NTT DOCOMO, Xiaomi, HONOR, Lenovo/MotM (Cap2), Spreadtrum, CMCC, Sharp, OPPO, MediaTek, NEC, New H3C, KDDI, Kyocera, Concern: Huawei/HiSi, Fujitsu (Cap1), Apple (Cap1) K: Support/fine: Huawei/HiSi, Fujitsu (Cap1), Apple (Cap1), Samsung (2nd), vivo, OPPO, Google (Cap 1), Lenovo/MotM (Cap1), Concern: Qualcomm 1 and K (UE indicates): Support/fine: Apple, Fujitsu, Qualcomm, Concern: Huawei/HiSi, Samsung, Ericsson, CMCC, vivo, |
| 1.2.2 | [117] Agreement For the Rel-19 Type-I SP and Type-II codebook refinements (except based on Rel-18 Type-II Doppler) for 48, 64, and 128 CSI-RS ports, regarding CPU occupation - For Capability 1 timeline: OCPU = ceil(P/32) - For Capability 2 timeline: OCPU = 1 Proposal 1.B.2: For the Rel-19 Type-II codebook refinement based on Rel-18 Type-II Doppler for 48, 64, and 128 CSI-RS ports, - For Capability 1 timeline: - OCPU is the legacy Rel-18 Type-II Doppler OCPU multiplied by ceil(P/32) and replace K with KDOPP for AP-CSI-RS, while still not exceeding 8 - Active resource counting follows the legacy Rel-18 Type-II Doppler - For Capability 2 timeline: - OCPU is the legacy Rel-18 Type-II Doppler OCPU and replace K with KDOPP for AP-CSI-RS - Active resource counting follows the legacy Rel-18 Type-II Doppler FL assessment: The timeline (Capability 1 and 2) has been agreed in RAN1#116bis. | Support/fine: Huawei/HiSi, MediaTek, ZTE, vivo, Xiaomi, CATT, Lenovo/MotM, NEC, NTT DOCOMO, IDC, New H3C, Samsung, OPPO (for Cap2), Spreadtrum, Nokia/NSB, Intel, CMCC, Apple, Ericsson, HONOR, Fujitsu, Google, Sharp, Fraunhofer IIS/HHI, Not support: OPPO (Cap1 use same solution as Cap2) |
| 1.2.3 | [117] Agreement For the Rel-19 Type-I multi-panel (MP) codebook refinement for 48, 64, and 128 CSI-RS ports, for RI=1-4, support the following (compromise between Scheme1 and Scheme2 described in RAN1#116bis): - W1 structure: Independent SD basis selection across all the Ng=K NZP CSI-RS resources, reusing legacy Rel-15 Type-I Pcsi-rs<16 SP SD basis selection rules with L=1 for RI=1-4 - Ng = K = {2, 4} denotes the number of NZP CSI-RS resources associated with the Ng panels - W2 structure: - Legacy Rel-15 Type-I inter-polarization co-phasing rules independently in each resource, - Layer-common wideband inter-resource QPSK co-phasing independently reported for resource n = 2,…,K with respect to the first resource. Rel-19 Type-I MP does not support RI=5-8. Reuse Rel-15 Type-I MP legacy designs for UCI omission, and CBSR. For CSI calculation, reuse Rel-18 Type II CJT CSI-RS port ordering for UE assumption on the transmitted PDSCH symbols across antenna ports. Proposal 1.B.3: For the Rel-19 Type-I MP codebook refinements for 48, 64, and 128 CSI-RS ports, regarding OCPU, timeline, active resource counting, and CMR restriction (FDM/TDM, EPRE offset), fully reuse those for the Rel-19 Type-I SP codebook FL assessment: This proposal is technically sound since Rel-19 Type-I MP is analogous to a Type-I version of Rel-18 Type-II CJT codebook – seems to be aligned with MediaTek’s proposal | Support/fine: MediaTek, Intel, vivo, Lenovo/MotM, NTT DOCOMO, IDC, Samsung, OPPO, Spreadtrum, Nokia/NSB, CMCC, Apple, Ericsson, Xiaomi, HONOR, Huawei/HiSi, CATT, ZTE, Fujitsu, Google, Fraunhofer IIS/HHI, Not support: |
| 1.3.1 | [117] Agreement For the Rel-19 Type-I codebook refinement for 48, 64, and 128 CSI-RS ports, for RI=v=1, support the following: - for each group of X1X2 SD basis vectors, a 3-bit scaling factor can be NW-configured via higher-layer (RRC) signalling, where the scaling factors are defined as scalings on the power control offset configured for the associated CSI-RS resources - The values of X1 and X2 for this feature are separately configured from those for CBSR - Separate configuration (RRC signalling) from CBSR - The candidate values of X1 and X2 are the same as those agreed for CBSR - The codepoints of each of the group-specific 3-bit scaling factors are mapped to values of 1, 1/2, 1/3 ,1/4,1/6,1/8,1/12,1/16 - Note: This feature is a separate UE capability FFS: Whether this can be extended to RI=v>1 … Question 1.C.1: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding the 3-bit scaling factor, please share your view whether this can be extended to RI=v>1 (and provide justification) Yes: Ericsson, Nokia/NSB, Tejas, No: IDC, TCL, OPPO, Xiaomi, Huawei/HiSi, ZTE, Fujitsu, Need more discussion/study: Qualcomm, CMCC, Samsung, Apple, NTT DOCOMO, Xiaomi (ok), HONOR, Google, Lenovo/MotM, Fraunhofer IIS/HHI, FL assessment: This FFS needs resolution |  |
| 1.3.2 | [117] Agreement For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, on CBSR, the following (X1, X2) values are supported: (1,1), (2,1), (2,2), (4,1), (4,2), (4,4), - FFS: (1,2), (1,4), (2,4) [117] Agreement For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, on CBSR, - N1⋅N2X1⋅X2-bit group-based CBSR - the following (X1, X2) values are supported: P; (N1, N2); (X1, X2). 48; (8,3); (1,1), (2,1), (4,1). (6,4); (1,1), (2,1), (2,2),. 64; (16,2); (1,1), (2,1), (2,2), (4,1), (4,2). (8,4); (1,1), (2,1), (2,2), (4,1), (4,2). 128; (16,4); (1,1), (2,1), (2,2), (4,1), (4,2). (8,8); (1,1), (2,1), (2,2), (4,1), (4,2). - FFS: (1,2), (1,4), (2,4), when applicable Conclusion 1.C.2: For the Rel-19 Type-I SP and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, on CBSR, there is no consensus on supporting any of the following additional (X1, X2) values: (1,2), (1,4), (2,4) Question 1.C.2: For the Rel-19 Type-I SP and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, on CBSR, please share your view whether any of the following (X1, X2) values should be supported: (1,2), (1,4), (2,4) (1,2): Tejas, Fraunhofer IIS/HHI (16,2), Xiaomi, NEC, NTT DOCOMO, Apple, Huawei/HiSi, (1,4): IDC (8,8), Tejas, Fraunhofer IIS/HHI (6,4)(8,4), Xiaomi, NEC, NTT DOCOMO, Apple, Huawei/HiSi, (2,4): IDC (8,8), Tejas, Fraunhofer IIS/HHI (16,4)(8,4),(8,8), Xiaomi, NEC, NTT DOCOMO, Apple, Huawei/HiSi, None: TCL, Nokia/NSB, HONOR, ZTE, Google, Lenovo/MotM, FL assessment: This FFS needs resolution. The same should hold for Type-I SP and Type-II |
| 1.3.3 | [117] Agreement For the 3-bit scaling scheme for Type-I codebook, the following (X1, X2) values are supported: (2,1), (2,2), (4,1), (4,2), (4,4), - FFS: (1,2), (1,4), (2,4), (8,1) Conclusion 1.C.3: For the 3-bit scaling scheme for Type-I SP codebook, there is no consensus on supporting any of the following additional (X1, X2) values: (1,2), (1,4), (2,4) Question 1.C.3: For the 3-bit scaling scheme for Type-I SP codebook, please share your view whether any of the following (X1, X2) values should be supported: (1,2), (1,4), (2,4), (8,1) (1,2): Apple, (1,4): Apple, (2,4): Apple (8,1): Ericsson, Samsung, Apple, ZTE (ok), Google, None: TCL, Nokia/NSB, HONOR, FL assessment: This FFS needs resolution. |  |
| 1.3.4 | Question 1.C.4: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, please share your view whether a UE can be configured with both the group-based hard CBSR and the 3-bit SD basis group-based scaling factor (assuming the UE is capable of both)? - Yes: ZTE, Google, Qualcomm, Ericsson, Lenovo/MotM, Tejas, Huawei/HiSi - No: MediaTek, |  |
| 1.4 | [117] Agreement For the Rel-19 Type-I single-panel (SP) codebook refinement for 48, 64, and 128 CSI-RS ports, for Scheme-A RI=3-4 only, the legacy mapping of i1,3 to (k1,k2) for (N1=3,N2=2) from Table 5.2.2.2.1-4 of TS 38.214 is used for all of the newly supported (N1,N2) values. - FFS: whether the i1,3 table (Table 5.2.2.2.1-4 of TS 38.214) needs to be further extended. Conclusion 1.D: For the Rel-19 Type-I single-panel (SP) codebook refinement for 48, 64, and 128 CSI-RS ports, for Scheme-A RI=3-4 only, there is no consensus in further extending the i1,3 table (Table 5.2.2.2.1-4 of TS 38.214) Question 1.D: For the Rel-19 Type-I single-panel (SP) codebook refinement for 48, 64, and 128 CSI-RS ports, for Scheme-A RI=3-4 only, please share your view whether the i1,3 table (Table 5.2.2.2.1-4 of TS 38.214) needs to be further extended. And if so, please be specific No need: Samsung, vivo, Nokia/NSB, CMCC, Fujitsu, Google, MediaTek, Need: - Scale by K: Huawei/HiSi, - Other: Xiaomi, CATT, NTT DOCOMO, - Open: ZTE FL assessment: This FFS needs resolution. |  |
| 1.5.1 | [117] Agreement For the Rel-19 Type-I multi-panel (MP) codebook refinement for 48, 64, and 128 CSI-RS ports, for RI=1-4, support the following (compromise between Scheme1 and Scheme2 described in RAN1#116bis): - W1 structure: Independent SD basis selection across all the Ng=K NZP CSI-RS resources, reusing legacy Rel-15 Type-I Pcsi-rs<16 SP SD basis selection rules with L=1 for RI=1-4 - Ng = K = {2, 4} denotes the number of NZP CSI-RS resources associated with the Ng panels - W2 structure: - Legacy Rel-15 Type-I inter-polarization co-phasing rules independently in each resource, - Layer-common wideband inter-resource QPSK co-phasing independently reported for resource n = 2,…,K with respect to the first resource. Rel-19 Type-I MP does not support RI=5-8. Reuse Rel-15 Type-I MP legacy designs for UCI omission, and CBSR. For CSI calculation, reuse Rel-18 Type II CJT CSI-RS port ordering for UE assumption on the transmitted PDSCH symbols across antenna ports. Proposal 1.E.1: For the Rel-19 Type-I MP codebook refinement for 48, 64, and 128 CSI-RS ports, the UCI parameters are captured in the tables below: - Note: The second column includes the location of the parameters when reported with two-part UCI Parameter; UCI; Details/description. RI; Part 1; Same as Rel-15 Type-I SP: RI=v={1,2,3,4}. Wideband CQI for the first TB; Part 1; Same as Rel-15 Type-I SP. Subband differential CQI for the first TB (*); Part 1; Same as Rel-15 Type-I SP. First SD basis vector selection indicator; Part 2 Wideband; One indicator per each of the Ng resources: - Each of the Ng indicators is the same as Rel-15 Type-I SP with the scheme following < 16-port design of Rel-15 Type-I SP codebookMode=1. Second SD basis vector selection indicator; Part 2 Wideband; One indicator per each of the Ng resources: - Each of the Ng indicators is the same as Rel-15 Type-I SP with the scheme following < 16-port design of R15 Type-I SP codebookMode=1. Inter-pol co-phase selection indicator; Part 2 Wideband or Subband (**); One (set of) indicator per each of the Ng resources: - Same as Rel-15 Type-I SP with the scheme following < 16-port design of R15 Type-I codebookMode=1. Inter-resource co-phasing; Part 2 Wideband; QPSK (2-bit) co-phasing on resource k=2,…,K relative to the first resource, layer-common. (*): Not included when CQI reporting granularity is set to ‘wideband’ (**): Wideband when PMI reporting is set to ‘wideband’, Subband when PMI reporting granularity is set to ‘subband’ FL assessment: This is a natural outcome of the previous agreement | Support/fine: MediaTek, NEC, NTT DOCOMO, Samsung, vivo, Qualcomm, Nokia/NSB, CMCC, Apple, Ericsson, Xiaomi, HONOR, Huawei/HiSi, CATT, ZTE, Fujitsu, Google, Sharp, Lenovo/MotM, Fraunhofer IIS/HHI, Tejas, Not support: |
| 1.5.2 | [117] Agreement For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports with RI=5-8, support the following schemes: - The same O1=O2 value(s) as RI=1-4 are supported - Scheme-A (based on Scheme3 described in RAN1#116bis): - W1 structure: - The 1st SD basis vector is freely selected and subsequent 2 (RI=5-6) or 3 SD basis vectors (RI=7-8) are freely selected such that they are orthogonal in at least one dimension (horizontal or vertical). - The v layers are mapped to the selected SD basis vectors following legacy Rel-15 Type-I for RI=5-8. - W2 structure: - Following legacy Rel-15 Type-I RI=5-8 Proposal 1.E.2: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, the UCI parameters are captured in the tables below for Scheme-A for RI=5-8: - Note: The second column includes the location of the parameters when reported with two-part UCI Scheme-A Parameter; UCI; Details/description. First SD basis vector selection indicator; Part 2 Wideband; v=5-8: Decide (by RAN1#118) Alt1 vs Alt2 Alt1. Same as legacy Rel-15 Type-I SP, separate (i1, i2) indicators of log2N1O1 bits and log2N2O2 bits, respectively Alt2: Separate (i1, i2) indicators of log2N1 bits and log2N2 bits, respectively; and separate (q1, q2) indicators of log2O1 bits and log2O2 bits, respectively.. Second SD basis vector selection indicator; Part 2 Wideband; v=5-8: Freely select the other nSDBV=2 (v=5-6) or 3 (v=7-8) SD basis vectors such that they are orthogonal in at least one dimension (horizontal or vertical). Decide (by RAN1#118) the indication scheme.. Inter-pol co-phase selection indicator; Part 2 Wideband or Subband (**); v=5-8: Same as legacy Rel-15 Type-I SP. (*): Not included when CQI reporting granularity is set to ‘wideband’ (**): Wideband when PMI reporting is set to ‘wideband’, Subband when PMI reporting granularity is set to ‘subband’ FL assessment: This list is to complete UCI design for Type-I SP –Scheme-A RI=5-8. | Support/fine: MediaTek, ZTE, Xiaomi, IDC, NEC, NTT DOCOMO, vivo, Qualcomm, Nokia/NSB, CMCC, Samsung (ok), Apple, Ericsson, HONOR, Huawei/HiSi, CATT, Fujitsu, Google, Sharp, Lenovo/MotM, Tejas, Not support: |
| 1.5.3 | [117] Agreement For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports with RI=5-8, support the following schemes: - The same O1=O2 value(s) as RI=1-4 are supported - Scheme-A (based on Scheme3 described in RAN1#116bis): - W1 structure: - The 1st SD basis vector is freely selected and subsequent 2 (RI=5-6) or 3 SD basis vectors (RI=7-8) are freely selected such that they are orthogonal in at least one dimension (horizontal or vertical). - … Proposal 1.E.3: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, for Scheme-A RI=5-8, by RAN1#118, decide, from the following alternatives, the second SD basis vector selection scheme (in Part 2 UCI, wideband): - Alt1: log2N1-bit (i1) and log2N2-bit (i2) indicators for each of the nSDBV other selected SD basis vectors, and the (q1,q2) for the nSDBV other selected SD basis vectors is indicated via a 1-bit flag f∈{0,1}, and a q2 (if f=0) or q1 (if f=1) - f=0 indicates q1 is same as the first SD basis vector - f=1 indicates q2 is same as the first SD basis vector - Alt2: log2N1-1N2O2+N2-1N1O1-(N1-1)(N2-1))-bit indicator for each of the nSDBV other selected SD basis vectors - Alt3: log2N1-1N2O2+N2-1N1O1-N1-1N2-1nSDBV-bit indicator, selecting nSDBV SD basis vectors from the beams orthogonal to first beam - Alt4: log2N1-bit (i1) and log2N2-bit (i2) indicators for each of the nSDBV other selected SD basis vectors, and One-bit orthogonal beam group indicator, i3 and separate (i1,i2) indicator per SD basis vector, with - i1∈{0,1,…,N1-1}, i2∈{0,1,…,N2O2-1}, if i3=0, or - i1∈{0,1,…,N1O1-1}, i2∈{0,1,…,N2-1}, if i3=1 Other alternatives are not precluded Note (from previous agreement): nSDBV=2 (v=5-6) or 3 (v=7-8) FL assessment: This needs to be resolved. The previous agreement leaves the details on indication open. While there are other slightly different alternatives, the above represent the ones that are aligned with the previous agreement and specific enough not to leave any open issue. Alt1: ZTE, Qualcomm, Ericsson, MediaTek, Fujitsu, Google, Lenovo/MotM, Fraunhofer IIS/HHI, Tejas, Intel, Alt2: Xiaomi, vivo Alt3: Huawei/HiSi, Samsung, vivo - Concern (combinatorial memory): Qualcomm, MediaTek, Fraunhofer IIS/HHI, Alt4: Nokia/NSB, | Support/fine: Huawei/HiSi, vivo, Xiaomi, ZTE, Nokia/NSB, Qualcomm, Samsung, Ericsson, MediaTek, Fujitsu, Google, Lenovo/MotM, Fraunhofer IIS/HHI, Tejas, Not support: |
| 1.6 | [116bis] Agreement For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding NZP CSI-RS resource aggregation to attain 32 < P (or PCSI-RS) ≤ 128, support the following refinement on the K>1 CSI-RS resources associated with a same CSI-RS resource set: - Allow per-resource configuration of evenPRBs or oddPRBs for 0.5 RE/RB/port density - For AP-CSI-RS, allow resource-specific slot offset when the K NZP CSI-RS resources are located in two consecutive slots - FFS: details on how to configure/determine the slot offsets Proposal 1.F: For the Rel-19 Type-I and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, regarding NZP CSI-RS resource aggregation to attain 32 < P (or PCSI-RS) ≤ 128, for AP-CSI-RS where the K NZP CSI-RS resources are located in two consecutive slots, - Except for codebook refinement based on Rel-18 Type-II Doppler, introduce per-resource higher-layer (RRC) configuration to indicate (via 1-bit per resource) whether 1-slot offset relative to the legacy resource-set-level slot offset configuration should be assumed or not - For codebook refinement based on Rel-18 Type-II Doppler, introduce per-resource higher-layer (RRC) configuration to indicate (via 1-bit per resource) whether 1-slot offset relative to the resource group slot offset should be assumed or not FL assessment: This FFS pertains to the detailed configuration mechanism. The above seems to be the simplest scheme while still keeping the legacy mechanism to configure resource-set-level slot offset. Note that either 0 (no offset) or 1-slot offset relative to the set-level offset is possible. Note that the companies not supporting propose additional constraint that would revert previous agreement (partially or fully) to optimize/remove the need for RRC ignalling. | Support/fine: Samsung, IDC, OPPO, vivo, Qualcomm, Nokia/NSB, Intel, Apple, NTT DOCOMO (ok), Ericsson, Xiaomi, HONOR, Huawei/HiSi, CATT, ZTE (open), Google, MediaTek, Lenovo/MotM, Tejas, Not support: Spreadtrum, |
| 1.7 | Proposal 1.G: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding the port subset indication (from the Rel-18 SD NES Type-1), extend the bitmap (i.e., port-subsetIndicator) by adding p48, p64, and p128 along with their appropriate lengths - Note: Per previous agreements, K is the number of aggregated NZP CSI-RS resources to attain 32 < P (or PCSI-RS) ≤ 128 FFS: For SD NES Type-2 FL assessment: This seems to be the simplest method to extend Rel-18 SD NES port subset selection for Rel-19 Type-I SP codebook | Support/fine: Google, Samsung, IDC, OPPO, vivo, Spreadtrum, Qualcomm, Nokia/NSB, Intel, CMCC, Apple, Ericsson, Xiaomi, HONOR, Huawei/HiSi, ZTE (open), [Fujitsu], MediaTek, Tejas (open), Not support: NTT DOCOMO, Lenovo/MotM, |
| 1.8 | [117] Agreement For the Rel-19 Type-I multi-panel (MP) codebook refinement for 48, 64, and 128 CSI-RS ports, for RI=1-4, support the following (compromise between Scheme1 and Scheme2 described in RAN1#116bis): - W1 structure: Independent SD basis selection across all the Ng=K NZP CSI-RS resources, reusing legacy Rel-15 Type-I Pcsi-rs<16 SP SD basis selection rules with L=1 for RI=1-4 - Ng = K = {2, 4} denotes the number of NZP CSI-RS resources associated with the Ng panels - W2 structure: - Legacy Rel-15 Type-I inter-polarization co-phasing rules independently in each resource, - Layer-common wideband inter-resource QPSK co-phasing independently reported for resource n = 2,…,K with respect to the first resource. Rel-19 Type-I MP does not support RI=5-8. Reuse Rel-15 Type-I MP legacy designs for UCI omission, and CBSR. For CSI calculation, reuse Rel-18 Type II CJT CSI-RS port ordering for UE assumption on the transmitted PDSCH symbols across antenna ports. Proposal 1.H: For the Rel-19 Type-I multi-panel (MP) codebook refinement for 48, 64, and 128 CSI-RS ports, regarding the W2 structure, clarify that: - Inter-polarization co-phasing is based on legacy Rel-15 Type-I SP codebookMode=1 for PCSI-RS<16 - QPSK inter-resource co-phasing alphabet is {+1,+j,-1,-j} FL assessment: This clarification is useful to avoid future ambiguities | Support/fine: MediaTek, IDC, NTT DOCOMO, Samsung, OPPO, vivo, Spreadtrum, Qualcomm, Nokia/NSB, CMCC, Apple, Ericsson, Xiaomi, HONOR, Huawei/HiSi, CATT, ZTE, Fujitsu, Google, Lenovo/MotM, Not support: |

Table 1B SLS results: issue 1

| Company | SLS results |  |  |
| --- | --- | --- | --- |
|  | Issue # | Metric | Observation |
| Huawei/HiSi | 1.1 (on 4 SD basis for RI=5-6) | UPT gain | The simulation results in Figure 1 and Figure 2 show that Ues with rank 6 can have 11% performance gain if up to 4 SD bases can be used compared to 3 SD bases only, and Ues with rank 5 with about 6% performance gain. Figure 1 Performance of Ues with RI=6. Figure 2 Performance of Ues with RI=5. |
|  | 1.1 (on SD basis indicator) | UPT gain | The simulation results in Figure 3 show that there is small performance gain (~0.6% gain) with reporting an orphan-layer selection. Figure 3 Performance of orphan-layer indication. |
|  | Other (SRS port grouping) | Normalized throughput (LLS) | It is observed in Figure 4 that 43% performance gain can be achieved by PDSCH reception with SRS port grouping compared to max Rank-4. The performance of 8R rank-8 is also shown in the figure as an upper bound, which is difficult to be implemented due to the high complexity currently. Moreover, it can be observed from figure 9 that the performance of low complexity receiver (two antenna groups) without SRS port grouping enhancement is very poor even at high SNR. Figure 4 Performance of 8Rx UE with different receiver schemes under practical interference |
| MediaTek | 1.1 | Cell average UPT gain | For Rel-19 Type I codebook Scheme B RI=5-8, regarding beam-to-layer mapping, full layer ordering information (Alt 4-6) results in ~2 % UPT gain, and orphan layer indication (Alt 2) results in ~1% UPT gain over fixed mapping (Alt 1) for specific traffic loads only (around 20~25 % RU). |
| ZTE | 1.1 | Average UPT gain | Regarding SD basis vector indication of Scheme-B for RI = 5-8, Alt 2 provides 2%~3% average UPT gain over Alt1 for RI = 5/7 with only 2-bit overhead increased, while Alt3 barely provide further UPT gain over Alt2. |
| Xiaomi | 1.4 | Mean and cell-edge UPTs | For 64 ports, about 5% and 23% performance gain are respectively obtained for mean users and cell-edge users. For 128 ports, about 5% and more than 72% performance gain are respectively obtained for mean users and cell-edge users. |
| CATT | 1.4 | Average throughput gain and 5%-tile throughput gain | According to the simulation results below (Tables), performance improvement (1%~1.5%) can be achieved for N1=8,N2=4 with a slightly modified beam offset i1,3 from N1=3,N2=2. Comparison of average and 5%-ile throughput performance, rank=3 Average throughput gain; 5%-ile throughput gain. rank=3, Case 0; 100%; 100%. rank=3, Case 1; 101.15%; 101.59%. rank=3, Case 2; 100.18%; 100.4%. Comparison of average and 5%-ile throughput performance, rank=4 Average throughput gain; 5%-ile throughput gain. rank=4, Case 0; 100%; 100%. rank=4, Case 1; 101.84%; 99.74%. rank=4, Case 2; 99.6%; 100.17%. |
| Samsung | 1.1 | Avg UPT gain vs overhead | for Rel-19 Type-I SP Scheme-B with RI=1-8, it is shown in the SLS results below that: - Indicating orphan layer yields 0.7% UPT gain while incurring 2-bit more overhead - Full ordering information yields 1.5% UPT gain while incurring 4-bit more overhead (assuming Alt4). |
|  | Other (SRS port grouping) | Avg UPT gain | The case of low complexity 8 RX receiver w/o SRS port grouping incurs 65% UPT loss com-pared to 4RX scenario. This basically implies that it is not possible to work for RI>4 without SRS port grouping assumption for low-complexity 8RX receiver |
| Nokia/NSB | 1.1 | Mean UE throughput and edge UE throughput | In it shown in the SLS results below that for the lower RU, Alt2 performs within 1% of Alt1, whereas Alt3 show about 3% UPT gain over Alt1 in both average and cell-edge throughput. For higher utilisation, Alt2 and Alt3 show about 2% and 4% UPT gain, respectively over Alt1. Comparison of UPT gain between different SD basis signalling solutions for Scheme-B and ranks 5 to 8. 45% RU Comparison of UPT gain between different SD basis signalling solutions for Scheme-B and ranks 5 to 8. 65% RU. |
| Ericsson | 1.1 | Mean user relative throughput | From the evaluation results above, we observe that there is no statistically significant gains between the different alternatives of orphan layer mapping for Type-I Scheme-B. |
|  | 1.3.1 | Mean user throughput gain, 5%-tile throughput gain | The table below shows the relative throughput gain of different schemes at 50% and 70% resource utilization. As seen in the result, such an approach provides large throughput gains. |
|  | 1.3.3 | Mean user throughput gain, 5%-tile throughput gain | In the table below, it is shown that the impact of using a group size larger than 1 along the N2 dimension, i.e., X2>1. As seen in the results, the mean and 5th-percentile throughput decrease with increasing group sizes along the N2 dimension. |

Table 1C Additional inputs: issue 1

| Company | Input |
| --- | --- |
| Mod V0 | Please share your inputs on each of the issues and, if applicable, proposals in TABLE 1A |
| InterDigital | Proposal 1.B.2: Support Proposal 1.B.3: Support Proposal 1.E.2: Support Proposal 1.F: Support Proposal 1.G: Support Proposal 1.H: Support |
| New H3C | Proposal 1.B.2:：OK |
| NEC | Proposal 1.B.2: We are fine with the proposal in principle, while for Capability 1, due to the maximum number of CPUs limited to be 8, there will be more cases exceeding the maximum value, for example, - Aperiodic CSI-RS for CMR, - 48 ports, KDOPP =8 or 12 - 64 ports, KDOPP =8 or 12 - 128 ports, KDOPP =4 or 8 or 12 - Semi-persistent or periodic CSI-RS for CMR - 48 ports, N4 =8 - 64 ports, N4 =8 - 128 ports, N4 =4 or 8 So we would like to update the proposal as: (or we can make a formula with min( , 8)) Updated Proposal 1.B.2: For the Rel-19 Type-II codebook refinement based on Rel-18 Type-II Doppler for 48, 64, and 128 CSI-RS ports, - For Capability 1 timeline: - In case of following configurations, OCPU is 8, - Aperiodic CSI-RS for CMR, - 48 ports, KDOPP =8 or 12 - 64 ports, KDOPP =8 or 12 - 128 ports, KDOPP =4 or 8 or 12 - Semi-persistent or periodic CSI-RS for CMR - 48 ports, N4 =8 - 64 ports, N4 =8 - 128 ports, N4 =4 or 8 - otherwise, OCPU is the legacy Rel-18 Type-II Doppler OCPU multiplied by ceil(P/32) and replace K with KDOPP for AP-CSI-RS - Active resource counting follows the legacy Rel-18 Type-II Doppler - For Capability 2 timeline: - OCPU is the legacy Rel-18 Type-II Doppler OCPU and replace K with KDOPP for AP-CSI-RS - Active resource counting follows the legacy Rel-18 Type-II Doppler Question 1.C.2: We are fine to support (1,2), (1,4), (2,4) Proposals 1.E.1/1.E.2: Support. Proposal 1.F: It can also be achieved with fix configuration, for example, a first half of resources located in first slot, and remaining resources in second slot, as this is mainly to avoid all resources in same slot. While if majority companies support per resource configuration, we are also fine. |
| NTT DOCOMO | Question 1.C.2: OK to further introduce other (X1,X2) values, e.g., (1,4), (2,4) for (8,8). Proposal 1.E.1/2: Support. Proposal 1.F: We do not think resource-specific ignalling is needed. Only 1-bit additional ignalling is sufficient to indicate between 1 slot or 2-slot. And predefined rule can be applied when 2-slot is indicated, e.g., first K/2 CSI-RS resources are located in first slot, and rest CSI-RS resources are located in second slot. [Mod: Your proposal is not according to the agreement (see below) which says resource-specific. You are proposing yet another restriction/structure on the resource-specific slot offset which has not been agreed. Proposal 1.F is simply to implement what we have agreed and leave the signalling details to RAN2. The only additional component in this proposal is RRC signalling – since we are collecting RAN2 impact for MIMO, this is needed 😊 “For AP-CSI-RS, allow resource-specific slot offset when the K NZP CSI-RS resources are located in two consecutive slots”] Proposal 1.G: Maybe we need separate discussion on support of NWES and 128 ports CSI. Proposal 1.H: Support. |
| Samsung | Proposal 1.B.2 Ok. Proposal 1.B.3 OK. Question 1.C.3 We are okay with (8,1). Question 1.D. We don’t think it is not needed, as the performance gain is limited. Also, it should be beneficial to use the same table to shorten development time for Rel-19 Type-I Scheme-A, as it is aligned with the design principle for Type-I Scheme-A for RI=1-4 that follows the legacy Rel-15 Type-I mostly (other than new (N1,N2) configuration) to facilitate the legacy implementation/algorithm to be easily utilized. Proposal 1.E.1 Support Proposal 1.F Support. By enabling the RRC parameter or not for each of the K NZP-CSI-RS-Resource in a NZP-CSI-RS resource set, resource-specific slot offset for the K NZP-CSI-RS resources can be allowed while satisfying they are located in 1 slot or 2 consecutive slots. Proposal 1.H Support. |
| OPPO | Conclusion 1.A.1: Support Proposal 1.A.2: Support Proposal 1.B.2: O_CPU obtained by legacy Rel-18 Type-II Doppler O_CPU multiplied by ceil(P/32) seems too large since the legacy O_CPU is Y*K and Y can be up to 3, which can lead to a very large value for 128 ports. Our preference is to reuse legacy Rel-18 Type-II Doppler O_CPU formula with separate UE reporting for Y and with K replaced by K_DOPP. Proposal 1.B.3: Fine Question 1.C.1: Prefer RI=1 only. The signaling overhead and performance impact to RI>1 is more significant than RI=1. Proposal 1.F: Fine. Proposal 1.G: Based on the latest RRC parameters, is it correct understanding that three bit strings (p48, p64, p128) should be added instead of increasing the current length? portSubsetIndicator-r18 CHOICE { p2 BIT STRING (SIZE (2)), p4 BIT STRING (SIZE (4)), p8 BIT STRING (SIZE (8)), p12 BIT STRING (SIZE (12)), p16 BIT STRING (SIZE (16)), p24 BIT STRING (SIZE (24)), p32 BIT STRING (SIZE (32)) } [Mod: Correct. Revised wording to reflect your comment] Proposal 1.H: We are fine though we think the previous wording is clear enough. |
| Vivo | 1.B.1 We are okay with counting as either 1 or K, but two capabilities should have same counting, we don’t think it is related with timeline. Further, we don’t support to let UE indicate one of {1, K} as UE capability. To count it as 1 or K is not related with UE complexity, it is just related with how to treat the aggregated resources in specification. We anyway will have a corresponding triplet counting rule to cap the UE buffer size on CSI-RS once we decide ARC is 1 or K. - D We think the current i1,3 table is sufficient. 1.E.1 & 1.E.2 OK 1.F.1 OK. To understand correctly, this RRC signalling is 1 bit per resource? If so, it is better to clarify it to avoid misunderstanding. [Mod: Correct] - G OK - H OK |
| Spreadtrum | Proposal 1.B.2: Support. Proposal 1.B.3: Support. Proposal 1.F: we think a pre-defined rule can be used to determine the slot offsets. For example, NW indicates the triggering offset for the first ceil(K/2) CSI-RS resources, and the other CSI-RS resources are transmitted in the second slot. The benefit of introducing RRC signalling is not clear to us. Proposal 1.G: We have similar understanding with OPPO. Proposal 1.H: OK with the clarification. |
| Qualcomm | Proposal 1.A.2: Based on our updated simulation with more outcoming result samples, the gain of “orphan beam” indication is reduced from 3.34% to 2.61% We can compromise to be OK. Proposal 1.B.3: Generally OK, and some additional: …, regarding OCPU, timeline, and active resource counting, and NZP-CSI-RS (CMR) restriction (FDM/TDM, EPRE offset), fully reuse …. Question 1.C.1: According to our analyses, we think this beam-specific scaling factor, if applied to rank>1, may result in unequal power between layers. But we are not sure whether our understanding is aligned with the proponent company’s intended usage on how to apply it to rank>1. While we are open to discuss, more detailed clarification/definition is needed. Proposal 1.E.1:OK Proposal 1.E.2: OK Proposal 1.F: OK Proposal 1.G: OK Proposal 1.H: OK |
| Nokia | Proposal 1.A.2 Support Proposal 1.B.1 Support and agree with FL arguments Proposal 1.B.2 Support. This is a natural extension of the timeline agreed for non-Doppler Type-II extension in the last meeting Proposal 1.B.3 Support. Question 1.C.1 For RI>1 the power scaling factor configured per beam groups complicates the SD bases selection because the CQI does not scale in general with the configured 3-bit factor when more than 1 SD basis is selected. In our view, hard CBSR is more effective to control interference, but from network perspective, we are ok to extend the 3-bit power scaling to all ranks Question 1.C.2 We don’t see a strong need for these extra values Question 1.C.3 We don’t see a strong need for these extra values Question 1.D We don’t see the need for this optimisation. It was agreed that scheme-A for ranks 1-4 follows legacy to reduce time-to-market. Optimising the candidate set for the second SD basis should not be considered at this stage. Proposal 1.E.1 OK. UCI table looks correct Proposal 1.E.2 Support Proposal 1.F Support Proposal 1.G Support Proposal 1.H Ok |
| Intel | Proposal 1.B.2: The number of CPUs for capability 1 timeline is too large. Given that the number of occupied CPUs for the Rel-18 eType II Doppler codebook is indicated via UE capability, we think that legacy rules (with KDOPP instead of K) can be directly reused with a separate UE capability. We are also fine to accept the proposal from NEC. Proposal 1.F: Support Proposal 1.G: Support. |
| CMCC | Conclusion 1.A.1: Support Proposal 1.A.2: Support Proposal 1.B.1: Support Proposal 1.B.2: The maximum OCPU UE can report is 8, and this proposal seems will cause OCPU larger than 8. Proposal 1.B.3: Support Question 1.C.1: We are open to study how RI =v>1 works. Question 1.D: We don’t see the need for this optimization. If it is for more performance gain, we already have Scheme-B. Proposal 1.E.1: Support Proposal 1.E.2: Support Proposal 1.F: Support, per resource signaling can give gNB scheduling more freedom. Proposal 1.G: We are open to discuss whether to support Rel-18 SD NES for 128 ports. Proposal 1.H: OK |
| Samsung | Question 1.C.1 We also have similar view with Qualcomm for v>1. The scaling constraint may result in different power allocation across layers, which seems different from the legacy definition and makes complicated UE implementation. Proposal 1.G @Oppo, your understanding is correct and the direct extension should be baseline. Proposal 1.F @vivo, your understanding is correct. We need just 1 bit parameter to enable 1-bit offset or not. Proposal 1.E.2 Although we think that there is a way to utilize an indicator in part 1, which is more efficient in terms of overhead for Scheme-B with RI=5-8, we are OK with the proposal. |
| Mod V18 | Added conclusion 1.D. Revised proposals 1.B.2/3, 1.F, 1.G per comments from Qualcomm, NEC, OPPO, vivo |
| Apple | Conclusion 1.A.1: Support Proposal 1.A.2: Support Proposal 1.B.1: We still feel that 1 active resource for capability 1 timeline could be too restrictive. If possible, UE capability report is a better alternative Proposal 1.B.2: We are fine Proposal 1.B.3: We are fine Question 1.C.1: It is not very clear on how to apply power control offset scaling to more than 1 layer. Honestly speaking, this is very similar as soft CBSR. We are open to discuss further but limiting to RI=1 is a reasonable compromise. Question 1.C.2: We are open to support those limitations, even though no strong view Question 1.C.3: We are open to support those limitations, even though no strong view Question 1.D: No strong view Proposal 1.E.1: We are fine Proposal 1.E.2: We are fine Proposal 1.F: We are fine Proposal 1.G: We are fine Proposal 1.H: We are fine |
| Ericsson | Proposal 1.B.2 Ok. Agree that the OCPU for Capability 1 shall not exceed 8. Proposal 1.B.3 Ok. Question 1.C.1 @OPPO: Some replies regarding your comment ‘Prefer RI=1 only. The signaling overhead and performance impact to RI>1 is more significant than RI=1.’, There is no additional signaling overhead for supporting this feature for RI>1. The 3-bit scaling factors are signaled per groups of beams once, and the same ignallin per groups of beams are used for the cases of RI=1 and RI>1. Regarding performance, we showed performance gains of 20-25% in mean throughput gain for using this feature with RI>1 over a baseline solution based on network implementation (results copied below). @Qualcomm/Samsung: If the beams corresponding to different layers for RI>1 are chosen from the same beam group, then the power between layers will still be equal. But if the beams corresponding to different layers are chosen from different beam groups, the power between layers may be different. As shown in our paper, one way to determine the power scaling factor to be applied for the ith beam is min1,ϑPiri, where Pi denotes the amplitude scaling factor signalled for the ith beam and ri is the number of layers that are transmitted using the ith beam. But how exactly to do this can be left to UE implementation. If there are concerns with implementation deviating from legacy approach, we suggest to make support of this feature for RI>1 as a UE capability. Question 1.C.3: We hare evaluation results showing the benefit of supporting X1,X2=(8,1) which are copied below. Supporting X1,X2=(8,1) gives the network to ignalling the scaling factor to be signalled with finer resolution in the second dimension when compared to for example X1,X2=(4,2). Based on the results, we suggest to support X1,X2=(8,1). Proposal 1.E.1 Ok Proposal 1.E.2 Ok Proposal 1.F: Ok Proposal 1.G: Support Proposal 1.H: Ok |
| Xiaomi | Proposal 1.B.2: In current spec, for Rel-18 Type II Doppler codebook, the CPU occupation could be is Y*N4 or Y*K depends on type of CSI-RS resource, and the value of Y can be {1, 2, 3}. We can see it have includes multiple candidate values for different UE capabilities. So, we think legacy Ocpu for Rel-18 Type II Doppler codebook could be reused. But, considering the maximum value is restricted, we are fine with the proposal as well. Proposal 1.B.3: Support Question 1.C.1: We have also concern on the complexity if RI>1 is supported for 3-bits scaling factor, and are fine with more discuss the issue. Question 1.D: According to our evaluation, extending i1,3 table could bring larger performance gain with a little overhead increase. The specific i1.3 table could be further discussed. Proposal 1.E.1: We are general fine with the proposal. However, we only agreed that SD basis is independently selected for each panel in the last meeting. But, we have not discussed how to indicate the selected SD basis vectors. In our view, the selected SD basis vectors are mostly same since the angle information of channel may be same from the Ng panels to a UE. Such channel feature could be used to reduce the indication overhead of SD basis vectors. For example, Ng bits is used to indicate whether the selected SD basis vectors for Ng panels are same or not. If same, only one indication information of the selected SD basis vector is reported. [Mod: The table indeed includes a proposal for the scheme for SD basis selection. Basically following legacy.] Proposal 1.F: Support Proposal 1.G: Support Proposal 1.H: Support |
| HONOR | Conclusion 1.A.1: Support Proposal 1.A.2: Support Proposal 1.B.1: Support Proposal 1.B.2: Support Proposal 1.B.3: Support Question 1.C.1: More discussions are needed. As commented by Qualcomm and Samsung, the power spilt between layers are not equal when RI>1. Question 1.C.2/1.C.3: We don’t see the strong need to extend the agreed values. Proposal 1.E.1/1.E.2: Agree. Proposal 1.F: Fine. Proposal 1.F: Fine. Proposal 1.H: Support. |
| Huawei, HiSilicon | Proposal 1.B.3: We are fine with it. Question 1.C.1: for rank>1, different beams/layers may have different power. However, as agreed in last meeting, the scaling is applied to power control offset, which implies the same power among layers. Questions 1.C.2: support (1,2), (1,4), (2,4) for better flexibility. Question 1.D: With increase of N1/N2, it’s more reasonable to extend also the range of SD basis vectors that UE can select. Proposal 1.E.1/1.E.2: Fine with it. Proposal 1.F: Fine with it, and the same principle can be applied to Doppler codebook, which is different from typeI/regular typeII codebook because of the resource group. - For codebook refinement based on Rel-18 Type-II Doppler, introduce per-resource higher-layer (RRC) configuration to indicate (via 1-bit per resource) whether 1-slot offset relative to the resource group slot offset should be assumed or not [Mod: Thanks for spotting this out!] Proposal 1.G: Fine with it. Proposal 1.H: Fine with it. |
| Mod V27 | Proposal 1.B.1: Cap1 is now FFS. While it’s good to agree on both Cap1 and Cap1 together, at leats for Cap2 there is no raised concern for ARC=1 Proposal 1.H: Refinement to ensure proper terminology and unit (resource-group for Doppler, resource for the rest – thanks @Yubo for pointing this out) |
| CATT | Proposal 1.B.3: OK Proposal 1.E.1: OK Proposal 1.E.2: OK Proposal 1.F: OK Proposal 1.H: OK |
| Mod V29 | Added question 1.C.4 (based on input from NEC) and proposal 1.E.3 (based on further Tdoc review). |
| ZTE | 1.A.1: Support. 1.A.2: Although we prefer indicating the SD basis vector for the orphan layer, we can accept current proposal considering this is the majority view. [Mod: Thank you Guangyu] 1.B.2: We are generally OK with the proposal. However, we did NOT see the necessity of limiting OCPU to 8, since we do NOT have such limitation for legacy Doppler Type-II. [Mod: I think we have a similar restriction] 1.B.3: Support. 1.C.1: Do NOT support extending the 3-bit scaling factor for RI=v>1. 1.C.2/1.C.3: At least for Type-I/Type-II CBSR, do NOT support the configurations that X1<X2. Considering that N1≥N2, introducing X1<X2 will cause much larger grid size in N1 direction than in N2 direction, which is NOT beneficial for interference management. 1.C.4: Based previous discussion, our understanding is that they are two separate UE features, and can be enabled together. - D: Open to discuss the extension of the i1,3 table. 1.E.1: Support. 1.E.2: Considering that the SD basis vector indication method of scheme-A for RI = 5-8 is still pending, we suggest to consider two alternatives for the first SD basis vector selection indicator: Parameter; UCI; Details/description. First SD basis vector selection indicator; Part 2 Wideband; v=5-8 Alt1: Same as legacy Rel-15 Type-I SP, separate (i1, i2) indicators of log2N1O1 bits and log2N2O2 bits, respectively v=5-8 Alt2: Separate (i1, i2) indicators of log2N1 bits and log2N2 bits, respectively; and separate (q1, q2) indicators of log2O1 bits and log2O2 bits, respectively.. [Mod: Sure] - F: Open to discuss. - G: Open to discuss. - H: Support. |
| Fujitsu | 1.B.2: Support. 1.B.3: Support. 1.C.1: Prefer R=v=1 only. Conclusion 1.D: Fine with conclusion. Proposal 1.E.1/2: Support. Proposal 1.E.3: Prefer Alt 1 Proposal 1.G: According to Rel-18 NES agreements, both SD type 1 and type 2 adaptation were supported in Rel-18 for different energy saving scenarios. For this proposal, supporting to extend to128 ports in port-subsetIndicator implies to support NES SD type 1 by increasing up to 128 ports in Rel-19. We think we need further discussion, at least we should decide to whether to support NES SD type 1 and/or type 2 in Rel-19 firstly, and then we also need the enhancement for NES SD type 1 and type 2 separately. [Mod: Added clarification for Type-1 and FFS for Type-2 which should address your point] - H: Support. |
| Google | Conclusion 1.A.1: OK Proposal 1.A.2: OK Proposal 1.B.1: OK Proposal 1.B.2: OK Proposal 1.B.3: OK Question 1.C.1: We can be open to RI up to 4, but the indication of 3-bit scaling factor needs to be clarified. For high rank case, the Tx power for each layer is already scaled by sqrt(v). Whether the sqrt(v) is included or excluded for the 3-bit scaling factor or not is unclear. Further, another potential issue is whether we allow certain power borrowing, e.g., power boosting for the layers without power scaling. Question 1.C.2: We do not see the use case for the additional values. Question 1.C.3: We are fine with (8, 1). Question 1.C.4: It seems fine for the concurrent configuration, since the 3-bit scaling factor does not support “0”. Conclusion 1.D: OK Proposal 1.E.1: OK Proposal 1.E.2: OK Proposal 1.E.3: OK. Slightly prefer Alt1 Proposal 1.F: Support. Proposal 1.G: Support. Proposal 1H: OK. |
| OPPO1 | Proposal 1.B.1 The same value for different capabilities is preferred, either 1 or K. If 1 is applied, more memory would be assumed for one active resource, then UE would report less number of supported active resources as capability. If K is applied, less memory would be consumed for one active resource, then more AR can be reported as capability. Considering this value would impact the UE capability on AR, dependency among different UE capabilities is not preferred [Mod: I tend to agree and anyway having separate ARCs for Cap1 and 2 isnt agreeable – check the blue text. But we need to progress step-by-step. In this case, Cap2 with ARC=1 isn’t controversial] |
| Sharp | Conclusion 1.A.1: Support Proposal 1.A.2: Support. Proposal 1.B.2: Support. Proposal 1.E.1: Support. Proposal 1.E.2: Support |
| Qualcomm | Question 1.C.4: Seems OK, since the “soft CBSR” does not comprise 0 (as also commented by some other company) Proposal 1.E.3: OK with listing the alternatives. We support Alt1, and we want to express our concern on Alt3’s combinatorial indication: Note the value of N1-1N2O2+N2-1N1O1-N1-1N2-1 can be as large as 399 (for the case N1=N2=8) – this can cause a high complexity on calculating / storing the en/decoding the combinatorial value table. Secondly, Alt3 itself is incomplete (the mapping b/w codepoints to SD basis index is not defined, but only a total is defined). Last but not least, this is for rank>4, thus two CWs. We tend to think there can be some performance loss for combinatorial-based SD basis selection, compared with one-by-one selection (so that stronger SD bases’ layers can be put into a same CW) |
| MediaTek | Proposal 1.B.1 OK Proposal 1.B.3 OK Question 1.C.4 Our initial thinking is that configuration of both is not needed. The 3-bit restriction may be applied only when hard CBSR configuration does not address the interference issue. Conclusion 1.D OK Proposal 1.E.1 A small typo in the description for Inter-pol co-phase selection indicator: One (set of) indicator per each of the Ng resources: Same as Rel-15 Type-I SP with the scheme following < 16-port design of R16 R15 Type-I codebookMode=1 [Mod: Thanks] Proposal 1.E.3 OK to list the alternatives, and our preference is Alt 1. For combinatorial indicator Alt 3, we agree with Qualcomm about the encoding and decoding issue. Further, for both Alt 2, 3, we think there is additional effort involved in indexing the N1-1N2O2+N2-1N1O1-N1-1N2-1 candidate beams. For Alt 1, we are wondering why separate indicators of log2N1 and log2N2 bits for the other SD basis vectors are needed rather than a single joint log2N1N2 bit indicator, given that such joint indexing of SD beams is already supported for Scheme B. [Mod: The intention from proposing companies is to make is as close as possible to legacy, which uses separate indications for i1 and i2. Anyway this is a minor issue] Proposal 1.F OK Proposal 1.G. OK Proposal 1.H Following the SD bases selection and the UCI parameters in Proposal 1.E.1, the inter-pol co-phasing can be clarified further as: Inter-polarization co-phasing is based on legacy Rel-15 Type-I SP codebookMode=1 for Pcsi-rs < 16 [Mod: OK] |
| Ericsson | Question 1.C.1 Some responses to Google’s question. In our understanding, the sqrt(v) is excluded from the signaled 3-bit scaling factors. Regarding power boosting, we are not considering any power boosting for layers without power scaling. Question 1.C.4: In our understanding, the group-based hard CBSR and the 3-bit SD basis group-based scaling factors are independent features. If a UE is capable of supporting both, then it should be possible to configure the UE with both. |
| Mod V40 | Added conclusions 1.C.2 and 1.C.3 considering the situation for (1,2), (1,4), (2,4) Added Alt2 on 1st basis for Proposal 1.E.2 per ZTE request Minor edits per companies’ inputs |
| Samsung | Proposal 1.E.3 Support the proposal. In our view, feedback overhead should be prioritized to decide the indicator, since Scheme-A is designed aiming for an ‘eco’ mode and the ordering info isn’t really beneficial in companies’ previous results. Based on the overhead analysis below, we prefer Alt3 which has the smallest overhead for all configurations of (N1,N2) among all alternatives. nSDBV=3 (N1,N2); (8,3); (6,4); (16,2); (8,4); (16,4); (8,8). Alt1; 22; 22; 22; 22; 25; 25. Alt2; 24; 24; 24; 24; 27; 27. Alt3; 19; 19; 20; 21; 24; 24. nSDBV=2 (N1,N2); (8,3); (6,4); (16,2); (8,4); (16,4); (8,8). Alt1; 15; 15; 15; 15; 17; 17. Alt2; 16; 16; 16; 16; 18; 18. Alt3; 14; 14; 14; 15; 17; 17. Regarding the combinatorial indicator, there are implementation ways to design large combinatorial values using small combinatorial values only in the current table of the spec (via Vandermonde identity). So, a large table is not needed and it shouldn’t be an issue, in terms of both memory and computational complexity. |
| Lenovo/ MotM | Proposal 1.A.2: Support Proposal 1.B.1: Our preference is K for capability 1 Proposal 1.B.2: OK Proposal 1.B.3: Support Question 1.C.1: Needs further discussion, agree with Ericsson if all beams belong to the same group or even to different beam groups with the same amplitude scaling no implementation issues would occur Conclusion 1.C.2/3: Fine Question 1.C.4: Supporting both simultaneously is very much like soft CBSR in legacy Type-II CB, but seems to be needed after all Proposal 1.E.1/2: Fine Proposal 1.E.3: Prefer Alt1 Proposal 1.F: Support Proposal 1.G: Needs more discussion, especially Type-2 as this notably increases the number of CSI-RS resources applied Proposal 1H: Fine |
| Fraunhofer IIS/HHI | Proposal 1.B.2: Support Proposal 1.B.3: Support Question 1.C.1: open to study further Proposal 1.E.1: Fine Proposal 1.E.3: Share same view as MediaTek regarding Alt 3. We think a single joint indicator (similar to the one to be agreed for scheme B) is sufficient to indicate nSDBV SD basis vectors and a separate indicator can be used to indicate the rotation factor for each SD beam separately (similar to Alt 1). Proposal 1.H: Support |
| Samsung | On SRS port grouping issue, our updated SLS results are further provided below. The reason for the low-complexity 8R w/o SRS port grouping performs much worse than our previous results is as follows: - In the previous results of w/o SRS port grouping case, PMI is calculated under assuming w/o SRS port grouping, but MCS (for RI>4) is calculated under assuming with SRS port grouping (Note that the MCS can be obtained via CQI reporting based on additional use of Bfed CSI-RS transmission, but this is far less practical), so mismatch was only happening in PMI calculation. - For the current results of w/o SRS port grouping case below, MCS calculation is also calculated under assuming without SRS port grouping, and now there are all mismatch in PMI/MCS (for RI>4) calculation. In this case, similar observations to the results Huawei/HiSi provided have been identified. - The case of low complexity 8 RX receiver w/o SRS port grouping incurs 65% UPT loss compared to 4RX scenario. This basically implies that it is not possible to work for RI>4 without SRS port grouping assumption for low-complexity 8RX receiver. |
| Mod V45 | No revision |
| Mod V46 | Proposal 1.E.3: offline editorial suggestion (“downs-select” “decide”) just as before |
| Tejas | Question 1.C.1: open for further study Question 1.C.4: Based on our understanding beam restriction and beam scaling are two separate features, and hence UE can be configured with both hard CBSR and scaling. We think that further discussion is needed to understand if scaling indication is limited to non-restricted groups alone. Question 1.D: Open to recommendations based on results Proposal 1.E.1: Support Proposal 1.E.2: Support the proposal and our preference is Alt2 with separate (q1, q2) indicators which is beneficial for indicating (q1, q2) as well for the nSDBV SD basis vectors. Proposal 1.E.3: Support the proposal and our preference is Alt1. We think that selecting nSDBV SD basis vectors orthogonal to first SD basis vector is critical, and it also important that there is orthogonality between nSDBV SD basis vectors in at least one dimension. In our view, Alt 1 eases to meet this requirement. Proposal 1.F: Support Proposal 1.G: Open |
| Mod V51 | No revision |
| Nokia | Proposal 1.E.3 We propose a slightly different formulation for Alt1, to be aligned with “Alt1” indication of the first SD basis vector, without the need to introduce q1,q2: Alt1 One-bit orthogonal beam group indicator, i3 and separate (i1,i2) indicator per SD basis vector, with i1∈{0,1,…,N1-1}, i2∈{0,1,…,N2O2-1}, if i3=0, or i1∈{0,1,…,N1O1-1}, i2∈{0,1,…,N2-1}, if i3=1 |
| Huawei, HiSilicon | Question 1.C.4: We didn’t see any issue to configure both CBSR and scaling factor, it should be supported if UE supports both. Proposal 1.E.3: we support Alt 3 as it have the least UCI overhead in all scenarios. Regarding the concerns on complexity of combinatorial numbers, we share similar view with Samsung that it can be easily resolved by implementation of Vandermonde identity. On the decoding/encoding, we don’t think it’s a real problem either. Although the details can be up to editor, on example is that firstly count the beams in vertical dimension (4 beams per cluster) excluding the column and the row clusters of the first beam, then count the beams in horizontal dimension (3 beams per cluster) excluding the column and row clusters of the first beam, then the column of the first beam (4 beams per cluster) and then the line of the first beam (4 beams per cluster). Another way to simply it is that we only need count the beams in vertical dimension (4 beams per cluster) excluding the column clusters of the first beam, and then count the beams in horizontal dimension (4 beams per cluster) excluding the row clusters of the first beam. Then the nSDBV SD basis vectors are selected from N1-1N2O2+N2-1N1O1 candidates. Although there is some kind of redundancy, the counting is simpler. |
| Mod V54 | Proposal 1.E.3: Added Alt4 per Nokia’s late comment + “Other alternatives are not precluded” |

### Issue 2 (WID objective 2c): CRI-based CSI for hybrid beamforming (HBF)

Table 2A Summary: issue 2

| # | Issue | Companies’ views |
| --- | --- | --- |
| 2.1.1 | Proposal 2.A.1: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding UCI parameters when two-part UCI/CSI is used: - Part 1: M CRI(s), M RI(s), M sets of CQI values for 1st CW - When MR is configured, the CRI(s) associated with the MR resource(s) are not reported - Part 2: M sets of {PMI, LI (if applicable), CQI values for 2nd CW (if applicable)} FL assessment: This issue was discussed OFFLINE [2] | Support/fine: Huawei/HiSi, MediaTek, Ericsson, vivo, Tejas, OPPO, NEC, Lenovo/MotM, CATT, Xiaomi, Intel, ZTE, Apple, Fujitsu, NTT DOCOMO, Spreadtrum, Qualcomm, New H3C, TCL, Google, CMCC, IDC, Sharp, KDDI, Samsung (ok), Tejas, Kyocera, HONOR, LG, Nokia/NSB, Not support: |
| 2.1.2 | [117] Agreement For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, for A-CSI only, the NW can configure MR (<M) of KS CSI-RS resources to be selected as part of reporting the M “quadruplets”: - (M–MR) CRIs, each with log2KS-MR bits are reported, along with the M sets of CQI/PMI/RI/(if applicable) LI - The value of MR is NW-configured via higher-layer (RRC) signaling - The MR selected resources are NW-configured via higher-layer (RRC) signaling - FFS: value of MR - This is an optional UE capability Proposal 2.A.2: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, if MR is configured, the supported value(s) of MR are as follows: - Rel-15 Type-I SP: {1, 2} - Rel-16 eType-II: {1} FL assessment: This FFS needs resolution. | Support/fine: Huawei/HiSi, IDC, MediaTek, ZTE, CATT, NTT DOCOMO, LG, IDC, OPPO, vivo, Spreadtrum, Qualcomm, Nokia/NSB, Intel, CMCC, Samsung, Apple, Ericsson, Xiaomi (ok), HONOR, Fujitsu, Google, Sharp, Lenovo/MotM, Tejas (ok) Not support: TCL (only 1), |
| 2.2.1 | Proposal 2.B.1: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding CBSR and RI restriction, support resource-specific specific CBSR - FFS (by RAN1#118): Whether RI restriction is resource-common or resource-specific FL assessment: This issue was discussed OFFLINE [2] | Support/fine: Lenovo/MotM, CATT, Xiaomi, Qualcomm, Apple, New H3C, TCL, Samsung, MediaTek, Ericsson, NEC, Fujitsu, ZTE, NTT DOCOMO, Spreadtrum, Huawei/HiSi, Google, CMCC, IDC, Sharp, KDDI, Tejas, Kyocera, HONOR, LG, vivo (ok), Nokia/NSB, Intel, OPPO (ok), Not support: |
| 2.4.1 | Proposal 2.D.1: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding timeline: - Multiply legacy Z’ by a factor of max(M,2). - Z is increased by max(M–1),1)*Z’ to match the increase in Z’ FL assessment: This proposal is a synthesis between proposals from vivo and Qualcomm | Support/fine: vivo, Qualcomm, Huawei/HiSi, Apple, NTT DOCOMO, LG, IDC, Spreadtrum, Qualcomm, Nokia/NSB, CMCC, Samsung, Ericsson (as long as Alt3 OCPU), Sharp, MediaTek, Lenovo/MotM (ok), Not support: MediaTek (legacy), Intel (legacy), Xiaomi (legacy), HONOR (legacy), ZTE (Ks.Z’), Google (legacy), |
| 2.4.2 | Proposal 2.D.2: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, OCPU = KS FL assessment: Reusing the legacy OCPU =Ks is reasonable as long as Z/Z’ is adjusted accordingly (2.D.1) considering the extra computations needed for M>1. I is also a middle ground between Alt4/5 (less than legacy) and Alt2/6 (more than legacy). Question 2.D.2: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, please share your preference on the following alternatives and justify your choice: - Alt2: OCPU = M + KS – 1 - Alt3 (legacy): OCPU = KS - Alt4: OCPU = M + 1 - Alt5: OCPU = X.M where X={1,2,3} based on UE capability - Alt6: OCPU = Ks + X.(M-1), where X={0,1} based on UE capability Alt2: vivo, Alt3 (legacy): ZTE, TCL, Xiaomi, Apple, NTT DOCOMO, Qualcomm, NEC (2nd), OPPO, Spreadtrum (if 2.D.1), Nokia/NSB, Intel, CMCC, Samsung, Ericsson, HONOR, Fujitsu, Sharp, Alt4: Huawei/HiSi, NEC (1st), MediaTek, Alt5: Google, Alt6: LG | Support/fine: ZTE, TCL, Xiaomi, Apple, NTT DOCOMO, Qualcomm, NEC (ok), OPPO, Spreadtrum (if 2.D.1), Nokia/NSB, Intel, CMCC, Samsung, Ericsson, HONOR, Fujitsu, Sharp, MediaTek (ok), Lenovo/MotM (ok), Not support: Huawei/HiSi, LG, vivo, Google |
| 2.5 | [117] Agreement For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, for M>1, support the following: - Resource-specific RI, i.e. RI is independently calculated and indicated for each of the selected M NZP CSI-RS resources - FFS: If resource-common RI indication is also supported Conclusion 2.E: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, for M>1, there is no consensus to support resource-common RI indication (in addition to the agreed resource-specific RI) Question 2.E: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, for M>1, please share your view whether resource-common RI indication should also be supported (in addition to the agreed resource-specific RI) Yes: Xiaomi, TCL, Huawei/HiSi, CATT, Tejas, NTT DOCOMO, LG, New H3C, No: vivo, Samsung, NEC, Qualcomm, Lenovo/MotM, Ericsson, Nokia/NSB, Google, Intel, CMCC, MediaTek, Fujitsu, Sharp, OPPO, Spreadtrum, Apple, ZTE, FL assessment: This FFS needs resolution. If the situation hasn’t changed from RAN1#116bis and 117, we will conclude no consensus in Round-1 |  |

Table 2B SLS results: issue 2

| Company | SLS results |  |  |
| --- | --- | --- | --- |
|  | Issue # | Metric | Observation |
| Huawei | 2.1.2 | Throughput | For Type I codebook, with MR =1/2 (M=4) gives performance gain of around 9.5/16% respectively compared to MR =0 For eType II codebook, with MR =1 gives performance gain of around 17.5% compared to MR =0 (a) Type-I codebook (M=4) (b) eType-II codebook (M=2) Relative throughput with different MR |

Table 2C Additional inputs: issue 2

| Company | Input |
| --- | --- |
| Mod V0 | Please share your inputs on each of the issues and, if applicable, proposals in TABLE 2A |
| InterDigital | Proposal 2.A.2 Support Proposal 2.D.1 Support |
| New H3C | Proposal 2.A.1：OK Proposal 2.B.1：OK Question 2.E：Yes |
| NEC | Question 2.D.2: Our first preference is Alt4, as resource selection can be achieved based on such as RSRP measurement (1 CPU occupied), and then M CPUs can be occupied for CSI measurement. We can also be fine to introduce two capabilities similar as other enhancement, capability 1 with Alt 3, and capability 2 with Alt 4. [Mod: Having 2 caps on this is an overkill. We will choose 1] |
| NTT DOCOMO | Proposal 2.A.2: support. Proposal 2.D.1: The extended timeline is okay to us if occupied CPU is not further increased. Question 2.E: we support NW to configure between resource-common RI restriction and resource-specific RI restriction. |
| LG | Proposal 2.A.1: Support Proposal 2.A.2: Support Proposal 2.B.1: Support Proposal 2.D.1: Support Question 2.D.2: Since M CSIs are reported, it makes sense to increase the number of CPU. Meanwhile, depending on UE implementation, legacy CPU calculation, Alt 3, can also be reused. Therefore, we suggest the following to merge Alt 2 and Alt 3: OCPU = Ks + X.(M-1), where X={0,1} based on UE capability Question 2.E: Support resource-common RI |
| OPPO | Proposal 2.A.1: Support. Proposal 2.A.2: We could live with it though we prefer the same value of Mr (=1 only) for both codebooks. Proposal 2.B.1: As compromise, is it possible to agree on Alt 3 (resource-specific CBSR and resource common RI restriction)? Question 2.D.2: We are fine with Alt3 considering at most Ks CSI should be calculated. Question 2.F: No. |
| vivo | 2.A.2 OK 2.B.1 Given majority view, we are ok to accept resource-specific CBSR. |
| Spreadtrum | Proposal 2.A.2: Fine with the proposal. Proposal 2.D.1: Support. Question 2.D.2: This question depends on the outcome of proposal 2.D.1. If proposal 2.D.1 is agreed, Alt3 is preferred. Question 2.E: No need to additionally support resource-common RI indication. |
| Qualcomm | Proposal 2.A.2: OK Proposal 2.D.1: Simplify our proposal as following: Increase by Z’ over legacy Z or Z’ The reason is, current max # ports per resource is doubled w.r.t. legacy, for either Ks=2 or 2<Ks<=8 Legacy max # ports per resource; Rel-19 max # ports per resource. Ks=2; 16; 32. 2<Ks<=8; 8; 16. We can be flexible with current proposal, if the following changes (i.e. at least doubled) can be considered: Proposal 2.D.1: For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding timeline: - Multiply legacy Z’ by a factor of maxmin(M,2). - Z is increased by maxmin(M–1,1)*Z’ to match the increase in Z’. |
| Nokia | Proposal 2.A.1 Support Proposal 2.A.2 Ok Proposal 2.B.1 Ok Proposal 2.D.1 Ok Proposal 2.D.2 Preference for Alt3 because it counts the number of CSI calculations and is consistent with the OCPU for cri-based reporting |
| Qualcomm | (Correct a typo above in our reply) |
| Intel | Proposal 2.A.1: Support Proposal 2.A.2: Fine. Further, given that there is no additional complexity/overhead, we are fine to include MR to 1,…,M. Proposal 2.B.1: Fine to accept the proposal. Proposal 2.D.1: In our understanding, higher complexity for CSI is already captured in CPU occupancy. Thus, increase of computation time might not be needed at least for some UEs. Thus, we propose to support legacy timeline. As a compromise, we are fine to support 2 timeline capabilities as for PMI codebook extension. Question 2.D.2: Our preference is Alt 3 (legacy). To moderator: could you please remove our name from Alt2 and add to Alt3. Thanks. Question 2.E: Resource-common RI is not needed in our view. |
| CMCC | Proposal 2.A.1 Support Proposal 2.A.2 Support Proposal 2.B.1 Support and prefer RI restriction is also resource-specific. Proposal 2.D.1 Ok Proposal 2.D.2 Prefer Alt3 to align with legacy. At most Ks CSIs will be calculated and the value of M only matters the reporting behaviour. |
| Samsung | Proposal 2.A.2 Support Proposal 2.B.1 Support Proposal 2.D.1 Ok. Question 2.D.2: Support Alt 3, OCPU = KS CPU occupation |
| Mod V18 | Added conclusion 2.E Revised proposal 2.D.1 (same essence, just to ensure values are in range) per Qualcomm input |
| Apple | Proposal 2.A.1: Support Proposal 2.A.2: We are fine Proposal 2.B.1: We are fine. But we need to discuss whether CBSR of all the resources have to be configured, similar as Rel-18 discussion [Mod: Thanks for pointing this out. Will do in later round(s)] Proposal 2.D.1: We are fine Question 2.D.2: Alt3, to follow legacy rule Conclusion 2.E: We are fine |
| Ericsson | Proposal 2.A.2: Ok Proposal 2.D.1: Has similar question as Intel. Need to ensure that we don’t increase both the timeline as well as CPU occupancy. Suggest to discuss this proposal together with Question 2.D.2 |
| Xiaomi | Proposal 2.A.2: We are fine if major companies could live with it. [Mod: Thank you Zhengxuan] Proposal 2.D.1: We think legacy timeline could be reused. The only difference is the number of CSI-RS ports is different from legacy. Multiple CRI-based reporting does not have impact on timeline, since UE still calculate CSI for the Ks resources. |
| HONOR | Proposal 2.A.1: Support. The mapping order for the UCI in second part should be clarified. The M sets of {PMI, LI (if applicable), CQI values for 2nd CW (if applicable)} is ordered according to the mapping order of M CRIs in the Part 1. [Mod: Later round(s) a part of UCI omission 😊] Proposal 2.A.2: Support. Proposal 2.B.1: Generally fine. We prefer to further support resource-common CBSR in addition to resource-specific CBSR to reduce signaling overhead. Proposal 2.D.1 and Question 2.D.2: Prefer to extend the number of CPU and reuse the legacy timeline, i.e., Support Alt3 in Question 2.D.2 and no enhancement on timeline. |
| Huawei, HiSilicon | Proposal 2.A.1: Support. Proposal 2.A.2: Support. Proposal 2.B.1: Support. More specifically, as elaborated in [2], two-level CBSR that satisfying the requirement of both inter- and intra-cell interference handling with significant RRC overhead reduction should be considered. [Mod: Later round(s) 😊] Proposal 2.D.1: Prefer to also scale Z by M. Question 2.D.2: Support Alt 4. As discussed in our contribution, given that the required CPU of channel estimation and CRI selection procedure for multi-beam CSI reporting is 1 (similar to the RSRP/SINR measurement), while that of RI/PMI/CQI calculation procedure is M (complete CSI calculation is only needed for the selected M resources), Alt 4 is a reasonable candidate value. |
| Mod V27 | Added proposal 2.D.2 Overall, agreeing both 2.D.1 and 2.D.2 is a good compromise for almost all companies 😊 |
| ZTE/ZTE2 | 2.A.1: Support. 2.A.2: Support. 2.B.1: Support. For the FFS point, since different analog beams point in different directions, the channel conditions would be significantly different for different beams. Therefore, resource-specific RI restriction is more suitable for the CRIs-based CSI reporting. 2.D.1: We are fine to extend Z/Z’. However, our views are as follows: - Regarding Z', since the computational complexity is mainly related to the number of CSI-RS resources KS (UE needs to measure the channel and calculate the CSI for each CSI-RS resource), we prefer to relax the timeline restriction as KS*Z’. (A typo is fixed, i.e., from ‘Kz*Z’ to ‘Ks*Z’’). - Regarding Z, considering multiple CSI-RS resources are configured for CRIs-based CSI reporting, Z should be extended as Z+ w to cover all the resources, where w can be a fixed value or determined by the extension of Z’. However, if Z is only increased by the extension of Z’, Z may NOT be capable of covering all the resources. 2.D.2: Support Alt 3, since UE need to measure all the KS CSI-RS resources. 2.E: OK with the conclusion. |
| Fujitsu | 2.A.1: Support. 2.A.2: Support. 2.B.1: Prefer resource-specific RI restriction only. Question 2.D.2: Support Alt 3. Conclusion 2.E: Support. |
| Google | Proposal 2.A.1: Support. Proposal 2.A.2: Support. Proposal 2.B.1: Support. Proposal 2.D.1: We do not quite understand the motivation for the equations. What is the problem for legacy approach? Proposal 2.D.2: We think the Ocpu should be based on the value of M instead of Ks. Our first preference is Alt5, and second preference is Alt5. Conclusion 2.E: Support |
| OPPO1 | Proposal 2.B.1: We could be fine with the proposal. Proposal 2.D.2: Support the proposal. |
| Sharp | Proposal 2.A.2: Support. Proposal 2.D.1: Support the proposal to scale Z by the increased number of reported CRIs. Proposal 2.D.2: Support Alt 3. |
| MediaTek | Proposal 2.D.1, 2.D.2 OK |
| Mod V40 | No revision |
| Lenovo/ MotM | Proposal 2.A.1: Support. Proposal 2.A.2: Prefer {1,2} for both CB types, given that Type-II has already been restricted to RI=1 @M=2 [Mod: Added FFS for {2} for Type-II. Since you are ok with the proposal (just want 1 more value), this can be discussed in round-2] Proposal 2.B.1: Support. Proposal 2.D.1/2: We can live with it but prefer legacy timeline and only increase OCPU Conclusion 2.E: Our preference is resource common RI restriction. Motivation to support per resource RI restriction is not clear |
| Mod V45 | Added FFS for {2} for Type-II in proposal 2.A.2 per Lenovo’s comment |
| Tejas | Proposal 2.A.2: We want to clarify that the motivation behind (floor(M/2)) is to have an implied implementation based on the value of M which in turn decides the UE co-scheduling opportunities for transmission. Hence, if larger number of beams are reported, say 3 or 4, implicit reporting is for 2 beams, else if it is for 1 or 2, MR = 1 which may also be logical. In this sense, supported values remain {1, 2}. It may also eliminate the need for configuring MR value via higher layer signalling. We are fine to support {1, 2} as well, being explicitly assigned for Type I and eType II. [Mod: Thank you] |
| Mod V51 | No revision |

### Issue 3 (WID objective 3): CJT calibration reporting for non-ideal synchronization and backhaul

Table 3A Summary: issue 3

| # | Issue |  | Companies’ views |
| --- | --- | --- | --- |
| 3.1 | [117] Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), if >1 (sub-band reporting) is supported, select one from option 1 and option 2, by RAN1#118, from the following: - A sub-band size is selected from {8,16} PRBs - The sub-band size is NW-configured via higher-layer (RRC) signalling - Denoting the number of sub-bands within the configured CSI reporting band as NSB-P, and the sub-bands are indexed as {0, 1, …, NSB-P –1}, decide, by RAN1#118, from the following reporting options: - Opt1: {(n,, n), n=0, 1, …, NTRP – 1, n≠nref}, where n,is the phase offset corresponding to sub-band 0 and the phase offset for sub-band  can be calculated as n, + n - Γn∈0,2πMΓ, ….,2π(MΓ-1)MΓ, where MΓ={64, 128} - Opt2: = NSB-P, i.e. {(n,, n,, n,NSB-P), n=0, 1, …, NTRP – 1, n≠nref} - The alphabet for n, follows the previously agreed alphabet for =1, including the ‘invalid’ state - The maximum NSB-P is [4] - FFS: For all the above reporting options, the UE performs measurement over the entire configured CSI reporting band - FFS: Further restriction on CSI-RS (e.g. RE density) Note: Companies to report whether their evaluation is based on precoded or non-precoded CSI-RS Proposal 3.A.1: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the support of sub-band reporting (>1): - Denoting the number of reported sub-band phase-offset values as NSB-P, and the sub-bands are indexed as {0, 1, …, NSB-P –1}, support, as a separate UE capability from wideband (=1) phase offset reporting, reporting, in one CSI reporting instance, {(n,, n,, n,NSB-P), n=0, 1, …, NTRP – 1, n≠nref} - The alphabet for n, follows the previously agreed alphabet for =1, including the ‘invalid’ state - FFS (by RAN1#118): - Supported sub-band size(s), e.g. following the legacy CSI sub-band definition, vs {4, 8, 16 PRBs} - Whether the UE performs measurement over the entire configured CSI reporting band WCSI - If needed, mechanism to limit CSI reporting overhead (e.g. maximum NSB-P) Note: UE is not required to perform DO/FO compensation for sub-band phase offset calculation FL assessment: This issue was discussed OFFLINE [2]. Supporting BOTH Opt1 and Opt2 have been precluded in RAN1#117 agreement – see above (therefore I will not accommodate such proposal – kindly please don’t re-propose this again 😊). |  | Support/fine: CATT, Samsung, NTT DOCOMO, Ericsson, OPPO (ok), Lenovo/MotM, Intel (ok), Apple, Fujitsu, TCL, Huawei/HiSi, Google, Sharp, KDDI, CMCC, NEC (ok), NICT, Nokia/NSB (ok), New H3C, Xiaomi (ok), ZTE (ok), Not support: Qualcomm (Opt1), Sony (Opt1), IDC (no sub-band), |
| 3.2 | [117] Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), - Regarding the number of configured associated SRS resource(s) (=Q) for antenna switching xTyR, support at least Q=1 where: - the configured associated SRS resource is selected from all the y/x SRS resources and all the configured resource set(s) - FFS (by RAN1#118): whether Q>1 is also supported - FFS (by RAN1#118): the supported value(s) of x - Regarding how to determine the SRS port corresponding to the ‘reference UE antenna port’, support PSRS =1 SRS port selected from all the ports from the configured Q associated SRS resource(s) - FFS (by RAN1#118): Whether PSRS >1 is also supported Conclusion 3.B.1: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the number of configured associated SRS resource(s) (=Q) for antenna switching xTyR and the number of SRS ports (=PSRS) corresponding to the ‘reference UE antenna port’, there is no consensus to support the following: - Q>1 - PSRS >1 Note: This implies no support for configuring >1 SRS resource sets for associated SRS when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset) Proposal 3.B.2: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), - The supported value(s) of x follows the supported configuration(s) of SRS resource for antenna switching xTyR used for reciprocity-based DL CSI acquisition - Note: This doesn’t have any spec impact (no new RRC parameter is needed) - For the selection of PSRS=1 SRS port corresponding to the ‘reference UE antenna port’ (out of the available port(s)), decide, by RAN1#118, from the following schemes: - Scheme1: NW-configured via higher-layer (RRC) signalling - Scheme1B: NW dynamic selection via MAC CE or A-CSI triggering (DCI) - Scheme2: UE-selected, the selection included in the phase offset report - Scheme3: Fixed rule, e.g. the first out of the available port(s) FFS: - Whether/how to identify the transmission occasion of the Q=1 associated SRS resource to determine the reference UE antenna port in relation to the CSI request and/or SRS triggering - The supported time-domain behaviour(s) for the associated SRS resource (periodic, semi-persistent, aperiodic) - In case the NW configures the Q=1 associated SRS resource from an existing SP and/or AP SRS antenna switching resource configuration for DL CSI acquisition (which utilizes dynamic signalling), whether/how to identify the Q=1 associated SRS resource FL assessment: This issue was discussed OFFLINE [2]. Summary re 3.B.1/2: Support Q>1? - Yes: Qualcomm, - No (can accept no): Samsung, NTT DOCOMO, Ericsson, vivo, OPPO, NEC, Lenovo/MotM, CATT, Intel, ZTE, Apple, Huawei/HiSi, TCL, Xiaomi, Sony, Sharp, Google (ok), KDDI, IDC, CMCC, ETRI, New H3C, Nokia/NSB, Support PSRS>1? - Yes: Qualcomm, - No (can accept): Samsung, NTT DOCOMO, Ericsson, vivo, OPPO, NEC, Lenovo/MotM, CATT, Intel, ZTE, Apple, Huawei/HiSi, TCL, Xiaomi, Sony, IDC, Sharp, Google (ok), KDDI, IDC, CMCC, ETRI, New H3C, Nokia/NSB, x following xTyR? - Yes: NTT DOCOMO (open), Ericsson, vivo, OPPO, CATT, ZTE, Apple, Qualcomm, Huawei/HiSi, TCL, Google, Xiaomi, Sony, Sharp, Samsung (ok), KDDI, IDC, CMCC, New H3C, NEC, Nokia/NSB, - No (only x=1): Intel, |  | Conclusion 3.B.1 is the inevitable logical outcome whether one likes/supports it or not. Note that this was already discussed since RAN1#117 3.B.2: Support/fine: NTT DOCOMO, Ericsson, vivo, OPPO, CATT, ZTE, Apple, Huawei/HiSi, TCL, Xiaomi, Lenovo/MotM, Sharp, Google, KDDI, IDC, Samsung, Qualcomm, New H3C, NEC, Nokia/NSB, Intel, Sony, CMCC, Not support: |
| 3.3 | Proposal 3.C.1: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, to facilitate UE-specific delay offset pre-compensation on PDSCH by the NW, decide, by RAN1#118, whether to support configuring a UE (via RRC signaling) to perform PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported delay offset (when ReportQuantity is ‘cjtc-Dd’). And if supported, whether any of the following is additionally supported or not: - NW indicates the delay offset value to be compensated for the Rel-18 eType-II CJT CSI report, and/or - The two separately configured reports (i.e. Rel-18 eType-II CJT CSI report and the CJTC delay offset report) are always jointly triggered and carried on a same PUSCH (hence on a same slot) - The delay offset value to be compensated is the latest reported DO before the DCI triggering the CJT CSI reporting Only AP-CSI-RS can be configured for the Rel-18 eType-II CJT report The above only applies when the CMRs do not share common QCL source for average delay indication Proposal 3.C.2: For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, to facilitate UE-specific frequency offset pre-compensation on PDSCH by the NW, decide, by RAN1#118, whether to support configuring a UE (via RRC signaling) to perform PMI calculation for the Rel-18 eType-II CJT CSI report assuming pre-compensation using the UE-reported frequency offset (when ReportQuantity is ‘cjtc-F’). And if supported, whether any of the following is additionally supported or not: - NW indicates the frequency offset value to be compensated for the Rel-18 eType-II CJT CSI report, and/or - The two separately configured reports (i.e. Rel-18 eType-II CJT CSI report and the CJTC frequency offset report) are always jointly triggered and carried on a same PUSCH (hence on a same slot) Only AP-CSI-RS can be configured for the Rel-18 eType-II CJT report The above only applies when the CMRs do not share common QCL source for average delay indication FL assessment: The above proposals further enhance the utility of delay and frequency offset reporting by co-configuration with Rel-18 Type-II CJT. This was included in Round-1 summary for RAN1#117 but only discussed briefly. Assuming that companies have looked into this issue, we will discuss more in RAN1#118 and aim for decision whether to support or not, also in RAN1#118. UE-specific PDSCH delay offset compensation: - Support/fine: vivo, ZTE, Qualcomm, Ericsson, CATT, Nokia/NSB, IDC, Samsung (ok), Huawei/HiSi (ok), Xiaomi, - Not support: MediaTek, OPPO, Intel, Apple (too early), UE-specific PDSCH frequency offset compensation: - Support/fine: vivo, ZTE, Ericsson, Samsung (ok), Xiaomi, - Not support: MediaTek, CATT, OPPO, Qualcomm, Nokia/NSB, Intel, Apple (too early), Huawei/HiSi (ok), |  | 3.C.1: proposal for discussion, not support - Support/fine: vivo, ZTE, Qualcomm, Ericsson, CATT, Nokia/NSB, IDC, Huawei/HiSi, MediaTek, Samsung, OPPO, Intel, Apple, Xiaomi, Sony, Fujitsu, Google, Sharp, Lenovo/MotM, CMCC, TCL, - Not support: 3.C.2: proposal for discussion, not support - Support/fine: vivo, ZTE, Qualcomm, Ericsson, MediaTek, Samsung, CATT, IDC, OPPO, Nokia/NSB, Intel, Apple, Xiaomi, Sony, Huawei/HiSi, Fujitsu, Google, Sharp, Lenovo/MotM, TCL, - Not support: |
| 3.4 | Conclusion 3.D.1: For the Rel-19 aperiodic standalone CJT calibration reporting, there is no consensus to support the following reporting formats: - [Joint Dd + wideband PO reporting] - Joint FO + wideband PO reporting - Joint Dd + FO + wideband PO reporting Conclusion 3.D.1 is the inevitable logical outcome whether one likes/supports it or not. Note that this was already discussed since RAN1#117 and the situation doesn’t change much (in fact the support is even weaker now) FL assessment: This issue was discussed OFFLINE [2] and since RAN1#117. The situation hasn’t changed (even more opposing companies) Summary: Joint Dd + wideband PO reporting: - Support/fine: OPPO (if SB PO not supported), vivo, Qualcomm, Sony (if SB PO not supported), Samsung (ok), ZTE, Google, - Not support: Huawei/HiSi, Lenovo/MotM, MediaTek, NTT DOCOMO, Ericsson, NEC, Intel, Apple, Fujitsu, TCL, Xiaomi, IDC, Sharp, KDDI, CMCC, ETRI, Joint FO + wideband PO reporting: - Support/fine: - Not support: Samsung, OPPO, Huawei/HiSi, vivo, Lenovo/MotM, Sony, MediaTek, NTT DOCOMO, Ericsson, NEC, Intel, Apple, Fujitsu, TCL, Xiaomi, IDC, Sharp, KDDI, CMCC, ETRI, Joint Dd + FO + wideband PO reporting: - Support/fine: - Not support: Samsung, OPPO, Huawei/HiSi, vivo, Lenovo/MotM, Sony, MediaTek, NTT DOCOMO, Ericsson, NEC, Intel, Apple, Fujitsu, TCL, Xiaomi, IDC, Sharp, KDDI, CMCC, ETRI, |  |  |
| 3.5.1 | [116bis] Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, the dynamic range and resolution parameters for frequency offset reporting FOn, i.e. (AFO, MFO), are NW-configured via higher-layer (RRC) signalling from the following candidate values: - … - MFO = {16,32} Proposal 3.E.1: For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the resolution for frequency offset reporting FOn, additionally support MFO = 256 FL assessment: The above proposal is motivated by additional results showing that the agreed 5 bits isn’t sufficient in some important deployment scenarios |  | Support/fine: vivo, Ericsson, Samsung, NTT DOCOMO, Qualcomm (UE cap), Nokia/NSB, Intel, Apple, Sony, Huawei/HiSi, CATT, ZTE, Google, Sharp, Lenovo/MotM, CMCC, KDDI, TCL, Not support: |
| 3.5.2 | [117] Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the dynamic range for delay offset reporting Dn,offset, i.e. AD, at least support the following values: {0.5CP, CP} - Decide, by RAN1#117, whether any of the following candidate values are supported: {0.75CP, 1.5CP, 14∆f, 112∆f,124∆f [117] Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the dynamic range for frequency offset reporting FOn, i.e. AFO, at least support the following values: {0.1ppm, 0.2ppm} - Decide, by RAN1#117, whether any of the following candidate values are supported: {0.025ppm, 0.05ppm, 1/(8t), 1/(16t), 1/(32t)} Conclusion 3.E.2: For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the dynamic range for delay offset reporting Dn,offset and frequency offset reporting FOn, there is no consensus on supporting additional values of dynamic range Question 3.E.2: For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the dynamic range for delay offset reporting Dn,offset and frequency offset reporting FOn please share your views on the support of the remaining candidates below - While having a single unit looks “nice” from spec perspective and uniformity, there is no value in terms of system implementation complexity for both NW and UE. Everyone agrees that having a single unit is preferred, but cannot agree on which unit 😊 Parameter; Value; Company view. AD; 0.75CP; Support/fine: Intel, Ericsson, NTT DOCOMO, Xiaomi, Google, NEC, Sharp, KDDI, IDC, OPPO Not support: ZTE, Nokia/NSB, Samsung. 1.5CP; Support/fine: Samsung, Ericsson, NTT DOCOMO, Fujitsu, NEC, KDDI, Intel, Qualcomm (optional), Sony Not support: ZTE, Nokia/NSB, Xiaomi, Google, Sharp, IDC, OPPO,. 14∆f; Support/fine: Huawei/HiSi, Nokia/NSB, CATT Not support: Samsung, OPPO, Apple, Intel, Xiaomi, Google, Fujitsu, NEC, Sharp, KDDI, IDC, Kyocera,. 112∆f; Support/fine: ZTE, Huawei/HiSi, Nokia/NSB, CATT Not support: Samsung, OPPO, Apple, Intel, Xiaomi, Google, Fujitsu, NEC, Sharp, KDDI, IDC, Kyocera,. 124∆f; Support/fine: ZTE, Huawei/HiSi, Qualcomm, vivo, CATT Not support: Samsung, OPPO, Apple, Intel, Xiaomi, Google, Fujitsu, NEC, Sharp, KDDI, IDC, Kyocera,. AFO; 0.025ppm; Support/fine: Qualcomm, Sony Not support: Samsung, OPPO,. 0.05ppm; Support/fine: Qualcomm, Sony Not support: Samsung, OPPO,. 1/(8t); Support/fine: ZTE, Huawei/HiSi, CATT Not support: Samsung, Apple, Intel, Ericsson, Xiaomi, Fujitsu, NEC, IDC, Kyocera,. 1/(16t); Support/fine: ZTE, Huawei/HiSi, Nokia/NSB, CATT Not support: Samsung, Apple, Intel, Ericsson, Xiaomi, Fujitsu, IDC, NEC, Kyocera,. 1/(32t); Support/fine: Huawei/HiSi, Nokia/NSB, CATT Not support: Samsung, Apple, Intel, Ericsson, Xiaomi, Fujitsu, IDC, Kyocera,. FL assessment: This is from RAN1#117 Round-3. Please check if your view remains the same or changes. If the situation isn’t much different, I’d have to conclude no consensus on supporting additional values in Round-1. |
| 3.6 | [117] Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the applicable type(s) of the configured NTRP NZP CSI-RS resources when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), one ‘CSI-RS for CSI’ resource set with NTRP resources is supported - FFS: 1 CSI-RS resource set with J⋅NTRP single-port CSI-RS resources where J>1 is the number of CSI-RS resources per TRP - FFS: 1 or NTRP TRS resource sets can also be used when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset) Conclusion 3.F: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), there is no consensus on supporting the following optimization schemes (in addition to 1 ‘CSI-RS for CSI’ resource set with NTRP resources): - Optimization 1: 1 CSI-RS resource set with J⋅NTRP single-port CSI-RS resources where J>1 is the number of CSI-RS resources per TRP - Optimization 2: 1 or NTRP TRS resource sets can also be used when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset) Question 3.F: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), please share your views on the need for supporting the following optimization schemes (in addition to 1 ‘CSI-RS for CSI’ resource set with NTRP resources): - Optimization 1: 1 CSI-RS resource set with J⋅NTRP single-port CSI-RS resources where J>1 is the number of CSI-RS resources per TRP - Support/fine: CATT, Qualcomm, NTT DOCOMO (open), Google (open), - Not support: Samsung, OPPO, Intel, Sony, Huawei/HiSi, ZTE, Sharp, CMCC, - Optimization 2: 1 or NTRP TRS resource sets can also be used when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset) - Support/fine: Nokia/NSB, Apple, NTT DOCOMO, ZTE, - Not support: Samsung, OPPO, Intel, Sony, Huawei/HiSi, Google, Sharp, CMCC, FL assessment: This FFS needs resolution. |  |  |
| 3.7.1 | [117] Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, the UCI parameters are captured in the tables below: - … - FFS: {When ReportQuantity is ‘cjtc-Dd’ (Doffset+d)} Whether it is possible not to report dn Conclusion 3.G.1: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-Dd’ (Doffset+d), there is no consensus on supporting the following optimization schemes: - Optimization 1: Additional configuration for not reporting {dn} (to save up to NTRP –1 bits) - Optimization 2: Additional configuration for using dn to either extend the dynamic range (by 2x) or increase the resolution (by 2x) of Dn,offset Question 3.G.1: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-Dd’ (Doffset+d), please share your view on the following optimization schemes: - Optimization 1: Additional configuration for not reporting {dn} (to save up to NTRP –1 bits) - Support/fine: Qualcomm, TCL, Sharp, NEC, OPPO, - Not support: NTT DOCOMO, Samsung, Nokia/NSB, Intel, Apple, Xiaomi, Huawei/HiSi, ZTE, Fujitsu, Google, CMCC, KDDI, - Optimization 2: Additional configuration for using dn to either extend the dynamic range (by 2x) or increase the resolution (by 2x) of Dn,offset - Support/fine: CATT, NTT DOCOMO (open), Qualcomm, - Not support: Samsung, OPPO, Nokia/NSB, Intel, Apple, Xiaomi, Huawei/HiSi, ZTE, Fujitsu, Google, CMCC, KDDI, FL assessment: This FFS needs resolution. Optimization 1 was initially proposed by Qualcomm and 2 by CATT. |  |  |
| 3.7.2 | [117] Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, the UCI parameters are captured in the tables below: - FFS: Mapping order - … Proposal 3.G.2: For the Rel-19 aperiodic standalone CJT calibration reporting, the UCI mapping order is as follows: - When ReportQuantity is ‘cjtc-Dd’ (Doffset+d): - nref, - {Dn,offset, n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID, - {dn, n=0, 1, …, N TRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID - When ReportQuantity is ‘cjtc-F’ (FO): - nref, - {FOn, n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID, - When ReportQuantity is ‘cjtc-Dd-F’ (joint Doffset+d and FO): - nref1, - nref2, - {Dn,offset, n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID, - {dn, n=0, 1, …, N TRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID - {FOn, n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID, FL assessment: The UCI table and ordering for PO-related reports will have to wait until the associated SRS port selection as well as sub-band PO issues are resolved. | Support/fine: Huawei/HiSi, ZTE, Ericsson, Google, Sharp, Lenovo/MotM, TCL, Not support: |  |
| 3.8 | [116bis] Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, regarding QCL assumptions, each of the NTRP DL-RSs can be configured with a TCI state via RRC ignalling - ‘The DL-RS’ refers to the CSI-RS configured for CJT calibration measurement - FFS: Whether additional constraints are needed for the TCI state of the ‘DL-RS’ Question 3.H: For the Rel-19 aperiodic standalone CJT calibration reporting, regarding QCL assumptions, please share your view whether additional constraints are needed for the TCI state of the ‘DL-RS’ - Yes: Google - No: ZTE, CMCC, FL assessment: This FFS needs resolution. |  |  |
| 3.9.1 | [116bis] Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the applicable type(s) of the configured NTRP NZP CSI-RS resources/resource sets when ReportQuantity is ‘cjtc-Dd’ (Doffset+d) or ‘cjtc-F’ (frequency offset), periodic TRS (‘CSI-RS for tracking’) resource set is used for each of the NTRP NZP CSI-RS resource sets - … - FFS: Whether aperiodic TRS resource set can also be used - … - FFS: Whether different RE locations (FDM) are supported for the RSs - … Conclusion 3.I.1: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-Dd’ (Doffset+d) or ‘cjtc-F’ (frequency offset), there is no consensus on additionally supporting the following optimization schemes: - Optimization 1: Aperiodic TRS resource set can also be used - Optimization 2: Different RE locations (FDM) are supported for the RSs Question 3.I.1: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-Dd’ (Doffset+d) or ‘cjtc-F’ (frequency offset), please share your view on the need for additionally supporting the following optimization schemes: - Optimization 1: Whether aperiodic TRS resource set can also be used - Optimization 2: Whether different RE locations (FDM) are supported for the RSs Optimization 1: - Yes: IDC, Intel, - No: Spreadtrum, CMCC, CATT, Samsung, OPPO, Qualcomm, Apple, Xiaomi, Huawei/HiSi, Fujitsu, Google, Lenovo/MotM, KDDI, Optimization 2: - Yes: CATT, ZTE, - No: CMCC, IDC, Samsung, OPPO, Intel, Apple, Xiaomi, Huawei/HiSi, Fujitsu, Google, Lenovo/MotM, KDDI, FL assessment: The two FFS points need resolution. They were already discussed in RAN1#117 and no consensus was reached. However, two companies requested to postpone to RAN1#118. This is the situation in RAN1#117. If there is no significant change, I will conclude no consensus in round-1 RAN1#118: - Whether aperiodic TRS resource set can also be used - Yes: Intel, Fujitsu, Samsung, Sharp, Ericsson, NTT DOCOMO, Nokia/NSB, Lenovo/MotM, Sony, TCL, - No: Spreadtrum, CATT, OPPO, Qualcomm, - Whether different RE locations (FDM) are supported for the RSs - Yes: Huawei, ZTE, CATT, Nokia/NSB, Qualcomm, - No: Intel, Spreadtrum, NTT DOCOMO, Samsung, OPPO, Fujitsu, Lenovo/MotM, Sony, Intel, Google, |  |  |
| 3.9.2 | [116bis] Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the applicable type(s) of the configured NTRP NZP CSI-RS resources/resource sets when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), single-port CSI-RS(s) for CSI is used - … - FFS: Whether different RE locations (FDM) are supported for the RSs - … Conclusion 3.I.2: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), there is no consensus on additionally supporting the following optimization scheme: - Different RE locations (FDM) are supported for the RSs Question 3.I.2: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), please share your view on the need for additionally supporting the following optimization scheme: - Whether different RE locations (FDM) are supported for the RSs FL assessment: This FFS point needs resolution. It was already discussed in RAN1#117 and no consensus was reached. However, one company requested to postpone to RAN1#118. This is the situation in RAN1#117. If there is no significant change, I will conclude no consensus in round-1 RAN1#118: - Whether different RE locations (FDM) are supported for the RSs - Yes: ZTE, Nokia/NSB, CATT, - No: Samsung, OPPO, Fujitsu, Lenovo/MotM, IDC, Nokia/NSB, Intel, Apple, Xiaomi, Sony, Huawei/HiSi, Google, Lenovo/MotM, CMCC, KDDI, |  |  |

Table 3B LLS/SLS results: issue 3

| Company | LLS/SLS results |  |  |
| --- | --- | --- | --- |
|  | Issue # | Metric | Observation |
| Google | 3.2 | CDF of RSRP gain | Figure 1 illustrates the system simulation results for the RSRP gain for PO measurement and report. It can be observed that up to 4 dB performance gain can be achieved if multiple ports are configured compared to single port configuration. Figure 1: RSRP gain based on multiple configured ports with port selection compared to a RRC configured port |
| ZTE | 3.3 | Average UPT and cell-edge UPT | The figure below shows the SLS performance evaluation results for non-compensated CJT and UE-specific DO/FO pre-compensated CJT. It is observed that, the UE-specific DO/FO pre-compensation provides 56.41% throughput gain for cell-edge UE and 17.81 throughput gain in average. |
| vivo | 3.1 | Cell mean SE | The following observation can be obtained by the simulation results below. - For 8Tx per TRP and 2Rx per UE, when residual TE is 20ns and residual PE is 0.05pi, there is more than 5% performance loss. And, with the increase of residual error, the performance loss also increases. - When the number of Tx per TRP and the number of Rx per UE are doubled, even if the residual TE is 80ns, the performance loss is less than 5%. The comparison of SE performance for different maximum residual phase errors and maximum residual timing errors The comparison of SE performance for different maximum residual phase errors and maximum residual timing errors |
|  | 3.3 | Cell mean SE | The following observation can be obtained by the simulation results below. - UE compensation achieves larger performance gains of mort than 7% compared to the baseline. - gNB compensation using UE-specific CSI-RS has the worst performance gain. Compared to UE compensation, the performance loss is about 11% at 30k SCS and about 30% at 15k SCS. This is mainly due to the impact of more RS overhead on the data transmission. - gNB compensation of the average DO value does not result in a significant performance gain, compared to no DO compensation. - The comparison of SE performance for different approaches The comparison of SE performance for different approaches |
| CATT | 3.1 | Average throughput gain vs overhead | The performances of different reporting options versus overhead in precoded/non-precoded CSI-RS scenario, with ideal/realistic channel estimation are shown in Figure 1 to Figure 4 below, respectively. It is observed that Opt2 with 4 reporting subbands outperforms Opt1 and baseline scheme in all cases. Figure 1 Performances of different reporting options in precoded CSI-RS scenario with ideal channel estimation Figure 2 Performances of different reporting options in non-precoded CSI-RS scenario with ideal channel estimation Figure 3 Performances of different reporting options in precoded CSI-RS scenario with realistic channel estimation Figure 4 Performances of different reporting options in non-precoded CSI-RS scenario with realistic channel estimation |
| Sony | 3.1 | Average throughput | As shown in the figure below, 4% throughput gain of SB PO is observed over WB PO in a LLS simulation. |
| Samsung | 3.1 | Avg UPT gain vs overhead | For (a) non-precoded CSI-RS transmission approach: - Even in an intra-site inter-cell scenario where channel frequency-selectivity across TRPs is very mild, the performance degradation of option 1 is significant (6% loss from the ideal case) whereas option 2 incurs only 1% loss from the ideal case when Nsb=8. For (b) BFed CSI-RS transmission approach, - option 1 (6~7% loss at 0-6 dB SNR regime) is vulnerable to measurement error compared to option 2 (2-5% loss at 0-6dB SNR regime), due to the underlying assumption of linear phase drift for option 1 being broken at low SNR regime. For (a) non-precoded CSI-RS transmission approach, in an inter-site inter-cell scenario, UPT loss for option 1 is observed, in the figure below, compared to WB PO reporting due to large difference of channel frequency selectivity across TRPs, which results in that the underlying assumption of linear phase drift in FD for option 1 does not hold. |
| Nokia/NSB | 3.3 | Mean UE throughput vs overhead | It is shown in the SLS results below that ~4% performance loss is encountered in the one-sided case compared to no delay compensation, due to the mismatch between the channel seen by the PDSCH (after precoder compensation at gNB side) and the PMI actually calculated at UE side. Performance comparison with and without delay compensation of CSI-RS resources at the UE side for CSI calculation. Delay compensation is also applied at the gNB to calculate PDSCH precoder. |
|  | 3.1/3.2 | Average UE throughput and cell-edge UE throughput | Simulation results below comparing wideband and subband phase offset calibration reporting for a non-MRT-precoded CSI-RS network implementation show that wideband PO calibration reporting allows to recover virtually all the performance loss caused by inter-TRP PO misalignment and subband reporting does not provide any additional benefit. Comparison between wideband and subband PO measured from one receive antenna. Network implementation with non-MRT-precoded CSI-RS for calibration measurement. Simulation results below comparing wideband phase offset calibration reporting with multiple UE receive antennas, for a non-MRT-precoded CSI-RS network implementation, show that a single antenna wideband PO calibration reporting allows to recover virtually all the performance loss caused by inter-TRP PO misalignment if the UE antenna radiation patter is assumed isotropic and without blockages Comparison between wideband PO calculation measured from one or more UE antennas. Network implementation with non-MRT-precoded CSI-RS for calibration measurement. The performance by reporting subband PO calibration measurement averaged over multiple UE antennas was also evaluated, by using a non-MRT-precoded CSI-RS network implementation. The combination of these two features does not provide any significant improvement in performance, as shown in the figures below. Comparison between subband PO calibration measured from one or more receive antennas. Network implementation with non-MRT-precoded CSI-RS for calibration measurement. |
| Ericsson | 3.3 | Throughput vs SNR | Comparing the CJT performance with and without UE side DO/FO pre-compensation, the simulation results below demonstrate a 15% performance gain at SNR 10 dB with DO/FO pre-compensation. The figure below provides simulation results which illustrate that synchronizing CJT PMI reporting periodicity and DO/FO reporting periodicity does not provide any meaningful performance gain over the case when CJT PMI reporting periodicity and DO/FO reporting periodicity are different. |
|  | 3.1 | Throughput vs SNR | The figure below shows the performance impact by selecting two sets of subbands for subband phase offset reporting with subband size=8RBs. It can be seen that there is a large performance degradation when a wrong set of subbands are selected for subband phase offset reporting. Option 2 subband phase reporting with two different sets of subbands {2,3,4,5} vs {1,2,3,4}. Subband size = 8. |
| Qualcomm | 3.1 | Relative UPT vs DL SNR | From the LLS results below, the following observations can be made: - For CJT-TDD Scheme-A (FD-precoded CSI-RSs), Opt1 (phase+delay/TAE) outperforms the case of all 16 subbands (which is with massive UCI overhead). - For the proposed “artificial” restriction on # subbands, it may not work for some large-BW case. - For CJT-TDD Scheme-B (FD-non-precoded CSI-RS), the benefit of Opt1 (phase+delay/TAE) over Opt2 (subband phase) is reduced. |
|  | 3.2 | Relative UPT vs DL SNR | From the LLS results below, the following observation can be made. - For CJT-TDD Scheme-A (FD-precoded CSI-RSs), UE combining of >1 antenna ports provide performance gain over single antenna port. - For CJT-TDD Scheme-B (FD-non-precoded CSI-RSs), UE combining does not provide significant benefit over single antenna port. |

Table 3C Additional inputs: issue 3

| Company | Input |
| --- | --- |
| Mod V0 | Please share your inputs on each of the issues and, if applicable, proposals in TABLE 3A |
| New H3C | Proposal 3.A.1:：OK Conclusion 3.B.1:：OK Proposal 3.B.2:：OK |
| NEC | Proposal 3.B.2: Support Question 3.G.1: Optimization 1: Support. |
| NTT DOCOMO | Proposal 3.E.1: OK Question 3.F: Support Optimization 2. Open to Optimization 1. Question 3.G.1: We do not see the strong need for Optimization 1. For Optimization 2, we are open. |
| InterDigital | Proposal 3.A.1: Not support Conclusion 3.B.1: Support Proposal 3.B.2: Support, prefer Scheme 1 Proposal 3.C.1: Support Proposal 3.C.2: Support Conclusion 3.D.1: Support Question 3.E.2: Only a single unit for time and frequency should be specified. Question 3.I.1: Optimization 1: Support Optimization 2: Not support Question 3.I.2: Not support |
| Samsung | Proposal 3.A.1. Support. Proposal 3.B.2. Support. Re the scheme for the selection of Psrs=1 SRS port, the need of dynamic port selection by UE is weak, given that the antenna diversity gain at a given UE is marginal. Scheme 1 or scheme 3 should suffice in our view. Proposal 3.C.1 Support. Initially, our preference was not supportive on the proposal, considering some case (case 1) where there exist PDP values exceeding CP. This case will be suffered from ISI and have undesirable performance when measuring CSI-RS for CSI acquisition, hence delay-compensated CSI-RS by NW should be transmitted in this case. But, we admit that for another case (case 2) where PDP values are all in within CP but some of them exceeds the resolvable delay that PMI subband size offers, UE can perform PMI calculation assuming delay pre-compensation using the UE-reported delay offset. In such a case, NW doesn’t need to perform pre-compensation for CSI-RS transmission, hence the excessive CSI-RS overhead can be avoided, and thus a sufficient UPT gain is expected (as some companies provided). One potential issue would be that whether case 1 or case 2 is only known to the UE since delay offset reporting does not contain full PDP information. Thus, gNB could not fully understand whether the UE can perform delay-compensated PMI calculation or not based on the reported delay offset. In our view, this potential issue could be resolved by allowing a joint triggering to report CJT CSI and CJTC-Dd so that UE can indicate such case 1 by TRP selection via small d value. Hence, we support a joint-triggering scheme, rather than the scheme that NW indicates delay offset value to be compensated for CJT CSI reporting (since when NW configures it for case 1, the CSI calculation wouldn’t be correct). Proposal 3.C.2 Support. Similar to the case of delay offset, it is also expected to be beneficial. On issue 3.4 After further thought, we are fine with joint Dd+wideband PO reporting. It can achieve the same functionality that option 1 of SB PO offers. For the other two join reports, we couldn’t find any useful use case on them. Question 3.E.2 We still have the same view as we provided previously. Question 3.F Optimization 1: not supported. We fail to see the need of this optimization, given that 1) UE combining gain will be marginal while CSI-RS overhead will increase 2x, 2) PO calibration is cell-specific and can be done by one of high-SNR-regime Ues, which provides sufficiently good-quality of PO reporting. Optimization 2: not supported. 1-port CSI-RS can support both schemes (non-precoded/precoded schemes) Question 3.G.1 Optimization 1: not supported. (it saves only 1-3bits, which isn’t an issue especially on PUSCH reporting) Optimization 2. Not supported. Dn has important information whether its delay spread exceeds CP or not, which ensures UE to prevent from ISI. But if dn is changed to indicate dynamic range by x2 or by x1, the information will be lost and the reported information does not guarantee to prevent from ISI. Question 3.I.1/3.I.2 We haven’t changed our view. |
| OPPO | Proposal 3.A.1: We think option 2 is much better than option 1 for down selection. If majority companies are fine with the proposal, we could live with it (but we are not proponent company). Conclusion 3.B.1: Fine. Proposal 3.B.2: Scheme 1 is preferred. Scheme 2 needs additional CSI overhead but no additional gain compared to scheme 1 can be obtained. Scheme 3 may lead to some loss if the best antenna port cannot be selected. Proposal 3.C.1/2: In our understanding, UE-specific delay/frequency offset pre-compensation on PDSCH can be performed based on the Rel-18 eType-II CJT CSI without assumption on pre-compensation (e.g. gNB compensation on the precoding matrix) and without compensation on CSI-RS. It seems enhancement on CSI feedback is not needed. Question 3.E.2: We are fine with no additional candidate. Question 3.F: For Optimization.1, the benefit is unclear to us. For Optimization 2, we don’t think it is needed considering CSI-RS resource set for CSI can be precoded CSI-RS (e.g. 1 port) or non-precoded CSI-RS. Question 3.G.1: For Optimization.1, we support it to make the reporting of dn optional to save the overhead. For Optimization 2, we don’t think it is needed. Question 3.I.1: Both optimizations are not needed without clear benefit. Question 3.I.2: No needed. |
| Vivo | 3.C.1 We support this proposal and linkage between DO report and CJT PMI report. For a certain CJT PMI reporting, as limited by the subband configuration in frequency domain, if inter-TRP DO (including propagation delay) is beyond the granularity of the subband configuration, performance loss is expected as UE cannot cancel the phase shift that cannot be quantized by subband granularity. If UE knows the inter-TRP DO by linking to a CJTC DO reporting, such performance loss can be avoided. Hence, to have UE specific DO compensation for CJT CSI reporting is beneficial from system-level performance perspective. We simulate the following cases in our tDoc. - Case 1: no delay offset compensation during CJT CSI calculation. If DO>=CP is modelled, the performance is close to this if TRP selection based on DO is performed as large-DO TRPs are filtered. - Case 2: UE common delay offset compensation in CSI calculation. gNB uses a common delay offset based on Ues’ CJTC reporting on CSI-RS. - Case 3: UE specific delay offset compensation when performing eType II CJT reporting. - Case 4: UE specific CSI-RS DO pre-compensation performed by gNB for CJT PMI reporting while considering UE specific CSI-RS overhead. It can be seen that compared with Case 1, Case 2 basically provides very small gain, and Case 3 can provide attractive gain. In addition, UE specific CSI-RS pre-compensation has the worst system-level performance given the high CSI-RS overhead. This set of results can prove UE specific digital-domain DO compensation provides gain, and it is beneficial to have linkage between CJTC report and CJT PMI report. Without such linkage, UE and gNB can have misaligned compensation on DO, i.e., the reported precoder is not aligned with the channel after DO pre-compensation. Further, the above results doesn’t include the case that DO>CP. If DO>CP exists, we agree TRP selection provides benefit. In such case, the performance without TRP selection will be the worst for sure, and the performance with only TRP selection (i.e., without compensation) is similar as Case 1 performance. On the detailed mechanism, to configure a linked CJTC DO reporting in the CJT PMI report to enable UE’s processing/assumption of DO compensation is a must and can be a first step, which is the main bullet of 3.C.1. We can further discuss how to make sure UE and gNB’s understanding on the detailed DO value to be compensated is aligned. There could be multiple options including, gNB’s explicit signaling to indicate the DO value, joint trigger of the linked CJTC DO reporting and CJT PMI reporting, using the latest DO report occasion before CJT PMI reporting, etc. Last, we agree that if this linkage is supported, it should be a separate UE capability. 3.C.2 Similar as 3.C.1, as the final FO can include both frequency error and Doppler component, where the later is UE specific. We support this given the benefit UE specific compensation provides. 3.D.1 We don’t have an issue with the conclusion including the first sub-bullet. |
| Qualcomm | Proposal 3.A.1: Lengthy discussion already 😊 – hope you haven’t been bored (in work, as well as in life). Sorry for bringing in some new inputs and appreciate your time. For subband phase reporting, we did some simple calculation (@30kHz SCS) for max unambiguous TAE (w/o 2π ambiguity) that opt2 can support (assuming all subbands reported, thus we always have two adjacent subbands) Subband size (#RBs); Max unambiguous TAE (w/o 2π ambiguity). 4; 1/360kHz/4/2: ±347 nsec. 8; 1/360kHz/8/2: ±174 nsec. 16; 1/360kHz/16/2: ±97 nsec. *Note: Since it is inter-TRP, it can be ±. Which is to say, a subset of subbands harms the performance by reducing the max unambiguous TAE. For example, when 16-RB subband has freq-interval e.g. 2 subbands (32-RB), the max unambiguous TAE is only ±43nsec – cannot even satisfy ±65nsec EVM assumption. The above is the reason why we observe UPT loss with a subset of subbands. Therefore, Proposal 1: For subband phase opt2, the subbands definition/config fully reuse legacy. Anyway, if NW don’t think full set of subbands are needed, it still has the freedom of bitmap config – but standard does not need to set such a constraint. - Some rough calculation of UCI overhead: For 4-TRP (3 Pos), 19 subbands x 15 bits = 285, far less than so many Type-II codebooks we’ve done. Based on our observation during last RAN1#117, companies’ concern on supporting both opt1 and opt2 is due to “UE complexity” – thanks for the consideration since we are also one of the UE vendors. Actually we don’t insist opt1, but we can reuse the already-agreed delay quantization (since either opt1 or delay quantization shows the best UPT for FD-precoded CSI-RS, over all cases across both FD-precoded and FD-non-precoded CSI-RS) – sorry to re-paste here for a convenient look If we have to down-select to one option, we have difficulty to accept another one than the best UPT with relatively much smaller UCI overhead (than the one of full subbands with massive overhead). However, we don’t think reusing delay quantization is supporting both – this is not reverting RAN1#117 agreement. Therefore, Proposal 2: Support reusing delay quantization for CMR of PO report. To summarize, we propose editorial: Proposal 3.A.1: For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), regarding the support of sub-band reporting (>1): - A sub-band size is selected from {[4],8,16} PRBs - The sub-band size is NW-configured via higher-layer (RRC) signalling - Denoting the number of reported sub-band phase-offset values as NSB-P, and the sub-bands are indexed as {0, 1, …, NSB-P –1}, support reporting, in one CSI reporting instance, {(n,, n,, n,NSB-P), n=0, 1, …, NTRP – 1, n≠nref} - The alphabet for n, follows the previously agreed alphabet for =1, including the ‘invalid’ state - Support reusing delay quantization for joint report of initial phase + D - The alphabet for initial phase follows the previously agreed alphabet for =1, including the ‘invalid’ state - FFS (by RAN1#118): - Whether the UE performs measurement over the entire configured CSI reporting band WCSI - The maximum NSB-P value, [e.g. 4 or 8], or full reuse legacy subband definition (i.e. table 5.2.1.4-2 in 214) and config. Besides, some more input for your interest below. Appendix for Round1 input: During this pre-RAN1#118 meeting Round0 offline discussion, there are arguments that channel frequency selectivity would impact the phase linearity of Scheme-A (FD-precoded CSI-RS). Some analyses based on 2 CDL-C channel with different delay offsets (-152nsec, -652nsec, -1.15usec, -2.15usec) are provided below. It can be observed that at a same SNR (30dB), the inter-TRP delay (i.e. the effective delay spread) does not impact the inter-TRP phase linearity in frequency-domain. The following figure shows that, for Scheme-A (FD-precoded CSI-RS), the linearity of inter-TRP phase is indeed impacted by SNR – for low SNR like 0dB, not only phase is non-linear anymore due to noise, but also more delay components appear in the PDP spectrum due to the noise. But it should be noted that anyway SNR is already taken into consideration for performance comparison between different Schemes (FD-precoded v.s. not) and phase options (opt1 initial phase + TAE/delay v.s. opt2 subband phases), as shown previously in Figure above. Proposal 3.B.2: While we are fine with this proposal in general, we understand companies are generally OK with selection of one UE antenna port out of multiple UE antenna ports (details FFS scheme1,2,3). Then seems a little weird to only allow select one from x, not the total y - At least for Scheme1 (NW config), NW need to config both the SRS resource (out of Q>1) and the SRS port out of x – which basically means configuring one out of y Therefore, editorial suggestion: … - For the selection of PSRS=1 SRS port corresponding to the ‘reference UE antenna port’ (out of x y port(s)), decide, by RAN1#118, from the following schemes: - Scheme1: NW-configured via higher-layer (RRC) signalling - Scheme2: UE-selected, the selection included in the phase offset report - Scheme3: Fixed rule, e.g. the first out of x y port(s) …. Proposal 3.C.2: Please change our position from “support” to “not support” Regarding impact to Type-II-CJT CSI calculation, FO is quite minor, compared to delay offset (3.C.1). Proposal 3.E.1: While we think higher FO resolution is beautiful, we doubt whether UE can measure such a high-resolution FO just with 14-symbol gap within a TRS, under a practical SNR. (Proponent companies please do some calculations.) While we also want to propose longer symbol-gap for higher-resolution FO measurement (something similar as what Rel-18 TDCP did), we understand this is not likely to happen within Rel-19. We can be supportive of such large MFO value (256) based on UE capability. Question 3.G.1: We support one aspect of “Optimization 2”: Increase the resolution of Dn,offset by 2x. Current quantization level MD={32,64}, thus this means we propose to increase one value: MD={128} – the purpose is for TAE quantization: 0.5CP/127=9.23nsec, which shows performance improvement in our evaluation. (BTW, we are also OK with the other aspect of this of “Optimization 2”: Increase the range of Dn,offset by 2x – as long as it is based on UE capability) Question 3.I.1: Optimization 1: No AP-TRS; |
| Nokia | Proposal 3.A.1 Ok Conclusion 3.B.1 Ok Proposal 3.B.2 Ok Proposal 3.C.1 Support Proposal 3.C.2 We don’t think UE-side compensation of FO is needed for CSI calculation Proposal 3.E.1 Ok Question 3.F We support opt. 2 because from NW perspective, reusing the existing TRS sets for all calibration measurements, including PO, avoids needing a dedicated CSI-RS resource for PO measurement Question 3.G.1 We don’t see the need for these proposed optimisations |
| Intel | Proposal 3.A.1: OK, we are also okay to not have sub-band reporting. Conclusion 3.B.1: Support Proposal 3.B.2: We think x=1 is sufficient, we can support this proposal with minimum spec. impact which is scheme-3. Proposal 3.C.1/3.C.2: Not sure this is practically useful – how is the pre-compensation indicated – dynamically ? A better work-around is to use pre-compensated CSI-RS (which could be for a group of Ues) Conclusion 3.D.1: Support Proposal 3.E.1: OK Question 3.E.2: Prefer time unit (CP) for DO and frequency unit (ppm) for FO – ok with additional 0.75CP, 1.5CP, no big preference on FO Question 3.F: we do not see strong need for further optimization of the CSI-RS resource set configuration. Question 3.G.1: no strong need for further optimization. Question 3.I.1: aperiodic TRS resource set can be supported. No different RE locations for delay/freq. offset measurement. Question 3.I.2: no different RE locations for phase offset measurement. |
| Mod V18 | Revised proposals 3.A.1, 3.B.1 per Qualcomm’s inputs Added conclusions 3.E.2, 3.G.1, 3.I.1, 3.I.2 (inevitable outcome) |
| Apple | Proposal 3.A.1: We are fine Conclusion 3.B.1: In principle we are okay. But we are little confused with the Note. xTyR antenna switching can be configured with different SRS resource sets with different time domain behavior (periodic, semi-persistent and aperiodic). For the same time domain behavior (aperiodic), it can be configured with multiple SRS resource sets if there is not enough UL symbols in a slot. Not sure why this limitation is needed. [Mod: The note applies only when cjtc-P (PO report) is configured and only for this purpose. Not for general usage of SRS AS for DL CSI acquisition] Proposal 3.B.2: We are fine. Scheme1 is slightly preferred Proposal 3.C.1: We are open to discuss, but it is pre-mature to agree that such a linkage is needed Proposal 3.C.2: We are open to discuss, but it is pre-mature to agree that such a linkage is needed Conclusion 3.D.1: We are fine Proposal 3.E.1: We are fine Conclusion 3.E.2: We are fine Question 3.F: We support to consider TRS (Optimization 2) since TRS is anyway needed for connected UE are fine Conclusion 3.G.1: We are fine Conclusion 3.I.1: We are fine Conclusion 3.I.2: We are fine |
| Ericsson | Proposal 3.A.1 We don’t think we can simply agree legacy sub-band definition without deciding on the maximum value of NSB-P. We showed some results for PO for different subband sizes (1 RB, 2RB, and 8 RB) for different timing error values of 130 ns and 300 ns. As you can see in figure below the PO can wrap around and this can cause ambiguity depending on which subbands and how many subbands are reported. Consider the example which shows the phase as a function of subband index for a subband size of 8 RBs. Assume the Pos, for subbands 1, 4, 7, and 10 are reported by the UE. If the gNB tries to perform linear interpolation to determine timing error, then it may determine a wrong value of timing error. The correct timing error would be determined if the UE reports Pos for subbands 2, 3, 4, 5. If the UE reports Pos for all the subbands, I guess gNB can figure out when the wrap arounds happen. But then we have to consider the overhead as well. So, we don’t agree with the current formulation to agree legacy subband sizes. Our suggestion is to further study the subband size and the maximum value of NSB-P together jointly. [Mod: I understand and sympathize with your argument. It is FFS now.] |
| Xiaomi | Proposal 3.A.1 Our first preference is no sub-band report since the performance will be degraded with the limited number of reported sub-band for Opt 2. If majority companies support this proposal, we can accept it for progress. Conclusion 3.B.1 Support Proposal 3.B.2 Support and prefer scheme 1 for the selection of 1 SRS port. Proposal 3.C.1/3.C.2 we are fine to perform PMI calculation assuming pre-compensation by UE. But as for association between CJTC report and CSI report, we are not clear about the benefit of jointly triggered. What is the problem to use the latest CJTC report? In addition, if combination of delay offset and frequency offset is configured, does it mean CJTC delay offset, CJTC frequency offset and CSI report will be jointly triggered? [Mod: It was argued (e.g. ZTE, Samsung) that joint triggering avoids error propagation and complex priority rules inherent in linked reports performed across >1 slots. I added your preference – also proposed by Huawei, for discussion] Conclusion 3.D.1 Support Conclusion 3.E.2 Support Question 3.F It is better to explain the benefit clearly. Conclusion 3.G.1 Support Conclusion 3.I.1/3.I.2 Support |
| Sony | Proposal 3.A.1 Our preference is still Opt 1 since we believe Opt 2 requires a lot of overhead to outperform Opt 1. We therefore do not support this proposal. Proposal 3.B.2 Support. Proposal 3.C.1/3.C.2 We are okay to discuss these proposals. Issue 3.4 Support of Dd+wideband PO reporting might be needed if sub-band PO reporting is not agreed. Perhaps this item could be discussed separately from the other two. [Mod: Correct. We will try to conclude on 3.A.1 first] Proposal 3.E.1 Okay Question 3.F We do not see a clear benefit of these optimizations. Conclusion 3.I.2 We are fine. |
| Ericsson2 | Regarding AFO, the currently agreed values of {0.1ppm, 0.2ppm} were motivated by the frequency errors specified in RAN4 specifications. In practice, some gNBs may support smaller frequency errors than what is specified in RAN4 specifications. Hence, in our view an additional value of AFO=0.05ppm should be supported which will result in reduced quantization error. |
| Huawei, HiSilicon | Proposal 3.C.1: We support the linkage of DO and CJT CSI reporting, since the delay offset difference between TRPs may result in severe frequency selectivity with the subband, and this will degrade the difference as CJT CSI cannot handle this. Linkage between DO and CJT CSI reporting can reduce the frequency selectivity. On the DO used for pre-compensation, we think a candidate can be added as below: - The delay offset value to be compensated is the latest reported DO before the DCI triggering the CJT CSI reporting. [Mod: Added] Proposal 3.C.2: The phase change due to frequency offset will be accumulated with time after UE reporting. And gNB needs to re-compensate the FO when scheduling the PDSCH. Hence there is no need to link the frequency offset reporting to the CJT CSI reporting. Proposal 3.E.1: fine with it. Question 3.E.2: We believe some new values for AFO is needed. The ranges of 0.1ppm and 0.2ppm agreed last meeting may be very small, just 400Hz and 800Hz for 4GHz carrier, which is enough compared to potential FO difference between TRPs. In fact for a 15kHz SCS, the range of FO that UE can measure is about 3kHz. So we support { 18∆t, 116∆t, 132∆t} for AFO. [Mod: I understand, sadly companies cannot find middle ground on this issue. So conclusion of no consensus is inevitable] Question 3.F: We don’t see the need for the optimizations. Conclusion 3.G.1: fine with the conclusion. Question 3.H: don’t see the need for additional constraints. Conclusion 3.I.1/2: fine with the conclusion. |
| Mod V27 | Proposal 3.A.1: Sub-band size is now FFS per opposing preference from Ericsson vs Qualcomm Proposal 3.C.1: Added one more possibility per Huawei’s comment Added Conclusion 3.F given the clear situation |
| CATT | Proposal 3.A.1: support We are fine to consider candidate subband size of {4,8,16}RBs from legacy CSI sub-band configuration as starting point. Noticing that the time alignment error is bounded by 65ns according to RAN4 spec, the inter-TRP TAE should be around ±130ns at most. As is calculated by Qualcomm, subband size of 4,8,16 RBs for 30kHz SCS can accommodate maximum ±347/±174/±97ns TAE without introducing wrap around phase ambiguity between adjacent subband. Finer granularity of 1RB and 2RBs per subband as mentioned by Ericsson can be considered for small BWP too. Regarding @Ericsson’s illustration, we want to mention that since NW configures the subband for PO reporting, phase wrap around ambiguity should be anticipated by NW for choosing wisely the reporting band. One possible way for NW to resolve the phase wrap around ambiguity is to configure a portion of adjacent subbands for PO reporting. Finally, if PO reporting subbands are wisely configured by NW, the 2pi wrap around ambiguity can also be resolved during the linear regression with implementation specific algorithms. This is because the “true” subband phases can be denoted as φi+2πki, where each ki has limited candidate integer values due to bounded maximal TAE. Linear regression based on the “true” subband phases can be solved with exact wrap up phases 2πki recovered at the NW side. Proposal 3.E.1: OK |
| Mod V29 | Added proposal 3.G.2 (after further Tdoc review) |
| ZTE | 3.A.1: Our original preference is to support both Opt-1 and Opt-2. If we have to down select one option, our preference is Opt-1. Although Opt-1 requires FD-precoded CSI-RS, it requires less reporting overhead and provides better performance (according to the results provided by QC). 3.B.1: Support the conclusion. 3.B.2: Support the proposal, and support only scheme 1 (NW-side SRS port selection) for SRS port selection. Based on previous agreement and conclusion 3.B.1, Q = 1 SRS resource with x SRS ports is configured for PO measurement. So, NW already selects x SRS ports from y SRS ports. If scheme 2 is supported, the final SRS port for PO measurement is actually first selected by NW and then further selected by UE. However, the basic assumption of PO measurement is UL/DL channel reciprocity, and NW can acquire all the channel information. Then UE-side SRS port selection is totally unneeded. Besides, we believe that SRS port selection based on the channel information is beneficial for PO measurement. Therefore, fixed rule of SRS port selection should NOT be supported. 3.C.1/3.C.2: We are fine with both proposals to start the discussion, and we agree with vivo’s analysis that both proposals are essential to enable UE-specific DO/FO pre-compensation. Besides, we believe that at least one of the two additional points is needed. - Firstly, NW explicitly configuring the DO/FO can guarantee the same understanding of DO/FO at both NW and UE sides, and enable NW adopting DO/FO that are different from those reported by UE in CJT pre-compensation. For instance, the DO/FO reported by one specific UE can be NOT reliable enough. Then NW can trigger DO/FO reporting for multiple Ues located in a same area, and use the averaged results to perform CJT pre-compensation. - Secondly, simultaneously triggering both DO/FO reporting and CJT CSI reporting can avoid introducing complicated timeline restriction between the DO/FO reporting and CJT CSI reporting. Besides, it also avoids the case where DO/FO reporting is dropped while CJT CSI reporting is NOT dropped, so that NW and UE have different understanding of DO/FO used for pre-compensation. [Mod: Thanks for the good analysis – I concur with your concern on error propagation and complex timeline without the above] 3.D.1: We believe at least Joint Dd and PO reporting should be supported. 3.E.1: We are open to support MFO = 256 in addition. 3.E.2: Since the maximum measurable value and resolution of DO/FO is determined by 1∆f/1∆t, the dynamic ranges in the formula of 1x∆f/1x∆t should be also supported. [Mod: This was already discussed for 3 meetings and the situation hasn’t changed] - F: Optimization 1 is NOT needed. J = 1 is sufficient for PO measurement. Optimization 2 can be further considered, since TRS is actually 1-port CSI-RS. [Mod: This was already discussed for 3 meetings and the situation hasn’t changed] 3.G.1: Fine. 3.G.2: Support. - H: We did NOT see the necessity of introducing any constraints. 3.I.1/3.I.2: We think different RE locations (FDM) can be considered to avoid the estimation error caused by channel variation over time, especially for PO measurement. [Mod: This was already discussed for 3 meetings and the situation hasn’t changed] |
| Ericsson3 | Regarding Proposal 3.G.2, so far, TRS is agreed as the measurement resource for both DO and FO reporting. Given TRS is a NZP-CSI-RS resource set with ‘trs-info’ on, each TRP is represented by a ‘CSI-RS resource set ID’ instead of a ‘CSI-RS resource ID’. Suggest to make the following change: Proposal 3.G.2 (Rev): For the Rel-19 aperiodic standalone CJT calibration reporting, the UCI mapping order is as follows: - When ReportQuantity is ‘cjtc-Dd’ (Doffset+d): - nref, - {Dn,offset, n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID, - {dn, n=0, 1, …, N TRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID - When ReportQuantity is ‘cjtc-F’ (FO): - nref, - {FOn, n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID, - When ReportQuantity is ‘cjtc-Dd-F’ (joint Doffset+d and FO): - nref1, - nref2, - {Dn,offset, n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID, - {dn, n=0, 1, …, N TRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID - {FOn, n=0, 1, …, NTRP – 1, n≠nref} ordered from the lowest to highest CSI-RS resource set ID, [Mod: Thanks for the good catch] |
| Fujitsu | 3.C.1/3.C.2: Support in principle. Re the first bullet for NW indication, we want to clarify this indication. For AP CSI reports, DCI will trigger one and more AP CSI reports, but for each CSI report configuration will be configured in RRC. So there are two ways to indicate compensation DO/FO values. If this compensation DO/FO values are in RRC, the values will be outdated due to RRC latency; And if this compensation DO/FO values are in DCI, DCI overhead will be increase, especially DO and FO are jointly reported. Hence, we slight prefer the second bullet and the third bullet. 3.E.2: Support the conclusion 3.G.1: Support the conclusion. 3.I.1&3.I.2: Support the conclusion. |
| Google | Proposal 3.A.1: We think a more important aspect is to clarify whether UE needs to perform DO/FO compensation for the PO calculation. It seems the intention for subband PO is to avoid such operation. Therefore, we suggest adding a bullet as follows. Otherwise, the benefit for the subband PO is unclear. - Note: UE is not required to perform DO/FO compensation for subband PO calculation. Conclusion 3.B.1: OK with the conclusion. Regarding the note, our understanding is that it does not preclude configuring >1 SRS resource sets, but it just precludes the PO report to be associated with >1 SRS resource sets. Please correct me if the understanding is incorrect. Proposal 3.B.2: OK in principle. But we think a better way for PSRS selection is dynamic selection. Since only SP/P PO report is supported, the selection can be indicated by MAC CE or DCI triggering the PO report. Proposal 3.C.1: We think this should only be applicable when the CMRs do not share common QCL source for average delay. Otherwise, the whole proposals become unnecessary. We suggest adding a bullet as follows: - The above only applies when the CMRs do not share common QCL source for average delay indication. Proposal 3.C.2: Similar to previous proposal, we think this should only be applicable when the CMRs do not share common QCL source for Doppler shift. Otherwise, the whole proposals become unnecessary. We suggest adding a bullet as follows: - The above only applies when the CMRs do not share common QCL source for Doppler shift indication. [Mod: OK] Conclusion 3.D.1: We also support joint Dd+PO reporting. Proposal 3.E.1: OK. Conclusion 3.E.2: OK. Conclusion 3.F: We are open to optimization 1. Conclusion 3.G.1: OK Proposal 3.G.2: OK Question 3.H: We think we can start from the following proposals. Current timeline for DO/FO/PO report does not count the processing delay for time/frequency offset calculation and compensation. Therefore, the DO/FO/PO report should not require the UE to perform certain compensation. For the TRSs configured for DO report, support the NW configure the TCI states for the TRSs based on the following constraints: - (With pre-compensation of FO): All the TRSs are configured with a common TCI state for QCL-TypeC indication without average delay, and each TRS is configured with a separate TCI state for QCL-TypeC indication without Doppler shift For the TRSs configured for FO report, support the NW configure the TCI states for the TRSs based on the following constraints: - (With pre-compensation of DO): All the TRSs are configured with a common TCI state for QCL-TypeC indication without Doppler shift, and each TRS is configured with a separate TCI state for QCL-TypeC indication without average delay For the single-port CSI-RS resources configured for PO report, support the NW configure the TCI states for the CSI-RS resources based on the following constraints: - (With pre-compensation of DO and FO): All the CSI-RS resources are configured with a common TCI state for QCL-TypeC indication [Mod: Noted for later rounds] Conclusion 3.I.1: OK Conclusion 3.I.2: OK |
| OPPO1 | Proposal 3.C.1/3.C.2 As in our previous comment, we are FINE with UE specific compensation on PDSCH for delay and frequency offset. However, whether Rel-18 eType II CJT CSI needs to be enhanced needs further study. Current evaluation result only focuses on whether UE specific compensation is performed or not. But even with legacy Rel-18 CJT CSI (with compensation) and Rel-19 DO/FO reporting, gNB is possible to perform UE specific compensation on PDSCH via compensated precoding. Now we are open for further study. [Mod: Proposals are to decide i.e. discuss further 😊 so your point will be discussed] |
| Sharp | Conclusion 3.B.1: Support. Proposal 3.C.1/Proposal 3.C.2: In our understanding, UE-specific DO/FO pre-compensation on PDSCH can be performed by reporting CJT PMI. The UE can calculate a PMI with the UE-side pre-compensation on the DO on the digital domain. However, we think the FO is compensated at the TRP side to adjust the frequency. Proposal 3.E.1: Fine. Conclusion 3.F: Support the conclusion. Proposal 3.G.2: Support Ericsson’s suggestion. |
| Qualcomm | Proposal 3.A.1 Thanks @Ericsson and @CATT for the good discussion. We agree with the analyses of @Ericsson, regarding phase ramp over larger subband size. (BTW, nice figure for illustration). Actually this is also the reason why we think directly reporting TAE is better than subband phase - For TAE, as long as the quantization range (e.g. 0.5CP or CP, which is about 128Δf or 114Δf) is large enough, the un-wrapped frequency unit size is like 1 or 2 RBs (about 124Δf or 112Δf); - However, existing smallest subband size is 4 RBs Then the question is, what would be the maximum TAE we are targeting to tackle? - For TAE report (Opt1-like), reusing existing quantization from delay can handle as large as e.g. 0.5CP=1.17 usec @30kHz SCS; - For subband phase (Opt2), 1 or 2-RB new subband size needs to be defined if larger TAE needs to be handled. Besides, we think this FFS bullet on UE measure eighbour should be carefully differed between FD-precoded (MRT-precoded) CSI-RS, and FD-non-precoded (non-MRT-precoded) CSI-RS: FFS (by RAN1#118): - Whether the UE performs measurement over the entire configured CSI reporting band WCSI due to that: MRT-precoded CSI-RS; Non-MRT-precoded CSI-RS. Subband phase can be aided by measuring eighbour subbands (interpolation); Subband phase is measure on the subband itself. |
| MediaTek | Proposal 3.C.1, 3.C.2 Since the proposal is for deciding to support the linkage, is it correct understanding that there will be a follow up proposal on supporting the linkage along with further details? [Mod: Yes] The reason to ask this question is that we are OK to have this linkage as a separate UE capability, independent of the Rel-19 CJT-c and Rel-18 Type II CJT features. And this capability can be included as part of the later proposal. Further, since many companies now recognize that Rel-19 Type I MP codebook can be potentially used in CJT scenarios, we think that the linkage can support Rel-19 Type I MP CSI as well. [Mod: Noted] |
| Ericsson | Proposal 3.A.1 Regarding CATT’s comment ‘Noticing that the time alignment error is bounded by 65ns according to RAN4 spec, the inter-TRP TAE should be around ±130ns at most.’, we checked this with our RAN4 colleagues. This statement is not true in general. We understand that the TAE of 65ns mentioned in TS 38.104 is for single node MIMO (e.g., intra-cell collocated MIMO mTRP) where antennas typically be co-located (i.e., at the same site roof top) [please refer to a previous LS RP-180557]. We also understand that the 65 ns is not for non-collocated intra-cell or inter-cell mTRP MIMO. In Rel-18, for UE supporting RTD> CP, as part mTRP MIMO, RAN4 has defined MRTD value as a total budget and not a specific TAE value. In fact, the TAE for mTRP MIMO can be higher than 3µs as long as MRTD budget is maintained at UE. We need to take this into account when discussing subband sizes, etc. in the next round of discussion. |
| Mod V40 | Added Alt1B to proposal 3.B.2 per Google’s input Minor revisions to address companies’ inputs |
| Lenovo/ MotM | Conclusion 3.B.1: OK Proposal 3.B.2: Agree. Not clear why any alternative other than Alt1/3 are needed [Mod: Next round, to be discussed] Proposal 3.C.1/2: We are OK with the linkage in general, however, in our understanding, the periodicity of reporting phase/freq offsets for synchronization would be larger than that needed for reporting PMI via CJT CB, hence, the same FO/PO report can be utilized for multiple CSI reports based on CJT CB. Reporting both report types on the same slot is too restrictive. Open to discuss during meeting Conclusion 3.D.1: Fine Proposal 3.E.1: Support Conclusion 3.E.2: OK Proposal 3.G.2: Support Conclusion 3.I.1/2: OK |
| Mod V45 | No revision |
| CMCC | Proposal 3.A.1 Support. Regarding the FFS: - For the supported sub-band size(s), we prefer to follow the legacy CSI sub-band definition. - We support that the UE performs measurement over the entire configured CSI reporting band WCSI. - No need for a mechanism to limit CSI reporting overhead considering the CJT calibration reporting is on PUSCH Conclusion 3.B.1: Support. Proposal 3.B.2: Support. For the selection of PSRS=1 SRS port corresponding to the ‘reference UE antenna port’ (out of the available port(s)), we support Scheme3: Fixed rule, e.g. the first out of the available port(s). For this scheme, the definition of the “first” should be further clarified, e.g., it can be considered from the aspects of time, port ID, or SRS resource ID, etc. We have some concerns on Scheme 1, e.g., the measurement of phase offset cannot be finished if the port configured by NW is dropped (consider the port dropping mechanism in Rel-18). Besides, regarding the supported time-domain behaviour(s) for the associated SRS resource, we support all the three types, i.e., periodic, semi-persistent, aperiodic. Proposal 3.C.1: Support. Conclusion 3.D.1: Support. Proposal 3.E.1: Support. Conclusion 3.F: Support. Conclusion 3.G.1: Support. Question 3.H: From our point of view, additional constraints are NOT needed for the TCI state of the ‘DL-RS’. Conclusion 3.I.1: Support. Conclusion 3.I.2: Support. |
| KDDI | Proposal 3.E.1: Support. Conclusion 3.G.1: fine. We do not think these optimizations are necessarily mandatory. Conclusion 3.I.1/2: fine. |
| TCL | Proposal 3.C.1 Support Proposal 3.C.2 Support Proposal 3.E.1 Support MFO = 256 depends on the UE’s capability. Proposal 3.G.2 Support |
| Mod V51 | Proposal 3.A.1: added “Separate UE capability” per inputs from some UE vendors |

# References

| 1 | RP-240087 | Revised WID: NR MIMO Phase 5 | Samsung (Moderator) |
| --- | --- | --- | --- |
| 2 | R1-2406643 | Moderator Summary for OFFLINE discussion on Rel-19 CSI enhancements | Moderator (Samsung) |
| 3 | R1-2405871 | On 128 CSI-RS ports and UE reporting enhancement | Huawei, HiSilicon |
| 4 | R1-2405876 | On Rel-19 Enhancements of CSI | InterDigital, Inc. |
| 5 | R1-2405888 | CSI enhancements to support up to 128 CSI-RS ports | MediaTek Inc. |
| 6 | R1-2405904 | Discussion on CSI enhancements | Spreadtrum Communications |
| 7 | R1-2405936 | CSI enhancements | Tejas Networks Limited |
| 8 | R1-2405943 | CSI enhancements for Rel. 19 MIMO | Fraunhofer IIS, Fraunhofer HHI |
| 9 | R1-2405955 | CSI Enhancement for NR MIMO | Google |
| 10 | R1-2405981 | Discussion on CSI enhancements | CMCC |
| 11 | R1-2406024 | CSI enhancements for MIMO | Intel Corporation |
| 12 | R1-2406029 | Discussion on CSI enhancements | ZTE Corporation, Sanechips |
| 13 | R1-2406178 | Discussion on Rel-19 CSI enhancements | vivo |
| 14 | R1-2406261 | CSI enhancements for Rel-19 MIMO | OPPO |
| 15 | R1-2406280 | Views on CSI enhancement | Xiaomi |
| 16 | R1-2406311 | Discussion on Rel-19 CSI enhancements | Fujitsu |
| 17 | R1-2406364 | Views on Rel-19 MIMO CSI enhancements | CATT |
| 18 | R1-2406431 | CSI enhancements | TCL |
| 19 | R1-2406468 | More views on CSI enhancements | Sony |
| 20 | R1-2406522 | Discussion on CSI enhancements | Lenovo |
| 21 | R1-2406548 | Discussion on CSI enhancements | NEC |
| 22 | R1-2406577 | Discussion on CSI enhancements | HONOR |
| 23 | R1-2407262 | Views on Rel-19 CSI enhancements | Samsung |
| 24 | R1-2406723 | Discussion on Rel-19 CSI enhancements | ETRI |
| 25 | R1-2406746 | CSI enhancement for NR MIMO Phase 5 | Nokia |
| 26 | R1-2406832 | Views on R19 MIMO CSI enhancement | Apple |
| 27 | R1-2406907 | CSI enhancements for large antenna arrays and CJT | Ericsson |
| 28 | R1-2406926 | Discussion on CSI enhancements | NTT DOCOMO, INC., NTT CORPORATION |
| 29 | R1-2407003 | CSI enhancements | Sharp |
| 30 | R1-2407184 | CSI enhancements for >32 ports and UE-assisted CJT | Qualcomm Incorporated |
| 31 | R1-2407074 | CSI Enhancements | CEWiT |
| 32 | R1-2407126 | Discussion on CSI enhancements for NR MIMO Phase 5 | KDDI Corporation |
| 33 | R1-2407144 | CSI enhancements | NICT |