3GPP TSG RAN WG1 Meeting #118	R1-2406774

Maastricht, NL, August 19th – 23rd, 2024

Source:	MediaTek Inc.

Title:	Frame structure and timing aspects

Agenda item:	9.4.2.2

Document for:	Discussion and decision

# Introduction

In this contribution, frame structure and timing aspects of Ambient IOT designs will be addressed, covering the following sections:

- Section 2: Energy Harvesting Impact and Resulting Device States

- Section 3: Timing Behavior and Synchronization for a Single Device

- Section 4: TDMA D2R Transmission

- Section 5: Summary

# Energy Harvesting Impact and Resulting Device States

In RAN1#107 and RAN#104 meetings, the following are captured [1][2]. In this section, the energy harvesting impact and the resulting device states are to be investigated.

Conclusion (RAN1#117) RAN1 discussion related to the potential impact of device unavailability due to charging by energy harvesting will occur in agenda item 9.4.2.2.

(RAN#104) Note: there was involved discussion regarding device energy harvesting in Section 3.4 of RP-241688, although no conclusions were reached.

Radio Frequency (RF) energy harvesting has emerged as a promising technique for ultra-low-power devices. This ultra-low power consumption of Device 1 opens up possibilities for devices that can operate solely on harvested RF energy, eliminating the need for traditional battery solutions:

Observation 1: Device 1 has low active power (~1uW), and it is possible to rely merely on RF energy harvesting (EH).

However, not all devices can operate within such stringent power constraints:

Observation 2: Device 2a/2b has 100 times higher active power (~100 uW), and it is necessary to exploit more energy source and assume much higher energy harvesting capability than device 1.

The significant difference in power requirements between Device 1 and Device 2a/2b underscores the need for diverse energy harvesting strategies. While RF energy harvesting may suffice for Device 1, Device 2a/2b requires additional energy sources to meet its higher power demands. The varying energy harvesting capabilities of these devices also impact their behavior during inactive periods:

Observation 3: Different EH capability should be considered in describing the device OFF state characteristics.

To address these differences and provide a comprehensive framework for device operation, we propose the introduction of distinct operational states:

Proposal 1: For describing EH impact, two device states, ON and OFF, are introduced for device 1 and device 2a/2b.

For Device 1, which operates at ultra-low power levels, we propose the following state definitions:

Proposal 2: For device 1:

- ON state: The available energy is enough for supporting transmission and reception. Device can maintain a clock and perform RF EH

- OFF state: The available energy is not enough for supporting transmission and reception. Device can perform RF EH only

- Transfer from OFF to ON: based on the available energy level

- FFS whether the available energy of device 1 can always be assumed enough during inventory round(s)

These states reflect the energy-constrained nature of Device 1, where the OFF state allows for energy accumulation through RF harvesting, preparing the device for active operation in the ON state.

Recent advancements in low-power oscillator technology have implications for devices with higher energy harvesting capabilities:

Observation 4: In [3][4], there show the feasibility of <1 uW oscillator that can remaining running during OFF state of device 2 with higher EH capability.

Leveraging this technology, we propose the following state definitions for Device 2:

Proposal 3: For device 2:

- ON state: The available energy is enough for supporting transmission and reception. Device can maintain a clock and perform EH (from more than RF)

- OFF state: The available energy is not enough for supporting transmission and reception. Device can maintain a clock and perform EH (from more than RF)

  - Note: EH of Device 2 is large enough during OFF state to maintain a clock

- Transfer from OFF to ON: based on device clock or available energy level

  - Note: The clock can be utilized for controlling OFF state duration for device 2

The ability to maintain a clock during the OFF state provides Device 2 with enhanced control over its power management and operational timing.

To effectively manage these diverse device capabilities within the NR ambient IoT ecosystem, we propose the development of specialized inventory mechanisms:

Proposal 4: RAN1 to develop inventory mechanisms utilizing the following two capabilities of OFF states:

- Capability 1: Device OFF to ON switch is based on energy level harvested

- Capability 2: Device OFF to ON switch is based on device clock counting

Proposal 5: Mechanism for indicating Capability 2 is introduced, while Capability 1 is default device capability reader assumes for a device before Capability 2 indication is received.

These inventory mechanisms will enable more efficient network management and resource allocation, taking into account the unique energy harvesting and power management characteristics of different NR ambient IoT devices.

# Timing Behavior and Synchronization for a Single Device

The R2D link in NR ambient IoT systems demonstrates remarkable robustness in data reception, even under challenging conditions. This is primarily due to the implementation of Manchester coding and the device's ability to track timing along with the processing of Manchester on-off chips.

Observation 6: For R2D link, device can track the PRDCH timing along the processing of Manchester on-off chips. PRDCH data reception can be performed even with SFO of 10000 PPM, as illustrated in the below figure:

Figure 3-1: R2D data reception robustness by timing tracking based on Manchester coding

This observation highlights the resilience of the R2D link against significant Sampling Frequency Offset (SFO). The Manchester coding scheme allows devices to maintain synchronization even with considerable timing discrepancies. Based on this robustness, we can propose a relaxed SFO requirement:

Proposal 6: From data reception performance perspective, residue SFO requirement after device receiving R2D preamble can be set to 10000 PPM.

Determining the end of PRDCH transmission is crucial for efficient communication. RAN1 has agreed to study two options:

Agreement in R1#116b - To determine or derive the end of PRDCH transmission, study at least following options: - Option 1: R2D postamble immediately follows the PRDCH to indicate the end of the PRDCH. - Option 2: Based on R2D control information

Given the device's ability to track timing effectively, we can simplify the design:

Proposal 7: Given that device can track timing well along demodulating Manchester on-off chips, there is no need of additional postamble for R2D, and Option-2 (Based on R2D control information) is adopted to determine or derive the end of PRDCH transmission.

The D2R link presents unique challenges and opportunities for timing synchronization and SFO compensation. The A-IoT reader can leverage the known time offset between D2R and R2D preambles to enhance synchronization:

Observation 7: For A-IOT reader, comparing the estimated time offset between D2R preamble and R2D preamble with the known time offset can be used to derive and compensate SFO.

Observation 8: Given X-us time error of D2R preamble detection by the reader and Y-us time offset between R2D and D2R preambles, D2R residue SFO can be derived as X/Y * 10^6 PPM.

The impact of residue SFO on D2R data reception performance varies with the transmission block size (TBS):

Observation 9: To confine D2R data reception performance loss, smaller residue SFO is required for longer TBS transmission, as can be checked in the below figure [5] (500 PPM for 96 bits, 100 PPM for 400 bits, and 50 PPM for 1000 bits):

Figure 3-2: D2R performance w.r.t. different TBS and residue SFO values

To address these varying requirements, we propose a flexible approach:

Proposal 8: RAN1 design allows flexible indication of the time offset between R2D preamble and D2R preamble to adapt the residue SFO w.r.t. targeted transmission time (related to selected TBS and data rate/MCS).

To ensure precise timing, we propose setting a target for D2R preamble design:

Proposal 9: To avoid excess time offset between R2D preamble and D2R preamble, RAN1 targets D2R preamble design with 90%-tiled estimated time error no larger than X us, X = [1].

Furthermore, we suggest supporting multiple time offset indications:

Proposal 10: At least support indicating time offset between R2D preamble and D2R preamble as Y*X us, where Y = [2000], [10000], [20000] (and X = [1] as in the previous proposal). FFS explicitly or implicitly.

Regarding the time interval between R2D and D2R transmissions, the 3GPP agreement outlines two options for study:

Agreement Study the following options for the time interval between a R2D transmission and the corresponding D2R transmission following it: - Option 1: Define a maximum time TR2D_max between a R2D transmission and the corresponding D2R transmission following it, so that the device transmits D2R transmission within [TR2D_min, TR2D_max]. - FFS: maximum time is common or different for different A-IoT devices - FFS: maximum time for different traffic types/command types (e.g. DT or DO-DTT) and/or different use case (e.g., Inventory or Command) - Option 2: The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D TR2D_min - FFS the maximum value(s) for TR2D

Based on our analysis, we propose:

Proposal 11: To support maximum TBS no smaller than 96 bits, Option 2 is supported, i.e., The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D  TR2D_min and TR2D_min = [2000*X us] where X is defined as in Proposal 9. FFS the maximum value(s) for TR2D.

# TDMA D2R Transmission

Agreement in RAN1#116 For A-IoT contention-based access procedure, at least slotted-ALOHA based access is studied.

TDMA for multiple D2R transmissions is a basic scheme used in RFID, e.g., slotted ALOHA, where each device maintains a slot counter and will decrease the counter by 1 after receiving a command, e.g., QueryRep, and if slot = 0 after counter decrementing, a D2R transmission can be started. This contention-based slotted ALOHA manner is useful for avoiding the collision among devices, especially considering the large initial SFO of the device, and has already been agreed upon before.

Observation 10: The contention-based slotted-ALOHA access for multiple D2R transmissions is supported, which requires frequent R2D signalling for slot counter decrement in the device.

A frequent R2D signalling in contention-based slotted-ALOHA access is not an efficient manner and extra latency is introduced. Therefore, multiple TDMA D2R transmissions triggered/scheduled by one R2D signalling should be supported. In that sense, a coarse synchronization among devices during a specific period is required, which we think is possible after the device performs synchronization per the received preamble from R2D signalling. Meanwhile, it is inevitable some residual SFO/timing errors still exist even after synchronization. In this case, a gap between two adjacent slots for D2R transmissions can be used to alleviate the potential collision among devices when they access the channel in a TDMA manner. The following figure is shown to illustrate the idea.

Figure 4-1: Illustration of TDMA D2R transmissions

Observation 11: Multiple TDMA D2R transmissions scheduled by one R2D signaling is a more efficient manner regarding spectrum utilization and latency.

Observation 12: A small gap in the time domain between two adjacent D2R transmission slots is useful for avoiding the collision among devices when accessing the channel in a TDMA manner considering the residual timing error.

Proposal 12: For TDMA D2R transmissions, when multiple devices are scheduled by one R2D signalling, study the mechanism to solve the impact of residual SFO/timing error, e.g., a small gap in the time domain between two adjacent D2R transmission slots.

# Summary

In this contribution, we have the following observations and proposals:

Observation 1: Device 1 has low active power (~1uW), and it is possible to rely merely on RF energy harvesting (EH).

Observation 2: Device 2a/2b has 100 times higher active power (~100 uW), and it is necessary to exploit more energy source and assume much higher energy harvesting capability than device 1.

Observation 3: Different EH capability should be considered in describing the device OFF state characteristics.

Proposal 1: For describing EH impact, two device states, ON and OFF, are introduced for device 1 and device 2a/2b.

Proposal 2: For device 1:

- ON state: The available energy is enough for supporting transmission and reception. Device can maintain a clock and perform RF EH

- OFF state: The available energy is not enough for supporting transmission and reception. Device can perform RF EH only

- Transfer from OFF to ON: based on the available energy level

- FFS whether the available energy of device 1 can always be assumed enough during inventory round(s)

Observation 4: In [3][4], there show the feasibility of <1 uW oscillator that can remaining running during OFF state of device 2 with higher EH capability.

Proposal 3: For device 2:

- ON state: The available energy is enough for supporting transmission and reception. Device can maintain a clock and perform EH (from more than RF)

- OFF state: The available energy is not enough for supporting transmission and reception. Device can maintain a clock and perform EH (from more than RF)

  - Note: EH of Device 2 is large enough during OFF state to maintain a clock

- Transfer from OFF to ON: based on device clock or available energy level

  - Note: The clock can be utilized for controlling OFF state duration for device 2

Proposal 4: RAN1 to develop inventory mechanisms utilizing the following two capabilities of OFF states:

- Capability 1: Device OFF to ON switch is based on energy level harvested

- Capability 2: Device OFF to ON switch is based on device clock counting

Proposal 5: Mechanism for indicating Capability 2 is introduced, while Capability 1 is default device capability reader assumes for a device before Capability 2 indication is received.

Observation 6: For R2D link, device can track the PRDCH timing along the processing of Manchester on-off chips. PRDCH data reception can be performed even with SFO of 10000 PPM, as illustrated in the below figure:

Proposal 6: From data reception performance perspective, residue SFO requirement after device receiving R2D preamble can be set to 10000 PPM.

Proposal 7: Given that device can track timing well along demodulating Manchester on-off chips, there is no need of additional postamble for R2D, and Option-2 (Based on R2D control information) is adopted to determine or derive the end of PRDCH transmission.

Observation 7: For A-IOT reader, comparing the estimated time offset between D2R preamble and R2D preamble with the known time offset can be used to derive and compensate SFO.

Observation 8: Given X-us time error of D2R preamble detection by the reader and Y-us time offset between R2D and D2R preambles, D2R residue SFO can be derived as X/Y * 10^6 PPM.

Observation 9: To confine D2R data reception performance loss, smaller residue SFO is required for longer TBS transmission, as can be checked in the below figure [5] (500 PPM for 96 bits, 100 PPM for 400 bits, and 50 PPM for 1000 bits):

Proposal 8: RAN1 design allows flexible indication of the time offset between R2D preamble and D2R preamble to adapt the residue SFO w.r.t. targeted transmission time (related to selected TBS and data rate/MCS).

Proposal 9: To avoid excess time offset between R2D preamble and D2R preamble, RAN1 targets D2R preamble design with 90%-tiled estimated time error no larger than X us, X = [1].

Proposal 10: At least support indicating time offset between R2D preamble and D2R preamble as Y*X us, where Y = [2000], [10000], [20000] (and X = [1] as in the previous proposal). FFS explicitly or implicitly.

Proposal 11: To support maximum TBS no smaller than 96 bits, Option 2 is supported, i.e., The corresponding D2R transmission timing TR2D following a R2D transmission is determined based on the control information in the R2D transmission, where TR2D  TR2D_min and TR2D_min = [2000*X us] where X is defined as in Proposal 9. FFS the maximum value(s) for TR2D.

Observation 10: The contention-based slotted-ALOHA access for multiple D2R transmissions is supported, which requires frequent R2D signalling for slot counter decrement in the device.

Observation 11: Multiple TDMA D2R transmissions scheduled by one R2D signaling is a more efficient manner regarding spectrum utilization and latency.

Observation 12: A small gap in the time domain between two adjacent D2R transmission slots is useful for avoiding the collision among devices when accessing the channel in a TDMA manner considering the residual timing error.

Proposal 12: For TDMA D2R transmissions, when multiple devices are scheduled by one R2D signalling, study the mechanism to solve the impact of residual SFO/timing error, e.g., a small gap in the time domain between two adjacent D2R transmission slots.

# References

- “Draft Report of 3GPP TSG RAN WG1 #117 v0.3.0,” MCC Support; on-line available: https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_117/Report

- RP-241688, “Moderator's summary for offline discussion about Ambient IoT”, Moderator (RAN1 Vice-Chair), RAN #104 meeting, June 2024

- Jiang H, Wang P H P, Mercier P P, et al. A 0.4-V 0.93-nW/kHz relaxation oscillator exploiting comparator temperature-dependent delay to achieve 94-ppm/° C stability[J]. IEEE Journal of Solid-State Circuits, 2018, 53(10): 3004-3011.

- Tokairin T, Nose K, Takeda K, et al. A 280nW, 100kHz, 1-cycle start-up time, on-chip CMOS relaxation oscillator employing a feedforward period control scheme[C]//2012 Symposium on VLSI Circuits (VLSIC). IEEE, 2012: 16-17.

- R1-2406775, “On downlink and uplink channel/signal aspects for A-IoT”, MediaTek Inc., RAN1 #118 meeting, August 2024

- R1-2405381, “FL summary #3 on frame structure and timing aspects for Rel-19 Ambient IoT”, Moderator (vivo), RAN1 #117 meeting, May 2024

- 3GPP TR38.848, “Study on Ambient IoT (Internet of Things) in RAN (Release 18),” V18.0.0, September 2023