3GPP TSG RAN WG1 #120             	                      R1-2500132

Athens, Greece, February 17th – 21st, 2025

Source: 	ZTE Corporation, Sanechips

Title:	Discussion on LP-WUS operation in IDLE/INACTIVE mode

Agenda item: 	9.6.2

Document for:	Discussion and decision

Introduction

In RAN #105 meeting, a updated WID on low-power Wake-up Signal and Receiver for NR was revised in [1]. In RAN1 #119 meeting, the following agreements were made [2].

Agreement Confirm the following working assumption with the addition of the note at the bottom: Working Assumption From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - FFS the serving cell measurement metrics - The entry/exit thresholds can be configured separately for different types of LR - It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB. - Note: This may be revisited based on the RAN2/RAN4 discussion. Note: this does not intend to impact any agreements and working assumptions made in RAN1/RAN2 after the working assumption. Agreement At least support the case that the number of beams for LP-WUS/LP-SS is the same as the number of SSB beams. Conclusion Do not support one-to-multiple mapping between the LP-WUS/LP-SS beams and the SSB beams. Conclusion No additional RAN1 specification impact specific to support multiple-to-one mapping between the LP-WUS/LP-SS beams and the SSB beams. R1-2409815 Summary #2 on LP-WUS operation in IDLE/INACTIVE mode Moderator (Apple) Agreement For the offset value(s) between an LO and a reference PO/PF, down-select between - Option 1-1 - Configurability between Option 1-1 and Option 1-2 - Option 2B-1 Agreement At least for the 1:1 LO to PO mapping, - The maximum value of M for Option A or Option B with G=1 (if either of them is supported) is 4. Conclusion There is no RAN1 consensus to support additional codepoints indicating wake-up at least for 1:1 LO to PO mapping. R1-2409816 Summary #3 on LP-WUS operation in IDLE/INACTIVE mode Moderator (Apple) For future meetings: Consider the following alternatives for UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[80ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [80ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - Alt 2: For the wake-up delay capability report, UE reports a pair of values for main radio ramp up and the number of SSBs needed for synchronization (D, NSSB). Support the following 3 candidate capabilities: {[(20ms, 3), (400ms, 5), (800ms, 5)]}. - Alt 3: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The number of SSBs needed for synchronization is defined in specs, and different number of SSBs can be defined for different UE capability on the main radio ramp up time. - 3 candidate values for the main radio ramp up time: {[20ms, 400ms, 800ms]} - The number of SSBs needed for synchronization is defined as [3, 5, 5] for [20ms, 400ms, 800ms] main radio ramp up time, respectively. - Alt 4: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The time needed for synchronization will be defined by RAN4 requirements later. Agreement Each LP-WUS or LP-SS is QCLed with an SSB with [select one from ‘typeA’, ‘typeC’], and when applicable, ‘typeD’ with the same SSB. R1-2410861 Summary #4 on LP-WUS operation in IDLE/INACTIVE mode Moderator (Apple) Working Assumption If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration. Conclusion From RAN1 perspective, there is no consensus on the following proposal: UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if the UE has not received LP-WUS for a long time. R1-2410862 Summary #5 on LP-WUS operation in IDLE/INACTIVE mode Moderator (Apple) For future meetings: Companies are encouraged to consider section 3.5 of R1-2410862 for further discussions in future meetings.

In this contribution, open issues on LP-WUS operation in IDLE/INACTIVE mode are discussed.

RRM measurement

In RAN1 117 meeting, we have the following WA

Working Assumption From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. - Above is applicable for both time-domain processing or frequency-domain processing - Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

During the discussion and FL summary, some contributions think the current definition for RSRP and RSRP means we need to continue to use SMTC for OFDM WUR according to TS 38.215,

| Definition | SS reference signal received power (SS-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry secondary synchronization signals. The measurement time resource(s) for SS-RSRP are confined within SS/PBCH Block Measurement Time Configuration (SMTC) window duration. If SS-RSRP is used for L1-RSRP as configured by reporting configurations as defined in TS 38.214 [6], the measurement time resources(s) restriction by SMTC window duration is not applicable. For SS-RSRP determination demodulation reference signals for physical broadcast channel (PBCH) and, if indicated by higher layers, CSI reference signals in addition to secondary synchronization signals may be used. SS-RSRP using demodulation reference signal for PBCH or CSI reference signal shall be measured by linear averaging over the power contributions of the resource elements that carry corresponding reference signals taking into account power scaling for the reference signals as defined in TS 38.213 [5]. If SS-RSRP is not used for L1-RSRP, the additional use of CSI reference signals for SS-RSRP determination is not applicable. SS-RSRP shall be measured only among the reference signals corresponding to SS/PBCH blocks with the same SS/PBCH block index and the same physical-layer cell identity. If SS-RSRP is not used for L1-RSRP and higher-layers indicate certain SS/PBCH blocks for performing SS-RSRP measurements, then SS-RSRP is measured only from the indicated set of SS/PBCH block(s). For frequency range 1, the reference point for the SS-RSRP shall be the antenna connector of the UE. For frequency range 2, SS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported SS-RSRP value shall not be lower than the corresponding SS-RSRP of any of the individual receiver branches. |
| --- | --- |
| Applicable for | If SS-RSRP is used for L1-RSRP, RRC_CONNECTED intra-frequency. Otherwise, RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_INACTIVE intra-frequency, RRC_INACTIVE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

However, the RSRP definition just refer to the following sentence

“SS reference signal received power (SS-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry secondary synchronization signals. ”

The reasons include:

- LP-RSRP and LP-RSRQ are defined as the same way. The above assumption only refer to the RSRP and RSRP definition which is only based on linear average.

- If all the paragraphs are referred, obviously, SSB related description is not applied for OFDM WUR.

Observation 1: Using the same definition of SS-RSRP and SS-RSRQ for OFDM WUR only means the following is reused

- SS reference signal received power (SS-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry secondary synchronization signals.

- Secondary synchronization signal reference signal received quality (SS-RSRQ) is defined as the ratio of N×SS-RSRP / NR carrier RSSI, where N is the number of resource blocks in the NR carrier RSSI measurement bandwidth.

Regarding SMTC introduction for OFDM WUR, we do not think it is necessary. In the WID, it is stated that the serving cell measurement can be offloaded to WUR, instead of neighbor cell measurement. Moreover, it would be infeasible for OFDM WUR to do the neighbor cell measurement due to the coverage limitation.

- Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

Observation 2: OFDM WUR only perform serving cell measurement and neighbor cell measurement by WUR is not considered in Rel-19.

Additionally, the measurement based on OFDM WUR or OOK WUR could be applied for different scenarios.  As shown in the following, for WUR, it is needed to determine which scenario is supported.

RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_INACTIVE intra-frequency, RRC_INACTIVE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency

Currently RRC_IDLE intra-frequency and RRC_INACTIVE intra-frequency can be supported. If separate band is supported, then RRC_IDLE inter-frequency, and RRC_INACTIVE inter-frequency is not considered in Rel-19 according to the RAN plenary conclusion.

Proposal 1:

- For OFDM WUR,

- LP-SSS-RSRP is defined as the linear average over the power contributions (in [W]) of the resource elements that carry secondary synchronization signals.

- LP-SSS-RSRQ is defined as the ratio of N×SS-RSRP / NR carrier RSSI

- SMTC is not needed for OFDM WUR and OOK WUR for serving cell measurement

- Serving cell measurement for OFDM-based WUR and OOK-based WUR can be applied for RRC_IDLE intra-frequency and RRC_INACTIVE intra-frequency

LP-WUS monitoring schemes

LO and PO offset

We firstly clarify the following alternatives, and then further analyze based on the conclusion of this issue.

Agreement For the offset value(s) between an LO and a reference PO/PF, consider the following options: - Definition: The gap between an LO and a PO is considered to be no less than the wake-up delay a UE supports if the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PO is no less than the wake-up delay. - Option 1: gNB configures a single offset value. - If the gap between an LO and the PO associated with the offset is no less than the wake-up delay a UE supports, the UE monitors the PO associated with the offset after receiving a wake-up indication in a LP-WUS. - Otherwise, - Option 1-1: the UE follows the legacy paging monitoring procedure. - Option 1-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - Option 2: gNB configures one or multiple offset values. - For the same PO, each offset corresponds to a LO. - This does not preclude the possibility that the same LO may correspond to different POs with different offset values. - Option 2A: A UE does not expect that the gap between the LO associated with the largest offset and the corresponding PO is less than the wake-up delay the UE supports. The UE monitors the LO associated with one offset that has a gap between the LO and the corresponding PO no less than the wake-up delay. - This implies that the gNB needs to configure at least one offset value that is no less than the largest wake-up delay supported by the UEs. - FFS exactly how to choose the offset - Option 2B: - If the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay a UE supports, the UE monitors the LO associated with one offset that has a gap between the LO and the PO associated with the offset no less than the wake-up delay. - FFS exactly how to choose the offset - Otherwise, - Option 2B-1: the UE follows the legacy paging monitoring procedure. - Option 2B-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - FFS exactly how to choose the offset - FFS the UE shall monitor the LO associated with additional offset(s) Note: The PO mentioned above refers to legacy PO configured for the UE.

Agreement For the offset value(s) between an LO and a reference PO/PF, down-select between - Option 1-1 - Configurability between Option 1-1 and Option 1-2 - Option 2B-1

Based on option1, to cover all the wake-up delays UE supports, the gNB generally would configure a large offset, e.g., 800ms. Without wake-up delay report, option 1 also works. In this case, actually, option1 invalidate the wake-up delay report.

If the gNB configure a small offset value, e.g.,40ms, based on option 1-1, the UE is not allowed to go to UDS and is required to monitor PO. In this case, the power saving gain would degrade since the UE may not have more time stay in UDS. Based on option 1-2, the UE PSG is not impacted, but the there is impact on the legacy paging implementation mechanism. More specifically, if the gNB page a UE who support LP-WUS, the gNB may need to send paging trice, in associated PO and next PO, since the gNB does not know whether this UE is monitoring LP-WUS or not. If the gNB mandate the UE who support LP-WUS in any condition to monitor the next PO and would not transmit the paging in first PO, then the UE who has the LP-WUS capability but not in LP-WUS mode would be impacted and also the gNB paging mechanism is changed.

Based on option 2, different offsets can address the different latency requirements and PSG requirements. For example, for the UE reporting 20ms as wake-up delay, the gNB can configure offset as 20ms to make sure that the UE has the smallest latency and largest PSG. For option 2B-1, the NW may configure all the offsets less than a UE’s wake-up delay, in this case, the UE would monitor legacy paging behavior. That’s to say, the gNB deactivate LP-WUS monitoring for some of the UEs via the offset configuration, which can be viewed as another exit condition. And further clarification on UE measurement is needed, e.g., whether the UE should return to MR measurement. Obviously, if only one offset is applied in option 2B-1, it is the same as option 1-1.

Observation 3:

- Option 2 can address the latency requirements and PSG requirements from different UEs.

- For option 2B-1, the gNB can deactivate LP-WUS monitoring for some of the UEs via the offset configuration

Proposal 2: Option 2 with multiple offsets is prioritized.

More specifically, for option 2B-1, regarding the FFS part, there are two alternatives

Alt1: The UE monitors the LO associated with the minimum offset that has a gap between the LO and the corresponding PO no less than the wake-up delay.

Alt2: The UE monitors the LO associated with the minimum offset and larger offset that has a gap between the LO and the corresponding PO no less than the wake-up delay.

Obviously, alt1 is the baseline and alt2 is kind of optimization which can provide more NW scheduling flexibility.

Figure 1. UE procedure based on option 2

Proposal 3: Further clarify the offset selection based on following

- Alt1: The UE monitors the LO associated with the minimum offset that has a gap between the LO and the corresponding PO no less than the wake-up delay.

- Alt2: The UE monitors the LO associated with the minimum offset and larger offset that has a gap between the LO and the corresponding PO no less than the wake-up delay.

Repetition and beams determination

A potential issue would be how to exactly determine the beam index and repetition. there are mainly two methods for the beam index and repetition determination if they are supported simultaneously.

Option1:beam index increase first and then repetition

Option2:repetition first and then beam index increase

Figure 2. beam sweeping and repetition

For option2, it is more appropriate for WUR to detect the LP-WUS. And if failed, discard the memory and then start the next detection with another beam. For option1, it requires the WUR to restore all the data for different beams until the transmission ends, which is not friendly to the WUS receiver.

Proposal 4: Consider option 2 to determine the beam index and repetition order.

LO and PO mapping

LO and PO mapping

In last meeting, the working assumption indicates to preclude the option3.

Working Assumption - If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration.

Regarding multiple PO associated to one LO, assuming only one offset is configured, three interpretations should be further confirmed for this case.

- Interpretation 1: its own multiple POs across multiple paging cycles, and multiple POs is associated to one LO

- Interpretation 2: multiple POs are associated to one LO, and all the UEs corresponding to the multiple POs monitor the same LO, wherein in one paging cycle, one UE may monitor multiple LOs due to multiple POs.

- Interpretation 3: multiple POs in its PF are associated to one LO, and all the UEs corresponding to the multiple POs monitor the same LO, wherein in one paging cycle, one UE only monitor one LO.

Figure 3. Interpretations for UEs corresponding to different POs

Based on interpretation1, if the UE does not receive the paging message in the first PO, automatically, the UE would monitor the second PO. Therefore, there is not much motivation to associate more POs.

- Based on interpretation2, the offset could be defined between LO and PO. If 4 POs is associated with one LO, the UE would have 4 LOs to monitor. In this case, actually the gNB does not need to transmit the LP-WUS in the first 3 LOs since anyway the UE is required to monitor its own PO. Then there is no need to further consider interpretation 2.

Based on interpretation3, the offset could be defined between LO and PF, wherein the PF contains multiple POs. In this case, the UE corresponding to the POs in the PF only need to monitor one LO.

Proposal 5: Clarify that if multiple POs associate to one LO,

- from the UE perspective, one LO associates to one PO in a cycle

- from the system perspective, one LO associates all the POs in a cycle

The main benefit for considering multiple POs associating to one LO is that the system overhead could be saved if the NW need to wake up all the UEs, which is anlyzed in subclause 4.2.

Observation 4: The main benefit for multiple POs associating to one LO is to save the system overhead when NW need to wake up all the UEs

Multiple POs for cell specific wake-up

From NW perspective, cell specific wake-up is quite beneficial for the system overhead. For example, if the gNB update the system information, e.g., LP-WUS configuration update, without cell specific wake-up, the system overhead would be quite large. Based on LP-WUS indication, the following three cases could be considered for cell specific wake-up.

Case1: LP-WUS can indicate all subgroups per PO

If LP-WUS is used for cell specific wake-up, and if one LO is associated with one PO, then to wake-up all the UEs in one -IDRX cycle, assuming N POs are configured in total in one I-DRX cycle, N additional LP-WUS also should be at least transmitted.

Case2: LP-WUS can indicate all subgroups for M POs

If LP-WUS is used for cell specific wake-up, and if one LO is associated with M POs, assuming N POs are configured in total in one I-DRX cycle, then at least N/M additional LP-WUS is needed to achieve cell specific wake-up during one I-DRX cycle.

Case3: LP-WUS can indicate all subgroups for all POs

If LP-WUS is used for cell specific wake-up, and if one LO is associated with all the POs in one I-DRX cycle, then at least one LP-WUS is needed. However, it would burden the LP-WUS design, since more information is needed to indicate that this LO is targeting all the POs, not one of them.

Above overhead calculation assumes the LP-WUS can be used to wake-up all the subgroups for one or more POs. The overhead comparison among above three cases could be

N*X1 : N/M*X2 : 1*X3

Wherein the X1 is the occupied OFDM symbols/resources for 6bit LP-WUS payload

X2 is the occupied OFDM symbols for 8bits LP-WUS payload, including 2bits for PO indication assuming M=4, 2^2=4.

X3 is the occupied OFDM symbols for 12bits LP-WUS payload, including 6bits for PO indication, 2^6=64

Currently, it is not sure whether CRC bits is supported, and to simplify this, it is assumed X1=6, X2=8, X3=12.

Then the overhead ratio would be 6N: 2N: 12. With this assumption, supporting more POs associated with one LO would bring system overhead reduction. And cell specific wake-up also bring less system overhead.

Assuming that the LP-WUS occupies 2slots to achieve msg3 coverage,

Table 4. Assumptions for system overhead calculation

| LP-WUS | 2slots 11RB Payload size: 6bit |
| --- | --- |
| SCS | 30KHz |
| System bandwidth | 20MHz |
| Number of POs in 1280ms | 64, 8 |
| Number of beams | 8 |

Based on case 1, the system overhead occupied by LP-WUS is around 1.25% for 8POs and 10% for 64POs. Then to achieve the system overhead less than 3%, 4POs would be needed.

Observation 5: In case of cell specific wake-up via LP-WUS

- If LP-WUS can indicate all subgroups per PO to wake-up, the system overhead occupied only by LP-WUS could be up to 10%

- If LP-WUS can indicate all subgroup for 4POs to wake-up, the system overhead occupied by LP-WUS could be less than 3%.

Additionally, we also can consider to use LP-SS for cell specific wake-up. In this case, both the OOK based WUR and OFDM based WUR need to monitor the LP-SS to make sure that the cell specific wake-up is not missed. Based on LP-SS cell specific wake-up, no additional LP-WUS is needed. And also, LP-SS has better coverage than LP-WUS.

Observation 6: Using LP-SS for cell specific wake-up would not cause additional system overhead, and has better coverage.

Proposal 6: Discuss and decide using LP-SS or LP-WUS to achieve cell specific wake-up.

- Slightly prefer to use LP-SS for cell specific wake-up

- If LP-WUS is used to achieve cell specific wake-up, at least, 4POs would be needed to be associated to LO and LP-WUS design and coverage would be further impacted.

Wake-up delay capability

Consider the following alternatives for UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[80ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [80ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - Alt 2: For the wake-up delay capability report, UE reports a pair of values for main radio ramp up and the number of SSBs needed for synchronization (D, NSSB). Support the following 3 candidate capabilities: {[(20ms, 3), (400ms, 5), (800ms, 5)]}. - Alt 3: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The number of SSBs needed for synchronization is defined in specs, and different number of SSBs can be defined for different UE capability on the main radio ramp up time. - 3 candidate values for the main radio ramp up time: {[20ms, 400ms, 800ms]} - The number of SSBs needed for synchronization is defined as [3, 5, 5] for [20ms, 400ms, 800ms] main radio ramp up time, respectively. - Alt 4: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The time needed for synchronization will be defined by RAN4 requirements later.

From the UE perspective, different SSB periodicity would bring different sync time. moreover, different UE implementation requires different sync time. For example, for OFDM WUR and OOK WUR, the required sync time would be different due to the different timing error estimation performance. Therefore, the required sync time based on alt 1,2,3 would be highly depends on the UE implementation and SSB periodicity.

Based on alt 1, it is a simple way to define the wake-up delay and there is no need to touch the number of SSBs for sync, since it may cause the confusion that other SSB periodicity may have different assumption.

based on alt2, reporting number of SSBs would make UE behavior complicated when we consider the UE is not expected to monitor LP-WUS during the wake-up delay. That’s to say, if the number of SSBs during the gap brought by the offset is less than the UE capability, the UE may also not monitor the corresponding PO.

Based on alt3, similar as alt2, number of SSB is defined and the UE behavior would be complicated. based on Alt4, the UE behavior is clear in RAN1.

Proposal 7: Support Alt4 for wake-up delay report and secnd an LS to RAN4.

Interaction with eDRX

According to the TR, IoT use cases and wearable cases are the target use cases. For these IoT uses cases, the eDRX is an important feature for UE power saving.

The following characteristics for target use cases are considered in the study item: - IoT cases including e.g., industrial wireless sensors, controllers, actuators and etc, including the following characteristics, - primary for small form devices - power-sensitive - static, nomadic or limited mobility - Wearable cases including e.g., smart watches, rings, eHealth related devices, and medical monitoring devices etc., - primary for small form devices, - power-sensitive - low/medium speed, FFS: high speed - eMBB cases including e.g., XR/smart glasses, smart phones and etc., - devices form is various and not restricted - power-sensitive - low/medium speed, FFS: high speed

Without RRM measurement relaxing for LP-WUS, the PSG by LP-WUS has no advantage over the eDRX. Therefore, considering the premature WUR deployment and the WUR coverage, we think the eDRX feature is still important to be supported especially for IoT devices.

During the SI stage, there two schemes for eDRX case. 1) LP-WUS monitoring within PTW and legacy PO determination. 2) LP-WUS monitoring is not restricted in PTW and dynamic PO determination. Based on the RRM measurement relaxing, LP-WUS still can bring some power saving gain compared with eDRX.

Observations: 1) LP-WUS monitoring within PTW and legacy PO determination - Effective per UE paging arrival rate <=1% - Results in [8A-3], [8A-4], [8A-8], [8A-12], [8A-16] show that compared with eDRX, LP-WUS provide mean power saving gain (average: 67.5%, range: 28%~92%) - Results in [8A-4], [8A-8] show that compared with eDRX mean paging latency (16.697~25.92) second, LP-WUS will result in mean paging latency (19.863~31.24) second, which increases mean paging latency (19%~21%) - Results in [8A-3], [8A-16] show that compared with eDRX, LP-WUS will result in mean paging latency (42.679~161.73) second without baseline eDRX latency. - 1%<Effective per UE paging arrival rate <=8% - Results in [8A-8] show that compared with eDRX, LP-WUS provides mean power saving gain (75%). - Results in [8A-8] show that compared with eDRX mean paging latency (16.927) second, LP-WUS will result in mean paging latency (16.953) second, which increases mean paging latency (0.2%)

For eDRX, there would be multiple IDRX cycles in a PTW. If the LP-WUS is located before each IDRX, then the LP-WUS monitoring would be frequent as shown in Figure 5, which is not so power efficient.

Figure 4. Duty cycle LP-WUS monitoring periodicity aligned with I-DRX

If LP-WUS is configured with a large periodicity, e.g., same as eDRX periodicity, then the WUR is not needed to frequently monitor as shown in above figure. In this case, we need to further consider how LO is configured to align with the PTW.

Figure 4. Duty cycle LP-WUS monitoring periodicity aligned with PTW

Based on the figure, if LO monitoring occasions are overlap with the paging PDCCH monitoring occasions, it seems not necessary to monitor the LP-WUS in the overlapped occasions.

Proposal 8: For eDRX, discuss

- the LO periodicity could be the same as IDRX

- the LO periodicity could be the same as eDRX periodicity

Given that RAN1 has no consensus to support dynamic PO, dynamic PTW also is not considered in this release. Then eDRX support should be discussed to align the current IDRX design.

TDD, QCL and power control

FDD/TDD support

In the WID, it is not clear whether the WUR could be deployed in FDD or TDD bands. From the initial study, FDD bands at least are feasible and could be supported. As for TDD bands, since TDD bands are commercially deployed widely, LP-WUS also should be designed to be compatible with TDD DL-UL configuration. An illustration is shown as follows.

Figure 5. LP-WUS in TDD

Proposal 13: Support LP-WUS deployed in FDD/TDD bands

For FDD, the multiple MOs in one LO could be contiguous. However, for TDD, the situation could be different, since the LP-WUS could be overlap with UL slots or symbols. For example, if LP-WUS is configured with 8 beams, it could be challenging for the NW to configure 8 contiguous DL slots in a frame for LP-WUS transmission in TDD bands. Moreover, if repetition is considered, more contiguous slots would be needed. In this case, the LO or MO definition may be impacted which may be based on different DL slots.

For example, for FDD, if LO is a window with N*K monitoring occasions, then the LP-WUS may occupy N*K DL slots. For TDD, we need to discuss whether N*K monitoring occasions only is configured on DL slots, or it could be on the flexible slots.

Proposal 9: The LO and MO definition should be applicable for both FDD and TDD

- FFS whether MO could be based on flexible symbols or slots in TDD

QCL relationship

Agreement Each LP-WUS or LP-SS is QCLed with an SSB with [select one from ‘typeA’, ‘typeC’], and when applicable, ‘typeD’ with the same SSB.

For LP-WUS and LP-SS, the QCL source and QCL type may need to be further discussed. In idle/inactive mode, the available QCL resource could be CD-SSB. As for the QCL type,

-	'typeA': {Doppler shift, Doppler spread, average delay, delay spread}

-	'typeB': {Doppler shift, Doppler spread}

-	'typeC': {Doppler shift, average delay}

-	'typeD': {Spatial Rx parameter}

Type D is mainly used for above 6G scenario, which may not be suitable for LP-SS/LP-WUS in FR1. But in FR2, type D QCL could be considered.

Currently, the LP-SS and LP-WUS could be allocated in the same carrier with SSB, but maybe different frequency location. Also, they have different bandwidth, we do not think same Doppler spread and delay spread could be assumed .

Proposal 10: The QCL source for LP-SS/LP-WUS should be CD-SSB and the QCL type could be type C in FR1 and type D in FR2

Power control

The power control for LP-SS/LP-WUS is mainly for the gNB side. For LP-SS, the power control may be needed for RSRP measurement, threshold determination, or AGC control. e.g., with the passloss by SSB, and the LP-SS power, the WUR can perform the AGC control and have the expected RSRP, which would be helpful for WUR decoding.

Moreover, generally speaking, for all the NR signals, the power should be controled including PDCCH as shown in the following figure in [3]

Figure 6. Power control for NR signals

Therefore, at least for LP-SS, the power control would be needed. And given that the low complexity of WUR, the power control shoul be as simple as much.

Proposal 11: Consider the power offset between LP-SS and SSB

- FFS the power control for LP-WUS

Conclusions

In this contribution, we have discussed issues on LP-WUS operation in IDLE/INACTIVE modes. We make the following observations and proposals:

Observations:

Observation 1: Using the same definition of SS-RSRP and SS-RSRQ for OFDM WUR only means the following is reused

- SS reference signal received power (SS-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry secondary synchronization signals.

- Secondary synchronization signal reference signal received quality (SS-RSRQ) is defined as the ratio of N×SS-RSRP / NR carrier RSSI, where N is the number of resource blocks in the NR carrier RSSI measurement bandwidth.

Observation 2: OFDM WUR only perform serving cell measurement and neighbor cell measurement by WUR is not considered in Rel-19.

Observation 3:

- Option 2 can address the latency requirements and PSG requirements from different UEs.

- For option 2B-1, the gNB can deactivate LP-WUS monitoring for some of the UEs via the offset configuration

Observation 4: The main benefit for multiple POs associating to one LO is to save the system overhead when NW need to wake up all the UEs

Observation 5: In case of cell specific wake-up via LP-WUS

- If LP-WUS can indicate all subgroups per PO to wake-up, the system overhead occupied only by LP-WUS could be up to 10%

- If LP-WUS can indicate all subgroup for 4POs to wake-up, the system overhead occupied by LP-WUS could be less than 3%.

Observation 6: Using LP-SS for cell specific wake-up would not cause additional system overhead, and has better coverage.

Proposals：

Proposal 1:

- For OFDM WUR,

- LP-SSS-RSRP is defined as the linear average over the power contributions (in [W]) of the resource elements that carry secondary synchronization signals.

- LP-SSS-RSRQ is defined as the ratio of N×SS-RSRP / NR carrier RSSI

- SMTC is not needed for OFDM WUR and OOK WUR for serving cell measurement

- Serving cell measurement for OFDM-based WUR and OOK-based WUR can be applied for RRC_IDLE intra-frequency and RRC_INACTIVE intra-frequency

Proposal 2: Option 2 with multiple offsets is prioritized.

Proposal 3: Further clarify the offset selection based on following

- Alt1: The UE monitors the LO associated with the minimum offset that has a gap between the LO and the corresponding PO no less than the wake-up delay.

- Alt2: The UE monitors the LO associated with the minimum offset and larger offset that has a gap between the LO and the corresponding PO no less than the wake-up delay.

Proposal 4: Consider option 2 to determine the beam index and repetition order.

Proposal 5: Clarify that if multiple POs associate to one LO,

- from the UE perspective, one LO associates to one PO in a cycle

- from the system perspective, one LO associates all the POs in a cycle

Proposal 6: Discuss and decide using LP-SS or LP-WUS to achieve cell specific wake-up.

- Slightly prefer to use LP-SS for cell specific wake-up

- If LP-WUS is used to achieve cell specific wake-up, at least, 4POs would be needed to be associated to LO and LP-WUS design and coverage would be further impacted.

Proposal 7: Support Alt4 for wake-up delay report and secnd an LS to RAN4.

Proposal 8: For eDRX, discuss

- the LO periodicity could be the same as IDRX

- the LO periodicity could be the same as eDRX periodicity

Proposal 9: The LO and MO definition should be applicable for both FDD and TDD

- FFS whether MO could be based on flexible symbols or slots in TDD

Proposal 10: The QCL source for LP-SS/LP-WUS should be CD-SSB and the QCL type could be type C in FR1 and type D in FR2

Proposal 11: Consider the power offset between LP-SS and SSB

- FFS the power control for LP-WUS

References

RP-241824, “Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, RAN#105

Draft Report of 3GPP TSG RAN WG1 #118bis

R1-2301599, On NW energy saving techniques in spatial and power domains, MediaTek Inc.