3GPP TSG RAN WG1 #120                                                                                           R1- 2500231

Athens, Greece, February 17th – 21st, 2025

Source:            CATT

Title:                 Design of LP-WUS and LP-SS

Agenda Item:   9.6.1

Document for: Discussion and Decision

Introduction

In RAN1#119, the following agreements were achieved for the design of the LP-WUS and LP-SS design [1]:

Agreement Update the existing agreement as shown below: In case of overlaid OFDM sequence carrying information, support option 2: - Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options for potential down-selection. - Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK ‘ON’ symbols. - Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s) - FFS the how the information bits are carried by the overlaid OFDM sequence(s) Note: the overlaid OFDM sequence in each OOK ‘ON’ symbol can be different according to information bits to be carried by the overlaid OFDM sequence within the LP-WUS. Agreement For the overlaid OFDM sequence in time domain, the sequence is generated based on , - M=1, is given by the largest prime number such that,is the overlaid OFDM sequence length. - The base overlaid sequence is generated by extension of , - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s) ,, - M=2, 4, is down-selected from the following: - Alt1: is given by the largest prime number such that,is the overlaid OFDM sequence length. - The base overlaid sequenceis generated by extension of , - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s) , - Note it doesn’t preclude any pulse shaping scheme if any. - Alt2:is given by the largest prime number such that, is the overlaid OFDM sequence length. - The base overlaid sequenceis generated by inserting zeros before and/or after . The total number of zeros is. For example,, - With CS(s) applied to the base overlaid OFDM sequence if any: denotes the potential cyclic shift (s) For example,, - where, - FFS the value of G, with G>0. - FFS value(s) of root q and/or CS for generating the candidate sequences if applied - FFS on whether changes are needed to handle intra-/inter-cell interference - Note: the overlaid OFDM sequence in time domain is based on potential modification to ZC sequence as listed above. - Above overrides any previous RAN1 agreement / working assumption Agreement At least for M>1, for the overlaid OFDM sequence in time domain, down-select the sequence length LZC from the following: - Alt1:, with - FFS how to map the generated overlaid OFDM sequence to the frequency domain - Alt2: - Note: X is the number of RBs of LP-WUS/LP-SS bandwidth (blanked guard RBs are not included) Agreement CS(s) and/or root(s) used for overlaid OFDM sequence in the time domain are derived from RRC signalling: - FFS: the set of values of CS and root for configuration - FFS: details of the RRC signaling Agreement For the length L of LP-SS binary sequence, limit the selection to the following: - M=1, L= {4, 6, 8} - M=2, L= {8, 12, 16} - M=4, L= {16, 32} Agreement To compare LP-SS binary sequence performance, cross-correlation is deprioritized over sync accuracy, i.e., cross-correlation within a set of 4 sequences is further compared for different sets of sequences with similar sync accuracy. Agreement For the M value for LP-WUS and LP-SS, down-select one alternative from the following in RAN1#120: - Alt1: The M values for LP-WUS and LP-SS are always same. - Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS. - Alt3: The M values for LP-WUS and LP-SS can be configured to be same or different. Agreement Regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell: - support maximum 4 candidates overlaid sequences for M=4 Agreement Regarding whether to support additional sync signal to LP-SS, down-selection from the following: - Option 1A: No additional sync signal, LP-SS periodicity =320ms - Additionally support at least one of [80ms,160ms] - FFS: whether different values can be applicable for different M values - Option 1B: No additional sync signal, LP-SS periodicity =320ms - No additional support of other periodicities - Option 2: With additional sync signal, LP-SS periodicity =320ms - FFS additionally support other LP-SS periodicities - FFS details on additional sync signal - FFS additional sync signal is configurable and/or conditionally present Agreement For RRC idle/inactive state, for the case where associated CD-SSB and initial DL BWP have the same SCS at least when the associated CD-SSB and LP-WUS are on the same carrier - The single SCS for LP-WUS/LP-SS is same as the associated CD-SSB - FFS: Case where associated CD-SSB and initial DL BWP have different SCSs - FFS: Which initial BWP for RedCap Agreement For collecting LP-SS binary sequence results - Endorse the template for collecting LP-SS binary sequence results in R1-2410925 - Email discussion for collecting LP-SS binary sequences by 10th Jan. - Please include your cross-checking results (e.g., a separate excel sheet) in the t-doc for #120.

In this contribution, we discuss the waveform design and sequence design of LP-WUS and LP-SS.

LP-WUS design

In this section, we discuss the LP-WUS signal design and LP-WUS structure.

LP-WUS signals design

This section discusses the detailed design of the LP-WUS, including the OOK waveform and overlaid OFDM sequences design.

OOK-1/OOK-4 waveform

- M value of OOK-4

In RAN1#118, OOK-4 with M=4 for 15KHz SCS was agreed. Whether the OOK-4 with M=1 is supported still need to be discussed. The DFT processor of OOK-4 with M=1 can reduce the PAPR of LP-WUS compared to that of OOK-1. It can allow gNB transmit LP-WUS with higher transmission power, which can enhance the coverage of LP-WUS. Furthermore, considering same OOK chip rate for both OOK-1 and OOK-4 with M=1, the OOK-1 can be specified by a special case of OOK-4, e.g., while M is not configured by gNB. The OOK-4 with M=1 can be supported with configurable M value without bringing additional spec effort. Thus, the OOK waveform of LP-WUS can be specified by a configurable variable M with value range {1, 2, 4}.

Proposal 1: OOK-4 with M=1 is the DFT-S-OFDM waveform with low PAPR and should be supported.

Proposal 2: The OOK waveform of LP-WUS can be specified by a configurable variable M with value range {1, 2, 4}.

- SCS configuration for LP-WUS

In RAN1#119, the single SCS for LP-WUS/LP-SS is same as the associated CD-SSB was agreed for the case where associated CD-SSB and initial DL BWP have the same SCS for RRC_IDLD/INACTIVE states. The case where associated CD-SSB and initial DL BWP have different SCS need to be discussed. If the SCS of PDCCH monitoring on the initial BWP has different SCS to that of SSB, the timing and the offset value of PDCCH related to the SSB needs to be known by the UE. Since the PDCCH location cannot be derived from the timing information from SSB directly, it would be complex for LP-WUR determining the MO location of LP-WUS. Thus, it is reasonable that only support the case of same SCS between initial DL BWP and CD-SSB if the LP-WUS is configured.

Proposal 3: Not support the case of different SCS between initial DL BWP and CD-SSB for the LP-WUS feature.

Overlaid OFDM sequence of LP-WUS

- How to overlaid OFDM sequences with OOK waveform

In RAN1#118, Manchester coding was supported for LP-WUS. The OFDM sequence overlaid with OOK waveform before or after Manchester coding at each OOK ON chip is needed to be further clarified. It is a general understanding that OFDM sequences are overlaid after Manchester coding at each OOK ON chip. Thus, the OFDM sequences are overlaid after Manchester coding at each OOK ON chip should be clarified.

Proposal 4: It is needed to be clarified that the OFDM sequences are overlaid after Manchester coding at each ON chip.

- Number of candidates overlaid OFDM sequences per OOK ON symbol

As agreed in RAN1#119, the maximum 4 candidates overlaid sequences per OOK ON chip was supported for OOK-4 with M=4. The maximum number of candidates overlaid sequences for M=1, 2 needs to be discussed.

Agreement—RAN1#118bis Regarding the maximum number of candidate overlaid sequences to carry LP-WUS information per OOK ON chip for one cell, down-select from the below: - 4 - 8 - 16 - 32 - 64 for M not larger than 2 - FFS whether the same or different set of candidate overlaid sequences are used for different cells Agreement-RAN1#119 Regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell: - support maximum 4 candidates overlaid sequences for M=4

The maximum number of overlaid sequences would depend on the maximum number of wakeup indication in a LP-WUS and the overlaid ZC sequence length. Generally, the maximum number of available overlaid ZC sequence can be derived from the ZC sequence length, i.e. Lzc-1, where Lzc is the overlaid ZC sequence length. Accordingly, the maximum number of overlaid sequences can be 127 (Lzc=128) and 63 (Lzc=64) for M=1 and M=2, respectively. The derivation of sequence length Lzc is discussed in following section. For RRC_IDLE/INACTIVE modes, the maximum number of wakeup indication in a LP-WUS would depend on the number of the subgroup indication per PO. It was agreed in RAN1#118bis that the maximum number of subgroups per PO is 32. The maximum number of wakeup indication in a LP-WUS is 33 for carrying 32 subgroups specific indication and 1 common indication for all subgroups in the paging group. Thus, 33 candidates overlaid sequences can be supported for M=1 and M=2.

However, the support of 4 maximum candidates overlaid sequences for M=1 and M=2 was discussed with consideration on larger number of candidates overlaid OFDM sequences leading to higher LP-WUR detection complexity and obvious performance degradation. These aspects are analyzed as following:

- Detection complexity: The OFDM-LP-WUR only needs to blind detect one subgroup specific sequence associated with its configured subgroup index and one common sequence for RRC_IDLE/INACTIVE modes UEs, regardless of the number of maximum candidate overlaid sequences. Thus, the 33 candidate sequences would not increase the detection complexity. Also, it would not increase the memory footprint since only two sequence needed to be stored regardless of the maximum number of candidate overlaid OFDM sequence.

- Detection performance: The detection performance of LP-WUS mainly depends on the number of sequences that UE aims to detect. It is not related to the maximum number of sequences that can be transmitted from gNB side. Regardless of the maximum number of candidate overlaid OFDM sequence, only two sequences are configured to be detected. Thus, the detection performance cannot be degraded by the maximum number of candidate overlaid OFDM sequence.

As above discussion, maximum 33 candidates overlaid sequences to carry LP-WUS information per OOK ON chip for M=1 and 2 can be supported.

Proposal 5: Support maximum 33 candidates overlaid sequences to carry LP-WUS information per OOK ON chip for M=1 and M=2.

- The length of Overlaid OFDM sequences

For sequence length LZC of overlaid OFDM sequences, the following two alternatives were agreed in RAN1#119:

- Alt1:, with

- Alt2:

#### The main difference between Alt1 and Alt2 is the overlaid sequences length depends on the FFT size with 2k or the number of PRB X. The processing ability of the FFT processor for OOK-4 is simpler and faster than that of DFT/LS processor. It is much easier for gNB to apply the FFT processor than DFT/LS processor with single FFT engine sharing by all signal processing. Though the DFT/LS processor is more flexible for matching the size with the number of available REs, the DFT/LS processor is one of the intensive and complicated calculations at gNB. Considering the sequence length after FFT may be less than the available REs for LP-WUS, the RE mapping to the FFT size could be achieved by zero padding before the sequence and/or after the end of sequence. For example, the sequence length of s(n) is 128 bits after 128 points FFT, the sequence after 4 bit zero padding s’(n)=[0 0 s(n) 0 0] can be mapping to the 132 REs in the bandwidth of LP-WUS.

Proposal 6: Support Alt1:, with .

- Information carried by overlaid OFDM sequences

For information carried by overlaid OFDM sequences, the following Option 2-2 was agreed in RAN1#119. How the information carried by overlaid OFDM sequences is discussed in this section.

- Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options for potential down-selection.

  - Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s).

For OOK-4 with M=1/2, the maximum number of overlaid OFDM sequence(s) can be 33 as analysed above. Thus, one sequence can be selected from 32 subgroup specific sequences and 1 common sequence and repeated at each OOK ON chip after Manchester coding. The OFDM-based LP-WUR can obtain the full LP-WUS information by the OFDM sequence in one OOK ON chip.

For OOK-4 with M=4, there are 4 candidates sequences carrying LP-WUS information in each OOK ON chip. How the information bits carried by 4 candidates overlaid OFDM sequences is FFS and following two schemes are discussed:

- Scheme 1: One overlaid sequence can carry part of subgroup specific indication or the common indication for all subgroups independently in one PO.

- Scheme 2: The codepoint is determined by N1 number of overlaid sequences jointly to identify the specific subgroup specific or common signalling for all subgroups in one PO.

For scheme 1, one overlaid sequence can be configured with 3 codepoints carrying 3 subgroups indication (or 2 subgroup specific indication and 1 common indication) and 1 codepoint with no wake up information, as shown in Table 1. The overlaid sequences in each OOK ON chip carry the codepoints indicating the different subset of subgroups. It needs 11 OOK ON chips with sequences in different chips representing different wakeup information for 32 subgroup UEs in one PO. An example of the mapping between candidate sequence indexes in each OOK ON chip and wakeup information for 32 subgroup UEs is shown in Table 1. An OFDM-based LP-WUR only needs to detect the overlaid OFDM sequences on one or two OOK ON chips, which is associated with its own subgroup index and common indication.

Table 1: The mapping between candidate sequence indexes in each OOK ON chip and wakeup information for 32 subgroup UEs (Scheme 1)

| OOK ON symbol index | Candidate Sequence index #1 | Candidate Sequence index #2 | Candidate Sequence index #3 | Candidate Sequence index #4 |
| --- | --- | --- | --- | --- |
| 1 | Subgroup #1 | Subgroup #2 | Subgroup #3 | No wake up |
| 2 | Subgroup #4 | Subgroup #5 | Subgroup #6 | No wake up |
| i | Subgroup #(i-1)*3+1 | Subgroup #(i-1)*3+2 | Subgroup #(i-1)*3+3 | No wake up |
| 11 | Subgroup #31 | Subgroup #32 | Common indication | No wake up |

For scheme 2, a codepoint is determined by the combination of multiple overlaid OFDM sequences jointly and needs at least 3 sequences from 3 different OOK ON chips to indicate the 32 subgroup indexes or the common indication. The 3 sequences are transmitted separately on 3 OOK ON chips and the group of 3 overlaid OFDM sequences are repeatedly transmitted in every 3 OOK ON chips in the subsequent OOK ON chips. An example of the mapping between candidate sequence indexes in each OOK ON chip and wakeup information for 32 subgroup UEs are shown in Table 2. An OFDM-based LP-WUR needs to determine the wakeup information by 3 OFDM overlaid sequences in 3 OOK ON chips. Compared to the required one or two sequences detection in scheme 1, this scheme would increase the FAR due to the multi-sequence joint detection. Furthermore, if one of three sequences is miss detected in scheme 2, the OFDM-based LP-WUR cannot derive the wakeup information correctly. Thus, the BLER of scheme 2 would be worse than that of scheme 1.

Table 2: The mapping between OFDM sequence indexes and wakeup information subgroup for 32 subgroup UEs (Scheme 2)

| Wakeup information | Candidate Sequence index in First OOK ON symbol（N1） | Candidate Sequence index in Second OOK ON symbol（N2） | Candidate Sequence index in Third OOK ON symbol（N3） |
| --- | --- | --- | --- |
| Subgroup #1 | Seq#2 | Seq#1 | Seq#1 |
| Subgroup #2 | Seq#3 | Seq#1 | Seq#1 |
| Subgroup #3 | Seq#4 | Seq#1 | Seq#1 |
| Subgroup #4 | Seq#1 | Seq#2 | Seq#1 |
| … | … | … | … |
| Subgroup #32 | Seq#1 | Seq#3 | Seq#3 |
| Common(#33) | Seq#2 | Seq#3 | Seq#3 |
| Note: Subgroup index = (N3-1) * 8 +(N2-1)*4+N1-1, |  |  |  |

As analyzed above, the scheme 1: One overlaid OFDM sequence carries subset of subgroups specific indication or the common indication in one PO is preferred due to better detection performance. The subset of the paging subgroups would be different at different OOK ON chips. The RRC_IDLE/INACTIVE modes UEs need to detect one or two of the configured overlaid OFDM sequences to derive the wakeup information for the specific subgroup or common signalling for all subgroups.

Proposal 7: Support scheme 1: One overlaid OFDM sequence carries subset of subgroups specific indication or the common indication in one PO.

- CS/root(s)

In RAN1#119, the CS and/or root(s) used for overlaid OFDM sequence in the time domain derived from RRC signaling are needed to be further discussed.

Considering the CS of time domain ZC sequence is to have different frequency subband after DFT signal processing at the receiver, the CS would be fixed according to the bandwidth location of LP-WUS. Since the choice of root sequence index had been discussed in Rel-15 RACH Preamble, it is straightforward to reuse the root values defined in RACH Preamble in [2], in where the root value with ith index is i/2+1 (when i is even) and LRA-(i/2+1) (when i is odd) and i is the logical index and LRA is the Preamble sequence length. For LP-WUS, the root value for overlaid OFDM sequence in the root set can be derived from the root index i by i/2+1 (when i is even) and Bzc-(i/2+1) (when i is odd), in where Bzc is the base ZC overlaid sequence length as agreed in RAN1#119. Accordingly, the set of root values for M=1/2/4 are shown in Table 3, which can be specified in TS38.211 similar to Rel-15 RACH Preamble. For example, the Bzc is 31 for M=4 (Lzc=32), four values in the root set can be derived by {0/2+1,31-(1/2+1), 2/2+1,31-(3/2+1)}={1, 30, 2 , 29}.

Different candidates of OFDM sequences per OOK ON chip should be mapped to different roots index in the set of root values. The mapping between candidates of OFDM sequences and the root value can be discussed after the principle of ZC root values selection is agreed.

Table 3: The set of root values for different M

| M value | Set of root value |
| --- | --- |
| 1 | {1, 126, 2, 125, 3, 124, 4, 123, 5, 122, 6, 121, 7, 120, 8, 119, 9, 118, 10, 117, 11, 116, 12, 115, 13, 114, 14, 113, 15, 112, 16, 111,17} |
| 2 | {1, 62, 2, 61, 3, 60, 4, 59, 5, 58, 6, 57, 7, 56, 8, 55, 9, 54, 10, 53, 11, 52, 12, 51, 13, 50, 14, 49, 15, 48, 16, 47,17} |
| 4 | {1, 30, 2, 29} |

Proposal 8: The CS of overlaid OFDM sequence should be fixed according to the bandwidth location of LP-WUS.

Proposal 9: The set of root values for overlaid ZC sequence can reuse the principle of the root values specified for RACH Preamble in TS38.211 as shown in Table 3.

Table 3: The set of root values for different M

| M value | Set of root value |
| --- | --- |
| 1 | {1, 126, 2, 125, 3, 124, 4, 123, 5, 122, 6, 121, 7, 120, 8, 119, 9, 118, 10, 117, 11, 116, 12, 115, 13, 114, 14, 113, 15, 112, 16, 111,17} |
| 2 | {1, 62, 2, 61, 3, 60, 4, 59, 5, 58, 6, 57, 7, 56, 8, 55, 9, 54, 10, 53, 11, 52, 12, 51, 13, 50, 14, 49, 15, 48, 16, 47,17} |
| 4 | {1, 30, 2, 29} |

Proposal 10: The mapping between wakeup information and the root values should be discussed after the principle of the ZC root sequences selection are agreed.

LP-WUS structure

General structure for LP-WUS

The LP-WUS structure should be designed to carry the information for UE waking up the MR and the additional information to assist UE synchronization. The low-cost LP-WUS expects to have the oscillator with high frequency instability. In particular, the LP-SS periodicity might be long. The LP-WUR might not maintain synchronization with the network. Synchronization is critical for LP-WUS detection, especially for the case when information is carried by encoded bits. If LP-WUR uses a low accuracy clock to keep lower power consumption, it may not keep synchronization with gNB. The necessity of preamble for LP-WUS is analyzed with evaluation sync performance of LP-SS in section 3.3.

Moreover, the preamble is an on-demand signal associated with wake-up codepoint. Thus, specified preamble has little impact on the resource overhead of LP-WUS. It can improve the tolerance to the T/F error of LP-WUR, which can allow longer LP-SS periodicity being configured. Considering the LP-SS is an always-on signal, the resource overhead of LP-SS can be reduced effectively with longer periodicity.

Figure 1: Structure of LP-WUS

Proposal 11: It is recommended to support a LP-WUS structure with wake-up information preceded by a fixed preamble sequence for assisting synchronization.

Information module design

- RRC_IDLE/INACTIVE modes

For RRC_IDLE/INACTIVE modes, a codepoint can consist of one or more sequence to carry wakeup information. The following two schemes i.e. Scheme 1: one stage sequence-based codepoint design and Scheme 2: multi-stage sequence-based codepoint design are introduced as following:

- Scheme 1: One-stage sequence-based codepoint design

  - A LP-WUS indicates one subgroup UE wake-up (subgroup-specific codepoint) or all subgroup wakeup (subgroup-common codepoint) from N subgroups by a one-stage codepoint.

Figure 2: One stage sequence-based codepoint design

For one-stage sequence-based codepoint as shown in Figure 2, the information module is a single orthogonal/quasi-orthogonal sequence as a codepoint representing the wakeup information for one subgroup UEs or all subgroup UEs. A LP-WUS indicates a codepoint value corresponding to a subgroup-specific codepoint or a subgroup-common codepoint. The subgroup-specific codepoint indicates only one subgroup wakeup and the subgroup-common codepoint indicate all subgroup wakeup. For example, there are 8 codepoints represent 8 single subgroup wakeup and one common codepoint represents all 8 subgroups wakeup, respectively. The total number of required codepoint is 9. When only one subgroup is paged, one of the 8 subgroup-specific codepoints for the paging subgroup is transmitted. If more than one subgroup is paged, the subgroup-common codepoint is transmitted by gNB to wake up all subgroups.

- Scheme 2: Multi-stage sequence-based codepoint design

  - A LP-WUS indicates one subgroup UE wake-up (subgroup-specific codepoint) or all subgroup wakeup (subgroup-common codepoint) from N subgroups by a multi-stage codepoint.

For scheme 2, the information module can consist of multiple orthogonal/quasi-orthogonal sequences to indicate a subgroup UEs. The codepoint is determined by multiple sequences jointly to identify the specific subgroup specific or common signaling for all subgroups in one PO. The OOK-based LP-WUR can derive the subgroup UE index from combining the sequence index in each stage. It would increase the number of the information carried by LP-WUS with the combined multiple information modules. An example of the mapping between wakeup information and index of two-stage sequence-based codepoint is shown in Table 4. Assuming a PO consists of 32 subgroups UE, the LP-WUS can carry a common indication by a single Walsh sequence, and a subgroup specific indication by two-stage Walsh sequence. Compared to scheme 1, the multi-stage sequence design can increase the capacity of LP-WUS and also can reduce the resource overhead of LP-WUS. Thus, the scheme 2 can be supported.

Table 4: The mapping between wakeup information and index of two-stage sequence-based codepoint

| Wake up information | Index of OOK codepoint | Codepoint Structure （8 bit walsh_1 + 4 bit walsh_2） |
| --- | --- | --- |
| Subgroup UE #1 | 1 | [seq_1; seq_1] |
| Subgroup UE #2 | 2 | [seq_2 ; seq_1] |
| …… | ….. | ……. |
| Subgroup UE #32 | 32 | [seq_8 ; seq_4] |
| Common | None | [seq_1;None] |
| Note: Codepoint index = (sequence index in second stage -1) * sequence number of second stage + sequence index in first stage; |  |  |

Observation 1: A two-stage sequence based LP-WUS can indicate 32 subgroup UEs by 8 bit Walsh sequence combine with 4 bit Walsh sequence.

Observation 2: A codepoint value can by concatenated multiple sequences can increase the capacity of wakeup indications.

Proposal 12: Support scheme 2: The codepoint based on Multi-stage sequence joint detection can carry subgroup specific indication and common signaling indication for all subgroups.

- RRC_CONNECTED mode

For RRC_CONNECTED mode UEs, two following options were discussed in RAN1#119:

- Option 1: A bitmap with each bit corresponding to [one or more] UEs.

- Option 3: A codepoint value corresponding to [one or more] UEs.

For option 1, it would have the following considerations:

- Improving detection performance by forward error correction coding: The bitmap with each bit corresponding to single information of wakeup indication of one or more UEs requires high reliability of detection performance in terms of bit error rate (BER). The information module of LP-WUS needs to be protected by channel coding to improve the detection performance in fast fading channel. The detection performance would be heavily degraded from bad channel condition, e.g., larger timing/frequency error.

- Additional error detection by CRC: The error detection scheme, such as CRC, is needed to ensure the correctness of bit detection to meet the target FAR for the bitmap wakeup information. At least 6 bits CRC is needed to satisfy 1% FAR but carry no information. The error detection and error correction would increase the overhead of LP-WUS.

- High detection complexity: The LP-WUR first needs to decode the bit of the received OOK signals at each OOK chip. The length of the bit sequence after the OOK demodulation would feed into the channel decoder if channel coding is used for error correction in battling the fast fading channel. After channel decoding, the CRC check is performed for the error detection of the decoding bits. This operation would increase the detection complexity and increase the power consumption.

For option 2, there are the following aspects to be considered:

- The detection performance: The LP-WUS after OOK demodulation at each symbol in time domain forms an orthogonal/quasi-orthogonal sequence. The LP-WUS sequence detection is a non-coherent detection through simple gate operation with spreading gain. Moreover, the FAR from noise can be controlled below 1% through the determination of gate by UE implementation without additional resource overhead.

- The capacity of wakeup indication: The information module can consist of multiple orthogonal/quasi-orthogonal sequences with each orthogonal/quasi-orthogonal sequence represents one set of information. It would increase the number of the information carried by LP-WUS with the combined multiple information modules.

- Low detection complexity: The UE can receive sequences based on a simpler adder. By accumulating the information of each OOK chip received with the local sequence by the adder, the reception information is obtained at the end of the signal reception occasion by comparing with a pre-defined threshold. This method not only simplifies reception but also has extremely low power consumption.

Proposal 13: The sequence-based LP-WUS should be supported for RRC_CONNECTED mode.

The same design of wakeup indication information carried by codepoints for both RRC_CONNECTED mode and RRC_IDLE/INACTIVE modes would reduce the standardization effort and gNB/UE implementation complexity of LP-WUS. The wakeup indication could be achieved by multi-stage sequence joint indication to increase the capacity of wakeup indication.

Proposal 14: Multi-stage sequence-based LP-WUS can provide large capacity and flexibility in wakeup indication for UEs in both RRC_IDLE/INACTIVE modes and RRC_CONNECTED mode.

Evaluation results

Three schemes of LP-WUS structures including one-stage Walsh sequence based LP-WUS, two-stage Walsh sequences based LP-WUS and bitmap-based LP-WUS are evaluated in this section. The detail of simulation assumption is shown in Table 8 in Appendix. The three schemes are introduced as following:

- Scheme 1: One-stage Walsh sequence based LP-WUS: A 16-bits Walsh sequence to carry the wakeup information for 16 UEs.

- Scheme 2: Two-stage Walsh sequences based LP-WUS: A 8-bits Walsh sequence and another 8-bits Walsh sequence are concatenated to carry the wakeup information for 64 UEs.

- Scheme 3: Bitmap-based LP-WUS consist of 10 bits information indicating 10 UEs information and 6 bits CRC to satisfy 1% FAR.

Figure 3 shows the detection performance of the three schemes, it can be observed that the detection performance of the one-stage Walsh sequence and two-stage Walsh sequences can obtain 6.4dB gain and 4.9dB gain than that of bitmap-based LP-WUS at 1% BLER with the ideal assumption of perfect timing and frequency compensation in the coherent demodulation, respectively. One-stage Walsh sequence has advantage of double the processing gain but less capacity of wakeup indication comparing to those of two-stage Walsh sequences. However, the BLER performance gain of one-stage Walsh sequence is less than 3 dB over that of two-stage Walsh sequences (1.5dB at 1% BLER). The capacity of wakeup indication for two-stage Walsh sequence is 4 times of that of one-stage Walsh sequence, which implies the overhead of wakeup indication is 4 times smaller with two-stage Walsh sequences.

Figure 3: Detection performance of Scheme 1~3

Observation 3: The one-stage Walsh sequence and two-stage Walsh-sequence show better detection performance with 6.4dB gain and 4.9 dB gain over that of bitmap-based LP-WUS at 1% BLER with the ideal assumption of perfect timing and frequency compensation in the coherent demodulation, respectively.

In Table 5, three schemes are compared with the aspects of resource overhead, target SNR, FAR, capacity and detection complexity. The detailed assumptions of LP-WUS are provided in Table 8 in Appendix

Table 5: Comparison of scheme 1 ~ scheme 3 for RRC_CONNECTED mode

| Scheme | Resource overhead | Target SNR | FAR | Capacity | Detection complexity |
| --- | --- | --- | --- | --- | --- |
| Scheme 1: One-stage Walsh sequence | 8 OFDM symbols | 2.8dB | <1% FAR through the determination of gate | 16 UEs | Simple: - non-coherent detection through simple gate operation |
| Scheme 2: Two-stage Walsh sequence | 8 OFDM symbols | 4.3dB | <1% FAR through the determination of gate | 64 UEs | Simple: - non-coherent detection through simple gate operation |
| Scheme 3: bitmap-based LP-WUS | 8 OFDM symbols | 9.2dB | <1% FAR through CRC | 10 UEs | Complex: - Decode CRC bit - Channel estimation - coherent demodulation |

Overall, scheme 2: Two-stage Walsh sequence based LP-WUS among the three schemes would provide higher capacity of wakeup indication and simpler detection complexity with acceptable detection performance.

Proposal 15: Multi-stage LP-WUS design is supported for RRC_CONNECTED mode.

LP-SS design

In this section, we would discuss the OOK waveform of LP-SS signal design and LP-SS sync procedure.

LP-SS signal design

Signal structure of LP-SS

The cell-specific LP-SS structure can be same as LP-WUS as shown in Figure 4, serving cell ID information preceded by a fixed preamble sequence to trigger the detection of the sequence without hypothesis testing for synchronization. The information module is used to carry truncated Cell ID information by 4 candidate LP-SS binary sequences.

Figure 4: Structure of LP-SS

Proposal 16: For structure of LP-SS, a fixed known preamble sequence concatenated with truncated cell ID information module could be considered in the LP-SS sequence design.

Proposal 17: The LP-SS should have the unified design with LP-WUS for low complexity in implementation and detection with low power consumption.

OOK waveform

In RAN1#116bis, the following two options for OOK waveform of LP-SS were discussed and the following working assumption was agreed:

- Option 1: OOK-1;

- Option 2: OOK-4 with M= 2, 4, FFS:1,8,16;

Considering the OOK-based LP-WUR would detect both LP-SS and LP-WUS, the OOK waveform configuration for LP-SS is same as that of LP-WUS would be simpler for LP-WUR detection and low standardization complexity, e.g., unified OOK waveform for both LP-SS and LP-WUS. Thus, both OOK-1 and OOK-4 with M=1/2/4 can be supported for LP-SS waveform as the OOK waveform of LP-WUS.

Proposal 18: Both OOK-1 and OOK -4 with M=1/2/4 can be supported for LP-SS waveform.

For the M value for LP-WUS and LP-SS, the following three alternatives were agreed in RAN1#119:

- Alt1: The M values for LP-WUS and LP-SS are always same.

- Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS.

- Alt3: The M values for LP-WUS and LP-SS can be configured to be same or different.

For Alt1, it would be simpler in UE implementation for LP-WUR detection for both LP-WUS and LP-SS by same sampling rate and chip duration for OOK symbol. Moreover, the sync performance of LP-SS is not related to the M value. The synchronization performance only depends on the sampling rate as shown in Table 6. Different M-value configuration for LP-SS and LP-WUS does not show any benefit for LP-SS sync accuracy in Alt 2 and Alt 3.

Proposal 19: Unified OOK waveform for LP-SS and LP-WUS can be supported.

Proposal 20: Support Alt1: The M values for LP-WUS and LP-SS are always same.

The binary LP-SS sequence type

In RAN1#118, three candidate sequences, such as GOLD sequence, M sequence and Computer searched sequence, were considered to be the binary LP-SS sequence. In which, both GOLD sequence and M sequence are NR existing sequences and have low cross-correlation for both synchronous and asynchronous multiple signal receptions. Compared to computer searched sequence, the two existing NR sequences can ease the spec work for discussion and evaluation and have little impact to existing gNB and UE implementation.

Proposal 21: Either Gold-sequence or M-sequence can be supported for binary LP-SS sequence design.

Coding

Channel coding uses the redundant information to battle the fading channel for the LP-WUR, which can improve the detection and decoding performance of OOK waveform. Most of the channel coding schemes requires high complexity decoding algorithm (i.e. Polar code, LDPC code) and high power consumption for improving the signal detection and decoding performance. A simple channel coding scheme i.e. Manchester coding with simple decoding algorithm and negligible power consumption was studied in Rel-18 LP-WUS/WUR with substantial gain in LP-SS detection performance. Thus, the Manchester coding should be the candidate as the LP-SS channel coding method.

Proposal 22: The Manchester coding can be the supported for LP-SS as coverage enhancement same with LP-WUS.

Sync procedure of LP-SS

Timing/frequency offset estimation algorithm of LP-SS

The LP-SS is designed for the LP-WUR to perform the timing and frequency synchronization. The LP-WUR would have limited capability in computation for the algorithm of the time and frequency synchronization to avoid the large power consumption. The timing and frequency estimation algorithm for LP-WUR should be simple without complicated mathematical operations to minimize the UE power consumption. The timing and frequency domain sliding window algorithm is a simple algorithm based on comparison of correlation peaks of samples with the sliding window in time from the reference time or with the sliding window in center frequency from the reference sub-band frequency.

The procedure of timing offset estimated by LP-SS is shown in Figure 5 as following steps:

- Step 1: LP-WUR obtains sampling points from the timing reference in a sliding window with a constant or over the sampling rate of the OOK-1/4 waveform.

- Step 2: LP-WUR decodes the OOK symbols within the sliding window by the energy of the envelop detector.

- Step 3: LP-WUR performs the correlation by sum the energy of each sampling point and obtain the peak point of correlation output in the sliding window as the timing offset.

- Step 4: The timing value can be estimated by the timing offset between the peak point and of the correlation output relative the reference point, in which the reference point is the original reference time of the LP-SS waveform.

The peak of the correlation from each point would be within the range of +/- 0.5 OOK symbol width from the reference point assuming 1/2 Manchester coding is applied. The refined timing could be done with over-sampling within an OOK symbol.

Figure 5: Procedure of timing offset estimation by LP-SS

The procedure of frequency offset estimated by LP-SS is shown in Figure 6 as following steps:

- Step 1: LP-WUR obtains sampling points from sync point #1 to sync point #N, respectively.

- Step 2: LP-WUR decodes the OOK symbols with the sync point by envelop detector.

- Step 3: LP-WUR performs the correlation by summing up the energy of each sampling point and obtains the peak point of correlation output in the sync point as the frequency after estimation.

- Step 5: The frequency offset value can be estimated by the offset between frequency location of sync point by selecting the max peak point and frequency reference point, in which the reference point is the center frequency of LP-SS.

Figure 6: Procedure of frequency offset estimation by LP-SS

Evaluation results of LP-SS

- Timing error estimation performance

The binary sequence for LP-SS is determined to satisfy sync accuracy requirement. The synchronization performance of LP-SS with different M values and sequence length (bit length after Manchester coding) are shown in Table 6. The details of simulation assumptions of LP-SS are shown in Table 9 in Appendix. The initial timing error is assumed to be random with uniform distribution (-6.4us, 6.4us). It can be observed that:

- The synchronization performance of LP-SS with time domain M-sequence and overlaid ZC OFDM sequence are same at 0.23us for OOK-4 with M=1/2/4 when 15.36MHz oversampling is performed for all M values of OOK-4.

- The synchronization performance of LP-SS with time domain M-sequence and overlaid Zadoff-Chu OFDM sequence are same at 0.23us for different number of OFDM symbols duration of LP-SS when 15.36 MHz oversampling is performed for different number of OFDM symbols.

Since the sequence length L and the M value have no impact to the sync accuracy performance, the shortest sequence length with lower resource overhead can be supported.

Table 6: Sync accuracy T of LP-SS based on M-sequence (Sliding window= (-25us, 25us))

| M | Sequence length L (after Manchester coding) | LP-SS duration | 90% CDF of sync accuracy T (us) |
| --- | --- | --- | --- |
| 1 | 4 | 4 OFDM symbols | 0.23us |
|  | 8 | 8 OFDM symbols | 0.23us |
| 2 | 8 | 4 OFDM symbols | 0.23us |
|  | 12 | 6 OFDM symbols | 0.23us |
|  | 16 | 8 OFDM symbols | 0.23us |
| 4 | 16 | 4 OFDM symbols | 0.23us |
|  | 32 | 8 OFDM symbols | 0.23us |

Observation 4: The 90% CDF of sync accuracy T can be achieved at 0.23us for M-sequence based LP-SS with M =1/2/4 with 15.36 MHz oversampling.

Observation 5: The 90% CDF of sync accuracy T can be achieved at 0.23us for M-sequence based LP-SS with 4~14 OFDM symbols duration of LP-SS with 15.36 MHz oversampling.

Proposal 23: The sequence length L=4/8/16 can be supported for M=1/2/4, respectively.

- Frequency error estimation performance

- The LP-WUR can have frequency and timing offset compensation through the frequency and timing offset estimation from LP-SS, which the timing drift is derived by the sliding window of transient region, i.e., ΔT = Fr*T ±0.5 * F’ *T2, where Fr is residual frequency error, T is interval between LP-WUS and last LP-SS, and F’ is frequency wonder with 0.1ppm/s.

- The frequency error estimation performance of LP-SS and maximum interval between LP-SS and LP-WUS are shown in Table 7. The detail simulation assumptions of LP-SS are shown in Table 10 in Appendix. The maximum tolerant timing error (timing drift ΔT) is assumed to be 5us, 2us and 1us for OOK-1, OOK-4 with M=2 and M=4 [3], respectively. The maximum interval between LP-SS and LP-WUS can be derived from the uniform distributed timing drift model.

- It can be seen the maximum interval between LP-SS and LP-WUS is 165ms, 66ms and 33ms for M=1, M=2 and M=4, respectively. In other words, the detection performance of LP-WUS would be degraded heavily caused by timing error while the periodicity of LP-SS is larger than165ms, 66ms and 33ms for OOK-1, OOK-4 with M=2 and M=4, respectively.

- Table 7: 90% CDF of residual frequency error (sync point= [-200:10:200], initial frequency error = random (-200, 200)ppm)

| (OOK waveform, Sequence length L (after Manchester coding)) | 90% CDF of Residual frequency error(ppm) | Maximum interval between LP-SS and LP-WUS(ms) |
| --- | --- | --- |
| (OOK-1 , 4bit) | 30.3ppm | 165ms |
| (OOK-4 with M=2, 8bit) | 30.3ppm | 66ms |
| (OOK-4 with M=4, 16bit) | 30.3ppm | 33ms |

Observation 6: The 90% CDF of residual frequency error can be reduced at 30.3ppm for M-sequence based LP-SS with M=1, M=2 and M=4.

Observation 7: The maximum interval between LP-SS and LP-WUS is 165ms, 66ms and 33ms for M=1, M=2 and M=4, respectively.

Observation 8: The detection performance of LP-WUS would be degraded heavily caused by timing error while the periodicity of LP-SS is lager than165ms, 66ms and 33ms for M=1, M=2 and M=4, respectively.

Additional sync signal

The LP-WUR has the oscillator with high frequency instability. In particular, the LP-SS periodicity might be long. The LP-WUR might not maintain synchronization with the network. Synchronization is critical for LP-WUS detection, especially for the case when information is carried by encoded bits. LP-SS can provide synchronization. Whether additional synchronization signal is needed highly depends on whether LP-SS alone is sufficient for meet the time and frequency synchronization requirement for LP-WUS detection. The following three options were discussed in RAN1#119.

- Option 1A: No additional sync signal, LP-SS periodicity =320ms

  - Additionally support at least one of [80ms,160ms]

    - FFS: whether different values can be applicable for different M values

- Option 1B: No additional sync signal, LP-SS periodicity =320ms

  - No additional support of other periodicities

- Option 2: With additional sync signal, LP-SS periodicity =320ms

  - FFS additionally support other LP-SS periodicities

  - FFS details on additional sync signal

  - FFS additional sync signal is configurable and/or conditionally present

As the frequency error estimation performance of LP-SS evaluated in Table 7, the LP-SS period cannot longer than 66ms for guaranteeing the frequency synchronization requirement for LP-WUS detection. Moreover, the preamble is an on-demand signal associated with wake-up codepoint. Thus, specified preamble has little impact on the resource overhead of LP-WUS. It can improve the tolerance to the T/F error of LP-WUR, which can allow longer LP-SS periodicity being configured. Considering the LP-SS is an always-on signal, the resource overhead of LP-SS can be reduced effectively with longer periodicity. For simplify standard effort, the sequence design for LP-SS can be reuse for the additional sync signal.

Proposal 24: Support additional sync signal for LP-SS period longer than 165ms, 66ms and 33ms for M=1, M=2 and M=4, respectively.

Proposal 25: The sequence design for LP-SS can be reused for the additional sync signal.

Coverage improvement schemes for LP-WUS/LP-SS

In RAN1#118bis, the following schemes were discussed for LP-WUS/SS coverage enhancement:

- Power boosting: The power boosting scheme can be implemented based on the power allocation by the gNB when transmitting LP-WUS/SS, without the need to inform the LP-WUR. The LP-WUR can receive signals based on energy detection without knowing the transmission power of the base station. Therefore, it can be implemented at the gNB without spec impact.

- Time domain repetition: Time-domain repetition transmission is a classic and simple method for coverage enhancement, which can be directly applied to LP-WUS signals. This scheme should be supported.

- Frequency domain diversity with time domain repetition: Frequency domain diversity transmission may not enhance the coverage performance of LP-WUS signals. This is because LP-WUS is designed to be a low-power signal for waking up devices, and the benefits of frequency diversity, which typically counteract frequency-selective fading, might not be as critical for the short and infrequent transmissions of LP-WUS.

- Multiple beam repetition/sweeping: Similar to that of PEI, multiple beam repetition/sweeping should be supported at least for RRC_ILDE/INACTIVE mode UEs for LP-WUS/SS.

Proposal 26: Support at least time domain repetition and multiple beam repetition/sweeping for coverage enhancement for LP-WUS/LP-SS.

Conclusion

In this contribution, we discuss the waveform design and sequence design of LP-WUS and LP-SS and give the following observations and proposals:

Observation 1: A two-stage sequence based LP-WUS can indicate 32 subgroup UEs by 8 bit Walsh sequence combine with 4 bit Walsh sequence.

Observation 2: A codepoint value can by concatenated multiple sequences can increase the capacity of wakeup indications.

Observation 3: The one-stage Walsh sequence and two-stage Walsh-sequence show better detection performance with 6.4dB gain and 4.9 dB gain over that of bitmap-based LP-WUS at 1% BLER with the ideal assumption of perfect timing and frequency compensation in the coherent demodulation, respectively.

Observation 4: The 90% CDF of sync accuracy T can be achieved at 0.23us for M-sequence based LP-SS with M =1/2/4 with 15.36 MHz oversampling.

Observation 5: The 90% CDF of sync accuracy T can be achieved at 0.23us for M-sequence based LP-SS with 4~14 OFDM symbols duration of LP-SS with 15.36 MHz oversampling.

Observation 6: The 90% CDF of residual frequency error can be reduced at 30.3ppm for M-sequence based LP-SS with M=1, M=2 and M=4.

Observation 7: The maximum interval between LP-SS and LP-WUS is 165ms, 66ms and 33ms for M=1, M=2 and M=4, respectively.

Observation 8: The detection performance of LP-WUS would be degraded heavily caused by timing error while the periodicity of LP-SS is lager than165ms, 66ms and 33ms for M=1, M=2 and M=4, respectively.

Proposal 1: OOK-4 with M=1 is the DFT-S-OFDM waveform with low PAPR and should be supported.

Proposal 2: The OOK waveform of LP-WUS can be specified by a configurable variable M with value range {1, 2, 4}.

Proposal 3: Not support the case of different SCS between initial DL BWP and CD-SSB for the LP-WUS feature.

Proposal 4: It is needed to be clarified that the OFDM sequences are overlaid after Manchester coding at each ON chip.

Proposal 5: Support maximum 33 candidates overlaid sequences to carry LP-WUS information per OOK ON chip for M=1 and M=2.

Proposal 6: Support Alt1:, with .

Proposal 7: Support scheme 1: One overlaid OFDM sequence carries subset of subgroups specific indication or the common indication in one PO.

Proposal 8: The CS of overlaid OFDM sequence should be fixed according to the bandwidth location of LP-WUS.

Proposal 9: The set of root values for overlaid ZC sequence can reuse the principle of the root values specified for RACH Preamble in TS38.211 as shown in Table 3.

Table 3: The set of root values for different M

| M value | Set of root value |
| --- | --- |
| 1 | {1, 126, 2, 125, 3, 124, 4, 123, 5, 122, 6, 121, 7, 120, 8, 119, 9, 118, 10, 117, 11, 116, 12, 115, 13, 114, 14, 113, 15, 112, 16, 111,17} |
| 2 | {1, 62, 2, 61, 3, 60, 4, 59, 5, 58, 6, 57, 7, 56, 8, 55, 9, 54, 10, 53, 11, 52, 12, 51, 13, 50, 14, 49, 15, 48, 16, 47,17} |
| 4 | {1, 30, 2, 29} |

Proposal 10: The mapping between wakeup information and the root values should be discussed after the principle of the ZC root sequences selection are agreed.

Proposal 11: It is recommended to support a LP-WUS structure with wake-up information preceded by a fixed preamble sequence for assisting synchronization.

Proposal 12: Support scheme 2: The codepoint based on Multi-stage sequence joint detection can carry subgroup specific indication and common signaling indication for all subgroups.

Proposal 13: The sequence-based LP-WUS should be supported for RRC_CONNECTED mode.

Proposal 14: Multi-stage sequence-based LP-WUS can provide large capacity and flexibility in wakeup indication for UEs in both RRC_IDLE/INACTIVE modes and RRC_CONNECTED mode.

Proposal 15: Multi-stage LP-WUS design is supported for RRC_CONNECTED mode.

Proposal 16: For structure of LP-SS, a fixed known preamble sequence concatenated with truncated cell ID information module could be considered in the LP-SS sequence design.

Proposal 17: The LP-SS should have the unified design with LP-WUS for low complexity in implementation and detection with low power consumption.

Proposal 18: Both OOK-1 and OOK -4 with M=1/2/4 can be supported for LP-SS waveform.

Proposal 19: Unified OOK waveform for LP-SS and LP-WUS can be supported.

Proposal 20: Support Alt1: The M values for LP-WUS and LP-SS are always same.

Proposal 21: Either Gold-sequence or M-sequence can be supported for binary LP-SS sequence design.

Proposal 22: The Manchester coding can be the supported for LP-SS as coverage enhancement same with LP-WUS.

Proposal 23: The sequence length L=4/8/16 can be supported for M=1/2/4 , respectively.

Proposal 24: Support additional sync signal for LP-SS period longer than 165ms, 66ms and 33ms for M=1, M=2 and M=4, respectively.

Proposal 25: The sequence design for LP-SS can be reused for the additional sync signal.

Proposal 26: Support at least time domain repetition and multiple beam repetition/sweeping for coverage enhancement for LP-WUS/LP-SS.

References

- RAN1#119, 3GPP TSG RAN WG1 Meeting #119, Orlando, US, November 18th – 22nd, 2024.

- TS 38.211, Physical channels and modulation, v18.3.0.

- TR38.869, v18.0.0, Study on low-power wake-up signal and receiver for NR.

Appendix

Simulation assumption

Table 8: Simulation assumption of LP-WUS

| Parameter | Configuration |
| --- | --- |
| Carrier Frequency | 2.6GHz |
| BW of LP-WUS | 11 RB |
| SCS | 30kHz |
| Channel | TDL-C |
| TxRx | 1Tx1Rx |
| Sampling rate | 15.36MHz |
| Waveform of LP-WUS | OOK-4 with M =2 |
| One-stage Walsh sequence based LP-WUS | 16 bits Walsh sequence |
| Two-stage Walsh sequence based LP-WUS | 8 bits Walsh sequence #1+ 8 bits Walsh sequence #2 |
| bitmap-based LP-WUS | 10 bits information + 6 bits CRC |
| Coding method | 1/2 Manchester coding |

Table 9: Simulation assumption of LP-SS

| Configuration | Value |
| --- | --- |
| Maximum frequency error | -20ppm |
| Timing error | Random(-6.4 ,6.4)us |
| BW of LP-SS | 4.32 MHz |
| Channel | TDL-C |
| LP-SS sequence type | M-sequence |
| Overlaid OFDM sequence type | ZC-sequence |
| Coding method | 1/2 Manchester coding |
| (LP-SS length, M) | (L=4/8 M =1) (L=8/16/24, M =2) (L=16/32/56, M =4) |
| SNR | SNR=-3dB |
| Sampling rate | 15.36 MHz |
| Sliding Window | (-25us, 25us) |

Table 10: Simulation assumption of LP-SS

| Configuration | Value |
| --- | --- |
| Initial frequency error | Random(-200ppm ,200ppm) |
| LP-SS sequence type | M-sequence |
| Overlaid OFDM sequence type | M-sequence(reuse PSS sequence) |
| Coding method | 1/2 Manchester coding |
| (Sequence length L (after Manchester coding), OOK waveform) | (4, OOK-1) (8, OOK-4 with M =2) (16, OOK-4 with M =4) |
| KPI | 95% CDF of residual frequency error |
| SNR | SNR=-3dB |